3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210469

# e-Meeting, October 10th - 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | _ | rev | _ | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL BWP in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.SL BWP and subcarrier spacing information can be configured and obtained by UE via dedicated configuration information or cell-specific configuration information as specified in TS38.331. In current TS 38.213, only the parameter ‘SL-BWP-Config’ providing UE dedicated configuration information is captured for UE to obtain the SL BWP and subcarrier spacing configuration information, while the parameter ‘SL-BWP-ConfigCommon’ providing cell-specific configuration information is missing. 2. In TS 38.213, some parameter name alignment correction according to TS 38.331 is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Adding ‘SL-BWP-ConfigCommon’ as one parameter providing SL BWP and subcarrier spacing configuration information; 2. Changing Parameter ‘sl-BWP-Config’ to ‘SL-BWP-Config’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1.The UE behavior obtaining SL BWP and subcarrier spacing information from cell-specific configuration information is missing from TS 38.213. 2.The subcarrier spacing configuration information cannot be obtained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16, 16.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

16	 UE procedures for sidelink

A UE is provided by SL-BWP-Config or SL-BWP-ConfigCommon a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS 38.211]. For a resource pool within the SL BWP, the UE is provided by sl-NumSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by sl-SubchannelSize. The first RB of the first sub-channel in the SL BWP is indicated by sl-StartRB-Subchannel. Available slots for a resource pool are provided by sl-TimeResource and occur with a periodicity of 10240 ms. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by sl-StartSymbol and be within a number of consecutive symbols indicated by sl-LengthSymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined.

<Unchanged parts are omitted>

16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a SL BWP on a carrier, and an active UL BWP on the primary cell, as described in clause 12, a UE determines a set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on:

a)	a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1

b)	the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in SL-BWP-Config or SL-BWP-ConfigCommon and BWP-Uplink for the SL BWP and the active UL BWP, respectively

c)	a configured sidelink resource pool bitmap

d)	a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period

<Unchanged parts are omitted>