3GPP TSG RAN WG1 #110	-Bis							R1-2210522

e-Meeting, October 10th - October 19th, 2022

Agenda item:	9.5.1.2

Source: 	Moderator (Qualcomm)

Title: 	Moderator Summary #3 on potential solutions for SL positioning

Document for:	Discussion and Decision

# Introduction

The study on expanded and improved NR positioning introduces sidelink positioning as an objective [1]:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

The focus on the 9.5.1.2 Subagenda is the following objective:

- Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

- Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

In this paper, we summarize some common elements in the contributions and identify some areas and positions where contributing companies are aligned from which some agreements could be derived at this meeting.

# Discussion Information

The proposals are ranked according to HIGH, MEDIUM, LOW. The intention is to start in the offline/online sessions from the HIGH proposals. Companies are encouraged to reply in all of them, but feel free to prioritize your input according to the ranking.

# Sidelink Positioning Methods & Measurements, including combination with other RAT dependent positioning measurements

## SL Positioning Methods & Measurements

#### 3.1.1 General Proposals on SL Positioning Methods & Measurements

The following agreements were reached the previous meetings:

Agreement With regards to the Positioning methods supported using SL measurements study further the following methods: - RTT-type solutions using SL - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT - SL-AoA - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study - SL-TDOA - SL-AoD - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu. - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study - Consider in the study at least the following aspects: - Definition(s) of the corresponding SL measurements for each method - Which method is applicable to absolute or relative positioning or ranging, including whether such categorization is needed to be discussed. - For angle-based methods, antenna configuration consideration(s) using practical UE capabilities - Per-panel location, if UE uses multiple panels. - UE’s mobility, especially for V2X scenarios - Impact of synchronization error(s) between UEs - Existing SL measurements (e.g. RSSI, RSRP), and UE ID information etc, may be used. - Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified, or whether there will be a unified SL Positioning method. - Note: When the study of carrier phase positioning and the evaluations of sidelink positioning have progressed, it can be reviewed whether carrier phase for sidelink can be considered in further work. Checkpoint at RAN1#110-e-Bis to see if sufficient information is available for this review. - Note: Companies are encouraged to describe the role of SL nodes and their interaction/coordination participating in each method. Agreement With regards to the Positioning methods supported using at least SL measurements, potential candidate positioning methods include at least the following: - RTT-type solution(s) using SL - SL-AoA - SL-TDOA - Note: other methods can still be studied - Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified.

Based on the submitted contributions, the following statements/proposals are identified to be related to this topic:

| Nokia, NSB | Proposal 1: Study impact of the multiple antenna panels to LoS/NLoS conditions and positioning measurement for SL-AoA, SL-TDOA, SL-RTT, and SL-AoD positioning techniques. Proposal 2: Support the SL-TDOA with both concepts of DL-TDOA and UL-TDOA. |
| --- | --- |
| Futurewei | Proposal 2: With regards to the SL positioning methods supported using at least SL measurements, potential candidate positioning methods include SL-AoD based methods. |
| Huawei, HiSilicon | - Proposal 2: Phase difference based AoD positioning should be supported for SL positioning. RAN1 should further study Tx antenna switching based scheme. - Proposal 3: Support the Rx-Rx time difference based method for absolute SL positioning. - Proposal 4: To support the above SL positioning methods, the following measurements should be reported: - Rx-Tx timing difference for both single-sided and double-sided RTT based method - Phase difference for AoD-based method - Rx-Rx timing difference for Rx-Rx timing difference based positioning method. - Proposal 5: Study mechanisms to enable SL positioning under UE distributed antenna system for TDOA, AoA, AoD, and RTT based positioning methods. |
| vivo | - SL-RTT and SL-AoA are supported for sidelink positioning. - With distributed antenna systems, the positioning performance will be improved compared to single-panel antenna systems. - Two antenna panels can be considered for V2X positioning. - Multiple antenna panels’ measurement and SL-PRS configuration can be further studied. Similar to the TRP measurement result reporting in TS 38.455, to support one or more measurement results reporting based on measurement request and independent with positioning method. |
| CATT | Proposal 1: SL-AoD positioning method on FR1 should be one of the potential candidate positioning methods for Rel-18 SL positioning. Proposal 2: SL-ECID positioning method is not supported for Rel-18 SL positioning. |
| OPPO | - RTT-type solution and SL-AoA method should be supported in SL positioning, whether SL-AoD can be supported or not in Rel-18 should be reviewed when enhanced sidelink operation on FR2 has progressed. - SL-TDOA method should be supported for absolute SL positioning. |
| Sony | Proposal 1: Support RTT-type solution(s) using SL, SL-AoA, and SL-TDOA as the positioning method for SL positioning measurement. Proposal 2: Consider SL-RSRP for positioning purpose. |
| NEC | Proposal 11: The movement of reference points should be taken into account for the study of sidelink positioning methods including OTDOA, RTT, AOA/D, etc, especially for DL positioning. Proposal 12: Timing/power/AoA/AoD based positioning methods can be combined with carrier phase based positioning method to firstly used to reduce the searching space of the integer ambiguity and corresponding measurement should be conducted. |
| xiaomi | Proposal 1: One-to-one direct ranging shall be focused in RAN1 study of SL positioning solutions. |
| CMCC | Proposal 1: RTT and AoA methods should be supported for both absolute positioning and relative positioning/ranging. - TDOA can be supported only in the case where the anchor devices are deployed and controlled by NW (e.g., RSUs) in Rel-18 SL positioning for absolute positioning; - Whether AoD can be supported during Rel-18 should be subject to the progress in NR sidelink evolution. |
| Lenovo | Proposal 12: Support angular-based SL positioning methods, SL-AoA and SL-AoD, considering antenna calibrated entities/nodes and fixed anchor nodes, e.g., gNBs or RSUs. Observation 6: SL fingerprinting, E-CID type methods can satisfy low latency and coarse SL positioning requirements. Proposal 13: RAN1 to support the SL fingerprinting method to satisfy coarse positioning requirements and to be used as input for other SL positioning methods, e.g., SL-TDoA, to enhance UE position estimation. |
| Interdigital | Proposal 10: Down-prioritize SL AoD-based positioning in FR1. |
| Apple | Proposal 8: RAN1 should discuss the specification impact of the SL positioning techniques based on the existing RAN-dependent techniques including the associated signaling, measurements and procedures for the new SL-positioning schemes.. Issues to be addressed include: - Identification of the positioning set and the target UE - Establishment of a common synchronization reference and the positioning reference UE (for TDOA based schemes) - Establishment of new SL measurements. - Configuration, transmission and measurement of SL positioning reference signals to or from the target UE - Feedback of positioning measurement and assistance information to the positioning estimator |
| Mediatek | Proposal 4-2: Support SL-AOD as one of the SL positioning methods Proposal 5-2: Support and define the measurement types of SL RSTD, SL PRS-RSRP, SL PRS-RSRPP, SL RTOA and SL AOA |
| ITL | Proposal 1: Potential solutions for sidelink positioning, followings should be considered. - Among UEs which transmit sidelink RS (e.g., UE#1, UE#2 and UE#3 in Figure 1), whether each UE is located on in-coverage, partial-coverage or out-of-coveage? - Among UEs which transmit sidelink RS (e.g., UE#1, UE#2 and UE#3 in Figure 1), is an anchor UE (or master UE) needed? - Is On-demand request from target UE which receives sidelink RS for positioning needed? - Is Mode 2 operation needed? |
| CEWiT | Proposal 4: SL-TDOA should be defined for sidelink position at least for absolute and relative positioning. RSTD should be used as measurement quantity. |

Based on the submitted tdocs, and proposals summarized above, in the table below we provide a list of methods that were proposed, along with a first categorization of what is companies’ preferences:

| Method | Support | “include it as part of potential candidate methods” | Deprioritize or not support |
| --- | --- | --- | --- |
| RTT-type solutions using SL | Vivo, OPPO, Sony, Lenovo, CEWiT |  |  |
| SL-AoA | Vivo, OPPO, Sony, Lenovo |  |  |
| SL-TDOA | Nokia, NSB, OPPO, Sony, Lenovo, Mediatek, CEWiT |  |  |
| SL-AoD | Lenovo, Mediatek | Futurewei, CATT | Interdigital |
| SL-E-CID / SL fingerprinting | Lenovo |  |  |
| Rx-Rx measurement method | Huawei, HiSilicon |  |  |

It seems a lot of the companies consider that all three candidate positioning solutions (RTT, AoA, TDOA) should be introduced, and they actually move to specific proposals on each one.

Only a few companies commented with regards to SL-AoD. My suggestion is to try one more time to see the status with regards to this method and i am writing 3 alternatives below for the companies to provide their views.

[CLOSED] Feature Lead Proposal 3.1.1-v0

- From the potential candidate Positioning methods using at least SL measurements, the following should be introduced:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- With regards to SL-AoD,

  - Alt. 1: It should also be introduced

  - Alt. 2: It should be considered a potential candidate positioning method and continue its study

  - Alt. 3: It should be deprioritized

Companies views

| MTK | Support Alt. 1, and also okay for the first main bullet |
| --- | --- |
| CATT | We are fine with the first main bullet. For the second main bullet, we prefer Alt.2. |
| ZTE | For the first bullet, we prefer to add the following note agreed in RAN1#110 meeting. - From the potential candidate Positioning methods using at least SL measurements, the following should be introduced: - RTT-type solution(s) using SL - SL-AoA - SL-TDOA - Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified. With regards to SL-AoD, Alt.2 is preferred. According to the discussion in the last meeting, companies mainly have concerns about the need for beam index information for SL-AoD method. Therefore, we think Alt. 2 can be revised as follows: - Alt. 2: It should be considered a potential candidate positioning method in FR 1 and continue its study in FR2 until there are progresses in AI 9.4.3 (Enhanced sidelink operation on FR2 licensed spectrum). |
| CMCC | We support this proposal. For SL-AoD, we prefer Alt.3. |
| LGE | For the 1st bullet, we support FL proposal. For the 2nd bullet, we support Alt 3, the deprioritization of AoD. In our understanding, AoD can be implemented by two ways. In the first way, AoD can be realized based on beam measurement in FR2, of which the discussion just started in SL evolution WI and we need to wait for the output of it. In the second way, AoD can be realized by measuring the phase difference between the signals from the different TX antennas. It’s related to the carrier phase measurement feature, so we also need to wait for its output. |
| Huawei, HiSilicon | For SL-AoD, we support Alt.1. Since beamforming is not likely performed in FR1, phase difference based AoD positioning should be introduced. In addition, the use of multiple panels on a UE may also be considered as well for SL-AoD based positioning. |
| Spreadtrum | We are fine with the proposal. For SL-AoD, we prefer Alt.2. |
| Fraunhofer | Support Alt.1 for SL-AoD. |
| Apple | We are fine with the proposal. We support Alt 2. |
| Futurewei | Support Alt 1 and OK with main bullet. |
| Intel | We are fine with the proposal. We prefer Alt. 2. SL-AoD is dependent on the antenna configuration at the transmitter |
| NEC | We support the proposal, and prefer Alt.2. |
| DCM | OK and we prefer Alt 3. |
| Samsung | For SL-TDOA, we want to add the following subbullet. with possible solutions for minimizing synchronization error between SL UEs. For SL-AoD, we prefer Alt 3. |
| Sony | We support the first main bullet. SL-AoD has not been thoroughly evaluated, we are OK with Alt.2 for the second main bullet. |
| Sharp | We support FL proposal. For SL-AoD, we are not sure if all the SL posinioning capable UEs need to support SL-AoA, so we prefer continuing the study of SL-AoD (Alt.2). |
| OPPO | Support the first bullet. For the 2nd bullet, we prefer Alt.3, at least beam measurement based SL-AoD should be deprioritized. |
| InterDigital | We support the 1st bullet. For the 2nd bullet, we support Alt 3 to deprioritize SL-AoD-based positioning. Given the remaining TU and work load, we consider it prudent to focus our discussion the methods listed in the 1st proposal. |
| Xiaomi | We support the FL proposal and prefer Alt 2. |
| CEWiT | We are fine with first bullet and for second bullet we prefer Alt 2. |
| Qualcomm | We agree with the proposal and prefer Alt 1. |
| Philips | We are fine with the first bullet and prefer Alt 1 or Alt 2. |
| Nokia, NSB | Support Alt3 |

[ONLINE] Feature Lead Proposal 3.1.1-v1

- From the potential candidate Positioning methods using at least SL measurements, the following should be introduced:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- With regards to SL-AoD,

  - Alt. 1: It should also be introduced

    - MTK, Huawei, HiSilicon, Futurewei, Qualcomm, Philips (5)

  - Alt. 2: It should be considered a potential candidate positioning method and continue its study

    - CATT, Fraunhofer, Apple, Intel, Sony, Sharp, Xiaomi, CEWiT, Philps (8)

  - Alt. 3: It should be deprioritized

    - CMCC, LGE, DCM, Samsung, OPPO, Interdigital, Nokia, NSB (7)

#### 3.1.2 RTT-type solutions using SL

| Nokia, NSB | Observation 1: The worst-case error for single sided SL RTT due to clock drift is on the order of 0.1 ns. Proposal 7: Only support single-sided SL RTT (vs double-sided RTT). |
| --- | --- |
| Futurewei | Proposal 5: For RTT -based SL-PRS shared pool resource-based solutions consider the PSFCH channel resources for the SL-PRS transmission. |
| Huawei, HiSilicon | - Proposal 1: For double-sided RTT based method, support not to restrict the SL-PRS transmission order. Observation 6: For RTT-based SL positioning, there are several associated SL-PRS transmissions. Proposal 20: Support to request SL-PRS transmission from UE-A to UE-B. Proposal 21: For one RTT session, the inter-UE coordination mechanism can be used to indicate the associated SL-PRS transmission among different UEs, e.g. - The transmitting UE can reserve the next associated SL-PRS resources for itself within the reserved period; - The transmitting UE can allocate/recommend the associated SL-PRS resources for the responding UE within the reserved period. |
| vivo | - Double-sided RTT should be studied and supported for SL positioning. Suggest UE Rx – Tx time difference for SL positioning can be defined as T, which is the timing gap between transmission of SL-PRS and receiving SL-PRS to/from other UE. |
| Intel | - The definition of the measure SL UE Rx – Tx time difference for RTT follows the definition for “UE Rx – Tx time difference” for Uu positioning. - Study the need and means to ensure that reception and transmission occasions for RTT are sufficiently close in time. |
| ZTE | Proposal 2: Adequate study and evaluation are needed before introducing DS-RTT method for SL positioning. |
| Xiaomi | Proposal 2: One-way/two-way RTT is supported for SL positioning/ranging. |
| Lenovo | Proposal 9: Support the following two types of SL-RTT type solutions for SL positioning: - One-way SL-RTT - Two-way SL-RTT Proposal 10: Support the Multi-node SL-RTT type solution where multiple anchor nodes/UEs may perform SL-RTT with a target-UE. |
| LGE | Proposal 8: It needs to be studied whether the order of SL PRS in double-side RTT can be changed for more flexible use of SL PRS resources and possibly shortened latency. |
| Interdigital | Proposal 4: Study the impact of UE reference timing offset and mobility on single-sided RTT-based positioning. Proposal 5: Study mechanism to support two-sided RTT-based positioning. |
| Fraunhofer | Proposal 1: With regards to RTT methods, consider both UE Rx-Tx and Report-free based approaches. Proposal 2: Study required signaling and procedures to enable report free RTT. Proposal 3: Consider Rx-Rx timing measurements within the framework of SL RTT solution and study the impact of frequency offsets and multipath propagation effects to the accuracy. |
| Apple | Proposal 2: Single/Double Sided Multi-RTT for absolute positioning: - In SL-Multi-RTT positioning method, the UE position is estimated based on measurements performed at multiple SL-UEs/RSUs/PRUs (in the SL Multi-RTT scheme), or a mix of SL-UE/RSU/PRUs and gNBs/TRPs (for the hybrid SL-Multi-RTT scheme). - The measurements performed at the SL-UE/RSU/PRUs and TRPs are SL-UE/SL-UE Rx-Tx time difference measurements and is typically used for absolute positioning. - A procedure to select the candidate anchor UEs should be studied Proposal 3: Single/Double Sided RTT for relative / absolute positioning: - In SL-RTT positioning method, the UE position is estimated based on measurements performed between the target SL-UEs and a (one) supporting SL-UE/RSU/PRU. The measurements performed at the SL-UE, RSUs, PRUs and TRPs are SL-UE/SL-UE Rx-Tx time difference measurements, SL-UE/gNB Rx-Tx time difference measurements (and optionally SL-PRS-RSRP, DL-PRS-RSRP and and UL-SRS-RSRP) of SL-PRS, DL-PRS and UL-SRS signals. The AoA/AoD may also be measured. It is typically used for both absolute and relative positioning. |
| NTT DOCOMO | Observation 1: - Double-sided RTT has disadvantage on latency performance compared to single-sided RTT. - Necessity to consider clock drifts with RTT is unclear. Proposal 1: - For RTT-type solution using SL, single-sided RTT is prioritized. Proposal 2: - For RTT-type solution using SL, support target UE-based mechanism, i.e., - Step 1: A target UE sends SL-PRS and a request to become anchor UE - Step 2: The anchor UE reports RX-TX timing difference with SL-PRS transmission - Step 3: The target UE obtains its own location |
| Qualcomm | Proposal 1: Both single-sided and double-sided SL-RTT methods should be introduced for supporting SL positioning/ranging. |
| CEWiT | Proposal 5: SL-M-RTT should be supported for sidelink positioning at least for relative positioning and distance-based ranging. Measurement quantity for SL-M-RTT is UE Rx-Tx time difference. Proposal 6: Double-sided RTT should be supported to minimize the sidelink synchronization error. |

With regards to RTT-type solutions, one main item is whether single-sided or double-sided RTT should be defined. It seems there is some majority into introducing both methods, even though some companies still express some concerns whether indeed it is needed, as shown in the table below.

| Method | Support |
| --- | --- |
| Single-sided RTT only | Nokia, NSB, NTT DOCOMO |
| Single & Double-sided RTT | Huawei, HiSilicon, vivo, Lenovo, Xiaomi, Qualcomm, CEWiT |

[CLOSED] Feature Lead Proposal 3.1.2-v0

- With regards to the RTT-type solutions using SL, downselect between the following 2 alternatives:

  - Alt. 1: it corresponds to a single-sided (also known as one-way) RTT method

  - Alt. 2: it may correspond to either a single-sided or double-sided (also known as two-way) RTT method

- Study which one or more of the following measurements could be applicable, and their corresponding definitions: SL UE Rx-Tx, SL RSRP/RSRPP, SL Rx-Rx measurement. Include in the study a report-free SL RTT procedure.

  - Note: Additional measurements are not precluded to be studied

- With regards to the double-sided RTT,

  - companies are encouraged to analyze and evaluate the effect in performance for the single-sided SL RTT due to clock drift

  - Study the order of the SL-PRS transmissions for double-sided RTT

Companies views

| MTK | In our view, the definition of a RTT should be target to anchor and then anchor to target, which is one sided From real implementation to tackle some impairments, it seems to us that, one sided or two sided, or more iterations can be done through the configuration. If this item is to discuss the “definition” of SL-RTT, we prefer one sided. |
| --- | --- |
| CATT | Support the proposal in principle. We are open to further discuss the DS-RTT, including its performance and specs impacts. |
| ZTE | We have several comments about Proposal 3.1.2-v0: Comment 1: For Alt. 1 in the first bullet, the term one-way ranging is used for TDOA method , instead of RTT. The content in bracket should be changed to “also known as onetwo-way”. Single-sided two-way ranging method used in UWB positioning is what we called single-sided RTT method in SL positioning, and double-sided two-way ranging method used in UWB positioning is what we called double-sided RTT method in SL positioning. Comment 2: For Alt. 2 in the first bullet, an FFS is needed to further study the spec impact of double-sided RTT method. - FFS study whether there or what is the spec impact of double-sided RTT method Comment 3: For the second bullet, we do not think SL Rx-Rx measurement is necessary. Is SL Rx-Rx measurement calculated between two antennas or two time stamps? If the answer is time stamp, then what is the difference between UE reporting two or multiple Tx/Rx time stamps and SL Rx-Rx measurement? In our understanding, there is no extra spec impact and it can up to implementation on how LMF or UE calculation the position considering mobility. Comment 4: For the second bullet, we are confused about “report-free SL RTT procedure” and we prefer to delet it or put it in FFS, further clarification is needed. Comment 5: The third bullet should be the subbullet of Alt. 2. To sum up, we suggest to revise the above proposal: ##### [HIGH] Feature Lead Proposal 3.1.2-v0-revised - With regards to the RTT-type solutions using SL, downselect between the following 2 alternatives: - Alt. 1: it corresponds to a single-sided (also known as onetwo-way) RTT method - Alt. 2: it may correspond to either a single-sided or double-sided (also known as two-way) RTT method - With regards to the double-sided RTT, - companies are encouraged to analyze and evaluate the effect in performance for the single-sided SL RTT due to clock drift - Study the order of the SL-PRS transmissions for double-sided RTT - FFS study whether there or what is the spec impact of double-sided RTT method - Study which one or more of the following measurements could be applicable, and their corresponding definitions: SL UE Rx-Tx, SL RSRP/RSRPP, SL Rx-Rx measurement. Include in the study a report-free SL RTT procedure. - FFS: SL Rx-Rx measurement, report-free SL RTT procedure - Note: Additional measurements are not precluded to be studied - With regards to the double-sided RTT, - companies are encouraged to analyze and evaluate the effect in performance for the single-sided SL RTT due to clock drift - Study the order of the SL-PRS transmissions for double-sided RTT |
| CMCC | We are generally fine with this proposal since we think in this stage both of the single and double sided RTT should be kept in the scope. For the order of the SL-PRS transmissions for double-sided RTT, we propose not to put any limitation on it to provide more flexibility for resource allocation. |
| LGE | For the 1st bullet, we support Alt 2. Both single- and double-sided RTT method should be introduced. For the 2nd bullet, we support SL UE Rx-Tx time difference and the related SL RSRP/RSRPP for SL RTT. But Rx-Rx time difference needs further study including the feasibility and the performance analysis. Concerning the report-free SL RTT, the pre-determined UE Rx-Tx time difference cannot be guaranteed in Scheme 2 resource allocation, which are based on sensing. Also for Scheme 1, fixing the UE Rx-Tx time difference may introduce interference to Uu link. We suggest not to consider it any more. For the 3rd bullet, we support FL proposal. |
| vivo | We think double side RTT should be supported. According to our analysis, if two UEs have the same synchronization source, the clock error between 2 UEs would be within ±0.1 PPM. Even though the clock error between two UEs is 0.2 PPM (that is eUE1-eUE2=0.2 PPM), the error can be 50ns if the T_reply is assumed as 1s. Which corresponds to 15m. In sidelink, if two UEs have different synchronization sources, the effect of clock drift would be larger. Therefore, we think double side RTT should be supported and alt 2 is preferred |
| Huawei, HiSilicon | General OK with the proposal. For RTT-type alternatives, we support Alt.2. |
| Lenovo | Supportive of FL’s Proposal with Alt. 2, additionally we showed some performance gains of DS-RTT over OS-RTT |
| Ericsson | OK with the proposal, with preference to alt 2. |
| Sharp | We support the proposal. We are fine with keeping both the single and double sided RTT for now. |
| Futurewei | OK with the proposal. We prefer Alt 2. |

There were questions about the measurements for Single-sided/double-sided RTT. A few concerns were raised about Rx-Tx and “report-free” RTT.

Companies are encouraged to provide their views.

[CLOSED] Feature Lead Proposal 3.1.2-v0

- Study which one or more of the following measurements could be applicable, and their corresponding definitions: SL UE Rx-Tx, SL RSRP/RSRPP, SL Rx-Rx measurement. Include in the study a report-free SL RTT procedure.

  - Note: Additional measurements are not precluded to be studied

Companies views

| Fraunhofer | Support. For the report-free RTT the concerns raised in the online session and comment4 above are not shared. For example, in comment1 ZTE lists some established RTT systems which follow a report-free procedure. |
| --- | --- |
| Apple | The last element is not a measurement but a procedure. Separate bullet point ? |
| CATT | OK to study these measurements. |
| DCM | Probably, ‘For SL-RTT’ is necessary in the main bullet. Rx-Tx should be clarified as Rx-Tx time difference. Definition of ‘report-free’ is quite unclear. We prefer to remove it. |
| CEWiT | Agree in principle. For clarity agree with DCM. |
| Qualcomm | We agree with Docomo’s proposed updates. |
| Philips | Agree with Docomo. |
| Nokia, NSB | “report-free SL RTT procedure”, if kept, should be a separate bullet point, not under measurements. It is not clear how report-free can work for SL: Such a procedure is possible if the responding UE can start its SL-PRS transmission at any point in time; for SL, however, our expectation is that SL-PRS transmissions can only start at certain locations with at least symbol granularity. |

[CLOSED] Feature Lead Proposal 3.1.2-v1

- With regards to the RTT-type solutions using SL,

  - Study which one or more of the following measurements could be applicable, and their corresponding definitions: SL UE Rx-Tx measurement, SL RSRP/RSRPP measurement, SL Rx-Rx measurement.

  - [Include in the study a report-free SL RTT procedure.]

  - Note: Additional measurements are not precluded to be studied

Comments are encouraged to comment whether to keep the sentence in the bracket or not

Companies views

| vivo | We wonder whether the main bullet will implicit the per method-specific measurement/reporting is needed for SL positioning. Considering the complex reporting per method in Rel-17 and hybrid measurement metrics (e.g RSRP and Rx-Tx measurement) in each method, we prefer to define a unified SL Positioning reporting other than method-specific positioning reporting. |
| --- | --- |
| LGE | We support FL proposal with two comments. First, we’re not sure why/how SL Rx-Rx time difference measurement is related to SL RTT. Is the proposal meant to repeat the whole set of SL PRS transmission for SL Rx-Rx time difference measurement? It just causes a large overhead and latency without a clear benefit. We suggest to remove it. We already agreed to study the impact of UE mobility for RTT-type positioning. If necessary, SL Rx-Rx time difference measurement can be done through that study. Second, we share the view with DCM and Nokia that a report-free RTT is not feasible. Also suggest to remove it. ##### [MEDIUM] Feature Lead Proposal 3.1.2-v1 - With regards to the RTT-type solutions using SL, - Study which one or more of the following measurements could be applicable, and their corresponding definitions: SL UE Rx-Tx measurement, SL RSRP/RSRPP measurement, SL Rx-Rx measurement. - [Include in the study a report-free SL RTT procedure.] - Note: Additional measurements are not precluded to be studied |
| CATT | We share the same view with LGE that SL Rx-Rx time difference measurement and the report-free RTT may be not suitable for this proposal. We are fine with LGE’s version. |
| Fraunhofer | The study on report-free RTT procedure should not be precluded. SL-RTT for V2X and InF scenarios is coupled with high reliability and low latency requirements and for that such procedure can be essential. We agree with the comments that the feasibility should NOT conflict with the OFDM symbol timing requirements. That said the responding UE can for example apply a cyclic shift SL-PRS so that the anchor UE can determine the RTT without neither violating the timing requirements nor the Rx-Tx responder report. |
| ZTE | We share the similar concern with vivo and that is also why we want to add a note in Proposal 3.1.1 as agreed in RAN1#110 meeting: - Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified. According to TS 37.355, all three positioning methods which need measurement results transferred from UE to the LMF (e.g., DL-TDOA, Multi-RTT, DL-AoD) include measurement elements “DL-PRS-RSRP”, “DL-PRS-RSRPP”, “Time stamp” and “LOS/NLOS information” and etc. In our understanding, it is definitely NOT signaling-efficient to report measurements per method given the condition that position may be calculated/derived from both timing-based positioning method and angle-based positioning method and they may share the same report elements. We provided measurement report example in our paper (similar as the measurement information transfer framework defined in TS 38.455): SL positioning measurement report { UE identification information Mandatory SL-PRS ID info Mandatory SL-PRS RSTD Optional SL-PRS AOA Optional SL-PRS RSRP Optional SL-PRS RSRPP Optional Rx-Tx time difference Optional Time stamp Mandatory Timing quality Mandatory … }. Therefore, we suggest that companies can just discuss the content of SL positioning measurement report considering those agreed SL positioning methods, instead of separately discuss measurements per method. We prefer not to keep the sentence in the bracket or put it in FFS to let companies further consider. Similar as LGE and CATT, SL Rx-Rx time difference measurement here is weird and we prefer to either delete it or put it in FFS. |
| Qualcomm | SL Rx-Rx measurement is equivalent to measuring RSTD at two different times from the same anchor in our understanding. Hence, we think it can be already be supported by RSTD measurements in the proposal on TDoA. We support ZTE’s proposal to include the note on single vs. multiple methods. |
| Intel | The report-free SL RTT case is a signalling enhacment using other knowledge of Rx or Tx time. Thus it might be better to discuss it together with measurement report functionalities. |
| CMCC | OK. However, how the “report-free SL RTT procedure” can work should be clarified. LGE’s version is more preferred. |
| Huawei, HiSilicon | We support the proposal also to include the study of report-free procedure. Regarding report free, our understanding is that UE SL-PRS transmission timing can be based on reception timing of another SL-PRS, so that the UE Rx – Tx time difference is equivalent to the Timing Advance of SL-PRS. |
| OPPO | At this stage we are fine to keep SL Rx-Rx measurement and report-free SL RTT procedure for further study, however, it is preferable to give some clarification on the 2 terminology to ensure companies are on the same page. |
| Samsung | OK. We prefer deleate the second sub-bullet (i.e. report-free SL RTT). We can discuss this sepearately if necessary. Also, prefer to remove SL Rx-Rx measurement as other companies are commented. We are OK with LGE’s version. |
| Spreadtrum | Support |
| DCM | Agree with LGE. SL RX-RX time difference and report-free are unnecessary in this stage. As commented by other companies, report-free should be discussed in other section since it seems that this proposal is made for measurement perspective. |
| Apple | Prefer LGE’s version with removal of two extra bullets |
| Nokia, NSB | OK |
| Sharp | OK |

#### 3.1.3 SL-AoA

| Nokia, NSB | Proposal 4: Support AoA method for SL positioning, considering UEs that can have multiple antenna elements (e.g., vehicles, RSUs). Proposal 5: SL-AoA positioning should take into account the geometric location of the antenna panels of the target UE and anchor Ues, and any necessary exchange of related information between the Ues and/or between the Ues and the network. |
| --- | --- |
| Vivo | - AoA for SL positioning is needed for SL-PRS measurement and reporting. |
| Intel | - The definition of the measure SL Angle of Arrival (SL-AoA) follows the definition for UL AoA for Uu positioning. |
| Xiaomi | Proposal 3: SL-AoA is supported for SL positioning/ranging. |
| Lenovo | Proposal 11: RAN1 to define SL-AoA measurements in terms of azimuth and vertical (elevation) angles for both Local coordinate systems (LCS) and Global coordinate systems (GCS). |
| Interdigital | Proposal 9: Study the SL node capability required for SL AoA-based positioning in FR1. |
| Apple | Proposal 4: SL-AoA positioning : - SL-AoA positioning is based on the angle of arrival of transmission or angle of departure of transmission to or from a SL UE target. In hybrid SL-AoA/AoD, one or more of the positioning set may be a gNB. - The measurements performed can include both Azimuth of arrival (AoA) and zenith of arrival (ZoA). - SL-AoA can be used to estimate the relative direction between two SL devices or to estimate absolute positioning with up to 3 sidelink devices or when combined with a direction estimator e.g. RTT. |
| CEWiT | Proposal 7: For SL-AOA, Angle estimated over the at least first path should be reported as measurement. Other measurements including related TOA and LOS/NLOS should be reported. |

[CLOSED] Feature Lead Proposal 3.1.3-v0

With regards to the measurements for SL-AoA positioning,

- Both SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement for the first arrival path should be included

  - FFS: Definition of the measurements

- Study further whether other measurements can be applicable

Companies views

| CATT | Support. |
| --- | --- |
| ZTE | “for the first arrival path” in the first bullet should be deleted, further study is needed for multi-path measurement report. |
| CMCC | We are generally fine with the proposal, except that “for the first arrival path” part. What is the intention to emphasize “for the first arrival path” here? I think maybe at this stage we can simply study SL-AoA/ZoA measurements and others can be details for FFS. |
| LGE | We support FL proposal with the following modification. Similar to SL RTT and SL TDOA, the angle estimation and the related RSPR per path can provide information for decision of the first path and the reliability of the measurements based on the channel status (e.g. Rician factor) - Both SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement per path, and RSRP/RSRPP for the first arrival path should be included |
| vivo | Support |
| Huawei, HiSilicon | OK |
| Lenovo | Support |
| Ericsson | We agree that these measurements should be included in the study. |
| Sharp | Support. We are also fine to remove “the first arrival path”. |
| Spreadtrum | Support |
| NEC | Support |

#### 3.1.4 SL-TDOA

| Nokia, NSB | Proposal 3: Study the detailed procedure of necessary signallingignaling for SL-TDOA, including a reference for RSTD, and synchronization source information of UE such as gNB/GNSS. |
| --- | --- |
| Intel | - Study if SL-TDOA is defined based on UL-TDOA, or DL-TDOA or both. - Study the definition of the measure SL Relative Time of Arrival (SL RtoA) for UL-TDOA-like SL-TDOA. - Study the definition of the measure SL RSTD for DL-TDOA-like SL-TDOA. |
| ZTE | Proposal 1: Study the enhancement of sidelink synchronization considering the time synchronization requirement of reference Ues, selection of the reference Ues, and the signaling procedures. |
| Xiaomi | Proposal 4: The condition of using SL-TDOA shall be studied including the synchronization among anchor Ues. |
| Lenovo | Proposal 7: RAN1 to define SL RSTD measurements for supporting SL-TDOA. Proposal 8: RAN1 to at least introduce the support for a reference anchor node/UE for SL RSTD measurements and SL relative time difference (RTD) information. |
| Interdigital | Proposal 6: Study mechanisms to support both DL-like (many to one) and UL-like (one to many) SL TdoA-based positioning. Proposal 7: Prioritize design of SL TdoA-based positioning in a SL positioning group of stationary anchor Ues (e.g., RSUs), and a target UE. Proposal 8: Study impact on SL TdoA-based positioning by the synchronization error between anchor Ues and the uncertainty of the absolute positioning information of anchor Ues. |
| LGE | Proposal 5: SL-TDOA positioning: - SL-TdoA positioning is based on the time delay of arrival to or from a SL UE target with SL Ues in a positioning set (at least 2 for 2-D positioning and at least 3 for 3-D positioning). In hybrid SL-TDOA, one or more of the positioning set may be a gNB. - The measurements performed include the TdoA of the positioning reference signal with a reference SL device and is typically used for absolute positioning - For timing-based positioning, the impact of synchronization errors between the anchor Ues should be studied to ensure accuracy of the estimated position. To mitigate the impact of synchronization errors RAN1 should study and adopt one or more of the following: - Enable a procedure to identify anchor Ues with a common synchronization reference - Enable a procedure to allow for a change of anchor Ues to a common synchronization reference - Enable a procedure to estimate the target UE position in the presence of anchor Ues with different synchronization references and compensate for the difference Proposal 29: SL synchronization procedure is performed by the UE that performs SL positioning. Proposal 30: Further study is needed on the solution that mitigates the performance loss of SL positioning due to the synchronization timing offset between Ues. |
| Samsung | Proposal 1: For SL-TDOA, further study necessity, whether it is feasible or not with possible solutions for minimizing synchronization error between SL Ues. |
| NTT DOCOMO | Proposal 3: - For SL-TDOA, do not discuss/study how to mitigate impact of synchronization error between two anchor Ues. Proposal 4: - For SL-TDOA/SL-AoA, support both target UE-based mechanism and anchor UE-based mechanism, i.e., - Target UE-based mechanism: - Step 1: A target UE sends a request to become anchor UE - Step 2: The anchor UE transmits SL-PRS - Step 3: The target UE obtains its own location - Anchor UE-based mechanism - Step 1: Anchor UE transmits SL-PRS periodically - Step 2: A target UE obtains its own location |
| Qualcomm | Observation 5: SL-TDOA requires synchronization across the SL anchors in order to result into good positioning performance; a drawback that SL-RTT and SL-AoA methods do not have. Synchronization across SL anchors is simpler to achieve for fixed-location anchors. Observation 6: Methods of achieving synchronization amongst anchor Ues may need to be looked into if SL-TDOA is considered to be specified. |
| Mediatek | Proposal 4-1: Support SL-TDOA for the direction of anchor(s) to target, and target to anchor(s) Proposal 5-1: We expect to have a unified measurement type, if both SL-RTT and SL-TDOA with target-to-anchor(s) transmission direction are supported |

[CLOSED] Feature Lead Proposal 3.1.4-v0

With regards to SL-TDOA positioning method,

- It corresponds to a method where SL-PRS is being transmitted from one or more device(s) to a target UE (i.e., DL-TDOA-like operation), or from a target UE to one or more device(s) (i.e. UL-TDOA-like operation)

  - Study the detailed procedure,  necessary signallingignaling for SL-TDOA, method(s) to  mitigate impact of synchronization error between two anchor Ues including whether such method(s) are needed.

- Study further which one or more of the following measurements can be applicable, and their corresponding definitions:

  - SL RSTD, SL RTOA, SL RSRP/RSRPP, SL Rx-Rx measurement

  - Note: Other measurements are not precluded to be studied

Companies views

| MTK | We are generally okay for the proposal. Just a small question for first main bullet. We see “one or more devices”, and for the subbullet corresponding to the first main bullet, we see “two anchor Ues”. Any potential difference between “devices” and “anchor Ues” ? |
| --- | --- |
| CATT | Support the proposal in principle. We share the same view with MTK, the “devices” in the first main bullet and the “anchor Ues” in the subbullet had better to be aligned with each other, e.g., both of them can be revised into anchor Ues or reference Ues. |
| ZTE | Generally fine. Similar as our comment 3 for Proposal 3.1.2, we have concerns about SL Rx-Rx measurement. As for the question raised by MTK and CATT, we think current wording is fine considering hybrid positioning and device can be either UE or gNB. |
| CMCC | Support |
| LGE | Support the FL proposal One comment on the proposal summary table, the proposal 5 in the row of LGE was miscaputured. It’s not LGE’s proposal. Only the proposal 29 and 30 are ours. |
| Vivo | In our understanding, it is difficult to use the relative positioning between VUE. So, we prefer it only be considered for absolute positioning from a target UE to more RSU(s). Otherwise, the proponent needs to clarify how to mitigate the synchronization error. |
| Huawei, HiSilicon | When we say “one or more devices”, are these devices expected to be the agreed RAN1 terminology “anchor UE”? |
| Lenovo | Suportive of FL’s proposal, we also understand that device(s) may translate to anchor Ues. Regarding 1st sub-bullet of 2nd bullet , Should it rather read “…, SL PRS RSRP/RSPP, …” measurements |
| Ericsson | OK |
| Sharp | Support. In our view, SL-TDOA positioning method is suitable for absolute positioning. We are wondering if companies intend to use it for relative positioning and/or ranging as well. |
| Spreadtrum | Support |
| Futurewei | Support. As we mentioned during the GTW the clarification of anchor UE definition is necessary |
| NEC | Support |

[CLOSED] Feature Lead Proposal 3.1.4-v1

- Study further which one or more of the following measurements can be applicable, and their corresponding definitions:

  - SL RSTD, SL RTOA, SL RSRP/RSRPP, SL Rx-Rx measurement

  - Note: Other measurements are not precluded to be studied

Companies views

| Apple | Fine with this proposal |
| --- | --- |
| CATT | OK to further study these measuments. |
| DCM | Probably, ‘For SL-TDOA’ is necessary in the main bullet. |
| OPPO | OK |
| Qualcomm | OK and agree with Docomo’s suggestion |
| Philips | OK with Docomo’s modification. |
| Nokia, NSB | OK, agree with DCM. |

[CLOSED] Feature Lead Proposal 3.1.4-v1

- For SL-TDOA, study further which one or more of the following measurements can be applicable, and their corresponding definitions:

  - SL RSTD, SL RTOA, SL RSRP/RSRPP, SL Rx-Rx measurement

  - Note: Other measurements are not precluded to be studied

Companies views

| vivo | Similar view as 3.1.2 |
| --- | --- |
| LGE | We support FL proposal with one comment. We’re not convinced about the benefit of Rx-Rx time difference measurement. As we commented earlier, if the simultaneous reception of the multiple SL PRSs is done at time T1, that’s quite enough for SL TDOA regardless of UE mobility. There is no need for additional reception of the whole set of SL PRSs at time T2. It wastes SL resources and increases the latency. We suggest to remove it. ##### [MEDIUM] Feature Lead Proposal 3.1.4-v1 - For SL-TDOA, study further which one or more of the following measurements can be applicable, and their corresponding definitions: - SL RSTD, SL RTOA, SL RSRP/RSRPP, SL Rx-Rx measurement - Note: Other measurements are not precluded to be studied |
| MTK | SL RSTD measurement, SL RTOA measurement, SL RSRP/RSRPP measurement |
| CATT | We share the same view with LGE that SL Rx-Rx time difference measurement may be not suitable for this proposal,especially for SL-TDOA. We are fine with LGE’s version. |
| ZTE | We suggest that companies can just discuss the content of SL positioning measurement report considering those agreed SL positioning methods, instead of separately discuss measurements per method. Also fine with deleting SL Rx-Rx time difference measurement. |
| Qualcomm | We propose to capture a note on single vs. multiple methods as in the RTT proposal. |
| Intel | We are fine with the proposal. |
| CMCC | OK |
| Locaila | We are fine with the proposal and also fine with LGE’s version. |
| Huawei, HiSilicon | We are fine with the proposal. |
| OPPO | Similar comments as that for 3.1.2-v1 |
| Samsung | We support LGE’s version |
| Spreadtrum | Support |
| DCM | OK |
| Apple | Support LGE |
| Xiaomi | We are fine with the proposal but prefer to LGE version to delete SL RX-RX measurement. |
| Nokia, NSB | OK |
| Sharp | We are fine with the proposal |

#### 3.1.5 SL-AoD

| Nokia, NSB | Proposal 6: Study if/how to expand the current NR RSRP/RSRPP measurement definition to SL PRS measurements. Study of SL-AOD can be deprioritized given that it requires FR2 support. |
| --- | --- |
| Intel | - Study the definition of the measures SL PRS-RSRP and SL PRS-RSRPP for SL-AoD if SL-AoD is supported. |
| Huawei, Hisilicon | - Proposal 2: Phase difference based AoD positioning should be supported for SL positioning. RAN1 should further study Tx antenna switching based scheme. |

FL Note: We will wait progress in section 3.1.1 before proceeding with treating these proposals.

#### 3.1.6 SL-UE-ID

| Apple | Proposal 6: SL-E-CID type positioning: - SL-E-CID type positioning is based the UE reporting only the measurements that it has available rather than being required to take additional measurement actions. - Note that new measurements such as a sidelink SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ may be required. |
| --- | --- |
| Lenovo | Observation 6: SL fingerprinting, E-CID type methods can satisfy low latency and coarse SL positioning requirements. Proposal 13: RAN1 to support the SL fingerprinting method to satisfy coarse positioning requirements and to be used as input for other SL positioning methods, e.g., SL-TDoA, to enhance UE position estimation. |

Only limited companies have commented on the use of SL-E-CID (also referred to as SL Fingerprinting from another company)

[CLOSED] Feature Lead Proposal 3.1.6-v0

With regards to SL-E-CID,

- Opt. 1:  consider as a potential candidate method a SL Positioning method which uses already-specified SL measurements (e.g. SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ) without the need of any new measurements to be performed and without the need of new reference signals.

  - FFS: Details

- Opt. 2: Study further which of the already-specified SL measurements (e.g., SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ) could be reported as part of any of the remaining SL Positioning methods.

- Opt. 3: Such a method/pproach should be considered a lower priority.

Companies views

| ZTE | Opt.3 |
| --- | --- |
| LGE | We need clarification on what exactly SL-E-CID means. If it means SL positioning with reference to a certain UE’s location based on the identification of the UE, we support the option 1. If so, we recommend to change the terminology such as SL UE-ID based positioning, to make the meansing clearer. |
| Huawei, HiSilicon | Is SL-E-CID actually referring to SL-E-UID? Saying E-CID would imply that UE reports Uu RRM measurement towards cell(s) to another UE in our understanding, which does not seem the case in common understanding. |
| Apple | Open to changing the name. Prefer Opt. 2. |
| Futurewe | We agree that the name could be better. We support Opt 2 |
| CATT | Opt.3 |
| DCM | Opt.3 |
| Samsung | Support Opt. 3 |
| Sony | Is this limited to SL-E-CID ? or about the possibility to use already-specified SL measurements (e.g. SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ) for SL-positioning? If it is about the later one, we think it can be beneficial. It may not be accurate but it can provides positioning information. How about rephrasing as follow: With regards to SL-E-CID, Study further the already-specified SL measurements (e.g., SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ) could be usedreported as part of the any of the remaining SL Positioning methods. |
| OPPO | Option 3 |
| InterDigital | We support Opt 3. |
| Nokia, NSB | Prefer Opt. 3 |

[CLOSED] Feature Lead Proposal 3.1.6-v1

Study further which of the already-specified SL measurements (e.g., SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ) could be reported as part of any of the remaining SL Positioning methods.

Companies views

| vivo | Clarification is needed for what is the remaining SL Positioning methods |
| --- | --- |
| LGE | We prefer the previous form of the proposal. The current proposal is not clear. What is the meaning of the “remaining SL positioning methods”? Except SL UE-ID based positioning, those measurements need not be reported. Otherwise, we may separate two questions into separate proposals. ##### [MEDIUM] Feature Lead Proposal 3.1.6-v1-1 Study further whether/which of the already-specified SL measurements (e.g., SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ) could be reported as part of any of the remaining SL Positioning methods. ##### [MEDIUM] Feature Lead Proposal 3.1.6-v1-2 With regards to SL UE-ID based positioning, - Opt. 1: consider as a potential candidate method a SL Positioning method which uses already-specified SL measurements (e.g. SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ) without the need of any new measurements to be performed and without the need of new reference signals. - FFS: Details - Opt. 2: Such a method/pproach should be considered a lower priority. |
| CATT | The remaining SL Positioning methods are not clear for us, such a method/pproach(e.g., using SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ for positioning purpose) should be considered a lower priority. |
| ZTE | Generally, we still think this discussion should be considered as a lower priority. Does this proposal trying to express: Study further which of the already-specified SL measurements (e.g., SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ) could be reported as part of any of the remaining SL Positioning measurement report methods. |
| Qualcomm | We propose the following update since RAN1 could conclude to not report any of the measurements. Study further which of the already-specified SL measurements (e.g., SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ), if any, could be reported as part of any of the remaining SL Positioning methods. |
| Intel | We are fine with the update from ZTE. |
| CMCC | Not support We prefer the original option 3, all the SL measurement should be based on the SL-PRS. |
| Locaila | Agree with Qualcomm’s view |
| Huawei, HiSilicon | We are fine with adding existing SL measurement. However, considering this is no longer geometry based positioning, we assume no need to perform any evaluation for it (similar to NR E-CID positioning). Now the question is what else would require study then? |
| OPPO | SL SSB RSRP/RSRQ and SL-CSI-RS RSRP/RSRQ are not specified, already specified SL measuremnts only include PSBCH-RSRP, PSSCH-RSRP, PSCCH-RSRP, SL RSSIy and SL CR, we do not think any or them can be used for positioning. |
| Samsung | We agreed that “A new reference signal should be introduced for supporting SL positioning/ranging.” We understand that this means the existing SL measurements are depriotized for SL positioning. |
| DCM | Same feeling with LGE. Original proposal was intended for E-CID, but now the proposal becomes general one while ‘remaining’ is kept. Clarification is necessary. |
| Apple | Agree with Oppo. Measurements needed for SL postioning “e-CID” may need to be specified. |
| Xiaomi | We prefer to consider this as low priority. SL positioning using new SL-RPS shall be prioritized. |
| Nokia, NSB | Multiple problems here: - Not clear what the “remaining SL Positioning methods” are - Not a single one of the examples listed for “already-specified SL measurements” is currently specified – SL SSB RSRP probably is meant to read PSBCH-RSRP, but the others? |

After 2 rounds on this proposal, it seems majority do not seem interested in looking into such methods.

- In Round 1,

  - Opt. 3 (low priority) was chosen by ZTE, CATT, DCM, Samsung, OPPO, Interdigital, Nokia, NSB

  - Opt. 2: Apple, Futurewei

- In Round 2:

  - a lot of companies focused on what “other positioning methods” mean and i couldn’t determine whether indeed there is some kind of convergence.

  - A few companies considered it again as low priority (in addition to Round 1): Xiaomi, CMCC

So, I am updating the proposal based on the comments, and marking it as LOW.

##### [LOW] Feature Lead Proposal 3.1.6-v2

With regards to SL-UE-ID based Positioning,

- Opt. 1: Study further which of the already-specified SL measurements could be used

  - Companies are encouraged to provide their views on which methods could be applicable

- Opt. 2: Such a method/approach should be considered a lower priority.

##### Companies views

| ZTE | Opt.2 |
| --- | --- |
| Intel | We are fine with this proposal. |
| InterDigital | We prefer Opt. 2. |
| CATT | Opt.2 |
| OPPO | Opt. 2 |
| Apple | Fine with the proposal. Prefer Opt. 1 |

#### 3.1.7 SL Carrier Phase

| Apple | Proposal 7: Based on the current progress in the dedicated sub-agenda item, carrier phase for sidelink should not be considered for further work in Rel-18 |
| --- | --- |

Only limited companies have commented on the use of carrier phase for sidelink. We are supposed to do a check this meeting to see the status of such a method. Please see a related proposal below:

[CLOSED] Feature Lead Proposal 3.1.6-v0

With regards to SL Carrier phase Positioning

- Alt. 1: not enough progress has been made in the dedicated sub-agenda item, and therefore, do not consider it further for Rel-18 SL Positioning

- Alt. 2: Revisit during RAN1 111 meeting to check whether enough progress has been made.

Companies views

| ZTE | Alt. 2 |
| --- | --- |
| CMCC | We are OK with this proposal. Between the two Alts, we prefer Alt. 1. |
| LGE | We support Alt. 1. After CPM is developed for Uu link based positioning, SL CPM can be studied in a later release. |
| CATT | We prefer Alt.2. |
| DCM | Alt 1 |
| Samsung | Prefer Alt2. |
| Sony | Alt 1 |
| Sharp | We are OK with this proposal and support Alt.1. |
| OPPO | Alt.1 |
| InterDigital | We prefer Alt 1. |
| Nokia, NSB | We prefer Alt. 1 |

[ONLINE] Feature Lead Proposal 3.1.6-v0

With regards to SL Carrier phase Positioning

- Alt. 1: not enough progress has been made in the dedicated sub-agenda item, and therefore, do not consider it further for Rel-18 SL Positioning

  - CMCC, DCM, Sony, OPPO, Sharp, Interdigital, LGE

- Alt. 2: Revisit during RAN1 111 meeting to check whether enough progress has been made.

  - ZTE, CATT, Samsung

#### 3.1.8 SL Positioning Measurements

From the revised proposals in 3.1.2 & 3.1.4, I make the following observations:

- Do not split the proposal per method or keep the note

  - Vivo, ZTE, Qualcomm

- With regards to the Rx-Rx measurement

  - Keep: OPPO, Huawei, HiSilicon

  - Remove the FFS: LGE, CATT, Samsung, DCM, Apple, ZTE

- With regards to the Report-Free SL RTT

  - Keep: Fraunhofer, Huawei, HiSilicon, OPPO

  - Remove the FFS: LGE, CATT, Intel, CMCC, Samsung, DCM, Apple

From moderator perspective, my understanding of

- Rx-Rx measurement: Difference of receive time of PRS transmitted from the same anchor on two difference instances. It is like an RSTD but from PRS transmitted from the same anchor. If there are not enough anchors, using such an “RSTD”, given prior knowledge of UE’s mobility/direction could provide information on UE’s location.

- Report-Free RTT: The measurement is still Rx-Tx, it is just not reported, and it is “encoded” in the transmission timing of the SL-PRS.

Overall, we are still in the study item, and it makes sense to allow companies to study different options and measurements. I am giving one more try to those 2 points to be further studied for one more round, by being more descriptive into what those 2 aspects correspond to.

##### [MEDIUM] Feature Lead Proposal 3.1.8-v0

With regards to the Positioning methods supported using SL measurements

- at least the following measurements are considered:

- SL Rx-Tx measurement

- SL RSTD measurement

- SL RSRP measurement

- SL RSRPP measurement

- Companies are encouraged to study other measurements (e.g., time difference of arrival of 2 SL-PRS transmitted at 2 differnet times from the same anchor) and provide their analysis into why they are needed in light of the above measurements.

- Companies are encouraged to study potential signaling enhancements in which SL Rx-Tx measurement is not reported but the transmit time of SL-PRS is adjusted based on the measurement.

##### Companies views

| LGE | Thank FL for the update. For the 2nd bullet, we got better understading based on the update. But we’re still not convinced that Rx-Rx time difference measurement reporting is necessary. We share the view with other companies that the existing Rx-Tx time difference or RSTD measurement at different time already do the same role, and Rx-Rx time difference measurements can be calculated by implementation. With the reason above, we keep our position on deletining the 2nd bullet. For the 3rd bullet, the time adjustment of SL PRS transmission to remove the reporting of Rx-Tx time difference measurement will cause interference to the transmission/reception of other UEs. For example, if two UE’s SL PRSs are multiplexed with different RE offset or symbol, timing adjustment different between UEs will cause performance degradation due to interference to each other. With the reason above, we also we keep our position on deletining the 3rd bullet. |
| --- | --- |
| MTK | 1, add SL RTOA measurement within first main bullet |
| ZTE | Generally ok. SL-RTOA should also be the potential measurement. Also, can we list SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement in the above Proposal 3.1.8? Position can be calculated/derived from both timing-based positioning method and angle-based positioning method. With regards to the Positioning methods supported using SL measurements - at least the following measurements are considered: - SL Rx-Tx measurement - SL RSTD measurement - SL RSRP measurement - SL RSRPP measurement - SL RTOA measurement - SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement For the last bullet, we suggest the following update: Companies are encouraged to study potential signaling enhancements in which(e.g. SL Rx-Tx measurement is not reported but the transmit time of SL-PRS is adjusted based on the measurement). |
| Intel | We can accept the current proposal. |
| FL Update | Dear all, please continue commenting according to the following Update: ##### [MEDIUM] Feature Lead Proposal 3.1.8-v1 With regards to the Positioning methods supported using SL measurements - at least the following measurements are considered: - SL Rx-Tx measurement - SL RSTD measurement - SL RSRP measurement - SL RSRPP measurement - SL RTOA measurement - SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement - Companies are encouraged to study other measurements (e.g., time difference of arrival of 2 SL-PRS transmitted at 2 differnet times from the same anchor) and provide their analysis into why they are needed in light of the above measurements. - Companies are encouraged to study potential signaling enhancements (e.g., SL Rx-Tx measurement is not reported but the transmit time of SL-PRS is adjusted based on the measurement) |
| InterDigital | We agree with ZTE’s update to the proposal. In our view, the RX-RX timing difference (2nd bullet) and “report-free” RTT (3rd bullet) should be down-prioritized. In addition, could we clarify that the examples mentioned in the last two bullets are relevant to RTT-type solutions? |
| Qualcomm | We are ok with the updated proposal. |
| CATT | We are live with the updated proposal above, but we think the last two bullets maybe not needed since current SL-Pos measurement may be enough for SL positioning. |
| OPPO | Fine with the updated version. |
| Fraunhofer | Can we add an FFS on measurements related to SL-AoD (subject to discussion on the SL Positioning methods)? |
| vivo | We are generally ok with the updated proposal. A typo in second bullet:“2 differnet times” |
| Samsung | Thanks FL for your updates. Our preference is first bullet only. However, we can accept another two bullets to make progress |
| Apple | Fine with the proposal |
| LGE | If FL proposal is just to “consider” instead of “accept” both RSTD and RTOA for SL positioning, we’re fine with the updated proposal. |
| Spreadtrum | We are fine with the proposal. |

# Sidelink Reference Signals for Positioning Purposes (SL-PRS)

## Design of a new SL reference Signal (SL-PRS)

Agreement Study new reference signal for SL positioning/ranging using the existing PRS/SRS design and SL design framework as a starting point. - The study could at least include: Sequence design, frequency domain pattern, time domain pattern (e.g. number of symbols, repetitions, etc), time domain behavior, configuration/triggering/activation/de-activation of the SL-PRS, AGC time, Tx-Rx Turanround time, supportable bandwidth(s), multiplexing options with other SL channels, randomization/orthogonalization options. - Note: The study of existing SL reference signal for SL positioning/ranging is not precluded. Companies are encouraged to perform performance evaluation/comparison to investigate whether such reference signals can meet the positioning accuracy requirements. Agreement A new reference signal should be introduced for supporting SL positioning/ranging.

#### SL-PRS Sequence design Proposals

The following agreement was reached with regards to this topic:

Agreement For the sequence of the new reference signal for SL positioning/ranging, down select between Alt 1 and Alt 2: - Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point. - Alt. 2: ZC-based (SRS sequence as a starting point)

The following proposals were found in the contributions:

| Nokia, NSB | Proposal 8: Consider Alt. 2: SL PRS design based on UL SRS (ZC-based sequence), with any necessary modifications to be studied. |
| --- | --- |
| Spreadtrum | Proposal 1: Gold sequence based SL-PRS sequence design should be considered. |
| Vivo | The pseudorandom-based (e.g., Gold sequence) should be used for the new SL reference signal (i.e., SL PRS), and the existing sequence of DL-PRS as a starting point. The NID of SL PRS should be associated with some UE information (e.g., pre-configured sequence ID, source ID). |
| China Telecom | Proposal 1: support either pseudorandom-based or ZC-based sequence for SL positioning reference signal generation. But prefer ZC-based RS considering its properties for potential location performance improvement. |
| OPPO | - For the sequence of the new reference signal for SL positioning/ranging, DMRS sequence of sidelink PSCCH/PSSCH (Alt.1 agreed in RAN1#110) should be selected. |
| CATT | Proposal 3: The sequence design of SL-PRS should reuse the existing sidelink/positioning reference signals design as much as possible to reduce the workload of standardization and the complexity of UE implementation. Proposal 4: Gold sequence should be adopted as the sequence of SL-PRS in order to resist the high doppler shift for V2X use cases. |
| Intel | - Pseudo-random sequence is preferred for SL-PRS design. |
| Sony | Proposal 3: Support UL-SRS as the sidelink reference signal for positioning, at least as the baseline during the evaluation. |
| ZTE | Proposal 7: If gold sequence is supported for SL-PRS, further study can focus on pseudo-random sequence initialization functions considering: - SL-PRS sequence ID: considering UE ID information or CRC for sidelink positioning control information - Offset in high-bit: when the bit size of SL-PRS sequence ID is larger than 10 - Whether SL-PRS configuration can be associated |
| Xiaomi | Observation 2: Compared with pseudorandom SN, ZC based design has lower PARA but less commonality with the current SL RS design. Proposal 5: RAN1 shall down-select between ZC-based and pseudorandom SN for SL PRS sequence design. |
| CMCC | Proposal 3: For the sequence of the new reference signal for SL positioning/ranging, Alt.1 should be selected, i.e., use existing sequence of DL-PRS as a starting point. |
| Lenovo | Proposal 4: RAN1 to adopt ZC-based sequences with SRS as a baseline design for SL Positioning. |
| LGE | Proposal 21: Either NR DL PRS sequence or UL SRS sequence for positioning is the starting point for SL PRS sequence design. |
| Interdigital | Proposal 29: Support pseudorandom-based sequence for the new SL-PRS using existing sequence of DL-PRS as a starting point (Alt. 1). |
| Fraunhofer | Proposal 9: The SL-PRS design shall adapt the SRS-Pos sequence design, frequency domain pattern and time domain pattern. |
| Apple | Proposal 9: RAN1 should use the exiting DL-PRS sequence design for the SL-PRS i.e. the SL-PRS should be pseudo-random based. |
| Samsung | Proposal 3: For the sequence of the new reference signal for SL positioning/ranging, pseudorandom-based sequence is used. |
| Qualcomm | Proposal 3: Use the DL-PRS design as baseline for SL-PRS, i.e. select Alt 1 from the alternatives for SL-PRS waveforms. |
| Mediatek | Proposal 3-3: Weil sequence is not considered |
| Ericsson | - Reuse the UL SRS for positioning design for SL PRS, with ZC sequences and the same comb configurations. |

We make the following observations with regards to what each company supports:

| Method | Support |
| --- | --- |
| Alt. 1 (PN-based) | Spreadtrum, vivo, OPPO, CATT, Intel, ZTE, CMCC, Interdigital, Apple, Samsung, Qualcomm (11) |
| Alt 2 (ZC-based) | Nokia, NSB, Sony, Lenovo, Fraunhofer, Ericsson (5) |
| Either one | China Telecom (1st priority ZC), LGE, Xiaomi |

It seems there is majority for Alt. 1 over Alt. 2. It also seems that no company has identified a technical problem with either one of the alternatives. Both sequences have been used in 3GPP for Positioning before, and it seems more of a preference/taste that each company has. Therefore, I make the following proposal:

[CLOSED] Feature Lead Proposal 4.1.1-v0

For the sequence of the new reference signal for SL positioning/ranging, use

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

Companies views

| ZTE | Support. |
| --- | --- |
| CMCC | Agree. Alt1 follows the design principle of using downlink signals as reference in NR sidelink and easy to be implemented. |
| LGE | Support FL proposal. The existing DL PRS sequence should be reused as much as possible for compatibility of DL PRS and reducing the workload on the sequence design. |
| vivo | Support. |
| Huawei, HiSilicon | OK. |
| Spreadtrum | Support |
| Fraunhofer | Do not support. Can proponents explain the reasoning for selecting Alt.1 over Alt.2? At least from standardization complextlty ZC-based will be more straightforward. |
| Apple | Support |
| Intel | Support. For easy implementation and robustness in high Doppler case. |
| CATT | Support. |
| DCM | OK |
| Samsung | OK. |
| Sony | Shouldn’t we need to assess between Alt.1 and 2, For example on the performance (accuracy) and PAPR aspect? |
| Sharp | Support |
| OPPO | support |
| InterDigital | We agree with the proposal. |
| Xiaomi | We are fine with the proposal. |
| Qualcomm | We agree with the proposal. Sidelink does not currently include transmission or reception of ZC sequences. |
| Philips | We are OK with the proposal. |
| Nokia, NSB | It is not clear why pseudorandom-based sequence is preferred over ZC-based scheme, since ZC-based sequence yields smaller PAPR leading to wider coverage and has better autocorrelation characteristics. However, since majority prefers Alt1 we can compromise. |
| Lenovo | We don’t see any obvious benefit/gain of pseudorandom-based sequence in comparison to ZC-based sequences, especially with regard to the PAPR. Although we prefer Alt.2, we’ll be also willing to go with Alt. 1. |

[ONLINE] Feature Lead Proposal 4.1.1-v0

For the sequence of the new reference signal for SL positioning/ranging, use

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

  - Support: ZTE, CMCC, LGE, vivo, Huawei, HiSilicon, Spreadtrum, Apple, Intel, CATT, DCM, Samsung, Sharp, OPPO, Interdigital, Xiaomi, Qualcomm, Philips (17)

  - Do not support: Fraunhofer, Sony (?)

  - Do not support but can compromise: Nokia, Lenovo

#### 4.1.2 SL-PRS Frequency & Time-domain Pattern

The following agreement was reached with regards to this topic:

Agreement With regards to the frequency domain pattern, study further a Comb-N SL-PRS design. Study at least the following aspects: - N>=1 (where N=1 corresponds to full RE mapping pattern) - Fully staggered SL-PRS pattern (e.g., M symbols of SL-PRS with comb-N with M=N and, at each symbol a different RE offset is used), Partially staggered SL-PRS pattern (e.g., M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used), Unstaggered SL-PRS patterns (e.g., M symbol(s) of SL-PRS with comb- N, at each symbol a same RE offset is used, N > 1) - The number of symbols of SL-PRS within a slot - Any relation to the comb-N option - RE offset pattern repetitions within a slot - FFS: Other frequency domain pattern(s) Agreement With regards to the frequency domain pattern, a Comb-N SL-PRS occupying M symbol(s) design should be introduced for the support of NR SL positioning - Note: there could be multiple values for M, N Agreement With regards to the frequency domain pattern for multi-symbol SL-PRS, prioritize partially and fully staggered SL-PRS. - Note: this does not preclude comb N=1 - FFS: single symbol SL-PRS, if supported

The following proposals were found in the contributions:

| Nokia, NSB | Proposal 9: Consider supporting the following parameters of SL-PRS structures: - The comb size N of SL-PRS: 1, 2, 4, 8. - The number of symbols (M) of SL-PRS: 1, 2, 4, 8, 12. Other values of M (1~12) can also be considered depending on resource allocation and SL-PRS multiplexing to be agreed.. Proposal 10: Consider inheriting all or part of UL SRS structures for SL PRS. Proposal 11: Consider supporting fully staggered SL-PRS pattern and partially staggered SL-PRS pattern. Don’t support unstaggered SL-PRS pattern. Proposal 12: Consider (pre-)configuring a subset of N and M from the whole set specified. |
| --- | --- |
| Huawei, HiSilicon | Proposal 7: The partial and full staggering Comb-N SL-PRS structure should be extended with one additional symbol that has the same frequency domain RE mapping as the first symbol to facilitate the Doppler frequency shift estimation for V2X scenario. - Proposal 8: Support subslot based SL-PRS pattern. - For example: The number of subslots within a slot can be 2, and the number of SL-PRS symbols within a subslot can be 5 for comb-2, comb-4, and comb-12. |
| vivo | - The candidate comb sizes of SL-PRS can be 2,4,6,12 - The candidate numbers of SL PRS symbols can be 2,4,6. - Partial staggered pattern can be considered for SL-PRS considering SL structure. |
| China Telecom | Proposal 2: refer to the existed PRS pattern in TS 38.211 Table 7.4.1.7.3-1 when determine the sets of values of M and N as well as RE offset. Specifically, N∈{2,4,6,12}, M∈{1,2,…,12}, and determine together the value of RE offset. Proposal 3: support the time domain repetition of SL-PRS within a PRS resource set. Proposal 4: support the introduction of muting pattern to SL-PRS in order to increase the flexibility. The existed two muting patterns of NR PRS can be considered as a starting point for SL-PRS muting configuration design. |
| OPPO | - Both full-RE mapping SL PRS pattern and comb-like (N>1) SL PRS pattern should be supported in SL positioning. |
| CATT | Proposal 5: Partially staggered pattern should be used as SL-PRS frequency domain pattern due to limited transmission distance of SL-PRS. - M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used. Proposal 6: Single symbol SL-PRS should be supported in order to improve the flexibility of SL-PRS configuration and transmission. Proposal 7: The candidate values of number of symbols of SL-PRS within a slot should be {1, 2, 4, 8}. |
| Intel | - Both fully and partially staggered pattern can be considered for SL-PRS. |
| NEC | Proposal 1: The resource allocation of SL-PRS in time and frequency domain should support non-contiguous resource allocation. |
| ZTE | Proposal 9: For SL-PRS frequency domain pattern, - Reuse the comb pattern of DL-PRS as much as possible; - Support both fully staggered SL-PRS pattern and partially staggered SL-PRS pattern. - One symbol SL-PRS excluding the AGC-symbol and Gap symbol should be supported - Consider Comb size N > 12, e.g. N = 16, 24, 32 and 64 |
| Xiaomi | Following the design in Uu interface, fully staggered SL-PRS pattern shall be at least supported. It can be further investigated whether partial SL-PRS pattern can be additionally supported, based on the evaluation on the performance of different SL-PRS patterns. |
| Lenovo | Proposal 5: Partial and Fully staggered comb patterns should consider the number of available symbols within a SL slot considering number of available symbols of 9 or 10. FFS whether a comb size of 12 symbols may be still supported. Proposal 6: RAN1 to consider at least the staggered SL PRS design option. FFS the positioning performance of partially staggered designs. Proposal 23: Mini-slot of various symbol length of SL-PRS e.g., 2, 4, and 6 can be further studied for a dedicated resource pool. |
| Interdigital | Proposal 32: Study RE-level frequency multiplexing of SL-PRS transmissions. |
| Fraunhofer | Proposal 10: Support staggered patterns for the sidelink positioning reference signal. |
| Samsung | Specifically, Uu PRS supports the comb size N={2,4,6,12} while Uu SRS supports the comb size N={2,4,8}. Similar comb sizes can be considered for SL PRS but we can consider N=1 for SL PRS. The advantage of supporting N=1, is to allocate all subcarriers to one UE, when it is not feasible to multiplex users on different combs within the same PRB. Next, Uu PRS supports the symbol length M={2,4,6,12} while Uu SRS supports the symbol length M={1,2,4,8,12}. These M value can be considered at first. However, other new M values can be considered also if it's necessary is clearly identified. Proposal 4: For the design of Comb-N SL-PRS occupying M symbol(s), consider to reuse Uu PRS/SRS design at first. - FFS: N=1 and other values for M |
| NTT DOCOMO | Proposal 7: - Comb-N SL-PRS occupying M symbol(s) design is discussed after details of resource pool for SL-PRS and signal structure including multiplexing with other channel/signal are concluded. |
| Qualcomm | Proposal 4: SL-PRS design has the following properties: N > 1, M >1, where M does not include the symbol used for AGC training. Proposal 5: Resuse at least a subset the frequency and time domain patterns from DL-PRS. |
| Mediatek | Proposal 3-1: The symbol(s) for AGC tuning could implicitly be resided in the SL-PRS pattern. Then AGC tuning and measurement could be performed within a SL-PRS resource Proposal 3-2: The more flexible symbol number for SL-PRS under full staggering structure could be considered, for example comb-4 with 6 symbols to allow using the first two symbols for AGC adjustment, and the remaining four symbols for measurement |

Based on the above proposals, we make the following observation:

- A lot of companies are suggesting to reuse the Uu PRS/SRS design as a starting point.

- A few companies are bringing up that FDMing of uEs may be a problem due to in-band emissions ,near-far effect, so they are suggesting to include comb-1 (Full RE pattern).

- 1 company (NEC) suggested that within a slot, a single SL-PRS resource should be non-consecutive symbols

- 1 company (Huawei, HiSilicon) suggested that we should to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol for the purpose of doppler estimation.

- 1 company suggested to discuss the number of SL-PRS symbols “excluding the AGC symbol”. It may be a way to focus our discussion without connecting it with the AGC symbol for now.

- With regards to fully-staggered & partially-staggered patterns, we observe the following.

- At least support fully staggered patterns: Xiaomi, Qualcomm

- Support both fully and partially staggered: Nokia, NSB, CATT, Intel

My suggestion is to leave both fully and partially staggered patterns for discussion.

[CLOSED] Feature Lead Proposal 4.1.2-v0

With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics:

- With regards to the value N (comb-type) and the number of SL-PRS symbols excluding the symbol(s) used for AGC training:

  - At least the following values are considered as candidate values: N = {1,2,4,6,8,12}

  - At least the following values are considered as candidate values: {1,2,4,6,8,12}

  - FFS: which of the above combinations shall be supported

  - FFS: Whether to consider N>12 as candidate value(s)

  - FFS: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol

- The symbols of a SL-PRS resource within a slot can be

  - Alt. 1: consecutive symbols

  - Alt. 2: consecutive or non-consecutive symbols

- RE-Offset sequence within a SL-PRS resource across the symbols should follow the Uu DL-PRS/SRS-Pos principle shown in the table below:

| NcombSL-PRS | Symbol number within the SL-PRS resource l-lstartSL-PRS |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| 2 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 4 | 0 | 2 | 1 | 3 | 0 | 2 | 1 | 3 | 0 | 2 | 1 | 3 |
| 6 | 0 | 3 | 1 | 4 | 2 | 5 | 0 | 3 | 1 | 4 | 2 | 5 |
| 8 | 0 | 4 | 2 | 6 | 1 | 5 | 3 | 7 | 0 | 4 | 2 | 6 |
| 12 | 0 | 6 | 3 | 9 | 1 | 7 | 4 | 10 | 2 | 8 | 5 | 11 |

Companies views

| MTK | 1, comb-1 is not preferred. Since SL transmission is mainly for short range, the full observation range of a OFDM symbol time is not actually needed. 2, we are okay for comb 2,4,6,8,12, which is a union of DL-PRS and SRS for POS 3, for the second subbullet within first main bullet, is it for M? If so, 12 is not preferred 4, N > 12 is not considered 5, for this term “whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol ”. Our view is, the Doppler estimation may rely on how large the time span of RS is. This is why in Rel-15 TRS is designed across two consecutive slots, having 2 symbols in each slot. We understand the intention of this proposal and okay for further study 6, for 2nd main bullet, we prefer consecutive symbols for a resource |
| --- | --- |
| CATT | We prefer to divide this proposal into 3 separate proposals for further discussion, in order to achieve the consensus one by one. For the first main bullet, the first sub-bullet and the second sub-bullet seems to be duplicated. For the second main bullet, we prefer Alt.1 For the third main bullet, we prefer to discuss it after the first main bullet was agreed, since it is related to the candidate values of N. |
| ZTE | Suggest using M to represent the number of SL-PRS symbols excluding the symbol(s) used for AGC training to align with previous agreements. With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics: - With regards to the value N (comb-type) and the number of SL-PRS symbols M excluding the symbol(s) used for AGC training: - At least the following values are considered as candidate values: N = {1,2,4,6,8,12} - At least the following values are considered as candidate values: M = {1,2,4,6,8,12} The last FFS seems like a special case of M>N. We need to discuss the relationship between the size of M and N before determining the SL-PRS pattern of an additional symbol. |
| CMCC | We are generally fine with this proposal, but the first two FFS bullets of candidate values under the 1st bullet are confusing, please see modifications below: - With regards to the value N (comb-type) and the number of SL-PRS symbols excluding the symbol(s) used for AGC training M: - At least the following values are considered as candidate values: N = {1,2,4,6,8,12} - At least the following values are considered as candidate values: M = {1,2,4,6,8,12} - FFS: which of the above combinations shall be supported - FFS: Whether to consider N>12 as candidate value(s) - FFS: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol For the FFS “Whether to consider N>12 as candidate value(s)”, we think if it is supported, partial stagger should also be supported, since one SL-PRS across multiple slots is not feasible. Besides, between the two alternatives for the symbols of a SL-PRS resource within a slot, we prefer Alt.1 since it is not so clear to support a non-contiguous SL-PRS within a slot. |
| LGE | For the 1st bullet, maybe we need clarification by adding M to represent the number of SL-PRS symbols. Then M needs to be added for the second sub-bullet. - With regards to the value N (comb-type) and the number of SL-PRS symbols M excluding the symbol(s) used for AGC training: - At least the following values are considered as candidate values: N = {1,2,4,6,8,12} - At least the following values are considered as candidate values: M ={1,2,4,6,8,12} We don’t support the 4th sub-bullet, which requires cross-slot scheduling. It causes quite complex scheduling issues, and we don’t think it’s needed. Suggest to remove the 4th sub-bullet. For the 5th sub-bullet, such a pattern does not exist in Uu DL PRS. We don’t think it’s necessary and suggest to remove the 5th sub-bullet. It could be separately discussed later if really necessary. - FFS: Whether to consider N>12 as candidate value(s) - FFS: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol For the 2nd bullet, We support Alt.1 consecutive symbols. We don’t seed the needs for non-consecutive symbols. The 3rd bullet is supported if the corresponding comb size N is used. |
| vivo | For the value N (comb-type), the comb size of DL-PRS with N∈{2,4,6,12} can be reused. For the candidate numbers of SL PRS symbols, considering SL structure (e.g., excluding the PSCCH symbol, AGC, or GP symbol for SL-PRS transmission), the candidate numbers of SL PRS symbols can be 2,4,6. For the symbols of a SL-PRS resource, we think alt 1. Consecutive symbols should be supported. Compared with consecutive symbols, non-consecutive symbols of SL-PRS may be more sensitive to Doppler shift. Therefore, we prefer Alt1( consecutive symbols of a SL-PRS). |
| Huawei, HiSilicon | Our preference is to only discuss the comb size first. Regarding the symbol number, it should consider the following factors: Available symbols in a slot available for SL-PRS The precence of PSCCH or PSSCH in a slot, which is related to dedicated resource pool as well as shared resource pool discussion We suggest to revise the proposal as below: With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics: - With regards to the value N (comb-type) and the number M of SL-PRS symbols excluding the symbol(s) used for AGC training: - At least the following values are considered as candidate values: N = {1,2,4,6,8,12} - FFS thevalues considered as candidate values for M - FFS: which of the combinations of (N,M) shall be supported - FFS: Whether to consider N>12 as candidate value(s) - FFS: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol - The symbols of a SL-PRS resource within a slot can be - Alt. 1: consecutive symbols - Alt. 2: consecutive or non-consecutive symbols NcombSL-PRS; l-lstartSL-PRS. |
| Lenovo | Generally feel that the supported SL-PRS resources within a slot should aim to consider the symobls allocated for AGC and Rx-Tx turnaround time (gap symbol), as this would affect the length of PRS symbols as deonted in 2nd sub-bullet of the first main bullet, where we are doubtful if M=12 can even be supported. We also do not think it is straight-forward to re-use the DL-PRS/SRS-Pos principle of SL-PRS. |
| Ericsson | OK with the proposal. |
| Sharp | We are fine with the proposal. We also prefer using “M” for the 2nd sub-bullet. As for the relationship between N and M, we are OK to keep it FFS as a part of “FFS: which of the above combinations shall be supported” for now. |

Lets continue the discussion with the following revised proposals:

[CLOSED] Feature Lead Proposal 4.1.2-v1

With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics:

- With regards to the value N (comb-type) and the number M of SL-PRS symbols excluding the symbol(s) used for AGC training:

  - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12}

  - FFS: thevalues considered as potential candidate values for M

  - FFS: which of the combinations of (N,M) shall be supported

  - FFS: Whether to consider N>12 as a potential candidate value(s)

  - FFS: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol

- The symbols of a SL-PRS resource within a slot can be

  - Alt. 1: consecutive symbols

  - Alt. 2: consecutive or non-consecutive symbols

- FFS: RE-Offset sequence within a SL-PRS resource

Companies views

| Fraunhofer | Okay |
| --- | --- |
| Apple | Fine with proposa; |
| Futurewei | OK |
| Intel | We are generally fine with the proposal. However, it is not clear to us why a single SL-PRS resource is not consecutive? What is the benefit of supporting such a design? |
| CATT | OK. |
| NEC | We are fine with the proposal. |
| DCM | OK, if M is FFS. As commented by HW, M is dependent on presence of PSCCH/PSSCH. |
| Samsung | For first sub-bullet, we would like to clarify if the gap symbol(s) is also excluded. We suggest the following update: - With regards to the value N (comb-type) and the number M of SL-PRS symbols excluding the symbol(s) used for AGC training and/or Rx-Tx turnaround time: For the second bullet, we prefer to decide with Alt 1. |
| LGE | We’re fine to focus on the comb size first. Then the 3rd sub-bullet is not necessary as we don’t have any candidate proposal for M in the 2nd sub-bullet. We’re still not convinced whether N>12 is really necessary. Also not convinced whether SL PRS should be used as TRS role because other channel/RS for SL positioning, e.g. for SLPP or measurement report can be used instead. Let’s remove it, or make it more generalized. ##### [HIGH][ONLINE] Feature Lead Proposal 4.1.2-v1’ With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics: - With regards to the value N (comb-type) and the number M of SL-PRS symbols excluding the symbol(s) used for AGC training: - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12} - FFS: thevalues considered as potential candidate values for M - FFS: which of the combinations of (N,M) shall be supported - FFS: Whether to consider N>12 as a potential candidate value(s) - FFS: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol - FFS: whether SL-PRS should be used for phase tracking purpose - The symbols of a SL-PRS resource within a slot can be - Alt. 1: consecutive symbols - Alt. 2: consecutive or non-consecutive symbols - FFS: RE-Offset sequence within a SL-PRS resource |
| Sharp | OK |
| OPPO | Basically fine, but for the second last bullet, seems it should be rephrased as “The symbols of a SL-PRS resource within a slot can be one of”, and we believe at least in the dedicated resource pool there is no point to support non-consecutive. |
| InterDigital | We support in general the proposal. For the 2nd bullet, we prefer Alt 1. For the 1st bullet, in our view, it would be beneficial to study the pattern in tandem with a SL-PRS structure and slot format considering multiplexing with AGC symbol, PSCCH symbol (if an associated PSCCH is defined in the same slot/mini-slot) and gap symbol, e.g., M=12 is not feasible if SL-PRS and PSCCH are (pre)configured to be TDMed. |
| CEWiT | Fine with the proposal |
| Qualcomm | We support in general but would like to reduce the number of FFSs by removing the following ones: - FFS: Whether to consider N>12 as a potential candidate value(s) - FFS: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol - FFS: RE-Offset sequence within a SL-PRS resource For the first FFS, it is not clear how an SL-PRS pattern with more than 12 symbols can fit in a 14 symbol slot while still providing time for ACG training and Rx-Tx turnaround. The other two FFSs are part of the sequence design. The part of the proposal on the SL-PRS resource is not clear. If it refers to a single SL-PRS transmission, then Alt 2 is inefficient from resource utilization since it requires additional AGC training and Rx/Tx turnaround time and should be removed. If it refers to multiple SL-PRS transmissions, then it is ok. We think it is for a single SL-PRS transmission and propose the following: - The symbols of a SL-PRS resource within a slot can be - Alt. 1: consecutive symbols - Alt. 2: consecutive or non-consecutive symbols |
| Nokia, NSB | OK |

[CLOSED] Feature Lead Proposal 4.1.2-v1

With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics:

- With regards to the value N (comb-type) and the number M of SL-PRS symbols within a slot excluding the symbol(s) used for AGC training:

  - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12}

  - FFS: the values considered as potential candidate values for M

  - [FFS1: Whether to consider N>12 as a potential candidate value(s)]

  - [FFS2: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol]

- The symbols of a SL-PRS resource within a slot can be

  - Alt. 1: consecutive symbols

  - [Alt. 2: consecutive or non-consecutive symbols]

- FFS: RE-Offset sequence within a SL-PRS resource

Companies are encouraged to comment whether they are OK with FFS1, FFS2, and Alt. 2 to be kept in the proposal or should be removed.

Companies views

| vivo | We prefer to remove FFS1, FFS2, and alt.2, but we can accept to be kept in the proposal |
| --- | --- |
| LGE | We support to remove all the brackets in FL proposal, as we commented in the earlier round. We share the view with QC on removing [Alt. 2]. |
| MTK | For FFS2, it seems to us that a SL-PRS resource could be designed to have the first symbol of excluding AGC symbol to have same RE offset as the last symbol. So we prefer to remove FFS2 |
| CATT | We can live with keeping the FFS1, FFS2 and Alt.2 at this stage, since we can further study them and see whether they are feasible. |
| Fraunhofer | Support the proposal and keep Alt.2 for further discussions |
| ZTE | For FFS1: We think FFS1 should remain in the proposal. Considering limited UE transmit power, we suggest to further consider Comb size N > 12 for SL-PRS to achieve larger power boosting and to support more uEs’ RS multiplexing. Especially now we are in the SI stage, the proposal should be inclusive and allow companies to further study. For FFS 2: We prefer to remove FFS2, we are also open to study the frequency pattern of SL-PRS symbols, but we may not need to be thas specific, can we try the following: - FFS: the values considered as potential candidate values for M - FFS2: study the frequency pattern of SL-PRS resource when M>N. For Alt.2: Prefer to remove Alt.2, we are confused about why do we need non-consecutive symbols and what’s the benefit. |
| NEC | Regarding the original intention of our proposal to support non-contiguous SL-PRS resource allocation in frequency domain or time domain, we think a SL-PRS resource at least may include non-consecutive sets of symbols for carrying more SL-PRSs in a TDM manner within a slot. Further, if “only slot-based allocation is considered” according to Alt.2 in the Proposal 5.1.1.D-v1, then a SL-PRS resource allocation should support nonconsecutive symbols of different SL-PRS within a slot for efficient resource utilization. And for the non-consecutive symbols for a single SL-PRS, we are also open to discuss further at this stage. |
| Qualcomm | We support removal of the items in brackets. |
| InterDigital | We prefer to remove Alt.2 in the 2nd main bullet. In our view, a SL-PRS resource is the granularity of SL-PRS transmission and resource selection and should comprise of consecutive symbols. One or more such SL-PRS resources can be TDMed within a slot and we are fine with that. We prefer to also remove FFS1 and FFS2 as the benefits of the study are not clear to us. |
| Intel | We support to remove FFS2 and Alt. 2. We share similar view as other companies that the benefit of non-consecutive SL-PRS symbols for a SL-PRS resource is not clear. It would introduce additional overhead on AGC and/or gap symbols. For FFS1, we can be open for further study. This may be beneficial to improve multiplexing capacity for SL-PRS transmission, especially when considering the IBE issues. |
| CMCC | We are generally fine As replied in the 1st round, for the FFS “Whether to consider N>12 as candidate value(s)”, we think if it is supported, partial stagger should also be supported accordingly, since one SL-PRS across multiple slots is not feasible under the limatition of “within one slot”. Besides, for the symbols of a SL-PRS resource, Alt 2 should be removed because for one SL-PRS, why it can be non-consecutive is not clear to us. |
| Locaila | We prefer to keep FFS1 and to remove FFS2 and Alt.2, but open to keeping FFS2 for further study |
| Huawei, HiSilicon | We are fine with the proposal including the two FFSs and the Alt.2 |
| OPPO | Prefer to remove FFS 1 and FFS2, as removing them does not prevent proponent companies to further study. We are fine to keep alt 2 for further study, it seems a bit early to preclude non-consecutive although it looks not so necessary. |
| Samsung | Is it common understanding that M is not include the gap symbol(s) for Rx-Tx turnaround time? If we see the next proposal as - With regards to Rx/Tx turnaround - Alt. 1: One symbol after the set of SL-PRS symbols can be used by a UE - Alt. 2: Time at the end of the set of SL-PRS symbols can be used by a UE - Note: “Time at the end” means that part of the last symbol is being used - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only) We can ask for clarification whether alt 2 means M include the Rx-Tx turnaround time, and alt 1 means not include. We prefer to remove all FFS bullets and Alt 2. |
| DCM | At least Alt 2 should be kept in this stage, which is dependent on outcome of discussion of section 5.1.1. When SL-PRS is multiplexed with PSSCH, non-contiguous would be necessary. |
| Lenovo | Supportive of the FL’s revised proposal |
| Ericsson | We prefer to remove FFS1, FFS2, and Alt 2 Regarding the comment from NEC “at least may include non-consecutive sets of symbols for carrying more SL-PRSs in a TDM manner within a slot.”, in our understanding this proposal is only discussing frequency and time domain pattern of a single SL-PRS resource. So, the proposal does not rule out TDM of multilple SL-PRS resources. So Alt 2 is not needed. |
| Xiaomi | We prefer to remove both FFS bullets. And we prefer to Alt 1 in the second bullet. |
| Nokia, NSB | OK |

We make the following observations:

FFS1:

- Keep: CATT, Fraunhofer, ZTE, Intel, Localia, Huawei, HiSilicon

- Remove: vivo, LGE, Qualcomm, Interdigital, Samsung, Ericsson, Xiaomi, OPPO

FFS2:

- Keep: CATT, Fraunhofer, Huawei, HiSilicon

- Remove: vivo, LGE, ZTE, Qualcomm, Interdigital, Intel, Localia, Samsung, Ericsson, Xiaomi, OPPO, MTK

Alt. 2:

- Keep: CATT, Fraunhofer, NEC, DCM, Huawei, HiSilicon, OPPO

- Remove: vivo, LGE, ZTE, Qualcomm, Interdigital, Intel, CMCC, Localia, Samsung, Ericsson, Xiaomi

#### 4.1.3 AGC and Rx-Tx Turnaround

The following agreement was reached during the previous meeting:

Agreement For a potential new SL PRS, study further the following - Number of symbol(s) for AGC and/or Rx-Tx turnaround time - Conditions under which AGC training and/or Rx-Tx turnaround time are needed

The following proposals were found in the contributions:

| Spreadtrum | Observation 1: AGC training and Rx-Tx turnaround time are always needed for SL-PRS transmission. Proposal 4: The number of symbol(s) for AGC and/or Rx-Tx turnaround time can be pre-configured per resource pool. |
| --- | --- |
| vivo | - One symbol for AGC and one symbol for GP symbol are assumed for one SL PRS. - Support multiple AGC symbols and one GP symbol in a slot. |
| CATT | Proposal 8: For the SL-PRS, one AGC symbol is needed before the SL-PRS transmission/reception, and one GP symbol is needed after the SL-PRS transmission/reception. Proposal 10: One AGC symbol should be always allocated before every SL-PRS transmission/reception or SCI+SL-PRS transmission/reception. And one GP symbol should be always allocated after every SL-PRS transmission/reception or SCI+SL-PRS transmission/reception. |
| Intel | - Support AGC and guard symbol for Tx-Rx turnaround time for a SL-PRS transmission in a dedicated SL-PRS resource pool. - Multiple AGC and guard symbols can be considered if SL-PRS transmissions from different uEs are multiplexed in a TDM manner. |
| Sony | Proposal 7: SL-PRS can be allocated within a resource pool and separated with a guard period. |
| ZTE | Proposal 8: For the design of AGC symbol: - AGC symbol is a duplication of the expected symbol next to the final symbol of fully/partially staggered SL-PRS. |
| CMCC | Proposal 5: Slot structure in NR sidelink should be reused as much as possible for SL-PRS slot, which including one AGC symbol, one GP symbol and the potential PSCCH symbols, the remaining symbols can be regarded as candidates for positioning RS. |
| LGE | Proposal 17: AGC symbol and TX/RX switching gap symbol are always inserted before the first and after the last SL PRS symbol of a SL PRS resource respectively. Proposal 18: Further study is needed how to construct the AGC symbol and the TX/RX switching gap symbol. |
| Qualcomm | Proposal 6: Sidelink PRS transmissions accommodate AGC training at the receiver by introducing a dedicated AGC training symbol prior to the first SL-PRS symbol. Observation 8: There are no conditions under which Rx-Tx turnaround time is not required for sidelink signals. Proposal 7: Sidelink PRS transmission accommodate Rx-Tx turnaround time and RAN1 to further study the details. |
| Mediatek | Proposal 3-1: The symbol(s) for AGC tuning could implicitly be resided in the SL-PRS pattern. Then AGC tuning and measurement could be performed within a SL-PRS resource Proposal 3-2: The more flexible symbol number for SL-PRS under full staggering structure could be considered, for example comb-4 with 6 symbols to allow using the first two symbols for AGC adjustment, and the remaining four symbols for measurement |

[CLOSED] Feature Lead Proposal 4.1.3-v0

  - At least for the dedicated SL-PRS resource pool,

- The first one or more symbol(s) of a set of contiguous SL-PRS symbols can be used for AGC training

  - FFS how to configure AGC symbol(s)

- Time at the end of a set of continuous SL-PRS symbols can be used by a UE for Rx/Tx turnaround

  - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only)

- Note: Multiple AGC and Rx/Tx turnaround time(s) can be considered if SL-PRS transmissions from different uEs are multiplexed in a TDM manner. Such discussions will take place as we progress more on the SL-PRS structure and slot format.

- Note: Related discussion for shared resource pool (if agreed) will happen separately

Companies views

| MTK | Generally okay |
| --- | --- |
| ZTE | ok |
| CMCC | We are OK with this proposal.By the way, we think the number of symbols for AGC should be kept as 1, similar to NR sidelink design. We are not so clear about whether and when more than one AGC symbols are needed. |
| LGE | For the number of symbols or time duration for AGC and Tx-Rx turnaround, the existing rule used in SL communication should be reused. We don’t need to reopen this issue, which were already specified in SL communication after quite long discussions between RAN1 and RAN4. For the 1st sub-bullet, following the principle above, only one symbol is necessary for AGC time. For the 2nd sub-bullet, following the principle above, only one symbol is necessary for Tx-Rx turnaround time. AGC and Tx-Rx turnaround symbol are always attached before the start and after the end of every SL PRS resource, for full flexibility in Tx/Rx and TDM of multiple SL PRSs within a slot. ##### [MEDIUM] Feature Lead Proposal 4.1.3-v0’ - At least for the dedicated SL-PRS resource pool, - The first one or more symbol(s) of a set of contiguous SL-PRS symbols can be is used for AGC training - FFS how to configure AGC symbol(s) - Time One symbol duration at the end of a set of continuous SL-PRS symbols can be is used by a UE for Rx/Tx turnaround - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only) |
| vivo | Firstly, we would like to know more about the case of using more symbols for AGC. In addition, more clarification is needed about “Time at the end of a set of continuous SL-PRS symbols”, whether it means one symbol after a set of continuous SL-PRS symbols |
| Huawei, HiSilicon | On AGC symbols, it may cause some confusion whether the generic term “SL-PRS symbols” includes AGC symbols or not. The same for the Rx/Tx turnaround. We suggest the following revision. - At least for the dedicated SL-PRS resource pool, - One or more symbol(s) preceding a set of contiguous SL-PRS symbols can be used for AGC training - FFS how to configure AGC symbol(s) - Time after a set of continuous SL-PRS symbols can be used by a UE for Rx/Tx turnaround - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only) |
| Spreadtrum | We are fine with HW’s version. |
| Apple | Okay in general. Agree with CMCC and LG that only 1 AGC symbol should be sufficient. |
| Futurewei | We support Huawei’s version for first sub-bullet, it is clearer. For the second sub-bullet the intention is not clear yet. Same question as Vivo. |
| Intel | We do not think more than one symbol for AGC and gap is needed. |
| CATT | We are fine with LGE’s revision. We prefer to use only one symbol for both AGC and GP, which is aligned with legacy Rel-16 sidleink design. This issue had been discussed in Rel-16 sidelink, no need to reopen this issue in Rel-18. |
| NEC | We are fine with the proposal in principle, and support one AGC symbol to be used. Regarding “if SL-PRS transmissions from different uEs are multiplexed in a TDM manner ” in the first Note, we believe “from the same UE” is also reasonable. |
| DCM | Based on 4.1.2-v1, SL-PRS may be contiguous, may not. In our understanding, if SL-PRS is always multiplexed with other channel/signal, SL-PRS is non-contigous. Then, in this case, AGC symbol is unnecessary for each contiguous SL-PRS set. Thus, ‘contiguous’ should be removed from 1st bullet, and also from 2nd bullet. Besides, more than one symbol would be unnecessary as commented by companies. |
| Samsung | At first RAN1 need to decide/discuss which additional channels and signals will be included in the dedicated SL-PRS resource pool (e.g., for SCI). We need to ask to RAN4 about how many AGC symbol are necessary based on the RAN1 decision above. |
| Sharp | Generally OK. Agree with CMCC, LG and Apple. |
| OPPO | 1. In SL communicaiton 1 symbol is used for AGC or Rx-Tx turnaround, we agree with several other companies that this should be resued for SL pos, no reason to consider other length. 2. we tend to agree with Huawei’s phrase, which is clearer and avoids some potential confusion. 3. As this proposal is for dedicated resource pool, as we commented in 4.1.2, we think in this case SL PRS should use consecutive symbols, otherwise, we tend to agree with DCM’s comments above. 4. The FFS for SL-TDOA should be kept, in SL commununiction UE needs to receive in all slots except the slots for its own transmission, because UE has to perform sensing or receive SL data from other uEs, but in SL-TDOA we see anchor UE/target UE does not need to receive/transmit SL PRS, it is not advisable to waste 1 symbol (7% of 1 slot) for Rx-Tx turnaround. |
| InterDigital | We support the principle of the proposal with the comments below. We agree with LG/Huawei that one AGC symbol as used in SL baseline is sufficient. For the 1st Note, we’d like to have a clarification on this below. At the current stage of discussion, we think multiple TDMed SL-PRS transmissions within one slot from one UE are not precluded and can be included in this Note. Note: Multiple AGC and Rx/Tx turnaround time(s) can be considered if SL-PRS transmissions from different uEs are multiplexed in a TDM manner. Such discussions will take place as we progress more on the SL-PRS structure and slot format. |
| Xiaomi | We support a single AGC symbol and a single GP symbol, similar as that of SL communication. |
| Qualcomm | We are ok with Huawei’s changes. We’d like to clarify what configuring the AGC symbols entails: is it only about the number of PRS symbols or something else? We suggest replacing that FFS with a general FFS details. - FFS how to configure AGC symbol(s) - FFS details |
| Nokia, NSB | OK with LGE’s revision. |

Note the properties of a SL-PRS resource are described in 4.1.2-v1, and the plan is to first treat that proposal.

[CLOSED] Feature Lead Proposal 4.1.3-v1

  - At least for the dedicated SL-PRS resource pool, and assuming a SL-PRS resource with contiguous symbols,

- With regards to AGC training

  - Alt. 1 One or more symbol(s) preceding the set of SL-PRS symbols can be used

    - Note: Whether it will be only one or it can be one or more is up for further discussions

  - Alt. 2 One symbol preceding the set of SL-PRS symbols can be used

- With regards to Rx/Tx turnaround

  - Alt. 1: One symbol after the set of SL-PRS symbols can be used by a UE

  - Alt. 2: Time at the end of the set of SL-PRS symbols can be used by a UE

    - Note: “Time at the end” means that part of the last symbol is being used

  - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only)

- Note: Multiple AGC and Rx/Tx turnaround time(s) can be considered if SL-PRS transmissions from the same or different uEs are multiplexed in a TDM manner. Such discussions will take place as we progress more on the SL-PRS structure and slot format.

Companies are encouraged to comment which alternative they are supporting.

Companies views

| vivo | Alt 2+Alt 1 In addition, we still wonder how to use the part of the symbol for Rx/Tx turnaround, which means the UE may receive the part of a symbol of SL-PRS and then switch to Tx by remaining part of the symbol. |
| --- | --- |
| LGE | We support Alt. 2 for AGC, and Alt. 1 for Rx/Tx turnaround. It’s much simpler and is aligned with the SL communication slot structure. We don’t think FFS is needed. At least one symbol each for AGC and Rx/Tx turnaround is need in a slot, similar to the SL communication slot. Suggest to remove the FFS. We can discuss this issue later after deciding the TDM issue described in note. - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only) |
| MTK | For the description “Note: “Time at the end” means that part of the last symbol is being used” is strange to us. For Huawei’s version: “Time after a set of continuous SL-PRS symbols can be used by a UE for Rx/Tx turnaround” it seems to say it is not limited to one symbol time, or partial of a symbol time. If we want to create alt. 2, we prefer using Huawei’s version |
| CATT | We prefer to remove the Alt.1 for AGC and remove the Alt.2 for Rx/Tx turnaround, i.e., using only one symbol for both AGC and GP, which is aligned with legacy Rel-16 sidleink design. This issue had been discussed in Rel-16 sidelink, no need to reopen this issue in Rel-18. |
| ZTE | Alt. 2+Alt. 1 With regards to AGC training, more than one symbols preceding a SL-PRS resource is unnecessary and we do not see any technical reasons for this design. As indicated in the note, multiple AGC and Rx/Tx turnaround time(s) can be considered if SL-PRS transmissions from the same or different uEs are multiplexed in a TDM manner. However, this “multiple” means maybe more than one AGC and Rx/Tx turnaround times are needed per slot. Each UE may occupy a SL-PRS resource and only one AGC and Rx/Tx turnaround time is needed per SL-PRS resource. With regards to Rx/Tx turnaround, how does it work if part of the last symbol is used for Rx/Tx turnaround? |
| NEC | We prefer to Alt.2 + Alt.1 to align with legacy SL slot structure, and share the similar view as CATT that no need to reopen the discussed issue. |
| Qualcomm | We are ok with the proposal. While we prefer Alt 2 for AGC, we are ok to keep the alternative for futher study at this point. For Rx/Tx we are ok to study both at this stage. The benefit of Alt 2 is that it could overhead per SL-PRS transmission and allow more SL-PRS transmissions to occur in a given time period. The change from MediaTek/Huawei would make Alt 2 equivalent to Alt 1 in our understanding. |
| InterDigital | We support Alt. 2 for AGC and Alt 1 for RX/TX switch symbol. We agree with LG on the alignment with baseline SL slot structure. We prefer to remove FFS in the 2nd main bullet to simply the design. |
| Intel | As commented by many companies, we do not see the need to introduce more than one symbols for AGC and gap for turn-around time. We suggest to remove Alt. 1 for AGC and Alt. 2 for Rx/Tx turnaround. Further, given the main bullet is for a single SL-PRS resource, we suggest to modify the alt. 2 as - Alt. 2 One symbol preceding the set of SL-PRS symbols SL-PRS resource can be used - Alt. 1: One symbol after the set of SL-PRS symbols SL-PRS resource can be used by a UE In addition, it is not clear to us whether “the same” is needed in the note. If SL-PRS is from the same UE, we do not need multiple AGC and Rx/Tx turnaround time. |
| CMCC | For the AGC training, in Alt 1, we do not understand why more than 1 symbol is required, since most companies have concern on this, we think the number of AGC symbol should be fixed as 1, no further discuss is needed. For Alt1 and Alt 2 for both the AGC and Rx/Tx turnaround, we think Alt 1 should be baseline, whether Alt 2 is feasible may depend on whether a UE can perform contiguous SL-PRS transmission within a slot or consecutive slots, this is also related to whether contiguous resource allocation can be achieved by both Scheme 1 and Scheme 2. |
| Huawei, HiSilicon | We are fine with the proposal. |
| NEC | @Intel In our understanding, if different SL-PRS transmissions from the same UE with significant difference(e.g. different pattern for different Rx uEs and/or related to different positioning methods with significant change in the number of RE occupied), additional AGC may need to be considered. |
| OPPO | Regarding the lengh of time for AGC and Rx/Tx turnaround, we shared the view that legacy design should be followed, we did not see any motivation to use a different design for SL PRS. We prefer to keep the FFS under the 2nd sub-bullet, it would give companies an impression that Rx/Tx turnaround time is always needed after each SL PRS resource if removed, as we elaborated in the last round, adding one symbol for Rx/Tx turnaround unnecessarily would lose 7% resource of a slot, it should be avoided whenever possible. |
| Samsung | As we commented before, for AGC symbols, we need to decide/discuss at first which additional channels and signals are included in the dedicated SL-PRS resource pool (e.g., for SCI). We think this is important to decide the number of AGC symbols. In addition, we would like to ask what’s the criteria to determine the number of AGC symbol(s). It that’s based on UE capabiality of AGC processing time, since single symbol is sufficient in legacy SL system, the motivation of more symbols is unclear. Also, we are not clear on the meaning of: “One symbol after the set of SL-PRS symbols can be used by a UE” The guard symbol should not be used by the UE. |
| Spreadtrum | Support the proposal. |
| DCM | Alt 2 + Alt 1. Same as legacy structure is enough. |
| Lenovo | Generally supportive of FL’s proposal. One clarification on the Rx/Tx Turnaround for Alt. 1 +2, when we refer to “can be used by a UE” we are actually descibing it relation to the Rx or Tx UE perspective ? It was a bit ambigous. |
| Ericsson | Regarding the number of symbols for AGC training, we are wondering why we need more than one symbols? We think it is better to align this design with with SL communications. So we prefer Alt 2 only for AGC training. For similar reasoning, we prefer Alt 1 for Rx/Tx turnaround. |
| xiaomi | Alt 2+ Alt 1. We share other companies view that legacy design of 1 symbol AGC and GP shall be reused if there is no critical issue. |
| Nokia, NSB | OK with Alt.2 for AGC, and Alt. 1 for Rx/Tx turnaround following the legacy SL design. |

We make the following observations:

- Alt. 1:

- Alt. 2: vivo, LGE, CATT, ZTE, NEC, Qualcomm, Intel, CMCC, OPPO, DCM, Ericsson, Xiaomi, Nokia, NSB

- Alt. 1: vivo, LGE, MTK, CATT, ZTE, NEC, Intel, CMCC, DCM, Ericsson, Xiaomi, Nokia, NSB

- Alt. 2: Qualcomm

2 companies said are OK with the current proposal: Huawei, HiSilicon, Spreadtrum

Samsung commented that the expression “can be used by a UE” may not be appropriate.

There was a comment by one to remove the “FFS”. It is just an FFS, and only one commented this. Lets keep it for now. Based on the above, there is very strong majority to already go with Alt. 2 in the first and Alt. 1 in the 2nd. For the sake of progress, i am suggesting the following.

##### [HIGH] Feature Lead Proposal 4.1.3-v2

  - At least for the dedicated SL-PRS resource pool, and assuming a SL-PRS resource with contiguous symbols,

- With regards to AGC training

  - One symbol preceding the set of SL-PRS symbols can be used

- With regards to Rx/Tx turnaround

  - One symbol after the set of SL-PRS symbols can be used by a UE

  - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only)

- Note: Multiple AGC and Rx/Tx turnaround time(s) can be considered if SL-PRS transmissions from the same or different uEs are multiplexed in a TDM manner. Such discussions will take place as we progress more on the SL-PRS structure and slot format.

##### Companies views

| LGE | Support FL proposal with the following comments. 1) We understand the set of SL-PRS symbols in the proposal as the set of symbols belonging to a single SL PRS resource. 2) We prefer to remove FFS with the reason commented in the previous round, but for the progress let us suggest the following modification. In our point of view, the FFS applies when multiple SL PRS resources are multiplexed in a slot. This corresponds to the case of Note, so we suggest to include such case into Note. We can discuss FFS after the SL PRS resource multiplexing discussion is completed, as Note said. ##### [HIGH] Feature Lead Proposal 4.1.3-v2 - At least for the dedicated SL-PRS resource pool, and assuming a SL-PRS resource with contiguous symbols, - With regards to AGC training - One symbol preceding the set of SL-PRS symbols can be used - With regards to Rx/Tx turnaround - One symbol after the set of SL-PRS symbols can be used by a UE - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only) - Note: Multiple AGC and Rx/Tx turnaround time(s), or no Rx/Tx turnaround time can be considered if SL-PRS transmissions from the same or different uEs are multiplexed in a TDM manner. Such discussions will take place as we progress more on the SL-PRS structure and slot format. |
| --- | --- |
| ZTE | Does “the set of SL-PRS symbols” represent “a SL-PRS resource with contiguous symbols”? if so, we do not think the note is needed. Same or different UEs may be configured with multiple SL-PRS resources which are multiplexed in a TDM manner, then it is natural that each SL-PRS resource related to one AGC and Rx/Tx turnaround time. Or we should specify “Multiple AGC and Rx/Tx turnaround time(s) can be considered per slot” |
| Intel | We support this proposal and are also fine with the changes proposed by LGE. |
| InterDigital | We agree with the principle of the proposal. Our understanding based on the wording of the proposal is a SL-PRS resource includes a set of contiguous symbols and these contiguous symbols include both AGC and Rx/Tx switch symbols and SL-PRS symbols. If that is correct understanding, then “multiple AGC and Rx/Tx turnaround time” will be needed when multiple SL-PRS resources are applied for SL-PRS transmission. And furthermore, the Notes can be modified as indicated below. - At least for the dedicated SL-PRS resource pool, and assuming a SL-PRS resource with contiguous symbols, - With regards to AGC training - One symbol preceding the set of SL-PRS symbols can be used - With regards to Rx/Tx turnaround - One symbol after the set of SL-PRS symbols can be used by a UE - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only) - Note: Multiple SL-PRS resources multiplexed in a TDM manner in a slot AGC and Rx/Tx turnaround time(s) can be considered for if SL-PRS transmissions from the same or different uUEsare multiplexed in a TDM manner. Such discussions will take place as we progress more on the SL-PRS structure and slot format. |
| Qualcomm | We accept the FL’s proposal. We prefer to remove the FFS since even a UE transmitting only SL-PRS might have to receive other SL signaling, e.g. for configuration. However, we can accept the FL’s proposal or LG’s version for progress. |
| CATT | We prefer to remove the FFS, the Rx/Tx turnaround should still be included. |
| CEWiT | We too prefer to remove FFS, as turn around time will be needed and agnostic to the SL positioning method. |
| OPPO | We support FL’s proposal in general, for progress, we can also accept LG’s proposal on the FFS bullet. In addition, we’d like to suggest following changes for clarity. - At least for the dedicated SL-PRS resource pool, and assuming a SL-PRS resource with contiguous symbols, - With regards to AGC training - One symbol preceding the a SL-PRS resource set of SL-PRS symbols can be used - With regards to Rx/Tx turnaround - One symbol after the SL-PRS resource set of SL-PRS symbols can be used by a UE - FFS: Discuss further scenarios that this may not be needed (e.g. SL-TDOA where a UE receives only) - Note: Multiple AGC and Rx/Tx turnaround time(s) can be considered if SL-PRS transmissions from the same or different uUEs are multiplexed in a TDM manner within a slot. Such discussions will take place as we progress more on the SL-PRS structure and slot format. |
| vivo | Share the same view as Qualcomm to remove the FFS |
| Samsung | For the note, it needs to clarify whether “multiple AGC and Rx/Tx turnaround time(s)” means in a slot there can be multiple symbols or multiple positions used for the purpose of AGC and/or Rx/Tx turnaround, or it means the length of AGC and Rx/Tx turnaround time can have multiple values. |
| NEC | We support the proposal in principle, and also think the clarification of “a SL-PRS resource” is needed. |
| Apple | Given that th FFS is a “discussion”, we are fine with this as supporting companies can bring their justification for the elimination of the Rx/Tx turnaround. On Oppo’s update, it has been mentioned that we do not have a definition for the SL-PRS resource yet. |

#### SL-PRS Configuration/Triggering/Activation/Reservation

The following agreement was reached with regards to this topic:

Agreement With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed. Agreement - With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further. - With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement.

The following proposals were found in the contributions:

| Nokia, NSB | Proposal 21: (Pre-)configured SL PRS can be activated/deactivated based on indications coming from the higher layers, e.g., based on parameters that are (pre-)configurable by the network. Proposal 22: Study (re-)configuration of SL-PRS as per changing target-anchor link conditions. |
| --- | --- |
| Futurewei | Observation 3: High-layer and lower-layer signaling involvement in the SL-PRS configuration (Option 2) offers a more flexible solution that can cope with in coverage and out of coverage scenarios of SL positioning. |
| Huawei, HiSilicon | - Proposal 15: Support Option 2: high-layer and lower-layer signaling involvement in the SL-PRS configuration, where - SL-PRS resource pool configurations is provided by high-layer signaling. - Activation/triggering information is provided by lower-layer signaling. - Proposal 16: Support both periodic and one-shot resource reservation for SL-PRS transmission. - At least for RTT based positioning, one-shot reservation should be supported. |
| Spreadtrum | Proposal 3: Combining high-layer and lower-layer signaling to achieve the configuration/activation/ deactivation/triggering of SL-PRS should be supported. |
| vivo | - For Scheme 1, if configured grant type 2 or dynamic scheduling like mechanism are supported, RRC and DCI is required. - For Scheme 2, the SCI is required to reserve resource and sensing mechanism. - Option 2 (i.e., high-layer and lower-layer signaling) should be supported in the SL-PRS resource reservation. |
| China telecom | Proposal 5: Support option2 for SL-PRS configuration/activation/deactivation/triggering. The indication mechanism can refer to DL-PRS. |
| CATT | Proposal 9: SCI which indicates the resource allocation of a SL-PRS resource should occupy at least one symbol before the transmission of the SL-PRS resource. Proposal 21: The SCI used for the scheduling of SL-PRS, should be transmitted in the dedicated resource pool for sidelink positioning. Proposal 22: For the SCI content, the information transmitted by legacy 1st-stage SCI and 2nd-stage SCI defined in Rel-16 V2X should be as a starting point for Rel-18 . |
| Sony | Proposal 8: Support high-layer and lower-layer signalling involvement in the SL-PRS configuration. Proposal 9: High-layer and lower-layer signalling is provided from serving gNB to the UE. |
| ZTE | Proposal 15: At least support using SCI to trigger/reserve SL-PRS resource(s) - One SCI may schedule multiple SL-PRS occasions for overhead reduction Proposal 16: If periodic SL-PRS or high-layer-only signaling involvement of SL-PRS is supported, how to resolve resource collision should be further addressed and discussed especially for resource selection scheme 2. |
| CMCC | Proposal 9: SL-PRS should also be transmitted along with PSCCH within a same slot in the dedicated resource pool for SL-PRS to reserve further resources and mitigate the resource collision possibility. Proposal 13: RAN1 should further study the following alternatives for SL-PRS reception: - Alt 1: The NR sidelink principle is followed. - SCI is used to indicate the current PSSCH for receiver UE, and reserve subsequent resources for the resource exclusion procedure of other transmitter UE(s) only. - Alt 2: The NR Uu principle is followed. - SCI is used to trigger one or more transmissions and to notify the receiver UE where to perform resource reception. Proposal 14: With regards to the configuration/activation/deactivation/triggering of SL-PRS, option 2 in the agreements from RAN1#109-e should be selected. Proposal 6: Parameters of SL-PRS, e.g., number of symbols, RS comb size, and RS BW, should be (pre)configured on resource pool level. Proposal 7: To improve the positioning accuracy, it can be considered defining the BW of SL-PRS to be equal to that of the resource pool. |
| Lenovo | Proposal 15: RAN1 to support Option 2 of configuration/activation/deactivation/triggering of SL-PRS, i.e., higher and lower layer involvement. Proposal 16: RAN1 to consider the resource allocation triggering mechanism of SL PRS transmission(s) in terms of which entity, e.g., anchor-UE, target-UE, gNB, LMF and the type of trigger, e.g., higher-layer SL positioning/ranging service request. RAN2 input may be required. |
| LGE | Proposal 6: At least for broadcast type of SL PRS transmission, SL PRS configuration is (pre-)configured per SL PRS resource pool, which includes at least the following fields. Other fields of SL PRS configuration is dynamically signaled by a PHY layer signaling. - SL PRS bandwidth - Set of the allowed configurations of SL PRS resource - Resource timing - Periodicity and offset - Comb pattern - Number of symbols - etc. - whether the SL PRS retransmission is allowed - whether the multiple SL PRS configurations are allowed Proposal 13: New 2nd SCI format is introduced for a control channel associated with SL PRS. - 2nd SCI format indicator in 1st SCI indicates that 2nd SCI is used for SL PRS control channel. |
| Interdigital | Proposal 14: Study a new SCI format associated with SL-PRS transmission. |
| Samsung | Proposal 6: In Scheme 1 (network centric SL-PRS resource allocation), gNB and/or LMF makes a decision for SL PRS resource allocation and the corresponding information is indicated to UE by DCI. - It is up to RAN2 whether Scheme 1 is performed at gNB and/or LMF. Proposal 8: For indication/reservation of SL-PRS resource(s), consider the following options for SL control information - Option 1: Same-slot SL control information signalling - Option 2: Cross-slot SL control information signalling |
| Asustek | Proposal 2: For providing scheduling/reserving information of SL PRS resources, introduce new SCI on new PSCCH in dedicated resource pool for SL PRS. |
| NTT DOCOMO | Proposal 5: - For option 1 of SL-PRS configuration/indication, periodic SL-PRS transmission is (pre-)configured per UE. - For option 2 of SL-PRS configuration/indication, SL-PRS properties are (pre-)configured per resource pool and can be configured by PC5-RRC signaling. Aperiodic SL-PRS transmission is indicated/requested by SCI. |
| Qualcomm | Proposal 11: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers only. Observation 12: An explicit trigger is not needed for the reception of SL-PRS. Reservation information suffices to inform the receiver when to expect SL-PRS. Proposal 12: Support SL-PRS transmission activation/deactivation at the physical layer by the UE’s own higher layers only. Observation 13: PC5-RRC configuration is limited to unicast and could only be applied for that cast type. Hence it is not suitable as the sole method for SL-PRS configuration. Proposal 14: SL-PRS resource reservations are indicated to the receiver UE by a higher layer message. |
| CEWiT | Proposal 12: Regarding the configuration/activation/deactivation/triggering of SL-PRS, option 2 (High-layer and lower-layer signalling involvement in the SL-PRS configuration) should be supported. Proposal 13: For sidelink positioning SL-PRS configuration/activation/deactivation/triggering, higher layers will specify the common assistance information, and set of PRS configurations. Proposal 14: At least PRS configuration triggering should be supported with lower layer signalling. This signalling will be through DCI in scenario A and through SCI in scenarios B and C. |
| Ericsson | - With regards to the configuration/activation/deactivation/triggering of SL-PRS, consider Option 2 (High-layer and lower-layer signaling involvement in the SL-PRS configuration) in NR Rel-18. |

Activation/triggering may need to be treated separately for scheme 1 and scheme 2, so i am suggesting to continue the discussion of these aspects in Section 5.2. So for now, lets continue on the configuration and reservation.

- With regards to configuration: It is clear that at least some part related to SL-PRS needs to be configured by higher layers. Some companies suggest it should be the resource pool for positioning, or some others may think that also bandwidth should be configured by higher layers, or periodicity, etc.

- With regards to the reservation/indication for the resources to the receiving UE, the majority of companies think that the transmitting UE shall inform the receiving UE through some lower-layer SL signaling, and specifically SCI signaling.

Based on the discussison above, I make the following proposal for configuration & reservation signaling:

[CLOSED] Feature Lead Proposal 4.1.4.A-v0

With regards to the configuration of SL-PRS:

  - A transmitting UE may receive at least part of SL-PRS related configuration from higher layers

    - FFS: which part(s) of the SL-PRS configuration; e.g. dedicated resource pool for positioning configuration, SL-PRS bandwidth configuration, number of SL-PRS symbols, periodicity, sequence-IDs, comb, repetitions, etc.

Companies views

| MTK | Generally okay. But it seems a receiving UE may also need to receive some higher layer configuration related to SL-PRS |
| --- | --- |
| CATT | We share the same with MTK that both transmitting UE and receiving UE may receive part of SL-PRS related configuration from higher layers. Then, we prefer the main bullet can be updated as follows, - A transmitting UE involving SL positioning may receive at least part of SL-PRS related configuration from higher layers |
| ZTE | Fine with CATT’s update. |
| CMCC | Some clarifications are needed for this proposal from our side. This proposal reads like only configuration of SL-PRS for Tx UE is discussed, is it the common understanding that the SL-PRS configuration we discussed so far is from Tx UE point of view? Then what about the Rx UE side? In Uu positioning, the PRS configuration is provided by LMF via LPP to UE for reception. In sidelink, e.g., SL CSI-RS, the configuration is from Tx UE to Rx UE via PC5-RRC, all configuartions consideres Rx UE. Or, are we talking about the sidelink data reception mechanisms, where some necessary configurations can be RP level, the Rx UE performs blind detection in all Rx RPs? In addition, the meaning of “dedicated resource pool for positioning configuration” in the FFS part should be clarified, does this mean a exact resource pool for SL-PRS transmission, or the resource pool-level configuration which is configured along with the resource pool. |
| LGE | We support FL proposal with the following clarification. The SL PRS configuration is applied to both TX UE and RX UE. So we don’t need to restrict the proposal only to TX UE. And it also needs to include pre-configuration for out-of-coverage case, where the pre-configuration can be inserted into the chip from the manufacturing stage. In this case, it does not come from a higher layer. To represent this case in SL communication, we usually use the terminology (pre-)configuration, which includes the case of the configuration by higher layer. As a summary, we suggest the followings. ##### [HIGH] Feature Lead Proposal 4.1.4.A-v0’ With regards to the configuration of SL-PRS: - A transmitting UE may receive is (pre-)configured at least part of SL-PRS related configuration from higher layers. - FFS: which part(s) of the SL-PRS configuration; e.g. dedicated resource pool for positioning configuration, SL-PRS bandwidth configuration, number of SL-PRS symbols, periodicity, sequence-IDs, comb, repetitions, etc. |
| vivo | Based on the previous discussion, the majority support higher-layer and lower-layer, we don’t know why we only discuss higher-layer and without lower-layer. So, can we add a sub-bullet for the lower layer, for example, A receive UE may receive part of the time and frequency information of SL-PRS from lower layers |
| Huawei, HiSilicon | OK |
| Lenovo | Supportive of FL’s proposal but also share the view that both Tx/Initiator UE and Responder/Rx UE(s) may receive the SL-PRS related configurations. So we could keep this aspect general at this stage. |
| Ericsson | OK. |
| Sharp | We have similar views with MTK that the proposal should cover not only transmitting UE but also receiving UE. As for LGE’s modification, we prefer separately discussing the pre-configuration case. |
| Spreadtrum | Support |
| FL Response | To vivo: The fact that this says “a part of the configuration”, it means that at least part of it will be higher-layer. For the remaining “part of the configuration”, it’s the next proposal as you see below where we are explicitly saying that the Rx UE is learning the reservation/indication of SL-PRS through SCI. To all: Indeed, the Rx UE should be getting part of the configuration also through higher layers. Please comment using the following also in your considerations: To LGE: Preconfiguration is passed to the PHY-layer of a UE as higher layer message (e.g. in SL communication, its RRC) from the PHY-point of view. The intention of this proposal is not to exclude the SL-communication-like (pre-)configuration signaling (i.e. RRC-based), but also to leave open the case that a different higher layer protocol is being used (e.g. the one being discussed in RAN2). At this point, the proposal need to be inclusive enough for both signaling solutions. If I remove the word “configuration” with the word “parameters” and add a Note, would it be more clear? With regards to the configuration of SL-PRS: - A transmitting UE may receive at least part of SL-PRS related configuration parameters from higher layers - FFS: which part(s) of the SL-PRS configuration; e.g. dedicated resource pool for positioning configuration, SL-PRS bandwidth configuration, number of SL-PRS symbols, periodicity, sequence-IDs, comb, repetitions, etc. - Note: This includes the (pre-)configuration case also. |
| Apple | Fine with updated proposal |
| Futurewei | OK with the revised Proposal |
| Intel | We are generally fine with the updated proposal. We prefer the wording “configuration” instead of parameters for SL-PRS. |
| CATT | We share the same view with Intel. We also prefer use the the wording “configuration” instead of parameters for SL-PRS. |
| NEC | Support the updated version by FL. |
| DCM | OK with the updated version. |
| Samsung | For the FFS bullet, it is unclear why dedicated resource pool is separately mentioned here and how to handle the shared pool case, if support. We suggest to modify it as: FFS: which part(s) of the SL-PRS configuration; e.g. dedicated resource pool for positioning configuration, SL-PRS bandwidth configuration, number of SL-PRS symbols, periodicity, sequence-IDs, comb, repetitions, etc. |
| Sony | Fine with the updated proposal |
| Sharp | OK with the updated proposal. |
| OPPO | Fine in general, a couple of comments on the FFS bullet: The first example should be “dedicated resource pool for positioning configuration” as this is needed for shared pool case. It is also better to clarify what “comb” mean, it refers to comb size, or both comb size and RE offset? |
| InterDigital | We agree with LG’s update on the proposal. |
| Xiaomi | We are fine with the FL updated proposal. |
| CEWiT | Fine with Updated proposal from FL. |
| Qualcomm | We are ok with the FL’s updated proposal. |
| Philips | Agree with FL’s updated proposal. |
| Nokia, NSB | OK with the revised proposal. |

[ONLINE] Feature Lead Proposal 4.1.4.A-v1

With regards to the configuration of SL-PRS:

  - A UE may receive at least part of SL-PRS related [configuration/parameters] from higher layers

    - FFS: which part(s) of the SL-PRS configuration; e.g. dedicated resource pool for positioning configuration, SL-PRS bandwidth configuration, number of SL-PRS symbols, periodicity, sequence-IDs, comb-size, repetitions, etc.

    - Note: This includes the (pre-)configuration case also.

For reservation, lets focus first on the dedicated resource pool, until there is more clarity on the status of the shared resource pool. In the proposal below we just try to agree that some SCI signaling will be there (any detail on 1st-stage SCI, or 2nd-stage, or cross-pool indication/reservation, or indicating multiple resources in the same slot with single SCI, etc, is FFS).

[CLOSED] Feature Lead Proposal 4.1.4.B-v0

With regards to the reservation of SL-PRS at least for the case of dedicated resource pool for positioning:

  - A transmitting UE may send SCI to a receiving UE for indicating a SL-PRS resource to be measured

    - FFS: Details

  - FFS: Reservation signaling if shared resource pool is agreed to be introduced

  - FFS: Whether a higher layer signaling reservation message could also be considered

Companies views

| MTK | Generally fine. The reservation through SCI is not just to let any receiving UE to know. It lets the uEs after decoding know there are resources to be reserved. |
| --- | --- |
| CATT | Support this proposal in principle. For the first main bullet, the SCI may be sent to several receiving Ues considering the boardcast and groupcast cases of SL-PRS. So, we prefer the first main bullet to be updated as follows, - A transmitting UE may send SCI to one or more a receiving UE(s) for indicating a SL-PRS resource to be measured |
| ZTE | We share the similar view with MTK and CATT. |
| CMCC | Comments. For the FFS about reservation, we think even for dedicated resource pool, the reservation signaling is also needed at least for Scheme 2; For the last FFS, we think it should be removed since in NR sidelink only SCI can reserve further resources, this should be the baseline for SL-PRS. |
| LGE | We support FL proposal with the following clarification. SCI can be monitored by not only the intended RX UE but also any UE that monitors to avoid the resource collision. So we may clarify this point to avoid unnecessary confusion in the proposal. ##### [HIGH] Feature Lead Proposal 4.1.4.B-v0’ With regards to the reservation of SL-PRS resource(s) at least for the case of dedicated resource pool for positioning: - A transmitting UE may send SCI to a receiving UE for indicating indicates a the SL-PRS resource(s) to be measured - FFS: Details - FFS: Reservation signaling if shared resource pool is agreed to be introduced - FFS: Whether a higher layer signaling reservation message could also be considered |
| vivo | In the first bullet, the wording “to a receiving UE” and “to be measured” can be removed. The usage of SCI may also be used for sensing. It is not necessary to limit the usage of the SCI transmitted by TX UE. So, we propose With regards to the reservation of SL-PRS at least for the case of dedicated resource pool for positioning: - A transmitting UE may send SCI to a receiving UE for indicating a SL-PRS resource to be measured - FFS: Details - FFS: Reservation signaling if shared resource pool is agreed to be introduced - FFS: Whether a higher layer signaling reservation message could also be considered |
| Huawei, HiSilicon | Generally OK. A small suggestion is to change “indicating a SL-PRS resource” to “indicating one or more SL-PRS resources”. The reservation of SL-PRS also could be used for indicating receving UE a SL-PRS resource to be transmited, especially for RTT based positioning, so the reserved SL-PRS transmission occasion is actually to be used for receiving UE to transmit SL-PRS in this case. |
| Lenovo | Supportive of FL’s proposal but also share the view that both Tx/Initiator UE and Responder/Rx UE(s) may receive the SL-PRS related configurations. So we could keep this aspect general at this stage. It is also beneficial to indicate if the SL-PRS resources to be reserved is for measurement or transmission purposes. |
| Ericsson | We need to understand what is specific to share resource pools that would require a separate agreement. Considering that the proposal is high level, we think we can include shared resource pools in the proposal. We are OK with the proposed rewordings by Huawei and vivo. |
| Sharp | Some more clarification on the purpose of the “reservation” may be helpful to understand the proposal. There may be the following possible purposes: - Inform the receiving UE of the SL-PRS resources for measurement; - Inform the other receiving Ues of the resources which are not valid for its reception (e.g., SL communication); - Prevent other transmitting UE to transmit SL-PRS on the same resources; |
| Spreadtrum | We are fine with vivo’s version. |
| Apple | Okay with Vivo’s update and Huawei’s modification. Ideally we should have a unified design for both dedicated and shared resources. However, with shared resources, the interaction with the current SCI/PSSCH needs to be discussed. |
| Futurewei | We are ok with the additional changes proposed by Vivo and HW, i.e. remove “to a receiving UE” and change to “indicating one or more SL-PRS resources” |
| Intel | We suggest a minor change in the main bullet to clarify that this is also for indication of SL-PRS. We are fine with Vivo/Huawei’s update in the sub-bullet. With regards to the reservation/indication of SL-PRS at least for the case of dedicated resource pool for positioning: |
| NEC | We share the similar views about SCI receiving object and fine with the vivo and Huawei’s version. |
| DCM | LGE/vivo’s version seems to be better. This signaling would be a kind of reservation. |
| Samsung | We are OK to consider SCI signaling but suggest the following modification including other option as With regards to the reservation of SL-PRS at least for the case of for dedicated resource pool and shared resource pool (if supported) for positioning: - Option 1: A transmitting UE may send SCI to a receiving UE for indicating a one or more SL-PRS resource(s) to be measured - FFS: Details - Option 2: A transmitting UE may receive SCI from a receiving UE for indicating a one or more SL-PRS resource(s) to be used for SL-PRS - FFS: Details - FFS: Reservation signaling if shared resource pool is agreed to be introduced - FFS: Whether a higher layer signaling reservation message could also be considered |
| OPPO | We did not see any need to differentiate dedicated resource pool and shared resource pool for this issue, and we share the view that the SCI could target multiple uEs. |
| InterDigital | We support the proposal and agree with view of Vivo regarding having a SCI to indicate the reserved SL-PRS resource for both measurement and sensing purpose. Thus, we agree with Vivo’s update on the proposal. |
| Xiaomi | We are fine with revisions from vivo and HW. |
| CEWiT | Agree with CATT and Huawei modifications as we may have more one receiver and SL-PRS resource can be more than one. |
| Qualcomm | We think the question of where this reservation information is transmitted should be resolved first since that places limitation on what can be done with SCI while maintaining backward compatibility. For example, cross resource pool reservations are not supported in sidelink and cannot be introduced in a backward compatible way using SCI. Higher-layer reservations can be done in the same or in different resource pools without introducing backward compatibility issues. Separately, the previous agreement mentioned lower-layer signaling, which included MAC-CE as well, and it is not clear why the proposal limits the options to SCI only. |
| Nokia, NSB | OK with Vivo’s revision. |

Some observations:

- Even though there was some support of a few companies with relatively benigh changes in the proposals, it appears to me that we may need to step back and consider a proposal that keeps the SL-MAC-CE or high-layer for downselection, and also keep the “message direction” more general (see Samsung’s reply).

[CLOSED] Feature Lead Proposal 4.1.4.B-v1

With regards to the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning:

  - Option A.1: SCI can be used for indicating one or more SL-PRS resource(s)

  - Option A.2: SL-MAC-CE can be used for indicating one or more SL-PRS resource(s)

  - Option A.3: Higher layer signaling can be used for indicating one or more SL-PRS resource(s)

For either of the above options, the reservation/indication signaling could correspond to

- Option B.1: A transmitting UE sending the reservation/indication, and then transmitting the SL-PRS resource(s) according to that.

- Option B.2: A transmitting UE receiving the reservation/indication, and then transmitting the SL-PRS resource(s) according to that.

Companies are encouraged to provide which Option from the A series they are supporting and which from the B series.

Companies views

| vivo | If the SCI can not be supported, we have some concerns about proposal 4.1.4.A-1 too since the two proposals are the evolution of the following proposal, and now the configuration only says high layer, and the proposal step back a high layer or low layer. In this case, we wonder about the difference between the previous agreement and prefer to discuss the two proposals together. Agreement With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. |  |
| --- | --- | --- |
| LGE | Regarding the first proposal, we’re not sure whether MAC-CE is higher layer or lower layer signaling. In that sense, we need the same level of details also for option A.3. What is the exacting signaling of the higher layer? Is it RRC or SLPP/RSP? Our preference is option A.1 For the second proposal, our understanding is that option B.1 is based on sensing and option B.2 on e.g. IUC. We think both options are necessary depending on the SL positioning method and associated SL PRS transmission. We may need some clarification on whether the proposal is for scheme 1 and/or scheme 2, or which UE/entity sends SL PRS resource reservation/indication to the transmitting UE. For either of the above options, the SL PRS resource reservation/indication signaling could correspond to - Option B.1: A transmitting UE sends the SL PRS resource reservation/indication, and then transmits the SL-PRS resource(s) according to the reserved/indicated SL PRS resource(s). - Option B.2: A transmitting UE receivs the SL PRS resource reservation/indication, and then transmits the SL-PRS resource(s) according to the reserved/indicated SL PRS resource(s). - FFS: from which UE/entity the transmitting UE receives the SL PRS resource reservation/indication |  |
| MTK | Our view is for the 3 options within series A, is it NOT to downselect for one option. Similarly for series B, it is also NOT to select one. Both options maybe needed. Further, for B.2, we want to further check, whether the TX UE may receive indication probably through higher layer, and also the TX UE needs to send indication In our view, the higher layer could also be used to coordinate between TX uEs and RX uEs for determining the SL-PRS configuration. Once determined, The reservation could be through SCI so that any other uEs could know after decoding |  |
| CATT | Both the three Option A.x and the two Option B.x should be not precluded at this stage, they are possible solutions for different scenarios. |  |
| ZTE | One clarification for the first part of this proposal: Does the proposal also not exclude the possibility of using multiple signalings (Option A.1, Option A.2, Option A.3) for SL-PRS resource reservation/indication? For the last part of this proposal, please see our comment/revision: For either of the above options, the reservation/indication signaling could correspond to - Option B.1: A transmitting UE sending the reservation/indication, and then transmitting the SL-PRS resource(s) according to that. - Option B.2: A transmitting UE receiving the reservation/indication, and then transmitting the SL-PRS resource(s) according to that. (Comment: this option is not very clear for us, does this means a transmitting UE do not send the reservation/indication?) |  |
| Qualcomm | We prefer higher layers sending the reservation (Option A.3) but are ok to keep all three options at this stage. Simiarly, we prefer Option B.1 (with modification below) but are ok to keep both options for now to study further. Does Options B.1 preclude the reservation being transmitted simultaneously with SL-PRS? This is problematic particularly for SCI-based reservation (A.1) as well as for shared resource pools. To not preclude this operation as this stage, we propose the following: - Option B.1: A transmitting UE sending the reservation/indication, and then transmitting the SL-PRS resource(s) according to that. |  |
| InterDigital | We support Option A.1 and we are okay with Option A.2 for making progress. We prefer to remove Option A.3, as this proposal pertains only to the lower layer signaling described in Option 2 in the agreement listed by Vivo. We support both Option B.1 and the principle of Option B.2 presuming Option B.2 is intended to for IUC-based SL-PRS resource determination. However, we would like to clarify first if “reserved/indicated SL PRS resource(s)” covers both Scheme 1 and Scheme 2. For Scheme 2, a transmitting UE will perform Option B.1 first and receive a conflict indication which prompts a re-selection. Also, in IUC framework, the resources provided by UE-A to UE-B (e.g., preferred resources) are NOT reserved by UE-A. If Option B.2 suggests a transmitting UE performs SL-PRS transmission in resource(s) reserved by another UE, then it is beyond the IUC baseline and we may need to discuss it sepeartely. |  |
| Intel | We feel that this is one step back on the updated proposal compared to the original FL proposal. We support Option A.1. |  |
| CMCC | We prefer Option A.1. In NR sidelink, only SCI can indicate and reserve resource for transmission, according to the WID, this should be a baseline for SL-PRS transmission. Between Option B.1 and Option B.2, both of them can be supported. Option B.1 is applicable for normal case for SL-PRS transmission, Option B.2 maybe applicable for some special case, for example, a UE select resource for the peer UE in when RTT type positioning method is used, or a group header UE select a set of resources for the group member uEs. So we think both of them can be supported and no down-selection is needed. |  |
| Huawei, HiSilicon | Just would like to confirm the understanding here, the reservation/indication is between uEs according to the description Option A.1 and A.2, but Option A.3 does not explicitly mention the higher layer is PC5 higher layer signaling which we assume so. Anyway, considering the broadcast nature of reservation, at least to our understanding, the reservation should adopt Option A.1. Another question for clarification on Option B.2 is whether this is considered as inter-UE coordination. If the answer is ‘Yes’, we think both option B.1 and option B.2 are needed, not need to select one of them. For option B.1, it is described from the reservating/indicating UE; while for option B.2, it is described from the reserved/indicated UE. At least in SL-RTT, either reserving/indicating UE or reserved/indicated UE can be the transmitting UE. |  |
| OPPO | At this stage we are fine to keep option A.1~3 for further study, but option A.3 should be revised to differentiate from option A.2, i.e. to “Higher layer signaling excluding SL-MAC-CE can be used for indicating one or more SL-PRS resource(s)”. Furthermore, we believe the reservation/indication signaling could correspond to both Option B.1 and Option B.2. |  |
| Samsung | We prefer option A.1 Also, we prefer to study both option B.1 and B.2. However, the wording of option B.1/2 looks can be refined. - Option B.1: A transmitting UE sending the reservation/indication, and then transmitting on the SL-PRS resource(s) according to that. - Option B.2: A transmitting UE receiving the reservation/indication, and then transmitting on the SL-PRS resource(s) according to that. |  |
| DCM | We are not sure higher layer signaling can be used for reservation. Probably, before determination of which layer, what behavior is intended should be clarified; e.g., like legacy reservation/indication or high-layer configuration + PHY layer indication or etc. We are OK to discuss B1 and B2. |  |
| Lenovo | We are supportive of FL’s proposal as it reads. In addition, we think that both B.1 and B.2 can be deemed feasible from a positioning point of view. |  |
| Ericsson | For A series, we prefer Option A.1 For B series, we have a preference for B.1. We agree with Qualcomm that ‘then’ should be removed from B.1. |  |
| Apple | We prefer Option A.1 We are open to discussing both B.1 and B.2 |
| Xiaomi | We prefer to option A.1 and think both option B.1 and B.2 can be supported. |
| Nokia, NSB | At this point we prefer to keep the options open. E.g. if SL PRS is transmitted in a shared resource pool with legacy UEs present then it is not clear how SCI can be used to indicate the SL-PRS resources. |

Observations: There is strong majority for Option A.1:

  - Option A.1: SCI can be used for indicating one or more SL-PRS resource(s)

    - Apple, Xiaomi, Ericsson, Samsung, Huawei, HiSilicon, CMCC, Intel, Interdigital (1st preference), MTK

  - Option A.2: SL-MAC-CE can be used for indicating one or more SL-PRS resource(s)

    - Interdigital (2nd preference)

  - Option A.3: Higher layer signaling excluding SL-MAC-CE can be used for indicating one or more SL-PRS resource(s), without any lower-layer signaling

    - Qualcomm

There was also a request for further clarification that in Option A.1, there can still be higher layer. Yes this is my understanding.

There was another request to try to clarify a bit more the Options in Series B. I am making them a “study” for now together with the suggested changes to clarify further the procedure. There was also a suggestion to add an “FFS”. Sinc ehte whole proposal is a “study” there can be a lot of different FFS one could add and i prefer not to go down this path.

Based on the above, my suggeston is to try the following in this round:

##### [HIGH] Feature Lead Proposal 4.1.4.B-v2

With regards to the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning:

  - Option A.1: SCI can be used for indicating one or more SL-PRS resource(s)

    - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS [configuration/parameters].

Study further the  following options with regards to the SL PRS resource reservation/indication signaling:

- Option B.1: A transmitting UE sends the SL PRS resource reservation/indication, and then transmits the SL-PRS according to the reserved/indicated SL PRS resource(s)..

- Option B.2: A transmitting UE receives the SL PRS resource reservation/indication, and then transmits the SL-PRS according to the reserved/indicated SL PRS resource(s).

- Note: Up to further discussion whether one or both shall be introduced.

##### Companies views

| LGE | For option A.1, in scheme 1, DCI is also used for network to indicate the resources for UE to use, it would be more complete to add DCI. For option B.2, it’s unclear from what entity UE receives such information. In scheme 1, it should be network as it schedules UE’s resources. In scheme 2, it could be UE to assist the transmitting UE’s SL PRS transmission. We’re fine to explicitly mention the aboves, but if FL feels it’s too early, then we suggest FFS for the entity from which UE receives such information. ##### [HIGH] Feature Lead Proposal 4.1.4.B-v2 With regards to the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: DCI and/or SCI can be used for indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS [configuration/parameters]. Study further the following options with regards to the SL PRS resource reservation/indication signaling: - Option B.1: A transmitting UE sends the SL PRS resource reservation/indication, and then transmits the SL-PRS according to the reserved/indicated SL PRS resource(s).. - Option B.2: A transmitting UE receives the SL PRS resource reservation/indication, and then transmits the SL-PRS according to the reserved/indicated SL PRS resource(s). - FFS: how the transmitting UE receives the SL PRS resource reservation/indication in scheme 1 and scheme 2. - Note: Up to further discussion whether one or both shall be introduced. |
| --- | --- |
| MTK | Generally okay. It seems that, for example at least for SL-RTT, the target UE could reserve resource for anchor UE for the transmission by anchor UE. But we feel that, the coordination before reservation maybe needed. |
| ZTE | For Option B.2, we have two interpretations: 1. A transmitting UE receives the SL PRS resource reservation/indication for its transmission, and transmits the SL-PRS according to the reserved/indicated SL PRS resource(s). 2. A transmitting UE receives the other UE’s SL PRS resource reservation/indication, and transmits the SL-PRS according to the reserved/indicated SL PRS resource(s). |
| Intel | We can accept the FL’s proposal. As pointed out by other companies option B.2 has many potential interpretations and it might be beneficial to list option to encourage companies to study these. |
| FL Update | TO LGE: This proposal is about the signaling between UEs, not from network to a UE (which is a separate proposal). I can just add the “on SL” in first sentence. ##### [HIGH] Feature Lead Proposal 4.1.4.B-v3 With regards to the reservation/indication on SL of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS [configuration/parameters]. Study further the following options with regards to the SL PRS resource reservation/indication signaling: - Option B.1: A transmitting UE sends the SL PRS resource reservation/indication, and then transmits the SL-PRS according to the reserved/indicated SL PRS resource(s).. - Option B.2: A transmitting UE receives the SL PRS resource reservation/indication, and then transmits the SL-PRS according to the reserved/indicated SL PRS resource(s). - Note: Up to further discussion whether one or both shall be introduced. |
| InterDigital | For Option B.2, as we indicated in the last round discussion, it involves a mechanism not supported by IUC schems, which is that a UE reserves resource(s) for transmission of another UE. Thus, we agree with FFS suggested by LGE to further study. |
| Qualcomm | The interpretation that Option B.2 in inter-UE coordination contradicts Option A.1 since inter-UE coordination messages are sent as MAC-CE (and optionally, up to Rx UE capability and up to Tx UE implementation, as SCI-2). Hence, we do not think that both can be kept simultaneously in the proposal. Option A.1 needs clarification whether this is SCI-1 or SCI-2. Our understanding form the companies comments about existing SL reservation signaling is that the intention is SCI-1. This should be made clear. On the other hand, if the interpretation is SCI-2, then it is not clear to us how this work, both in dedicate and shared resource pools. Lastly, the note under Option A.1 seems redundant since there are already separate proposals for configuration. We prefer to remove it but can accept it for progress. We propose the following changes below. ##### [HIGH] Feature Lead Proposal 4.1.4.B-v3 With regards to the reservation/indication on SL of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: 1st SCI can be used for indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only 1st SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS [configuration/parameters]. Study further the following options with regards to the SL PRS resource reservation/indication signaling: - Option B.1: A transmitting UE sends the SL PRS resource reservation/indication, and then transmits the SL-PRS according to the reserved/indicated SL PRS resource(s).. - Option B.2: A transmitting UE receives the SL PRS resource reservation/indication, and then transmits the SL-PRS according to the reserved/indicated SL PRS resource(s). - Note: Up to further discussion whether one or both shall be introduced. |
| CATT | The updated FL proposal is fine for us. In fact, all the three Option A.x should be kept, but for the sake of progress, we can live with the updated FL proposal which Option A.1 was kept with the note to say other options are not be precluded. |
| CEWiT | We do not support this proposal. For First part, we are not clear with the intension of only listing A.1 from previous proposal. Is intention to agree on SCI based reservation/ indication in this meeting and further study other options? Then other options should be listed under further study. Note is not clarifying this. SCI based solution will be suitable for ranging but for absolute and relative positioning UE should have coordination and only SCI solution will not be sufficient. So we are of the opinion that all options from last or at least 2 options from last round of proposal should be supported. For second part, as pointed by Intel all interpretation should be listed or should have scope to add new options in next meeting. |
| OPPO | Support the updated FL proposal. As the proposal is about signaling between UEs, Option B.2 is in fact “A transmitting UE receives the SL PRS resource reservation/indication from another UE”, it is not any mechanism used in exting SL communication, however, in SL positioning some inter-operations may be involved between multiple anchor UEs or between anchor UE and target UE, which is new on SL, hence we are open to further study this option. |
| vivo | We are okay with the FL proposal, and maybe the first part can be agreed upon first if companies have concerns about the second part. In addition, for the type of SCI, we prefer to further discuss. |
| Samsung | Proposal OK in general. we would like to add FFS: Whether SCI is single stage SCI or two stage SCI. |
| Apple | Fine with the proposal. Okay with Samsung’s addition. |
| LGE | We’re fine with the updated FL proposal with the intention of SL only. We prefer the 2nd SCI instead of the 1st SCI in order to minimize the change of the 1st SCI for backward compatibility. The only field that needs to be changed in the 1st SCI may be the indication of the 2nd SCI. All the necessary information for SL PRS control information can be carried by the 2nd SCI. However, we’re open for further discussion, and OK with Samsung’s suggestion of FFS: Whether SCI is single stage SCI or two stage SCI. |

#### SL-PRS Time domain Behavior

The following proposals were found in the contributions:

| Nokia, NSB | Proposal 13: To enable backward compatibility in terms of slot design, SL PRS can be transmitted via one or more of the following: - In PSSCH, e.g., if an accompanying positioning meta-data will be sent in the same slot - In PSFCH, using remaining resources from legacy SL transmissions, depending on the size of SL PRS - In a new “mini-slot” occupying the last symbols of a legacy slot, which contains dedicated symbol(s) for SL PRS, accompanied with AGC and guard symbols. Proposal 14: To enable backward compatibility in terms of slot design, control information associated with SL PRS can be transmitted via one or more of the following: - Using SCI (PSCCH and/or PSSCH), e.g., in case SL PRS will be transmitted in associated PSSCH together with positioning meta-data in the same slot - In PSFCH, using remaining resources from legacy SL transmissions, depending on the size of the control information - In a new “mini-slot” occupying the last symbols of a legacy slot, which contains dedicated symbol(s) for control info. Associated with SL PRS, accompanied with AGC and guard symbols. |
| --- | --- |
| Spreadtrum | Proposal 2: On-demand SL-PRS transmission at least should be supported. |
| Vivo | - Periodic and aperiodic SL-PRS should be supported for SL-PRS. |
| CATT | Proposal 23: Periodic, semi-persistent and aperiodic SL-PRS should be supported in Rel-18. Proposal 24: The semi-persistent SL-PRS and aperiodic SL-PRS transmission procedure should be triggered explicitly by the related signaling procedure. |
| Intel | - For a shared resource pool, periodic, aperiodic, and semi-persistent transmissions of SL-PRS are supported for both resource allocation schemes 1 and 2. |
| ZTE | Proposal 14: RAN1 should first study whether to support periodic SL-PRS, semi-persistent SL-PRS or aperiodic SL-PRS. |
| CMCC | Proposal 4: For the study of time domain behaviour, on-demand SL-PRS has higher priority than always-on SL-PRS - All of the three types of time domain behavior can be considered in Rel-18 for SL positioning RS, including periodic, semi-persistent, and aperiodic. |
| Lenovo | Proposal 17: RAN1 to consider at least the following configurability options for SL PRS: - Support of periodic/semi-persistent/aperiodic transmission of SL PRS - Unambiguous identification of initiator Ues and responder Ues, e.g., defining new IDs or re-using existing IDs |
| LG | Proposal 4: At least the following types of SL PRS transmission are supported for SL positioning. - Periodic transmission - Semi-persistent transmission - Dynamic transmission - On-demand transmission |
| Apple | Proposal 3: RAN1 to consider sending SL-PRS to support on-demand operation. |

I think it may be useful to be more descriptive with regards to the periodic/semi-persistent/aperiodic; mainly due to the fact that we have diverse group of people contributing in thie subagenda. For some people), P/SP/AP is about RRC-based configuration vs MAC-CE vs DCI-based signaling. However, my understanding is that the better way to discuss this topic is not with what signaling is being used, but what is the procedure. For periodic SL-PRS, getting that configuration is enough for the transmitting UE to initiate the transmissions. For Semi-persistent SL-PRS, an activation message maybe needed (this could be higher-layer or lower layer based on further agreements/discussions) but the UE will keep on transmitting until a deactivation message is received. In either Periodic or semi-persistent, i think it is common ground that there is going be a periodicity. For Aperiodic SL-PRS, a triggering message is needed (which again could be higher-layer or lower layer based on further agreements/discussions) for one or more SL-PRS instance to be transmitted, but no deactivation message is needed. A first thing we could discuss is that both one-shot and a periodic SL-PRS could be useful to be considered. How a SL-PRS with a periodicity configuration is activated/deactivated can be part of further discussion and also in relation to Scheme 1 and Scheme 2.

If the above make sense, my suggestion is the following proposal for now:

[CLOSED] Feature Lead Proposal 4.1.6-v0

With regards to the SL-PRS time domain behavior, at least consider the following two behaviors:

- Periodic SL-PRS: SL-PRS is transmitted periodically with a transmission periodicity

  - FFS: Details, Activation/Deactivation

- Aperiodic SL-PRS: SL-PRS is expected to be transmitted at least once after a triggering message is received.

  - FFS: Details

- FFS: Applicability of the above time behaviors for scheme 1 & scheme 2

Companies views

| CMCC | We are generally fine. To provide some clarifications, we think the “Activation/Deactivation” does not exist in Sidelink, especially when the Activation/Deactivation signalling is conveyed by MAC CE; Semi-persistent SL-PRS can be achieved by Tx UE’s stopping the transmission for next periodicity, and do not reserve the corresponding resource by indicating “0” in the periodicity reservation field. |
| --- | --- |
| LGE | Our understanding of FL proposal is as follows. The 1st bullet includes both always-on and configurable periodic transmission of SL PRS. The 2nd bullet includes both aperiodic/dynamic and on-demand transmission of SL PRS. If our understanding is correct, we support FL proposal with the following modification. The term ‘message’ is usually used for higher layer signaling. As we didn’t agree anything on the specific realization of the triggering, we suggest to make it open by removing the term ‘message’ ,and add triggering details to FFS. ##### [MEDIUM] Feature Lead Proposal 4.1.6-v0’ With regards to the SL-PRS time domain behavior, at least consider the following two behaviors: - Periodic SL-PRS: SL-PRS is transmitted periodically with a transmission periodicity - FFS: Details, Activation/Deactivation - Aperiodic SL-PRS: SL-PRS is expected to be transmitted at least once after a triggering message is received. - FFS: Details, triggering - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 |
| Huawei, HiSilicon | We expect that this proposal is described from Tx UE perpective. For aperiodic SL-PRS, from Tx UE perspective, the triggering could be from an internal source within the UE rather from a triggering message. So we suggest the following revision. With regards to the SL-PRS time domain behavior, at least consider the following two behaviors from Tx UE perspective: - Periodic SL-PRS: SL-PRS is transmitted periodically with a transmission periodicity - FFS: Details, Activation/Deactivation - Aperiodic SL-PRS: SL-PRS is transmitted without strictly following a transmission periodicity. - FFS: Details - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. In addition, the proposal saying ‘consider the two behaviors’ intended to say to be considered to support SL positioning or to be considered for the continuing study?? |
| Apple | To clarify, the 1st bullet covers periodic and SPS (with SPS in the FFS with activation/de-activation?). Given the use cases and possible power saving issues, we do not see why SPS should not be considered up front. |
| Futurewei | We also agree that should be from the Tx UE perspective, therefore to remove “is received”, OK with either LGE or HW changes |
| Intel | It seems that if we discuss activation/deactivation, it is for SPS based SL-PRS transmission. It would be good to clarify this. |
| CATT | For the periodic SL-PRS, the candidate values of periodicity of SL-PRS should be consider. We prefer the following revision for periodic SL-PRS case: - Periodic SL-PRS: SL-PRS is transmitted periodically with a transmission periodicity - FFS: Details, Activation/Deactivation, the candidate values of periodicity |
| DCM | For periodic, we do not hink any activation/deactivation signaling is necessary. If such a signaling is introduced, it is rather semi-persistent. For aperiodic, trigger may come from higher layer, e.g., a target UE sends SL-PRS and requests an anchor UE to report the measurement result. LGE’s update is better. |
| Samsung | We would also like to consider semi-persitent also. Thus, we propse With regards to the SL-PRS time domain behavior, at least consider the following two behaviors: - Periodic SL-PRS: SL-PRS is transmitted periodically with a transmission periodicity - FFS: Configuration - Semi-persistent SL-PRS: SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - FFS: Details, Activation/Deactivation - Aperiodic SL-PRS: SL-PRS is expected to be transmitted at least once after a triggering message is received. - FFS: Details - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 |
| Sharp | Generally OK with the FL’s proposal and either modification by LGE or Huawei is fine. |
| OPPO | We share the view that periodic/aperiodic is from transmitter perspective. |
| InterDigital | We support the proposal with one comment regarding the triggering condition for aperiodic SL-PRS transmission. The current proposal indicate it is triggered “after a triggering message is received”. Our interpretation is this “triggering message” can be received from higher layers or included in a request transmission from another UE. The triggering conditions are still for further study, so we suggest to update the bullet as below. - Aperiodic SL-PRS: SL-PRS is expected to be transmitted at least once after a triggering message is received such SL-PRS transmission is triggered. FFS: Details including at least triggering condition(s) for aperiodic SL-PRSs |
| Xiaomi | We also think aperiodic SL-PRS transmission can be triggered by internal source or conditionally. So we prefer HW’s revision. |
| Qualcomm | We support LG’s modifications to include internal triggering as well. |
| Philips | Generally fine with the proposal. |
| Nokia, NSB | OK with LGE’s revision for aperiodic SL-PRS,. |

[CLOSED] Feature Lead Proposal 4.1.6-v1

With regards to the SL-PRS time domain behavior, at least consider the following two behaviors from Tx UE perspective:

- Periodic SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity

- Semi-persistent SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation

- Aperiodic SL-PRS

  - SL-PRS is expected to be transmitted at least once after triggering message is received. [without strictly following a transmission periodicity]

- FFS: Applicability of the above time behaviors for scheme 1 & scheme 2

- FFS: Rx UE behavior is separately discussed.

Companies views

| vivo | We also support including the internal triggering for aperiodic SL-PRS without signaling and wonder whether a note is needed to explain it. |  |
| --- | --- | --- |
| LGE | We’re fine with the FL proposal. Maybe we need FFS for the details of each operation. ##### [HIGH] Feature Lead Proposal 4.1.6-v1 With regards to the SL-PRS time domain behavior, at least consider the following two behaviors from Tx UE perspective: - Periodic SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity - Semi-persistent SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - Aperiodic SL-PRS - SL-PRS is expected to be transmitted at least once after triggering message is received. [without strictly following a transmission periodicity] - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. - FFS: details on the configuration of periodicity, activation/deactivation, and triggering condition/event |  |
| MTK | Okay for this version and also fine for LGE’s |  |
| CATT | We are fine with the proposal. |  |
| Fraunhofer | Support |  |
| ZTE | For aperiodic SL-PRS, we suggest to delet the sentence in bracket. After triggering, a periodic PRS transmission is also possible. |  |
| Qualcomm | Higher-layer (application or protocol) control is needed over any sidelink transmission since there is no centralized system control. Hence, the UE cannot start transmitting upon receiving (pre-)configuration, but also needs a trigger from higher layers. This makes periodic and semi-persistent equivalent on sidelink. For example, regular SL-PRS transmissions might need to temporarily disabled to allow for emergency vehicle transmissions. Our first preference is to put periodic and semi-persistent under a single label as in the previous version of the proposal and in SL communications. If that is not agreeable to the group, we can accept the current structure with the following updates. With regards to the SL-PRS time domain behavior, at least consider the following two behaviors from Tx UE perspective: - Periodic SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity - Transmission of SL-PRS can be enabled/disabled by the higher layer. FFS details. - Semi-persistent SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - Aperiodic SL-PRS - SL-PRS is expected to be transmitted at least once after triggering message is received. [without strictly following a transmission periodicity] - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. - FFS: whether periodic and semi-persistent SL-PRS are captured separately or using the same behavior. |  |
| InterDigital | We support the proposal. |  |
| Intel | We are fine with the updated proposal. We think the [without strictly following a transmission periodicity] can be removed. |  |
| CMCC | First, we would like to clarify the definition of periodic and aperiodic transmission in NR sidelink. In NR sidelink, if a high layer notified parameter Prsvp_Tx=0, means the transmission is a aperiodic transmission, the corresponding field in SCI should indicate no periodic resource is reserved; If the high layer notified parameter Prsvp_Tx≠0, means the transmission is a periodic transmission, the corresponding field in SCI should indicate there is further periodic resource is reserved corresponding to the periodicity value; Even though the transmission may be stopped or the periodic resources may be changed, this is still called as a “periodic transmission”. Thus, in NR sidelink, no Semi-persistent transmission with explicit activation/deactivation signalling is supported. Since NR sidelink should be baseline for SL-PRS, we think the bullet for Semi-persistent transmission should be removed. |  |
| Locaila | We are OK with the proposal. |  |
| Huawei, HiSilicon | Question for clarification on activation and triggering. Does it mean that activation/deactivation for SP SL-PRS is like DL MAC CE activation/deactivation or SPS activation/deactivation from network, while the triggering is like DCI-based triggering from network or triggering internally within the UE? |  |
| NEC | We support the proposal. |  |
| OPPO | Support. |  |
| Samsung | OK. We prefer to delete [without strictly following a transmission periodicity]. |  |
| Spreadtrum | Support the proposal. |  |
| DCM | Although we are not sure semi-persistent is necessary, we can live with it for progress. |  |
| Lenovo | Generally supportive of the proposal. However, the triggering aspect is not clear from the propoal, i.e. is it from the higher layer or lower layer. Therefore, we prefer to add more clarity to the triggering aspect. |  |
| Ericsson | Do we need to support both periodic SL-PRS and semi-persistient SL-PRS? Also, isn’t periodic SL-PRS always on? In this case, periodic SL-PRS may not be preferrable. |  |
| Apple | We are fine with this |
| Nokia, NSB | OK. We agree with Qualcomm’s point that higher layer control over periodic SL-PRS is needed. |

Observations: There is still a debate about whether periodic and semi-persistent are needed for SL. My suggestion is that it should be part contribution drive in the next meetings to understand what is needed. It seems there support to write down all three at this stage for the purpose of the study.

##### [MEDIUM] Feature Lead Proposal 4.1.6-v2

With regards to the SL-PRS time domain behavior, at least consider study the following two behaviors from Tx UE perspective:

- Periodic SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity

  - Transmission of SL-PRS can be enabled/disabled by the higher layer. FFS details.

- Semi-persistent SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation

- Aperiodic SL-PRS

  - SL-PRS is expected to be transmitted at least once after triggering message is received. [without strictly following a transmission periodicity]

    - Note: This “triggering” may include UE’s internal triggering without signaling

- FFS: Applicability of the above time behaviors for scheme 1 & scheme 2

- FFS: Rx UE behavior is separately discussed.

- FFS: whether periodic and semi-persistent SL-PRS are captured separately or using the same behavior.

- FFS: What signaling is used for activation/deactivation/triggering is part of the study

##### Companies views

| LGE | We support FL proposal with one comment. In the sub-bullet of the aperiodic SL PRS, triggering should include the request from other UE. In SL RTT for example, one UE’s SL PRS transmission can trigger or request the subsequent SL PRS transmission from the other UE. We suggest to clarify this point by adding ‘request’ in addition to ‘triggering’. - Aperiodic SL-PRS - SL-PRS is expected to be transmitted at least once after triggering or request message is received. [without strictly following a transmission periodicity] - Note: This “triggering” may include UE’s internal triggering without signaling |
| --- | --- |
| MTK | We agree with LGE. We also wonder that it seems FL has the tendency to ignore “request” based on some earlier discussion? |
| ZTE | Generally ok with LGE’s revision. |
| Intel | We support this proposal. The difference between request and triggered is not clear to us. We expect that a request is followed by a trigger inside the tx UE itself. |
| FL Update | Please assume the word “request” is added in the aperiodic bullet as LGE suggests. To MTK: It was not pointed that there is a need to add “request” in this proposal in the previous round. Not from LGE, not from MTK. |
| Qualcomm | We do not think that RAN1 should define a request mechanism for SL-PRS. This is related to positioning sessions and in the domain of RAN2. Nor should RAN1 define details of triggering. Instead from the PHY layer’s point of view, there is a trigger to transmit SL-PRS but the reason for trigger (internal, request, …) is not needed. We propose to keep the original wording and not introduce “request”. |
| CATT | OK to add the word “request” into the aperiodic bullet. But for the following FFS, periodic and semi-persistent SL-PRS should be the different resource-type of SL-PRS, just as the definination in Rel-16 positioning, we prefer to remove the following FFS. - FFS: whether periodic and semi-persistent SL-PRS are captured separately or using the same behavior. |
| CEWiT | With FLs update based on LGE’s comment we are fine with the proposal. |
| OPPO | We tend to agree that from PHY point of view all SL PRS transmission is triggered by higher layer, but this does not mean that RAN1 cannot discuss triggering/request mechanism, one precedenct is IUC information transmission triggered by explicit request. In the sense, we support the updated proposal from FL. |
| Fraunhofer | Okay with the updated version |
| vivo | In our view, maybe only configuration signaling can be involved for the periodic SL-PRS. Based on the following sub-bullet, it is more like additional signaling for enabling/disabling SL-PRS is needed. In this case, more clarification is needed for us - Transmission of SL-PRS can be enabled/disabled by the higher layer. FFS details. |
| Samsung | Okay in general. BTW, for the update, actually, we are not sure the big difference between triggering and request. |
| Apple | OK |
| Spreadtrum | Support |

#### SL-PRS cast type

The following proposals were found in the contributions:

| vivo | - Unicast, groupcast and broadcast should be considered for SL positioning in Rel-18. |
| --- | --- |
| CATT | Proposal 32: Groupcast and broadcast SL-PRS should be considered for SL positioning. - FFS: Unicast SL-PRS |
| LGE | Proposal 5: The following cast types of SL PRS transmission are supported for SL positioning. - Unicast - Groupcast - Broadcast |
| Interdigital | Proposal 3: Support at least unicast for SL-PRS transmission and related signalling. |
| Apple | Proposal 15: The SL PRS/SRS may be unicast, broadcast or multi-cast. For broadcast and multicast SL PRS/SRS, RAN1 should study the following two options: • Option 1: Grouping is the same as PSSCH i.e. the PSSCH and the PRS are bundled together • Option 2: Grouping is independent of the PSSCH |
| Denso | Proposal 2: RAN1 to consider sending SL-PRS to support broadcast/multicast. |

A few companies had a proposal on the SL-PRS cast type. It should be noted that also RAN2 had an agreement that “RAN2 will study the question of cast type for positioning signalling, but for SL-PRS, they will follow RAN1 decision”.

It is reasonable to consider that SL-PRS can be a unicast reference signal for the purpose of one-to-one ranging and groupcast/broadcast for one-to-many SL-PRS reception (e.g. single RSU transmitting single SL-PRS to multiple vehicles). So, i make the following proposal for discussion:

[CLOSED] Feature Lead Proposal 4.1.7-v0

The following cast types of SL-PRS transmission should be introduced for SL positioning:

- Unicast, Groupcast, Broadcast

Companies views

| ZTE | OK |
| --- | --- |
| CMCC | We can support at least unicast at this stage, and further study the applicable scenario for Groupcast, and Broadcast. |
| LGE | Support FL proposal. |
| vivo | Support |
| Huawei, HiSilicon | We have some concern over supporting groupcast and broadcast in this release, especially for the aperiodic SL-PRS discussed in 4.1.6-v0. |
| Spreadtrum | Support |
| Apple | Support |
| Futurewei | OK |
| Intel | Agree with the FL’s proposal. |
| CATT | OK |
| NEC | Support |
| DCM | OK |
| Samsung | We suggeset the following modification as The following cast types of SL-PRS transmission should be introduced can be studied for SL positioning: - Unicast, Groupcast, Broadcast - FFS: Applicability of the above case types |
| Sharp | Support |
| OPPO | OK |
| InterDigital | We agree with the proposal. |
| Xiaomi | OK |
| CEWiT | Fine with proposal |
| Qualcomm | We support the proposal. Some of the methods being discussed and used in evaluations, e.g. multi-RTT, SL-TDoA, and SL-AoA could become very inefficient and consume too many system resources if limited to unicast. |
| Philips | OK |
| Nokia, NSB | OK |
| Lenovo | Support |

[ONLINE] Feature Lead Proposal 4.1.7-v1

The following cast types of SL-PRS transmission should be introduced for SL positioning:

- Opt. 1: Unicast, Groupcast, Broadcast

  - ZTE, LGE, vivo, Spreadtrum, Apple, Futurewei, Intel, CATT, NEC, DCM, Sharp, OPPO, Interdigital, Xiaomi, CEWiT, Qualcomm, Philips, Nokia, NSB

- Opt. 2: At least unicast. FFS: Groupcast, Broadcast

  - CMCC, Huawei, HiSilicon

- Opt. 3: All three cast types (Unicast, Groupcast, Broadcast) can be studied

  - Samsung

#### Additional aspects with regards SL-PRS

The following proposals were found in the contributions:

| CATT | Proposal 12: If the RB configurations of PSCCH and SL-PRS are different when PSCCH and SL-PRS are TDMed, one symbol should be used for gap right after PSCCH transmission. Proposal 11: The slot structrue of multiple SL-PRS resources for several UEs in one slot should be considered. |
| --- | --- |
| Intel | - Specification support of up to 100 MHz and 400 MHz SL-PRS transmission bandwidths for FR1 and FR2 should be considered for SL positioning, respectively. - RAN1 should also study the performance for SL positioning in limited bandwidth scenarios, e.g., up to 20 MHz. |
| Sony | Proposal 10: Consider supporting multiplexing of SL-PRS from multiple UEs within a given set of resource |
| NEC | Proposal 2: Consider jointly utilizing the resources on multiple shared resource pools for SL-PRS transmission. |
| ZTE | Proposal 12: In Rel-18, only TDM between SL-PRS transmission and SL-data transmission is considered. Proposal 22: FDMed PRS multplexing between UEs having positioning serving interaction should NOT be supported. |
| CMCC | Proposal 10: Mapping relationship b/w PSCCH and SL-PRS resources may need to be re-designed for NR SL positioning. - PSCCH-PSSCH mapping relationship can be a reference; - RE-level multiplexing for SL-PRS should be considered when designing the mapping relationship. |
| LGE | Proposal 7: It needs to be studied whether the multiple SL PRS configurations can be allowed in a SL PRS resource pool. Proposal 15: If RX UE fails to receive SL PRS or the quality of the received SL PRS does not meet the requirement, it is supported that RX UE sends a feedback to TX UE so that TX UE retransmits the SL PRS. Proposal 19: A single SL PRS configuration is allowed at least within a slot. - FFS whether a single SL PRS configuration is (pre)configured per resource pool. Proposal 20: The boundary of SL PRS resource within a slot are aligned across the UEs based on SL PRS configuration. Proposal 27: A minimum time gap for RF switching between different resource pools or BWPs needs to be considered in SL PRS resource allocation/selection. Proposal 33: When TX and RX SL PRS resources collide each other, a prioritization rule is necessary to drop either operation. Proposal 34: When SL PRS resource and UL transmission resource collide each other, a prioritization rule is necessary to drop either operation. |
| Spreadtrum | Proposal 15: Considering energy efficient of SL-PRS transmission, feedback mechanisms should be considered. |
| Interdigital | Proposal 30: Study slot-based SL-PRS transmission with a SL-PRS slot format including symbols for AGC, SL-PRS and associated SCI transmission in a slot. Proposal 31: Study symbol-level granularity of SL-PRS transmission multiplexing PSSCH/PSCCH transmission. |
| Fraunhofer | Proposal 5: For use-cases targeting a high accuracy, resource allocation strategies supporting an effective bandwidth beyond 40MHz for the reference signal shall be considered including the following options: - Option 1: Consider SL resource pools with higher bandwidth for positioning reference signals - Option 2: For bands supporting intra-band concurrent operation of Uu and PC5 (currently the band n79) support joint allocation of resources of UL-SRS and SL positioning reference signal - Option 3: Consider carrier aggregation of PC5 bands Proposal 6: Support the increase of the SL bandwidth for SL-PRS using a low EIRP. Proposal 8: Study carrier aggregation between PC5 carrier/BWPs at least for PC5 carrier using the same band. |
| Apple | Proposal 14: For SL positioning, the frequency resources allocated to the SL positioning reference signal should also be (pre-)configured based on sub-channels. - It can be set equal, larger or smaller than the shared channel frequency domain allocation size - It can be set independent of the shared channel frequency domain allocation size within its own dedicated slot |
| Samsung | Proposal 5: For SL PRS configuration, at least the following parameters are considered - SL PRS comb size, comb offset - SL PRS muting pattern - SL PRS symbol length - SL PRS repetition - SL PRS transmission periodicity - Starting PRB and subchannel for SL PRS - SL PRS max power - Latency bound for SL PRS transmission or measurement report - FFS: which parameters are supported - FFS: how the parameters can be decided for SL PRS transmission |
| Sharp | Proposal 2: - In a slot containing SL-PRS resources, each SL-PRS resource is mapped to a distinct PSCCH in the same slot. |
| Asustek | Proposal 3: Resource mapping between new PSCCH and SL PRS needs further design for dedicated resource pool for SL PRS. |
| Qualcomm | Proposal 8: There is no data/control transmission in time-frequency grid of SL PRS resources. Proposal 9: FDM multiplexing with other signals at RE level inside of SL PRS time-frequency grid is precluded. Proposal 10: Transmission of other SL signals and channels in the same OFDM symbol as SL PRS is precluded. |

FL Note: We will wait progress in the previous sections before proceeding with these proposals.

# SL Positioning Resource Allocation

Based on the submitted contributions, it may be beneficial to start the topic of SL Positioning Resource Allocation, by discussing it in two subtopics:

## Resource pool for SL Positioning

The following agreement was reached with regards to this topic:

Agreement With regards to the SL Positioning resource allocation, study further the following 2 options for SL Positioning resource (pre-)configuration: - Option 1: Dedicated resource pool for SL-PRS - Include in the study at least the following aspects: - which slots can be used, SL frame structure, SL positioning slot structure, multiplexing of SL-PRS with control information (if included in the same slot) - positioning measurement report - whether a dedicated frequency allocation (e.g., layer/BWP) is needed for SL PRS - resource allocation procedure(s) of SL-PRS - This option may or may not include control information (i.e., configuration/activation/deactivation/triggering of SL-PRS) for the purpose of SL positioning operation - Option 2: Shared resource pool with sidelink communication. - Include in the study at least the following aspects: - co-existence between SL communication and SL positioning, backward compatibility - Multiplexing considerations of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH) and any modifications in the SL-slot structure Agreement With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging: - Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS - Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed

The following proposals were found in the contributions:

| Nokia, NSB | Proposal 15: Support Alt. 2 (both dedicated and shared resource pool(s) with SL communication can be (pre-)configured for SL PRS transmissions). Proposal 16: Study transmission of SL PRS that occupy wide bandwidth over multiple SL resource pools, including mechanisms to enable resource pool aggregation. Proposal 17: Study multiplexing of SL PRS transmitted by different UEs in vicinity in code domain, including additional mechanisms to determine whether such resource re-use would be acceptable. Proposal 18: Study SL PRS transmission over segmented parts in time domain, including any necessary mechanisms for resource allocation. |
| --- | --- |
| Futurewei | Proposal 3: For SL-PRS transmission support Alt2 i.e., support both dedicated and shared resources-based solutions. |
| Huawei, HiSilicon | Proposal 12: Study mechanisms to support multiplexing of SL-PRS and sidelink communication at least in a Rel-18 shared resource pool. |
| Spreadtrum |  |
| vivo | - Dedicated resource pool for SL PRS can be supported first. |
| China Telecom | Proposal 6: With regards to the SL positioning resource allocation, we prefer a dedicated resource pool for SL-PRS for the consideration of the performance of localization. Proposal 7: The dedicated resource pool for SL-PRS should be associated to the BWP that is identical to the SL communication resource pool. The multiplexing manner between them is preferred to be TDM. |
| OPPO | - With regards to the SL Positioning resource allocation, either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS (i.e. Alt 2). |
| CATT | Proposal 13: For resource pool configured for SL-PRS, only dedicated resource pool(s) (Alt.1 in RAN1#110) should be supported, due to the issues of limitation, complexity and compatibility of the shared resource pool(s) (Alt.2 in RAN1#110). |
| Intel | - For SL positioning resource allocation, Alt. 2 is supported, i.e., either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-) configured for SL-PRS. |
| Sony | Proposal 5: Support either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-) configured for SL-PRS (known as Alt. 2). |
| NEC | Proposal 3: Either dedicated resource pool(s) and/or shared resource pool(s) with sidelink communication can be (pre)configured for SL-PRS, namely the Alt.2 regarding the resource pool(s) for SL positioning resource allocation should be supported. Proposal 4: If both dedicated resource pool(s) and shared resource pool(s) are supported, the association between two kinds of resource pools for SL positioning control information transmission(s) should be considered. |
| ZTE | Proposal 10: In Rel-18, prioritize dedicated SL-PRS resource pool associated with the BWP of a carrier that is the same as that of the SL communication resource pool |
| CMCC | Proposal 8: In Rel-18, only dedicated resource pool for SL-PRS (Alt 1) should be supported. - Only support TDM configuration b/w SL data and SL-PRS resource pools. |
| Lenovo | Proposal 19: Support Alt.2 of SL positioning resource allocation where dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. |
| LGE | Proposal 11: A dedicated resource pool is used for SL PRS transmission to provide a sufficient bandwidth for SL positioning accuracy requirement. Proposal 12: Other signals/channels associated with SL PRS (e.g. the control channel, the measurement report, etc.) are transmitted in a resource pool different from the one for SL PRS transmission. - Further study is needed whether the resource pool can be shared with the one for SL communication. |
| Interdigital | Proposal 27: Either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS (Alt. 2). |
| Apple | Proposal 10: With regards to the SL Positioning resource allocation, either dedicated resource pool(s) and/or shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. |
| Samsung | Proposal 9: Either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. |
| Sharp | Proposal 1: - A dedicated resource pool for SL-PRS should be “self-contained”, i.e. control signaling in a SL positioning procedure, e.g., signalling for SL-PRS resource reservation, or for scheduling SL-PRS transmission, or for triggering of SL-PRS measurement, should be transmitted in the same resource pool. |
| Asustek | Proposal 1: Adopt Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS. |
| NTT DOCOMO | Proposal 6: - For SL-PRS, either dedicated resource pool(s) or shared resource pool(s) with SL communication can be (pre-)configured. - The same structure is prioritized. |
| Qualcomm | Proposal 16: Support SL PRS-only resource pools where only SL-PRS transmissions can take place without other SL signals or channels. |
| CEWiT | Proposal 10: With regards to the SL Positioning resource allocation, option 1 dedicated resource pool for SL-PRS should be preferred. |
| Ericsson | - Regarding SL positioning resource allocation, consider Alt 2 where either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. |

With regards to the topic of dedicated-only vs dedicated and shared resource pool, we make the following observations from the contributions:

|  | Supporting companies |
| --- | --- |
| Alt. 1: Dedicated-Only resource pool | Vivo, China Telecom, CATT, ZTE, CMCC, LGE, Apple, Asustek, Qualcomm, CEWiT (10) |
| Alt. 2: Both dedicated and shared resource pool | Nokia, NSB, Futurewei, Huawei, HiSilicon (only for Rel-18 shared resource pool), Spreadtrum, OPPO, Intel, Sony, NEC, Lenovo, Interdigital, Samsung, NTT DOCOMO, Ericsson (13) |

It seems there is no strong majority either way. I would like to check if the companies supporting only dedicated resource pool would accept going with Alt. 2.

Companies are encouraged to clarify in their responses the following:

- when they mean “shared resource pool”, do they mean Rel-18 shared resource pool (i.e., no backward compatibility considerations), or it should also be applicable to previous releases SL communication resource pools.

[CLOSED] Feature Lead Proposal 5.1-v0

With regards to the SL Positioning resource allocation,

- either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS

Companies views

| ZTE | Ok. This proposal seems not making any significant progress because it includes two alternatives: - Alt. 1: either dedicated resource pool(s) or shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS - Alt.2: both dedicated resource pool(s) and shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS For share resource pool, we tend to think SL-PRS configuration information is specified in the IE “SL-ResourcePool ”(the same IE used for SL communication pool). |
| --- | --- |
| CMCC | Comments. Even a shared resource pool can be supported, the resource for SL data and SL-PRS should be TDMed distinguished by semi-static configuration, for example, SL-PRS can be configured in every N slots and occupying a set of symbols within the corresponding slot, like PSFCH configuration in NR sidelink. |
| LGE | From the companies’ proposal, it is clear that all the companies support the dedicated resource pool, but some companies have concern on the shared resource pool. In this case, I think the reasonable proposal should be such as follows. ##### [HIGH] Feature Lead Proposal 5.1-v0 With regards to the SL Positioning resource allocation, - At least a dedicate resource pool is supported - FFS whether a shared resource pool and/or a (pre-)configuration of either of those pools is additionally supported - either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS |
| vivo | For us, the dedicated resource pool is simpler and not need to consider complexity backward compatibility. It may need much time to discuss the backward compatibility with NR SL. We prefer to discuss the dedicated resource pool first. If there is timing remaining, the shared resource pool can be considered further. In addition, what is “(pre-)configured for SL-PRS” needs to be further clarified, it is more like pre-configuration PRS in Rel-17 for on-demand PRS or not? |
| Huawei, HiSilicon | OK |
| Ericsson | OK |
| Sharp | OK with the proposal. Also, fine with LGE’s modification. |
| Spreadtrum | Support |
| Apple | support |
| Futurewei | OK with proposal in principle. Agree with ZTE, maybe should have two clear alternatives and the proposal should be: “Downselect one of these two alternative...” at least will give a clear direction for the next step, which should be the downselect. |
| Intel | We are fine with the proposal. Regarding the question from FL, our understanding is that Rel-18 shared resource pool should ensure backward compatibility. |
| CATT | We prefer only dedicated RP should be supported in Rel-18. There are a lot of issues need to be solved for shared RP. Since all companies support dedicated RP, and 10 companies object to shared RP, we prefer to take the dedicated RP as the baseline, and leave the shared RP issue as the low priority in the discussion. We can live with LGE’s version above. |
| NEC | Support |
| DCM | Support |
| Samsung | Support |
| Sony | Support |
| OPPO | Support the proposal, in our view backward compatibility should be guaranteed in the shared reousrce pool, otherwise the benefit (i.e., resource efficiency) of shared resource pool would dilute. We are open to discuss how to reduce the complexity in the shared resource pool, e.g., a UE does not transmit both PSSCH and SL PRS in one slot. |
| InterDigital | We support the proposal. |
| Xiaomi | We are fine with the proposal. Regarding to the question on shared resource pool, we think Rel16/17 SL communication UEs can still perform SL communication in Rel-18 SL resource pool which supports SL-PRS. |
| CEWiT | Agee with CATT and can accept LGE’s proposal |
| Qualcomm | Given the previous discussion, the term shared pool needs to be further clarified. In our understanding, it is a pool with Rel-16 and Rel-17 sidelink communication UEs as well as Rel-18 sidelink communication and sidelink positioning UEs. Otherwise, the motivation for having a shared pool is not clear. That said, we share the concern about the workload introduced by a shared resource pool and prefer to focus on the dedicated resource pool. |
| Philips | OK but prefer dedicated resource pool. |
| Nokia, NSB | OK |

Note: I think at least 2 companies (ZTE, FW), might have confused that the current wording means that further downselection is possible.  The proposal picks Alt. 2 from the previous discussion, so there is no further downslection. The “and/or” means that the spec supports both and a UE could configured with either one or both. It was also clarified during the previous meeting.

If other companies were confused also, i plan to treat this online. We ll have to do a check on the supporting companies online.

[ONLINE] Feature Lead Proposal 5.1-v1

With regards to the SL Positioning resource allocation,

- Alt. 1: at least dedicated resource pool(s) can be (pre-)configured for SL-PRS. FFS whether a shared resource pool can be additionally supported

  - LGE, vivo, CATT, CEWIT, Qualcomm, Philips (but OK for Alt. 2) (5)

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS

  - Huawei, HiSilicon, Ericsson, Spreadtrum, Intel, NEC, DCM, Samsung, Sony, OPPO, Interdigital, Xiaomi, Nokia, NSB (12)

#### Dedicated Resource Pool for Positioning

The following agreement was reached with regards to this topic:

| Futurewei | Observation 4: For the dedicated SL-PRS resource pools, is beneficial if the control associated with SL-PRS transmissions is transmitted both in the shared (data) resource pools and SL-PRS dedicated pool resources. |
| --- | --- |
| Huawei, Hisilicon | - Proposal 9: Support PSCCH plus SL-PRS slot structure within the dedicated SL-PRS resource pool. - Proposal 10: Support to design the association between PSCCH candidate position and the scheduled SL-PRS resource for the dedicated resource pool. Proposal 11: Consider the reserved slots as a part of the dedicated SL-PRS resource pool. |
| Spreadtrum | Proposal 5: In dedicated resource pool for SL-PRS, one SL-PRS resource or one SL-PRS resource set can be used as the granularity of resource allocation for SL-PRS transmission. Proposal 6: The bandwidth of PRS can be configured at the resource pool level or can be fixed to the bandwidth of dedicated resource pool for SL-PRS. Proposal 7: The frequency domain range of SCI blind detection needs to be limited in dedicated resource pool for SL-PRS. |
| vivo | In NR SL, only one SL BWP is supported on a carrier. And resource pool is configured in the BWP. In our opinion, the SL PRS is transmitted in the BWP, and restricted in a resource pool. - The resource for SL PRS is restricted in a resource pool. - Control information(i.e., SCI) associated with SL-PRS should be included in the dedicated resource pool for SL-PRS. |
| OPPO | - No PSSCH is transmitted in the dedicated resource pool for SL PRS. |
| Intel | - For dedicated resource pool for SL-PRS, - SL-PRS resource pool and SL communication resource pools can be multiplexed in a TDM manner. - Support multiplexing of PSCCH and SL-PRS transmission in a TDM manner, i.e., PSCCH is used to schedule SL-PRS transmission in the resource pool. - For SL-PRS transmissions in a dedicated resource pool periodic, aperiodic, and semi-persistent transmissions containing SL-PRS are supported for both resource allocation schemes 1 and 2. |
| Somy | Proposal 6: Support a BWP carrying dedicated resource pool(s) for positioning (e.g., to carry wideband SL-PRS). |
| NEC | Proposal 5: Multiplexed SL-PRS resource on dedicated resource pool should be supported for parallel positioning procedures. Proposal 6: If a dedicated resource pool is supported for SL-PRS, only SL-PRS is expected to be transmitted and there is no data transmission but potentially with PSCCH carrying first stage SCI and PSSCH carrying second stage SCI, being used to configure/activate/deactivate/trigger SL-PRS. Proposal 9: The flexible SL positioning slot structure feasible to varied SL-PRS should be supported for dedicated resource pool. |
| ZTE | Proposal 10: In Rel-18, prioritize dedicated SL-PRS resource pool associated with the BWP of a carrier that is the same as that of the SL communication resource pool Proposal 11: Pool level TDMed restriction between SL-PRS resource pool and SL communication pool should be avoided. |
| Xiaomi | Proposal 6: A separate SL positioning BWP or frequency layer can be defined which is different from SL communication BWP. Proposal 7: Physical layer sidelink control information is transmitted in the dedicated SL positioning pool to indicate or reserve the resource of current/future SL-PRS transmission(s) - SL PRS transmission and the associated control information transmission are TDMed |
| Lenovo | Proposal 20: RAN1 to further study the SL PRS resource configuration considering the relationship between SL Positioning frequency layers or SL BWPs with the agreed SL Positioning dedicated resource pools. |
| Interdigital | Proposal 28: Consider at least an associated PSCCH transmission in the dedicated resource pool (pre)configured for SL-PRS. |
| Apple | Proposal 13: For SL positioning reference signal configured in dedicated SL positioning slots a study the need for a new SL positioning slot and includes: - whether any legacy channels can be added to this slot - the need for a new AGC symbol and additional gap symbols in the slot The new resource allocation methods apply to the dedicated slots. |
| Denso | Proposal 1: RAN1 to consider to support that only dedicated resource pool(s) can be (pre-)configured for SL-PRS(Alt.1). |
| Samsung | Proposal 10: For dedicated resource pool(s) for SL-PRS, new PHY structure is considered for SL PRS transmission including at least - AGC/GAP symbols - PSCCH - SL PRS symbols - FFS: PSFCH, 2nd stage SCI - FFS: how to multiplex channels/signals in a slot Proposal 11: At least for dedicated resource pool(s) for SL-PRS, consider SL PRS RE level multiplexing on same slot/subchannel to improve SL PRS capacity and resource utilization efficiency in a resource pool. |
| Qualcomm | Proposal 17: Support the dedicated SL-PRS resource pool being on a carrier that can be different from that of the sidelink communications resource pool. |
| Mediatek | Proposal 2-1: Support dedicated resource pool(s) for SL-PRS transmission |
| CEWIT | Proposal 11: PRS slot configuration over dedicated resources should be defined for SL positioning. FFS is the slot structure considering PRS resource selection, number of assisting UEs, AGC, GP symbol requirements, control channel, etc. |

There can be a few things that we can discuss here based on the above proposals:

- 1st Topic: Which other channels are in the resource pool for positioning? It seems there are 3 options: SL-PRS only, SL-PRS + SCI, SL-PRS + SCI + data.

- 2nd topic: With regards to the SCI indicating to the receiving UE the SL-PRS allocation, does it need to be from the same dedicated resource pool, or some other dedicated resource pool, or even from a shared resource pool (if agreed) it can even be from a shared resource pool?

- 3rd topic: Is SCI reserving the whole slot, or are we just reserving “SL-PRS resource”? E.g., if a SL-PRS resource is X-symbols, only those X-symbols are considered reserved.

- 4th topic: Can a SL-PRS be smaller BW than the BW of the resource pool for positioning, or is the bandiwdht fixed equal to the BW of the pool?

- 5th topic: Is the RP for positioning in a separate SL-BWP or the same SL-BWP in a carrier? Can it be on carrier that doesn’t have any RP for communication?

We make separate proposals below for each one of these topics. My first intention is to gather views to see if there is any consensus we can make this meeting for any of the topics. If not, we need to at least write down the alternatives in order to start having some structure on what a resource pool for positioning is.

[CLOSED] Feature Lead Proposal 5.1.1.A-v0

For a dedicated resource pool for positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS:

  - Alt. 1: No other channel can be included beyond SL-PRS

  - Alt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) can be included

  - Alt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH can be included

Companies views

| ZTE | Alt.3 should be excluded first. There is no need to introduce dedicated SL-PRS resource pool if PSSCH which carries SL communication data is transmitted in the same resource pool with SL-PRS. Among Alt.1 and Alt. 2, we are open to discuss and slightly prefer Alt.1. |
| --- | --- |
| CMCC | Comments. First, it should be discussed whether the PSCCH which carried the 1st SCI for reservation (i.e., maybe the next question should be discussed first) can be transmitted in SL data resource pool or not, i.e., whether a SCI in a SL data resource pool can indicate/reserve SL-PRS in a dedicated SL-PRS resource pool, if so, Alt 1 can be agreed; otherwise, only Alt 2 and Alt 3 can be selected. Next, it should be discussed whether 2nd SCI in NR sidelink is needed for SL-PRS or not, if so, since it is carried by PSSCH in NR sidelink, PSSCH is also needed for SL-PRS, then Alt 3 should be selected; otherwise, Alt 2 should be selected. |
| vivo | We prefer alt 2. In our opinion, if the control information (e.g., PSCCH) of the associated SL-PRS is not transmitted in that resource pool, UE needs to monitor two resource pools to detect the control information and SL-PRS. It would increase the UE complexity and have some impacts on the Rel-16 SL specification which is not expected. Therefore, we support control information in the dedicated resource pool. |
| Huawei, HiSilicon | OK. We support Alt. 2. Note however that this proposal may be pending on first agreeing the proposed HIGH] Feature Lead Proposal 4.1.4.B-v0 We doubt whether Alt.3 is still for a dedicated resource pool. |
| Spreadtrum | We prefer Alt. 2. |
| Apple | We are fine with Alt-2 |
| Futurewei | Support Alt 2. |
| Intel | Alt. 2. Depending on the resource allocation considerations, we think the SCI is needed to facilitate sensing. |
| CATT | Alt.3. PSCCH can be used to indicate the resource allocation of SL-PRS. PSSCH can be used to carriy the measument report in the dedicated RP. |
| DCM | Alt 3. Agree with CATT. Measurement report should be conveyed on the RP. |
| Samsung | We suggest the following modification by adding additional alternative as For a dedicated resource pool for positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS: - Alt. 1: No other channel can be included beyond SL-PRS - Alt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) can be included - Alt. 3: PSCCH/PSSCH which carries SCI associated with SL-PRS transmission(s) and PSSCH with higher layer SL data can be included - Alt. 4: PSCCH/PSSCH which carries SCI associated with SL-PRS transmission(s) can be included |
| LGE | We prefer Alt.1 especially when SL PRS comb size of 12 is supported. There will be no available resource for SCI when N=12 is used for SL PRS. One advantage of Alt. 1 compared to Alt. 2 is power saving required for monitoring SCI. As long as SCI in a different pool does not indicate the existence of SL PRS, UE doesn’t need to jump to the dedicated pool. In Alt. 2, UE should monitor a quite large BW of the dedicated pool for monitoring SCI. We don’t think Alt. 3 is feasible considering lack of resources for multiplexing all those channels. |
| Sony | We prefer Alt.2 |
| Sharp | We are OK to have PSCCH in dedicated resource pool. However we’d like to clarify that there is a case that PSCCH transmission in the resource pool is not required for some SL operataion. |
| OPPO | At this stage, we are fine to keep all the alternatives for futher discussion. For a dedicated resource pool configured for resource selection scheme 2, PSCCH should be included to avoid too much chages on the SL mode 2 resource slection mechanism. For a dedicated resource pool configured for resource selection scheme 1, PSCCH may not be needed. As to Alt 3, it could be considered only for 2nd stage SCI in case of 1st stage SCI is not sufficient to convery all information, but it should not be used for SL data transmission (including positioning related higher layer signling or message). |
| InterDigital | We support Alt. 2. |
| Xiaomi | We are fine with the proposal. In SL communication, 1st stage SCI is transmitted in PSCCH while 2nd stage SCI is transmitted in PSSCH. Information including source/destination ID, etc. are carried in 2nd stage SCI. At the current stage, we think both alt 2 and alt 3 shall be further studied. |
| CEWiT | We prefer Alt 3 |
| Qualcomm | We support the company proposals to remove Alt 3. If PSSCH is included, then this is no longer a dedicated resource pool. We ok to continue discussing Alt 1 and Alt 2. |
| Nokia, NSB | For Alt. 3, it should be clarified whether the PSSCH is restricted to carry only 2nd stage SCI. |

We observe the following:

- With regards to which channels can be included in the resource pool in addition to SL-PRS:

  - Alt. 1: No other channel can be included beyond SL-PRS

    - Support: ZTE (1st preference), LGE, Qualcomm

    - Against:

  - Alt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) can be included

    - Support: ZTE, vivo, Huawei, HiSilicon, Spreadtrum, Apple, Futurewei, Intel, Sony, Interdigital, Xiaomi, Qualcomm

  - Alt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH can be included

    - Support: CATT, DCM, Xiaomi, CEWiT, Nokia, NSB (for 2nd SCI)

    - Against: ZTE, Huawei, HiSilicon, LGE, Qualcomm

  - Keep all Alternatives at this stage

    - OPPO

  - Split Alt. 3 to 2 alternatives  3.A & 3.B

    - Samsung

  - Depending on further progress on other items, downselection should happen later

    - CMCC

[CLOSED] Feature Lead Proposal 5.1.1.A-v1

For a dedicated resource pool for positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS support one or more of the following alternatives:

  - Alt. 1: No other channel can be included beyond SL-PRS

  - Alt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) can be included

  - Alt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH can be included

    - 3A: PSSCH carries higher layer SL data

    - 3B: PSSCH carries SCI associated with SL-PRS transmission(s)

- Note: Companies are encouraged to provide their analysis and views on the above alternatives

Companies views

| FL | Please comment if the proposal is OK for agreement without downselection this meeting. |
| --- | --- |
| LGE | The proposal itself without downselection is fine for us. Our preference is Alt. 1, which is simplest and causes no waste of resources. Alt. 2 may cause some wast of resources in that SCI will be transmitted through too-wide spectrum for SL PRS transmission. Also, there will be no room for SCI transmission if the number of SL PRS symbols equals 12. We’re not convinced yet whether multiplexing SL communication data and SL PRS into a single slot is feasible in Alt. 3 |
| CATT | OK with this proposal, we prefer Alt.3. |
| ZTE | Alt.1 is preferred. We can compromise to this proposal and keep all the alternatives in this meeting. |
| Qualcomm | Alt 3 does not seem aligned with a dedicated resource pool. Once PSSCH (which also includes PSSCH DMRS, CSI-RS, SCI-2, …) is included, then there no reason to distinguish between a dedicated resource pool and a shared one. Given that SCI-2 and PSSCH DMRS are needed to decode SL-SCH, this option also introduces large overhead. The choice between Alt 1 and Alt 2 depends on the agreed reservation mechanism. If the reservations are to use SCI, then Alt 2 is needed. Otherwise, Alt 1 has lower overhead and is the preferred option. |
| InterDigital | We support Alt. 2, as the design will be simpler with the associated SCI within the same resource pool. We do not prefer Alt. 3A, as the higher layer SL data can have different traffic pattern and their transmission in the dedicated pool will conceivably increase congestion and intereferences to SL-PRS. Regarding Alt 3.B, we’d like to clarify why PSSCH includes another SCI associate with SL-PRS transmission while PSCCH already includes such a SCI. Does it mean SCI-2? |
| Intel | We are fine with the proposal and do down-selection in the upcoming meeting. We support Alt. 2 and do not support Alt. 1 and 3. |
| CMCC | The proposal is OK. Maybe we should discuss next question first, then go back to whether a SCI in the dedicated resource pool is needed. And as if SCI is agreed to be transmitted in the dedicated resource pool, whether 2nd SCI is needed should also be decided by RAN1, if so, option 3B should be adopted;otherwise, option 2 should be adopted. The intention of Option 3A is not clear to us so it should be removed. |
| Huawei, HiSilicon | At least Alt. 2 is needed. For Alt 3, we think in the dedicated resource pool, there should be no legacy transmission. The benefits of the PSSCH in the dedicated resource can decouple communication resource pool and SL-PRS resource pool. i.e. one SL positioning dedicated resource pool can work both for SL-PRS transmission and related control singaling and measurement report. This solution can potentially simiplify the resource pool configuration, and potentially reduce the latency. By the way, we have some question on Alt3 as: On 3A of Alt.3, does this higher layer SL data also includes MAC PDU containing MAC subheader and MAC CE? In general in this agenda, MAC layer is excluded from higher layer concept. |
| OPPO | OK |
| Samsung | OK |
| Spreadtrum | We support Alt. 2. |
| DCM | We think Alt 3 is necessary so that measurement report is possible in this RP; otherwise, measurement report shall be performed in communication pool. Which UE uses which RP is uncelar, so the mechanism does not work well. |
| Apple | OK |
| Xiaomi | We prefer Alt 2 or Alt 3B, depending on RAN1 decision on whether 2nd stage SCI will be used in SL positioning. |
| Nokia, NSB | OK |

There was an attempt to go the above proposal for Email endorsement and there were a lot of comments for the details of Alt. 3. We need to keep it high-level in this meeting, otherwise no progress. I am suggesting to consider the following options and put FFS in the details for Opt. 3.

##### [MEDIUM] Feature Lead Proposal 5.1.1.A-v4

For a dedicated resource pool for positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS consider the following options:

  - Opt. 1: No other channel can be included beyond SL-PRS

  - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH is included

    - FFS: Details

- Note: Companies are encouraged to provide their analysis and views on the above

##### Companies views

| LGE | We support FL proposal in general. Just one clarification is needed for Opt.3. As the proposal is intented for a dedicated resource pool for SL positioning, it would be better to clarify that PSSCH is also only for SL positioning. We support Opt.1, but we’re ok with the proposal itselt without downselection for this meeting. ##### [MEDIUM] Feature Lead Proposal 5.1.1.A-v4 For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) is are included - FFS: Details - Note: Companies are encouraged to provide their analysis and views on the above |
| --- | --- |
| MTK | We are okay for LGE’s revision. Basically, these options should be concluded in next meeting, not this time We prefer option 2. Just let the decidated pool focus on measurement purpose. |
| ZTE | Fine. |
| Intel | We support the FL’s proposal. Which options are viable depends on decisions made for resource allocation thus cannot be downselected before further agreements on resource allocation are reached. |
| FL Update | Please comment on the latest proposal from LGE |
| InterDigital | We support in general the proposal. We prefer Option 2. We prefer to remove Option 3 unless more information is provided regarding the relationship between the PSSCH and SL-PRS/SL positioning. Does this PSSCH carry SL-PRS measurement reporting or other information relevant to SL positioning? Or carry SL-PRS configuration messages? |
| Qualcomm | We still prefer to remove Option 3 but can accept having all three options for discussion at this stage. However, the proposal needs to make it clear that not all are supported: ##### [MEDIUM] Feature Lead Proposal 5.1.1.A-v4 For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS consider the one of following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) is are included - FFS: Details - Note: Companies are encouraged to provide their analysis and views on the above |
| CATT | OK with the LGE’s version. We prefer Opt.3. |
| CEWiT | Okay with LGE’s proposal. Support Option 3. |
| OPPO | Support the verion proposed by QC. |
| vivo | We are okay with the modification from Qualcomm |
| Samsung | QC version is OK to make progress. |
| Apple | OK with LG’s version. On QC’s version, this may be very restrictive. It seems that if we choose a higher option, we automatically choose a lower option as well. e.g. even if Option 3 is supported, do not see why Option 2 cannot be supported if there is no PSSCH to send at that point. So “if not all are supported” does that mean that we are not allowing Option 2 if we choose option 3 ? |
| Spreadtrum | We are fine with QC’s version. |

[CLOSED] Feature Lead Proposal 5.1.1.B-v0

For a dedicated resource pool for positioning,

- The SL-PRS could be indicated to a receiving UE using

  - Alt. 1: an SCI received in the same resource pool

  - Alt. 2: an SCI received in the same resource pool or some other resource pool

    - Note: the other resource pool could be another dedicated resource pool or a shared resource pool, depending on further agreements.

Companies views

| ZTE | We prefer Alt.2. |
| --- | --- |
| CMCC | Alt 2. At least SCI received in the same resource pool can be supported to indicate the SL-PRS; FFS whether SL-PRS can be inidicated by SCI from another resource pool, e.g., from a SL data resource pool. |
| Vivo | Alt 1 The SCI receiving in the same resource pool can decrease the scheduling complexity. In NR R16/R17 SL, the slot index is the logical index in a resource pool. If the cross-resource pool scheduling is supported, the alignment of slot index in different resource pools should be defined at least. |
| Huawei, HiSilicon | For Alt.2, one question for clarification is whether it means both should be supported, instead of a further down-selection between the same RP and some other RP? |
| Spreadtrum | We prefer Alt. 2. |
| Apple | Atl2. Support CMCCs update |
| Futurewei | Support Alt 2. |
| Intel | Alt. 1. We have some concern on Alt. 2 with some other resource pool, i.e., the sensing operation is not very clear as SCI in other pools may be missed. This would also complicate the design and implementation. |
| CATT | Alt.1. the dedicated resource pool should contain both the SCI and its related SL-PRS. |
| DCM | Alt 1. Linkage between two RPs are not preferred; the two RPs in Alt 2 should be treated as a single RP. |
| LGE | We think the proposal is overlapping with Proposal 5.1.1.A-v0. They depend on each other. Let’s discuss this proposal after decision of Proposal 5.1.1.A-v0. |
| Sony | Alt.1 |
| Sharp | One question for the clarification. Does the term “indicated” here cover an activation of periodic SL-PRS? |
| OPPO | Seems the proposal is related to Proposal 5.1.1.A-v0 above, in our view Alt. 1 should be supported, Alt.2 would cause huge impact on UE autonomous resource selection scheme (assuming SL mode 2 is baseline). |
| InterDigital | WE support Alt. 1. We think it is more complext to introduce an association between SCI and corresponding SL-PRS in different resource pools. Regarding the Note, we would like to have a clarification on “another dedicate resource pool” for SCI transmission. Is this “another dedicated resource pool” dedicated to SL-PRS? If it is, then this dedicate resource pool already support both SCI and SL-PRS transmission, albeit the SCI can be used to indicate SL-PRS in a different resource pool? |
| Xiaomi | Alt 1. Supporting cross pool indication will increase a lot of standardization complexity. And we did not see the necessity to support such function. |
| CEWiT | Prefer Alt 2 |
| Qualcomm | Could you clarify how the proposal differs from 4.1.4.B-v0? We also agree with the comments that cross-resource pool reservation using SCI is a complex issue that is best avoided. |
| Nokia, NSB | Alt. 1. Cross-pool SCI would create backward compatibility problems if the other resource pool is not restricted to Rel-18 UEs. |

The following companies supported the following alternatives:

- Alt. 1:

  - Vivo, Intel, CATT, DCM, Sony, OPPO, Interdigital, Xiaomi, Qualcomm, Nokia, NSB

- Alt 2

  - Spreadtrum, Futurewei, CEWiT

We may need to wait for progress in 4.1.4.B first. Difference from 4.1.4.B is that this treats the topic of same vs cross respurce pool indication, and it assumes SCI is being used for reservation.

[CLOSED] Feature Lead Proposal 5.1.1.B-v1

For a dedicated resource pool for positioning, for the case of SCI being used for indication/reservation

- The SL-PRS could be indicated using at least an SCI received in the same resource pool

- FFS: an SCI received in some other resource pool

  - Note: the other resource pool could be another dedicated resource pool or a shared resource pool, depending on further agreements.

Companies views

| LGE | As commented earlier,the proposal is overlapping with Proposal 5.1.1.A-v1, which treats the issue whether SCI can be transmitted with SL PRS in the same resource pool or not. We cannot discuss the same issue in multiple proposals. |
| --- | --- |
| ZTE | Disagree, this proposal is highly related to the previous proposal and let’s keep it inclusive. The following update is suggested: For a dedicated resource pool for positioning, for the case of SCI being used for indication/reservation, support one or more of the following options: - The SL-PRS could be indicated using at least an SCI received in the same resource pool - FFS: The SL-PRS could be indicated using an SCI received in some other resource pool - Note: the other resource pool could be another dedicated resource pool or a shared resource pool, or a resource pool not for SL-PRS (e.g. a SL data resource pool), depending on further agreements. |
| InterDigital | We support the proposal. We share LGE’s view that this proposal is closely related to the Proposal 5.1.1.A-v1. |
| CATT | Support the proposal. We prefer the SL-PRS should be indicated using an SCI received in the same resource pool(i.e., the same dedicated RP), and the cross-RP indication should not be supported. |
| CMCC | OK |
| Huawei, HiSilicon | We are fine with the proposal. |
| Samsung | OK |
| Spreadtrum | Support the proposal. |
| Apple | OK |
| xiaomi | OK |
| Nokia, NSB | OK |

[CLOSED] Feature Lead Proposal 5.1.1.C-v0

For a dedicated resource pool for positioning,

- With regards to the granularity of a resource allocation for SL-PRS transmission

  - Alt. 1: a SL-PRS resource can be allocated (SL-PRS-resource-based allocation)

  - Alt. 2: a slot can be allocated (slot-based resource allocation)

Companies views

| ZTE | Alt.1 to better utilize SL-PRS resources. |
| --- | --- |
| CMCC | Comments. Mayebe it should be discussed whether multiple SL-PRS can be TDMed in a slot first, i.e., whether a slot contains more than one SL-PRS candidate resources in time domain. |
| vivo | Alt 1 ##### In the previous discussion, the comb structure is supported to increase the capacity of SL PRS. To increase the scheduling flexibility, the SL-PRS can be allocated as the granularity of the resource allocation for SL-PRS. |
| Huawei, HiSilicon | Alt.1 and Alt.2 may contain each other. We prefer at this stage to discuss slot-based resource allocation and sub-slot based resource allocation. Whether the resource allocation contains a single SL-PRS resource or multiple SL-PRS resource can be further discussed. |
| Spreadtrum | We prefer Alt. 1. |
| Apple | Alt 2 |
| Futurewei | Alt 1 is preferrable, especially if an IUC approach is used for UE SL-PRS transmission coordination. The answer to this question is also related to the Sl-PRS congestion avoidance procedures. |
| Intel | Alt. 1. We think at least TDM multiplexing of SL-PRS resource should be supported in the dedicated resource pool. |
| CATT | Alt 1. This aligns with the Rel-16 DL-PRS/SRS-Pos design framework. |
| NEC | We prefer Alt. 1. |
| DCM | Slot-based allocation (i.e., Alt 2) is preferred. If resource efficiency is concern, higher SCS should be used instead. |
| Samsung | We think that the proposal is not clear enough. |
| LGE | We prefer Alt. 1 for better utilization. If N=1 and Alt.2 is supported, most of the resources in a slot will be wasted. |
| Sharp | Some more clarity may be needed. How is the SL-PRS resource defined? Is it a resource defined within a certain time duration (e.g. slot) ? |
| OPPO | Seems SL PRS resource here refers to a granulairy shorter than a slot, but we have no clear definition of SL PRS resource to date. Maybe following revision is needed: For a dedicated resource pool for positioning, - With regards to the granularity of a resource allocation for SL-PRS transmission - Alt. 1: a SL-PRS resource shorter than a slot can be allocated (SL-PRS-resource-based allocation) - Alt. 2: only a slot can be allocated (slot-based resource allocation) |
| InterDigital | We support in general Alt. 1. However, we suggest to discuss SL-PRS structure and slot format firt before addressing this proposal, because the definition of “a SL-PRS resource” may vary depending on what signals (AGC/gap) and/or channel (PSCCH) included in this resource. A SL-PRS transmission applies a set of resources first according the pattern and in addition we should discuss what more resources need to to enable such SL-PRS transmission and should be considered as a par tof “a SL-PRS resource” as the granularity of SL-PRS rsource selection and reservation. |
| Xiaomi | Alt 1 to support resource multiplexing among UEs. |
| Qualcomm | Alt 1 |
| Nokia, NSB | We prefer Alt. 1 |

Observations:

- Alt. 1: a SL-PRS resource can be allocated (SL-PRS-resource-based allocation)

  - ZTE, vivo, Spreadtrum, Futurewei, Intel, CATT, NEC, LGE,Xiaomi, Qualcomm, Nokia, NSB

- Alt. 2: a slot can be allocated (slot-based resource allocation)

  - DCM, Apple

- Clarifications needed

  - Huawei, HiSilicon, Samsung, Sharp, CMCC

[CLOSED] Feature Lead Proposal 5.1.1.D-v1

For a dedicated resource pool for positioning,

- With regards to the granularity of a resource allocation for SL-PRS transmission

  - Alt. 1: sub-slot-based allocation is considered

    - E.g. a SL-PRS resource shorter than a slot can be allocated

  - Alt. 2: only slot-based allocation is considered

- Note: Companies are encouraged to provide their analysis and views on the above alternatives

Companies views

| FL | Please comment if the proposal is OK for agreement without downselection this meeting. |
| --- | --- |
| LGE | We generally agree with FL proposal, but Alt. 1 needs to be clarified. The original intention of Alt. 1 is SL PRS resource based allocation. We think this terminology is more correct. The term ‘sub-slot-based’ may mislead that we need to define a ‘sub-slot’ within a slot. It’s not an intention, and we don’t support it. We propose the following clarification. We support Alt. 1. ##### [MEDIUM] Feature Lead Proposal 5.1.1.D-v1 For a dedicated resource pool for positioning, - With regards to the granularity of a resource allocation for SL-PRS transmission - Alt. 1: sub-slot SL PRS resource-based allocation is considered - E.g. a SL-PRS resource shorter than a slot can be allocated - Alt. 2: only slot-based allocation is considered - Note: Companies are encouraged to provide their analysis and views on the above alternatives |
| CATT | We prefer the sub-slot-based allocation is changed into SL-PRS resource in Alt.1, since no other case can be used. |
| ZTE | We do not prefer the wording in Alt.1 and it seems like including a new definition of subslot. We suggest the following: For a dedicated resource pool for positioning, - With regards to the granularity of a resource allocation for SL-PRS transmission - Alt. 1: Symbol-level allocation depending on the SL-PRS configuration is considered sub-slot-based allocation is considered - E.g. the number of symbols for a SL PRS resource a SL-PRS resource shorter than a slot can be allocated - Alt. 2: only slot-based allocation is considered - Note: Companies are encouraged to provide their analysis and views on the above alternatives |
| Qualcomm | We prefer the previous version of the proposal. We support Alt 1 to increase system resource utilization efficiency. For a dedicated resource pool for positioning, - With regards to the granularity of a resource allocation for SL-PRS transmission - Alt. 1: a SL-PRS resource can be allocated (SL-PRS-resource-based allocation) - E.g. a SL-PRS resource shorter than a slot can be allocated used and more than one SL-PRS resource can be within a slot. - Alt. 2: only slot-based allocation is considered - Note: Companies are encouraged to provide their analysis and views on the above alternatives |
| InterDigital | We understand this proposal is about the granularity of the SL-PRS resource selection and we support a SL-PRS resource selection based on a SL-PRS resource in both time and frequency domain. Thus, We’d like to have a clear definition of “a SL-PRS resource” to be discussed in connection with this proposal. In time domain, the inclusion or exclusion of PSCCH/AGC/Gap symbol in “a SL-PRS resource” will also impact the resource allocation. In frequency domain, does using SL-PRS-resource-based resource allocation imply a RE-level FDMed of SL-PRS transmissions? Or the granularity of SL-PRS frequency resource allocation is sub-channel-based? |
| Intel | We share similar view as LGE that Alt. 1 should be updated to SL-PRS-resource-based allocation |
| CMCC | We think at least for Scheme 2, only Alt 2 can be considered because if sub-slot transmission is supported, too much complexity will be brought to resource selection procedure. For scheme 1, maybe both of the Alts can be considered. |
| Huawei, HiSilicon | We are fine with the proposal. |
| NEC | We share the similar view as InterDigital about making a clear definition of a SL-PRS resource at first both in time domain and in frequency domain, because some specific features should be considered for SL communication compared with that of PRS resource allocation in NR positioning. |
| OPPO | We are fine with the proposal from FL, to use “SL PRS resource-based” seems not desirable, as the terminology “SL PRS resource” is not defined yet. Or how about the following: For a dedicated resource pool for positioning, - With regards to the granularity of a resource allocation for SL-PRS transmission - Alt. 1: sub-slot-based allocation is considered - E.g. a SL-PRS resource can be shorter than a slot can be allocated - Alt. 2: can only be slot-based allocation is considered - Note: Companies are encouraged to provide their analysis and views on the above alternatives |
| Samsung | OK |
| Spreadtrum | We prefer SL-PRS-resource-based allocation and agree with QC’s version. |
| DCM | OK, and which alt is better is highly-related to 5.1.1A. Both Alt should be kept now. |
| Apple | Fine with proposal since both are kept for now. |
| xiaomi | We are fine with FL proposal ,and fine if not downselect in this meeting. |
| Nokia, NSB | OK, but we prefer Qualcomm’s revision on Alt-1, and support Alt-1. |

Based on the above, i think we may also need to step back and have a first more generic agreement on the resource allocation granularity. What about the following?

##### [MEDIUM] Feature Lead Proposal 5.1.1.D-v2

For a dedicated resource pool for positioning,

- With regards to the granularity of a resource allocation for SL-PRS transmission, study further at least the following options:

  - Opt. 1: shorter than a slot allocation (e.g., symbol-level allocation)

  - Opt. 2: SL-PRS-resource-based allocation

  - Opt. 3: slot-based allocation

- Other Options are not precluded

- Note: Definition of SL-PRS resource needs to have progressed before discussing on downselecting amongst these options.

- Note: Companies are encouraged to provide their analysis and views on the above options

##### Companies views

| LGE | We support FL proposal in general. But please see our comments below. In original proposal, opt. 1 and opt.2 were identical in our understanding. But now they were separated listed as different option, and it seems that in option 1 a part of SL PRS resource can be allocated in a symbol level. I’m not sure it is the intention of FL, but anyway we’re ok with the current FL since anyway we can conclude this issue after finalizing the definition of SL PRS resource. |
| --- | --- |
| mtk | We don't expect larger symbol number for a SL-PRS resource. Then we are not sure what is the main difference between option 1 and option 2. We prefer option 1. |
| ZTE | Prefer Op. 2. Option 1 can be deleted, in the previous round, the “symbol-level allocation” we proposed is actually “the number of symbols for a SL PRS resource” which is the same as Option 2. Also, since we do not have agreement on whether larfer than 12 symbols SL-PRS is supported, option is not preferred. |
| Intel | We support the FL’s proposal. Dedicated resource pools for SL positioning have the target to multiplex full bandwidth SL-PRS transmissions of multiple users in the same slot. Thus a combination of option 1 and option 2 should be the target for this agreement. The exact configuration should depent on either the network for in coverage scenarios or preconfiguration in out of coverage scenarios. |
| FL Update | To LGE: Opt. 1 and Opt. 2 are not identical depending on further progress. Opt. 2 could mean that the actual resource elements are only reserved (e.g. comb-4/4 symbols, we only reserve every 4 RE for 4 symbols). Opt. 1 it is symbol-level reservation, e.g., 4 symbols, but all REs inside that symbol. |
| InterDigital | We prefer Opt 2. Also, we think Opt. 1 is identical to Opt .2 as ZTE indicates. |
| Qualcomm | While we prefer Option 2, we are ok with the current proposal. |
| CATT | OK with the proposal and we prefer Option 2(SL-PRS-resource-based allocation). In addition, the following note as follows looks like unnecessary, since the main bullet had said “study further at least the following options”, the note is duplicated with the main bullet, so we prefer to remove the note as follows, in order to make the proposal more concise. - Note: Companies are encouraged to provide their analysis and views on the above options |
| CEWiT | Fine with the proposal |
| OPPO | Although we understand the what Opt. 2 means, just as the Note says, there is no definition of SL PRS resource yet, given that we do not think it is appropriate to include Opt. 2 now. Our suggestion is to focus on time domain granularity now, the frequency domain granularity would be clear when SL PRS resource is defined. Furthermore, in proposal 4.1.3-v2 AGC and Rx-Tx turaround symbols are not viewed as SL PRS resource, and perhaps SCI would be transmitted with SL PRS, therefore, in our understanding this proposal is only regarding granularity for resource allocation, it does not mean UE would use all the allocated symbols for SL PRS resource allocation, clarification is necessary to avoid potential confusion. For a dedicated resource pool for positioning, - With regards to the time domain granularity of a resource allocation for SL-PRS transmission, study further at least the following options: - Opt. 1: shorter than a slot allocation (e.g., symbol-level allocation) - Opt. 2: SL-PRS-resource-based allocation - Opt. 3: slot-based allocation - Other Options are not precluded - Note: UE may not use all the allocated symbols for SL PRS transmission, i.e. symbols for other purposes (if any) will be excluded. - Note: Definition of SL-PRS resource needs to have progressed before discussing on downselecting amongst these options. - Note: Companies are encouraged to provide their analysis and views on the above options |
| Samsung | We also think Opt 2 is not clear. So, prefer to remove and discuss seperatley if needed. In addition, one question for clarification: if option 3 is adopted, how could UE determines the symbol used to transmit SL PRS for the case of M<12? Does it mean all symbols in the slot will be used for transmission e.g. by repeated M-symbol pattern? |
| NEC | We are fine with the proposal and prefer Opt. 2 in principle, and share the similar view as OPPO about the definition of SL-PRS resource which also has effect on Proposal 4.1.3-v2. |
| Apple | Okay with the current proposal. Based on the FLs’s explanation, there is a subtle difference between Opt 1 and Opt2. Companies can take this into account this in future discussions. |
| LGE | Thank FL for the comment on the difference between option 1 and 2. Then, we prefer to add “time domain” granularity to the first bullet. Even with this clarification, there is still difference between option 1 and 2 in that how many symbols of a SL PRS resource can be the unit for resource selection. For example, in option 1, there is still possibility that a part of a SL PRS resource can be selected, but in option 2, the whole set of symbols of a SL PRS resource needs to be selected. So we prefer to keep option 2. For a dedicated resource pool for positioning, - With regards to the time domain granularity of a resource allocation for SL-PRS transmission, study further at least the following options: - Opt. 1: shorter than a slot allocation (e.g., symbol-level allocation) - Opt. 2: SL-PRS-resource-based allocation - Opt. 3: slot-based allocation The note added by OPPO is unclear. The first part of the sentence means that the symbols within a slot may be used for other purpose than SL PRS. But the second part means the opposite? |
| Spreadtrum | We support the proposal. |

[CLOSED] Feature Lead Proposal 5.1.1.D-v0

For a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission

  - Alt. 1: The bandwidth of SL-PRS can be same or different than that of the resource pool

  - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool

Companies views

| ZTE | Alt.1. The bandwidth of SL-PRS can be flexibly configured and the bandwidth of resource pool is its upper bound. |
| --- | --- |
| CMCC | Alt 2. We prefer Alt 2 to obtain more positioning accuracy. |
| Vivo | The bandwidth of SL PRS resource pool is (pre-)configured and limited in the BWP. In our opinion, it should be limited in the resource pool to avoid the impact on the transmission in the other resource pool. Further, the bandwidth of SL PRS can be the same or smaller than that of the resource pool. It means that it cannot be larger than the resource pool. |
| Huawei, HiSilicon | OK with Alt.1 in principle. However, it should be clarified what does “different than that of the resource pool” mean, i.e., is the bandwidth < than that of the resource pool, or can the bandwidth be larger than that of the resource pool? We assume the former. |
| Spreadtrum | We prefer Alt.2. |
| FL Response | To Huawei, HiSilicon, vivo, ZTE: Yes, the intention is to say same or smaller: ##### [MEDIUM] Feature Lead Proposal 5.1.1.D-v1 For a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission - Alt. 1: The bandwidth of SL-PRS can be same or different smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool |
| Apple | Fine with the updated version |
| Futurewei | We would like to revisit this question after the BW evaluation of SL Pos requirements are discussed. A dedicated resource pool could be allocated for SL PRS transmissions to be used in different scenarios (commercial, public safety, IIoT, etc) that require different accuracies. In this case Alt 1 may offer better flexibility to select only the necessary BW. |
| Intel | We are okay with Alt. 2. |
| CATT | Alt.1. The bandwidth of SL-PRS depends on the positioning requirements and scenarios. The flecxibilty of SL-PRS bandwidth configuratrion should be ensured. |
| NEC | We are fined with the updated version and prefer Alt. 1. |
| DCM | Alt 1. |
| LGE | We prefer Alt. 2 as the dedicated resource pool is for transmission of SL PRS. Otherwise, the resources are wasted and the accuracy is degraded. |
| Sony | Alt.1 The bandwidth can be seen as the upper-bound of the resource pool |
| OPPO | Fine with the updated version from FL. |
| InterDigital | We support Alt. 1. We think the bandwidth requirement of a SL-PRS can vary depending on positioning method and/or accuracy and a configurable bandwidth within a resource pool is beneficial. |
| Xiaomi | We prefer Alt 2. FDM among SL-PRS can be supported by comb-based structure, we did not see the necessity to use smaller bandwidth for SL-PRS as higher bandwidth can achieve better performance. |
| CEWiT | Fine with the updated version |
| Qualcomm | We prefer Alt 2. |
| Nokia, NSB | We prefer Alt. 1. |

We make the following observations:

  - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool

    - ZTE, CATT, NEC, Huawei, HiSilicon, DCM, Sony, Interdigital, Nokia, NSB

  - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool

    - CMCC, Intel, Spreadtrum, Intel, LGE, Xiaomi, Qualcomm

Based on the above, downselection is not possible now.

[CLOSED] Feature Lead Proposal 5.1.1.D-v1

For a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission

  - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool

  - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool

- Note: Companies are encouraged to provide their analysis and views on the above alternatives

Companies views

| FL | Please comment if the proposal is OK for agreement without downselection this meeting. |
| --- | --- |
| LGE | The proposal itself without downselection is fine for us. We support Alt. 2. |
| CATT | OK with the proposal, we prefer Alt.1. |
| ZTE | Fine with this proposal and we prefer Alt.1 for configuration flexibility. |
| Qualcomm | We’d like to clarify whether the SL-PRS bandwidth in Alt 1 is dynamic or semi-static. Once that is clarified, we’d be ok with the proposal. For a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - FFS whether the bandwidth is (pre-)configured. - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives Our preference is Alt 2. FDMing SL-PRS transmissions in different RBs could degrade performance and should be avoided. In this case, it would be |
| InterDigital | We support Alt. 1. We think the bandwidth a SL-PRS can vary by different Ues depending on positioning method and/or accuracy and Alt.1 can enable FDMed of different SL-PRS transmissions. |
| Intel | We are fine with the proposal and we prefer Alt. 2. Performance degradation can be expected if SL-PRS BW is less than the dedicated resource pool |
| CMCC | The proposal is OK We prefer Alt 2 to guarantee the positioning accuracy. |
| NEC | We support Alt. 1 considering the FDMed SL-PRS transmissions may only partially overlap in time domain. |
| OPPO | OK |
| Samsung | OK |
| Spreadtrum | Support |
| DCM | Alt 1. Why always full-band shall be used as Alt 2 is unclear. |
| Apple | OK |
| Xiaomi | We prefer Alt 2 but OK to not downselect in this meeting. |
| Nokia, NSB | OK. We prefer Alt. 1. |

[CLOSED] Feature Lead Proposal 5.1.1.E-v0

With regards to the relation of the dedicated resource pool for SL-PRS to

- the BWP of a SL communication resource pool:

  - Alt. 1: The dedicated SL-PRS resource pool is associated with the one SL BWP of the carrier

  - Alt. 2: The dedicated SL-PRS resource pool can be associated with a BWP that can be the one SL BWP of the carrier or with a second additional BWP

- the carrier of a SL communication resource pool:

  - Alt. 1: The dedicated SL-PRS resource pool is on the carrier of the SL communication resource pool

  - Alt. 2: The dedicated SL-PRS resource pool can be on a carrier that can be the same or different  from the carrier of the SL communication resource pool

Companies views

| ZTE | Only one BWP and one carrier frequency is defined in current SL communication. At least in Rel-18, we should focus on Alt.1 first. |
| --- | --- |
| CMCC | Alt 1+Alt 1 To follow the principle in NR sidelink, do not support more than one BWP and focus on single carrier currently. |
| Vivo | For the BWP of a communication resource pool, Alt. 1 (i.e., The dedicated SL-PRS resource pool is associated with the one SL BWP of the carrier) should be supported. In R16/R17 NR SL, only one BWP is supported in a carrier. The bandwidth of that BWP can be configured as the same as the carrier, it is not necessary to define another BWP in the carrier. The dedicated SL-PRS resource pool can be configured in the BWP as other SL resource pools. And for the carrier of a SL communication resource pool, Alt. 1 (The dedicated SL-PRS resource pool is on the carrier of the SL communication resource pool) should be supported. |
| Huawei, HiSilicon | For BWP, we prefer Alt.1. One carrier only have one BWP for SL, how does Alt.2 works in the first bullet? For SL carrier, we support Alt.1, because Alt.2 is somehow SL CA case which is not supported yet for SL. |
| FL Response | The understanding in Alt. 2 in the first bullet is that we would have to define 2 BWP in a SL carrier, if this is agreed. |
| Apple | Alt 1 + Alt 1 |
| Futurewei | We agree with the other companies that we should focus on having one BWP of the carrier, and one dedicated resource pool in the carrier (Alt 1+ Alt 1). If the evaluation results will show that a CA is necessary, we could further extend the design to CA. |
| Intel | The SL communication resource pool is in the same SL BWP as the dedicated resource pool. Thus we suggest to change Alt. for the BWP to “The SL communication resource pool is in the same SL BWP as the dedicated resource pool”. For the second part, as the SL CA discussion was not started, we think this should not be discussed at this stage. |
| CATT | Alt 1 + Alt 1. Only one BWP and one carrier are needed. |
| NEC | Alt.1 + Alt. 1 |
| DCM | Alt 1 + Alt 1 |
| Samsung | Alt.1 + Alt. 1 |
| LGE | For BWP, we support Alt. 1 with the same reasons as other companies’. For carrier, we support Alt. 2 considering the case where ITS dedicated carrier is used for transmission of SCI or measurement report. |
| Sony | Alt 2 + Alt 2 . The problem with Alt.1, the legacy SL BWP may have smaller bandwidth. Hence, the accuracy can be compromised. |
| OPPO | Alt 1 and Alt 1, as UE cannot operate on 2 SL BWP so far, and SL CA is not supported yet. |
| InterDigital | We prefer Alt. 1 in both bullets. Currently one SL BWP is supported in one SL carrier. To understand better Alt. 2, we’d like to ask the FL if Alt. 2 implies the operator can configure a SL carrier/BWP dedicate to SL positioning only? |
| Xiaomi | Alt 2 + Alt 1. The bandwidth requirement of SL PRS and SL communication can be very different, and thus it would be beneficial at least on UE power consumption if different BWPs are configured for SL PRS and SL communication. In addition, higher SCS can be used for BWP of SL PRS, and thus more transmission occasions can be configured for SL PRS. Finally, due to large bandwidth requirement of SL PRS, resource pool of SL PRS and communication will be rather TDMed. The specification impact of introducing a SL PRS BWP will be small. |
| Qualcomm | Alt 1 for BWP. The issue of multiple sidelink BWPs was discussed an not adopted in Rel-16. It increased specification and implementation and the use of resource pool already provided the ability to limit the monitored bandwidth. Atl 2 for carriers. This is important to support high accuracy positioning for V2X and public safety. |
| Nokia, NSB | Alt. 1 for the BWP of a SL communication resource pool Alt. 2 for the carrier of a SL communication resource pool |

[ONLINE] Feature Lead Proposal 5.1.1.E-v1

With regards to the relation of the dedicated resource pool for SL-PRS to

- the BWP of a SL communication resource pool:

  - Alt. 1: The dedicated SL-PRS resource pool is associated with the one SL BWP of the carrier

    - ZTE, CMCC, vivo, Huawei, HiSilicon, Apple, Futurewei, Intel, CATT, NEC, DCM, Samsung, LGE, OPPO, Interdigital, Qualcomm, Nokia, NSB, CEWiT (17)

  - Alt. 2: The dedicated SL-PRS resource pool can be associated with a BWP that can be the one SL BWP of the carrier or with a second additional BWP

    - Sony, Xiaomi (2)

- the carrier of a SL communication resource pool:

  - Alt. 1: The dedicated SL-PRS resource pool is on the carrier of the SL communication resource pool

    - ZTE, CMCC, vivo, Huawei, HiSilicon, Apple, Futurewei, Intel, CATT, NEC, DCM, Samsung, Interdigital, OPPO, Xiaomi (14)

  - Alt. 2: The dedicated SL-PRS resource pool can be on a carrier that can be the same or different  from the carrier of the SL communication resource pool

    - LGE, Sony, Qualcomm, Nokia, NSB, CEWiT (5)

#### SL-PRS in the Shared Resource Pool

The following proposals were found in the contributions:

| Huawei, Hisilicon | Proposal 12: Study mechanisms to support multiplexing of SL-PRS and sidelink communication at least in a Rel-18 shared resource pool. Proposal 13: Support multiplexing of multiple SL-PRS transmissions in a slot at least in a Rel-18 shared resource pool. Proposal 14: Study mechanisms for supporting a shared resource pool with backward compatibility, i.e. transmitting SL-PRS in a resource pool configured to legacy Rel-16/Rel-17 Ues. |
| --- | --- |
| Spreadtrum | Proposal 8: For multiplexing of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH), the TDM method is baseline. |
| OPPO | - In the shared resource pool, remaining PRBs (if there are) should be used for SL PRS transmission. - PSCCH and related DMRS as SL communication should be transmitted along with SL PRS in the same slot in a shared mode 2 resource pool. - Priority value for SL PRS should be defined, in shared resource pool, the priority of SL PRS and the priority of SL data should be comparable. |
| Intel | - For shared resource pool for SL-PRS and SL communication, - SL-PRS transmission is associated with PSSCH and occupies same BW as the PSSCH. |
| NEC | Proposal 8: The similar slot based configuration of the shared resource pool should be supported as that of the associated dedicated resource pool. |
| ZTE | Proposal 13: If SL-PRS shares resource pool with SL communication, extending SL-PRS outside the SL resource pool in terms of frequency domain should be considered to satisfy the positioning accuracy requirement. |
| Xiaomi | Proposal 8: For shared resource pool design, study SL PRS transmission together with SL PSCCH/PSSCH transmission in a slot. |
| Lenovo | Proposal 21: In a shared resource pool, TDM multiplexing of PSCCH and SL PRS may be supported. Proposal 22: Further study whether SL-PRS is transmitted on legacy PSSCH region and/or PSFCH region. |
| Apple | Proposal 11: For SL positioning reference signal configured in same slot as SL PSSCH/PSCCH/PSFCH (uses same resource allocation method as PSSCH/PSCCH/PSFCH), a study the modifications to SL-slot structure. This could include: - the P-SRS placement within the slot e.g the PSSCH region, the PSFCH region or in an independent region. - The need for a new AGC symbol and additional gap symbols between the different signals/channels Proposal 12: For SL positioning reference signal configured in same slot as SL PSSCH/PSCCH/PSFCH, RAN1 should study the behavior of SL shared resources with legacy channels. This could include • specifying that only slots that are used for transmission to Rel-18 and above Ues may use a shared resource pool o signaling to indicate the resources that the PSSCH/PSFCH should avoid due to use by the positioning reference signal • limiting shared resources to aperiodic SL-PRS transmission and/or • modifying the behavior of shared periodic or semi-persistent SL-PRS in shared resources |
| Samsung | Proposal 12: For shared resource pool(s) with SL communication, SL PRS is transmitted in the existing slot structure with the following principle as - SL PRS can be transmitted in symbols for PSSCH. - SL PRS is not transmitted in symbols for 2nd SCI. - FFS: Whether SL PRS can be transmitted in symbols for PSCCH. - FFS: Whether SL PRS can be transmitted in symbols for PSSCH DMRS. Proposal 13: In a shared resource pool for SL PRS and SL communication, TX UE indicates whether SL-PRS is transmitted or not for co-existence with SL communication. - FFS: signalling details |

##### [MEDIUM] Feature Lead Proposal 5.1.2-v0

With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with sidelink communication

- Alt. 1: backward compatibility with legacy Rel-16/17 UEs should be supported

- Alt. 2: backward compatibility with legacy Rel-16/17 UEs does not need to be supported (i.e. introduce SL-PRS transmission only assuming a Rel-18 sidelink communication resource pool)

##### Companies views

| LGE | First of all, we need to add “if supported” in the main sentence. Second, restricting a resource pool only for Rel.18 UEs may not be a good solution in the resource utilization perspective. The SL positioning should be backward compatible with legacy UE operations. We support Alt. 1, but we’re ok with FL proposal itself without downselection for this meeting. ##### [MEDIUM] Feature Lead Proposal 5.1.2-v0 With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool (if supported) with sidelink communication - Alt. 1: backward compatibility with legacy Rel-16/17 UEs should be supported - Alt. 2: backward compatibility with legacy Rel-16/17 UEs does not need to be supported (i.e. introduce SL-PRS transmission only assuming a Rel-18 sidelink communication resource pool) |
| --- | --- |
| MTK | We may not need to discuss this item, before shared pool is supported |
| ZTE | Is this proposal really necessary, it is common understanding that SL-PRS transmission related information is transparent to Rel-16/17 UEs. How can we support “backward compatibility with legacy Rel-16/17 UEs”?? BTW, can we also clarity the definition of dedicated resource pool and shared resource pool? Right now we feel companies have diverse understandings. |
| Intel | The motivation of introducing SL-PRS transmissions in a shared resource pool is to enable sharing of resources. Thus, Alt. 1 should be supported. |
| FL Update | To ZTE: We are trying to define the differences betweens dedicated and shared resource pool by describing a few main properties. One main property of shared resource pool is whether backward needs to be taken into account. It was also the comment that was discussed online and the chair’s suggestion to try address what dedicated and shared resource pool mean. For dedicated resource pool, Feature Lead Proposal 5.1.1.A-v4 addresses what a dedicated resource pool could contain. For shared resource pool, backward compatibility with rel-16/17 does NOT mean that the Rel-16/17 UEs are going to transmit or receive SL-PRS. It means that a Rel-18 UE that supports SL-PRS could transmit/receive SL-PRS in a resource pool that contains Rel-16/17/18 SL Data communications and that the Rel-16/17 UEs can avoid resources to be used by SL-PRS with existing signaling. For example, Rel-18 UE reserves a slot with SCI-1 that Rel-16/17 can decode, and then the Rel-16/17 UEs avoid the resources reserved for SL-PRS without needing to know that those resources are for SL-PRS. Note that at least one company proposed that a shared resource pool for SL-PRS could correspond to a Rel-18 shared resource pool and will not accommodate the Rel-16/17 UEs. To MTK: Chair suggested that we need to provide more clarity about dedicated resource and shared resource pool before trying to downselect. I think, one of the main things to understand is the backward compabilitiy aspect of shared resource pool. It is important to arrive at a common understanding on a few main items to see how we can proceed on this debate. |
| Qualcomm | We support the Alt 1 interpretation and propose to remove Alt 2. Per-release resource pools without backward compatibility are not something that has been supported in sidelink. Furthermore, the Alt 2 definition will not work for services like V2X. In our view, the reason to have a shared resource is allow coexistence with legacy UEs. |
| CATT | Since shared resource pool has not been supported until now, so we prefer to add a note to say that this proposal don’t mean that shared resource pool has been supported and this proposal just for the clarification of definination of shared resource pool. |
| CEWiT | Support Alt 1. Agree with comment that share resource pool if allowed will be for coexistence with legacy UE. Therefore backward compatibility is important. |
| OPPO | We share the view that Alt.1 should be supported. |
| vivo | Alt 1 If backward compatibility is not considered, we don’t know how to do SL positioning in the shared resource pool since the Rel-16/17 UEs always can use the same resources for communications. |
| Samsung | We prefer Alt 1. |
| NEC | We support the Proposal and prefer Alt. 1. |
| Apple | Support Alt 1. |
| Spreadtrum | We support the proposal. |

## SL Positioning Resource Allocation Modes

The following agreement was made with regards to the topic of SL Positioning Resource Allocation Modes:

Agreement With regards to the SL-PRS resource allocation, study the following two schemes: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS - Applicable regardless of the network coverage - FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting Ues (e.g. IUC-like solutions). - Note: Other Schemes are not precluded to be studied - FFS how to handle resource allocation of SL-Positioning measurement report Agreement Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

#### 5.2.1 Scheme 1

The following proposals were made in the submitted documents:

| Huawei, HiSilicon | - Proposal 17: Support to request SL-PRS transmission for the UE itself or for another UE to the gNB. - SL-PRS transmission request to a gNB is indicated by MAC CE, which also includes the requested SL-PRS characteristics. - The grant of SL-PRS resource indicated by DCI. |
| --- | --- |
| Spreadtrum | Proposal 9: Dynamic grant and configured grant Type 1/Type 2 can be considered for network-centric operation SL-PRS resource allocation. |
| Vivo | - Scheme 1 resource allocation can be used for SL PRS resource allocation for in-coverage or even partial-coverage scenarios. - RRC signaling and DCI can be used for SL-PRS resource allocation. |
| China telecom | Proposal 8: Considering the amount of fields that should to be reserved for SL-PRS resource allocation, it is recommended to reuse DCI format 3_0 if the reserved fields is relatively small. Otherwise, it could be better to define a new DCI format exclusively for SL-PRS. |
| Intel | - For SL-PRS transmissions in a shared resource pool using scheme-1, the same resource allocation via DCI signaling is considered for transmissions containing SL-PRS as the starting point. - Study the definition of DCI formats including related SL-PRS signaling. |
| ZTE | Proposal 18: In scheme 1, support serving gNB of the UE to configure and send SL-PRS configuration to UE. Proposal 19: Support using DCI to dynamically or semi-persistently schedule SL-PRS and consider whether a new DCI format is needed. Proposal 20: Support LMF to resolve the SL-PRS resource conflict between different scheme 1 Ues in the same sidelink positioning session. |
| Lenovo | Proposal 25: RAN1 to use the existing Mode 1 resource allocation mechanisms bas baseline including dynamic grants and configured grants for the transmission of SL-PRS and SL positioning messages (e.g., assistance data, measurement reports). |
| LGE | Proposal 22: It is supported that the network schedules the SL PRS resources. Proposal 23: When LMF or gNB allocates the SL PRS resources to UE, DCI indicates at least the resource pool index and SL PRS resource information. - Further study is needed whether SL PRS configuration index is also indicated. |
| Samsung | Proposal 6: In Scheme 1 (network centric SL-PRS resource allocation), gNB and/or LMF makes a decision for SL PRS resource allocation and the corresponding information is indicated to UE by DCI. - It is up to RAN2 whether Scheme 1 is performed at gNB and/or LMF. |
| CEWiT | Proposal 8: For scheme 1, PRS resources will be configured by the network through gNB or LMF to all assisting and target UE. |
| Ericsson | - Regarding SL-PRS resource allocation, when the SL UE is in coverage, gNB configuration of which scheme to use for SL-PRS resource allocation can be considered in NR Rel-18. |

Based on the above we make the following proposal:

[CLOSED] Feature Lead Proposal 5.2.1-v0

Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE may receive an activation/triggering of a SL-PRS resource transmission(s) from the network

  - Opt. 1: through higher layers from the network

  - Opt. 2: through Dynamic grant

  - Opt. 3: through configured grant type 1/type 2

  - Note: Multiple options could be supported

  - FFS: Details

Companies views

| ZTE | This proposal is only for recource allocation signaling, the details of activation/triggering can be discussed in separate proposals. We suggest the following update: ##### [MEDIUM] Feature Lead Proposal 5.2.1-v0 Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE may receive an activation/triggering of a SL-PRS resource allocation signaling from the network - Opt. 1: through higher layers from the network - Opt. 2: through Dynamic grant from the network - Opt. 3: through configured grant type 1/type 2 from the network - Note: Multiple options could be supported - FFS: Details |
| --- | --- |
| CMCC | Comments. Further clarifications are needed for the listed options. To our understanding, option 1 indicates that the resources are allocated by LMF and ther higher layer signaling can be LPP, and options 2 and 3 indicate that the resources are allocated by gNB, such as the Mode 1 resource allocation mechanism defined in sidelink. Is it the correct understanding? If so, we think that the options should be refined to be clearer. |
| Vivo | Option 1 and/or option 2 can be supported. And think that resource is provided by gNB |
| Huawei, HiSilicon | Is CG type 1 in Opt.3 covered by Opt.1? |
| Lenovo | Ok with FL’s proposal, although the FFS on details could be better clarified, e.g., FFS: Details on Options 1,2 and 3. |
| Spreadtrum | We are fine with ZTE’s version. |
| Apple | Okay with ZTE’s update. Agree that FFS should be a little more defined. |
| Futurewei | The text requires clarifications as ZTE , CMCC and HW noticed. Not clear what Option 1 is referring to. We think that when in coverage of gNB and LMF, a SL-UE should support similar type of signaling as in R17 positioning. |
| Intel | Suggest to add “activation/triggering/release” to align the previous agreements. For note: does that mean a combination of options can be supported? For instance, higher layers + dynamic grant can be used to trigger SL-PRS transmission? |
| CATT | OK. We think all the options should be supported. |
| DCM | OK |
| Samsung | OK, we preper Opt.2 |
| LGE | As commented by other companies, Opt. 1 is not clear. If the intention is something similar to what CMMC commented, Opt. 1 needs to be clarified. In this case down-selection between Opt. 1 or Opt. 2/3 is needed. Opt. 2 and 3 can be combined as a single option if we agree woth both periodic and aperiodic SL PRS transmission. We prefer more modification on top of ZTE version. ##### [MEDIUM] Feature Lead Proposal 5.2.1-v0 Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE may receives an activation/triggering of a SL-PRS resource allocation signaling from the network. Down select one option. - Opt. 1: through higher layers from the network LMF - Opt. 2: through Dynamic grant, or - Opt. 3: through configured grant type 1/type 2 from gNB - Note: Multiple options could be supported FFS: Details |
| Sony | OK |
| Sharp | We support the FL proposal. Our understanding is that Option1 intends signaling from LMF to UE. |
| OPPO | Any difference between configured grant type 1 and Opt. 1, and any difference between configured grant type 2 and Opt.1+Opt. 2? |
| InterDigital | We are fine with the proposal. We have a same question as Huawei’s and it would be helpful if further clarification can be provided regarding the difference between Option 1 and Type 1 CG (using RRC signaling) included in Option 3. |
| CEWiT | Fine with the proposal |
| Qualcomm | We agree with the proposal and support Option 1. |
| Philips | We support ZTE’s update. |
| Nokia, NSB | OK |

Looking at the above comments, wondering if the following proposal could be aggreable

[MEDIUM] Feature Lead Proposal 5.2.1-v1

Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options:

  - Opt. 1: through higher layers from the LMF

  - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB

    - Up to further discussion which one or more of these shall be applicable

Companies views

| FL | Please comment if the proposal is OK for agreement without downselection this meeting. |
| --- | --- |
| Vivo | Opt.2, and we wonder how the LMF knows the resource of gNB that can be allocated. |
| LGE | The proposal itself without downselection is fine for us. We prefer Opt. 2. |
| CATT | We prefer Opt.2. |
| ZTE | OK to keep the proposal as it is and we support serving gNB of the UE to configure and send SL-PRS configuration to UE in scheme1 but LMF can be involved in resolving the SL-PRS resource conflict. To vivo, as analyzed in our paper, Different Ues may belong to different gNB’s reach, when serving gNB provides SL-PRS configuration to scheme 1 UE for transmitting, the scheme 1 Ues with different serving gNBs may possibly acquire SL-PRS configuration with some resource conflict. In such case, it is straightforward to support LMF to resolve the SL-PRS resource conflict between different scheme 1 Ues in the same sidelink positioning session. |
| Qualcomm | We are ok with the proposal as a compromise and to further discuss the options. We support Option 1. In addition to coordination between gNBs as pointed out by ZTE, this also allows a unified framework for both Scheme 1 and Scheme 2. |
| InterDigital | We prefer Opt. 2. |
| Intel | We are fine with the proposal. We prefer Option 2. |
| CMCC | OK |
| Locaila | We prefer Opt. 2. |
| Huawei, HiSilicon | OK with the proposal, but in general, we believe Option 2 is more reasonable. The reseaon is: the SL-PRS resource configuration is the gNB behaviour. Clarification for whether/which use cases need the LMF to do the configuration is needed.. |
| OPPO | OK |
| Samsung | OK |
| Spreadtrum | Support. We prefer Opt 2. |
| DCM | OK, and Opt 1 would be better. |
| Lenovo | Support |
| Apple | Fine with proposal |
| Xiaomi | We prefer option 2 |
| Nokia, NSB | OK |
| CEWiT | Okay |

#### 5.2.2 Scheme 2

The following agreement was reached with regards to this topic:

Agreement Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects: - Resource selection mechanism for SL-PRS - Inter-UE coordination - Aspects for congestion control mechanisms for SL-PRS

The following proposals were made in the submitted documents:

| Nokia, NSB | Proposal 19: Study assistance and avoidance of conflicts for SL PRS configurations among peer UEs, including mechanisms based on inter-UE coordination (IUC). Proposal 20: Study at least the following methods for SL PRS conflict avoidance: SL PRS muting, comb offset coordination, and SL PRS sequence selection. |
| --- | --- |
| Futurewei | Proposal 6: Use the IUC like protocol for the transmission of the preferred set and the non-preferred set of resources for SL-PRS resource selection purpose, and to trigger SL-PRS transmission and measuring reports. |
| Huawei, HiSilicon | Observation 1: The legacy design for UE autonomous resource allocation in sidelink R16/R17 should be reused as much as possible. Observation 2: A unified Scheme 2 mechanism and related signaling framework should be considered to support different sidelink positioning methods. Observation 3: To support the sensing-based Scheme 2 resource allocation for SL-PRS, at least the following information is needed: - Reservation period; - Priority. Observation 4: To support the sensing-based Scheme 2 resource allocation for SL-PRS, RSRP measurement should be defined. Proposal 18: The SL-PRS is transmitted together with the scheduling PSCCH in the dedicated resource pool, and at least the following information is carried by the SCI: - Reservation period; - Priority. Observation 5: An LS to SA2 and/or RAN2 may be needed to define the reservation period and priority for SL-PRS. Proposal 19: Both sensing and random selection based on Scheme 2 are supported according to UE capability or configuration. |
| Spreadtrum | Proposal 10: Multiple Ues occupying different PRS resources in the same time slot should to be considered. Proposal 11: The association between PSCCH frequency domain monitoring ranges and SL PRS in a slot should be considered for SL PRS resource selection. Proposal 12: Inter-UE coordination (IUC) in Mode 2 should be considered for SL-PRS resource allocation. |
| Vivo | - Scheme 2 resource allocation can be used for SL PRS resource allocation for in-coverage, partial-coverage, and out-of-coverage scenarios. - The SL-PRS can be used for RSRP measurement for sensing in Scheme 2. - Periodic and aperiodic resource reservations should be supported for SL-PRS. |
| OPPO | - Current mode 2 resource allocation in sidelink communication should be the baseline for UE autonomous resource allocation of SL PRS. - IUC scheme 1 should be applied to SL PRS resource selection at least when SL PRS is transmitted within the shared resource pool with SL communication, FFS for dedicated SL PRS resource pool. - Whether IUC scheme 2 should be applied to SL PRS resource selection or not should be review when: - Whether SL PRS supported resource reservation or not is determined; and, - If dedicated SL PRS resource pool, whether there is PSFCH resource to carry the conflict information. |
| CATT | Proposal 14: Resource selection mechanism for SL-PRS in the dedicated resource pool should reuse the existing resource selection mechanism for PSSCH in Rel-16/17 NR V2X as much as possible to reduce the workload of standardization and the complexity of UE implementation. Proposal 15: Existing resource sensing/exclusion/re-evaluation/pre-emption schemes for PSSCH in Rel-16/17 NR V2X should be the starting point for resource selection mechanism for PSCCH and SL-PRS. Proposal 16: Existing random resource selection schemes for PSSCH in Rel-16/17 NR V2X should also be considered for resource selection mechanism for SL-PRS. Proposal 17: In order to solve the issue of SL-PRS resource confliction, the inter-UE coordination mechanism for SL-PRS should be introduced in the dedicated resource pool. Proposal 18: Inter-UE coordination for SL-PRS in the dedicated resource pool should reuse the existing inter-UE coordination mechanism for PSSCH in Rel-17 NR sidelink enhancements as much as possible to reduce the workload of standardization and the complexity of UE implementation. Proposal 19: Existing inter-UE coordination Scheme 1 and Scheme 2 for PSSCH in Rel-17 NR sidelink enhancements should be the starting point for inter-UE coordination for SL-PRS. Proposal 20: Congestion control mechanism for SL-PRS should be studied after the resource allocation mechanism of SL-PRS was determined. |
| Intel | - For SL-PRS transmissions in a shared resource pool using scheme-2, reuse of the resource allocation methods defined for SL communication for single shot transmissions is considered as a starting point. - Study if a resource allocation enhancement for RTT is necessary. - Study the definition of scheme-2 resource allocation for periodic, aperiodic, and semi-persistent SL-PRS transmission in a dedicated resource pool. This procedure comprises of sensing, resource exclusion, and resource selection. - Depending on the number of available SL-PRS resources in a dedicated resource pool the half duplex problem for measurements as well as sensing can occur more frequently. - For sensing-based operation, when compared to periodic transmissions, more collisions and half-duplex problems can be expected for aperiodic transmissions. - Study the definition of pre-reservation of SL-PRS transmissions in the communication resource before the transmission in a dedicated resource pool. - Conflicts for the pre-reservation are resolved like IUC scheme 2. - Congestion control for SL positioning in a shared resource pool follows the congestion control measures and definitions of SL communication. - Study if congestion control is necessary for SL positioning in a dedicated resource pool. - Study how to include SL positioning transmission/reception into the SL intra-UE prioritization framework. |
| Sony | Proposal 11: Consider inter-multiple Ues coordination for sidelink positioning, especially in V2X use-case. |
| NEC | Proposal 10: The IUC method could be considered based on necessary enhancement for OCC SL positioning. |
| ZTE | Proposal 21: If SCI trigged multiple SL-PRS occasions is applied, the sensing window size should be no less than the interval between two SCIs triggering SL-PRS. Proposal 23: Study SL-PRS IUC to maximize resource utilization and minimize resource conflicts. |
| Xiaomi | Proposal 9: Sensing based SL PRS resource selection shall be supported for UE autonomous resource allocation. Proposal 10: Further study whether/how a UE allocates SL PRS resource for other UE transmission shall be supported. |
| CMCC | Proposal 11: The scope of Inter-UE coordination can be extended to mechanisms beyond those supported in Rel-17. Proposal 12: Centralized scheduling mechanism, e.g., mode 2(d) like method discussed in Rel-16 NR sidelink, can be considered for resource allocation for SL-PRS. |
| Lenovo | Proposal 26: RAN1 to use the existing Mode 2 sensing, (re-) selection and reservation as baseline for the transmission of SL-PRS and SL positioning messages (e.g., assistance data, measurement reports). Proposal 27: RAN1 to support resource allocation/coordination schemes for a SL Positioning Group, e.g., resource configuration sharing by a member UE, inter-UE coordination by a member Ues. |
| LGE | Proposal 16: It is supported that UE1 reserves UE2’s resources used for SL positioning (e.g. SL PRS or measurement report) and sends the SL PRS resource reservation information to UE2, at least when UE1 and UE2 participates in the same SL positioning group. Proposal 24: It is supported that UE selects the SL PRS resources based on sensing. Proposal 25: When UE selects the SL PRS resources based on sensing, the 2nd SCI indicates at least the source/destination ID and retransmission request flag in addition to the resource pool index and SL PRS resource information. - Further study is needed whether SL PRS configuration index is also indicated. Proposal 26: The mode-2 resource allocation procedure of SL communication is reused as much as possible for the resource allocation of the control channel and of the measurement report associated with SL PRS transmission. Proposal 28: The resource selection based on the inter-UE coordination message defined in Rel.17 SL is supported for SL PRS resource selection. Proposal 31: Congestion control in SL positioning needs to be studied. |
| Interdigital | Proposal 15: Study a SL-PRS-only resource selection based on legacy Mode 2 sensing procedure with enhancements to minimize SL-PRS collision in Schem 2. Proposal 16: Study a joint Scheme 2 resource allocation for multiplexed SL-PRS and PSSCH/PSCCH transmissions based on legacy Mode 2 sensing procedure. Proposal 17: Study SL-PRS resource allocation based on R17 IUC framework with Scheme 1 at a starting point. Proposal 18: Study CBR and CR measurement for SL-PRS transmissions. |
| Samsung | Proposal 7: In Scheme 2 (UE autonomous SL-PRS resource allocation), UE can decide SL PRS resource based on principle of the existing Mode 2 resource allocation mechanism and the corresponding information is indicated by 1st SCI. Proposal 14: For resource selection mechanism for SL PRS, use Rel-16 Mode-2 procedure as a starting point and further study if additional enhancements are needed. Also, Rel-17 IUC procedure can be studied as a starting point for IUC for SL positioning, but its benefit should be further studied. Proposal 15: In Scheme 2 (UE autonomous resource allocation), congestion control for SL PRS is supported and congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority. - The priority value for SL PRS transmission can be decided independently with the priority value of PSSCH transmission. - CBR measurement for SL PRS can be reported to gNB. - Congestion control for SL PRS can restrict CR limit of SL PRS by CBR and priority. - FFS: The definition of CBR/RSSI/CR for SL positioning. |
| Qualcomm | Proposal 15: Introduce potential mechanisms for SL-PRS resource coordination across a number of transmitting Ues (e.g. IUC-like solutions). |
| CEWiT | Proposal 9: For scheme 2, PRS resources will be configured in the following possible way, - Option 1: PRS resource will be provided by one of the assisting resources for all assisting UE - Option 2: Each assisting UE will find the PRS resource individually. |

[CLOSED] Feature Lead Proposal 5.2.2.A-v0

For Scheme 2, at least for the dedicated resource pool for positioning, with regards to Resource selection mechanism for SL-PRS, pick one or more of the following options:

- Option 1: A sensing based resource selection should be introduced

- Option 2: A random resource selection should be introduced

- In either option, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

Companies views

| ZTE | An additional FFS is needed: - FFS: whether the above sensing is based on SCI or based on SL-PRS, or based on both SCI and SL-PRS. |
| --- | --- |
| CMCC | Option 2 should be removed and more clarifications are needed. For option 1, in NR sidelink, PHY will do resource exclusion then generate a candidate resource set which is reported to MAC layer, MAC layer will do the random selection among the reported candidate resources; For option 2, in NR sidelink, it is mainly designed for power saving UE, in which PHY do not proceed the resource exclusion procedure and all the candidate resources in selection window are identified as available and reported, MAC layer will do the random selection among the reported candidate resources, since power saving is not a key issue here, we propose to focus on option 1 and remove option 2. |
| Vivo | At least Option 1 should be supported. Whether support option 2 can be FFS. |
| Huawei, HiSilicon | OK |
| Lenovo | Supportive of FL’s proposal, although we tend to lean toward Option 1 |
| Spreadtrum | Support. We prefer Option 1. |
| Apple | Support the proposal. |
| Futurewei | OK with the Proposal, At least Option 1 should be supported |
| Intel | We are fine with the proposal. Option 1 and Option 2 can be targeting for different use cases. |
| CATT | OK. Both Option 1 and Option 2 shoul be supported. For random selection, it can reduce both the positioning latency and power assumption. |
| NEC | Support the proposal. |
| DCM | Option 1. |
| Samsung | O.K, we prefer Option 1 to gurantee positioning accuracy. |
| LGE | Support FL proposal. Option 2 may be necessary for VRU, which is an important device type also in SL communication. |
| Sony | Support and our preference is Option 1. Option 2 can be for further study |
| Sharp | We are fine with the FL proposal. |
| OPPO | No need to emphasize “at least for the dedicated resource pool for positioning”, the 2 options are applicable to shared resource pool also. |
| InterDigital | We support the proposal. |
| Xiaomi | We think at least option 1 shall be supported. Random resource selection can only work in sparse network. |
| CEWiT | Support |
| Qualcomm | We think an additional option is to have higher layers negotiate and select resources for PRS. This ensures that scenario requirements are captured in the selection procedure and can dynamically adapt to the deployment. ##### [MEDIUM] Feature Lead Proposal 5.2.2.A-v0 For Scheme 2, at least for the dedicated resource pool for positioning, with regards to Resource selection mechanism for SL-PRS, pick one or more of the following options: - Option 1: A sensing based resource selection should be introduced - Option 2: A random resource selection should be introduced - Option 3: Higher-layer resource selection should be introduced. - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. |

[CLOSED] Feature Lead Proposal 5.2.2.A-v1

For Scheme 2, with regards to Resource selection mechanism for SL-PRS, pick one or more of the following options:

- Option 1: A sensing based resource selection should be introduced

- Option 2: A random resource selection should be introduced

- Option 3: Higher-layer resource selection should be introduced.

- In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

Companies views

| FL | Please comment if the proposal is OK for agreement without downselection this meeting. |
| --- | --- |
| Vivo | We wonder about the mechanism of Higher-layer resource selection for the out-of-coverage case |
| LGE | The proposal itself without downselection is fine for us. We support the legacy behavior, option 1 and 2. |
| CATT | OK with the proposal, but we don’t think Option 3 is needed. |
| ZTE | We propose one FFS in the first round discussion but it was not captured here. In our understanding, if indeed high-layer only resource selection is introduce and there is no SCI to reserve SL-PRS resources, the sensing might based on sensing SL-PRS instead of sensing SCI. therefore, we suggest an additional FFS: FFS: whether the above sensing is based on SCI or based on SL-PRS, or based on both SCI and SL-PRS. |
| Qualcomm | We are ok with the proposal without downselection this meeting. |
| InterDigital | We support Option 1 and Option 2. |
| Intel | We are fine with the proposal. We support Option 1 and Option 2 which can be targeting for different use cases. We do not support Opton 3. |
| CMCC | OK However, at least option 1 should be supported. Option 3 has no basline in NR sidelink, then we will worry about the workload and complexity when introduce such a new mechanism for SL-PRS transmission. |
| Locaila | We are OK with the proposal.Support Option 1 and 2. |
| Huawei, HiSilicon | One question for Option 3, is the higher layer corresponding to PC5 higher layer or network configuration or both? For legacy sidelink communcaiton, both network configuration and UE capability will be involved. |
| NEC | We support the proposal and prefer Option 1 and 2. |
| OPPO | We are open to further study any option, but we are not sure Option 3 is clear, note that in legacy UE autonomous resource selection random resource selection is fully performed at MAC layer, sensing based resource selection is performed by MAC layer within a resource set reported by PHY layer, in the sense, any difference between Option 3 and Option 1 and 2. |
| Samsung | Option 3 is not clear how to select resource(s). It needs to make clear. |
| DCM | Support Option 1. Accept Option 2. Not support Option 3. |
| Lenovo | Option 3 deviates from legacy behaviour and we prefer to focus on Options 1 and 2. |
| Apple | Fine with Options 1 and 2 |
| xiaomi | We prefer option 1 and 2, and also think option 3 needs to be clarified. |
| Nokia, NSB | We prefer Option 1 and Option 2. |

It seems there is strong majority for Option 1 and 2 to be considered. So, Option 3 is added as FFS.

##### [MEDIUM] Feature Lead Proposal 5.2.2.A-v2

For Scheme 2, with regards to Resource selection mechanism for SL-PRS, pick one or more of the following options:

- Option 1: A sensing based resource selection should be introduced

- Option 2: A random resource selection should be introduced

- In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

- FFS: Option 3: whether an additional option where Higher-layer resource selection could be included

##### Companies views

| LGE | We recognized there is another option in scheme 2, that is, IUC-based resource selection. As we have FFS in previous agreement to study the resource selection based IUC, it could be added also as a FFS. For Scheme 2, with regards to Resource selection mechanism for SL-PRS, pick one or more of the following options: - Option 1: A sensing based resource selection should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. - FFS: Option 3: whether an additional option where Higher-layer resource selection could be included - FFS: Option 4: Resource selection based on IUC scheme. IUC scheme and related procedure defined in Rel.17 SL is a starting point. |
| --- | --- |
| ZTE | We notice that our proposed FFS was ignored. We just want to make sure companies have the same understanding about sensing. In SL communication, UE selects the candidate resource(s) for SL data transmission in the selection window based on the sensing results (decoding SCI). However, for SL-PRS, the sensing might based on sensing SL-PRS instead of sensing SCI. At least in SI stage, we should not preclude this option. Or are we just briefly list options without any details? |
| Intel | We support the addition from LGE. |
| FL Update | To ZTE: “Sensing based” is very open in Opton 1. Companies will say what sensing means in their contribution. If we start adding FFS in one option, then other companies will want to add other FFS in other options. For now, we just need to be able to write down a few options for companies to comment for next meeting. Yes i assume supporters of Option 1 will comment how sensing is happening. Either way, since LGE asked to add one more option, lets add one more FFS inside Option 1. ##### [MEDIUM] Feature Lead Proposal 5.2.2.A-v3 For Scheme 2, with regards to Resource selection mechanism for SL-PRS, pick one or more of the following options: - Option 1: A sensing based resource selection should be introduced - FFS: whether the above sensing is based on SCI or based on SL-PRS, or based on both SCI and SL-PRS. - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. - FFS: Option 3: whether an additional option where Higher-layer resource selection could be included - FFS: Option 4: Resource selection based on IUC scheme. IUC scheme and related procedure defined in Rel.17 SL is a starting point. Please continue commenting based on the above |
| InterDigital | We support the FL’s original proposal. Also ok with the latest version. |
| Qualcomm | We can accept the proposal. |
| CATT | We prefer to only keep the Option 1 and Option 2 for the following reasons: Both Option 1 and Option 2 are the resource allocation mechanism specified in Rel-16 V2X, and they can be re-used in resource selection mechanism for SL-PRS, but Option 3 is not, we think Opiton 3 is not needed listed here. For IUC scheme, we tend to treat it as another proposal, IUC should be the enhancements on the top of Option 1 and Option 2. |
| CEWiT | Fine with updated proposal. IUC option also considered and support option 4 to study. |
| OPPO | IUC scheme is an enhancement for mode 2 resource selection in SL, where mode 2 resource selection includes sensing based resource slection and ransom resource selection, given that, Option 4 should not be in the same level as Option 1 and Option 2. For Scheme 2, with regards to Resource selection mechanism for SL-PRS, pick one or more of the following options: - Option 1: A sensing based resource selection should be introduced - FFS: whether the above sensing is based on SCI or based on SL-PRS, or based on both SCI and SL-PRS. - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs (including IUC scheme) for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. - FFS: Option 3: whether an additional option where Higher-layer resource selection could be included - FFS: Option 4: Resource selection based on IUC scheme. IUC scheme and related procedure defined in Rel.17 SL is a starting point. |
| Samsung | We prefer to remove two FFS bullet in the FL’s update and just say as FFS: other options are precluded. |
| Apple | Fine with LGE’s update |
| Spreadtrum | We support the proposal. |

From the above proposals, it is a bit unclear what companies are proposing with regards to the activation/triggering mechanism for a transmitting UE in scheme 2. My understanding that one option is that a transmitting UE receives an activation/triggering through the higher-layer request (i.e. based on the higher layer protocol that is being discussed in RAN2). Companies are encouraged to provide more information on other options in their responses.

[CLOSED] Feature Lead Proposal 5.2.2.B-v0

  - For scheme 2, a transmitting UE may receive an activation/triggering of SL-PRS transmission(s)

      - Opt. 1: through higher layers

        - Note: This does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request for initiating one or more SL-PRS transmission(s)

Companies views

| ZTE | Ok and we think this is common understanding. |
| --- | --- |
| CMCC | Agree. Maybe here we should clarify that the activation/triggering of SL-PRS transmission is come from a same UE, not received from other UEs, is it correct understanding? |
| Huawei, HiSilicon | strictly speaking the mode 2 sensing procedure in Rel. 16/17 does imply that the higher layers select the resource for PSSCH, based on the sensing measurements provided by the PHY layer. |
| Lenovo | Support, this request wording may be added to the main bullet “For scheme 2, a transmitting UE may receive an activation/triggering request of SL-PRS transmission(s)”. |
| Apple | Okay |
| Futurewei | OK, it the way Mode 2 operates. |
| Intel | The proposal is not clear to us which higher layer is triggering which (to it) lower layer. If the intention is to define something similar to the “resource (re)-selection” trigger from the MAC layer to the phy we could agree to this. |
| CATT | It seems that physical layer signaling also can be used to activate/trigger SL-PRS transmission(s). Hence, it is not clear for us why only mention higher layer signaling in Opt. 1. |
| NEC | Support |
| DCM | This proposal intends intra-UE perspective? or between two UEs? |
| Samsung | We suggest the following update: - For scheme 2, a transmitting UE may receive an activation/triggering of SL-PRS transmission(s) - Opt. 1: through higher layers of the transmitting UE - Opt. 2: by another SL UE via PHY or higher layer signaling - Note: This does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request for initiating one or more SL-PRS transmission(s) |
| LGE | We agree with Intel that more clarification is needed for the proposal. Does triggering mean a resource selection triggering to PHY layer from MAC layer? Or does triggering mean to request for UE to transmit SL PRS (in this case MAC receives that triggering after all, and MAC triggers PHY to select resources)? Does activation mean the start of SL PRS transmission based on the already selected resources? As it’s an issue of not only Scheme 2 but also Scheme 1, we suggest to focus on triggering first. If the triggering means the red part above, we think other UE can request a UE to transmit SL PRS. For example, a target UE can trigger SL PRS transmission of an anchor UE based on the 1st SL PRS in SL RTT, or RX UE can request TX UE to retransmit SL PRS based on the status of SL PRS reception (on-demand SL PRS). It could also be applied to Scheme 1. We suggest to add this option as follows. ##### [MEDIUM] Feature Lead Proposal 5.2.2.B-v0’ - At least for For scheme 2, a transmitting UE may receive an activation/triggering of SL-PRS transmission(s) - Opt. 1: through higher layers of the UE - Opt. 2: through other UE’s request - FFS details - Note: This does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request for initiating one or more SL-PRS transmission(s) |
| OPPO | With regard to activation/triggering of SL-PRS transmission, in our understanding it is independent of resource selection scheme, Opt.1 can be applied in both Scheme 1 and Scheme 2. |
| InterDigital | We agree with the proposal. |
| Qualcomm | We generally agree but would like to update the note and replace “request” with “trigger” to also include the case of internal triggering. - For scheme 2, a transmitting UE may receive an activation/triggering of SL-PRS transmission(s) - Opt. 1: through higher layers - Note: This does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request trigger for initiating one or more SL-PRS transmission(s) |
| Philips | We agree with LGE |
| Nokia, NSB | OK and prefer QC’s revision |

[CLOSED] Feature Lead Proposal 5.2.2.B-v1

  - At least for For scheme 2, a transmitting UE may receive an activation/triggering of SL-PRS transmission(s)

      - Opt. 1: through higher layers of the UE

      - Opt. 2: through other UE’s request

        - FFS details

      - Note: This does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request trigger for initiating one or more SL-PRS transmission(s)

Companies views

| FL | Please comment if the proposal is OK for agreement without downselection this meeting. |
| --- | --- |
| LGE | Note is only relevant to opt. 1. So it needs to be moved under sub-bullet of opt. 1. We’re fine with the following modification. We support both options. ##### [MEDIUM] Feature Lead Proposal 5.2.2.B-v1 - At least for For scheme 2, a transmitting UE may receive an activation/triggering of SL-PRS transmission(s) - Opt. 1: through higher layers of the UE - Note: This does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request trigger for initiating one or more SL-PRS transmission(s) - Opt. 2: through other UE’s request - FFS details |
| CATT | It seems that physical layer signaling also can be used to activate/trigger SL-PRS transmission(s). Hence, it is not clear for us why only mention higher layer signaling in this proposal. |
| MTK | If the higher layer message is from other UEs, using request seems better If it is a UE’s higher layer to do something, using trigger seems better Then if the note is an independent bullet, we suggest to use “higher-layer trigger/request” and it seems the note could be applicable to both options |
| Qualcomm | We are generally ok with the proposal without downselection this meeting. However, the proposal seems to imply that both options are supported without the possibility of downselection and update is needed. |
| InterDigital | We are fine with the proposal. |
| Intel | We prefer not to include Option 2. It is not clear to us the feature of UEs scheduling other UEs for SL-PRS transmissions. Since the “At least” was added to the proposal this can be studied at a later stage. |
| CMCC | OK |
| Locaila | We are OK with the proposal. |
| Huawei, HiSilicon | We are fine with the proposal. And we think at this stage, further study is needed, not to downselect either one of the options. |
| OPPO | The version from LG is fine with us. |
| Samsung | It is not clear why ‘at least’ is added in the main bullet. Also not clear why activation was deleted. |
| DCM | OK, and supporting both would be possible. It should be clarified. |
| Lenovo | Supprt LG’s revised update on the note |
| Xiaomi | This seems apply only to aperiodic transmission as in proposal 4.1.6-v1. We are fine with the proposal. |
| Nokia, NSB | OK |

To CATT: Physical layer is included in Option 2. Do you mean physical layer from the same UE?

##### [MEDIUM] Feature Lead Proposal 5.2.2.B-v2

  - At least For For scheme 2, a transmitting UE may receive an activation/triggering higher-layer trigger/activation/request of SL-PRS transmission(s)

      - Opt. 1: through higher layers of the UE

        - Note: This does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request trigger for initiating one or more SL-PRS transmission(s)

      - Opt. 2: through other UE’s request

  - FFS details

##### Companies views

| LGE | First of all, the proposed operation should be for study rather than for normative one as we have a proposal to study a higher layer triggering in Proposal 4.1.6-v2. Second, we don’t restrict the trigger/activation/request in the main sentence into a higher layer signaling. We already have a higher layer signaling in Opt. 1. Therefore it would be better to more generalize the mean of trigger/activation/request in the main sentence. It would open the possibility for Opt. 2 either through lower layer or higher layer signaling. ##### [MEDIUM] Feature Lead Proposal 5.2.2.B-v2 - At least For For scheme 2, study the case where a transmitting UE may receive an activation/triggering higher-layer trigger/activation/request of SL-PRS transmission(s) - Opt. 1: through higher layers of the UE - Note: This does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request trigger for initiating one or more SL-PRS transmission(s) - Opt. 2: through other UE’s request in higher or lower layer signaling - FFS details |
| --- | --- |
| MTK | 1, want to understand the Note under Opt. 1 is only applicable to option 1? |
| ZTE | OK. There are two “For” in the main bullet. |
| Intel | These two options overlap, in fact Option 2 requires Option 1 as in our understanding after a request is received the transmitting UE’s higher layer would trigger Option 1. |
| FL Update | The note moved below Option 1 because Option1 says “higher layer”. Option 2 has everything open. Let me adjust it as follows: ##### [MEDIUM] Feature Lead Proposal 5.2.2.B-v3 - At least For For scheme 2, study the case(s) where a transmitting UE may receive an activation/triggering higher-layer trigger/activation/request of SL-PRS transmission(s) - Opt. 1: through higher layers of the UE - Opt. 2: through other UE’s request in higher or lower layer signaling - Note: Forr either option, higher layer signaling does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request trigger for initiating one or more SL-PRS transmission(s) - FFS details |
| InterDigital | We support the latest version of the FL proposal. |
| Qualcomm | We do not agree with lower layer requests. Initiating a positioning operation requires higher layer involvement to ensure security and QoS and is not in the domain of lower layers. While we don’t think that distinguishing the cause of a trigger is needed by the PHY layer, we can accept this modified proposal below. ##### [MEDIUM] Feature Lead Proposal 5.2.2.B-v3 - At least For For scheme 2, study the case(s) where a transmitting UE may receive an activation/triggering higher-layer trigger/activation/request of SL-PRS transmission(s) - Opt. 1: through higher layers of the UE - Opt. 2: through other UE’s request in higher or lower layer signaling - Note: Forr either option, higher layer signaling does not mean that the exact time/frequency resources for SL-PRS are chosen by higher layers. It means that a transmit UE may receive a higher-layer request trigger for initiating one or more SL-PRS transmission(s) - FFS details |
| CATT | To FL: we are fine with physical layer signaling is included in Option 2. Physical layer signaling should come from other UE’s request, such as IUC signaling based on PSFCH. We don’t think only high-layer signaling can be involved, physical layer signaling should also be considered to extend the flexibility of triggering of SL-PRS. We can live the the updated proposal from FL. |
| CEWiT | Fine with updated proposal from FL. For Opt2, tend to agree with CATT comment. |
| OPPO | Support the updated version from FL above, lower layer signaling should in Opt. 2 should be kept for further study. As to Intel and QC’s comments, our understanding is that the proposl is NOT from PHY layer (of transmitting UE) perspective, but from the entire transmitting UE perspective. |
| vivo | We support the latest version of the FL proposal |
| Samsung | OK for FL’s updates |
| NEC | We are fine with the updated version from FL. |
| Apple | Support the updated FL proposal. Note: there are 2 “for”s at the start of the proposal - Update to “transmitting as shown below: - Add: Trigger/ activation/request It means that a transmitting UE may receive a higher-layer request trigger for initiating one or more SL-PRS transmission(s) |
| LGE | Thanks for the update. We’re fine with the updated FL proposal. |
| Spreadtrum | We are fine with FL’s updates. |

[CLOSED] Feature Lead Proposal 5.2.2.C-v0

For Scheme 2, at least for the dedicated resource pool for positioning, with regards to Inter-UE coordination for SL-PRS,

- Option 1: study Rel-17 Scheme 1 IUC procedure as a starting point

- Option 2: study Rel-17 Scheme 2  IUC procedure as a starting point

- Option 3: study Rel-17 Scheme 1 or 2  IUC procedure as a starting point

- Option 4: Inter-UE coordination procedures can be deprioritized

Companies views

| ZTE | Option3 |
| --- | --- |
| CMCC | We propose to add another option: For Scheme 2, at least for the dedicated resource pool for positioning, with regards to Inter-UE coordination for SL-PRS, - Option 1: study Rel-17 Scheme 1 IUC procedure as a starting point - Option 2: study Rel-17 Scheme 2 IUC procedure as a starting point - Option 3: study Rel-17 Scheme 1 or 2 IUC procedure as a starting point - Option 4: Inter-UE coordination procedures can be deprioritized - Option 5: study inter-UE coordination mechanisms extending Rel-17 Schemes 1/2. We propose to extend the scope of IUC procedure for SL-PRS, i.e, the resource can be allocated by a group header to avoid RE-level sensing/resource selection procedure, and mitigate the IBE issue. |
| Huawei, HiSilicon | Option 1 is sufficient, as PSFCH may not be available in the dedicated pool but rather in the communication pool, PSFCH is already used for communication IUC scheme 2. |
| Lenovo | Support Options 1-3 since it’s still in study phase. We could also study an additional Option, where the Intiator UEs may assist in providing scheduling information of other (Anchor) UEs for the purposes of SL-PRS transmissions in a coordinated manner, especially within a SL positioning group. We wonder if this aspect is still in the scope of coordinated UE procedures for SL-PRS transmission. |
| Spreadtrum | We prefer Option 3. |
| Apple | Okay with proposal. S;upport 1- 3 |
| Futurewei | OK Prefer Option 3. |
| Intel | We prefer to study Option 3. |
| CATT | We prefer Option 3. Both of IUC scheme 1 and IUC scheme 2 should be studied. |
| NEC | Supporth the proposal and prefer Option 1-3. |
| DCM | We prefer Option 4. Workload should be considered. |
| Samsung | We prefer to deffer this issue until we make progress on more essential features. |
| LGE | Option 3 is not clear. Does it mean either option or both option? We prefer both options to be studied, as in option 3 below. ##### [LOW] Feature Lead Proposal 5.2.2.C-v0’ For Scheme 2, at least for the dedicated resource pool for positioning, with regards to Inter-UE coordination for SL-PRS, - Option 1: study Rel-17 Scheme 1 IUC procedure as a starting point - Option 2: study Rel-17 Scheme 2 IUC procedure as a starting point - Option 3: study Rel-17 Scheme 1 or and 2 IUC procedure as a starting point Option 4: Inter-UE coordination procedures can be deprioritized |
| Sharp | OK with the proposal. |
| OPPO | Option 1, Scheme 2 IUC cannot be supported if there is no PSFCH in the resource pool, or resource reservation is not supported for SL PRS. |
| InterDigital | Support Option 3. We think both IUC scheme 1 and scheme 2 should be studied at this stage of discussion. |
| CEWiT | We prefer to have optionto extend/enhance Rel 17 ICU procedure |
| Qualcomm | We prefer to revisit this issue once more progress is made in other proposals. |
| Philips | We prefer options 1-3. |
| Nokia, NSB | OK. We prefer Option 3. Regarding Scheme 2 IUC, do companies expect that PSFCH resources are configured in the dedicated resource pool? |

##### [LOW] Feature Lead Proposal 5.2.2.C-v1

For Scheme 2, at least for the dedicated resource pool for positioning, with regards to Inter-UE coordination for SL-PRS,

- Option 1: study Rel-17 Scheme 1 IUC procedure as a starting point

- Option 2: study Rel-17 Scheme 2  IUC procedure as a starting point

- Option 3: study Rel-17 Scheme 1 or and 2  IUC procedure as a starting point

- Option 4: Inter-UE coordination procedures can be deprioritized

Note: For either option, companies are encouraged to describe what enhancements may be needed.

##### Companies views

| FL | Please comment if the proposal is OK for agreement without downselection this meeting. |
| --- | --- |
| LGE | The proposal itself without downselection is fine for us. We support both IUC scheme 1 and 2. |
| InterDigital | We support Option 3 in the proposal to study both IUC Scheme 1 and Scheme 2. |
| CATT | OK with the proposal, we prefer Option 3, i.e., both the scheme1 and scheme 2 should be supported. |
| CMCC | We still think the IUC procedure can be extended for some specific case, e.g., RTT type method or there is a group header, where UE can select resource for another UE’s transmission. |
| Samsung | We prefer to defer this issue until we make progress on more essential features. Current proposal does not provide meaningful information. |
| DCM | Accept |
| Lenovo | Support Options 1, 2 and 3 |
| Nokia, NSB | OK. We prefer Option-3. Regarding Scheme 2 IUC, do companies expect that PSFCH resources are configured in the dedicated resource pool? |
| ZTE | If we are going to downselect, we prefer option 3. |
| Qualcomm | The proposal does not it make it clear that there will be downselection in a future meeting. |
| CEWiT | Fine with proposal. |
| Apple | Fine with proposal. |
| Spreadtrum | Support |

[CLOSED] Feature Lead Proposal 5.2.2.D-v0

For Scheme 2, at least for the dedicated resource pool for positioning, with regards to congestion control mechanisms for SL-PRS

- Study congestion metrics definition for SL-PRS (e.g. CBR and CR)

- Study which parameters  of a SL-PRS configuration could be impacted by the congestion control mechanism

Companies views

| CMCC | Agree. |
| --- | --- |
| Huawei, HiSilicon | OK |
| Lenovo | Support |
| Apple | Okay |
| Futurewei | OK |
| Intel | Support |
| CATT | In our point of view, congestion control mechanism for SL-PRS should be studied in order to maintain the congestion of SL-PRS within a reasonable level. However, since the definitions of CBR and CR are closely related to resource occupation, we prefer congestion control mechanism for SL-PRS should be studied after the resource allocation mechanism of SL-PRS was determined. |
| DCM | OK |
| Samsung | O.K |
| LGE | Support in general |
| OPPO | OK |
| Inter | Support |
| CEWiT | Support |
| Qualcomm | Support |
| Philips | Support |
| Nokia, NSB | OK |

##### [LOW] Feature Lead Proposal 5.2.2.D-v0

For Scheme 2, at least for the dedicated resource pool for positioning, with regards to congestion control mechanisms for SL-PRS

- Study congestion metrics definition for SL-PRS (e.g. CBR and CR)

- Study which parameters  of a SL-PRS configuration could be impacted by the congestion control mechanism

- Note: Companies are encouraged to study the above in conjunction to the resource allocation mechanisms for SL-PRS.

##### Companies views

| LGE | We are fine with the proposal in general. One comment is that the existing SL congenstion control mechanism is the starting point. If possible, the existing SL congenstion control mechanism should be reused as much as possible. We want to add a note. ##### [LOW] Feature Lead Proposal 5.2.2.D-v0 For Scheme 2, at least for the dedicated resource pool for positioning, with regards to congestion control mechanisms for SL-PRS - Study congestion metrics definition for SL-PRS (e.g. CBR and CR) - Study which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism - Note: Companies are encouraged to study the above in conjunction to the resource allocation mechanisms for SL-PRS. - Note: Existing SL congestion mechanism is a starting point, and should be resused as much as possible. |
| --- | --- |
| ZTE | Basically ok but prefer to delay the study of congestion control. Right now we should focus on more general designs like SL-PRS configuration or resource allocation. |
| InterDigital | We support the proposal. |
| CATT | We admit QoS management is an important issue for SL-PRS. But it is too earlier to discss this issue. Congestion control mechanism for SL-PRS should be studied in order to maintain the congestion of SL-PRS within a reasonable level. However, since the definitions of CBR and CR are closely related to resource occupation, we prefer congestion control mechanism for SL-PRS should be studied after the resource allocation mechanism of SL-PRS was determined. |
| CMCC | Support |
| Locaila | We are fine with the proposal. |
| Samsug | support |
| DCM | OK |
| Lenovo | Support |
| Nokia, NSB | OK |
| Sharp | Support |
| Qualcomm | We are ok with the proposal |
| CEWiT | Fine with proposal. |
| Apple | Fine with proposal. |

# Positioning Measurements Report

The following agreement was reached the previous meeting:

Agreement With regards to the Sidelink Positioning measurement report, - Study the contents of the measurement report (e.g. time stamp(s), quality metric(s), ID(s), angular/timing/power measurements, etc) - Study the time domain behavior of the measurement report (e.g. one-shot, triggered, aperiodic, semi-persistent, periodic) - FFS whether the Sidelink Positioning measurement can be a high-layer report and/or a lower layer report. Agreement For the content of the sidelink positioning measurement report, potential elements may include at least the following: - One or more sidelink positioning measurement(s) - Timestamp(s) associated with a sidelink positioning measurement - Quality metric(s) associated with a sidelink positioning measurement - Identification Information for a sidelink positioning measurement - FFS any detail for the above

#### 6.1 Contents of the positioning measurement report

The following proposals were found with regards to the contents of a positioning measurement report:

| Futurewei | Observation 2: The quality indication of the anchor node’s coordinates or its synchronization accuracy may be used by the target node for the anchor node(s) selection. Proposal 1: Identify and study quality metrics usage for the anchor nodes selection. Observation 5: A hard value of the LOS/NLOS indication allows the selection of most reliable measurements for position estimation, while the soft value of this indicator may allow an estimation of the confidence/precision of the location determination. Observation 6: Provisioning of additional quality metrics characteristics for the anchor node helps to select the best measurements and to evaluate the precision of the location determination. Proposal 4: SL positioning should support reporting of quality metric for the LoS/NLoS of SL-PRS measurements and quality metrics of anchor characteristics. |
| --- | --- |
| OPPO | - Measurements defined in NR-Uu positioning for different positioning methods can be adapted for SL positioning. |
| CATT | Proposal 25: Support at least the following measurements for sidelink positioning: - SL Rx-Tx time difference - SL AOA - SL RSRP/RSRPP Proposal 28: For SL positioning, the reporting granularity for SL Rx-Tx time difference and SL RSTD should be configurable. And fixed reporting granularity is applied to SL AOA. Proposal 29: Periodic, semi-persistent and aperiodic measurement report should be supported in SL positioning. And the sidelink positioning measurement should be a high-layer report. |
| Intel | - Add the following information to the list of potential measurement report fields: - UE location information, that may include latitude/longitude/altitude information, or location coordinates, together with any qualifying information about the location, e.g., associated uncertainty, time-stamp, etc. |
| ZTE | Proposal 4: Do not support separate SL positioning methods, the following potential elements for sidelink positioning measurement report can be considered: - One or more sidelink positioning measurement(s): UE Rx-Tx time difference, SL-PRS RSRP, SL-PRS RSRPP, SL-AoA, SL-PRS RSTD - Time stamps at which the measurements are performed - Quality metric(s): timing quality or measurement quality - Identification Information - UE information: UE’s ID, PC5 unicast link identifier, V2X service identifier, UE’s application layer ID, destination layer-2 ID, source layer-2 ID, Source user info, target user info, etc. - SL-PRS information: SL-PRS resource ID and/or SL-PRS resource set ID |
| LGE | Proposal 9: For double-side RTT, the information about the unique identification of the measured SL PRS resource (e.g. SL PRS resource (set) ID combined with SL PRS resource repetition index) can be included in the measurement report. |
| Interdigital | Proposal 21: Support at least UE RX-TX time difference and SL-PRS-RSRP measurement for SL positioning. |
| Samsung | Proposal 17: Consider for UEs to indicate a reliability of known location information to the target UE for SL positioning. |
| NTT DOCOMO | Proposal 8: - Reported information includes RX-TX time difference, RSTD, AoA, received RSRP, transmission power of SL-PRS, absolute location information. |
| Qualcomm | Proposal 18: Support UE Rx-Tx time difference measurements derived on SL-PRS to enable SL-RTT ranging. Proposal 19: Support angle of arrival (both azimuth and zenith angles) measurements to enable SL-AoA positioning. Proposal 20: If SL-TDoA and SL-AoD are supported, introduce RSTD, RSRP, and RSRPP measurements. Proposal 22: Study information to be included in a sidelink positioning measurement report, in addition to the basic UE measurements, including at least the following: time stamp(s), quality metric(s), UE-ID(s), SL-PRS resource ID(s). |

From the proposals above, it seems there are a few more potential elements that companies are considering for the content of the SL positioning measurement report, in addition to what was added in the previous related agreement. Note the elements below are in addition to what we agreed as potential elements to be looked into further.

[CLOSED] Feature Lead Proposal 6.1.A-v0

For the content of the sidelink positioning measurement report, additional potential elements may include at least the following:

- UE location information (e.g. latitude/longitude/altitude, or location coordinates), and qualifying information about location, e.g., associated uncertainty, time-stamp, etc.

- LOS/NLOS indicators

- Transmission power of SL-PRS

Companies views

| Futurewei | Support first two bullets. Open to discuss the last one. |
| --- | --- |
| CATT | We support also to study the the reporting granularity for measurement. So our preferred revision as follows, ##### [LOW] Feature Lead Proposal 6.1.A-v0 For the content of the sidelink positioning measurement report, additional potential elements may include at least the following: - UE location information (e.g. latitude/longitude/altitude, or location coordinates), and qualifying information about location, e.g., associated uncertainty, time-stamp, etc. - LOS/NLOS indicators - Transmission power of SL-PRS - Configurable or fixed reporting granularity for SL positioning measurment |
| Samsung | Support the first sub-bullet. |
| InterDigital | We support the proposal regarding the first two bullets. However, we suggest to move SL-PRS transmit power into a FFS, as in our view it is pre-mature to include SL-PRS power in reporing. We suggest to discuss SL-PRS power control/setting first to evaluate the purpose/benefit to include it in SL-PRS measurement reporting. For the content of the sidelink positioning measurement report, additional potential elements may include at least the following: - UE location information (e.g. latitude/longitude/altitude, or location coordinates), and qualifying information about location, e.g., associated uncertainty, time-stamp, etc. - LOS/NLOS indicators - FFS: Transmission power of SL-PRS |
| Nokia, NSB | OK, with FFS for SL-PRS TX power as proposed by InterDigital |

##### [LOW] Feature Lead Proposal 6.1.A-v1

For the content of the sidelink positioning measurement report, additional potential elements may include at least the following:

- UE location information (e.g. latitude/longitude/altitude, or location coordinates), and qualifying information about location, e.g., associated uncertainty, time-stamp, etc.

- LOS/NLOS indicators

- FFS: Transmission power of SL-PRS

- Configurable or fixed reporting granularity for SL positioning measurment

##### Companies views

| InterDigital | We are fine with the proposal |
| --- | --- |
| CATT | Support the proposal. |
| Huawei, HiSilicon | We do not think UE location coordinates should be included in the “measurement report”. |
| Samsung | OK |
| Lenovo | Support, Perhaps a slight re-wording on the 1st bullet, we understand that the UE may provide location information via “Provide Location Information” type message when requested by another entity. |
| Nokia, NSB | OK |

Furthermore, it seems a couple of companies are interested in providing more details on the “Identification Information” that may be included in the measurement report. So, based on that i am suggesting the companies to study further both UE-related identification and SL-PRS related identification information.

[CLOSED] Feature Lead Proposal 6.1.B-v0

With regards to the content of the sidelink positioning measurement report, the “Identification Information for a sidelink positioning measurement” of the previous agreement, may correspond to one or more of the following:

- One or more UE-related identification information

- One or more SL-PRS related identification information

Companies views

| ZTE | ok |
| --- | --- |
| Huawei, HiSilicon | In general the UE ID is included in the generic PC5 communication, and we do not see justification to further indicate the UE ID in the measurement report. Is it about conveying PC5 measurement towards a UE in a report to LMF? Can proponent clarify? |
| CATT | OK to further clarifty the meaning of “Identification Information for a sidelink positioning measurement” in the previous agreement, e.g., source ID, destination ID, SL-PRS resource ID, etc. |
| Samsung | OK |
| InterDigital | We support the proposal. |
| Locaila | Support |
| OPPO | OK |

#### 6.2 Signaling Discussion for the Positioning Measurement report

The following proposals were found in the contributions:

| Huawei, HiSilicon | - Proposal 6: SL positioning measurement shall be treated as regular communication data, and be transparent to physical layer and transmitted over the PC5 communication resources. |
| --- | --- |
| China Telecom | Proposal 9: We prefer the time domain behavior of the Sidelink positioning measurement report to be semi-persistent and triggered. the contents of the measurement report can refer to the messages transferred in existed solutions with some changes on the transferred entities. |
| vivo | SL-PRS RSRP is needed for SL-PRS measurement and reporting. MAC CE or high layer signaling (e.g., RSPP) can be considered to carry SL-PRS measurement report as SL CSI report. SL-PRS measurement report is not transmitted in the dedicated SL-PRS resource pool. |
| Intel | - SL positioning measurement reports can be transmitted on using the PC5 or the Uu interface. |
| ZTE | Proposal 3: Support SL positioning measurement request/report as high-layer signaling. |
| LGE | Proposal 14: Measurement report between UEs can be transmitted by the existing higher layer signaling (e.g. PC5-RRC, PC5-S or MAC CE). |
| Interdigital | Proposal 22: A target and/or anchor UE reports SL positioning measurement to gNB/LMF in higher layer signalling in in-coverage scenario. Proposal 23: A target UE receives positioning measurement reporting from anchor UE(s) in out-of-coverage scenario. Proposal 24: Study resource allocation for SL-PRS measurement reporting. |
| Samsung | Proposal 16: Study the following as the candidate destination for SL measurement reporting - Another UE (FFS by SA2/RAN2 for a UE as a location server) - gNB - LMF (FFS by RAN2 whether it is a feasible option or not) |
| NTT DOCOMO | - Define latency bound of SL positioning measurement result. |
| Qualcomm | Proposal 21: From RAN1 perspective, the SL positioning report is a high-layer report. Up to RAN2 & SA2 working groups to identify and design the necessary architecture, signaling and protocols. Proposal 23: Support higher-layer one-shot measurement reporting. Proposal 24: Support higher-layer periodic measurement reporting. Proposal 25: SL positioning measurement reports are transmitted as any other sidelink data and no special resource allocation mechanism is introduced for them. |
| CEWiT | Proposal 15: For the Sidelink Positioning measurement report, use a dedicated sidelink positioning resource pool configured for sidelink PRS. Proposal 16: Sidelink positioning measurement report should be carried over PSSCH or PSFCH. Proposal 17: For sidelink Positioning measurement report configurations both the higher layer and lower layer signalling should be used. |

The following agreements were made in RAN2’s previous meeting:

Protocol:

- Introduce a new protocol for sidelink positioning procedures between UEs (name FFS, e.g., RSPP, SLPP).  FFS where it is specified.  The new protocol is a separate ASN.1 module from LPP (this does not necessarily imply whether it is included in 37.355).

- Study the potential impact to LPP for support of sidelink positioning procedures between UE and LMF.  FFS how much impact (if any), e.g., only to carry the new protocol, and if the PC5-only and hybrid PC5+Uu cases are the same or different

- RAN2 wait for SA2 on the triggering of the positioning procedures from upper layers

If there is a new protocol between UEs, it is natural to use that for the purpose of report, at least as a RAN1 assumption, while still considering, the option of a low-layer reporting if consensus is reached.

##### [LOW] Feature Lead Proposal 6.2-v0

With regards to the sidelink positioning measurement report,

- RAN1 assumes as a starting point that higher layer signaling for the positioning measurement report shall be used.

- FFS: Whether other signaling (e.g. low-layer signaling) could also be applicable.

##### Companies views

| CATT | Support. We prefer the sidelink positioning measurement report should be higher layer signaling, instead of low-layer singalling |
| --- | --- |
| Samsung | OK |
| InterDigital | We agree with the proposal. |
| Qualcomm | OK |
| Nokia, NSB | OK |
| Locaila | Support |
| DCM | We prefer to use PHY layer signalig so that measurement report can be performed in dedicated resource pool. |

# SL PHY-layer Positioning Procedures

## 7.1 SL-PRS Power control

The following agreement was reached previous meeting:

Agreement Study power control mechanisms for SL-PRS transmission, including whether it is necessary.

The following proposals were found in the contributions:

| Huawei, HiSilicon | Proposal 22: Support to design SL-PRS power control scheme based on DL pathloss and SL pathloss. Proposal 23: Support to use SL PRS as the pathloss reference signal. |
| --- | --- |
| vivo | - Open-loop power control scheme should be supported for SL-PRS. - The SL pathloss and the DL pathloss can be considered in the power control of the SL-PRS. |
| OPPO | - In sidelink positioning, SL PRS is at least subject to DL pathloss based power control. - For the proactively transmitted SL PRS, the transmission power should be upper bounded by the target coverage, and for the reactively transmitted SL PRS, SL pathloss based power control should also be applied. |
| CATT | Proposal 30: SL-PRS power control should be introduced in Rel-18. - The minimum pathloss between DL pathloss and SL pathloss can be adopted as the compensated pathloss for the SL-PRS power control. |
| Intel | - For SL-PRS transmissions in a shared resource pool, the power control mechanism of the PSSCH transmission associated with the SL-PRS transmission is used. - For SL-PRS transmissions in a dedicated resource pool, the power control mechanism follows the open loop power control principles of the power control for SL communication. |
| ZTE | Proposal 24: Support introducing the close loop power control in SL-PRS power control formula. |
| Lenovo | Proposal 18: RAN1 to further study SL power control (e.g., based on open-loop methods) and interference mitigation mechanisms (e.g., muting) to improve SL PRS detectability/hearability between anchor nodes and the target-UE. |
| LGE | Proposal 32: Further discussions are needed whether DL pathloss or SL pathloss or both are needed to adapt the transmission power of the SL PRS. |
| Interdigital | Proposal 19: Study OLPC for a SL-PRS transmission multiplexed with a PSSCH/PSCCH transmission, with R16/R17 PSSCH/PSCCH OLPC as a starting point. Proposal 20: Study unicast-based OLPC mechanism for stand-alone SL-PRS transmission. |
| Apple | Proposal 16:For the total sidelink Transmit Power if the SL-PRS is transmitted in the same slot as the SL communication signals the total sidelink transmit power is the same as in the symbols used for PSxCH transmissions in a slot (irrespective of the BW of the SL signal). If SL-PRS is in its own dedicated slot, the total power may be different. This may require a separate set of power control parameters for the SL-PRS. |
| Samsung | Proposal 18: Consider the following aspects for SL PRS power control. - SL PRS transmission bandwidth and multiplexing with other SL channels/signals - Whether only OLPC is applied - Whether only DL pathloss is applied or SL pathloss is applied additionally - Whether PMAX,CBR is applied for PSSCH and/or SL PRS |
| Qualcomm | Proposal 26: Study SL-pathloss based OLPC for SL-PRS transmission. Proposal 27: Support DL-pathloss based OLPC for SL-PRS transmission for in-coverage UEs. |

##### [LOW] Feature Lead Proposal 7.1-v0

For SL-PRS transmissions in a dedicated resource pool, with regards to the power control for SL-PRS at least Open Loop PC should be introduced.

- FFS: Closed loop PC, which reference signal to be used as pathloss reference signal, details

##### Companies views

| Apple | Support |
| --- | --- |
| CATT | OK with the proposal. |
| Samsung | Prefer to support open loop PC only. We suggest to add ‘as least’ as below For SL-PRS transmissions at least in a dedicated resource pool, with regards to the power control for SL-PRS at least Open Loop PC should be introduced. |
| InterDigital | We support the proposal. |
| Qualcomm | We agree with Samsung’s proposed changes. We propose removing the FFS since closed-loop power control does not exist in sidelink. |
| Nokia, NSB | OK |
| ZTE | Prefer to keeo close loop PC in the proposal. |
| Locaila | Support |
| OPPO | Support. |
| Sharp | OK |

## 7.2  SL-PRS Beam management

| Intel | - Deprioritize SL positioning discussions related to FR2 beam management till this feature is defined for SL communication. |
| --- | --- |
| ZTE | Proposal 26: Deprioritize Rel-18 NR sidelink positioning in FR2. |

## 7.3 Miscellanous proposal on PHY-layer Procedures

| Intel | - Study the definition of measurement assistance information for SL positioning. |
| --- | --- |

# Joint SL and Uu Positioning

Based on the submitted contributions, the following statements/proposals are identified to be related to the sub-topc of Joint SL & Uu Positioning:

| Spreadtrum | Proposal 14: The anchor UE needs to send indication information to the target UE to indicate whether it can act as a positioning node. |
| --- | --- |
| Sony | Proposal 12: Support hybrid positioning where the UE receives reference signal for positioning from both direct-link (Uu) and sidelink (PC5) and jointly utilize for positioning estimation purpose. |
| ZTE | Proposal 5: RAN1 confirm RAN2’s agreement that: - All three in-coverage, partial coverage and out-of-coverage scenarios shall be supported for SL positioning. - Support joint SL and Uu positioning method for in-coverage scenarios and partial-coverage scenarios. |
| CMCC | Proposal 2: Joint scheme b/w NR Uu and SL positioning can be considered to facilitate the positioning accuracy performance. |
| Lenovo | Proposal 14: RAN1 to support hybrid positioning under the following models: - Hybrid Positioning Model A - Hybrid positioning using hybrid interfaces including the support of Uu and SL measurements for SL position calculation. - Hybrid Positioning Model B - Hybrid positioning using hybrid technologies including the support of RAT-dependent and RAT-independent methods for SL position calculation. Proposal 24: RAN1 to discuss the relationship between Uu PRS and SL PRS configuration and associated measurements for hybrid positioning model A (using both Uu and SL interfaces). FFS whether such measurements are performed in a joint manner or separately measured. |
| Fraunhofer | Proposal 1: At least for in-coverage scenarios, in addition to standalone SL positioning methods, study hybrid positioning methods wherein one or more of the UE(s) perform SL measurements and UE position/ranging is estimated using measurements derived on both SL and Uu positioning. Uu positioning corresponds to RAT dependent methods. |
| Qualcomm | Proposal 2: RAN1 to define using a combination of Uu- and PC5-based positioning, as a positioning mode where at least one of the UEs performs SL positioning/ranging measurements and location/range is computed using measurements derived on both SL and Uu. |
| Ericsson | - To enable the possibility for combining SL based positioning measurements and uu based positioning measurements to improve positioning performance, consider the possibility for SL UEs to report SL positioning measurements to the network when in coverage. |

RAN2 #119-e reached the following agreements on sidelink positioning architecture, supported scenarios and protocol [2]:

Agreements on supported scenarios, architecture: - Confirm that for sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported. FFS if partial coverage case assumes anything about which UEs are in coverage. - Study the architecture and signaling procedures to enable at least the following two operation scenarios: - Operation Scenario 1: PC5-only-based positioning. - Operation Scenario 2: Combination of Uu- and PC5-based positioning. - RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server. FFS from RAN2 perspective if there are cases without a UE in the location server role.

FL Note: It seems it has already been agreed that the study includes PC5-only  and Combination Uu/PC5 Positioning, and also all the coverage scenarios. It is unclear whether a RAN1 proposal at this point on this will be useful given the number of RAN1-centric proposals that we have for discussion.

# Terminology Alignment

Agreement For the purpose of RAN1 discussion during this study item, at least the following terminology is used: - Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface). - Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information. - Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE. - Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes. - SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity. - Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion. Agreement For the purpose of RAN1 discussion during this study item, at least the following terminology is used: - Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. - FFS: clarification of the knowledge of the location of the anchor UE

| CATT | Proposal 33: The location coordinates of anchor UE should be known at least for SL absolute positioning. |
| --- | --- |
| Lenovo | Proposal 1: Request SA2 to adopt the terminology of the “Anchor UE” in their discussions to avoid fragmented definitions of the same concept of “Reference UE” and “Located UE”. Proposal 2: RAN1 to consider the following additional terminology during the study: - Initiator UE initiates a SL positioning/ranging session, e.g. anchor UE, target-UE, assistance UE - Responder UE responds to a SL positioning/ranging session from an initiator UE, , e.g. anchor UE, target-UE, assistance UE. |
| Interdigital | Proposal 1: Consider the following terminology for the purpose of continued RAN1 discussion: - Reference anchor UE: An reference entity e.g., anchor UE, based on which a target UE’s relative position and/or ranging information is determined. - Sidelink positioning group: A group of UEs including a target UE and one or more anchor UEs, who participate in same positioning procedures. |
| CEWiT | Proposal 2: For the purpose of RAN1 discussion during this study item, at least the following terminologies are used: Assisting UE group: Assisting UE group is a group of UEs that lie in the neighborhood of the target UEs or are connected to target UEs and can be configured to transmit and/or receive the PRS to/from the target UE. Further, they share measurements to anchor UE or LMF. Reference UE: It is one of the assisting nodes participating in positioning the target UE, which is used as a reference to perform positioning measurements from other assisting UEs. Proposal 3: In the sidelink position, for partial and out-of-coverage scenarios, the assisting UE group should be provided by either the anchor node or the target node. |
| Ericsson | - An Anchor UE as defined in the agreement in RAN1#109-e does not need to have known location as such UEs can be supporting positioning of a target UE at least for relative positioning. |

In Monday’s GTW session the following was captured as open item for discussion:

Proposed addition to the definition of anchor UE:

An anchor UE could be UE-type RSU.

Also, there is an unresolved FFS with regards to the location knowledge of the anchor UE. My understanding is that it may or may not be known. For example, for a typical scenario of absolute positioning, the location may be known, e.g. RSUs transmitting SL-PRS towards a vehicle; in this case, the RSU location may be known. In some other scenarios, e.g., a UE trying to do relative positioning with regards to an anchor, the location of the anchor may not be known either (e.g. Public safety scenario where a first responder is trying to find out the relative position of a target UE).

Based on the above, i am suggesting the following proposal:

##### [LOW] Feature Lead Proposal 9.A-v0

Proposed Conclusion:

- An anchor UE could be UE-type RSU

##### [LOW] Feature Lead Proposal 9.B-v0

Clarify that the Location of the anchor UE may or may not be known.

##### Companies views

| Futurewei | Support Proposal 9.A-v0 |
| --- | --- |
| CATT | For Proposal 9.A-v0, we are OK. For Proposal 9.B-v0, we believe all company agree that for absolute positioning, the location coordinates of anchor UE should be known. So we prefer the updated proposal as follows, ##### [LOW] Feature Lead Proposal 9.B-v0 Clarify that the Location coordinates of the anchor UE may or may not should be known for absolute positioning . |
| Qualcomm | We are ok with 9.A-v0 |
| Nokia, NSB | OK |
| Locaila | Support |
| Samsung | OK with 9.A-v0 |

# SL Positioning Architecture And Signaling Discussion

A lot of companies provided proposals and discussions on the high-layer architecture, procedures, and System-level proposals; topics that are already being discussed in SA2 (TR 23.700-86) and RAN2. It may be more efficient to try to avoid repetition of the discussions unless it is considered necessary.

| Sony | Proposal 13: Consider supporting UE-types classification for V2X positioning (e.g., RSU, VRU, Car). Proposal 14: Consider adapting positioning procedure based on the region/zone of the UE. Proposal 15: Consider supporting positioning procedure with the assistance of another UE for the estimation of relative positioning and relative angle. |
| --- | --- |
| ZTE | Proposal 17: Sidelink related information transfer between LMF and gNB should be supported. |
| Lenovo | Proposal 3: Support the following 4 different models for SL positioning, which distinguish the configuration entity and positioning calculation entity for in-coverage, and partial and out-of-coverage scenarios: - SL Positioning Model 1a - UE-assisted positioning (network-configured) - SL Positioning Model 1b – UE-assisted positioning (UE-configured ) - SL Positioning Model 2a – UE-based positioning (UE-configured) - SL Positioning Model 2b - UE-based positioning (network-configured) Proposal 32: RAN1 to further discuss the benefits of PRUs and assistance UEs in SL positioning/ranging procedures. |
| LGE | Proposal 1: It is supported that UE initiates the SL positioning procedure, which includes the SL positioning group formation and SL PRS configuration. Proposal 2: It is supported that LMF/gNB initiates the SL positioning procedure, which includes SL positioning group configuration and SL PRS configuration. Proposal 3: SL positioning group is comprised of a target UE and anchor UE(s). SL PRS is transmitted and received within the SL positioning group. - Further studies are needed on which UE can initiate or join the SL positioning group, how to generate and accept the join request, how to leave or release the SL positioning group, etc. |
| Interdigital | Proposal 2: Study LMF’s role in SL positioning and its interaction with the gNB for in-coverage SL positioning. Proposal 10: Support both MO-LR and MT-LR for in-coverage SL positioning. Proposal 11: Study SL mechanisms to support MO-LR for out-of-coverage SL positioning. Proposal 12: Study discovery mechanisms for a target UE to identify one or more anchor UEs in a SL positioning group. Proposal 26: An anchor UE reports its location and associated uncertainty information to LMF (for in-coverage) and a target UE (for out-of-coverage). |
| Franhofer | Proposal 7: Study the coordination of SL-PRS and SRS resources for bands supporting intra-band con-current operation of Uu and PC5. |
| NTT DOCOMO | Proposal 10: - Send an LS to RAN2/RAN3/SA2 to ask whether PHY layer signaling for SL Positioning Client UE / SL Positioning Location Server UE is necessary or not. |
| Mediatek | Proposal 6-1: Introduce the server UE to align the positioning framework between being over the Uu interface and being over the PC5 interface Proposal 6-2: The server UE may play the role of receiving the measurement results for further processing Proposal 6-3: The server UE may assist the target UE to determine a group of anchor UEs to support the positioning of the target UE |
| CEWiT | Proposal 1: For sidelink positioning over PC5 link, a procedure should be defined for selecting assisting UE/s which will assist anchor UE in finding location of target UE. |

The following agreements were reached in RAN2 discussion with regards to these issues:

Agreements: - Confirm that for sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported. FFS if partial coverage case assumes anything about which UEs are in coverage. - Study the architecture and signaling procedures to enable at least the following two operation scenarios: - Operation Scenario 1: PC5-only-based positioning. - Operation Scenario 2: Combination of Uu- and PC5-based positioning. - RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server. FFS from RAN2 perspective if there are cases without a UE in the location server role. - Align with SA2/RAN1 on the terms for sidelink positioning, and introduce the following terms of UE role as the baseline for further discussion: - Target UE: UE to be positioned - Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. FFS: clarification of the knowledge of the anchor UE. - Additional roles can be considered.

FL Note: It is unclear whether a RAN1 proposal at this point on this will be useful given the number of RAN1-centric proposals that we have for discussion.

# Proposal for Online Discussion

This section will contain proposals for discussion during online time:

##### [ONLINE] Feature Lead Proposal 4.1.1-v0

For the sequence of the new reference signal for SL positioning/ranging, use

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

  - Support: ZTE, CMCC, LGE, vivo, Huawei, HiSilicon, Spreadtrum, Apple, Intel, CATT, DCM, Samsung, Sharp, OPPO, Interdigital, Xiaomi, Qualcomm, Philips (17)

  - Do not support: Fraunhofer, Sony (?)

  - Do not support but can compromise: Nokia

>> Technicaly we have two issues with Alt.1

- the PAPR (peak2averagepower ratio)

- we are unsure if the number of  PN sequenzes

>>>  We understand that the majeority prefers Alt.1, My assumption is that majority can compromise to Alt2 as well. No need to conclunde on this issue for this meeting. If we want o have progress on this issue we can set RAN1#111 for downselection.

##### [ONLINE] Feature Lead Proposal 4.1.4.A-v1

With regards to the configuration of SL-PRS:

  - A UE may receive at least part of SL-PRS related [configuration/parameters] from higher layers

    - FFS: which part(s) of the SL-PRS configuration; e.g. dedicated resource pool for positioning configuration, SL-PRS bandwidth configuration, number of SL-PRS symbols, periodicity, sequence-IDs, comb-size, repetitions, etc.

    - Note: This includes the (pre-)configuration case also.

##### [ONLINE] Feature Lead Proposal 4.1.7-v1

The following cast types of SL-PRS transmission should be introduced for SL positioning:

- Opt. 1: Unicast, Groupcast, Broadcast

  - ZTE, LGE, vivo, Spreadtrum, Apple, Futurewei, Intel, CATT, NEC, DCM, Sharp, OPPO, Interdigital, Xiaomi, CEWiT, Qualcomm, Philips, Nokia, NSB

- Opt. 2: At least unicast. FFS: Groupcast, Broadcast

  - CMCC, Huawei, HiSilicon

- Opt. 3: All three cast types (Unicast, Groupcast, Broadcast) can be studied

  - Samsung

##### [ONLINE] Feature Lead Proposal 5.1.1.E-v1

With regards to the relation of the dedicated resource pool for SL-PRS to

- the BWP of a SL communication resource pool:

  - Alt. 1: The dedicated SL-PRS resource pool is associated with the one SL BWP of the carrier

    - ZTE, CMCC, vivo, Huawei, HiSilicon, Apple, Futurewei, Intel, CATT, NEC, DCM, Samsung, LGE, OPPO, Interdigital, Qualcomm, Nokia, NSB (16)

  - Alt. 2: The dedicated SL-PRS resource pool can be associated with a BWP that can be the one SL BWP of the carrier or with a second additional BWP

    - Sony, Xiaomi (2)

- the carrier of a SL communication resource pool:

  - Alt. 1: The dedicated SL-PRS resource pool is on the carrier of the SL communication resource pool

    - ZTE, CMCC, vivo, Huawei, HiSilicon, Apple, Futurewei, Intel, CATT, NEC, DCM, Samsung, Interdigital, OPPO, Xiaomi (14)

  - Alt. 2: The dedicated SL-PRS resource pool can be on a carrier that can be the same or different  from the carrier of the SL communication resource pool

    - LGE, Sony, Qualcomm, Nokia, NSB (4)

##### [ONLINE] Feature Lead Proposal 4.1.2-v1

With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics:

- With regards to the value N (comb-type) and the number M of SL-PRS symbols within a slot excluding the symbol(s) used for AGC training / RxTx Turnaround:

  - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12}

  - FFS: the values considered as potential candidate values for M

  - [FFS1: Whether to consider N>12 as a potential candidate value(s)]

    - Keep FFS1: CATT, Fraunhofer, ZTE, Intel, Localia, Huawei, HiSilicon

    - Remove FFS1: vivo, LGE, Qualcomm, Interdigital, Samsung, Ericsson, Xiaomi, OPPO

  - [FFS2: whether to add in the end of the SL-PRS pattern an additional symbol with the same RE-offset as the first symbol]

    - Keep FFS2: CATT, Fraunhofer, Huawei, HiSilicon

    - Remove FFS2: vivo, LGE, ZTE, Qualcomm, Interdigital, Intel, Localia, Samsung, Ericsson, Xiaomi, OPPO, MTK

- The symbols of a SL-PRS resource within a slot can be

  - Alt. 1: consecutive symbols

  - [Alt. 2: consecutive or non-consecutive symbols]

    - Keep Alt2: CATT, Fraunhofer, NEC, DCM, Huawei, HiSilicon, OPPO

    - Remove Alt2: vivo, LGE, ZTE, Qualcomm, Interdigital, Intel, CMCC, Localia, Samsung, Ericsson, Xiaomi

- FFS: RE-Offset sequence within a SL-PRS resource

##### [ONLINE] Feature Lead Proposal 5.1.1.A-v4

For a dedicated resource pool for SL positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS consider [the/one of] following options:

  - Opt. 1: No other channel can be included beyond SL-PRS

  - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) is are included

    - FFS: Details

- Note: Companies are encouraged to provide their analysis and views on the above

##### [ONLINE] Feature Lead Proposal 5.1.2-v0

With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with sidelink communication

- Alt. 1: backward compatibility with legacy Rel-16/17 UEs should be supported

- Alt. 2: backward compatibility with legacy Rel-16/17 UEs does not need to be supported (i.e. introduce SL-PRS transmission only assuming a Rel-18 sidelink communication resource pool)

##### [ONLINE] Feature Lead Proposal 5.1.1.D-v1

[At least] for a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives:

  - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool

  - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool

- Note: Companies are encouraged to provide their analysis and views on the above alternatives

##### [ONLINE] Feature Lead Proposal 3.1.8-v1

With regards to the Positioning methods supported using SL measurements

- at least the following measurements are considered:

- SL Rx-Tx measurement

- SL RSTD measurement

- SL RSRP measurement

- SL RSRPP measurement

- SL RTOA measurement

- SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

- Companies are encouraged to study other measurements (e.g., time difference of arrival of 2 SL-PRS transmitted at 2 different times from the same anchor) and provide their analysis into why they are needed in light of the above measurements.

- Companies are encouraged to study potential signaling enhancements (e.g., SL Rx-Tx measurement is not reported but the transmit time of SL-PRS is adjusted based on the measurement)

- FFS any additional measurements

##### [ONLINE] Feature Lead Proposal 3.1.6-v0

With regards to SL Carrier phase Positioning

- Alt. 1: not enough progress has been made in the dedicated sub-agenda item, and therefore, do not consider it further for Rel-18 SL Positioning

  - CMCC, DCM, Sony, OPPO, Sharp, Interdigital, LGE

- Alt. 2: Revisit during RAN1 111 meeting to check whether enough progress has been made.

  - ZTE, CATT, Samsung

# Proposal for Email Endorsement

# References

      - RP-213588 “Revised SID on Study on expanded and improved NR positioning,” Intel (Email discussion moderator), RAN #94-e.

      - R1-2208364	Potential solutions for SL positioning	Nokia, Nokia Shanghai Bell

      - R1-2208372	Potential solutions for sidelink positioning	FUTUREWEI

      - R1-2208453	Discussion on SL positioning solutions	Huawei, HiSilicon

      - R1-2208558	Discussion on potential solutions for SL positioning	Spreadtrum Communications

      - R1-2208648	Discussion on potential solutions for sidelink positioning	vivo

      - R1-2208773	Discussion on potential solutions for sidelink positioning	China Telecom

      - R1-2208821	Discussion on potential solutions for SL positioning	OPPO

      - R1-2208981	Discussion on potential solutions for SL positioning	CATT, GOHIGH

      - R1-2209058	Potential solutions for SL positioning	Intel Corporation

      - R1-2209105	Consideration on potential solutions for SL positioning	Sony

      - R1-2209151	Discussion on potential solutions for SL positioning	NEC

      - R1-2209213	Discussion on potential solutions for SL positioning	ZTE

      - R1-2209291	Discussion on sidelink positioning solutions	xiaomi

      - R1-2209341	Discussion on potential solutions for SL positioning	CMCC

      - R1-2209393	Potential SL Positioning Solutions	Lenovo

      - R1-2209483	Discussion on potential solutions for SL positioning	LG Electronics

      - R1-2209487	Potential solutions for SL positioning	InterDigital, Inc.

      - R1-2209533	Potential solutions for SL positioning 	Faunhofer IIS, Fraunhofer HHI

      - R1-2209589	Discussions on Potential solutions for SL positioning	Apple

      - R1-2209675	Discussion on potential solutions for SL positioning	DENSO CORPORATION

      - R1-2209736	Discussion on Potential Solutions for SL Positioning	Samsung

      - R1-2209783	Views on potential solutions for SL positioning	Sharp

      - R1-2209797	Discussion on sidelink positioning	ASUSTeK

      - R1-2209907	Discussion on potential solutions for SL positioning	NTT DOCOMO, INC.

      - R1-2209990	Potential Solutions for Sidelink Positioning	Qualcomm Incorporated

      - R1-2210097	The potential solutions for sidelink positioning	MediaTek Inc.

      - R1-2210102	Considerable solutions on sidelink positioning in NR	ITL

      - R1-2210112	Discussion on enhancements for sidelink based positioning	CEWiT

      - R1-2210175	On potential solutions for SL positioning	Ericsson

# Previous Agreements for SL Positioning Solutions

RAN1 #109-e

Agreement

Study power control mechanisms for SL-PRS transmission, including whether it is necessary.

Agreement

With regards to the Positioning methods supported using SL measurements study further the following methods:

  - RTT-type solutions using SL

    - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT

  - SL-AoA

    - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study

  - SL-TDOA

  - SL-AoD

    - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu.

    - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study

- Consider in the study at least the following aspects:

  - Definition(s) of the corresponding SL measurements for each method

  - Which method is applicable to absolute or relative positioning or ranging, including whether such categorization is needed to be discussed.

  - For angle-based methods, antenna configuration consideration(s) using practical UE capabilities

  - Per-panel location, if UE uses multiple panels.

  - UE’s mobility, especially for V2X scenarios

  - Impact of synchronization error(s) between UEs

  - Existing SL measurements (e.g. RSSI, RSRP), and UE ID information etc, may be used.

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified, or whether there will be a unified SL Positioning method.

- Note: When the study of carrier phase positioning and the evaluations of sidelink positioning have progressed, it can be reviewed whether carrier phase for sidelink can be considered in further work. Checkpoint at RAN1#110-e-Bis to see if sufficient information is available for this review.

- Note: Companies are encouraged to describe the role of SL nodes and their interaction/coordination participating in each method.

Agreement

With regards to the numerologies of the SL-PRS, limit the study to those supported for NR Sidelink.

- Note 1: NR Sidelink supports {15, 30, 60 kHz} in FR1 and {60, 120 kHz} in FR2

- Note 2: This doesn’t imply that SL-PRS FR2-specific optimization(s) are expected to be studied

Agreement

Study new reference signal for SL positioning/ranging using the existing PRS/SRS design and SL design framework as a starting point.

- The study could at least include: Sequence design, frequency domain pattern, time domain pattern (e.g. number of symbols, repetitions, etc), time domain behavior, configuration/triggering/activation/de-activation of the SL-PRS, AGC time, Tx-Rx Turanround time, supportable bandwidth(s), multiplexing options with other SL channels, randomization/orthogonalization options.

- Note: The study of existing SL reference signal for SL positioning/ranging is not precluded. Companies are encouraged to perform performance evaluation/comparison to investigate whether such reference signals can meet the positioning accuracy requirements.

Agreement

With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options:

- Option 1: High-layer-only signaling involvement in the SL-PRS configuration

  - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS.

  - Based on the study, this option may correspond to

    - A SL-PRS configuration that is a single-shot or multiple shots

    - A high-layer configuration that may be received from an LMF, a gNB, or a UE

- Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

  - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS.

- Option 3: Only lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

- Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed.

Agreement

With regards to the Sidelink Positioning measurement report,

- Study the contents of the measurement report  (e.g. time stamp(s), quality metric(s), ID(s), angular/timing/power measurements, etc)

- Study the time domain behavior of the measurement report (e.g. one-shot, triggered, aperiodic, semi-persistent, periodic)

- FFS whether the Sidelink Positioning measurement can be a high-layer report and/or a lower layer report.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface).

- Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information.

- Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE.

- Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes.

- SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity.

- Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.

  - FFS: clarification of the knowledge of the location of the anchor UE

Agreement

With regards to the frequency domain pattern, study further a Comb-N SL-PRS design. Study at least the following aspects:

- N>=1 (where N=1 corresponds to full RE mapping pattern)

- Fully staggered SL-PRS pattern (e.g., M symbols of SL-PRS with comb-N with M=N and, at each symbol a different RE offset is used), Partially staggered SL-PRS pattern (e.g., M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used), Unstaggered SL-PRS patterns (e.g., M symbol(s) of SL-PRS with comb- N, at each symbol a same RE offset is used, N > 1)

- The number of symbols of SL-PRS within a slot

  - Any relation to the comb-N option

  - RE offset pattern repetitions within a slot

- FFS: Other frequency domain pattern(s)

Agreement

For a potential new SL PRS, study further the following

- Number of symbol(s) for AGC and/or Rx-Tx turnaround time

- Conditions under which AGC training and/or Rx-Tx turnaround time are needed

Agreement

With regards to the SL Positioning resource allocation, study further the following 2 options for SL Positioning resource (pre-)configuration:

- Option 1: Dedicated resource pool for SL-PRS

  - Include in the study at least the following aspects:

    - which slots can be used, SL frame structure, SL positioning slot structure, multiplexing of SL-PRS with control information (if included in the same slot)

    - positioning measurement report

    - whether a dedicated frequency allocation (e.g., layer/BWP) is needed for SL PRS

    - resource allocation procedure(s) of SL-PRS

    - This option may or may not include control information (i.e., configuration/activation/deactivation/triggering of SL-PRS) for the purpose of SL positioning operation

- Option 2: Shared resource pool with sidelink communication.

  - Include in the study at least the following aspects:

    - co-existence between SL communication and SL positioning, backward compatibility

    - Multiplexing considerations of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH) and any modifications in the SL-slot structure

Agreement

With regards to the SL-PRS resource allocation, study the following two schemes:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

  - Applicable regardless of the network coverage

- FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting UEs (e.g. IUC-like solutions).

- Note: Other Schemes are not precluded to be studied

- FFS how to handle resource allocation of SL-Positioning measurement report

RAN1 #110-e

Agreement

With regards to the Positioning methods supported using at least SL measurements, potential candidate positioning methods include at least the following:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- Note: other methods can still be studied

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified.

Agreement

A new reference signal should be introduced for supporting SL positioning/ranging.

Agreement

Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS.

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

Agreement

With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging:

- Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS

- Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed

Agreement

For the content of the sidelink positioning measurement report, potential elements may include at least the following:

- One or more sidelink positioning measurement(s)

- Timestamp(s) associated with a sidelink positioning measurement

- Quality metric(s) associated with a sidelink positioning measurement

- Identification Information for a sidelink positioning measurement

- FFS any detail for the above

Agreement

For the sequence of the new reference signal for SL positioning/ranging, down select between Alt 1 and Alt 2:

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

- Alt. 2: ZC-based (SRS sequence as a starting point)

Agreement

With regards to the frequency domain pattern, a Comb-N SL-PRS occupying M symbol(s) design should be introduced for the support of NR SL positioning

- Note: there could be multiple values for M, N

Agreement

Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects:

- Resource selection mechanism for SL-PRS

- Inter-UE coordination

- Aspects for congestion control mechanisms for SL-PRS

Agreement

- With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further.

- With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement.

Agreement

With regards to the frequency domain pattern for multi-symbol SL-PRS, prioritize partially and fully staggered SL-PRS.

- Note: this does not preclude comb N=1

- FFS: single symbol SL-PRS, if supported