3GPP TSG RAN WG1 #110bis-e			R1-2210624

Electronic Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on conditions for UE to be UE-B for Condition 2-A-2 of Scheme 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (LG Electronics) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Condition 2-A-2 of inter-UE coordination scheme 2, further clarification on conditions for UE to be UE-B is needed, since it is infeasible to transmit the conflict indication if the PSFCH occasion for resource conflict information of the second UE is not valid and the conflict indication should not be transmitted if the conflict information receiver flag in SCI format 1-A from the second UE is set to 0 and if sl-IndicationUE-B = 'enabled'. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the related description on conditions for UE to be UE-B for Condition 2-A-2 in TS 38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential transmission of the conflict indication in an incorrect PSFCH occasion. Incomplete support of the conflict information receiver flag. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

16.3.0	UE procedure for transmitting PSFCH with control information

<Unchanged parts omitted>

A first UE determines a second UE for providing the conflict information to in a PSFCH as follows

-	if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot,

-	does not expect to perform reception on the sidelink due to half-duplex operation in the slot,

-	the PSFCH occasion for resource conflict information of the second UE is valid,

-	the conflict information receiver flag in SCI format 1-A from the second UE is set to 1, if sl-IndicationUE-B = 'enabled', and

-	determines to transmit to the second UE the PSFCH with the conflict information.

<Unchanged parts omitted>