3GPP TSG RAN WG1 #111	R1-2210862

Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | xxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the short message indicator when TRS availability indication is present |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to Table 7.3.1.2.1-1 in TS38.212, when the bit field of Short Message indicator is “01”, it describes that “only scheduling information for Paging is present in the DCI format 1_0”, and when the bit field of Short Message indicator is “10”, it describes that “only short message is present in the DCI format 1_0”. This is not correct in Rel-17 considering TRS availability indication field is also present in the DCI format 1_0 when the bit field of Short Message indicator is “01”, “10” or “11”. - The description of other fields in DCI fomat 1_0 needs to be also updated similarly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct that TRS availability indication is present when the bit field of Short Message indicator is “01”, “10” or “11”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TRS availability indication shall be ignored by the UE due to incorrect description in DCI format 1_0 and Table 7.3.1.2.1-1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 7.3.1.2.1	Format 1_0

#### < Unchanged parts are omitted >

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI:

-	Short Messages Indicator – 2 bits according to Table 7.3.1.2.1-1.

-	Short Messages – 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if TRS-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	Frequency domain resource assignment – bits.  If only the short message, and TRS availability indication if TRS-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	 is the size of CORESET 0

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message, and TRS availability indication if TRS-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5. If only the short message, and TRS availability indication if TRS-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message, and TRS availability indication if TRS-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	TB scaling – 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message, and TRS availability indication if TRS-ResourceSetConfig is configured, are carried, this bit field is reserved.

-	TRS availability indication – 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the TRS-ResourceSetConfig if configured; 0 bits otherwise.

-	Reserved bits – (8 – M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 – M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above.

#### < Unchanged parts are omitted >

Table 7.3.1.2.1-1: Short Message indicator

| Bit field | Short Message indicator |
| --- | --- |
| 00 | Reserved |
| 01 | Only scheduling information for Paging, and TRS availability indication if TRS-ResourceSetConfig is configured, are present in the DCI |
| 10 | Only short message, and TRS availability indication if TRS-ResourceSetConfig is configured, are present in the DCI |
| 11 | Scheduling information for Paging, TRS availability indication if TRS-ResourceSetConfig is configured and short message are present in the DCI |

#### < Unchanged parts are omitted >