3GPP TSG RAN WG1 #111	R1-2210966

Toulouse, France, November 14th – 18th, 2022

Source: 	vivo

Title:             Discussion on the number of CSI-IM resources

Agenda Item:	8.1

Document for:	Discussion and Decision

Introduction

In TS38.214 [1], for NCJT CSI enhancement, the UE can be configured a CSI reporting setting and the corresponding NZP CSI-RS Resource Set for channel measurement is configured with 2≤Ks≤8 resources.

According to the following description in TS38.214, a resource pair for a NCJT measurement hypothesis is corresponding to a CSI-IM resource. Therefore, the number of CSI-IM rsources are different form the number of CSI-IM resources and  may exceed 8 when the higher layer parameter sharedCMR-r17 is enabled, e.g., Ks=8 CSI-RS resources are configured in the CSI resource set with two resource groups and sharedCMR-r17 is enabled. However, in TS38.214 and TS38.331[2], the number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources and the maximum number of CSI-IM resources per set is 8, which is inconsistent with the conclusion of NCJT CSI enhancement.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with Ks≥2 resources, two Resource Groups with K1≥1 resources in Group 1, K2≥1 resources in Group 2, K1+K2=Ks, and N Resource Pairs: - each resource can contain, subject to UE capability, at most 32 CSI-RS ports. For two Resource Groups with Ki resources (i=1,2), if maxK1,K2=1, the resource in NZP-CSI-RS-ResourceSet shall contain at most 32 CSI-RS ports; if maxK1,K2=2, each resource in NZP-CSI-RS-ResourceSet shall contain at most 16 CSI-RS ports; if 2<maxK1,K2<8, each resource in NZP-CSI-RS-ResourceSet shall contain at most 8 CSI-RS ports. - each of the N Resource Pairs is associated to a CRI value. - The CSI-ReportConfig may be configured with higher layer parameter sharedCMR. M1 and M2 are the numbers of resources associated to a CRI value, other than the N CRIs defined above, in Group 1 and Group 2, respectively, with M=M1+M2, such that the total number of CRI values configured for the CSI-ReportConfig is M+N. - If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0}, M1=M2=0; otherwise, - if the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{1,2}, or if csi-ReportMode is set to 'Mode2', - if sharedCMR is configured: M1=K1 and M2=K2; otherwise - if sharedCMR is not configured, only the resources in Group 1 and Group 2 that are not referred to in any Resource Pair are associated to M CRI values other than the N CRIs defined above. - If interference measurement is performed on CSI-IM, M+N resources are configured in the corresponding csi-IM-ResourceSet. The M resources for channel measurement defined above are resource-wise associated with the first M CSI-IM resources by the ordering of the CSI-RS resources and CSI-IM resources in the corresponding Resource Set. The N Resource Pairs for channel measurement are associated to the last N CSI-IM resources by the ordering of the CSI-RS Resource Pairs and CSI-IM resources in the CSI-IM Resource Set. The UE may assume that the two CSI-RS resources for channel measurement in a Resource Pair and the associated CSI-IM resource for interference measurement are resource-wise QCLed with respect to 'typeD'.

In this contribution, these two issues are discussed and a potential solution is provided.

Discussion

For Rel-17 NCJT CSI measurement, M+N CSI-IM resources are configured in the corresponding csi-IM-ResourceSet, where M = M1+M2 and M1, M2 are the numbers of CSI-IM resources associated with M1, M2 CMRs for STRP measurement hypotheses in Group 1 and Group 2, respectively, and N is the number of CSI-IM resources associated with N CMR pair(s) for NCJT measurement hypotheses. The corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with 𝐾𝑠≥2 resources, two Resource Groups with 𝐾1≥1 resources in Group 1, 𝐾2≥1 resources in Group 2, 𝐾1+𝐾2=𝐾𝑠. According the current TS 38.214,

if the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to 𝑋∈{0}, 𝑀1=𝑀2=0 and the number of CSI-IM resources is N<Ks.

If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to 𝑋∈{1,2} and sharedCMR is configured, 𝑀1=K1 and 𝑀2=K2 and the number of CSI-IM resources is N+K1+K2>Ks.

If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to 𝑋∈{1,2} and sharedCMR is not configured, 𝑀1=K1-N and 𝑀2=K2-N and the number of CSI-IM resources is K1+K2-N<Ks.

If the higher layer parameter csi-ReportMode is set to 'Mode2' and sharedCMR is configured, 𝑀1=K1 and 𝑀2=K2 and the number of CSI-IM resources is N+K1+K2>Ks.

If the higher layer parameter csi-ReportMode is set to 'Mode2' and sharedCMR is not configured, 𝑀1=K1-N and 𝑀2=K2-N and the number of CSI-IM resources is K1+K2-N<Ks.

Therefore, the number of CSI-IM resources configured in a CSI-IM resource set includes the CSI-IM resource for NCJT CMR pairs and CSI-IM resources for STRP CMRs, which does not equal to the number of CSI-RS resources for channel measurement for NCJT measurement. To make the contradiction clear, a draft CR is provided in our companion contribution [3].

Adopt draft CR for RAN1 specification.

Regarding the number of CSI-IM resources in TS38.331, two RRC parameters, maxNrofCSI-IM-Resources and maxNrofCSI-IM-ResourcesPerSet, are provided in TS38.331, where maxNrofCSI-IM-Resources is equal to 32 and maxNrofCSI-IM-ResourcesPerSet is equal to 8. As described in TS38.214, when two Resource Groups and CMR pair(s) are configured for a CSI-RS resource set, M+N CSI-IM resources are configured in the corresponding csi-IM-ResourceSet. The M resources for channel measurement are resource-wise associated with the first M CSI-IM resources by the ordering of the CSI-RS resources and CSI-IM resources in the corresponding Resource Set. The N Resource Pairs for channel measurement are associated to the last N CSI-IM resources by the ordering of the CSI-RS Resource Pairs and CSI-IM resources in the CSI-IM Resource Set. Therefore, maxNrofCSI-IM-Resources may exceed 32 and maxNrofCSI-IM-ResourcesPerSet may exceed 8 when the higher layer parameter sharedCMR-r17 is enabled. Possible alternatives are listed as follows to solve the inconsistence:

Alt 1: the value of maxNrofCSI-IM-ResourcesPerSet is increased from 8 to 10, where 2 additional CSI-IM resources corresponds to the up to 2 CMR pairs. And the value of maxNrofCSI-IM-Resources is increased from 32 to 40.

Alt 2: the value of maxNrofCSI-IM-ResourcesPerSet is increased from 8 to 10, where 2 additional CSI-IM resources corresponds tothe up tp 2 CMR pairs, while keeping the value of maxNrofCSI-IM-Resources unchanged.

- In our understanding, the value of maxNrofCSI-IM-Resources associated with UE capability maxConfigNumberCSI-IM-PerCC can be kept unchanged while maxNrofCSI-IM-ResourcesPerSet associated with the number of NZP CSI-RS for NCJT CSI enhacncement should be enhanced.

For Rel-17 NCJT CSI, maxNrofCSI-IM-ResourcesPerSet associated with the number of NZP CSI-RS for NCJT CSI enhacncement is increased to 10, to consider the CSI-IM resources associated with up to 2 of CMR pairs.

Send a LS to RAN2 to inform RAN1 decision.

Conclusions

According to the discussion, following proposals are provided.

- Adopt draft CR for RAN1 specification.

For Rel-17 NCJT CSI, maxNrofCSI-IM-ResourcesPerSet associated with the number of NZP CSI-RS for NCJT CSI enhacncement is increased to 10, to consider the CSI-IM resources associated with up to 2 of CMR pairs.

Send a LS to RAN2 to inform RAN1 decision.

References

[1] TS 38.214, v17.3.0, Physical layer procedures for data, Sep 2022.

[2] TS 38.331, v17.2.0, Radio Resource Control (RRC) protocol specification, Sep 2022.

[3] R1-2210967, “Draft CR on the number of CSI-IM resources”, RAN1#111, vivo, Nov 2022.