# 3GPP TSG RAN WG1 Meeting #111                                                                    R1-2211059

# Toulouse, France, November 14th – 18th, 2022

# Source:              ZTE Corporation

# Title:                   Evaluation on AI for beam management

# Agenda Item:     9.2.3.1

# Document for:   Discussion and Decision

- Introduction

In RAN1#109e, RAN1#110, and RAN1#110bis-e, the evaluation methodology and KPIs for AI/ML based beam management were discussed and many fruitful consensuses have been achieved [1][2][3]. Particularly, system level simulation was agreed to be the baseline tool for dataset construction and most of the simulation assumptions have already been determined. Besides, many basic KPIs were agreed for performance evaluation of spatial-domain beam prediction and temporal beam prediction, such as beam prediction accuracy related KPIs, system performance related KPIs, etc. In this contribution, we will provide our views and preliminary simulation results of the AI/ML based beam prediction.

- Spatial domain beam prediction

General description:

The task of beam selection aims at aligning the best beam at both the transmitter and receiver, which is challenging due to mobility and large attenuation in mmWave communication systems. A typical way for beam selection is to exhaustively sweep all beam pairs to find the best beam pair. However, as the number of beams grows, the beam selection overhead and associated power consumption for measurement would be unacceptably high. Therefore, we utilize AI/ML technologies to assist beam selection and explore the spatial domain beam prediction with low overhead as shown in the figure below. The measured RSRPs of partial beams (or beam pairs) are used as the AI input and the predicted RSRPs of all beams (or beam pairs) are used as the AI output, where each beam pair includes a transmit beam at the gNB and a receive beam at the UE. Besides, the beam set for measurement (i.e., set B) is a subset of the whole beam set for prediction (i.e., set A).

Figure 2-1. Illustration of AI/ML based spatial domain beam prediction.

Dataset construction:

- To evaluate performance of the AI/ML based method for spatial-domain beam prediction, we generate the dataset for model training and validation according to agreed parameters in RAN1#109e, which is summarized in Table I in the appendix. Specifically, 10 UEs per sector per drop are randomly generated with a total of 500 simulation drops. 80% of UEs are located in indoor and 20% of UEs are located in outdoor as in TR 38.901. The UE speed is fixed to be 3 km/h. For antenna configuration, the baseline agreed in RAN#110bis-e is adopted directly, where a single panel with (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5)λ and 2 panels with (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) are used by the gNB and UE, respectively. Besides, the gNB and UE are equipped with 64 Tx beams in total and 4 Rx beams per panel, respectively.

AI/ML model and training methodology:

For the spatial domain beam prediction, a full connected network with 4 layers is used with measured RSRPs of partial beams (or beam pairs) being used as the AI input and predicted RSRPs of all beams (or beam pairs) being used as the AI output. Both the Tx-Rx beam pair prediction and Tx beam prediction are considered for comparison, which apparently corresponds to the P1 initial beam selection procedure and P2 Tx beam refinement procedure, respectively. For ease of understanding, we use a simple notation <Tx Ry> for AI model input in the following, where x denotes the number of transmission beams picked out of 64 Tx beams and y denotes the number of reception beams picked out of 8 reception beams. Particularly, if multiple Tx or Rx beams are used for beam measurement, we assume all measured beams are obtained by uniformly sampling the whole beam space <T64 R8> with a pre-determined beam pattern. If only one receive beam is used for beam measurement (i.e., y=1 in the case of Tx beam prediction), we assume that the receive beam is the realistic optimal Rx beam or a fixed Rx beam (the first Rx beam) during the beam measurement procedure. Furthermore, we also consider various numbers of beams in set B as model input for the evaluation of model generalization capability. In this generalization case, we use zero padding to the beams without measured RSRPs to align the model input size, resulting of a fixed input size of 64×8.

Performance KPIs:

To evaluate the performance of AI/ML in spatial domain beam prediction, we provide our preliminary simulation results in terms of KPIs related to beam predicting accuracy, which include average L1-RSRP difference of Top-1 predicted beam, beam prediction accuracy for Top-1 and/or Top-K beams, beam prediction accuracy with 1dB margin for Top-1 beam. Particularly, the beam prediction accuracy (%) for Top-K beams is defined as the percentage that the Top-1 genie-aided beam is one of the Top-K predicted beams. In this case, a refined second stage beam sweeping over these Top-K predicted beams may be needed to obtain an optimal beam. Nevertheless, whether to conduct the refined beam sweeping over Top-K beams is up to NW.

Besides, RS overhead reduction for beam measurement should also be considered as a basic KPI for evaluation. Specifically, RS overhead reduction can be defined as 1-N/M with N being the number of beams (or beam pairs) for measurement and M being the number of beams (or beam pairs) for prediction.

Observation 1: If Top-K beams are predicted by the adopted AI model, a refined beam sweeping over these Top-K predicted beams may be needed to obtain an optimal beam, which is up to NW.

Proposal 1: RS overhead reduction should be considered as a basic KPI for evaluation and can be defined as 1-N/M, where N is the number of beams (or beam pairs) for measurement and M is the number of beams (or beam pairs) for prediction.

Simulation results:

Both Tx-Rx beam pair prediction and Tx beam prediction mentioned above are simulated and compared in term of beam prediction accuracy performance. Except for the generalization evaluation, the sampling rate on the whole beam space in both cases is fixed to be 25%, which means that the associated RS overhead reduction would be 75%. Besides, corresponding non-AI cases are also simulated as baseline performance, where the optimal beam is obtained by measuring the uniformly sampled partial beams and directly choosing the one with the largest measured RSRP. For spatial beam prediction, 6 different cases are considered as follows.

- Case 1-1: In the <T32 R4> case for Tx-Rx beam pair prediction, 32 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by 4 receive beams picked out of 8 Rx beams by uniform sampling in the UE. In this case, RSRPs of selected beam pairs (32x4=128 beam pairs) are used as AI model inputs and predicted RSRPs of all beam pairs (64x8=512 beam pairs) are used as AI model outputs.

- Case 1-2: In the <T16 R8> case for Tx-Rx beam pair prediction, 16 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by all 8 receive beams in the UE. In this case, RSRPs of selected beam pairs (16x8=128 beam pairs) are used as AI model inputs and predicted RSRPs of all beam pairs (64x8=512 beam pairs) are used as AI model outputs.

- Case 1-3: In the <T16 R1> case for Tx beam prediction, 16 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal or kept fixed during the beam measurement procedure. In this case, RSRPs of selected beams (16x1=16 beams) are used as AI model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI model outputs.

- Case 1-4: Both training and testing with samples of mixed data samples, where the mixed data samples generated under <T8 R1>, <T16 R1>, and <T32 R1> cases are used as AI model inputs to predict RSRPs of all Tx beams.

- Case 1-5: In the <T8 R1> case for Tx beam prediction, 8 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal or kept fixed during the beam measurement procedure. In this case, RSRPs of selected Tx beams (8x1=8 beams) are used as AI model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI model outputs.

- Case 1-6: In the <T32 R1> case for Tx beam prediction, 32 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal or kept fixed during the beam measurement procedure. In this case, RSRPs of selected beams (32x1=32 beams) are used as AI model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI model outputs.

All simulation results are summarized in Table II. As can be seen, the spatial domain beam prediction with a fixed (i.e., the first Rx beam) or optimal Rx beam achieves a sufficiently high performance with only 25% beam overhead being used. Besides, even with a same sampling rate on the whole beam space, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pairs prediction. As can be concluded from the simulation results, the number of Rx beams is relatively small and any sampling at the Rx beam space may cause serve loss of the spatial channel characteristic. Thus, in the beam pair prediction, it’s better to take measurement results of all Rx beams as the model input instead of performing any sampling at the Rx beam space. We also note that other assistance information such as beam shape or beam usage are not evaluated since they are implementation-related information of the gNB or UE, which may be difficult to be disclosed to the opposite node [4].

Table II. Performance comparison of Tx-Rx beam pairs prediction and Tx beams prediction

| Cases |  | Fixed or optimal Rx beam | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Diff to genie-aided beam |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1-1 | Non-AI | / | 3.099 | 42.80% | 25.95% | 25.95% | 3.099 |
|  | AI | / | 1.171 | 77.60% | 60.35% | 87.15% | 0.756 |
| Case 1-2 | Non-AI | / | 2.604 | 43.25% | 27.55% | 27.55% | 2.604 |
|  | AI | / | 0.357 | 91.10% | 75.55% | 96.55% | 0.352 |
| Case 1-3 | Non-AI | Fixed | 3.013 | 42.00% | 25.45% | 25.45% | 3.391 |
|  |  | Optimal | 3.391 | 40.45% | 25.95% | 25.95% | 3.013 |
|  | AI | Fixed | 0.326 | 92.75% | 77.35% | 97.40% | 0.187 |
|  |  | Optimal | 0.096 | 98.10% | 88.25% | 99.75% | 0.335 |

Note: Diff to genie-aided beam is defined as the L1-RSRP difference between the predicted L1-RSRP

of Top-1 predicted beam and the ideal L1-RSRP of Top-1 genie-aided beam

Observation 2: The spatial domain beam prediction with a fixed beam pattern achieves a sufficiently high performance with only 25% beam overhead being used.

Observation 3: With the same sampling rate on the whole beam space, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction.

Proposal 2: In the beam pair prediction, it’s better to take measurement results of all Rx beams as the model input instead of performing any sampling at the Rx beam space.

For the evaluation of model generalization capability, we consider different number of beams in set B for AI model inputs. More specifically, mixed data samples generated under <T8 R1>, <T16 R1>, and <T32 R1> cases are input to the AI model to predict RSRPs of all transmit beams. For baseline comparison, corresponding non-AI cases are also simulated. The simulation results of generalization performance are summarized in Table III. As observed, the case of generalization AI can achieve a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>. Additionally, the case of generalization AI can obtain significant performance gain over the non-AI case. The AI based method with the optimal Rx beam can achieve a slight better performance than that of the AI based method with the fixed Rx beam.  For cross-checking and reproducibility, the description of the adopted AI model and the evaluation results are summarized in Table IX in the appendix.

Table III. Simulation results for generalization performance evaluation

| Cases |  | Fixed or optimal Rx beam | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4 | Non-AI | Fixed | 2.933 | 45.25% | 28.97% | 28.97% |
|  |  | Optimal | 3.405 | 44.17% | 30.03% | 30.03% |
|  | AI | Fixed | 0.675 | 87.67% | 72.68% | 95.83% |
|  |  | Optimal | 0.314 | 94.733 | 84.867 | 98.92% |
| Case 1-5 | Non-AI | Fixed | 5.186 | 20.95% | 12.50% | 12.50% |
|  |  | Optimal | 6.9198 | 19.80% | 12.60% | 12.60% |
|  | AI | Fixed | 1.561 | 72.300% | 55.60% | 89.80% |
|  |  | Optimal | 0.832 | 86.55% | 72.60% | 96.85% |
| Case 1-3 | Non-AI | Fixed | 3.013 | 42.00% | 25.45% | 25.45% |
|  |  | Optimal | 3.391 | 40.45% | 25.95% | 25.95% |
|  | AI | Fixed | 0.326 | 92.75% | 77.35% | 97.40% |
|  |  | Optimal | 0.096 | 98.10% | 88.25% | 99.75% |
| Case 1-6 | Non-AI | Fixed | 0.600 | 72.80% | 48.95% | 48.95% |
|  |  | Optimal | 0.627 | 72.25% | 51.55% | 51.55% |
|  | AI | Fixed | 0.112 | 97.60% | 85.10% | 99.15% |
|  |  | Optimal | 0.041 | 99.40% | 92.45% | 100.00% |

Observation 4: The case of AI generalization that model training and testing are performed in mixed datasets with different beam constructions can achieve a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>.

Observation 5: The AI based Tx beam prediction with the optimal Rx beam is slightly better than that of the AI based method with a fixed (e.g, first) Rx beam.

Observation 6: The AI/ML model can be utilized for spatial domain beam prediction, which can greatly reduce the RS overhead for measurement while maintain a high beam prediction accuracy in various beam constructions.

Proposal 3: Different beam constructions in Set B (number/pattern of beams (pairs) in Set B, etc) should be considered for the evaluation of model generalization capability.

- Temporal beam prediction

General description:

In high mobility environment, the task of fast beam tracking is more challenging. A typical way for beam tracking is periodic measurement and reporting, which introduces significant overhead that scales with the number of beams. Therefore, we utilize AI/ML technology to realize temporal beam prediction in this section. Specifically, recurrent neutral network is leveraged to implement the temporal beam prediction with assumption of a simple UE mobility pattern with random direction straight-line trajectories. The measured RSRPs of all or partial DL beams (or beam pairs) of some past time instances are used as the AI input and the predicted RSRPs of all DL beams (or beam pairs) of future time instances are used as the AI output. That is, the beam set for measurement and the beam set for prediction are the same, or the beam set for measurement is a subset of the beam set for prediction. Besides, for each time instance within the prediction window, Top-K beams predicted by the adopted AI model may be used for a refined beam sweeping.

Figure 3-1. Illustration of AI/ML based temporal beam prediction.

Dataset construction:

- The dataset for model training and evaluation is generated according to the agreed parameters in RAN1#109-e, which is summarized in Table I in the appendix. Specifically, one UE per sector per drop is randomly generated with a total of 20 simulation drops. For antenna configuration, the baseline agreed in RAN#110bis-e is adopted directly, where a single panel with (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5)λ and 2 panels with (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) are used by the gNB and UE, respectively. Besides, the gNB and UE are equipped with 32 Tx beams in total and 4 Rx beams per panel, respectively.

For temporal beam prediction, the random direction straight-line trajectories in Option #4 is adopted for modelling UE mobility, which means that multiple UEs are randomly dropped in multiple cells and move in a straight line trajectory along an initially random direction with fixed speed. More details about Option #4 is provided in the appendix for reference. Compared with other UE trajectory options, Option #4 is simpler and beneficial for model generalization. The modelling of UE orientation and rotation may cause the adopted model to be overly complex with excessive training overhead. Thus, it has not been considered for the time being for simplicity. Additionally, spatial consistency procedure B in TR38.901 is modeled along the mobility trajectory to ensure that propagation parameters maintain continuity across multiple realizations.

Table IV. Simulation assumptions for the temporal beam prediction

| UE distribution | 1 UE per sector per drop with a total of 21 sectors and 20 simulation drops |
| --- | --- |
| TTI number per simulation drop | 150000 |
| BS Antenna Configuration | One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5)λ |
| UE Antenna Configuration | Two panel: (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) |
| Spatial consistency | Procedure B in TR38.901 |
| UE Speed | 30 km/h, 60 km/h and 90 km/h |
| UE trajectory model | Option #4: Random direction straight-line trajectories |
| UE orientation | Initially random and keeps fixed |
| UE rotation speed | 0 |

Proposal 4: The random direction straight-line trajectories in Option #4 can be adopted for modelling UE trajectory as a baseline.

AI/ML model and training methodology:

For cross-checking purpose, the adopted AI model for temporal beam prediction is captured in Table V. The LSTM network is used, where the model input is measured RSRPs of all or partial DL beams (or beam pairs) of past 5 time instances, and the model output is predicted RSRPs of all DL beams (or beam pairs) of future 5 time instances. The sampling interval between two adjacent time instances is 160ms. For the evaluation of generalization performance, data samples generated by mixed UE speeds of 30km/h and 90km/h are used for AI model training and testing. The corresponding dataset size is 60,000 and 180,000 for the case of UE speed of 30km/h, 60km/h or 90km/h and for the case of mixed UE speeds of 30km/h, 60km/h and 90km/h, respectively. Besides, model complexity is also taken into consideration, which includes the model size and the amount of computation (measured in FLOPs). Among them, the former describes the required memory size and the latter describes the required computing complexity, which directly impacts the inference time of the model.

Table V. Description of the adopted AI model for temporal beam prediction

| Model architecture | LSTM, FC layer |
| --- | --- |
| Number of layers | 4 |
| Model input | Measured RSRPs of past time instances |
| Model output | Predicted RSRPs of future time instances |
| Observation window length | 5 |
| Prediction window length | 5 |

| Dataset size | For performance comparison of fixed Set B and variable Set B across training and inference: 140000 |
| --- | --- |
|  | For performance comparison of Tx-Rx beam pair prediction and Tx beam prediction: 140000 |
|  | For evaluation of generalization: 60,000 (for single UE speed of 30km/h or 60km/h or 90km/h) 180,000 (for mixed UE speeds of 30km/h, 60km/h, and 90km/h) |
| Percentage of training/validity/testing samples | 90%, 5%, 5% |
| Model size | 2.758M |
| FLOPs | 7.250M |

Performance KPIs:

To evaluate the performance of AI/ML in temporal beam prediction, we provide our preliminary simulation results in terms of beam prediction accuracy for Top-1 and Top-K beams. All adopted KPIs are appropriately defined according to the agreement achieved in RAN1#109e. Particularly, the beam prediction accuracy (%) for Top-K beams is defined as the percentage that the Top-1 genie-aided beam is one of the Top-K predicted beams. As with the spatial domain beam prediction, a second stage beam sweeping over these Top-K predicted beams is not considered for RS overhead counting. Otherwise, the RS overhead for the second stage beam sweeping would be the product of the number of UEs per sector and the number of Top-K beams. Thus, the RS overhead of the temporal beam prediction method may be even higher than that of an exhaustive sweeping of all beams.

Simulation results:

- Performance comparison of fixed Set B and variable Set B across training and inference

To study the selection of Set B of beams (pairs), performance comparison of fixed Set B and variable Set B across training and inference is conducted, where RSRPs of measured beam pairs in Set B are used as model input. Besides, Set B is a subset of Set A, with a size of 32 and 256, respectively. Two different cases of Set B are considered as follows.

- Case 2-1: Set B is fixed across training and inference

- Case 2-2: Set B is variable across training and inference, changed following a set of pre-configured patterns

Table VI.  Beam prediction accuracy for different cases (UE speed 30km/h)

| Cases |  | Top-K | Top-K Prediction Accuracy [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-1 | AI | 4 | 96.07 | 96.07 | 96.18 | 95.73 | 94.74 |
|  | Non-AI | 4 | 30.99 | 31.03 | 30.73 | 30.90 | 30.86 |
| Case 2-2 | AI | 4 | 97.08 | 96.44 | 96.59 | 96.10 | 95.47 |
|  | Non-AI | 4 | 32.13 | 31.96 | 32.28 | 32.48 | 32.36 |

Based on the simulation results shown in Table VI, significant performance gain can be achieved by the AI based temporal beam prediction in terms of beam prediction accuracy for Top-4 beams. Additionally, compared with the case of Set B is fixed, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

Observation 7: Compared with the case that Set B is fixed across training and inference, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

- Performance comparison of Tx-Rx beam pair prediction and Tx beam prediction

To compare the performance of Tx-Rx beam pair prediction and Tx beam prediction based on the optimal or fixed Rx beam, four different cases of AI model inputs for each past time instance and outputs for future time instance are considered as follows.

- Case 2-3: RSRPs of all Tx beams of 5 past time instances are used as model input and RSRPs of all Tx beams of 5 future time instances are used as model output, where the optimal Rx beam is assumed for measurement

- Case 2-4: RSRPs of all Tx beams of 5 past time instances are used as model input and RSRPs of all Tx-Rx beam pairs of 5 future time instances are used as model output, where the optimal Rx beam is assumed for measurement

- Case 2-5: RSRPs of all Tx beams of 5 past time instances are used as model input and RSRPs of all Tx beams of 5 future time instances are used as model output, where the fixed (i.e., first) Rx beam is assumed for measurement

- Case 2-6: RSRPs of all Tx beams of 5 past time instances are used as model input and RSRPs of all Tx-Rx beam pairs of 5 future time instances are used as model output, where the fixed (i.e., first) Rx beam is assumed for measurement

- Based on the simulation results shown in Table VII, Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction under the optimal Rx beam or the fixed Rx beam. In addition, compared with the case of the fixed Rx beam, a slight performance gain is observed if Tx beam prediction or Tx-Rx beam pair prediction is performed under the optimal Rx beam.

Table VII.  Beam prediction accuracy for different cases (UE speed 30km/h)

| Cases |  | Top-K | Top-K Prediction Accuracy [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-3 | AI | 4 | 99.17 | 98.99 | 98.85 | 98.50 | 98.25 |
|  | Non-AI | 4 | 97.06 | 95.48 | 94.05 | 92.19 | 90.37 |
| Case 2-4 | AI | 4 | 96.70 | 96.74 | 96.50 | 96.26 | 94.96 |
|  | Non-AI | 4 | 90.35 | 86.07 | 83.22 | 80.67 | 77.77 |
| Case 2-5 | AI | 4 | 98.62 | 98.32 | 98.13 | 98.09 | 97.61 |
|  | Non-AI | 4 | 96.61 | 94.98 | 93.13 | 91.29 | 89.35 |
| Case 2-6 | AI | 4 | 95.28 | 95.88 | 95.28 | 94.91 | 93.47 |
|  | Non-AI | 4 | 21.42 | 21.05 | 20.54 | 20.13 | 19.85 |

Observation 8: For AI/ML based temporal beam prediction, Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction under the optimal Rx beam or a fixed Rx beam.

- Evaluation for generalization

To evaluate the generalization performance of AI model, RSRPs of all measured beam pairs are used as model input, and the beam set for measurement and the beam set for prediction are the same. Datasets for training and testing can be the samples with UE speed of 30km/h or 60km/h or 90km/h or mixed. According to the type of datasets for training and testing, seven different cases of AI model inputs for each past time instance are considered as follows.

- Case 2-7: Training with samples of mixed UE speed, testing with samples of UE speed of 90km/h

- Case 2-8: Training with samples of mixed UE speed, testing with samples of UE speed of 60km/h

- Case 2-9: Training with samples of mixed UE speed, testing with samples of UE speed of 30km/h

- Case 2-10: Both training and testing with samples of mixed UE speed

- Case 2-11: Both training and testing with samples of UE speed of 90km/h

- Case 2-12: Both training and testing with samples of UE speed of 60km/h

- Case 2-13: Both training and testing with samples of UE speed of 30km/h

As can be seen from the simulation results in Table VIII, if the AI model is trained with samples of single UE speed or mixed UE speeds, a little performance loss is observed as the UE speed in testing samples increases from 30km/h to 90km/h. The AI based method in case 2-7, case 2-8, case 2-9, and case 2-10 can also achieve a better performance than that of the non-AI method in scenarios with mixed UE speeds. Additionally, a little performance loss is observed as the AI model is trained with a single UE speed and tested with mixed UE speeds. As with the spatial-domain beam prediction, the description of the adopted AI model and the evaluation results are summarized in Table IX in the appendix.

Table VIII.  Beam prediction accuracy for different cases

| Cases |  | Top-K | Top-K Prediction Accuracy [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-7 | AI | 4 | 93.16 | 92.07 | 90.70 | 89.49 | 86.84 |
|  | Non-AI | 4 | 91.31 | 84.63 | 78.37 | 73.44 | 68.64 |
| Case 2-8 | AI | 4 | 95.27 | 93.94 | 93.16 | 92.27 | 90.67 |
|  | Non-AI | 4 | 93.76 | 88.64 | 83.34 | 78.49 | 74.24 |
| Case 2-9 | AI | 4 | 96.69 | 96.03 | 95.70 | 95.14 | 93.47 |
|  | Non-AI | 4 | 95.86 | 92.36 | 89.01 | 86.36 | 83.27 |
| Case 2-10 | AI | 4 | 95.38 | 94.87 | 94.61 | 93.63 | 91.98 |
|  | Non-AI | 4 | 93.79 | 88.58 | 83.99 | 79.76 | 75.62 |
| Case 2-11 | AI | 4 | 94.70 | 94.13 | 93.00 | 92.40 | 91.13 |
|  | Non-AI | 4 | 92.60 | 85.30 | 79.30 | 74.43 | 69.47 |
| Case 2-12 | AI | 4 | 95.37 | 95.33 | 95.47 | 95.27 | 93.50 |
|  | Non-AI | 4 | 93.33 | 88.40 | 83.17 | 79.27 | 74.63 |
| Case 2-13 | AI | 4 | 97.40 | 97.20 | 97.37 | 96.80 | 96.60 |
|  | Non-AI | 4 | 96.60 | 93.50 | 90.53 | 86.47 | 83.67 |

Observation 9: If the AI model is trained with samples of single UE speed or mixed UE speeds, a little performance loss is observed as the UE speed in testing samples increases from 30km/h to 90km/h.

Observation 10: For the generalization evaluation, the AI based method can also achieve a better performance than that of the non-AI method in scenarios with mixed UE speeds.

Observation 11: A little performance loss is observed as the AI model is trained with a single UE speed and tested with mixed UE speeds.

- Conclusions

In this contribution, we discuss the potential specification impacts and enhancements for AI/ML based beam management. We have the following observations and proposals.

Observations:

Observation 1: If Top-K beams are predicted by the adopted AI model, a refined beam sweeping over these Top-K predicted beams may be needed to obtain an optimal beam, which is up to NW.

Observation 2: The spatial domain beam prediction with a fixed beam pattern achieves a sufficiently high performance with only 25% beam overhead being used.

Observation 3: With the same sampling rate on the whole beam space, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pairs prediction.

Observation 4: The case of AI generalization that model training and testing are performed in mixed datasets with different beam constructions can achieve a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>.

Observation 5: The AI based Tx beam prediction with the optimal Rx beam is slightly better than that of the AI based method with a fixed (e.g, first) Rx beam.

Observation 6: The AI/ML model can be utilized for spatial domain beam prediction, which can greatly reduce the RS overhead for measurement while maintain a high beam prediction accuracy in various beam constructions.

Observation 7: Compared with the case that Set B is fixed across training and inference, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

Observation 8: For AI/ML based temporal beam prediction, Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction under the optimal Rx beam or a fixed Rx beam.

Observation 9: If the AI model is trained with samples of single UE speed or mixed UE speeds, a little performance loss is observed as the UE speed in test samples increases from 30km/h to 90km/h.

Observation 10: For the generalization evaluation, the AI based method can also achieve a better performance than that of the non-AI method in scenarios with mixed UE speeds.

Observation 11: A little performance loss is observed as the AI model is trained with a single UE speed and tested with mixed UE speeds.

Proposals:

Proposal 1: RS overhead reduction should be considered as a basic KPI for evaluation and can be defined as 1-N/M, where N is the number of beams (or beam pairs) for measurement and M is the number of beams (or beam pairs) for prediction.

Proposal 2: In the beam pair prediction, it’s better to take measurement results of all Rx beams as the model input instead of performing any sampling at the Rx beam space.

Proposal 3: Different beam constructions in Set B (number/pattern of beams (pairs) in Set B, etc) should be considered for the evaluation of model generalization capability.

Proposal 4: The random direction straight-line trajectories in Option #4 can be adopted for modelling UE trajectory as a baseline.

- References

- R1-2205641, Feature lead summary #4 evaluation of AI/ML for beam management, Moderator (Samsung), 3GPP TSG RAN1#109-e, May 2022.

- R1-2208106, Feature lead summary #5 evaluation of AI/ML for beam management, Moderator (Samsung), 3GPP TSG RAN1#110, August 2022.

- R1-2210362, Feature lead summary #3 evaluation of AI/ML for beam management, Moderator (Samsung), 3GPP TSG RAN1#110bis-e, October 2022.

- R1-2211060, Discussion on other aspects for AI beam management, ZTE Corporation, 3GPP TSG RAN1#110bis-e, October 2022.

- Appendix

For dataset generation and performance evaluation for AI/ML in beam management, take the parameters (if applicable) in the following table for Dense Urban scenario for SLS.

Table I. Assumptions for Dense Urban scenario for AI/ML in beam management

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | For spatial domain beam prediction, 3km/h For time domain beam prediction: 30km/h (baseline), 60km/h (optional) Other values are not precluded |
| UE distribution | FFS UEs per sector/cell for evaluation. More UEs per sector/cell for data generation is not precluded. For spatial domain beam prediction: FFS: Option 1: 80% indoor ,20% outdoor as in TR 38.901 Option 2: 100% outdoor For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | · [One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline] · [Four panels: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ as optional] · Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | [Panel structure: (M,N,P) = (1,4,2)] · 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline · Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FFS: Option 1: Full buffer Option 2: FTP model Other options are not precluded |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | [40 dBm] |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

- UE trajectory model Option #4: Random direction straight-line trajectories. - Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed. - The initial UE location should be randomly drop within the following blue area where d1 is the minimum distance that UE should be away from the BS. - Each sector is a cell and that the cell association is geometry based. - During the simulation, inter-cell handover or switching should be disabled. - For training data generation - For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance. - The value of T (or D) can be further discussed - The trajectory sampling interval granularity depends on UE speed and it can be further discussed. - UE can move straightly along the entire trajectory, or - UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°] - If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated. - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded. - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.

Table IX.  Evaluation results for BM-Case1 and BM-Case2 without model generalization for DL Tx beam prediction or Tx-Rx beam pair prediction

|  |  |  | BM-Case 1 (the case 1-3 mentioned above) <T16 R1> case for Tx beam pair prediction, where 16 fixed Tx beams are measured by the optimal Rx beam. | BM-Case 2 (the case 2-13 mentioned above) Both training and testing with samples of UE speed of 30km/h, and Set B is same with Set A. |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of [beams/beam pairs] in Set A | 64 | 256 |
|  |  | Number of [beams/beam pairs] in Set B | 16 | 256 |
|  |  | Baseline scheme | Exhaustive beam sweeping | Exhaustive beam sweeping |
| AI/ML model input/output |  | Model input | RSRPs of selected beam pairs (16x1=16 beam pairs) | RSRPs of all measured Tx-Rx beam pairs of 5 past time instances |
|  |  | Model output | RSRPs of all beam pairs (64x1=64 beam pairs) | RSRPs of all Tx-Rx beam pairs of 5 future time instances |
| Data Size |  | Training | 72000 | 126000 |
|  |  | Testing | 4000 | 7000 |
| AI/ML model |  | [Short model description] | Full connected network with 4 layers | LSTM with 2 layers, full connected network with 2 layers |
|  |  | Model complexity | 0.149M | 2.758M |
|  |  | Computational complexity | 0.149M | 7.250M |
| Evaluation results [With AI/ML / baseline] | [Beam prediction accuracy (%)] | Top 1 | 88.25 | 83.9 |
|  |  | Top 4 | - | 97.4 |
|  |  | Top 5 | 99.75 | - |
|  | [L1-RSRP Diff] | Average L1-RSRP diff (dB) | 0.096 | 1.354 |
|  | [System performance] | [RS overhead Reduction (%)/ RS overhead] | 75% | 50% |