3GPP TSG RAN WG1 #111 	     		  R1-2211216

Toulouse, France, November 14th – 18th, 2022

Title:	[Draft] Reply LS on further questions on feMIMO RRC parameters

Response to: 	R1-2210808/R2-2211028

Release:	Rel-17

Work Items:	NR_feMIMO-Core

Source:	CATT [to be RAN WG1]

To:	RAN WG2

Cc:

Contact Person:

Name:

E-mail Address:

Attachments:

1. Overall Description:

RAN1 thanks RAN2 for the LS on further questions on feMIMO RRC parameters. Please find RAN1 answers on the questions below.

BWP and cell ID in unified TCI state for SRS For the case that the UL TCI state is present, RAN2 understands there are two alternatives associating the BWP and serving cell with the UL TCI state: - Add the separate fields BWP ID and Cell ID in srs-UL-TCIState-r17 - Clarify in the field description of srs-TCIState-r17 that the present UL-TCI-State is associated with the BWP and serving cell where the SRS-Config is configured. For the case that the Joint TCI state is present, RAN2 understands there are also two alternatives associating the BWP and serving cell with the Joint TCI state: - Add the separate fields BWP ID and Cell ID in srs-DlorJoint-TCIState-r17 - Clarify in the field description of srs-TCIState-r17 that the Joint TCI state is associated with the serving cell where the SRS-Config is configured and the current active DL BWP of this serving cell. Question 1 According to above RAN2 understandings, RAN2 would like to ask RAN1 to explain what the association of BWP and cell ID for srs-TCIState is.

Answer 1: BWP ID and Cell ID are supported to be configured in TCI-UL-State-r17 and TCI-State, which is flexible enough to allow cross-CC source RS configuration. In this way, it seems not necessary to further introduce BWP ID and Cell ID in srs-UL-TCIState-r17 or srs-DlorJoint-TCIState-r17. For both UL TCI state and Joint TCI state, Alt2 should be adopted.

codebookmode for codebookConfig Question 2 RAN2 would like to ask RAN1 - Is there a need to add the parameter codebookMode which selects mode1 or mode2 in the IE CodebookConfig-r17? - If a) is needed, which types of codebook the parameter is needed, for example: - typeI-SinglePanel-Group1-r17 - typeI-SinglePanel-Group2-r17 - typeII-PortSelection-r17 Additionally, if a) is needed, for each type of codebook that needs a parameter codebookMode, please confirm if a separate parameter is needed or if they share the same configuration of codebookMode.

Answer 2:

- Since the RRC parameter for CSI report mode configuration has been agreed in RAN1#106bis in Rel-17, the parameter codebookMode which selects mode1 or mode2 in the IE CodebookConfig-r17 is needed.

- Based the agreements on Rel-17 MTRP CSI, only the codebook type of ‘typeI-SinglePanel’ has been supported in Rel-17. Hence, only ‘typeI-SinglePanel-Group1-r17’ and ‘typeI-SinglePanel-Group2-r17’ need to add this new parameter.

- The common parameter for both ‘typeI-SinglePanel-Group1-r17’ and ‘typeI-SinglePanel-Group2-r17’ can be shared.

For configuring pathloss reference signal for unified TCI state RAN2 would like to inform RAN1 of how pathloss reference RS is configured for unified TCI state operation. In each BWP-UplinkDedicated UE may be configured with up to 64 pathloss reference RS by field pathlossReferenceRSToAddModList. Each joint or UL TCI state may be linked to one pathloss reference RS from this list. There is further MAC CE and DCI operation to indicate UE which pathloss reference RSs to follow. Question 3 RAN2 assumes RAN1 informs RAN2 in case the above described pathloss reference signal configuration for unified TCI state is not what RAN1 has intended.

Answer 3: The above described pathloss reference signal configuration for unified TCI state is what RAN1 has intended.

2. Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #112				Feb. 27 to Mar. 3, 2023		Athens, GR

TSG-RAN WG1 Meeting #112bis-e			Apr. 17 to Apr. 26, 2023		e-meeting