3GPP TSG RAN WG1 #111		  R1-2211221

Toulouse, France, November 14th – 18th, 2022

Agenda Item:     9.1.3.1

Source:	Spreadtrum Communications

Title:	Discussion on increased number of orthogonal DMRS ports

Document for:	Discussion and decision

Introduction

In RAN1#110bis-e meeting, RAN1 had an extensive discussion on increasing the maximum number of DMRS ports for PDSCH/PUSCH larger than Rel-15 for CP-OFDM. During the meeting, some important agreements have been achieved [1].

In this contribution, we provide our view on the detailed design issues of Rel-18 DMRS ports.

Discussion

In the last meeting, larger FD-OCC was supported to increase the number of DMRS ports for PDSCH/PUSCH. The corresponding agreement can be found below,

Agreement Confirm the working assumption in RAN1#110 with the following update: To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)). - FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2. - FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options).

According to the agreement, there’s an FFS on whether to support additional options to handle potential performance issues of Opt.1. In our views, we don’t think another option is needed. First of all, based on the simulation results provided during previous meetings, there’s no consensus on whether the performance loss between FD-OCC and other options is large enough to support additional schemes. Besides, if multiple schemes are supported, it will bring additional issues on whether/how to support MU scheduling between DMRS ports from different schemes. In summary, supporting multiple schemes will complicate the specification and also increase the work load too much.

Proposal 1: For Rel.18 DMRS enhancement, additional options other than Opt.1 is not supported.

Regarding FD-OCC code design, based on the agreement below, RAN1 will down select one from two options in this meeting.

Agreement For FD-OCC length 4 for DMRS of PDSCH/PUSCH for Rel.18 eType 1/eType 2 DMRS, support one from the following FD-OCCs (to be selected in RAN1#111): - Opt.1-1: Walsh matrix (Hadamard code): FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +1; -1; -1. 3; +1; -1; -1; +1. - Opt.1-2: Cyclic shift with {0, π, π/2, 3π/2}: FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +j; -1; -j. 3; +1; -j; -1; +j.

In our views, it is more desired to follow the legacy rule of Rel.15 DMRS. Besides, FD-OCC length 4 with Walsh matrix has been supported for other RS such as CSI-RS. From implementation perspective, Walsh matrix includes only +1 and -1 which will simplify the spreading/de-spreading operation. Therefore, Opt.1-1 is preferred.

Proposal 2: For FD-OCC length 4 for DMRS, Opt.1-1 is preferred.

During the last meeting, some companies raised concern on Walsh matrix based FD-OCC due to gNB implementation complexity. We are open to discuss whether to support cyclic shift based FD-OCC for UL.

MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports by different CDM groups has been supported with no scheduling restriction. For MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group, we think it is beneficial to bring additional scheduling flexibility.

In order to enable MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group, some scheduling restriction is needed to guarantee that the UE using Rel.15 DMRS can perform channel estimation with legacy algorithm. Which means although the co-scheduled UE is actually using Rel.18 DMRS, the UE using Rel.15 DMRS can assume that the co-scheduled UE is also using Rel.15 DMRS. Similarly, the UE using Rel.18 DMRS can assume that the co-scheduled UE is also using Rel.18 DMRS. In summary, MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group can be achieved by scheduling restriction at gNB and does not require additional enhancement for legacy UEs and Rel.18 UEs.

Specifically, for MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group, only the FD-OCC sequences [+1 +1 +1 +1] and [+1 -1 +1 -1] can be used for UE scheduled with Rel.18 DMRS. For example, if a UE is scheduled with Rel.15 DMRS using FD-OCC [+1 +1], another UE can be co-scheduled with Rel.18 DMRS using FD-OCC [+1 -1 +1 -1].

Observation 1: MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group can be achieved by scheduling restriction at gNB side and does not require additional enhancement for legacy UEs and Rel.18 UEs.

During the offline discussion, how to design antenna ports tables for Rel.18 DMRS for PDSCH has been discussed [2]. The last version of FL proposal#2.6a before the end of the last meeting is listed below.

FL proposal#2.6a: - Down select one of the following on how to enhance TS38.212 to indicate Rel.18 DMRS ports for PDSCH. - Scheme A: Specify new antenna ports tables similar to Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A in TS38.212. The maximum size of antenna ports field is increased by M (M>=0) bit(s). - For M>= 1, existing rows in Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A in TS38.212 are partially/fully copied to the new tables except for “Reserved” row. - FFS for other rows in the new tables. - FFS: The sizes of antenna port field and its mapping to antenna port tables. - Scheme B: Reuse the existing Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A in TS38.212 and keep the size of antenna ports field in DCI unchanged. Introduce new M(M>=1)-bit DCI field of “DMRS port(s) offset indicator” to indicate Rel.18 DMRS ports. - At least M=1 is supported. For M=1, - If “DMRS port(s) offset indicator” field is set “0”, DMRS port(s) are the same as indicated by antenna ports field in DCI format 1_1/1_2. - If “DMRS port(s) offset indicator” field is set “1”, DMRS port(s) are incremented with X from the indicated DMRS port(s) by antenna ports field in DCI format 1_1/1_2. - Value of X is 8 for Rel.18 eType 1 DMRS and X is 12 for Rel.18 eType 2 DMRS. - FFS: Whether/how to enhance the reserved field in antenna ports tables under different values of “DMRS port(s) offset indicator”. - FFS: Whether to support M>1 and its DMRS port combinations under different values of “DMRS port(s) offset indicator”. - Scheme C: Reuse the existing Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A in TS38.212 and keep the size of antenna ports field in DCI unchanged. Introduce new table to indicate Rel.18 DMRS ports including full 8/16 or 12/24 ports. - TDRA entry configured includes a entry indicate what DMRS ports is used for scheduling. - Scheme D: Reuse the existing Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A in TS38.212 and keep the size of antenna ports field in DCI unchanged. Introduce new tables to indicate Rel.18 DMRS ports with new DMRS port index. - At least one Rel-18 DMRS port with the new port index p is included in each row - FFS: the combination of Rel-18 DMRS ports with the new port index and legacy port index in one row - FFS: MU restrictions with the determined tables for DMRS ports indications. - FFS: How to enhance antenna ports tables in TS38.212 to indicate Rel.18 DMRS ports for PUSCH for rank = 1,2,3,4.

First of all, among the four candidate schemes, Scheme A will only use new antenna tables, Scheme B will only use old antenna tables with potential new rows, and Scheme C/D will use both old antenna tables and new antenna tables.

Scheme A is more straightforward and the same DMRS port indication mechanism is reused for Rel.18 DMRS. Besides, considering the possibility that not all of the DMRS port combinations in existing tables are needed, Scheme A can achieve a more compact design.

For Scheme B, we think fully reusing the existing tables with additional DMRS port index offset is not flexible enough. The reason is that based on Scheme B, the indicated DMRS port combinations within a CDM group cannot include both legacy port index values and new port index values. In order to solve this issue, the reserved fields of the tables need to be changed to new DMRS port index combinations. However, the number of reserved fields in each table may not be enough. For example, there’s no reserved field in Table 7.3.1.2.2-2A for single codeword.

For Scheme C, since the new tables for Rel.18 DMRS ports includes full 8/16 or 12/24 ports, we think TDRA based switching is to achieve switching between Rel.15 DMRS ports and Rel.18 DMRS ports, rather than table switching for Rel.18 DMRS ports. So Scheme C is not valid compared with other schemes. If the intention is using only new tables for Rel.18 DMRS port indication, the new tables design looks similar as Scheme A.

For Scheme D, the new DMRS port combinations will be categorized into new DMRS tables. However, the description is not complete since how to indicate DMRS ports from the new tables is not clear. Based on our understanding, if table switching is achieved by DCI, a new DCI field should be introduced. Compared with Scheme A, the difference is that each antenna ports table from Scheme A will be split into two tables, and the DMRS port indication mechanism will be complicated with no clear benefit.

Proposal 3: Regarding the indication of Rel.18 DMRS ports for PDSCH, support Scheme A.

On the antenna ports table design for > 4 layers PUSCH, RAN1 has discussed offline and the latest FL proposal#3.4b is copied below.

FL proposal#3.4b: - For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS. - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are down selected from the following: - Alt.1-1: Same DMRS port combinations as that for rank = 5,6,7,8 for PDSCH are reused at least for full or non-coherent UL codebook. - FFS: whether all or some/one of the current DMRS port combination(s) are reused. - Alt.1-2: New DMRS port combinations are used for rank = 5,6,7,8 (FFS: details). - For Rel.18 eType1/eType2 DMRS ports, - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details). - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked. - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group. - FFS: One or more than one DMRS port combination(s) for each rank and TPMI - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17. - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports.

Although it was not endorsed, it’s relatively stable. So, maybe we can endorse it quickly and discuss further details.

For Type 1/Type 2 Rel.15 DMRS ports, since only SU-MIMO is assumed for > 4 layers PUSCH, we think the DMRS port combinations for DL can be reused for UL, at least for full or non-coherent UL codebook. For partial coherent UL codebook, since the codebook has not been finished, we prefer to wait for the progress in 9.1.4.2.

Regarding the DMRS port indication for > 4 layers PUSCH, we prefer to reuse legacy rule that DMRS is indicated from ports combinations with total ports number equals to the number of layers indicated by TPMI/SRI.

Proposal 4: For > 4 layers PUSCH based on Type 1/Type 2 Rel.15 DMRS ports, support Alt1-1.

Conclusion

In this contribution, we gave our views on the detailed design of Rel-18 DMRS ports. The following proposals and observation are achieved:

Proposal 1: For Rel.18 DMRS enhancement, additional options other than Opt.1 is not supported.

Proposal 2: For FD-OCC length 4 for DMRS, Opt.1-1 is preferred.

Observation 1: MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group can be achieved by scheduling restriction at gNB side and does not require additional enhancement for legacy UEs and Rel.18 UEs.

Proposal 3: Regarding the indication of Rel.18 DMRS ports for PDSCH, support Scheme A.

Proposal 4: For > 4 layers PUSCH based on Type 1/Type 2 Rel.15 DMRS ports, support Alt1-1.

References

[1] Chair's notes RAN1_110bis-e v17

[2] R1-2210408, FL summary on DMRS#4, Moderator (NTT DOCOMO)