3GPP TSG RAN WG1#111                                                      	            R1- 2211297

Toulouse, France, Nov. 14th –18th, 2022

Agenda Item:	9.12.1

Source:	Lenovo

Title:	Discussion on L1 enhancements for inter-cell beam management

Document for:	Discussion

Introduction

One of the Rel-18 Mobility WI scops is to support L1/L2 signaling based as approved in RP-222332[1]:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In RAN1#110bis, the following agreements and assumptions were reached[2]

Agreement - For Rel-18 L1/L2 mobility, L1 intra-frequency measurement for candidate cell is supported - At least the following aspects are for RAN1 further study: - RAN1 assumes Rel-17 ICBM CSI measurement as starting point. - Whether and how to apply relaxation for the restrictions imposed on the Rel-17 intra-frequency L1 non-serving cell measurement defined in 9.13.2 of TS38.133, where RAN4 impact is foreseen, e.g. - SFN offset alignment compared with serving cell - BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP - Introduction of symbol level gap or SMTC for larger Rx timing difference (i.e. larger than CP length) - Commonality with intra-frequency L3 measurement - Commonality with L1 inter-frequency measurement for measurement configuration - Send an LS to RAN4 (CC RAN2) - RAN1 to ask RAN4 if the restriction on e.g., SFN offset alignment, BWP setting and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not. - RAN1 assumes Rel-17 ICBM CSI measurement as starting point. Agreement - For Rel-18 L1/L2 mobility, - SSB is supported for L1 intra-frequency measurement - SSB is supported for L1 inter-frequency measurement if inter-frequency L1 measurements are supported - Further study the following L1 measurement RS for candidate cell - CSI-RS for tracking, beam management, CSI and mobility, CSI-IM, which is for L1 intra-frequency and L1 inter-frequency (if supported) Agreement - For candidate cell measurement for Rel-18 L1/L2 mobility, - L1-RSRP is supported for intra-frequency candidate cell measurement. - Further study the following measurement quantities for candidate cell measurement - L1-RSRP for inter-frequency (if supported) - L1-SINR for intra-frequency and inter-frequency (if supported) - FFS: to assess the use case and the benefit of UL measurement instead of/in addition to DL L1 measurement, which includes: - How the UL measurement result is used, e.g. handover decision - Signals/channels used for UL measurement, e.g. SRS - Spec impact including other WGs, e.g. definition of gNB measurement, interface to transfer RS configuration or measurement results - Note: The next discussion will take place based on companies’ contribution in future meeting. Agreement - For Rel-18 L1/L2 mobility, further study the potential RAN1 spec impact of L1 inter-frequency measurement - The definition and scenarios of L1 inter-frequency measurement is determined by RAN4, and RAN1 assumes at least the following until receiving their confirmation - The scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios: - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE. - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE - At least the following aspect is studied: - Commonality with L1 intra-frequency measurement for measurement configuration - Send an LS to RAN4 (CC RAN2) - RAN1 would like to confirm our understanding that the supported scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios: - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE. - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE - It is RAN1 understanding that the introduction of measurement gap and SMTC for L1 inter-frequency measurement, if any, is expected to be a RAN4 issue - Note: this content is included in the LS agreed for intra-frequency L1 measurement Agreement - For L1 measurement report for Rel-18 L1/L2 mobility, further study the following mechanisms: - Report as UCI on PUCCH or PUSCH - Periodic report on PUCCH, semi-persistent report on PUCCH/PUSCH, and aperiodic report on PUSCH - Potential enhancements to Rel-17 ICBM report format to accommodate Rel-18 scenarios, e.g. - Inter-frequency measurement, if supported - Increasing the maximum number of reported beams, which is 4 for Rel-17 ICBM - Flexible size beam report, e.g., two-part UCI (e.g., the 1st part contains the best beam/cell and the number (e.g., N) of reported beams/cells, the 2nd part contains the rest (N-1) beams/cells - Reducing the reporting overhead by e.g. choosing beams/cells per frequency or across frequencies to report (FFS how) - Report on MAC CE - Both gNB scheduled and/or UE initiated (if supported) report are studied Agreement RAN1 to further study if the beam indication of candidate cell(s) L1/L2 mobility should be designed for a specific TCI framework below, and their potential RAN1 spec impact. - Option A: Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-17 TCI framework mechanism - Option B: Beam indication for Rel-18 L1/L2 mobility is designed based on Rel-15 TCI framework mechanism - Option C: Beam indication for Rel-18 L1/L2 mobility is designed based on both Rel-15 and Rel-17 TCI framework mechanisms Agreement Send an LS to RAN2/RAN3 asking the clarification on intra-/inter-DU scenario: - RAN1 has started the discussion on the configuration for L1 measurement and TCI states for candidate cells. Regarding the following RAN2 agreements captured in RAN2 LS (R1-2208331/R2-2209257), it is not clear for RAN1 which kind of information/configuration for candidate cell(s) are available at a serving cell for inter-DU case for Rel-18 L1/L2 mobility. Thus, companies have different understanding on the implication of the sentence “as much commonality as reasonable” in the LS. - The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different. - RAN1 respectfully asks RAN2 and RAN3 if the serving DU knows the measurement RS configuration and TCI state configuration of cells served by another DU. Agreement From RAN1 perspective, the following scenarios can be considered for Rel-18 L1/L2 mobility for beam indication timing. This will be updated depending on further RAN1 assessment and RAN2 decision on the time chart - Scenario 1: Beam indication before cell switch command - Scenario 2: Beam indication together with cell switch command - Scenario 3: Beam indication after cell switch command Interested companies are encouraged to further study the validity of the scenarios and the potential spec impact. Agreement Interested companies are encouraged to perform technical analysis of the cell switch command from a RAN1 point of view, e.g. - Necessary information included in the command, which is relevant for RAN1 discussion - Necessary number of bits for the information - L1 impact or concern to use DCI or MAC CE for L1/L2 cell switch command Agreement RAN1 to further study the potential RAN1 enhancements and spec impact to perform at least the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay / interruption - DL synchronization for candidate cell(s) - TRS tracking for candidate cell(s) - CSI acquisition for candidate cell(s) - Activation/Selection of TCI states for candidate cell(s), if feasible - Note: Uplink synchronization aspect will not be discussed under this A.I. - FFS: Whether the above procedures prior to the reception of L1/L2 cell switch command can be performed on candidate cell when it is deactivated SCell (if defined in RAN2) Detailed discussion will be commenced after receiving RAN2 LS.

In this contribution, we provide our views on L1 enhancements including beam reporting, beam indication, and dynamic switching mechanism among candidate serving cells.

Discussion

Dynamic serving cell change

It has been agreed that Rel-17 ICBM will be taken as the starting point of Rel-18 L1/L2 Mobility. In Rel-17 ICBM, the UE can be indicated with a joint TCI state or DL TCI state associated with a SSB associated with a PCI different from the serving PCI for all PDSCH and UE-dedicated PDCCH reception without serving cell change. And a Joint TCI state or UL TCI state associated with an additional SSB can be indicated for a UE in a BWP of a serving cell for the UE to transmit all the PUSCH and PUCCH to a TRP associated with an additional PCI. Even when the cell quality of the additional cell that is associated with the indicated TCI state is better than the serving cell, if the gNB can only send the handover command via RRC reconfiguration message, this leads to larger handover latency.

When the network detects that the quality of a candidate cell is better than the serving cell, a serving cell change command can be signaled to the UE along with the unified TCI indication signaling. For example, the serving cell change command can be indicated in the DCI with format 1_1/1_2 along with the TCI state change when a TCI state associated with an additional PCI is indicated. A new DCI field or a reserved DCI field, which can be used for DCI based TCI indication without DL assignment, can be used to indicate whether the serving cell is changed to the additional cell associated with the indicated TCI state. Further, RAN2 [3] has agreed that the candidate configuration index, TCI state and the BWP ID should be included in the serving cell change command, all of those signaling has been included in the unified TCI state indication DCI format 1_1/1_2. For example, the candidate configuration index can be associated with a PCI. Thus, at lease when the UE is configured with unified TCI framework, serving cell change command can be signaled by DCI.

- Support DCI based serving cell change at least for undifed TCI framework in NR Rel-18.

Inter-frequency measurement for L1/L2 mobility

In the last RAN2 meeting, RAN2 has confirmed to support L1/L2 mobility in inter-frerquency scenario. For example, the UE can perform an inter-frequency handover based on L1/L2 signaling. Inter-frequency L1 measurement is required to support such scenario. Inter-frequency mobility can be supported by legacy L3 procedure by employing inter-frequency RRM measurement, where measurement gap is required. To support inter-frequency L1 measurement, measurement gap for RF retuning is needed as well. For example, when an aperiodic beam measurement and beam report for candidate cells in another frequency is triggered by a DCI, a gap is required for the UE to switch to the frequency of the indicated candidate cell for performing beam measurement. And the UE cannot transmit and receives signals during this gap. How to determine the meaurement gap and the corresponding UE behavior can be further studied.

- Support measurement gap for L1 beam measurement for inter-frequency L1/L2 mobility. Details are FFS.

Beam measurement and beam report for L1/L2 signaling based serving cell change

One study point of L1 measurement for L1/L2 mobility is whether RS other than SSB can be used for L1 measurement of candidate cells. In R17 inter-cell mobility, only SSB from non-serving cell is used for inter-cell measurement. The benefit for reusing SSB for inter-cell measurement in R18 is that SSB is independent of BWP configuration of the non-serving cell. This makes the CSI configuration and CSI measurement easier for inter-cell mobility. In legacy L3 based mobility, CSI-RS for mobility can be used for L3 measurement for neighboring cells. L1/L2 based mobility focuses primarily on FR2 for handover latency reduction when a large number of potential beams are to be measured. In order to reduce the HO latency, the TCI state used for the target cell is expected to be configured before or along with the serving cell change command. The TCI state is used for PDSCH reception as well as PUSCH transmission. In Rel-17 QCL framework, the indicated TCI state shall indicate a QCL-TypeA TRS as well as a QCL-TypeD RS, which can be a TRS or a CSI-RS for beam management, and the QCL-TypeA TRS and the QCL-TypeD RS can be further QCLed with an SSB of the serving cell or an SSB of an additional cell.

If CSI-RS for mobility are used for L1 measurement for candidate cells, further measurement on SSB of the same candidate cell may still be needed for QCL configuration. However, this additional complexity can be compensated by performing CSI-RS-based measurements more frequently, which reduces the measurement gap overhead.

Furthermore, in RAN1#110bis, some companies also proposed to measure the CSI-RS for beam management, or TRS of the candidate cells, since they can be directly indicated as the source QCL RS for the data transmission in the target cell. Again, the use of CSI-RS in addition to SSB may allow reducing the measurement gap overhead.

- Support using CSI-RS for candidate cell measurements in addition to SSB.

It was agreed in RAN2#119 that L1/L2-centric serving cell change shall be supported based on L1 measurements. In Rel-17, SSB associated with additional PCIs can be configured in the CMR of a CSI-ReportConfig for non-serving cell beam measurement, and the UE can report one or more SSBRIs associated with additional PCI and the corresponding L1-RSRP in a CSI report. In our opinion, this feature should be the starting point for the L1 measurement for L1/L2 mobility triggering.

As pointed out by some companies in RAN2#119, robustness of  L1/L2-centric serving cell change should be ensured. In legacy systems, L3 filtered RSRP measurement results as well as L3 cell-level measurement results are used for L3 signaling based handover decision. Similar mechanism can be considered in L1 measurement for L1/L2 mobility triggering to avoid a ping-pong issue. Time domain filtering as well as spatial domain filtering can be introduced. For example, filtered L1 measurement of non-serving SSBs over multiple measurement occasions can be reported in a beam report to help the gNB to make decision on whether the serving cell should be changed. In additional, cell-level measurements of one or more candidate cells can be reported in a beam report according to gNB configuration. How to obtain the filtered and cell-level results can be further studied.

- Study to introduce filtered L1-RSRP and cell-level measurement results in L1 beam reporting for L1/L2 mobility triggering.

Rel-17 beam reporting is fully controlled by the network, for example period beam report is configured by RRC, semi-persistent beam report can be activated by DCI or MAC CE, and aperiodic beam report is triggered by DCI. In order to quickly identify a potential target cell, shorter beam report periodicity is expected. However, that leads to higher UE power consumption and UL resource overhead. Candidate beam identification procedure is similar with beam failure detection procedure specified in Rel-17. The UE only needs to report the measurement result when some predefined conditions are satisfied. In addition to the Rel-17 L1 beam report, event-based beam reporting can be considered for L1/L2-centric serving cell change since the UE may have the full measurement results for multiple candidate cells.

Measuring and reporting candidate cells depends on the UE mobility. When the UE is at or near the center of its serving cell, the L1-RSRPs measured from candidate cells are likely insignificant and are not reported. When the UE is in the overlapping area between the serving cell and a candidate cell near the edge of the serving cell, it is more likely to report L1-RSRP of RSs sent from candidate cells. To always reserve sufficient PUCCH resources for reporting measurements from both the serving cell and candidate cells will be a waste of PUCCH resources. Event-based measurement reporting enables a UE to report L1-RSRP from candidate cell using MAC-CE or RACH when the measured L1-RSRP exceeds a certain threshold. This is a good compromise between reducing the dedicated UL resources and reducing the report latency.

- Support event-based beam report for L1/L2 mobility.

Conclusion

Proposal 1	Support DCI based serving cell change at least for undifed TCI framework in NR Rel-18.

Proposal 2	Support measurement gap for L1 beam measurement for inter-frequency L1/L2 mobility. Details are FFS.

Proposal 3	Support using CSI-RS for candidate cell measurements in addition to SSB.

Proposal 4	Study to introduce filtered L1-RSRP and cell-level measurement results in L1 beam reporting for L1/L2 mobility triggering.

Proposal 5	Support event-based beam report for L1/L2 mobility.

References

- RP-222332 Revised WID on Further NR mobility enhancements.

- Chairman Note for RAN1#110bis

- R1-2210810(R2-2211061) LS on RAN2 agreements about L1/L2-triggered mobility (LTM)