3GPP TSG RAN WG1 Meeting #111 			                             R1-2211389

Toulouse, France, November 14th  – 18th, 2022

Agenda Item:	9.10.1

Source: 	Intel Corporation

Title:	Discussion on XR specific power saving techniques

Document for:        Discussion/Decision

- Introduction

In RAN1 # 110bis-e, the following were agreed with regards to power saving enhancements for XR [1]

Agreement For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1) - Prioritize semi-static solutions - FFS: Whether dynamic solutions will be also needed R1-2210338 Moderator Summary#2 on XR specific power saving techniques Moderator (Qualcomm Incorporated) Conclusion “Retransmission-less CG for UL pose transmission (Item 3.3-5)” is a RAN2 issue, leave the discussion to RAN2, RAN1 does not further investigate the issue. - Note: how to capture evaluation results and findings will be separately discussed Conclusion RAN1 does not further study jitter handling by LP-WUS (Item 2.2-4) in Rel-18 XR SI - Note: how to capture evaluation results and findings will be separately discussed Conclusion In addition to the values for jitter in Table 5.1-2 in TR 38.838, the following statistical parameters for jitter can also be optionally evaluated in Rel-18 XR SI. - Note: This optional assumption is not applicable to the evaluation of 90 FPS and above Parameter; unit; Optional value for evaluation. Mean; ms; 0. STD; ms; 5. Truncation range; ms; [-8, 8]. Conclusion RAN1 deprioritizes DCP indicated SSSG switching (Item 3.3-4) - Note: how to capture evaluation results and findings will be separately discussed

In this contribution, we discuss our views on potential power saving enhancements for XR.

- C-DRX enhancements

Due to periodic nature of XR packet arrival, C-DRX already seems to be a useful power saving method that can be deployed. The typical XR frame rates are 60, 120 frames per seconds (fps), of which frame periodicities are 16.67ms, 8.33ms., Existing C-DRX cycle lengths (multiple of 1ms) do not align with XR traffic periodicity. Figure 1 shows an example where XR frame arrival rate is 16.67ms and closest CDRX cycle length supported by Rel-15 is 16ms. Hence, even if ON duration of a cycle is aligned with burst arrival, every N cycles, burst could miss ON duration, where value of N depends on length of ON duration. Hence, XR latency may increase as in some cases, since packet scheduling may be delayed to next DRX cycle which is approximately 16ms later whereas XR packet PDB can be as low as 10ms.

Figure 1: CDRX cycle length does not quite match with XR frame arrival rate.

RAN1 already recommended to identify a solution to align CDRX with XR traffic periodicity. Several options can be considered such as non-integer (in ms) C-DRX cycle, or adaptation of C-DRX start offset by a pattern. In Table 1, we show performance of enhanced DRX (e-DRX) where a pattern (start offset of DRX ON is adjusted every 6th (3rd )cycle for DRX config {8,6,6} and {16, 4, 14}, respectively) is adopted to match with XR traffic arrival with respect to legacy C-DRX, for both jitter ON and OFF. As expected, with jitter ON, performance degradation occurs, however it is still better than using legacy CDRX. Presence of jitter results in non-alignment even if DRX cycle start offset is adjusted due to non-integer periodicity of XR traffic.  Hence, it is critical that range of ON duration covers jitter range as much as possible.

Observation 1: Capture the following into the TR:

- For DRX (cycle 8ms, IAT 6ms, ON 6ms), 6% (4.4%) and 2.7%  (3%) power saving gain (capacity gain) are observed with semi-static C-DRX alignment compared to legacy C-DRX for jitter OFF and jitter ON, respectively, for PDB 10ms.

On the other hand, there was some discussion in RAN1 # 110bis-e on whether dynamic indication of start offset and ON duration is necessary to address shift in center point of jitter range, so that C-DRX can be aligned better with pseudo-periodic XR traffic. In our view, gNB is expected to know jitter statistics and center point of jitter range is not expected to change dynamically. 5GC is expected to provide traffic jitter information to NG-RAN (as per SA2 discussion) and in our view, configuring ON duration to cover jitter range (based on jitter statistics and/or jitter information from 5GC) is expected to address the issue. Beyond that, we are not convinced dynamic adjustment would provide meaningful power saving gain.

Proposal 1: RAN1 deprioritizes dynamic adjustment of CDRX for alignment with XR traffic periodicity .

Table 1: Power consumption and capacity performance of enhanced DRX using semi-static CDRX alignment and legacy C-DRX. Results are presented for VR and CG in Dense Urban scenario, 4 UEs, DL only with SU-MIMO scheduler. DRX (Cycle, IAT, ON).

| Power Saving Scheme |  | Jitter ON |  |  |  |  |  | Jitter OFF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Power consumption (PC) |  |  |  | #satisfied UEs per cell/ #UEs per cell PDB 10ms | #satisfied UEs per cell/ #UEs per cell PDB 15ms | Power consumption (PC) |  |  |  | #satisfied UEs per cell/ #UEs per cell PDB 10ms | #satisfied UEs per cell/ #UEs per cell PDB 15ms |
|  |  | Baseline | Optional |  |  |  |  | Baseline | Optional |  |  |  |  |
|  |  | Mean PC | PC of 5%-tile UE in PC CDF | PC of 50%-tile UE in PC CDF | PC of 95%-tile UE in PC CDF |  |  | Mean PC | PC of 5%-tile UE in PC CDF | PC of 50%-tile UE in PC CDF | PC of 95%-tile UE in PC CDF |  |  |
| Always On |  | 116.76 | 108.24 | 116.13 | 127.31 | 3.93/4 | 3.95/4 | 115.19 | 107.43 | 114.42 | 126.9 | 3.97/4 | 3.99/4 |
| Legacy CDRX | DRX (8,6,6) | 111.36 | 98.98 | 109.41 | 127.46 | 3.72/4 | 3.81/4 | 109.67 | 98.19 | 107.4 | 126.03 | 3.80/4 | 3.89/4 |
|  | DRX (16,4,14) | 112.99 | 102.69 | 111.63 | 126.18 | 3.82/4 | 3.89/4 | 111.58 | 101.94 | 109.78 | 125.95 | 3.84/4 | 3.93/4 |
| w/ eDRX | DRX (8,6,6) | 108.36 | 98.04 | 107.36 | 121.26 | 3.83/4 | 3.9/4 | 103.05 | 93.34 | 102.26 | 116.97 | 3.97/4 | 3.99/4 |
|  | DRX (16,4,14) | 111.43 | 102.32 | 110.63 | 123.13 | 3.89/4 | 3.93/4 | 105.55 | 97.76 | 104.77 | 117.26 | 3.97/4 | 3.99/4 |

On the other hand, XR media may include multiple flows, each of which may have different periodicity and requirement. Hence, a given DRX configuration may not adapt well to the characteristics of multiple flows. To this end, multiple DRX configurations can be activated to address the issue.

Proposal 2: RAN1 studies multiple active DRX configurations to support XR traffic with multiple flows.

- Jitter handling

Jitter may cause extra delay to XR traffic or extra power consumption, depending on Jitter range and CDRX configuration. Jitter can be compensated if ON duration of long-DRX cycle can be configured to cover the jitter range. However, just configuring a longer ON duration may increase power consumption due to PDCCH monitoring activity over a longer ON duration.

In our view, Rel17 PDCCH monitoring adaptation solution can be considered as starting point. For example, inside the ON duration, UE could start with sparse PDCCH monitoring, and once traffic is scheduled, UE could switch to frequent monitoring, such as based on scheduling DCI indication.

Proposal 3: For jitter handling, Rel17 PDCCH monitoring adaptation solution seems sufficient.

- UE could start with sparse PDCCH monitoring, and once traffic is scheduled, UE could switch to frequent PDCCH monitoring

- PDCCH monitoring enhancements

Similar to the discussion in previous section, PDCCH monitoring activity should be concentrated over a window where XR packet arrival is expected. However, non-integer PDCCH monitoring periodicity is not supported till Rel-17. Hence, some adaptation to PDCCH monitoring periodicity, such as shown in Figure 2, can be considered so that UE could receive the scheduling DCIs during the monitoring window only.

Figure 2: PDCCH monitoring window should include expected XR packet arrival occasions.

However, Rel-17 schemes including both PDCCH skipping and SSSG switching, offers good flexibility in achieving different patterns of PDCCH monitoring adaptation, which in our view, can be adapted to XR traffic characteristics. An example is shown in Figure 3 on how Rel-17 solution can be applied.

- If PDCCH skipping is used only, scheduling DCI may indicate suitable skipping duration

- If SSSG switching is used, UE may be indicated to switch SSSG when adaptation in monitoring slot periodicity/offset is needed

Figure 3: Application of Rel-17 PDCCH monitoring adaptation solution for XR applications.

Observation 2: Rel-17 schemes including both PDCCH skipping and SSSG switching, offer good flexibility in achieving different patterns of PDCCH monitoring adaptation, which in our view, can be adapted to XR traffic characteristics.

On the other hand, it is imperative that UE enters sleep mode upon end of XR packet transmission. Currently, Rel-17 supports scheduling DCI based PDCCH skipping solution. It was discussed during Rel-17 that PDCCH skipping needs to consider retransmission. Since PDB of XR media can be short, it may not be desirable that retransmission is delayed since skipping starts from next slot of DCI reception. One option can be to support non-scheduling DCI providing the trigger for PDCCH skipping, i.e., when packet delivery is complete gNB can use a non-scheduling DCI to indicate PDCCH skipping so that UE can enter sleep state immediately. On the other hand, Rel-17 PDCCH monitoring adaptation solution does not support interaction of retransmission handling and PDCCH skipping. Since XR traffic is latency sensitive, there may be a need the PDCCH skipping starts after XR packet delivery is successfully completed. Instead of UE determining when to start PDCCH skipping such as based on outcome of ACK or NACK, we think it is much cleaner solution if gNB explicitly indicates PDCCH trigger with immediate effect. To this end, we think non-scheduling DCI also helps with protecting retransmissions. On the other hand, non-scheduling DCI can also support indication from more than three skipping durations, such as by repurposing some existing fields. This provides gNB added flexibility to further improve power saving at the UE.

Proposal 4: RAN1 considers supporting non-scheduling DCI to trigger PDCCH monitoring enhancements.

- Conclusions

In this contribution, we discuss the potential enhancements for XR power saving. The following proposals and observations are provided.

Observation 1: Capture the following into the TR:

- For DRX (cycle 8ms, IAT 6ms, ON 6ms), 6% (4.4%) and 2.7%  (3%) power saving gain (capacity gain) are observed with semi-static C-DRX alignment compared to legacy C-DRX for jitter OFF and jitter ON, respectively, for PDB 10ms.

Observation 2: Rel-17 schemes including both PDCCH skipping and SSSG switching, offer good flexibility in achieving different patterns of PDCCH monitoring adaptation, which in our view, can be adapted to XR traffic characteristics.

Proposal 1: RAN1 deprioritizes dynamic adjustment of CDRX for alignment with XR traffic periodicity .

Proposal 2: RAN1 studies multiple active DRX configurations to support XE media with multiple flows.

Proposal 3: For jitter handling, Rel17 PDCCH monitoring adaptation solution seems sufficient.

- UE could start with sparse PDCCH monitoring, and once traffic is scheduled, UE could switch to frequent PDCCH monitoring

Proposal 4: RAN1 considers supporting non-scheduling DCI to trigger PDCCH monitoring enhancements.

# Reference

[1] RAN1 # 110bis-e Chairman Notes