3GPP TSG RAN WG1 #111	    			            R1-2212018

Toulouse, France, November 14th – November 18th, 2022

Agenda item:	8.1

Source: 	Samsung

Title: 	SRS CLPC for unified TCI framework

Document for:	Discussion and Decision

# Introduction

In RAN1#110b-e, the issue of how to determine the CLPC index for SRS when using the unified TCI framework was discussed. There was no consensus as documented in [1]. The following options were considered [1]:

Option-1: On SRS close loop in unified TCI framework, per serving cell - If PUSCH -config is configured, only 2 close loops for SRS are supported o Candidate values of 'i0' and 'i1' in closedLoopIndex -r17 for SRS refers to first and second close loop; - Otherwise, only one close loop for SRS is supported Interpretation: - For the Case 1 that PUSCH is configured with i0 in a TCI state, and SRS is configured with i1 in the same TCI state, a DCI format 0_x would update the CLPC ID for PUSCH and/or SRS, depending on what is scheduled. If both PUSCH and SRS are scheduled, both closed loops would be updated. - For the Case 2 that PUSCH is configured with i0 in a TCI state, and SRS is configured with i0 in the same TCI state, a DCI format 0_x would update CLPC ID 0, irrespective of if PUSCH or SRS is scheduled - For the Case 3 that PUSCH is not configured in a CC, and SRS is configured with i0 in a TCI state, a DCI format 0_x would update CLPC ID 0 when SRS is scheduled

Option-2B (reusing existing parameter): On SRS close loop in unified TCI framework, - If srs-PowerControlAdjustmentStates is set to 'separateClosedLoop ' in a SRS resource set, the SRS is associated with a separate close loop; - Otherwise, closedLoopIndex -r17 for SRS in a joint/UL-TCI state is to indicate a SRS close loop tied with PUSCH o Note: In such case, candidate values of 'i0' and 'i1' in closedLoopIndex -r17 for SRS refers to first and second close loop tied with PUSCH

In this document we present our views on how the SRS CLPC index should be determined.

# Discussion

SRS CLPC (SRS power control adjustment state) is described in TS 38.213 in clause 7.3.1 [2]. This text applies to both Rel-15/16 TCI spatial relation framework as well as the unified TCI framework.

For the SRS power control adjustment state for active UL BWPb of carrier f of serving cell c and SRS transmission occasioni

-     hb,f,ci,l=fb,f,ci,l, where fb,f,ci,l is the current PUSCH power control adjustment state as described in clause 7.1.1, if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions; or

-           if the UE is not configured for PUSCH transmissions on active UL BWP b of carrier f of serving cell c, or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and if tpc-Accumulation is not provided, where

…

- hb,f,c(i)=δSRS,b,f,c(i) if the UE is not configured for PUSCH transmissions on active UL BWP b of carrier f of serving cell c, or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and tpc-Accumulation is provided, and the UE detects a DCI format 2_3 KSRS,min symbols before a first symbol of SRS transmission occasion i, where absolute values ofδSRS,b,f,c are provided in Table 7.1.1-1

…

According to the above text from TS 38.213:

- Based on the part in yellow, there can only be one independent CLPC loop for SRS as l is not a parameter in these equations (i.e., when SRS and PUSCH have separate loops). Therefore, the SRS PC adjustment state associated with the UL/Joint TCI state is not used for distinguishing two different SRS power control states.

- If SRS PC follows PUSCH CLPC according to the part in cyan, the SRS can follow either that of PUSCH power control adjustment state 0 (l=0), or PUSCH power control adjustment state 1 (l=1). This based on the SRS power control adjustment state in the TCI state.

This description seems to aligned with option 2B from [1].

Observation 1: For SRS CLPC, the corresponding text in clause 7.3.1 of TS 38.213 applies to both Rel-15/16 TCI spatial relation framework as well as the unified TCI framework. No spec change is needed.

# Conclusions

The following observation has been made regarding SRS CLPC for the unified TCI state framework:

Observation 1: For SRS CLPC, the corresponding text in clause 7.3.1 of TS 38.213 applies to both Rel-15/16 TCI spatial relation framework as well as the unified TCI framework. No spec change is needed.

# References

- R1-2210439, “Moderator Summary #1 for Maintenance on Rel-17 Multi-Beam”, Moderator (ZTE), 3GPP RAN1#110b-e, e-Meeting, October, 2022.

- TS 38.213, “Physical layer procedures for control”, V17.3.0, September 2022.