3GPP TSG RAN WG1 #111		R1-2212053

Toulouse, France, November 14th – 18th, 2022

Agenda item:	9.5.2.3

Source:	Samsung

Title:	Discussion on LPHAP

Document for:	Discussion and Decision

# Introduction

In RAN1 #110b [1], the following agreements and conclusions were made regarding the evaluation assumptions and observations for low power high accuracy positioning (LPHAP).

Conclusion

- From evaluations for a LPHAP device, RAN1 concludes that the existing Rel-17 positioning for UEs in RRC_INACTIVE state cannot satisfy the target battery life required by LPHAP use case 6 in the majority of the evaluation scenarios that were examined.

- Based on the evaluations, enhancements to meet the target battery life in Rel-18 are necessary.

Conclusion

- Evaluations show that UE (re)entering RRC_CONNECTED state to obtain SRS (re)configuration increases power consumption;

- Note: This intermediate conclusion may be updated before capturing it in the TR if new/different evaluations are provided and to add information about the number of sources.

Agreement

- For UL and DL+UL positioning for UEs in RRC_INACTIVE, study the potential benefits and performance gains of enhancements on SRS for positioning in order to avoid frequent SRS (re)configuration, including at least the following:

  - The (pre-)configuration of SRS for positioning. FFS details, e.g., signaling and procedure, whether/how it is applicable to an area across multiple cells, consideration of UL overhead/capacity implied by (pre-)configuration and multiple cells, etc;

  - SRS for positioning activation/request procedure(s), e.g., network activation of SRS via paging, UE request to obtain/update SRS via RACH-based procedure;

    - FFS: Events of invalidity of SRS configuration to trigger the UE request procedure.

  - FFS whether it is applicable to UEs in RRC_IDLE state.

Conclusion

- Evaluations show that extending paging DRX cycles beyond 10.24s provide power saving gains with respect to that with the baseline DRX cycle of 1.28s and is beneficial towards meeting the battery life requirement

- Note: This intermediate conclusion may be updated before capturing it in the TR if new/different evaluations are provided and to add information about the number of sources and to show the achievable gains.

Conclusion

- Evaluations show that minimizing gaps between PRS/SRS/paging/reporting/synchronization RS reduces the power consumption;

- Note: This intermediate conclusion may be updated before capturing it in the TR if new/different evaluations are provided and to add information about the number of sources.

Agreement

For the LPHAP study only:

- For the power consumption model of the ultra-deep sleep type, adopt the following option (i.e. revision of option 1 from previous agreement):

  - The relative power unit: 0.015

  - Additional transition energy: 10000

    - Note: Power consumption analysis from individual companies with additional transition energy of 5000 can be optionally evaluated and captured in the TR.

  - Total transition time: 400ms

- Note: Power consumption analysis from individual companies with Option 2 (revised from previous agreement) can be optionally evaluated and captured in the TR.

  - Option 2 additional transition energy is revised from 450 to 480.

- Note: No new device type is expected based on ultra-deep sleep power modeling.

This contribution further discusses the evaluation and technical aspects related to the support of LPHAP.

# Further Clarification of Study Scope

The study intends to focus on RRC_INACTIVE and/or RRC_IDLE state, as described in the SID [2], to further alleviate the study load. However, RAN1 may need RAN2 to clarify whether positioning for RRC_IDLE state is within the scope of Rel-18 study since there was no support of positioning on RRC_IDLE state in Rel-16 and Rel-17. After the clarification of study scope from RAN2, RAN1 can further study whether enhancement is needed for RRC_INACTIVE and/or RRC_IDLE state to support LPHAP.

Proposal 1: RAN1 shall wait for RAN2’s clarification on the scope of the study. Especially, one of the following options shall be clarified:

- Option 1: The study investigates potential enhancement to positioning in RRC_INATIVE state to support LPHAP.

- Option 2: The study investigates supporting of positioning in RRC_IDLE state and potential enhancement to support LPHAP.

- Option 3: Option 1 + Option 2.

# DL Positioning Evaluation Results

## 3.1 High SNR Scenario

The evaluations for high SNR scenario are summarized in this section, wherein single SS/PBCH block is sufficient for synchronization purpose. 4 cases are considered in the evaluations (details in Table 1), and the evaluation results with deep sleep and ultra deep sleep (Option 1 with transmission energy as 10000) are shown in Table 2 (details in Appendix 2).

Table 1 Summary of cases evaluated for DL positioning in high SNR.

| Case # | Description | Timeline Illustration |
| --- | --- | --- |
| Case DL-1 | DRX cycle = 1.28 s One PRS per DRX cycle |  |
| Case DL-2 | DRX cycle = 1.28 s One PRS per 8 DRX cycles |  |
| Case DL-3 | DRX cycle = 10.24 s One PRS per DRX cycle |  |
| Note 1: Common evaluation assumptions: - Frequency range: FR1 - SCS: 30 kHz - Carrier BW: 100 MHz - # TRP for positioning: 4 - Group paging rate: 10% - Device type: Type A (800 mAh) - Implementation factor: K=1 - Ultra deep sleep model: Option 1 with 10000 transition energy Note 2: Common legends in the figures |  |  |

Table 2 Summary of evaluation results for power consumption in DL positioning for high SNR.

| Type A LPHAP Device Battery Life (Month) | Case DL-1 | Case DL-2 | Case DL-3 |
| --- | --- | --- | --- |
| Deep sleep | 0.47 | 0.55 | 0.69 |
| Ultra Deep sleep | 0.17 | 0.18 | 1.33 |

## 3.2 Low SNR Scenario

The evaluations for low SNR scenario are summarized in this section, wherein multiple SS/PBCH blocks (e.g., 3) are used for synchronization purpose. 4 cases are considered in the evaluations (details in Table 3), wherein the timeline for a DRX cycle with positioning measurement and reporting is shown in Figure 1, and the evaluation results with deep sleep and ultra deep sleep (Option 1 with transmission energy as 10000) are shown in Table 4 (details in Appendix 2).

Table 3 Summary of cases evaluated for DL positioning in low SNR.

| Case # | Description |
| --- | --- |
| Case DL-1 | DRX cycle = 1.28 s, One PRS per DRX cycle, 3 SSB for synchronization |
| Case DL-2 | DRX cycle = 1.28 s, One PRS per 8 DRX cycles, 3 SSB for synchronization |
| Case DL-3 | DRX cycle = 10.24 s, One PRS per DRX cycle, 3 SSB for synchronization |
| Note 1: Common evaluation assumptions: - Frequency range: FR1 - SCS: 30 kHz - Carrier BW: 100 MHz - # TRP for poisitioning: 4 - Group paging rate: 10% - Device type: Type A (800 mAh) - Implementation factor: K=1 - Ultra deep sleep model: Option 1 with 10000 transition energy |  |

Table 4 Summary of evaluation results for power consumption in DL positioning for low SNR.

| Type A LPHAP Device Battery Life (Month) | Case DL-1 | Case DL-2 | Case DL-3 |
| --- | --- | --- | --- |
| Deep sleep | 0.29 | 0.35 | 0.62 |
| Ultra Deep sleep | 0.14 | 0.15 | 1.09 |

Figure 1 Illustration of timeline within a DRX cycle with 3 SSB for synchronization in low SNR.

# UL Positioning Evaluation Results

## 4.1 High SNR Scenario

The evaluations for high SNR scenario are summarized in this section, wherein multiple SS/PBCH blocks (e.g., 3) are used for synchronization purpose. 4 cases are considered in the evaluations (details in Table 5), and the evaluation results with deep sleep and ultra deep sleep (Option 1 with transmission energy as 10000) are shown in Table 6 (details in Appendix 2).

Table 5 Summary of cases evaluated for UL positioning in high SNR.

| Case # | Description |
| --- | --- |
| Case UL-1 | DRX cycle = 1.28 s, One PRS per DRX cycle, 1 SSB for synchronization |
| Case UL-2 | DRX cycle = 1.28 s, One PRS per 8 DRX cycles, 1 SSB for synchronization |
| Case UL-3 | DRX cycle = 10.24 s, One PRS per DRX cycle, 1 SSB for synchronization |
| Note 1: Common evaluation assumptions: - Frequency range: FR1 - SCS: 30 kHz - Carrier BW: 100 MHz - # TRP for poisitioning: 4 - Group paging rate: 10% - Device type: Type A (800 mAh) - Implementation factor: K=1 - Ultra deep sleep model: Option 1 with 10000 transition energy |  |

Table 6 Summary of evaluation results for power consumption in UL positioning for high SNR.

| Type A LPHAP Device Battery Life (Month) | Case UL-1 | Case UL-2 | Case UL-3 |
| --- | --- | --- | --- |
| Deep sleep | 0.52 | 0.55 | 0.70 |
| Ultra Deep sleep | 0.18 | 0.18 | 1.38 |

## 4.2 Low SNR Scenario

The evaluations for high SNR scenario are summarized in this section, wherein single SS/PBCH block is sufficient for synchronization purpose. 4 cases are considered in the evaluations (details in Table 7), and the evaluation results with deep sleep and ultra deep sleep (Option 1 with transmission energy as 10000) are shown in Table 8 (details in Appendix 2).

Table 7 Summary of cases evaluated for UL positioning in low SNR.

| Case # | Description |
| --- | --- |
| Case UL-1 | DRX cycle = 1.28 s, One PRS per DRX cycle, 3 SSB for synchronization |
| Case UL-2 | DRX cycle = 1.28 s, One PRS per 8 DRX cycles, 3 SSB for synchronization |
| Case UL-3 | DRX cycle = 10.24 s, One PRS per DRX cycle, 3 SSB for synchronization |
| Note 1: Common evaluation assumptions: - Frequency range: FR1 - SCS: 30 kHz - Carrier BW: 100 MHz - # TRP for poisitioning: 4 - Group paging rate: 10% - Device type: Type A (800 mAh) - Implementation factor: K=1 - Ultra deep sleep model: Option 1 with 10000 transition energy |  |

Table 8 Summary of evaluation results for power consumption in UL positioning for low SNR.

| Type A LPHAP Device Battery Life (Month) | Case UL-1 | Case UL-2 | Case UL-3 |
| --- | --- | --- | --- |
| Deep sleep | 0.34 | 0.35 | 0.65 |
| Ultra Deep sleep | 0.15 | 0.15 | 1.17 |

# Observations from Evaluation Results

Based on the evaluation results in Section 4 and Section 5 of this contribution, we have the following observations:

Observation 1: For Type A LPHAP device with implementation factor K=1:

- For all evaluated configuration cases in both DL and UL positioning, Rel-17 positioning technique cannot meet the target battery life requirement.

- For some of the evaluated configuration cases in both DL and UL positioning, ultra deep sleep can improve the battery life.

  - Especially, the improvement is significant for long DRX cycle (e.g., Case 3 in the evaluations).

- For all evaluated configuration in both DL and UL positioning, battery life in low SNR (or bad synchronization) scenario is far from the target requirement.

# Improvements Considered

Based on above observations, we studied the following enhancements to further improve the battery life.

## 6.1 eDRX

As observed in the evaluation results, long DRX cycle can help to reduce the power consumption. Since the maximum DRX cycle supported positioning is 10.24 s, which may not be sufficient for power saving purpose, eDRX can be considered in the study for further power reduction.

For this purpose, extra evaluation cases (i.e., Case DL-4 and Case UL-4) are considered in the simulation (an illustration of the procedure for DL positioning in high SNR is shown in Figure 2): the eDRX cycle is 20.48 s in the evaluation, and the remaining evaluation assumptions are the same as Section 3 and Section 4, for DL and UL positioning respectively.

A summary of evaluation results is shown in Table 9 to Table 12. Details of the evaluation results are shown in Appendix 2.

Figure 2 Illustration of eDRX based positioning related procedures in high SNR.

Table 9 Summary of evaluation results for power consumption in DL positioning for high SNR.

| Type A LPHAP Device Battery Life (Month) | Case DL-1 | Case DL-2 | Case DL-3 | Case DL-4 |
| --- | --- | --- | --- | --- |
| Deep sleep | 0.47 | 0.55 | 0.69 | 0.71 |
| Ultra Deep sleep | 0.17 | 0.18 | 1.33 | 2.59 |

Table 10 Summary of evaluation results for power consumption in DL positioning for low SNR.

| Type A LPHAP Device Battery Life (Month) | Case DL-1 | Case DL-2 | Case DL-3 | Case DL-4 |
| --- | --- | --- | --- | --- |
| Deep sleep | 0.29 | 0.35 | 0.62 | 0.68 |
| Ultra Deep sleep | 0.14 | 0.15 | 1.09 | 2.13 |

Table 11 Summary of evaluation results for power consumption in UL positioning for high SNR.

| Type A LPHAP Device Battery Life (Month) | Case UL-1 | Case UL-2 | Case UL-3 | Case UL-4 |
| --- | --- | --- | --- | --- |
| Deep sleep | 0.52 | 0.55 | 0.70 | 0.72 |
| Ultra Deep sleep | 0.18 | 0.18 | 1.38 | 2.69 |

Table 12 Summary of evaluation results for power consumption in UL positioning for low SNR.

| Type A LPHAP Device Battery Life (Month) | Case UL-1 | Case UL-2 | Case UL-3 | Case UL-4 |
| --- | --- | --- | --- | --- |
| Deep sleep | 0.34 | 0.35 | 0.65 | 0.69 |
| Ultra Deep sleep | 0.15 | 0.15 | 1.17 | 2.28 |

Based on above evaluations, we have the following observation and proposal for enhancements.

Observation 2: For Type A LPHAP device with implementation factor K=1:

- eDRX based positioning is beneficial in improving the battery life.

Proposal 3: To improve the battery life, it’s beneficial to study and support eDRX based positioning.

## 6.2 Paging/PEI Triggered Positioning

Another direction to further save UE’s power in positioning in RRC_INACTIVE mode is trying to align the positioning measurement/reporting/transmission procedure with the paging procedure, such that the UE can wake up to perform all necessary operations in a burst and goes back to sleep. For this purpose, we propose paging or PEI triggered positioning, wherein the indication to perform positioning related operations (e.g., PRS measurement, measurement report, or SRS transmission) can be explicitly included or implicitly carried by paging or PEI. The UE only needs to perform positioning related operations based on the indication, and skips performing positioning related operations otherwise.

Especially for PEI, it has been testified that the supporting for PEI is beneficial for reducing UE’s power in RRC_INACTIVE mode by supporting subgroup indication in the PEI (e.g., effectively lower paging rate), and in particular, due to its lower requirement on message decoding comparing to paging PDCCH/PDSCH, the location of PEI can be flexibly configured to be right after a single SSB reception even for low SNR scenario, which can remarkably reduce the UE power consumption for low SNR scenario (e.g., skipping extra SSB receptions). An illustration of PEI triggered cancellation of positioning related procedures is shown in Figure 3.

Figure 3 Illustration of PEI triggered cancellation of positioning related procedures in low SNR.

A summary of evaluation results for paging or PEI based positioning is shown in Table 13 and Table 14, for DL and UL respectively, wherein long DRX cycle (e.g., Case 3 and 4) and ultra deep sleep are focused in the evaluation. Details of the evaluation results are shown in Appendix 2.

Table 13 Summary of evaluation results for power consumption of DL positioning with enhancements.

| Type A LPHAP Device Battery Life (Month) | Case DL-3-UDS (high SNR) | Case DL-4-UDS (high SNR) | Case DL-3-UDS (low SNR) | Case DL-4-UDS (low SNR) |
| --- | --- | --- | --- | --- |
| w/o enhancement | 1.33 | 2.59 | 1.09 | 2.13 |
| Paging triggered | 1.41 | 2.73 | 1.18 | 2.30 |
| PEI triggered | 1.42 | 2.75 | 1.41 | 2.75 |

Table 14 Summary of evaluation results for power consumption of UL positioning with enhancements.

| Type A LPHAP Device Battery Life (Month) | Case UL-3-UDS (high SNR) | Case UL-4-UDS (high SNR) | Case UL-3-UDS (low SNR) | Case UL-4-UDS (low SNR) |
| --- | --- | --- | --- | --- |
| w/o enhancement | 1.38 | 2.69 | 1.17 | 2.28 |
| Paging triggered | 1.41 | 2.74 | 1.19 | 2.32 |
| PEI triggered | 1.42 | 2.76 | 1.41 | 2.75 |

Based on above evaluations, we have the following observation and proposal for enhancements.

Observation 3: For Type A LPHAP device with implementation factor K=1:

- Paging and PEI triggered positioning are beneficial in improving the battery life;

- PEI triggered positioning is beneficial especially for low SNR scenario.

Proposal 4: To improve the battery life in low SNR scenario, it’s beneficial to study and support paging or PEI triggered positioning.

## 6.3 Acquiring Positioning Configuration without RRC Connection

One proposal for study is to avoiding entering RRC_CONNECTED mode to acquire the reconfiguration of positioning, for RRC_INACTIVE mode positioning. For this purpose, a set of extra evaluations are considered in the simulation (an illustration of the procedure for DRX cycle with RRC reconfiguration in high SNR is shown in Figure 4): timer based RRC reconfiguration is used and the interval for RRC reconfiguration is assumed as 60 minutes in the evaluation (e.g., PeriodicRNAU-TimerValue = min60), wherein the power consumption of RRC reconfiguration block (e.g., including RACH procedure and RRC reconfiguration and releasing) is assumed as 10000 with 40 time units, and the remaining evaluation assumptions are the same as Section 3.

A summary of evaluation results is shown in Table 15. Details of the evaluation results are shown in Appendix 2.

Table 15 Summary of evaluation results for power consumption of DL positioning with enhancements.

| Type A LPHAP Device Battery Life (Month) | Case DL-3-UDS (high SNR) | Case DL-4-UDS (high SNR) | Case DL-3-UDS (low SNR) | Case DL-4-UDS (low SNR) |
| --- | --- | --- | --- | --- |
| w/ RRC re-config. | 1.33 | 2.58 | 1.09 | 2.12 |
| w/o RRC re-config. | 1.33 | 2.59 | 1.09 | 2.13 |

Figure 4 Illustration of DRX cycle with RRC reconfiguration in high SNR.

Based on above evaluations, we have the following observation.

Observation 4: For Type A LPHAP device with implementation factor K=1:

- Acquiring positioning configuration without RRC connection doesn’t show significant gain.

# Conclusion

The proposals and observations made in this contribution are summarized below:

Proposal 1: RAN1 shall wait for RAN2’s clarification on the scope of the study. Especially, one of the following options shall be clarified:

- Option 1: The study investigates potential enhancement to positioning in RRC_INATIVE state to support LPHAP.

- Option 2: The study investigates supporting of positioning in RRC_IDLE state and potential enhancement to support LPHAP.

- Option 3: Option 1 + Option 2.

Proposal 2: Support Option 1 for ultra deep sleep study:

- Option 1:

  - The relative power unit: 0.015

  - Additional transition energy: 2000

  - Total transition time: 400ms

Observation 1: For Type A LPHAP device with implementation factor K=1:

- For all evaluated configuration cases in both DL and UL positioning, Rel-17 positioning technique cannot meet the target battery life requirement.

- For some of the evaluated configuration cases in both DL and UL positioning, ultra deep sleep can improve the battery life.

  - Especially, the improvement is significant for long DRX cycle (e.g., Case 3 in the evaluations).

- For all evaluated configuration in both DL and UL positioning, battery life in low SNR (or bad synchronization) scenario is far from the target requirement.

Observation 2: For Type A LPHAP device with implementation factor K=1:

- eDRX based positioning is beneficial in improving the battery life.

Proposal 3: To improve the battery life, it’s beneficial to study and support eDRX based positioning.

Observation 3: For Type A LPHAP device with implementation factor K=1:

- Paging and PEI triggered positioning are beneficial in improving the battery life;

- PEI triggered positioning is beneficial especially for low SNR scenario.

Proposal 4: To improve the battery life in low SNR scenario, it’s beneficial to study and support paging or PEI triggered positioning.

Observation 4: For Type A LPHAP device with implementation factor K=1:

- Acquiring positioning configuration without RRC connection doesn’t show significant gain.

# Reference

[1] Chairman notes, RAN1#110b, e-Meeting, October, 2022

[2] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel

# Appendix 1: Evaluation cases and assumption

Table 16 Evaluation cases and assumption for DL positioning with deep sleep

| Evaluation assumption | Case DL-1-DS, FR1, UE-assisted DL positioning, Type A | Case DL-2-DS, FR1, UE-assisted DL positioning, Type A | Case DL-3-DS, FR1, UE-assisted DL positioning, Type A | Case DL-4-DS, FR1, UE-assisted DL positioning, Type A |
| --- | --- | --- | --- | --- |
| Sleep state | Deep sleep | Deep sleep | Deep sleep | Deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| paging reception | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| RS periodicity | 1.28 s | 10.24 s | 10.24 s | 20.48 s |
| M-sample | 1 | 1 | 1 | 1 |
| RRM measurement | No | No | No | No |
| BWP switching | No | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR |
| implementation factor K | 1 | 1 | 1 | 1 |

Table 17 Evaluation cases and assumption for DL positioning with ultra deep sleep

| Evaluation assumption | Case DL-1-UDS, FR1, UE-assisted DL positioning, Type A | Case DL-2-UDS, FR1, UE-assisted DL positioning, Type A | Case DL-3-UDS, FR1, UE-assisted DL positioning, Type A | Case DL-4-UDS, FR1, UE-assisted DL positioning, Type A |
| --- | --- | --- | --- | --- |
| Sleep state | Ultra deep sleep | Ultra deep sleep | Ultra deep sleep | Ultra deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| paging reception | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| RS periodicity | 1.28 s | 10.24 s | 10.24 s | 20.48 s |
| M-sample | 1 | 1 | 1 | 1 |
| RRM measurement | No | No | No | No |
| BWP switching | No | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR |
| implementation factor K | 1 | 1 | 1 | 1 |
| Ultra deep sleep model | Option 1 with 10000 as transition energy | Option 1 with 10000 as transition energy | Option 1 with 10000 as transition energy | Option 1 with 10000 as transition energy |

Table 18 Evaluation cases and assumption for UL positioning with deep sleep

| Evaluation assumption | Case UL-1-DS, FR1, UL positioning, Type A | Case UL-2-DS, FR1, UL positioning, Type A | Case UL-3-DS, FR1, UL positioning, Type A | Case UL-4-DS, FR1, UL positioning, Type A |
| --- | --- | --- | --- | --- |
| Sleep state | Deep sleep | Deep sleep | Deep sleep | Deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| paging reception | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| RS periodicity | 1.28 s | 10.24 s | 10.24 s | 20.48 s |
| RRM measurement | No | No | No | No |
| BWP switching | No | No | No | No |
| implementation factor K | 1 | 1 | 1 | 1 |

Table 19 Evaluation cases and assumption for UL positioning with ultra deep sleep

| Evaluation assumption | Case UL-1-UDS, FR1, UL positioning, Type A | Case UL-2-UDS, FR1, UL positioning, Type A | Case UL-3-UDS, FR1, UL positioning, Type A | Case UL-4-UDS, FR1, UL positioning, Type A |
| --- | --- | --- | --- | --- |
| Sleep state | Ultra deep sleep | Ultra deep sleep | Ultra deep sleep | Ultra deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| paging reception | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| RS periodicity | 1.28 s | 10.24 s | 10.24 s | 20.48 s |
| RRM measurement | No | No | No | No |
| BWP switching | No | No | No | No |
| implementation factor K | 1 | 1 | 1 | 1 |
| Ultra deep sleep model | Option 1 with 10000 as transition energy | Option 1 with 10000 as transition energy | Option 1 with 10000 as transition energy | Option 1 with 10000 as transition energy |

# Appendix 2: UE power consumption results for each evaluation case

Table 20 UE power consumption result for Case DL-1-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-1-DS (high SNR) | Deep Sleep | 1 | 1 | 2508 | 2508 | 2508 | 61.91% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 11.11% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.11% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 4.94% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 5.63% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 2.96% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 12.34% |
|  | Total (every power cycle) |  |  |  | 2560 | 4051 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.58 |  |  |
|  | Battery life (in month) |  |  |  | 0.47 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 21 UE power consumption result for Case DL-2-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-2-DS (high SNR) | Deep Sleep | 1 | 1 | 20092 | 20092 | 20092 | 72.32% |
|  | Deep Sleep Transition | 450 | 40 | 8 | 320 | 3600 | 12.96% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.16% |
|  | SSB Proc. | 50 | 1 | 32 | 32 | 1600 | 5.76% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 6.57% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.43% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 1.80% |
|  | Total (every power cycle) |  |  |  | 20480 | 27781 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.36 |  |  |
|  | Battery life (in month) |  |  |  | 0.55 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 22 UE power consumption result for Case DL-3-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-DS (high SNR) | Deep Sleep | 1 | 1 | 20428 | 20428 | 20428 | 92.98% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 2.05% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.20% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 0.91% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.04% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.55% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 2.28% |
|  | Total (every power cycle) |  |  |  | 20480 | 21971 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.07 |  |  |
|  | Battery life (in month) |  |  |  | 0.69 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 23 UE power consumption result for Case DL-4-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-DS (high SNR) | Deep Sleep | 1 | 1 | 40908 | 40908 | 40908 | 96.37% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.06% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.11% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 0.47% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.54% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.28% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 1.18% |
|  | Total (every power cycle) |  |  |  | 40960 | 42451 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.04 |  |  |
|  | Battery life (in month) |  |  |  | 0.71 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 24 UE power consumption result for Case DL-1-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-1-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 1748 | 1748 | 26.22 | 0.24% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 89.93% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.40% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.80% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.05% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.08% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 4.50% |
|  | Total (every power cycle) |  |  |  | 2560 | 11119.22 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 4.34 |  |  |
|  | Battery life (in month) |  |  |  | 0.17 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 25 UE power consumption result for Case DL-2-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-2-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 14012 | 14012 | 210.18 | 0.25% |
|  | UDS Transition | 10000 | 800 | 8 | 6400 | 80000 | 94.90% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.05% |
|  | SSB Proc. | 50 | 1 | 32 | 32 | 1600 | 1.90% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 2.16% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.14% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 0.59% |
|  | Total (every power cycle) |  |  |  | 20480 | 84299.18 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 4.12 |  |  |
|  | Battery life (in month) |  |  |  | 0.18 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 26 UE power consumption result for Case DL-3-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 19668 | 19668 | 295.02 | 2.59% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 87.81% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.40% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.76% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.00% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.05% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 4.39% |
|  | Total (every power cycle) |  |  |  | 20480 | 11388.02 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.56 |  |  |
|  | Battery life (in month) |  |  |  | 1.33 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 27 UE power consumption result for Case DL-4-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 40148 | 40148 | 602.22 | 5.15% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 85.51% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.38% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.71% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.95% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.03% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 4.28% |
|  | Total (every power cycle) |  |  |  | 40960 | 11695.22 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.29 |  |  |
|  | Battery life (in month) |  |  |  | 2.59 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 28 UE power consumption result for Case DL-1-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-1-DS (low SNR) | Deep Sleep | 1 | 1 | 2396 | 2396 | 2396 | 36.98% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 6.95% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 22.23% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.09% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.69% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 9.26% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 3.52% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.85% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 15.43% |
|  | Total (every power cycle) |  |  |  | 2560 | 6479 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.53 |  |  |
|  | Battery life (in month) |  |  |  | 0.29 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 29 UE power consumption result for Case DL-2-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-2-DS (low SNR) | Deep Sleep | 1 | 1 | 19252 | 19252 | 19252 | 43.99% |
|  | Deep Sleep Transition | 450 | 40 | 8 | 320 | 3600 | 8.23% |
|  | Light Sleep | 20 | 1 | 576 | 576 | 11520 | 26.32% |
|  | Light Sleep Transition | 100 | 12 | 16 | 192 | 1600 | 3.66% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.10% |
|  | SSB Proc. | 50 | 1 | 96 | 96 | 4800 | 10.97% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 4.17% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.27% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 2.29% |
|  | Total (every power cycle) |  |  |  | 20480 | 43761 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.14 |  |  |
|  | Battery life (in month) |  |  |  | 0.35 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 30 UE power consumption result for Case DL-3-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-DS (low SNR) | Deep Sleep | 1 | 1 | 20316 | 20316 | 20316 | 83.27% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.84% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 5.90% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 0.82% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.18% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 2.46% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.93% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.49% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 4.10% |
|  | Total (every power cycle) |  |  |  | 20480 | 24399 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.19 |  |  |
|  | Battery life (in month) |  |  |  | 0.62 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 31 UE power consumption result for Case DL-4-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-DS (low SNR) | Deep Sleep | 1 | 1 | 40796 | 40796 | 40796 | 90.90% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.00% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 3.21% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 0.45% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.10% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 1.34% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.51% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.27% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 2.23% |
|  | Total (every power cycle) |  |  |  | 40960 | 44879 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.10 |  |  |
|  | Battery life (in month) |  |  |  | 0.68 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 32 UE power consumption result for Case DL-1-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-1-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 1636 | 1636 | 24.54 | 0.18% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 73.22% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.54% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.46% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.33% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.39% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.67% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.88% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 7.32% |
|  | Total (every power cycle) |  |  |  | 2560 | 13657.54 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 5.33 |  |  |
|  | Battery life (in month) |  |  |  | 0.14 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 33 UE power consumption result for Case DL-2-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-2-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 13172 | 13172 | 197.58 | 0.20% |
|  | UDS Transition | 10000 | 800 | 8 | 6400 | 80000 | 79.12% |
|  | Light Sleep | 20 | 1 | 576 | 576 | 11520 | 11.39% |
|  | Light Sleep Transition | 100 | 12 | 16 | 192 | 1600 | 1.58% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.04% |
|  | SSB Proc. | 50 | 1 | 96 | 96 | 4800 | 4.75% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 1.80% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.12% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 0.99% |
|  | Total (every power cycle) |  |  |  | 20480 | 101106.6 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 4.94 |  |  |
|  | Battery life (in month) |  |  |  | 0.15 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 34 UE power consumption result for Case DL-3-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 19556 | 19556 | 293.34 | 2.11% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 71.81% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.34% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.44% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.32% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.31% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.64% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.86% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 7.18% |
|  | Total (every power cycle) |  |  |  | 20480 | 13926.34 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.68 |  |  |
|  | Battery life (in month) |  |  |  | 1.09 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 35 UE power consumption result for Case DL-4-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 40036 | 40036 | 600.54 | 4.22% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 70.26% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.12% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.41% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.32% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.22% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.60% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.84% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 7.03% |
|  | Total (every power cycle) |  |  |  | 40960 | 14233.54 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.35 |  |  |
|  | Battery life (in month) |  |  |  | 2.13 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 36 UE power consumption result for Case UL-1-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-1-DS (high SNR) | Deep Sleep | 1 | 1 | 2510 | 2510 | 2510 | 68.90% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 12.35% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.24% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 5.49% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 6.26% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 5.76% |
|  | Total (every power cycle) |  |  |  | 2560 | 3643 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.42 |  |  |
|  | Battery life (in month) |  |  |  | 0.52 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 37 UE power consumption result for Case UL-2-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-2-DS (high SNR) | Deep Sleep | 1 | 1 | 20094 | 20094 | 20094 | 73.41% |
|  | Deep Sleep Transition | 450 | 40 | 8 | 320 | 3600 | 13.15% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.16% |
|  | SSB Proc. | 50 | 1 | 32 | 32 | 1600 | 5.85% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 6.66% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.77% |
|  | Total (every power cycle) |  |  |  | 20480 | 27373 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.34 |  |  |
|  | Battery life (in month) |  |  |  | 0.55 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 38 UE power consumption result for Case UL-3-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-DS (high SNR) | Deep Sleep | 1 | 1 | 20430 | 20430 | 20430 | 94.75% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 2.09% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.21% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 0.93% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.06% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.97% |
|  | Total (every power cycle) |  |  |  | 20480 | 21563 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.05 |  |  |
|  | Battery life (in month) |  |  |  | 0.70 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 39 UE power consumption result for Case UL-4-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-DS (high SNR) | Deep Sleep | 1 | 1 | 40910 | 40910 | 40910 | 97.31% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.07% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.11% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 0.48% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.54% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.50% |
|  | Total (every power cycle) |  |  |  | 40960 | 42043 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.03 |  |  |
|  | Battery life (in month) |  |  |  | 0.72 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 40 UE power consumption result for Case UL-1-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-1-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 1750 | 1750 | 26.25 | 0.25% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 93.38% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.42% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.87% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.13% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 1.96% |
|  | Total (every power cycle) |  |  |  | 2560 | 10709.25 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 4.18 |  |  |
|  | Battery life (in month) |  |  |  | 0.18 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 41 UE power consumption result for Case UL-2-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-2-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 14014 | 14014 | 210.21 | 0.25% |
|  | UDS Transition | 10000 | 800 | 8 | 6400 | 80000 | 95.36% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.05% |
|  | SSB Proc. | 50 | 1 | 32 | 32 | 1600 | 1.91% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 2.17% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.25% |
|  | Total (every power cycle) |  |  |  | 20480 | 83889.21 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 4.10 |  |  |
|  | Battery life (in month) |  |  |  | 0.18 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 42 UE power consumption result for Case UL-3-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 19670 | 19670 | 295.05 | 2.69% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 91.09% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.41% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.82% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.08% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 1.91% |
|  | Total (every power cycle) |  |  |  | 20480 | 10978.05 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.54 |  |  |
|  | Battery life (in month) |  |  |  | 1.38 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 43 UE power consumption result for Case UL-4-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 40150 | 40150 | 602.25 | 5.34% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 88.61% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.40% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.77% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.02% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 1.86% |
|  | Total (every power cycle) |  |  |  | 40960 | 11285.25 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.28 |  |  |
|  | Battery life (in month) |  |  |  | 2.69 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 44 UE power consumption result for Case UL-1-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-1-DS (low SNR) | Deep Sleep | 1 | 1 | 2406 | 2406 | 2406 | 43.13% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 8.07% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 25.81% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.58% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.81% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 10.75% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.09% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 3.76% |
|  | Total (every power cycle) |  |  |  | 2560 | 5579 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.18 |  |  |
|  | Battery life (in month) |  |  |  | 0.34 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 45 UE power consumption result for Case UL-2-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-2-DS (low SNR) | Deep Sleep | 1 | 1 | 19262 | 19262 | 19262 | 44.94% |
|  | Deep Sleep Transition | 450 | 40 | 8 | 320 | 3600 | 8.40% |
|  | Light Sleep | 20 | 1 | 576 | 576 | 11520 | 26.88% |
|  | Light Sleep Transition | 100 | 12 | 16 | 192 | 1600 | 3.73% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.10% |
|  | SSB Proc. | 50 | 1 | 96 | 96 | 4800 | 11.20% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 4.26% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.49% |
|  | Total (every power cycle) |  |  |  | 20480 | 42861 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.09 |  |  |
|  | Battery life (in month) |  |  |  | 0.35 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 46 UE power consumption result for Case UL-3-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-DS (low SNR) | Deep Sleep | 1 | 1 | 20326 | 20326 | 20326 | 86.50% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.91% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 6.13% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 0.85% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.19% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 2.55% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.97% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.89% |
|  | Total (every power cycle) |  |  |  | 20480 | 23499 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.15 |  |  |
|  | Battery life (in month) |  |  |  | 0.65 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 47 UE power consumption result for Case UL-4-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-DS (low SNR) | Deep Sleep | 1 | 1 | 40806 | 40806 | 40806 | 92.79% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.02% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 3.27% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 0.45% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.10% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 1.36% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.52% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.48% |
|  | Total (every power cycle) |  |  |  | 40960 | 43979 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.07 |  |  |
|  | Battery life (in month) |  |  |  | 0.69 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 48 UE power consumption result for Case UL-1-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-1-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 1646 | 1646 | 24.69 | 0.19% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 78.45% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 11.30% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.57% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.35% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.71% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.79% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 1.65% |
|  | Total (every power cycle) |  |  |  | 2560 | 12747.69 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 4.98 |  |  |
|  | Battery life (in month) |  |  |  | 0.15 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 49 UE power consumption result for Case UL-2-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-2-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 13182 | 13182 | 197.73 | 0.20% |
|  | UDS Transition | 10000 | 800 | 8 | 6400 | 80000 | 79.84% |
|  | Light Sleep | 20 | 1 | 576 | 576 | 11520 | 11.50% |
|  | Light Sleep Transition | 100 | 12 | 16 | 192 | 1600 | 1.60% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.04% |
|  | SSB Proc. | 50 | 1 | 96 | 96 | 4800 | 4.79% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 1.82% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.21% |
|  | Total (every power cycle) |  |  |  | 20480 | 100196.7 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 4.89 |  |  |
|  | Battery life (in month) |  |  |  | 0.15 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 50 UE power consumption result for Case UL-3-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 19566 | 19566 | 293.49 | 2.25% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 76.83% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 11.06% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.54% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.35% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.61% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.75% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 1.61% |
|  | Total (every power cycle) |  |  |  | 20480 | 13016.49 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.64 |  |  |
|  | Battery life (in month) |  |  |  | 1.17 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 51 UE power consumption result for Case UL-4-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 40046 | 40046 | 600.69 | 4.51% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 75.05% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.81% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.50% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.34% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.50% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.71% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 1.58% |
|  | Total (every power cycle) |  |  |  | 40960 | 13323.69 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.33 |  |  |
|  | Battery life (in month) |  |  |  | 2.28 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 52 UE power consumption result for Case DL-3-UDS in high SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (high SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19668 | 19668 | 295.02 | 2.73% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 92.68% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.04% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.85% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.11% |
|  | PRS* | 12.135 | 1 | 1 | 1 | 12.135 | 0.11% |
|  | CG-SDT* | 25.135 | 1 | 2 | 2 | 50.27 | 0.47% |
|  | Total (every power cycle) |  |  |  | 20480 | 10790.06 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.53 |  |  |
|  | Battery life (in month) |  |  |  | 1.41 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 53 UE power consumption result for Case DL-4-UDS in high SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (high SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40148 | 40148 | 602.22 | 5.43% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 90.11% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.04% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.80% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.05% |
|  | PRS* | 12.135 | 1 | 1 | 1 | 12.135 | 0.11% |
|  | CG-SDT* | 25.135 | 1 | 2 | 2 | 50.27 | 0.45% |
|  | Total (every power cycle) |  |  |  | 40960 | 11097.26 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.27 |  |  |
|  | Battery life (in month) |  |  |  | 2.73 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 54 UE power consumption result for Case DL-3-UDS in low SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (low SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19556 | 19556 | 293.34 | 2.28% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 77.65% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 11.18% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.55% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.04% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.66% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.77% |
|  | PRS* | 12.135 | 1 | 1 | 1 | 12.135 | 0.09% |
|  | RA-SDT* | 100.135 | 10 | 1 | 10 | 100.135 | 0.78% |
|  | Total (every power cycle) |  |  |  | 20480 | 12878.25 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.63 |  |  |
|  | Battery life (in month) |  |  |  | 1.18 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 55 UE power consumption result for Case DL-4-UDS in low SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (low SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40036 | 40036 | 600.54 | 4.55% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 75.84% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.92% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.52% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.04% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.55% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.73% |
|  | PRS* | 12.135 | 1 | 1 | 1 | 12.135 | 0.09% |
|  | RA-SDT* | 100.135 | 10 | 1 | 10 | 100.135 | 0.76% |
|  | Total (every power cycle) |  |  |  | 40960 | 13185.45 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.32 |  |  |
|  | Battery life (in month) |  |  |  | 2.30 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 56 UE power consumption result for Case DL-3-UDS in high SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (high SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19664 | 19664 | 294.960 | 2.75% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 93.38% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.01% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.87% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 1.87% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.06% |
|  | PRS* | 1.515 | 1 | 1 | 1 | 1.515 | 0.01% |
|  | CG-SDT* | 3.140 | 1 | 2 | 2 | 6.280 | 0.06% |
|  | Total (every power cycle) |  |  |  | 20480 | 10709.39 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.52 |  |  |
|  | Battery life (in month) |  |  |  | 1.42 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 57 UE power consumption result for Case DL-4-UDS in high SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (high SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40144 | 40144 | 602.160 | 5.47% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 90.77% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.01% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.82% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 1.82% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.06% |
|  | PRS* | 1.515 | 1 | 1 | 1 | 1.515 | 0.01% |
|  | CG-SDT* | 3.140 | 1 | 2 | 2 | 6.280 | 0.06% |
|  | Total (every power cycle) |  |  |  | 40960 | 11016.59 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.27 |  |  |
|  | Battery life (in month) |  |  |  | 2.75 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 58 UE power consumption result for Case DL-3-UDS in low SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (low SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19556 | 19556 | 293.340 | 2.73% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 93.11% |
|  | Light Sleep* | 0.265 | 1 | 68 | 68 | 18.007 | 0.17% |
|  | Light Sleep Transition* | 1.25 | 12 | 2 | 24 | 2.5 | 0.02% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.01% |
|  | SSB Proc.* | 17.093 | 1 | 12 | 12 | 205.119 | 1.91% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 1.86% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.06% |
|  | PRS* | 1.515 | 1 | 1 | 1 | 1.515 | 0.01% |
|  | RA-SDT* | 12.515 | 10 | 1 | 10 | 12.515 | 0.12% |
|  | Total (every power cycle) |  |  |  | 20480 | 10739.63 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.52 |  |  |
|  | Battery life (in month) |  |  |  | 1.41 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 59 UE power consumption result for Case DL-4-UDS in low SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (low SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40036 | 40036 | 600.540 | 5.44% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 90.52% |
|  | Light Sleep* | 0.265 | 1 | 68 | 68 | 18.007 | 0.16% |
|  | Light Sleep Transition* | 1.25 | 12 | 2 | 24 | 2.5 | 0.02% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.01% |
|  | SSB Proc.* | 17.093 | 1 | 12 | 12 | 205.119 | 1.86% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 1.81% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.05% |
|  | PRS* | 1.515 | 1 | 1 | 1 | 1.515 | 0.01% |
|  | RA-SDT* | 12.515 | 10 | 1 | 10 | 12.515 | 0.11% |
|  | Total (every power cycle) |  |  |  | 40960 | 11046.83 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.27 |  |  |
|  | Battery life (in month) |  |  |  | 2.75 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 60 UE power consumption result for Case UL-3-UDS in high SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (high SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19670 | 19670 | 295.05 | 2.74% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 93.03% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.04% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.86% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.12% |
|  | SRS* | 21.135 | 1 | 1 | 1 | 21.135 | 0.20% |
|  | Total (every power cycle) |  |  |  | 20480 | 10748.82 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.52 |  |  |
|  | Battery life (in month) |  |  |  | 1.41 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 61 UE power consumption result for Case UL-4-UDS in high SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (high SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40150 | 40150 | 602.25 | 5.45% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 90.45% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.04% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.81% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.06% |
|  | SRS* | 21.135 | 1 | 1 | 1 | 21.135 | 0.19% |
|  | Total (every power cycle) |  |  |  | 40960 | 11056.02 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.27 |  |  |
|  | Battery life (in month) |  |  |  | 2.74 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 62 UE power consumption result for Case UL-3-UDS in low SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (low SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19566 | 19566 | 293.49 | 2.30% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 78.20% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 11.26% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.56% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.04% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.69% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.78% |
|  | SRS* | 21.135 | 1 | 1 | 1 | 21.135 | 0.17% |
|  | Total (every power cycle) |  |  |  | 20480 | 12787.26 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.62 |  |  |
|  | Battery life (in month) |  |  |  | 1.19 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 63 UE power consumption result for Case UL-4-UDS in low SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (low SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40046 | 40046 | 600.69 | 4.59% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 76.37% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 11.00% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.53% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.04% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.58% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.74% |
|  | SRS* | 21.135 | 1 | 1 | 1 | 21.135 | 0.16% |
|  | Total (every power cycle) |  |  |  | 40960 | 13094.46 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.32 |  |  |
|  | Battery life (in month) |  |  |  | 2.32 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 64 UE power consumption result for Case UL-3-UDS in high SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (high SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19666 | 19666 | 294.990 | 2.76% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 93.42% |
|  | Light Sleep* | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition* | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.01% |
|  | SSB Proc.* | 50 | 1 | 4 | 4 | 200 | 1.87% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 1.87% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.06% |
|  | SRS* | 2.640 | 1 | 1 | 1 | 2.640 | 0.02% |
|  | Total (every power cycle) |  |  |  | 20480 | 10704.27 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.52 |  |  |
|  | Battery life (in month) |  |  |  | 1.42 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 65 UE power consumption result for Case UL-4-UDS in high SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (high SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40146 | 40146 | 602.190 | 5.47% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 90.81% |
|  | Light Sleep* | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition* | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.01% |
|  | SSB Proc.* | 50 | 1 | 4 | 4 | 200 | 1.82% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 1.82% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.06% |
|  | SRS* | 2.640 | 1 | 1 | 1 | 2.640 | 0.02% |
|  | Total (every power cycle) |  |  |  | 40960 | 11011.47 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.27 |  |  |
|  | Battery life (in month) |  |  |  | 2.76 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 66 UE power consumption result for Case UL-3-UDS in low SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (low SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19562 | 19562 | 293.430 | 2.73% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 93.20% |
|  | Light Sleep* | 0.265 | 1 | 72 | 72 | 19.067 | 0.18% |
|  | Light Sleep Transition* | 1.25 | 12 | 2 | 24 | 2.5 | 0.02% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.01% |
|  | SSB Proc.* | 17.093 | 1 | 12 | 12 | 205.119 | 1.91% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 1.86% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.06% |
|  | SRS* | 2.640 | 1 | 1 | 1 | 2.640 | 0.02% |
|  | Total (every power cycle) |  |  |  | 20480 | 10729.39 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.52 |  |  |
|  | Battery life (in month) |  |  |  | 1.41 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 67 UE power consumption result for Case UL-4-UDS in low SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (low SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40042 | 40042 | 600.630 | 5.44% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 90.61% |
|  | Light Sleep* | 0.265 | 1 | 72 | 72 | 19.067 | 0.17% |
|  | Light Sleep Transition* | 1.25 | 12 | 2 | 24 | 2.5 | 0.02% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.01% |
|  | SSB Proc.* | 17.093 | 1 | 12 | 12 | 205.119 | 1.86% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 1.81% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.05% |
|  | SRS* | 2.640 | 1 | 1 | 1 | 2.640 | 0.02% |
|  | Total (every power cycle) |  |  |  | 40960 | 11036.59 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.27 |  |  |
|  | Battery life (in month) |  |  |  | 2.75 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 68 UE power consumption result for Case DL-3-UDS in high SNR with RRC re-configuration

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (high SNR, RRC re-config. every 60 mins) | Ultra Deep Sleep | 0.015 | 1 | 19628 | 19628 | 294.42 | 2.58% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 87.60% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.39% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.75% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.00% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.05% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 4.38% |
|  | RRC Re-config.** | 28.459 | 40 | 1 | 40 | 28.46 | 0.25% |
|  | Total (every power cycle) |  |  |  | 20480 | 11415.8794 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.56 |  |  |
|  | Battery life (in month) |  |  |  | 1.33 |  |  |
| Note*: value averaged based paging rate Note**: value averaged based on RRC re-config. frequency |  |  |  |  |  |  |  |

Table 69 UE power consumption result for Case DL-4-UDS in high SNR with RRC re-configuration

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (high SNR, RRC re-config. every 60 mins) | Ultra Deep Sleep | 0.015 | 1 | 40108 | 40108 | 601.62 | 5.12% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 85.10% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.38% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.70% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.94% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.02% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 4.25% |
|  | RRC Re-config.* | 56.904 | 40 | 1 | 40 | 56.90 | 0.48% |
|  | Total (every power cycle) |  |  |  | 40960 | 11751.5238 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.29 |  |  |
|  | Battery life (in month) |  |  |  | 2.58 |  |  |
| Note*: value averaged based paging rate Note**: value averaged based on RRC re-config. frequency |  |  |  |  |  |  |  |

Table 70 UE power consumption result for Case DL-3-UDS in low SNR with RRC re-configuration

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (low SNR, RRC re-config. every 60 mins) | Ultra Deep Sleep | 0.015 | 1 | 19516 | 19516 | 292.74 | 2.10% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 71.66% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.32% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.43% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.32% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.30% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.63% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.86% |
|  | CG-SDT | 1000 | 10 | 1 | 10 | 1000 | 7.17% |
|  | RRC Re-config.** | 28.459 | 40 | 1 | 40 | 28.46 | 0.20% |
|  | Total (every power cycle) |  |  |  | 20480 | 13954.2 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.68 |  |  |
|  | Battery life (in month) |  |  |  | 1.09 |  |  |
| Note*: value averaged based paging rate Note**: value averaged based on RRC re-config. frequency |  |  |  |  |  |  |  |

Table 71 UE power consumption result for Case DL-4-UDS in low SNR with RRC re-configuration

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (low SNR, RRC re-config. every 60 mins) | Ultra Deep Sleep | 0.015 | 1 | 39996 | 39996 | 599.94 | 4.20% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 69.98% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.08% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.40% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.31% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.20% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.60% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.84% |
|  | CG-SDT | 1000 | 10 | 1 | 10 | 1000 | 7.00% |
|  | RRC Re-config.* | 56.904 | 40 | 1 | 40 | 56.90 | 0.40% |
|  | Total (every power cycle) |  |  |  | 40960 | 14289.84 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.35 |  |  |
|  | Battery life (in month) |  |  |  | 2.12 |  |  |
| Note*: value averaged based paging rate Note**: value averaged based on RRC re-config. frequency |  |  |  |  |  |  |  |

Table 72 UE power consumption result for Case DL-3-UDS in high SNR without RRC re-configuration

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (high SNR, without RRC re-config.) | Ultra Deep Sleep | 0.015 | 1 | 19668 | 19668 | 295.02 | 2.59% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 87.81% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.40% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.76% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 2.00% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.05% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 4.39% |
|  | RRC Re-config.** | 28.459 | 40 | 0 | 0 | 0.00 | 0.00% |
|  | Total (every power cycle) |  |  |  | 20480 | 11388.02 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.56 |  |  |
|  | Battery life (in month) |  |  |  | 1.33 |  |  |
| Note*: value averaged based paging rate Note**: value averaged based on RRC re-config. frequency |  |  |  |  |  |  |  |

Table 73 UE power consumption result for Case DL-4-UDS in high SNR without RRC re-configuration

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (high SNR, without RRC re-config.) | Ultra Deep Sleep | 0.015 | 1 | 40148 | 40148 | 602.22 | 5.15% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 85.51% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.38% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 1.71% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.95% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.03% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 4.28% |
|  | RRC Re-config.* | 56.904 | 40 | 0 | 0 | 0.00 | 0.00% |
|  | Total (every power cycle) |  |  |  | 40960 | 11695.22 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.29 |  |  |
|  | Battery life (in month) |  |  |  | 2.59 |  |  |
| Note*: value averaged based paging rate Note**: value averaged based on RRC re-config. frequency |  |  |  |  |  |  |  |

Table 74 UE power consumption result for Case DL-3-UDS in low SNR without RRC re-configuration

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (low SNR, without RRC re-config.) | Ultra Deep Sleep | 0.015 | 1 | 19556 | 19556 | 293.34 | 2.11% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 71.81% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.34% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.44% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.32% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.31% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.64% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.86% |
|  | CG-SDT | 1000 | 10 | 1 | 10 | 1000 | 7.18% |
|  | RRC Re-config.** | 28.459 | 40 | 0 | 0 | 0.00 | 0.00% |
|  | Total (every power cycle) |  |  |  | 20480 | 13926.34 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.68 |  |  |
|  | Battery life (in month) |  |  |  | 1.09 |  |  |
| Note*: value averaged based paging rate Note**: value averaged based on RRC re-config. frequency |  |  |  |  |  |  |  |

Table 75 UE power consumption result for Case DL-4-UDS in low SNR without RRC re-configuration

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (low SNR, without RRC re-config.) | Ultra Deep Sleep | 0.015 | 1 | 40036 | 40036 | 600.54 | 4.22% |
|  | UDS Transition | 10000 | 800 | 1 | 800 | 10000 | 70.26% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 10.12% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 1.41% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.32% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 4.22% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.60% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.84% |
|  | CG-SDT | 1000 | 10 | 1 | 10 | 1000 | 7.03% |
|  | RRC Re-config.* | 56.904 | 40 | 0 | 0 | 0.00 | 0.00% |
|  | Total (every power cycle) |  |  |  | 40960 | 14233.54 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.35 |  |  |
|  | Battery life (in month) |  |  |  | 2.13 |  |  |
| Note*: value averaged based paging rate Note**: value averaged based on RRC re-config. frequency |  |  |  |  |  |  |  |

# Appendix 3: Summary for UE power consumption results

Table 76 Summary for UE power consumption results

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| Case DL-1-DS (high SNR) | 1.58 | 0.47 | No, 5.53 | No, 11.53 |
| Case DL-2-DS (high SNR) | 1.36 | 0.55 | No, 5.45 | No, 11.45 |
| Case DL-3-DS (high SNR) | 1.07 | 0.69 | No, 5.31 | No, 11.31 |
| Case DL-4-DS (high SNR) | 1.04 | 0.71 | No, 5.29 | No, 11.29 |
| Case DL-1-UDS (high SNR) | 4.34 | 0.17 | No, 5.83 | No, 11.83 |
| Case DL-2-UDS (high SNR) | 4.12 | 0.18 | No, 5.82 | No, 11.82 |
| Case DL-3-UDS (high SNR) | 0.56 | 1.33 | No, 4.67 | No, 10.67 |
| Case DL-4-UDS (high SNR) | 0.29 | 2.59 | No, 3.41 | No, 9.41 |
| Case DL-1-DS (low SNR) | 2.53 | 0.29 | No, 5.71 | No, 11.71 |
| Case DL-2-DS (low SNR) | 2.14 | 0.35 | No, 5.65 | No, 11.65 |
| Case DL-3-DS (low SNR) | 1.19 | 0.62 | No, 5.38 | No, 11.38 |
| Case DL-4-DS (low SNR) | 1.10 | 0.68 | No, 5.32 | No, 11.32 |
| Case DL-1-UDS (low SNR) | 5.33 | 0.14 | No, 5.86 | No, 11.86 |
| Case DL-2-UDS (low SNR) | 4.94 | 0.15 | No, 5.85 | No, 11.85 |
| Case DL-3-UDS (low SNR) | 0.68 | 1.09 | No, 4.92 | No, 10.92 |
| Case DL-4-UDS (low SNR) | 0.35 | 2.13 | No, 3.87 | No, 9.87 |
| Case UL-1-DS (high SNR) | 1.42 | 0.52 | No, 5.48 | No, 11.48 |
| Case UL-2-DS (high SNR) | 1.34 | 0.55 | No, 5.45 | No, 11.45 |
| Case UL-3-DS (high SNR) | 1.05 | 0.70 | No, 5.30 | No, 11.30 |
| Case UL-4-DS (high SNR) | 1.03 | 0.72 | No, 5.28 | No, 11.28 |
| Case UL-1-UDS (high SNR) | 4.18 | 0.18 | No, 5.82 | No, 11.82 |
| Case UL-2-UDS (high SNR) | 4.10 | 0.18 | No, 5.82 | No, 11.82 |
| Case UL-3-UDS (high SNR) | 0.54 | 1.38 | No, 4.62 | No, 10.62 |
| Case UL-4-UDS (high SNR) | 0.28 | 2.69 | No, 3.31 | No, 9.31 |
| Case UL-1-DS (low SNR) | 2.18 | 0.34 | No, 5.66 | No, 11.66 |
| Case UL-2-DS (low SNR) | 2.09 | 0.35 | No, 5.65 | No, 11.65 |
| Case UL-3-DS (low SNR) | 1.15 | 0.65 | No, 5.35 | No, 11.35 |
| Case UL-4-DS (low SNR) | 1.07 | 0.69 | No, 5.31 | No, 11.31 |
| Case UL-1-UDS (low SNR) | 4.98 | 0.15 | No, 5.85 | No, 11.85 |
| Case UL-2-UDS (low SNR) | 4.89 | 0.15 | No, 5.85 | No, 11.85 |
| Case UL-3-UDS (low SNR) | 0.64 | 1.17 | No, 4.83 | No, 10.83 |
| Case UL-4-UDS (low SNR) | 0.33 | 2.28 | No, 3.72 | No, 9.72 |
| Case DL-3-UDS (high SNR, paging triggered POS) | 0.53 | 1.41 | No, 4.89 | No, 10.89 |
| Case DL-4-UDS (high SNR, paging triggered POS) | 0.27 | 2.73 | No, 3.27 | No, 9.27 |
| Case DL-3-UDS (low SNR, paging triggered POS) | 0.63 | 1.18 | No, 4.82 | No, 10.82 |
| Case DL-4-UDS (low SNR, paging triggered POS) | 0.32 | 2.30 | No, 3.70 | No, 9.70 |
| Case DL-3-UDS (high SNR, PEI triggered POS) | 0.52 | 1.42 | No, 4.58 | No, 10.58 |
| Case DL-4-UDS (high SNR, PEI triggered POS) | 0.27 | 2.75 | No, 3.25 | No, 9.25 |
| Case DL-3-UDS (low SNR, PEI triggered POS) | 0.52 | 1.41 | No, 4.59 | No, 10.59 |
| Case DL-4-UDS (low SNR, PEI triggered POS) | 0.27 | 2.75 | No, 3.25 | No, 9.25 |
| Case UL-3-UDS (high SNR, paging triggered POS) | 0.52 | 1.41 | No, 4.59 | No, 10.59 |
| Case UL-4-UDS (high SNR, paging triggered POS) | 0.27 | 2.74 | No, 3.26 | No, 9.26 |
| Case UL-3-UDS (low SNR, paging triggered POS) | 0.62 | 1.19 | No, 4.81 | No, 10.81 |
| Case UL-4-UDS (low SNR, paging triggered POS) | 0.32 | 2.32 | No, 3.68 | No, 9.68 |
| Case UL-3-UDS (high SNR, PEI triggered POS) | 0.52 | 1.42 | No, 4.88 | No, 10.88 |
| Case UL-4-UDS (high SNR, PEI triggered POS) | 0.27 | 2.76 | No, 3.24 | No, 9.24 |
| Case UL-3-UDS (low SNR, PEI triggered POS) | 0.52 | 1.41 | No, 4.89 | No, 10.89 |
| Case UL-4-UDS (low SNR, PEI triggered POS) | 0.27 | 2.75 | No, 3.25 | No, 9.25 |
| Case DL-3-UDS (high SNR, with RRC re-config.) | 0.56 | 1.33 | No, 4.67 | No, 10.67 |
| Case DL-4-UDS (high SNR, with RRC re-config.) | 0.29 | 2.58 | No, 3.42 | No, 9.42 |
| Case DL-3-UDS (low SNR, with RRC re-config.) | 0.68 | 1.09 | No, 4.91 | No, 10.91 |
| Case DL-4-UDS (low SNR, with RRC re-config.) | 0.35 | 2.12 | No, 3.88 | No, 9.88 |
| Case DL-3-UDS (high SNR, without RRC re-config.) | 0.56 | 1.33 | No, 4.67 | No, 10.67 |
| Case DL-4-UDS (high SNR, without RRC re-config.) | 0.29 | 2.59 | No, 3.41 | No, 9.41 |
| Case DL-3-UDS (low SNR, without RRC re-config.) | 0.68 | 1.09 | No, 4.91 | No, 10.91 |
| Case DL-4-UDS (low SNR, without RRC re-config.) | 0.35 | 2.13 | No, 3.87 | No, 9.87 |