3GPP TSG RAN WG1 #111              	 R1-2212253

Toulouse, France, November 14th – 18th, 2022

Agenda Item: 9.10.1

Source: MediaTek Inc.

Title: On XR specific power saving techniques

Document for: Discussion and Decision

Introduction

During RAN1 #109e, the following can be drawn from the RAN1 #109e chairman’s note [1].

For PDCCH monitoring related power saving enhancements:

- High priority Issue 2-1: Enhancements to Rel-17 PDCCH monitoring adaptation

  - Focus on the techniques addressing XR-specific issues, e.g., jitter, re-transmission

During RAN1 #110, the following conclusion was made in the RAN1 #110 chairman’s note [2].

Conclusion

All the proposed PDCCH monitoring adaptation/reduction schemes including those for jitter handling need to be compared against the Rel-17 PDCCH monitoring adaptation which is to be used as performance reference.

During RAN1 #110bis-e, the following agreement was made in the RAN1 #110bis-e chairman’s note [3].

Agreement

For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1)

- Prioritize semi-static solutions

  - FFS: Whether dynamic solutions will be also needed

In this contribution, we further discuss the techniques for XR specific power saving enhancements.

Retransmission handling for Rel-17 PDCCH skipping

- 2.1 Background, Proposal, and Evaluation results

In Rel-17 the PDCCH skipping feature has been introduced under the power saving enhancements. The UE can be indicated to skip monitoring for a duration by the scheduling DCI format X-1/X-2. The feature brings good power saving gain while the scheduling is impacted especially for the retransmission requests.

Due to XR application’s stringent latency requirement, instant retransmission is critical. However, in the current Spec, no additional PDCCH monitoring for retransmission is allowed when PDCCH skipping is triggered. In one way, NW can handle the situation by applying a low MCS for last data scheduling to ensure a correct decoding while this comes with resource efficiency cost as the network needs to aim for a very low BLER target using a single shot transmission. Hence, further retransmission-aware mechanisms should be explored for XR traffic.

Observation 1: In current Spec, no additional PDCCH monitoring for retransmission is allowed when PDCCH skipping is triggered. Due to XR application’s stringent latency requirement, further retransmission-aware mechanisms should be explored.

We have two solutions to address the retransmission following the PDCCH skipping indication.

Solution 1: If UE is indicated to skip PDCCH monitoring for a specific duration based on the Rel-17 adaptation scheme, then if any NACK is transmitted, UE resumes the monitoring and cancels the indication of Skipping.

Proposal 1: (Solution 1) If UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted, UE resumes the monitoring and cancels the PDCCH skipping.

Solution 2: In solution 2, when UE is indicated to skip PDCCH monitoring for a duration by the Rel-17 adaptation scheme, then if any NACK is transmitted or if the scheduled UL data is transmitted, the UE has to resume PDCCH monitoring for a configured duration.

Proposal 2: (Solution 2) If UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted or if UL data is transmitted, UE resumes the PDCCH monitoring for a configured duration.

We have preference for solution 2 as it can be applied to both DL and UL. Also, it can get better power saving gain for an appropriate configured duration.

Observation 2: Solution 2 can be applied to both DL and UL. Also, it can get better power saving gain for an appropriate configured duration.

For Solution 2, we performed system level simulation (SLS) to evaluate the achievable power saving gain. The results are shown in Figure 1 below.

Figure 1 Power saving gain results by SLS for retransmission aware PDCCH skipping

In Figure 1, it can be seen that if UE does not resume PDCCH monitoring for the potential retransmission (y=0), the outage rate (ratio of unsatisfied UE) grows fast when the PDCCH skipping duration exceeds 4ms for cloud gaming (CG), and 2ms for XR. This means only short skip durations are acceptable for Rel-17 PDCCH skipping if there is no reTX handling.

Observation 3: In Figure 1, it can be seen that if UE does not resume PDCCH monitoring for the potential retransmission (y=0), the outage rate (ratio of unsatisfied UE) grows fast when the PDCCH skipping duration exceeds 4ms for cloud gaming (CG), and 2ms for XR. This means only short skip durations are acceptable for Rel-17 PDCCH skipping if there is no reTX handling.

At the same time, as shown in Figure 1, with Solution 2, UE performs PDCCH skipping with 12ms while resuming PDCCH monitoring for 5ms when a NACK is transmitted. With this ReTx handling method, it can achieve a significant power saving gain (20.78%~27.97%) w.r.t Rel-17 PDCCH skipping.

Observation 4: As shown in Figure 1, with Solution 2, UE performs PDCCH skipping with 12ms while resuming PDCCH monitoring for 5ms when a NACK is transmitted. With this ReTx handling method, it can achieve a significant power saving gain (20.78%~27.97%) w.r.t Rel-17 PDCCH skipping.

- 2.2 Resource impact of robust MCS for last data scheduling

As mentioned before, in one way, NW can handle this retransmission issue by applying a very low MCS for last data scheduling to ensure a correct decoding while this comes with resource efficiency cost as the network needs to aim for a very low BLER target using a single shot transmission.

Observation 5: In one way, NW can handle the retransmission issue by applying a low MCS for last data scheduling to ensure a correct decoding while this comes with resource efficiency cost as the network needs to aim for a very low BLER target using a single shot transmission.

To provide a quantified analysis for Observation 4, we developed some analysis for robust MCS for last data scheduling to ensure a correct decoding based on SLS simulation results. The target is to use a low MCS to make sure that UE can decode last data transmission successfully. The resource impact of this low MCS scheme is modelled by the following formula:

s=1,2,3,…ps×[s-1s+1s×SEbaselineSEadap]              (1)

where

- 𝑠: number of scheduled data transmissions (segments) for one frame, s = 1, 2, 3, ….

- ps: probability of one frame is transmitted by 𝑠 scheduled data transmissions (segments)

  - The value of 𝑠 and ps are derived from SLS results as shown in Figure 2

- SEbaseline :  The spectral efficiency (SE) from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [4], using the MCS calculated by gNB channel based on link adaptation algorithm in SLS

- SEadap :  The SE from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [4], using a low MCS 0 to 5

Figure 2 MCS index and number of scheduled data transmissions (segments) for one frame in XR application

The results of resource impact using formula (1) is shown below in Figure 3 below. It can be seen that the robust MCS scheme can consume 1.72 times to 7.31 times of resources with MCS 0 to MCS 5, which is a large overhead for XR application.

Figure 3 Resource impact of robust MCS for last data scheduling to ensure a correct decoding

Observation 6: Figure 3 (using Formula (1) below) shows that the robust MCS scheme for last data scheduling to ensure a correct decoding can consume 1.72 times to 7.31 times of resources with MCS 0 to MCS 5, which is a large overhead for XR application.

The resource impact is modelled by the following formula:

s=1,2,3,…ps×[s-1s+1s×SEbaselineSEadap]              (1)

where

- 𝑠: number of scheduled data transmissions (segments) for one frame, s = 1, 2, 3, ….

- ps: probability of one frame is transmitted by 𝑠 scheduled data transmissions (segments)

  - The value of 𝑠 and ps are derived from SLS results as shown in Figure 2

- SEbaseline :  The spectral efficiency (SE) from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [4], using the MCS calculated by gNB channel based on link adaptation algorithm in SLS

- SEadap :  The SE from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [4], using a low MCS 0 to 5

- 2.3 Evaluation results of retransmission aware PDCCH skipping with CDRX

During RAN1 #110bis-e discussion [5], Nokia/ZTE mentions that our SLS simulation result is not considering CDRX and the impact of transition of active/inactive time is not included. The reason we did not include CDRX is that legacy CDRX is not suited for XR traffic and would evidently increase the outage rate.

Observation 7: During RAN1 #110bis-e discussion [5], Nokia/ZTE mentions that MTK SLS simulation result is not considering CDRX and the impact of transition of active/inactive time is not included. The reason CDRX was not included is that legacy CDRX is not suited for XR traffic and would evidently increase the outage rate.

However, to demonstrate that retransmission aware PDCCH skipping also provides power saving gain when operating with CDRX, we performed SLS simulation to evaluate power saving gain of retransmission aware PDCCH skipping with (16,10,5) CDRX cycle.

Figure 4 Power saving gain results by SLS for retransmission aware PDCCH skipping with CDRX

It can be seen from Figure 4 that compared with Rel-17 PDCCH skipping, when CDRX comes into play, retransmission aware PDCCH skipping with CDRX still achieves higher power saving gain (15.29% compared to Rel-17) with lower outage rate increment.

Observation 8: To demonstrate that retransmission aware PDCCH skipping also provides power saving gain when operating with CDRX, SLS simulation is performed to evaluate power saving gain of retransmission aware PDCCH skipping with (16,10,5) CDRX cycle. From Figure 4, it can be seen that when CDRX comes into play, retransmission aware PDCCH skipping with CDRX still achieves higher power saving gain (15.29% compared to Rel-17 PDCCH skipping) with lower outage rate increment.

- 2.4 Evaluation results of retransmission aware PDCCH skipping with UL pose/control

During RAN1 #110bis-e discussion [5], ZTE mentions that UL transmission should be jointly considered/optimized. To evaluate the impact of UL transmission, we performed a joint DL/UL SLS simulation for VR application to see how much power saving gain can be obtained with retransmission aware PDCCH skipping in DL.

Figure 5 Power saving gain results by SLS for DL retransmission aware PDCCH skipping with UL pose/control

It can be seen from Figure 5 that compared with Rel-17 PDCCH skipping, when UL pose/control (4ms period) comes into play, DL retransmission aware PDCCH skipping still achieves 14.27% power saving gain compared to Rel-17 PDCCH skipping.

Observation 9: During RAN1 #110bis-e discussion [5], ZTE mentions that UL transmission should be jointly considered/optimized. To evaluate the impact of UL transmission, a joint DL/UL SLS simulation is performed for VR application. From Figure 5, it can be seen that when UL pose/control (4ms period) comes into play, DL retransmission aware PDCCH skipping still achieves 14.27% power saving gain compared to Rel-17 PDCCH skipping.

For UL retransmission optimization, we think it can be jointly considered/enhanced if necessary in R18 XR WI phase.

Proposal 3: RAN1 to specify a DL retransmission aware PDCCH skipping in R18 XR WI; UL retransmission optimization can be jointly considered/enhanced if necessary in R18 XR WI.

For potential enhancement of UL retransmission, we have the following observation.

Observation 10: For potential enhancement of UL retransmission, the following can be considered:

- For UL pose control: retransmission-less configured grant (CG) as proposed by Qualcomm in [6]

- For UL AR video: UL (re-)transmission handling in C-DRX as proposed by Huawei in [7]

CDRX enhancement for XR

In this section, we introduce some CDRX enhancement for XR. However, since the spec impact of CDRX enhancement mainly falls in RAN2, we would suggest RAN1 to capture the evaluation results (including the excel sheet) of CDRX enhancement into the TR and leave remaining works to RAN2.

Proposal 4: RAN1 to capture the evaluation results (including the excel sheet) of CDRX enhancement into the TR and leave remaining works to RAN2, since the spec impact of CDRX enhancement mainly falls in RAN2.

- 3.1 Aligning C-DRX on-duration with XR traffic

In TR 38.838, XR traffic is modelled as packets belonging to media frames arriving with a periodicity based on the frame rate and with a random jitter. The jitter follows truncated Gaussian distribution. The traffic model can be illustrated as below as in Figure 6:

Figure 6 XR traffic model

For XR, it is crucial that the C-DRX on-duration occasions match the traffic periodicity. Otherwise, the packets arriving at the transmitter will need to wait for the next on-duration, causing some packets to miss the target packet delay budget (PDB), therefore reducing the capacity. Both the PDB and the capacity are very limited in XR.

For example, with a frame rate of 60fps and jitter range of [-4,4]ms, the C-DRX on-durations aligned with the arrival of XR traffic would look like below in Figure 7.

Figure 7 C-DRX on-duration aligned with the XR traffic

In legacy (up to Rel-17) C-DRX, the DRX cycle can only take integer values between 10ms and 10240ms. Even if new integer values are introduced, such as 16ms to approximately match the 60fps frame rate (with traffic periodicity 100/60=50/3~16.67ms), the DRX on-duration would gradually drift out of sync with XR traffic as illustrated in Figure 8 below. This can cause a drop in UE satisfaction rate due to the misalignment with traffic and the increase in latency as shown in Table 1 below.

Figure 8 C-DRX on-duration drift using integer DRX cycles

Observation 11: It is not possible to align DRX on-duration occasions with XR traffic using legacy DRX cycles with integer values. This can cause a drop in UE satisfaction rate as shown in Table 1 below.

Instead, the DRX on-duration occasions can be aligned with XR traffic by introducing rational number (integer division) values for DRX cycles, for example 50/3ms. System level simulation (SLS) evaluation results for baseline (always-on), legacy C-DRX, and eC-DRX with rational number DRX cycle schemes are presented in Table 1 and Figure 9 below where

- eC-DRX: C-DRX matching the traffic periodicity, with DRX cycle = 50/3 ms

- PSG is calculated as:

PSG = (Baseline power - Power)/Baseline power × 100%

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | % of DL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AlwaysOn - baseline | 0 | 0 | 0 | 12 | 12 | 94.6% | 0% |
| R17 CDRX | 16 | 12 | 8 | 12 | 12 | 92.7% | 5.4% |
| eCDRX (rational DRX cycle) | (50/3) | 12 | 8 | 12 | 12 | 94.3% | 9.9% |

Table 1 SLS evaluation results with eC-DRX, FR1, DL-only, DU, CG 30Mbps

Figure 9 Comparison of SLS evaluation results with eC-DRX

From SLS evaluation, it is observed that eC-DRX using a DRX cycle with a rational value (such as 50/3) matching the CG traffic improves PSG by 4.5%, while improving UE satisfaction rate by 1.6% over Rel-17 C-DRX.

Observation 12: As shown by SLS results in Figure 9, eC-DRX using rational DRC cycle value matching CG traffic improves both power savings and UE satisfaction rate compared to Rel-17 C-DRX (PSG improved by 4.5%, UE satisfaction rate improved by 1.6%).

We therefore propose that the legacy C-DRX formulas should be enhanced to support non-integer (rational number) DRX cycles to match the periodicity of XR traffic.

Proposal 5: Introduce non-integer (rational number) DRX cycles to match typical XR traffic patterns.

- 3.1 Reduce C-DRX On-Duration after data arrival

According to the XR/CG traffic model, the packet arrival time includes a jitter. The jitter follows truncated Gaussian distribution and as baseline is within range [-4, 4] ms.

The UE needs to monitor PDCCH during the jitter period, which means that the UE will be awake for most of the DRX cycle. Therefore, drx-onDurationTimer (ODT) in C-DRX, which is started at the beginning of every DRX cycle, may need to be configured with a large value (e.g., 8ms).

On the other hand, after the first packet for a media frame is detected, the subsequent packets belonging to the same frame would usually be transmitted within a relatively short period of time. Therefore, drx-InactivityTimer (IAT), where the UE is waiting for a subsequent PDCCH indicating a new DL or UL transmission, can be configured with a relatively small value compared to the ODT. Note the UE monitors PDCCH during Active Time, which includes the time while either timer (ODT or IAT) is running.

To save the UE power, it can be beneficial to reduce the duration where the UE monitors for PDCCH as much as possible. As a potential enhancement, the UE can stop the ODT at the first data arrival and can rely on IAT while waiting for the next packet for the same media frame, as shown below in Figure 10. This can save power for UEs where the first packet for a media frame arrives early during the on-duration. Note that the IAT can still be running so that the UE monitors the PDCCH for any subsequent packet(s) belonging to the same media frame.

Figure 10 Stopping ODT early after data arrival

The system level simulation (SLS) evaluation results for this enhancement are presented in Table 2 and Figure 11 below where

- Stop ODT: Stop ODT early + eC-DRX with DRX cycle = 50/3 ms

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | % of DL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AlwaysOn - baseline | 0 | 0 | 0 | 12 | 12 | 94.6% | 0% |
| R15/16CDRX | 16 | 12 | 8 | 12 | 12 | 92.7% | 5.4% |
| eCDRX (rational DRX cycle) | (50/3) | 12 | 8 | 12 | 12 | 94.3% | 9.9% |
| eCDRX + stop ODT early | (50/3) | 12 | 8 | 12 | 12 | 92.4% | 16.6% |

Table 2 SLS evaluation results with stop ODT early, FR1, DL-only, DU, CG 30Mbps

Figure 11 Comparison of SLS evaluation results with Stop ODT early

From SLS evaluation, it is observed that stopping ODT early on traffic arrival + eC-DRX improves PSG by 11.2% with marginal reduction in UE satisfaction rate by 0.3% compared to Rel-17 C-DRX.

Observation 13: Stopping ODT early + eC-DRX provides significant power savings with marginal impact on UE satisfaction rate compared to Rel-17 C-DRX (11.2% PSG and 0.3% reduction in UE satisfaction rate) as shown in Table 2 and Figure 11.

We therefore propose the following:

Proposal 6: Reduce DRX on-duration after the arrival of data by stopping ODT to enable the UE to go to sleep early.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: In current Spec, no additional PDCCH monitoring for retransmission is allowed when PDCCH skipping is triggered. Due to XR application’s stringent latency requirement, further retransmission-aware mechanisms should be explored.

Proposal 1: (Solution 1) If UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted, UE resumes the monitoring and cancels the PDCCH skipping.

Proposal 2: (Solution 2) If UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted or if UL data is transmitted, UE resumes the PDCCH monitoring for a configured duration.

Observation 2: Solution 2 can be applied to both DL and UL. Also, it can get better power saving gain for an appropriate configured duration.

Observation 3: In Figure 1, it can be seen that if UE does not resume PDCCH monitoring for the potential retransmission (y=0), the outage rate (ratio of unsatisfied UE) grows fast when the PDCCH skipping duration exceeds 4ms for cloud gaming (CG), and 2ms for XR. This means only short skip durations are acceptable for Rel-17 PDCCH skipping if there is no reTX handling.

Observation 4: As shown in Figure 1, with Solution 2, UE performs PDCCH skipping with 12ms while resuming PDCCH monitoring for 5ms when a NACK is transmitted. With this ReTx handling method, it can achieve a significant power saving gain (20.78%~27.97%) w.r.t Rel-17 PDCCH skipping.

Observation 5: In one way, NW can handle the retransmission issue by applying a low MCS for last data scheduling to ensure a correct decoding while this comes with resource efficiency cost as the network needs to aim for a very low BLER target using a single shot transmission.

Observation 6: Figure 3 (using Formula (1) below) shows that the robust MCS scheme for last data scheduling to ensure a correct decoding can consume 1.72 times to 7.31 times of resources with MCS 0 to MCS 5, which is a large overhead for XR application.

The resource impact is modelled by the following formula:

s=1,2,3,…ps×[s-1s+1s×SEbaselineSEadap]              (1)

where

- 𝑠: number of scheduled data transmissions (segments) for one frame, s = 1, 2, 3, ….

- ps: probability of one frame is transmitted by 𝑠 scheduled data transmissions (segments)

  - The value of 𝑠 and ps are derived from SLS results as shown in Figure 2

- SEbaseline :  The spectral efficiency (SE) from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [4], using the MCS calculated by gNB channel based on link adaptation algorithm in SLS

- SEadap :  The SE from MCS index table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [4], using a low MCS 0 to 5

Observation 7: During RAN1 #110bis-e discussion [5], Nokia/ZTE mentions that MTK SLS simulation result is not considering CDRX and the impact of transition of active/inactive time is not included. The reason CDRX was not included is that legacy CDRX is not suited for XR traffic and would evidently increase the outage rate.

Observation 8: To demonstrate that retransmission aware PDCCH skipping also provides power saving gain when operating with CDRX, SLS simulation is performed to evaluate power saving gain of retransmission aware PDCCH skipping with (16,10,5) CDRX cycle. From Figure 4, it can be seen that when CDRX comes into play, retransmission aware PDCCH skipping with CDRX still achieves higher power saving gain (15.29% compared to Rel-17 PDCCH skipping) with lower outage rate increment.

Observation 9: During RAN1 #110bis-e discussion [5], ZTE mentions that UL transmission should be jointly considered/optimized. To evaluate the impact of UL transmission, a joint DL/UL SLS simulation is performed for VR application. From Figure 5, it can be seen that when UL pose/control (4ms period) comes into play, DL retransmission aware PDCCH skipping still achieves 14.27% power saving gain compared to Rel-17 PDCCH skipping.

Proposal 3: RAN1 to specify a DL retransmission aware PDCCH skipping in R18 XR WI; UL retransmission optimization can be jointly considered/enhanced if necessary in R18 XR WI.

Observation 10: For potential enhancement of UL retransmission, the following can be considered:

- For UL pose control: retransmission-less configured grant (CG) as proposed by Qualcomm in [6]

- For UL AR video: UL (re-)transmission handling in C-DRX as proposed by Huawei in [7]

Proposal 4: RAN1 to capture the evaluation results (including the excel sheet) of CDRX enhancement into the TR and leave remaining works to RAN2, since the spec impact of CDRX enhancement mainly falls in RAN2.

Observation 11: It is not possible to align DRX on-duration occasions with XR traffic using legacy DRX cycles with integer values. This can cause a drop in UE satisfaction rate as shown in Table 1 below.

Observation 12: As shown by SLS results in Figure 9, eC-DRX using rational DRC cycle value matching CG traffic improves both power savings and UE satisfaction rate compared to Rel-17 C-DRX (PSG improved by 4.5%, UE satisfaction rate improved by 1.6%).

Proposal 5: Introduce non-integer (rational number) DRX cycles to match typical XR traffic patterns.

Observation 13: Stopping ODT early + eC-DRX provides significant power savings with marginal impact on UE satisfaction rate compared to Rel-17 C-DRX (11.2% PSG and 0.3% reduction in UE satisfaction rate) as shown in Table 2 and Figure 11.

Proposal 6: Reduce DRX on-duration after the arrival of data by stopping ODT to enable the UE to go to sleep early.

References

- Chairman’s Notes, RAN1 #109-e (9.11.1 XR-specific power saving techniques)

- Chairman’s Notes, RAN1 #110 (9.11.1 XR-specific power saving techniques)

- Chairman’s Notes, RAN1 #110-bis-e (9.10.1 XR-specific power saving techniques)

- 3GPP TS 38.214 V17.3.0, 5G; NR; Physical layer procedures for data

- R1-2210338, “Moderator Summary#2 on XR specific power saving techniques”, Moderator (Qualcomm Incorporated), RAN1 #110-bis-e

- R1-2210002, “ Power Saving Techniques for XR”, Qualcomm, RAN1 #110-bis-e

- R1-2208420, “ Discussion on XR-specific power saving techniques, Huawei, RAN1 #110-bis-e

Appendix

Table 5.1.3.1-1 and 5.1.3.1-2 in 38.214 [3]