3GPP TSG RAN WG1 #111			R1-22122426

Meeting, Nov 14th – 18th, 2022

Source: 	CEWiT

Title:	Discussion on evaluation methodology and inital results for beam 	enhancement on sidelink FR2 operation

Agenda: 	9.4.3

Document for:	Decision

## Introduction

New radio (NR) sidelink (SL) communication is used to share information between device to device without or with only minimal involvement of the gNB. SL is evolved in NR Rel-15 and stabilized in Rel-17 for FR1 operation. But FR2 support for SL is limited. Rel-18 is planning to introduce FR2 enhancements/optimization mainly focusing on beam management aspects. In RAN#97, it is agreed to evaluate the enhancements for FR2 till RAN#98. The detailed objective of the study is as follows [1],

- Study and specify enhanced SL operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18)

  - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1]

  - Work is limited to the support of SL beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing SL-CSI framework and reusing Uu beam management concepts wherever possible.

    - Beam management in FR2 licensed spectrum considers SL unicast communication only.

Base on this objective in RAN # 110b-e initial simulation scenarios and parameters are agreed for analysing the FR2 enhancements. Following are the agreement from last meeting.

Agreement

- In evaluation methodology for commercial deployment scenario for sidelink operation on FR2

- Reuse indoor layout defined for SL-U with pairs topology and without WiFi nodes

  - FFS: total number of UEs deployed in the layout

  - Companies should report how UEs are paired

- FFS: whether to consider the cluster-based topology defined for SL-U

- Note: for the evaluation, there is no Uu link in this indoor layout

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with

- Option 1: 7 macro sites with 3 cells per site

- Option 2: a single site

- Companies should report how UEs are paired

- FFS: total number of UEs deployed in the layout

- FFS: whether Uu and PC5 use same carrier

FFS: ISD for this layout option 3

Agreement

For the indoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the total number of UEs is 12 pairs/20 MHz with scaling factors of 1, ½ or 1/3.

Agreement

For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the number of UEs per cell is 60 with scaling factors of 1, ½ or 1/3.

Agreement

For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, Uu link has different carrier as PC5 in the simulation is the baseline

- Optional: Uu link has same carrier as PC5 in the simulation.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the outdoor layout, the channel model reuses the procedures and parameters for UMi - Street Canyon specified in TR 38.901.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the indoor layout, the channel model reuses the procedures and parameters for InH mixed office specified in TR 38.901.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for UE antenna parameters, reuse the antenna element pattern and antenna array configuration for pedestrian UE and cellular UE as in Table 6.1.4-6 and Table 6.1.4-7 of TR 37.885.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, consider at least the following parameters:

- Carrier frequency: 30 GHz

- Sub-carrier spacing: 120 kHz(baseline), 60 kHz(optional)

- Simulation bandwidth: 100 MHz (baseline), 200 MHz (optional)

- UE receiver noise figure: 13 dB (baseline), 10 dB (optional)

- UE Tx power: 23 dBm (EIRP should not exceed 43 dBm)

- UE speed: 3 km/h

Agreement

For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, ISD is 200 meters.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, support at least the following traffic model:

- Option 1: periodic traffic mode 3

  - Packet size scaling factor is up to companies’ porting

- Option 2: FTP model 3 with arrival rate satisfying one of the followings:

  - BO low load: 10%-25%

  - BO mid load: 35%-50%

  - BO high load: above 55%

  - Packet size is up to companies’ reporting

- Option 3: XR traffic models including cloud gaming, virtual reality, and augmented reality.

- It is up to each company to use either Option 1 or 2 or 3 or mixed of them.

Agreement

When reporting the simulation results for sidelink operation on FR2, companies should report the used resource allocation scheme.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure.

FFS: UE satisfaction as section 7.2 in TR 38.838 for XR traffic evaluation

This contribution provides the initial evaluation results for agreed parameters and further discussion on evaluation methodology used for FR2 evaluation in sidelink enhancement.

## Sidelink Evaluation on FR2:

For analysing the performance of sidelink in FR2, we perform the simulation in indoor and outdoor scenarios for by considering all agreements from RAN1 #110b-e. We drop the users in the layouts randomly in both the cases and paired to generate the communication links between the UEs. The UEs pairing is explained in the following sub section.

### UE pairing:

- Indoor Scenario:

Random Pairing: We drop the users randomly in the layout. We choose the Tx-UEs and Rx-UEs randomly in the dropped users. We paired the Tx-UEs with Rx-UEs randomly.

Distance-based Pairing: We drop the users randomly in the layout. Pairing is performed by following steps

  - We first choose a Tx-UE1 randomly and paired that Tx-UE1 with the nearest user in the dropped users.

  - Next, we choose another Tx-UE2 randomly in the remaining dropped users and paired with the nearest user in the dropped users.

  - Similarly, for all the dropped users, we are choosing Tx-UE and Rx-UE pairs.

- Outdoor Scenario:

Random Pairing: We drop the users randomly in the layout. We choose the Tx-UEs and Rx-UEs randomly in the dropped users. We paired the Tx-UEs with Rx-UEs randomly.

Distance-based Pairing: We drop the users randomly in the layout. Pairing as following the steps below

  - We first choose a Tx-UE1 randomly and paired Tx-UE1 with the nearest user in the dropped users.

  - Next, we choose another Tx-UE2 randomly in the remaining dropped users and paired with the nearest user in the dropped users.

  - Similarly, for all the dropped users, we paired all the users.

### Evaluation Results:

We perform the simulation for the Indoor and Outdoor scenarios to compute PRR by transmitting the random data. We present the PRR performance with varying SNR values. While detecting the transmitting signal, we use the simple linear detector at the receiver. The Figure 1 represents the PRR performance for indoor scenario with consideration of the agreements. We observed from the Figure 1 that the distance-based paring case gave best PRR compared to random pairing. Similarly, for the outdoor scenario.

Observation 1: In commercial use cases for sidelink on FR2, the distance-based pairing of UEs gave best performance in both the indoor and outdoor scenarios.

In commercial use cases for sidelink on FR2, the two UEs are communicating directly with each other. In FR 2, there may be a possibility for blockage of signals from human body as well, particularly in outdoor scenarios there may be blockage of signals form vehicles. So, it is important to simulate the performance in presence of environmental blockages. Therefore, we propose to use the proper blockage models for the channel generation in the evaluation for both the scenarios.

Proposal 1: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the blockage models either Model A or Model B presented in sections 7.6.4.1 and 7.6.4.2 respectively in TR 38.901 can be used.

Proposal 2: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for link level simulations SNR can be used as the performance metric.

Figure 1: PRR for Indoor Scenario.

In Figure 2, we are presenting the comparison with the omni antenna configuration with parabolic model antenna configuration. It mimics the beamforming capability of UE by tilting the beam of the antenna as per the UE direction. We can observe from the Figure 2 the performance improvement with the parabolic antenna model. So, we can expect more performance improvement with the beam management in the sidelink communication system. So we propose to study the beam management in sidelink to enhance the performance in the system.

Observation 2: We observed that the performance improvement with the parabolic antenna model compared to the omni direction antenna model.

Figure 2: PRR comparison with omni and parabolic antenna configurations.

Proposal 3: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the beam management can be studied to enhance the performance of the sidelink.

In FR2, particularly for outdoor scenarios, there may be possibility for placement of more number of antennas with multiple panels. So, we propose to use more antenna configurations with more antennas and multiple panels for evaluation.

Proposal 4: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for outdoor scenario, more antenna configurations with more number of antennas and with multiple panels can be considered.

## Conclusion:

The contribution draws the following proposals,

Observation 1: In commercial use cases for sidelink on FR2, the distance-based pairing of UEs gave best performance in both the indoor and outdoor scenarios.

.

Proposal 1: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the blockage models either Model A or Model B presented in sections 7.6.4.1 and 7.6.4.2 respectively in TR 38.901 can be used.

Proposal 2: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for link level simulations SNR can be used as the performance metric.

Proposal 3: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the beam management can be used to enhance the performance of the sidelink.

Proposal 4: In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for outdoor scenario, more antenna configurations with more number of antennas and with multiple panels can be considered.

## References

- RP-221938 - Revised WID on NR sidelink evolution

- TS 38.845 V 2.0.0- Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases