3GPP TSG RAN WG1 #111	R1-2212433

Toulouse, France, November 14th – 18th, 2022

Agenda item:		9.11.4

Source:	Nordic Semiconductor ASA

Title:					Improved GNSS operation for IoT NTN

Document for:		Discussion and Decision

# Introduction

In RAN#95, work item in RP-220979 has been approved and updated in RP-221806. The objectives regarding improved GNSS operation are the following

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

In the previous meeting, RAN1#110, the following agreements were made under this agenda item:

Agreement

Support eNB to at least aperiodically trigger UE to make GNSS measurement.

Agreement

If eNB aperiodically triggers UE to make GNSS measurement, a MAC CE is used.

Agreement

UE reports GNSS position fix time duration for measurement at least during the initial access stage

- which message carries this information is up to RAN2

Agreement

In connected mode, UE may report GNSS validation duration with MAC CE.

In the following, we address the above issues and all other open issues pertaining to this agenda item.

# Improved GNSS operation

From UE point of view, Rel-17 IoT NTN system was designed for sporadic short transmissions in RRC Connected mode and after the short transmission UE goes back to Idle mode. From [1]:

For sporadic short transmission:

-	The idle UE wakes up from idle DRX / PSM, access the network, perform uplink and/or downlink communications for a short duration of time and go back to idle.

-	Before accessing the network, the UE acquires GNSS position fix and does not need to re-acquire a GNSS position fix for the transmission of the packets.

How long UE can stay in Connected mode depends on validity timer of UL synchronization and on validity of GNSS position fix of the UE. The UE autonomously determines its GNSS validity duration X and reports information associated with this validity duration to the network via RRC signalling. The validity duration X may get the values:

  - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

From power saving point of view, it would be beneficial if UE could stay in RRC Connected mode until an active data transmission has stopped, as is the case in TN. Therefore, it would be good if UE could acquire a new GNSS position fix while in Connected mode if GNSS validity timer expires or if the eNB notice that the UE is about losing its UL synchronization. Typically, an updated GNSS fix (“hot” fix) takes considerably less time than the first GNSS fix (“cold” fix) done before entering the Connected mode. However, even the acquisition of “hot” fix requires tens of radio frames and generally UE cannot resort to the “legacy” gaps in N/MPDCCH monitoring settings.

In the previous meeting, it was agreed that the closed loop time and frequency correction loops are considered in order to reduce the need for acquiring a new GNSS fix in RRC-connected mode. The closed loop timing correction has already been supported since Rel-17 for IoT NTN. Likewise, the frequency error due to UE velocity could be tracked by a closed loop correction loop similar to closed loop timing correction mechanism, as discussed e.g. in [2]. However, a mechanism to recover is needed in the case that the closed loop correction fails at some point in the RRC-Connected mode.

Proposal 1: In RRC connected mode, the closed loop time and frequency correction loops could be considered as a complementary mechanism to GNSS gaps

# Conclusions

In this contribution we discussed issues related to improved GNSS operation for IoT NTN and made the following proposals:

Proposal 1: In RRC connected mode, the closed loop time and frequency correction loops should be further studied.

# References

[1] RP-211456, 3GPP TR 36.763 v1.0.0 Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)

[2] R1-2205060, Improved GNSS Operations for IoT NTN