3GPP TSG RAN WG1 #111		R1- 2212759

Toulouse, France, November 14th – 18th, 2022

Agenda item:	9.1.1.1

Source: 	Moderator (MediaTek)

Title: 	Moderator summary on extension of unified TCI framework (Round 1)

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 WID of MIMO evolution for downlink and uplink is approved. In the approved WID, extension of unified TCI framework is a part of the RAN1 objectives, and the detailed scope of this agenda item (AI 9.1.1.1) includes the following highlighted objectives:

RAN1: - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

- Plan

Based on the contributions from companies [1]-[32], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow:

  - Issue 1 – General framework for unified TCI extension

  - Issue 2 – TCI state update and activation

  - Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal

  - Issue 4 – UL power control for UL MTRP

  - Issue 5 – PDSCH-CJT

  - Issue 6 – Beam reporting and beam failure recovery

- Observations and recommended proposals based on the summary of companies’ views

The summary will be used for our 2nd R18 MIMO online discussion on Wednesday afternoon (14:30-15:00 local time in Toulouse). Please upload your input to the draft folder, if any, by 13:00 local time before the launch break.

- Contact Person

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| MediaTek | Darcy | darcy.tsai@mediatek.com |
| Panasonic | Khalid | khalid.zeineddine@eu.panasonic.com |
| FGI | Cubie | wanchen.lin@fginnov.com |
| Ericsson | Claes | Claes.tidestav@ericsson.com |
| vivo | Yang | songyang@vivo.com |
| Sharp | Taka | fukui.takahisa@sharp.co.jp |
| ZTE | Bo | gao.bo1@ZTE.com.cn |
| OPPO | Jeffrey | caojianfei@oppo.com |
| MediaTek | Rebecca | rebecca.chen@mediatek.com |
| Google | Alex | alexliou@google.com |
| Qualcomm | Yan | yanzhou@qti.qualcomm.com |
| Futurewei | Zhigang | zrong@futurewei.com |
| NTT DOCOMO | Yuki | yuki.matsumura@docomo-lab.com |
| NTT DOCOMO | Weiqi | sunwq@docomolabs-beijing.com.cn |
| LG | Jaehoon | jhoon.chung@lge.com |
| NEC | Peng | guan_peng@nec.cn |
| CMCC | Jun | zuojun@chinamobile.com |
| Intel | Avik | avik.sengupta@intel.com |
| Huawei, HiSilicon | Keyvan | Keyvan.zarifi@huawei.com |
| InterDigital | Jonghyun | jonghyun.park@interdigital.com |
| Samsung | Dalin | dalin.zhu@samsung.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| Fujitsu | Jian | zhangjian1288@fujitsu.com |
| Fraunhofer IIS/HHI | Sutharshun | sutharshun.varatharaajan@iis.fraunhofer.de |
| Apple | Hong | hhe5@apple.com |
| Spreadtrum | Qiyishu Li | qiyishu.li@unisoc.com |
| Lenovo | Bingchao Liu | liubc2@lenovo.com |

- Proposals to be discussed in the 1st online session

Proposal 5.2 (offline consensus): On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT):

- Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA

- Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state

- Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

Proposal 3.4: On unified TCI framework extension for S-DCI based MTRP:

- If two SRS resource sets for CB/NCB are configured, the UE shall apply the two indicated joint/UL TCI states to the two SRS resource sets for CB/NCB, respectively.

  - FFS: The applying order of the indicated joint/UL TCI states to the two SRS resource sets for CB/NCB is fixed in specification or can be configured by RRC

- The UE shall determine the UL transmission filter, PL-RS, and if provided, UL PC parameter setting for PUSCH (including P0, alpha, and closed loop index) for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG-PUSCH and Type2 CG-PUSCH) from the indicated joint/UL TCI state(s) applied to the SRS resource(s) indicated by the DCI format 0_1/0_2

Proposal 3.4.1: On unified TCI framework extension for S-DCI based MTRP:

- Use the existing SRS resource set indicator in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 (including DG-PUSCH and Type2 CG-PUSCH)

- Note: The UL transmission filters used for the scheduled/activated PUSCH transmission and the indicated SRS resource are always identical based current specification that the UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the indicated SRS resource.

Proposal 3.7: On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH)  is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH

Proposal 3.2: On unified TCI framework extension for S-DCI based MTRP:

- For a CORESET other than a CORESET with index 0 that is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET can be provided with RRC configuration that informs that the UE shall apply the first one, the second one, or both of the indicated joint/DL TCI states to PDCCH reception on the CORESET

- For a CORESET other than a CORESET with index 0 that is associated at least with CSS sets other than Type3-PDCCH CSS sets, the CORESET can be provided with RRC configuration that informs that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET

- For a CORESET with index 0, the CORESET can be provided with RRC configuration that informs that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET

Proposal 6.1: On unified TCI framework extension for S-DCI based MTRP, if a BFD-RS set is not explicitly provided to a UE for TRP-specific BFR, the UE determines one BFD-RS for the BFD-RS set from an indicated joint/DL TCI state if the indicated joint/DL TCI state is configured to apply for PDCCH reception

- FFS: Determine from which one of the indicated joint/DL TCI states

- FFS: The case if CORESET(s) is configured to apply both indicated joint/DL TCI states for PDCCH-SFN

- Discussion

- Issue 1 – General framework for unified TCI extension

Void

- Issue 2 – TCI state update and activation

Table 2-1 Summary for Issue 2

| # | Issue | Companies’ view and summary |
| --- | --- | --- |
| 2.1 | TCI state update for S-DCI based MTRP, down-selection from the alternatives agreed in RAN1#110bis | Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS |
| 2.2 | For S-DCI based MTRP, UE behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP | Question 1: Whether the switching between STRP and MTRP can depend on the number of TCI states indicated in a beam indication DCI (e.g., the UE assumes STRP operation when beam indication DCI indicates TCI state(s) only for one TRP, the UE assumes MTRP operation when beam indication DCI indicates TCI states for both TRPs) - Yes: Qualcomm (if no new field for switch), vivo, Xiaomi, OPPO - No: Ericsson, Google, Huawei/HiSilicon, InterDigital, MTK, Spreadtrum, Panasonic, ZTE, CATT, LG, Sharp, Docomo,CMCC, Intel, Transsion Question 2: If the answer to Q1 is no, in what case(s)/scenario(s), the UE shall assume MTRP operation and maintain the indicated TCI states for both TRPs? Please share your answer o Q2 in Table 2-2. FL note: Please share your view on above two questions |
| 2.3 | Common TCI state ID activation/update for the set of CCs configured in the same CC list | Question 1: Whether a CC list can be comprised of a mix of STRP CC(s) and MTRP CC(s) - Yes: OPPO, Spreadtrum, FGI, IDC, Huawei - No: Xiaomi, QC, NEC Question 2: Whether a CC list can be comprised of a mix of S-DCI based MTRP CC(s) and M-DCI based MTRP CC(s) - Yes: OPPO, FGI, IDC, Huawei - No: Xiaomi, Spreadtrum, QC, NEC FL note: Please share your view on above two questions |
| 2.4 | Combinations of joint/DL/UL TCI states that can be mapped to a TCI codepoint of the existing TCI field for S-DCI based MTRP | FL note: Please check Proposal 2.4 directly |

Proposal 2.4: On unified TCI framework extension for S-DCI based MTRP:

- For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {the first joint TCI state, the second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

- For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {the first DL TCI state, the first UL TCI state, the second DL TCI state, the second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

Table 2-2 Company inputs for Issue 2

| Company | Input |
| --- | --- |
| Mod | Please update your view on Issue 2.3 in Table 2-1 |
| Xiaomi | Q1: no, since TCI state activation/update for STRP should follow the mechanism of M-DCI based MTRP. Q2: no, since different type of MAC CE for TCI state activation/update is used for S-DCI based MTRP and M-DCI based MTRP. |
| OPPO | Q1: Yes. There was no such restriction in legacy for MTRP operation. In addition, the STRP and MTRP operation can be dynamically switched, hence both should be allowed to be operated in different CCs in the same CC list. As for unified TCI state indication, we think this should be open for discussion. Q2: Yes. Even thin a single CC for MTRP operation, it’s up to a TRP to send S-DCI or M-DCI. For multiple CCs, we don’t think restriction should be introduced. Of course, RAN1 should figure out how to get it done. |
| Spreadtrum | For Q1 Yes. Since cc list is configured by RRC and dynamic STRP/MTRP switching may be supported, it is possible that both STRP and MTRP CCs are mixed in a CC list. For Q2 No. At least for CA, the CCs in a list are assumed to have similar channel propagation scenario (so it may be not happened that a TRP has an ideal back-haul in CC1 while has non-ideal backhaul in CC2), then a simultaneous TCI update can be enabled for sake of saving signaling overhead. |
| vivo | Issue 2.2: Q1: Yes. We think STRP/MTRP switching by the number of TRPs corresponding to the indicated TCI state(s) is needed. The TCI state pairs and a single TCI state which is not suitable for pairing should be switched dynamically. The TCI state pairs can be determined based on group-based beam reporting and the single TCI state can be determined by non-group-based beam reporting. If the answer is no, then any single TCI state should be one of the TCI state in a pair when at least one codepoint mapped with TCI states for 2 TRPs has been activated, which will degrade the performance by such a beam selection restriction. Issue 2.3: Q1: There can be the case where one CC is STRP and another CC is MTRP. Q2: From the perspective of UE complexity, such a mixture is not expected. |
| QC | For Q1: No, since the TCI activation/indication command will be applied to all CCs in the CC list, which can either be sTRP or sDCI mTRP mode, e.g. a TCI codepoint with 2 TCIs cannot be activated for a sTRP CC For Q2: No, since sDCI and mDCI have different TCI activation mechanism |
| FGI | Question 1: Yes. The purpose of updating TCI states for a CC list is to reduce the signaling overhead rather than placing the restrictions on number of TCI states for a CC. Furthermore, it seems that there is no such restriction in Rel-16 mTRP scheme, so we prefer to allow a mix of STRP CC and MTRP CC in a CC list. Question 2: Although different MAC CEs for s-DCI based and m-DCI based MTRP scheme are applied to the UE in Rel-16, we might have a different MAC CE for eUTCI in Rel-18. Thus, we prefer to allow a mix of SDCI based MTRP and MDCI based MTRP CC in a CC list, but we are open to both options. |
| ZTE | Issue 2.2: Q1: No Q2: Agree that, once MTRP operation is enabled through activating/indicating TCI state respective to different TRPs, TCI states for both TRPs should be applied. If totally switched to sTRP (rather than just being for a given PDSCH transmission of sTRP), the activated TCI codepoint should comeback to the legacy MAC-CE/schemes. Issue 2.3: No for both, if some companies can provide clear benefits/necessities of those combination cases. |
| Fujitsu | Issue 2.2: to Question 1, yes. We think the switching between STRP and MTRP depending on number of indicated TCI state in DCI could provide flexibility and have less spec impact by reusing Rel-16 scheme. |
| Panasonic | Issue 2.2, Q1 (No) & Q2 We think that the beam indication DCI should be used to update the indicated TCI states and not used for dynamic switching. But perhaps we should discuss how the update of the indicated TCI states should work and whether the indicated TCI states are sticky. But without discussing the latter, dynamic switching at the time of scheduling is better in our opinion, so we prefer to use the scheduling DCI for dynamic switching. We also prefer to discuss this issue together with Issue 3.8: For S-DCI based MTRP, the DCI field for informing which of the indicated joint/DL TCI states shall be applied to PDSCH reception since we can use the solution in issue 3.8 for dynamic switching |
| Mod | Proposal 2.4 is added for the further detail of TCI state update in S-DCI based MTRP |
| Sharp | Issue 2.2: Q1: No. Application information of the indicated TCI state(s) to the channel can inform to switch between mTRP and sTRP mode. Proposal 2.4: Support |
| IDC | Our views are updated in the table above. |
| Lenovo | Issue 2.2: Q1: No Q2: When two TCI states are indicated, if gNB wants to schedule a sTRP PDSCH transmission, gNB should firstly indicate another TCI codepoint mapped with single TCI state and it can only be applied after BAT. Unnecessary delay will be introduced in this mode. For proposal 2.4: Support |
| Huawei, Hisilicon | Issue 2.2: Question 1: No. We think that the network should have the flexibility to update the TCI state of only one TRP while UE keeps using the TCI state of the other TRP that has been indicated in a previous DCI. We don’t think it is a good idea that, in the mTRP operation regime, the TCI-indicating DCI has to always indicate a pair of joint TCI states corresponding to two TRPs even if the NW would like to update only one of the two TCI states. In our view, such a design has some issues as follows: - The sTRP/mTRP switching should be dynamically supported in the scheduling DCI and not in the TCI-indicating DCI. As an example, for UL, if SRS resource set indicator in the UL scheduling DCI for PUSCH is absent or indicate only one SRS resource set, sTRP operation is assumed. Otherwise, mTRP operation is assumed. - Such a design would be against the design principle in Rel-17. In Rel-17 uTCI framework, in the case of separate UL/DL TCI, NW can indicate only UL (DL) TCI in the TCI-indicating DCI while UE keeps using DL (UL) TCI from a previous TCI-indicating DCI. We don’t see any reason that while a memorizing mechanism is used for DL/UL TCI, a similar memorizing mechanism cannot be used for mTRP TCI indication. - Such a design unnecessarily increases the MAC-CE overhead and complicates its design: for the sake of simplicity, let us explain this by only focusing on the case that joint TCI is used for both TRPs and the current indicated TCIs are (TCI_1, TCI_2) where the first (second) entry of the pair corresponds to TRP1 (TRP2). To update one of the two TCI states (eg, TCI_1 to TCI_3 or TCI_2 to TCI_4), three pairs (TCI_1, TCI_2), (TCI_3, TCI_2), (TCI_1, TCI_4) have to be included in the MAC-CE which require 6 Octets. Question 2: This is already answered in Item 1 above: The sTRP/mTRP switching should be dynamically supported in the scheduling DCI and not in the TCI-indicating DCI. As an example, for UL, if SRS resource set indicator in the UL scheduling DCI for PUSCH is absent or indicates only one SRS resource set, sTRP operation is assumed. Otherwise, mTRP operation is assumed. In other words, UE maintains two TCI states associated with the two TRPs but, depending on the scheduling DCI, may use one or both of them. Issue 2.3: Question 1: Support. This can reduce the number of CC lists and hence reduce the number of TCI indication signaling. For example, in the case of the following figure, if sTRP cells and mTRP cells are configured in different cell list, gNB need to configured three CC lists {CC1}, {CC2, CC4}, {CC3, CC5}. The gNB need to configure/activate/indicate TCI states in three reference CCs. On the other hand, if the sTRP cells and mTRP cells are allowed to be configured in a CC list, two CC lists are enough, i.e., {CC1, CC2, CC4}, {CC1, CC3, CC5}. CC1 is configured in both CC lists. If CC1 is configured as the reference CC, once two TCI states are indicated in CC1, the two TCI states can be applied to the two CC list respectively. However, for instance, only the first TCI state applies to sTRP CC2 and CC4 in the first list and only the second TCI state applies to the sTRP CC3 and CC5 in the second list. This can largely reduce the signaling overhead. Question 2: We think this is also workable. Although TCI activation/indication signaling of sDCI MTRP CC or mDCI based MTRP CC is different, they can still share the same TCI state of the same TRP. Note that, only 4 CC lists can be configured according to current spec. If mixed configuration is not supported, the number of CC list may be not enough. Proposal 2.4: Support. |
| NEC | Issue 2.2. Q1: Yes, we think it is the method used for Rel-16 s-DCI based MTRP PDSCH, but we are open to have enhancements. Issue 2.3: Q1: NO. Q2: NO. Proposal 2.4: Support. |
| Xiaomi | Proposal 2.4: Support |
| QC | Suggest the following change. We do not support one codepoint to indicate only the 1st or 2nd TCI. Proposal 2.4: On unified TCI framework extension for S-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of a set of {the first joint TCI state, the second joint TCI state} or a single joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {the first DL TCI state, the first UL TCI state, the second DL TCI state, the second UL TCI state} any combination of up to 2 DL TCI states and/or up to 2 UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) |
| LG | Issue 2.2: Q1: No Q2: To our understanding, flexible application of STRP/MTRP schemes for each channel/signal can be possible based on the linkage between the TCI states and the corresponding channels/RSs with various configuration of M/N, not depending on the TCI state indication. Proposal 2.4: Fine with the proposal |
| vivo | We share same view as QC’s update on Proposal 2.4. Or we can trace back to the wording in previous meetings, like support following combinations - 2 joint TCI states for joint DL/UL TCI mode in the CC/BWP - 2 pairs of DL and UL TCI states for separate DL/UL TCI mode in the CC/BWP - 1 pair of DL and UL TCI states + 1 DL TCI state for separate DL/UL TCI mode in the CC/BWP - 1 pair of DL and UL TCI states + 1 UL TCI state for separate DL/UL TCI mode in the CC/BWP Note: 1 joint TCI state for joint DL/UL TCI mode, 1 pair of DL and UL TCI states, 1 DL TCI state or 1 UL TCI state for separate DL/UL TCI mode has been supported in Rel-17 unified TCI state framework. |
| Fraunhofer IIS/HHI | Issue 2.2: Q1 – yes. Issue 2.3: Q1 and Q2 – No. We don’t see a need to combine STRP and MTRP, or S-DCI-based and M-DCI-based CCs. Proposal 2.4: Fine with the revision from Qualcomm |
| Samsung | Issue 2.2: Q1: we view this operation, i.e., using the number of TCIs to indicate STRP/MTRP, is a restriction, and it seems that this restriction is not (always) needed. If one TCI is indicated, it can be a TCI update for one of the two TRPs. Q2: we suggest to discuss it along with dynamic switching for target channels Issue 2.3: Q1 & Q2: no Proposal 2.4: for the first bullet, we are unclear about the intention – if a single joint TCI state is mapped to a codepoint, how to tell if it is the 1st or 2nd? For the second bullet, as discussed before, we do not think all combinations are needed. If a union set is indicated, the UE can decide which combination(s) to apply. We are open to discuss it. |
| Google | Issue 2.3: Q1: Yes Q2: Yes Proposal 2.4: Either FL’s or QC’s proposal is fine to us. |
| Panasonic | - Proposal 2.4: We are okay to support but we feel this proposal does not bring anything new. We already had the following agreement earlier. I do not see what is new about proposal 2.4. We feel it is not needed and should not be discussed leaving room for other discussions. Agreement: On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS |

- Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal

Table 3-1 Summary for Issue 3

| # | Issue | Companies’ view and summary |
| --- | --- | --- |
| 3.1 | For S-DCI based MTRP, how to inform which of joint/DL TCI states indicated by DCI/MAC-CE shall be applied to PDSCH reception | Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. |
| 3.2 | For S-DCI based MTRP, whether Rel-17 rule is reused when provide the RRC configuration that informs the TCI selection for PDCCH reception | Question: Whether Rel-17 rule is reused when provide the RRC configuration that informs the TCI selection (the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states), i.e., - For a CORESET other than a CORESET with index 0 that is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET can be provided RRC configuration that informs that the UE shall apply the first one, the second one, or both of the indicated joint/DL TCI states to PDCCH reception on the CORESET - For a CORESET other than a CORESET with index 0 that is associated at least with CSS sets other than Type3-PDCCH CSS sets, the CORESET can be provided RRC configuration that informs that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET - For a CORESET with index 0, the CORESET can be provided RRC configuration that informs that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET FL note: Whether the Rel-17 RRC parameter followunifiedTCIstate in above RRC configurations may be left to RAN2 decision - Yes:Xiaomi, OPPO, vivo - No: FL note: Please share your answer to above question. |
| 3.3 | For S-DCI based MTRP, whether to support CORESET group configuration | Question: Whether to support CORESET group configuration for S-DCI based MTRP? - Support: Xiaomi, ZTE, Futurewei, Spreadtrum, LG, CMCC, Fraunhofer - Not support: OPPO, Fujitsu, Google, Panasonic, MTK, FGI, Lenovo, QC, Ericsson. Huawei/HiSilicon, CATT, Sharp, Docomo, Intel - Leave it to RAN2: Xiaomi, vivo FL note: Based on contributions, it seems still difficult to reach consensus on this issue. FL note: Please check Proposal 3.3, and share your view, if any. |
| 3.4 | For S-DCI based MTRP, PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG-PUSCH and Type2 CG-PUSCH), down-selection from the alternatives agreed in RAN1#110bis | Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Support (19): Apple, CATT, CMCC, Docomo, FGI, Fujitsu, Google, Hyundai, Intel, ITRI, Lenovo, MTK, OPPO, Qualcomm, Sharp, Spreadtrum, Transsion, vivo, Xiaomi Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - Support (11): Ericsson, Fraunhofer, Futurewei, Huawei/HiSilicon, LG, NEC, Nokia, Panasonic, Samsung, ZTE FL note: One observation from contributions is that if the TCI state(s) applied to the indicated SRS resource(s) and the TCI state(s) applied to PUSCH can be always aligned, there is no difference between Alt1 and Alt2. On the other hand, no use case if they are not aligned has been mentioned by any contribution. Thus, Proposal 3.4 is recommended for this issue as a potential compromise between the two alternatives. Please check and share your view, if any. Proposal 3.4.1 is also provided for the alternatives with more proponents if Proposal 3.4 cannot be accepted by this group. |
| 3.5 | For S-DCI based MTRP, PUCCH transmission, down-selection from the alternatives agreed in RAN1#110bis | Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design |
| 3.6 | For M-DCI based MTRP, channel(s)/signal(s) that has “implicit” association with a coresetPoolIndex value | DG-PUSCH and Type2 CG-PUSCH: The UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value - Support (16): Apple, CATT, CMCC, Docomo, Huawei/HiSilicon, Lenovo, MTK, Nokia, OPPO, Qualcomm, Sharp, Spreadtrum, vivo, Xiaomi, ZTE - Concern: FL note: Given the majority view. Proposal 3.6 is recommended for PUSCH transmission in M-DCI based MTRP. PUCCH with HARQ-ACK: The UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to HARQ-ACK transmission scheduled by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - Support: Huawei/HiSilicon, Nokia, OPPO, Panasonic, Qualcomm, Sharp, vivo, Xiaomi, LG, Fraunhofer IIS/HHI - Concern: Aperiodic CSI-RS: The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to aperiodic CSI-RS triggered by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - Support: CATT, Lenovo (if RRC-configured), Nokia (if RRC configured), OPPO, Qualcomm, vivo, LG, Sharp, Xiaomi - Concern: Aperiodic SRS: The UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to AP-SRS triggered by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - Support: CATT, Lenovo (if RRC-configured), Nokia, OPPO, Qualcomm, LG, Sharp, Xiaomi, vivo - Concern: FL note: Please share your view on above channels and signals. |
| 3.7 | For M-DCI based MTRP, the configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state | Question 1: Whether to reuse the same configuration/rule as in Rel-17 unified TCI framework? - Yes: Futurewei, Huawei/HiSilicon, Intel, QC, CATT, LG, Sharp, Docomo, NEC, Intel, Xiaomi, OPPO, vivo - No: Question 2: If the answer to Q1 is no, what is the configuration/rule that is used for configuring/allowing CORESET(s) to follow the indicated joint/DL TCI state? (Please share your answer on Q2 in Table 3-2) FL note: Please share your view on above two questions. |
| 3.8 | For S-DCI based MTRP, the DCI field for informing which of the indicated joint/DL TCI states shall be applied to PDSCH reception | Alt1: Introduce a new indicator field - Support: Xiaomi, Spreadtrum, vivo, QC, Sharp, Huawei, Lenovo Alt2: Reuse the existing TCI field - Support: OPPO FL note: Please share your view on above two alternatives |
| 3.9 | For S-DCI based MTRP, the default indicated joint/DL TCI state(s) for PDSCH reception when the DCI field is not present | Alt1: Use RRC to configure that the UE shall apply which one or both of the indicated joint/DL TCI states - Support: Sharp Alt2: The UE applies the first indicated joint/DL TCI state - Support: vivo, Xiaomi, Huawei Alt3: The UE applies both indicated joint/DL TCI state - Support: Alt4: The UE applies the same indicated joint/DL TCI state(s) configured to the CORESET used for PDCCH reception if the CORESET follows the indicated joint/DL TCI state(s) - Support: Xiaomi Alt.5: The UE continuously applies the applicable joint/DL TCI state(s) for PDSCH after receiving DCI without TCI field - Support: OPPO, QC, Fujitsu Alt 6: UE does not expect the DCI field is absent after the BAT when joint/DL/UL TCI states for both of the two TRPs are indicated in a CC/BWP or a set of CCs/BWPs in a CC list. How to ensure the restriction is FFS. - Support: vivo FL note: Please share your view on above three alternatives |

Proposal 3.3: On unified TCI framework extension for S-DCI based MTRP, support CORESET group configuration

Support: Xiaomi, Futurewei, ZTE, Spreadtrum, Samsung, IDC

Not support: QC, Ericsson, Panasonic, Google, CATT, Nokia, Lenovo, Sharp, Docomo, OPPO, Fujitsu, Huawei, FGI

Proposal 3.6: On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value

Table 3-2 Company inputs for Issue 3

| Company | Input |
| --- | --- |
| Mod | - Please provide your view on Proposal 3.4 and Proposal 3.4.1 - Please update your view on Issue 3.8 and Issue 3.9 in Table 3-1 |
| Xiaomi | As for proposal 3.4 and 3.4.1, we prefer proposal 3.4.1. For issue 3.8, we prefer to introduce a new field. Since the existing TCI field can’t be reused to indicate the association between PDSCH and indicated TCI states. It is necessary to separate the indication of the indicated TCI states and the association between PDSCH & indicated TCI states. For Issue 3.9, we prefer Alt 4, to apply the same indicated joint/DL TCI state as the CORESET used for the scheduling PDCCH transmission. |
| OPPO | Issue 3.2 We think Rel.17 rule on CORESET to follow unified TCI state should be reused. If there is no significant physically layer issues to be addressed in RAN1, it seems proper to respectively ask RAN2 handle the RRC parameters. Proposal 3.4.1 We are fine with this proposal. But we think P3.4 is the compromised solution which step bigger steps than P3.4.1. Issue 3.8 Support to reuse the existing TCI field. It may point to the codepoint of 1 or 2 DL/joint TCI state(s). Though the indicated TCI state(s) are not applicable yet before BAT, UE may interpret the number of indicated DL/joint TCI state(s) as an indication for which TCI state would be used. For example, when the number of indicated DL/joint TCI states is 2, then both applicable TCI states should be used by UE. Issue 3.9 Given the alternatives listed by FL, we understand that there was indicated DL/joint TCI state before the DCI without TCI field. There seems no need to set default TCI states. We suggest to continuously use the applicable DL/joint TCI state. If that’s the case, we suggest to add Alt.5 for consideration. Alt.5: The UE continuously applies the applicable DL/joint TCI state(s) for PDSCH after receiving DCI without TCI field |
| Spreadtrum | For proposal 3.4 and 3.4.1 Proposal 3.4.1 is preferred For issue 3.8 Support to introduce a new field to indicate the association between PDSCH and indicated TCI state(s). |
| vivo | Proposal 3.3 and Issue 3.3: We are not sure what the benefit of configure the CORESET group is. Can proponent clarity? Proposal 3.4: Do not support. Proposal 3.4.1: Support. Proposal 3.6: Support Issue 3.6: Better to add “following unified TCI state” for AP-CSI-RS and AP-SRS. Issue 3.7 to 3.9: We have provided our preference in the table. Issue 3.9: We’d like to add another alternative: UE does not expect the DCI field is absent after at least indicate joint/DL/UL TCI states for both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list. This alternative is reasonable since using an RRC signaling to configure the DCI field is present or not would cause a lot of latency. And how to ensure the restriction is FFS. |
| QC | Support Proposal 3.4.1. Don’t see the need to follow TCI for SRI. It is up to NW implementation as in R17 For 3.8, a new field is a cleaner solution For 3.9, support Alt5. Use the latest indicated TCI(s) as default, which is same as R17 |
| FGI | We prefer proposal 3.4.1. For issue 3.8: Introduce of a new indicator field can associate each field to each TRP, so we support Alt1. For issue 3.9: Support Alt 2, 3, and 4. |
| ZTE | Issue 3.1: The key issue for the next is relevant to whether the UE need to buffer the data by using both indicated TCI state(s). In our views, the answer is Yes, and then we do not need to further discuss the case of being less than a threshold. Issue 3.2: We have strong concerns on combination of “none” and other types of ‘apply the first one, the second one, both’ in followunifiedTCIstate. In our views, we only need to consider using ‘followunifiedTCIstate’ for enabling following or not. Then, ‘the first one, the second one, both’ should be indicated separately. Proposal 3.3: Regarding ‘CORESET group’ as mentioned by vivo, we think that the motivation is to reuse the similar solution as CORESETPoolId for mDCI-mTRP. Then, we are open to leave this issue for the future meetings or RAN2. Proposal 3.4: Support. Then, we need to add the following FFS for only one SRS resource set case in S-DCI based MTRP: - FFS: the case that only one SRS resource sets for CB/NCB is configured. Proposal 3.4.1: Not support. Proposal 3.6: Support. |
| Fujitsu | Issue 3.2: yes. The legacy rule in Rel-17 is still OK. Proposal 3.3: not support. We do not see clear necessities to introduce CORESET group in S-DCI based MTRP. Proposal 3.4 and Proposal 3.4.1: Proposal 3.4.1 is preferred. But we are also open to Proposal 3.4 for progress. Proposal 3.6: support. Issue 3.7: for Q1, yes. Issue 3.9: we support Alt.5 which is also similar with Rel-17 unified TCI. |
| Panasonic | Proposal 3.4 and 3.4.1 Perhaps we can explicitly say On unified TCI framework extension for S-DCI based MTRP: - Use the SRS resource set indicator field in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Issue 3.6 and the proposal 3.6 regarding M-DCI based MTRP and the channel(s)/signal(s) that have “implicit” association with a coresetPoolIndex value, the two multi DCI mutliTRP schemes are - PDSCH NCJT - STxMP For STxMP, using a coresetPoolIndex for SRS is not supported yet. So we think the discussion should be for PDSCH NCJT only and therefore we do not see the need to discuss PUSCH, aperiodic SRS and aperiodic CSI-RS. Further clarification on this issue from proponents would be appreciated. Issue 3.9 The agreement we had mentions two default behaviors highlighted in yellow which are to be determined. Issue 3.9 only targets the case when DCI is not present. How about the case when the DCI is not present, and the reception time is less than a threshold? We think both default behavior should be jointly discussed. Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. |
| Mod | Proposal 3.2 and Proposal 3.7 is added for Issue 3.2 and Issue 3.7 |
| Sharp | Proposal 3.2: Support Proposal 3.3: Not support CORESET group. whether follow unified TCI state is determined per a CORESET. Proposal 3.4: Not support. The UL/Joint TCI state should be applied to PUSCH independently from SRS. Proposal 3.4.1: Support Proposal 3.6: Support Proposal 3.7: Support Issue 3.8: We prefer Alt 1. The existing TCI field can be used for both or either of DL and UL channels/signals, not only PDSCH, to indicate TCI state(s). The new indicator field is used for PDSCH. These should be separated. Issue 3.9: We slightly prefer Alt 1. |
| Lenovo | Proposal 3.2: Support Proposal 3.4: Do not support Proposal 3.4.1: Support Proposal 3.6: Support Proposal 3.7: Support For issue 3.8, we support to introduce a new field for this purpose. |
| Huawei, Hisilicon | Issue 3.2: We think Rel-17 rule should be reused. But an easier formulation is that for CORESETs that adopt the indicated Joint/DL TCI state(s), i.e., CORESETs associated with USS sets or Type3-PDCCH CSS sets, or other CORESETs for which followUnifiedTCIState is enabled, a new RRC parameter in the CORESET configuration should be introduced to indicate that the CORESET shall adopt the first, the second, or both of the two indicated Joint/DL TCI states (the new RRC parameter does not indicate the value ‘none’). Further, we think above should apply to CORESETs with or without configured coresetPoolIndex. Issue 3.3: Not support. We don’t see the specific benefit of defining CORESET group. Issue 3.4: Neither of the proposals is our preference, but if we have to choose, we think Proposal 3.4.1 makes more sense since it does not restrict the SRS beam to the indicated beams and it has a similar behavior as in Rel-17. Issue 3.6: Proposal 3.6: Support. PUCCH with HARQ-ACK: Support Aperiodic CSI-RS: Not support. There are some cases in which one TRP can trigger CSI-RS for the other TRP or both TRP. For example, for MTRP BM, two CSI-RS resource sets can be triggered by one DCI. It is not reasonable to apply TCI state of the triggering TRP for both resource set. Similarly, for MTRP CSI measurement, two CSI-RS are configured in a pair for measurement. The two CSI-RS can be triggered by one DCI. It is not reasonable to apply TCI state of the triggering TRP for both CSI-RS. Aperiodic SRS: We would like to further study this. Issue 3.7: Question 1: Yes. We prefer to reuse the legacy mechanism to determine if a CORESET adopts the indicated joint/DL TCI state, that is: • for the CORESET associated with USS sets or Type3-PDCCH CSS sets, the indicated TCI state is applied to the CORESET. • for other CORESETs, the indicated TCI state is applied to the CORESET if the RRC parameter “followUnifiedTCIstate” is enabled in the CORESET configuration. • Note: Above applies to CORESETs with or without configured coresetPoolIndex. Issue 3.8: support Alt 1. Issue 3.9: support Alt 2. |
| NEC | Proposal 3.3: not support. Proposal 3.4: supportive. In addition, in FFS we think it is not the “application order” to be configured, instead, we suggest the following: FFS: for each CB/NCB SRS resource set, support RRC configurations on using the first one or the second one of the indicated joint/UL TCI states. Proposal 3.4.1: Not support, SRI in DCI can do the job. Proposal 3.6: OK. |
| Xiaomi | Proposal 3.2: we share same view as ZTE and HW that the new RRC will be configured for CORESET with enabled followunifiedTCIstate or follow unified TCI state directly, and it is unnecessary to indicate none of the two indicated joint/DL TCI states. Proposal 3.7: support Issue 3.9: for Alt 5, we think the proponents got a misunderstanding. It is about the default behavior if the DCI field for indicating which one or both of the indicated Joint/DL TCI states shall be applied to PDSCH reception is not present, not the TCI field. And for Alt 4, we add “if the CORESET follows the indicated Joint/DL TCI state(s)”. Else, Alt 2 will be used. |
| QC | For Proposal 3.2, support For Proposal 3.3, do not support For Proposal 3.4, do not support. We have concerns on both bullets. For Proposal 3.4.1, support For Proposal 3.6, support For Proposal 3.7, support |
| LG | Fine with the proposal 3.2 and 3.7. |
| vivo | Updated proposal 3.4.1: We’d like to add a sub-bullet. - SRS resource set indicator is reinterpreted. |
| Fraunhofer IIS/HHI | Issue 3.2: Support. The reuse of Rel. 17 rules for TCI state application is fine. Proposal 3.4 and 3.4.1: Prefer the compromise proposal 3.4 Proposal 3.6: Support Issue 3.7, Q1: Yes Issue 3.8: Prefer Alt. 1 Issue 3.9: We share a similar view on Alt. 5 as Xiaomi. We believe the discussion is about the absence of the DCI field in DCI format 1_1/1_2 that indicates which TCI-state(s) should be applied to PDSCH reception scheduled by the DCI format and not about the absence of the TCI field itself. And a clarification regarding Alt. 6. Since the presence of the DCI field itself is configurable via RRC, does Alt. 6 foresee a case where the DCI field is present in some cases and absent in some cases via RRC configuration (e.g., always present after BAT but can be absent before BAT)? |
| Samsung | Proposal 3.2: follow or not follow should be based on the higher layer parameter followUnifiedTCIState. For USS and type-3 CSS, ‘none’ should not be indicated/provided. Proposal 3.4: fine. Not sure if there is common ground/compromise can be made between 3.4 and 3.4.1 similar to PDSCH reception. Proposal 3.5: support Proposal 3.7: just to clarify. This proposal is to confirm the follow or not follow rule/configuration specified in Rel-17 for Rel-18 MDCI right? |
| Google | Issue 3.2: We share the same views as Huawei. Issue 3.8: Alt 1 Proposal 3.7: Support. But shouldn’t it be related to Issue 3.2? |
| Panasonic | - Proposal 3.4.1: Support We are more aligned with Proposal 3.4.1 now since we determined that the DCI field is actually the SRS resource indicator field, therefore this proposal ties the PUSCH transmission beam to the configured SRS resource sets through the SRI resource indicator, which is what proposal 3.4 was trying to achieve in our understanding. The only issue is that the SRS is not always configured to follow the uTCI framework, but this is a separate issue. - Proposal 3.6: Support with the following editorial change based the FL note Proposal 3.6: On unified TCI framework extension for PUSCH transmission in M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value In our understanding, the multi DCI multiTRP schemes are: - PDSCH NCJT - STxMP For STxMP, using a coresetPoolIndex for SRS is not supported yet. Finally, we feel these issues can be tackled together especially if we are about to introduce a new field similar to that of PUSCH. - Issue 2.2: Question 1: Whether the switching between STRP and MTRP can depend on the number of TCI states indicated in a beam indication DCI (e.g., the UE assumes STRP operation when beam indication DCI indicates TCI state(s) only for one TRP, the UE assumes MTRP operation when beam indication DCI indicates TCI states for both TRPs) - Issue 3.8: For S-DCI based MTRP, the DCI field for informing which of the indicated joint/DL TCI states shall be applied to PDSCH reception Alt1: Introduce a new indicator field Alt2: Reuse the existing TCI field |

- Issue 4 – UL power control for UL MTRP

Table 4-1 Summary for Issue 4

| # | Issue | Companies’ view and summary |
| --- | --- | --- |
| 4.1 | How to determine the UL PC parameter setting(s) and PL-RS(s) for S-DCI based PUSCH STxMP with SDM and SFN schemes | FL note: This issue can be discussed once RAN1 has conclusion on the TCI selection for PUSCH transmission (i.e., Issue 3.4 and Issue 3.6) |
| 4.2 | How to determine the UL PC parameter setting(s) if one or both indicated joint/UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH | Proposal 4.A: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applying to PUSCH/PUCCH transmission occasions in an UL BWP at least for S-DCI based PUSCH/PUCCH repetition with TDM is/are not associated with UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH, down-selection one alternative from the followings: - Alt1: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings configured in the corresponding UL BWP - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting - Alt2: No change from Rel-17 unified TCI framework Support/fine: Qualcomm, MTK, Futurewei, vivo, Nokia, Lenovo, ZTE, Apple, OPPO, Samsung, Xiaomi, Spreadtrum, Huawei, CMCC, Intel, Docomo, CATT, LG, CATT, Transsion, Sharp Not support: Ericsson FL note: This issue is not that critical at this stage, which can be discussed once RAN1 has conclusion on Issue 3.4 and Issue 4.1. |

- Issue 5 – PDSCH-CJT

Table 5-1 Summary for Issue 5

| # | Issue | Companies’ view and summary |
| --- | --- | --- |
| 5.1 | Whether PDSCH-CJT is considered as a S-DCI based MTRP operation | Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework |
| 5.2 | QCL type(s)/assumption(s) if two indicated joint TCI states are applied to PDSCH-CJT | Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Support: CATT, Spreadtrum, QC, Ericsson, vivo, LG, Intel - Concern: Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated TCI state - Support: Lenovo, Xiaomi, QC, Ericsson, vivo, ZTE - Concern: Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB - Support: ZTE, Xiaomi - Concern: FL note: Rel-18 CJT targets FR1 FL note: ‘QCL-TypeA’: {Doppler shift, Doppler spread, average delay, delay spread} FL note: ‘QCL-TypeB’: {Doppler shift, Doppler spread} FL note: Please update your view to above alternatives. FL note: Proposal 5.2 is recommended for this issue. |

Table 5-2 Company inputs for Issue 5

| Company | Input |
| --- | --- |
| QC | For issue 5.1, our answer is “Yes” For Proposal 5.2, both Alt1 and Alt2 can be supported, e.g. same as SFN PDSCH. We don’t support any delay pre-compensation without sufficient evaluation |
| Ericsson | Agree with Qualcomm |
| Xiaomi | Issue 5.1: Yes Proposal 5.2: support |
| vivo | Proposal 5.2: Support |
| Google | Issue 5.1: Yes Proposal 5.2: Support |
| ZTE | Issue 5.1: From transmission perspective, it may quite like SFN case. But, it should be highlighted that we still need consider how to further distinguish CJT transmission (e.g., two TCI state applies to the same layer/DMRS port), e.g., a new RRC parameter for enabling CJT. Issue 5.2: Support Alt2 and Alt3. Then regarding Alt3, please reviews several contributions in CJT where many companies suggest to consider the Delay compensation for CJT transmission, by gNB side. Clearly if pre-compensated, average delay/delay spread related parameters can be ignored. |
| Spreadtrum | For issue 5.1 PDSCH-CJT can be considered as S-DCI based MTRP operation. Since for M-DCI based MTRP, non-ideal backhaul is assumed, CJT transmission may not be achieved. For issue 5.2 In terms of TCI indication, the indication mechanism for CJT-PDSCH can keep the same as PDSCH-SFN in Rel-17. However, the pre-compensate scheme applied to “SFNSchemeB” may be not applicable for CJT, since the assumption that opposite Doppler shift exists between two TRPs is not valid. Since it is has not been concluded that the {Doppler shift, Doppler spread} or {delay shift, delay spread} can be pre-compensated perfectly by gNB in CJT. Alt 1 is preferred. |
| CATT | Proposal 5.1: Support Proposal 5.2: Support |
| Nokia | Issue 5.1: PDSCH-CJT seems to match with S-DCI based MTRP operation, but RAN1 shall first see how this differentiates from the mTRP PDSCH schemes (SDM, SFN, FDM, TDM) defined in Rel-16/17 before defining a new scheme. [Mod] This will be further discussed if it is agreed as a S-DCI based Tx scheme. Issue 5.2: Alt.1 Ok with proposal 5.2 |
| Lenovo | Proposal 5.1: Support Proposal 5.2: Support |
| LG | Issue 5.1: Yes Proposal 5.2: Support and prefer Alt1. |
| Docomo | Issue 5.1: Yes Proposal 5.2: support |
| OPPO | Issue 5.1: Yes. Proposal 5.2: Open to discuss these alternatives, but we are reluctant to see any newly introduced transmission schemes for MTRP PDSCH. |
| NEC | Issue 5.1: Yes Proposal 5.2: only alt 1 can work to our understanding, otherwise there will be no coherent transmission. |
| Huawei, HiSilicon | Proposal 5.1: Not required. PDSCH-CJT is a sDCI scheme and, in general, should follow agreements made in Rel-18 uTCI framework unless otherwise specified. However, depending on the considered issue, CJT-PDSCH may need a different treatment than SFN PDSCH or NCJT-PDSCH in some cases. Note that RAN1 already has an agreement regarding CJT-PDSCH which is different from other sDCI-based PDSCH schemes (Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability). Also, Proposal 5.2 is specific to CJT-PDSCH and not any other sDCI-based scheme. So, we don’t see a point to agree with the proposal. [Mod] One question, w/o this confirmation, which TCI selection scheme should be used for PDSCH-CJT? Is it based on coresetPoolIndex value or the DCI field we discussed in Issue 3.1? Proposal 5.2: Support Our preference is Alt. 1. Alt.2 assumes that the doppler shift is pre-compensated for at the gNB side and Alt. 3 assumes that average delay is pre-compensated for at the gNB side. In our view, neither of these assumptions are necessary for the CJT operation. Note that, a similar issue has been discussed in Rel-17 which led to supporting ‘sfnSchemeA’ or ‘sfnSchemeB’ where only in ‘sfnSchemeB’ it is assumed that the Doppler shift is pre-compensated for at the gNB side. |
| Apple | Proposal 5.1: Support in general. Proposal 5.2: Our understanding that this agreement is obviously applied for UE supporting 2 indicated joint TCI states for PDSCH-CJT and we support it. To make it clear, the following modification is needed: Proposal 5.2: On unified TCI framework extension for PDSCH-CJT for UE supporting 2 indicated joint TCI states, down-select at least one of the following alternatives:. |
| ZTE2 | Proposal 5.1: Support. In our views, CJT-related TCI state enhancement is only relevant to PDSCH transmission in Rel-18. If something wrong, please correct it. Proposal 5.2: Support. Alt2 and Alt3 are supported. After reviewing above companies’ comments, it is quite confusing that why gNB pre-compensation is not needed. If not, why we need to support 1 TCI state as a baseline which is a clear case for gNB pre-compensation. In our views, the system can not work well if there is a clear misalignment in the delay/Doppler-domain. Alt2 and Alt3 are good trade-off solution(s) between gNB/UE complexity and system performance. |
| CMCC | Proposal 5.1: Support Proposal 5.2: Support |
| Transsion | Proposal 5.1: Support Proposal 5.2: Support |
| Mod | No revision to Proposal 5.2 |

- Issue 6 – Beam reporting and beam failure recovery

Table 6-1 Summary for Issue 6

| # | Issue | Companies’ view and summary |
| --- | --- | --- |
| 6.1 | Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP | For S-DCI based MTRP, if a BFD-RS set is not explicitly provided for TRP-specific BFR, BFD-RS for the BFD-RS set is determined from a corresponding indicated joint/DL TCI state - Support: CATT, Docomo, Ericsson, Huawei/HiSilicon, MTK, Qualcomm, vivo, LG,CMCC, Intel - Concern: FL note: Given the support of several companies, Proposal 6.1 is recommended for this issue. Please check and share your view, if any. |
| 6.2 | Enhancement to beam update after NW response to TRP-specific BFR request | Beam update for all channels/signals associated with s failed TRP after NW response to TRP-specific BFR request for the failed TRP - Support: CATT, Docomo, Qualcomm, vivo, ZTE, Google, LG, OPPO, Apple,CMCC - Concern: FL note: Please update your view to above enhancement to TRP-specific BFR |
| 6.3 | Enhancement to beam reporting for STxMP | Question: This issue should be discussed in which Ais? - AI 9.1.1.1 : QC, OPPO, CMCC, Docomo, ZTE, vivo, LG, CATT, Apple, Transsion - AI 9.1.4.1: Ericsson, NEC, Xiaomi, Huawei/HiSilicon, MTK, Google, Docomo FL note: Please update your view to above question. |

Table 6-2 Company inputs for Issue 6

| Company | Input |
| --- | --- |
| QC | For Proposal 6.1, support For issue 6.2, support. This is extension from R17 sTRP beam resetting For issue 6.3, prefer 9.1.1.1, which is focusing on beam related issues. But we are also flexible on whichever is faster for treatment |
| Ericsson | Proposal 1: OK in principle, but propose to clarify that we rely on the agreed RRC configuration that determines which TCI states are used for PDCCH reception Proposal 6.1: On unified TCI framework extension for S-DCI based MTRP, if the first BFD-RS set or the second BFD-RS set is not explicitly provided to a UE, the UE determines BFD-RS from the indicated joint/DL TCI state configured to apply for PDCCH reception. Proposal 6.2: Support in principle. But “TRP” is imprecise. Propose a reformulation based on indicated TCI state. Proposal 6.3: We prefer that this is discussed in 9.1.4.1, since the reporting should be designed to solve the use case discussed in 9.1.4.1. Also, from a workload perspective, 9.1.4.1 seems more appropriate. |
| Xiaomi | Proposal 6.1: In our understanding, there are four types of CORESET in the scenario of S-DCI MTRP: - Type 1 CORESET: apply the first indicated TCI state - Type 2 CORESET: apply the second indicated TCI state - Type 3 CORESET: apply the first and the second indicated TCI state - Type 4 CORESET: apply the third TCI state different from the first and the second indicated TCI state, i.e., not follow unified TCI state. And we think the proposal 6.1 is reasonable only for following special cases - Case 1: configured with Type 1 and Type 2 CORESET. - Case 2: configured with Type 1 and Type 3 CORESET. - Case 3: configured with Type 2 and Type 3 CORESET. - Case 4: configured with Type 1, Type 2 and Type 3 CORESET. For other cases, such as following cases, proposal 6.1 is not reasonable. Since for some of them, it is unnecessary to support TRP-specific BFD-RS set. And for others of them, BFD-RS set should also include the RS of the TCI state different from the unified TCI state. - Case 5: configured with Type 1 CORESET. - Case 6: configured with Type 2 CORESET. - Case 7: configured with Type 3 CORESET. - Case 8: configured with Type 4 CORESET. - Case 9: configured with Type 1 and Type 4 CORESET. - Case 10: configured with Type 2 and Type 4 CORESET. - Case 11: configured with Type 3 and Type 4 CORESET. - Case 12: configured with Type 1, Type 2 and Type 4 CORESET. - Case 13: configured with Type 1, Type 3 and Type 4 CORESET. - Case 14: configured with Type 2, Type 3 and Type 4 CORESET. Issue 6.3 We prefer this issue to be discussed in 9.1.4.1 for a faster treatment, and also a joint discussion with the mechanisms of STxMP would be more beneficial. |
| Panasonic | Proposal 6.1: We support. Proposal 6.2: We support as long as ‘failed TRP’ wording is changed to for example spatial direction associated with a TCI state. |
| Futurewei | Proposal 6.1: Support in principle. Issue 6.3: This issue should be discussed in AI 9.1.4.1. |
| Google | Proposal 6.1: Need more discussions. Xiaomi’s concern could be an issue. Issue 6.2 and 6.3: We have updated our views above. |
| ZTE | Proposal 6.1: First of all, we need to agree whether we need to support implicit manner for S-DCI base mTPR-BFR. Then, we can further discuss the detailed issue. - Regarding question raised from Xiaomi, we think that, like mDCI-mTRP, a CORESETPooolID, or a configurable ID, should be configured/associated with each of CORESET (even including CORESET #0) (which can work together with legacy parameter of following unified TCI or not). Then, we can discuss how to handle above issues (i.e, exactly similar solution as mDCI). Otherwise, if considering very complicated solutions, the necessity of supporting implicit manner for determining BFD-RS set become unclear. |
| CATT | Proposal 6.1: Needs further discussion. In principle, we agree with Ericsson’s revision. Issue 6.2: Support. Issue 6.3: 9.1.1.1. |
| Nokia/NSB | in 6.1: FFS for SFN-PDCCH case in 6.2: There is also a need to consider case where no new candidate cannot be indicated. Beam update for all channels/signals associated with s failed TRP after NW response to TRP-specific BFR request for the failed TRP FFS: if no new candidate can be indicated in the BFR-request |
| Lenovo | Proposal 6.1: Need further discussion. Issue 6.2: Fine Issue 6.3: AI9.1.4.2 |
| LG | Fine with the proposal and Issue 6.2 |
| Docomo | Proposal 6.1: Support. Issue 6.2: Support. Issue 6.3: Either 9.1.1.1 or 9.1.4.1 is fine for us |
| OPPO | Proposal 6.1: we tend to think the examples from Xiaomi make sense. If one CORESET doesn’t follow the indicated TCI state, implying a different Tx beam for this CORESET, then the RS in the indicated TCI state cannot be considered as BFD RS by UE. It seems more discussion on this issue should be conducted by the group. Issue 6.2: Update our preference in the table. |
| Huawei, HiSilicon | Proposal 6.1: Support in principle although to also cover the case that neither of BFD-RS sets is provided, we suggest the following modification: Proposal 6.1 (modified): On unified TCI framework extension for S-DCI based MTRP, if the first BFD-RS set and/or the second BFD-RS set is not explicitly provided to a UE, the UE determines BFD-RS from the first indicated joint/DL TCI state for the first BFD-RS set and/or determines BFD-RS from the second indicated joint/DL TCI state for the second BFD-RS set. Issue 6.2: We don’t think the new beam found by BFR procedure should be applied for all channel/RS. If the BFD-RS is the QCL RS of the indicated TCI-state, the new beam should only be used to replace the indicated TCI state. For channel/RS that does not adopt the indicated TCI state (e.g., common PDCCH), they do not need to apply the new beam directly. Note to our FL: We are just wondering if our FL can add a new section for other issues. For instance, CC group-based TCI indication/application is proposed by more than one companies but not listed here for discussion. [Mod] I will add this in Issue 2 in the next round summary |
| Apple | Proposal 6.1: Ok. Issue 6.2: Support Issue 6.3: Our view is that beam related issues are handled in 9.1.1.1, which has been followed in the second agenda 9.1.1.2 for the two Tas aspects. This was actually discussed in GTW before and concluded to split in this way. |
| Samsung | For implicit BFD RS configuration, we agree that only the CORESETs that follow the indicated unified TCI(s) need to be considered here. For per TRP BFR under unified TCI, we also support to have beam update for all channels/signals. |
| ZTE | For Proposal 6.1, we need to discuss how to achieve association firstly for mTRP-BFR enhancement, and only flag of ‘first/second/both TCI states’ may not be sufficient after reviewing good discussion from other companies. |
| Mod | Proposal 6.1 is revised according to some above comments. As pointed out by some companies, it would be more proper to focus on the CORESETs that follow the indicated TCI state(s), instead all possible cases. Otherwise, the benefit from implicit determination is unclear. |
| CMCC | Proposal 6.1: Support Issue 6.3: Either 9.1.1.1 or 9.1.4.1 is fine. It’s better to have a decision in this meeting. |
| Intel | Proposal 6.1: We think the original formulation was clearer in how to determine the two BFD-RS sets. |
| Transsion | Proposal 6.1: Support Issue 6.2: Support Issue 6.3: AI 9.1.1.1 |
| Mod | No revision to Proposal 6.1 |
| Xiaomi | Suggest the following update Proposal 6.1: On unified TCI framework extension for S-DCI based MTRP, if a BFD-RS set is not explicitly provided to a UE, the UE determines one BFD-RS for the BFD-RS set from an indicated joint/DL TCI state if the indicated joint/DL TCI state is configured to apply for PDCCH reception - FFS: Determine from which one of the indicated joint/DL TCI states - FFS: The case if CORESET(s) is configured to apply both indicated joint/DL TCI states for PDCCH-SFN - FFS: determine another one BFD-RS for the BFD-RS set from TCI states different from indicated joint/DL TCI states if UE supports up to 2 BFD-RSs for the BFD-RS set. [Mod] Original proposal doesn’t preclude the case that both BFD-RS sets are not provided, it targets to any of the two BFD-RS sets. |
| Google | Proposal 6.1: Generally OK. But the current wordings seem unclear. Suggest the following revision. In addition, regarding the second FFS, we should firstly decide whether TRP-specific BFR is supported for PDCCH-SFN in Rel-18. Proposal 6.1: On unified TCI framework extension for S-DCI based MTRP, for TRP-specific BFR, if a BFD-RS set is not explicitly provided to a UE, the UE determines one BFD-RS for the BFD-RS set from an indicated joint/DL TCI state if the indicated joint/DL TCI state is configured to apply for PDCCH reception - FFS: Determine from which one of the indicated joint/DL TCI states - FFS: The case if CORESET(s) is configured to apply both indicated joint/DL TCI states for PDCCH-SFN |

- Appendix: Agreements/conclusions before/in RAN1#111

RAN1#111

Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group Note: Detail of the RRC configuration is left to RAN2 design Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework

RAN1#110b-e

Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support simultaneous configuration of both joint and separate DL/UL TCI modes in a serving cell Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support separate RRC-configured TCI state list(s) for each of TRPs Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - FFS: PL-RS(s), and UL PC parameter setting(s) (including P0, alpha, and closed loop index) for the PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUCCH transmission: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group associated with the same CORESET group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Note: the association indicates whether the UE shall apply the first one, the second one, or both of the joint/UL TCI states indicated by DCI/MAC-CE to a PUCCH resource/group Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 Agreement On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111: - Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Alt2: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) only specific to one of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Note: According to the agreement in RAN1#109-e, support of one additional TCI field or a field associating the TCI field to the TRP(s) is not precluded Note: It has been agreed to use the existing TCI field for TCI state indication for S-DCI based MTRP in RAN1#109e Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS FFS: The behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP

RAN1#110

Agreement On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP - Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1 - Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17 Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings: - Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s) - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions - Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE Switching between multi-TRP and single TRP operation is not precluded Agreement On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, down-selection one alternative from the followings: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - Alt3: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/UL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI format 0_1/0_2 is received in a CORESET group, the indicated joint/UL TCI state(s) associated with the CORESET group is applied to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - FFS: Details of CORESET group(s) FFS: PUSCH transmission scheduled/activated by a DCI format 0_0 and Type-1 CG-PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission, down-selection at least one alternative from the followings: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Alt4: Use DCI to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

RAN1#109e

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Detail of mapping joint/DL/UL TCI state ID(s) to a TCI codepoint, e.g., possible combinations of joint, DL, and/or UL TCI state IDs that can be mapped to a TCI codepoint - FFS: Whether to increase the max number of MAC CE activated TCI codepoints, i.e., more than 8 codepoints - FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits - Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded Note: The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for M-DCI based MTRP, consider the following alternatives for TCI state update: - Alt1: Reuse the same TCI state update scheme for S-DCI based MTRP - Atl2: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate the joint/DL/UL TCI state(s) corresponding to the same CORESETPoolIndex value - Alt3: Use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) to indicate all joint/DL/UL TCI states corresponding to both CORESETPoolIndex values - Study the association between the indicated joint/DL/UL TCI state(s) and a CORESETPoolIndex value - Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value. - Study whether the indicated joint/DL/UL TCI state(s) applies to the channels/signals associated with the same CORESETPoolIndex value or different CORESETPoolIndex value is indicated by DCI Agreement On unified TCI framework extension for S-DCI based MTRP, consider at least the following alternatives to map/associate a joint/DL TCI state to PDCCH reception(s) - Atl1: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt2: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a search space set - Alt3: Use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt4: Use DCI to inform the mapping/association between an indicated joint/DL TCI state and a CORESET or a CORESET group - Alt5: Based on a fixed mapping/association rule, e.g., the first indicated joint/DL TCI state always applies to PDCCH receptions Consider above alternatives for PDCCH repetition, PDCCH-SFN, PDCCH w/o repetition/SFN, and potential support of dynamic switching between S-TRP and M-TRP for PDCCH. It is not precluded to adopt one single alternative or multiple alternatives to support these cases. Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH/PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH/PUCCH (including P0, alpha for PUSCH, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.

- References

| 1 | R1-2211217 | Unified TCI framework extension for multi-TRP | PANASONIC |
| --- | --- | --- | --- |
| 2 | R1-2211114 | Discussion on unified TCI framework extension for M-TRP operation | Hyundai Motor Company |
| 3 | R1-2211219 | Discussion on unified TCI framework extension for multi-TRP | Spreadtrum Communications |
| 4 | R1-2211167 | Discussion on unified TCI framework extension for multi-TRP operation | CATT |
| 5 | R1-2211382 | Unified TCI Framework for Multi-TRP | Intel Corporation |
| 6 | R1-2211290 | Discussion of unified TCI framework for multi-TRP | Lenovo |
| 7 | R1-2211334 | Unified TCI framework extension for multi-TRP | Xiaomi |
| 8 | R1-2211425 | Unified TCI framework extension for multi-TRP | OPPO |
| 9 | R1-2210990 | Discussion on unified TCI framework extension for multi-TRP | vivo |
| 10 | R1-2211070 | Views on unified TCI extension for MTRP | Fujitsu |
| 11 | R1-2211051 | Unified TCI framework extension for multi-TRP | Ericsson |
| 12 | R1-2210845 | Unified TCI framework extension for multi-TRP | FUTUREWEI |
| 13 | R1-2210911 | Discussion on unified TCI framework extension for multi-TRP | Huawei, HiSilicon |
| 14 | R1-2210925 | Further Discussion on Unified TCI Extension for MTRP | InterDigital, Inc. |
| 15 | R1-2210935 | Enhancements on unified TCI framework extension for multi-TRP | ZTE |
| 16 | R1-2212026 | Views on unified TCI extension focusing on m-TRP | Samsung |
| 17 | R1-2212167 | Unified TCI framework extension for multi-TRP | Nokia, Nokia Shanghai Bell |
| 18 | R1-2212211 | Discussion on unified TCI framework extension for multi-TRP | Google |
| 19 | R1-2211969 | Discussion on unified TCI framework extension for multi-TRP | NTT DOCOMO, INC. |
| 20 | R1-2212099 | Extension of unified TCI framework for mTRP | Qualcomm Incorporated |
| 21 | R1-2211797 | Extension of unified TCI framework for multi-TRP | Apple |
| 22 | R1-2211886 | Unified TCI framework extension for multi-TRP | Sharp |
| 23 | R1-2211880 | Discussion on unified TCI framework extension for multi-TRP | FGI |
| 24 | R1-2211859 | Unified TCI framework extension for multi-TRP/panel | LG Electronics |
| 25 | R1-2211664 | Discussion on unified TCI framework extension for multi-TRP | CMCC |
| 26 | R1-2211588 | Multi-TRP enhancements for the unified TCI framework | Fraunhofer IIS, Fraunhofer HHI |
| 27 | R1-2212352 | Discussion on unified TCI framework extension for multi-TRP | NEC |
| 28 | R1-2212236 | Unified TCI framework extension for multi-TRP | MediaTek Inc. |
| 29 | R1-2212333 | Discussion on unified TCI framework extension for multi-TRP | ITRI |
| 30 | R1-2212420 | Discussion on Unified TCI framework extension for multi-TRP | CEWiT |
| 31 | R1-2212376 | Discussion on unified TCI framework extension for multi-TRP operation | TCL Communication Ltd. |
| 32 | R1-2211512 | Discussion on unified TCI framework extension for multi-TRP | Transsion Holdings |