3GPP TSG RAN WG1 #112			R1-2300331

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.3.2

Source:	InterDigital, Inc.

Title:	Discussion on subband non-overlapping full duplex operation

Document for:	Discussion and Decision

# Introduction

In Rel-18, a study item on evolution of NR duplex operation was approved [1], where the objectives identified for the study item are as follows:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In RAN1#111 meeting [3], the following agreements and conclusions were made:

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured. Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

In this contribution, we discuss subband non-overlapping full duplex (SBFD) operations, based on the analysis on effects of intra-/inter-subband CLI with simulation results, and discuss issues on power management, UL/DL timing alignment, UE-to-UE CLI mitigation, resource allocations in SBFD, collision handling, and benefits of SBFD operating through multiple BWPs.

# Discussions

## Subband non-overlapping FD operations

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-17, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-18, enhancements to support full duplex (FD) at least for gNB have been proposed and endorsed as the study item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on subband non-overlapping FD-gNB and HD-UEs in a cell

For a single UE perspective, a UE in a cell illustrated in Figure 2 can be informed of such a mixed D/U regions across RBs per symbol/slot, where the granularity of a subband also needs to be discussed where the granularity should at least be a group of RBs, or BWP-level of the granularity may also be considered. For the case of BWP-level SBFD, a subband indication for the SBFD operations can be based on reusing an existing BWP indicator in a DCI. Or, alternatively, the BWP indication can be interpreted as muted RBs/BWP region in terms of DL reception or UL transmission to be rate-matched around the muted RBs, as a simplified SBFD operation. Both the RB-level and BWP-level granularity of subband has its own trade-off benefits and need to be considered in this study.

Observation 1. Mixed D/U regions informed to a UE per symbol/slot in a cell can be used for a subband-wise UL transmission or DL reception, which results in UL coverage/capacity enhancement while achieving parallel DL transmissions over non-overlapped RBs.

### Explicit indication of the time and freq. location for UL subbands in SBFD

In previous meetings, the baseline for SBFD operation in RRC_CONNECTED mode was agreed, that is the time and frequency locations of subbands for SBFD operation is known to SBFD-aware UEs. As for the uplink and downlink BWPs, it was agreed that SBFD scheme within a single configured UL and DL BWP pair with aligned center frequencies is considered as the baseline. It was also agreed that at least the explicit indication of frequency locations of UL subband and time locations within a period are indicated to the UEs, as baseline.

Since the symbol-level TDD allocations are already possible by using SFI, the time and frequency indication of SBFD subbands can be performed based on a combination of an enhanced SFI and one or more frequency resource allocations. As such, the UE can determine the time domain resources in slot-level or symbol-level for SBFD operation based on an enhanced SFI configuration.

The frequency resource allocations can be configured using below alternatives:

- Alt. 1. Indication of the first and the last PRBs corresponding to the SBFD subbands.

- Alt. 2. Indication of the starting PRB and the number of PRBs corresponding to the SBFD subbands.

- Alt. 3. Indication of the PRB resources corresponding to the SBFD subbands based on a bitmap indication.

Proposal 1. Study mechanisms on how to inform UE of mixed D/U regions per symbol/slot to enable subband non-overlapping FD (SBFD) operations.

### Implicit indication of DL subband freq. location for SBFD operation

In previous meetings, it was agreed that the frequency location of the UL subbands will be at least explicitly indicated. However, the indication of the other subband types and whether it should be indicated implicitly or explicitly was left as FFS.

In our opinion, the frequency locations of the other subband types (e.g., DL subbands) does not need to be indicated explicitly. Considering the dynamic size of RB gaps (e.g., guard-bands) between the UL and DL subbands, explicit indication of the DL subbands may result in some resources left as unused. Instead, one possible way would be the part of RB gaps can be explicitly indicated when needed.

Considering the use-cases where the RB gap between the UL and DL subbands can be selected and indicated dynamically (based on the CLI), the UEs can determine the DL subbands accordingly. As such, a UE that is configured with the UL subbands and the RB gap size, can determine the DL subbands based on the remaining subbands.

Observation 2. In cases with dynamic RB gap indication, the DL subbands can be determined based on the indicated UL subbands and the RB gap.

Proposal 2. Consider implicit determination on the subband frequency locations of the DL subbands based on the indicated UL subband at least, where a parameter on RB gaps can instead be indicated when needed.

### BWP switching for SBFD operation within a TDD carrier

Semi-static configuration for SBFD subband time and frequency locations was agreed to be considered as baseline. However, the semi-static configuration may result in suboptimal NW operations. For example, the configuration cannot dynamically change based on the UL/DL traffic that can result in reduced scheduling flexibility, waste of resources, increased latency, and increased scheduling complexity due to the fixed resource allocations.

In RAN1#110bis-e, the working assumption was agreed that for SBFD operation within a TDD carrier, the SBFD will be studied within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

Considering the semi-static configuration of SBFD subbands/BWPs, in case more than one SBFD BWP is configured, it may be possible to schedule the UE to operate in different BWPs, e.g., based on NW UL/DL traffic and scheduling requirements. For example, in case UL traffic is more than DL traffic, the NW may indicate a BWP pair in which the UE to operate in an associated SBFD configuration comprising a larger UL subband. Alternatively, in case the DL traffic is more than UL, the NW may select the UE to switch to operate in another SBFD configuration that is associated with a different BWP pair comprising larger DL subband(s) (i.e., smaller UL subband).

Observation 3. Using SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration could result in improving scheduling flexibility, optimal resource allocations, lower latency, and so forth.

Proposal 3. Study the aspects of BWP switching for SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration.

### Collision Handling

In RAN1 #110bis-e, it was agreed to study the cases where a UE detects a conflict for the UL and DL operation in the same SBFD symbol. It was also agreed to study the ways to handle the conflicts in case of happening.

In our opinion, the UEs can handle the conflict in the UL and DL in the same SBFD symbol based on configured prioritizations. For example, the priority could be based on the physical channel priority between the uplink and downlink.

For example, PDSCH could have a higher priority than PUSCH. As such, in case a UE is scheduled to receive PDSCH in a SBFD symbol and the UE is also granted for PUSCH transmission in the same SBFD symbol, the UE may determine that downlink has higher priority than uplink. In another example, the UE may prioritize dynamic grant PDSCH over dynamic grant PUSCH without UCI. Alternatively, the UE may give lower priority to a dynamic grant PDSCH than a dynamic grant PUSCH with UCI. Moreover, the UE may consider dynamic grant PDSCH with lower priority than an uplink control channel or signal transmission, such as PUCCH or SRS.

In general, the UE might be configured to consider dynamic grant with higher priority than configured grant. In another example, the UE could determine that the downlink reference signals such as SSB, CSI-RS, PRS, and TRS have higher priority that the transmission of uplink reference signals such as SRS.

Observation 4. In case the UE is configured with physical channel priorities, the UE could use them to handle collisions in SBFD symbols.

Proposal 4. Consider configuration of physical channel priorities for handling the collisions in SBFD symbols, including uplink, downlink, control channels, data channels, dynamic grant, configured grant, and so forth.

### SBFD operation in SSB symbols

In RAN1#111, the support for the SBFD operation in SSB symbols was discussed. Based on the discussions, it was clear that not supporting the SBFD operation in SSB symbols may degrade the performance and objectives of the SBFD operation.

Contiguous UL subband resources

Considering the UL repetition, the contiguous UL subband resources should be provided in time domain. However, in case the SBFD operation is not allowed on SSB symbols, this requires switching to DL-only symbols each time there is an SSB transmission. As such, switching between SBFD symbols and DL-only symbols will affect the resources for UL repetition in SBFD symbols and will degrade the coverage enhancement purposes.

Latency in UL transmission

In addition to coverage enhancement, one of the objectives in SBFD operation is to reduce latency in UL transmissions. However, in case the SSB symbols are not transmitted in SBFD resources, this will change the TDD model back to DDDSU each time there is an SSB burst transmission. On the other hand, considering SSB transmission in SBFD symbols may improve the latency even in initial access or cell reselection procedures due to availability of resources for sending PRACH preamble and random-access procedures.

Observation 5. In case the SBFD operation is not supported in SSB symbols, the SBFD objectives such as UL coverage enhancement and latency reduction may be affected and degraded.

Proposal 5. Support SBFD operation in SSB symbols.

Simultaneous UL transmission in SSB symbols

Another aspect discussed in RAN1 #111 regarding the SBFD operation in SSB symbols was whether simultaneous UL transmission could be avoided in SSB symbols. It was proposed during the meeting that even if SBFD is supported in SSB symbols, there should be no UL transmission scheduled for those symbols.

In our opinion, such limitation is not necessary, as this could be managed by the implementation at the gNB. For example, the gNB can schedule UL transmission in the same symbols as SSB. It is just that the gNB that is sending an SSB beam in an SSB symbol needs to make sure that the UL transmission that is scheduled in the same symbol is for the UEs with beam direction far enough from the SSB’s beam direction. In other words, gNB has to make sure that the gNB does not schedule UEs for UL transmission in an SSB symbol with close-by or correlated beam directions. So, the beam directions for the SSB beam and the scheduled UL UEs can be properly determined based on gNB implementation.

Observation 6. In case the beam direction corresponding to a UE with UL transmission scheduled in the same symbol as an SSB transmission is far enough from the beam direction of the respective SSB, the simultaneous UL transmission in SSB symbols can be possible.

Proposal 6. Support the simultaneous UL transmission in SSB symbols, where the spatial collision avoidance can be performed via gNB implementation.

Cell (re)selection for cells with SBFD operation

It was discussed in the previous meeting that the cell reselection may be affected due to reduced EPRE for the SSB symbols that overlap with SBFD symbols. The reduced EPRE can be compensated by using scaling rules and corresponding parameters. That is, the UE that prefers to connect to an SBFD neighbour cell can receive configuration, e.g., from the current serving cell, on the ratio by which the SSB EPRE is reduced in the corresponding SBFD cell.

For example, the UE could receive an indication that the EPRE of the SSBs received from a neighbour cell is reduced (e.g., 3dB) due to SBFD configuration. As such, the SBFD-capable UE can compensate the measured RSRP and RSRQ (for the cell ranking) by using a scaling value that corresponds to the (e.g., 3dB) EPRE reduction.

Observation 7. In cell (re)selection for SBFD cells with reduced EPRE due to SSB transmission in SBFD symbols, in case the UE knows the ratio by which the EPRE is reduced, the UE can use scaling parameters for compensating the reduced EPRE in evaluating the RSRP or RSRQ during cell ranking.

During the cell (re)selection, in case the SSBs are not transmitted in SBFD symbols for an SBFD cell, the UEs cannot have an estimation of the CLI that they could expect after switching to RRC-Connected Mode in the corresponding cell. However, if the SSBs are transmitted in SBFD symbols during the cell (re)selection, the UE could measure the CLI, for example based on configured ZP CSI-RS resources. The UE may then decide if the threshold is within acceptable range and whether the UE can camp on or connect to the corresponding cell. This will avoid latency and ping-ponging effects. Once a UE decides to transmit a RACH to a cell, how to transmit a PRACH preamble including which RACH resource to choose to camp on the cell operating SBFD needs further investigations, with respect to the latency reduction and UL coverage enhancement offered by the SBFD operation.

Proposal 7. Support enhancements on random-access procedures and cell ranking for cell (re)selection including cells operating SBFD, to avoid latency and ping-ponging effects.

## Analysis on effects of intra-/inter-subband CLI

In this subsection, we discuss effects of intra-subband and inter-subband CLI based on initial LLS results, where we conducted the LLS to see how much negative impacts on DL reception on a subband (e.g., RBs) can be observed, when intra-subband CLI on the same overlapped subband or inter-subband CLI on adjacent or non-overlapped subband exist due to nearby other UE’s uplink transmissions.

Simulation assumptions for the LLS are summarized in Appendix (Table 3). We considered a duplexing scheme where the DL signal can be a couple of dBs less than, greater than or equal to the UL (CLI) signal. For a given UL (CLI) RB allocation, we evaluated the DL receiver performance for various RB allocations over the system bandwidth considering several degrees of intra and inter subband CLI overlaps as shown in Figure 3. For the initial results presented, no signal impairments due to PA nonlinearity or other imperfections, etc. are considered, where such impairments may further increase the adjacent inter-subband CLI level.

Additionally, the performance evaluations considered some timing advance (up to half an OFDM symbol) being applied between the DL and UL, which represents a realistic environment when UE-to-UE CLI exists, as an uplink (CLI) signal transmission timing is controlled by a timing advance mechanism from its serving cell/TRP.

Figure 3. Throughput vs SNR: DL signal and CLI having same power level

Figure 4. Throughput vs SNR: CLI power 18dB below DL signal

Figures 3 and 4 show throughput performance for the DL signal affected by intra- and inter-subband CLI (UL) with different power levels relative to the DL signal. Figure 3 is for the case when CLI and DL signal power are identical, while Figure 4 is for the case where CLI power level is 18dB below the DL signal. The throughput performance curves are generated based on applying the half symbol timing advance on the CLI signal in both figures.

It can be seen from both Figure 3 and Figure 4 that, for a given MCS index, i.e., MCS-27, the DL throughput performance suffers considerably from the effects of CLI and approaches almost zero when there is any amount of intra-subband CLI being overlapped with the DL signal. Moreover, the ICI as a result of the inter-subband CLI has a severe impact on the throughput when the inter-subband CLI is adjacent to the DL signal with 0-RB gap in between. Throughput performance improves significantly to 90% of the maximum throughput when the adjacent subband gap between the DL signal and the inter-subband CLI is at least 2RBs in this example scenario.

Figure 5. Normalized Throughput vs SNR, for various PRB allocations of DL signal (victim) 
and 25PRB allocation for CLI (UL)

Figure 5 shows the normalized throughput performance curves for the DL signal relative to the number of PRBs allocated to it. For the various number (2, 5, 10, 25, and 50) of PRB allocations to the DL (victim) signal, the CLI (UL) signal is allocated 25PRBs and it is assumed to be 18dB below the DL signal. The normalized throughput performance is shown for the case when there is no frequency gap (0PRB gap) and for a frequency gap of 1 PRB, between the DL signal and the CLI. It can be observed that as the frequency allocation (number of PRBs) of the DL signal relative to the frequency allocation of the CLI decreases, the performance of the DL signal degrades significantly. It can also be seen that the relative degradation with lower number of frequency (PRB) allocation is considerable when there is 0PRB frequency gap between DL signal and the CLI (UL) signal compared to a frequency gap of 1PRB.

Observation 8. DL throughput performance suffers considerably at high MCSs and approaches to almost zero when there is any degree of intra-subband CLI overlap with DL signal.

Observation 9. Inter-carrier interference, resulting from time advance on inter-subband CLI may impact DL throughput performance significantly, especially at high MCS indices, when there is no adjacent inter-subband distance between the DL signal and the CLI, i.e., 0-RB gap.

Observation 10. At high MCS indices, DL throughput performance recovers to 90% of the maximum throughput when the inter-subband distance between the DL signal and the CLI signal is at least 2-RB in this example scenario.

Observation 11. In addition to the frequency gap between the DL signal and the CLI (UL) signal, the amount of frequency resources (PRBs) allocated to the DL signal relative to that for the CLI signal also impacts the normalized throughput performance significantly.

Proposal 8. Study performance of applying a frequency gap or guard RBs for a UL transmission in an SBFD framework for interference mitigation with regards to adjacent DL subbands.

## Performance comparison between SBFD and legacy TDD

In this subsection we compare the performance of SBFD with legacy TDD via system level simulations. We consider the SBFD deployment case 1, where a single carrier is considered with all cells using the same SBFD subband configuration. We consider the FR1 - Urban macro deployment. We applied Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD) from the agreement made in RAN1#109-e [2], which includes the following:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Alt. 2 : Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

  - SBFD: Alt. 4: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

Legacy TDD uses a shared Tx/Rx antenna array for downlink and uplink transmission/receptions, whereas SBFD utilizes separate Tx/Rx antenna array for simultaneous downlink and uplink transmission. The total number of TxRUs is the same for both legacy TDD and SBFD, however in the SBFD case, each of the downlink and uplink uses half of the total number of TxRUs.

Regarding antenna elements, following options are considered for this evaluation:.

- SBFD Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- SBFD Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

Additionally, the effect of ratio of self-interference (RSI) at gNB receiver is considered. The effect of three different RSI values: 135 dB, 145 dB, 165 dB on UL performance are evaluated.

We consider two traffic ratios: (i) DL:UL ratio = 1:1, and (ii) DL:UL ratio = 2:1. Additionally, for both of these we consider varying DL:UL traffic arrival rates/loads. Tables 1-3 presents results for the DL:UL ratio of 1:1, whereas Table 4-6 presents results for the 2:1 ratio. Additional details regarding simulation assumptions can be found in the Appendix (Table 8).

Table 1. legacy TDD vs. SBFD for 1:1 (DL:UL) ratio – low load

| Reported parameters |  | DL/UL ratio 1/1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Legacy TDD | SBFD Opt.1 |  |  |  |  |  | SBFD Opt. 2 |  |  |  |  |  |
|  |  |  | Alt. 2 |  |  | Alt. 4 |  |  | Alt. 2 |  |  | Alt. 4 |  |  |
|  |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  |
|  |  |  | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 |
| DL UPT [Mbps] | Mean | 497.07 | 410.71 | 411.32 | 410.32 | 510.61 | 511.99 | 511.61 | 424.64 | 424.64 | 425.17 | 530.12 | 529.41 | 532.09 |
| UL UPT [Mbps] | Mean | 54.75 | 52.51 | 53.38 | 53.61 | 40.70 | 40.96 | 41.15 | 62.67 | 63.52 | 63.92 | 45.86 | 45.69 | 45.88 |
| DL Latency [s] | Mean | 0.012 | 0.015 | 0.015 | 0.015 | 0.012 | 0.012 | 0.012 | 0.014 | 0.014 | 0.014 | 0.011 | 0.011 | 0.011 |
| UL Latency [s] | Mean | 0.502 | 0.531 | 0.519 | 0.512 | 0.534 | 0.531 | 0.531 | 0.438 | 0.429 | 0.425 | 0.524 | 0.519 | 0.514 |
| RU (%) |  | 15.8 | 24.1 | 23.7 | 23.9 | 16.9 | 16.8 | 16.8 | 22.3 | 22.2 | 22.2 | 16.1 | 16.0 | 16.0 |
| λd/λu |  | 0.24/0.24 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Additional comments: DL:UL ratio 1:1 Legacy TDD Slot config: {DDDSU}, with S = [12D, 2G, 0U] SBFD slot config: Two options considered Alt. 2: {XXXXU}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Alt. 4: {XXXXX}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) SBFD Antenna configuration: Two options considered Option 1: (same # of antenna elements as TDD) Option 2: (# of antenna elements for SBFD is two times that of TDD) |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2. legacy TDD vs. SBFD for 1:1 (DL:UL) ratio – medium load

| Reported parameters |  | DL/UL ratio 1/1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Legacy TDD | SBFD Opt.1 |  |  |  |  |  | SBFD Opt. 2 |  |  |  |  |  |
|  |  |  | Alt. 2 |  |  | Alt. 4 |  |  | Alt. 2 |  |  | Alt. 4 |  |  |
|  |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  |
|  |  |  | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 |
| DL UPT [Mbps] | Mean | 319.41 | 283.38 | 285.0 | 284.76 | 379.21 | 380.53 | 380.47 | 302.33 | 302.02 | 302.12 | 401.92 | 401.62 | 401.88 |
| UL UPT [Mbps] | Mean | 52.25 | 47.37 | 53.29 | 55.29 | 30.73 | 37.80 | 39.11 | 54.89 | 59.17 | 62.10 | 37.47 | 43.31 | 44.19 |
| DL Latency [s] | Mean | 0.023 | 0.025 | 0.025 | 0.025 | 0.018 | 0.018 | 0.018 | 0.023 | 0.023 | 0.023 | 0.017 | 0.017 | 0.017 |
| UL Latency [s] | Mean | 0.764 | 0.773 | 0.736 | 0.719 | 0.853 | 0.781 | 0.744 | 0.745 | 0.671 | 0.612 | 0.772 | 0.745 | 0.738 |
| RU (%) |  | 39.7 | 52.0 | 51.6 | 51.6 | 35.6 | 35.5 | 35.6 | 50.4 | 50.2 | 49.8 | 34.5 | 34.5 | 34.5 |
| λd/λu |  | 1.43/1.43 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Additional comments: DL:UL ratio 1:1 Legacy TDD Slot config: {DDDSU}, with S = [12D, 2G, 0U] SBFD slot config: Two options considered Alt. 2: {XXXXU}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Alt. 4: {XXXXX}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) SBFD Antenna configuration: Two options considered Option 1: (same # of antenna elements as TDD) Option 2: (# of antenna elements for SBFD is two times that of TDD) |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3. legacy TDD vs. SBFD for 1:1 (DL:UL) ratio – high load

| Reported parameters |  | DL/UL ratio 1/1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Legacy TDD | SBFD Opt.1 |  |  |  |  |  | SBFD Opt. 2 |  |  |  |  |  |
|  |  |  | Alt. 2 |  |  | Alt. 4 |  |  | Alt. 2 |  |  | Alt. 4 |  |  |
|  |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  |
|  |  |  | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 |
| DL UPT [Mbps] | Mean | 288.10 | 223.06 | 223.69 | 223.93 | 310.97 | 313.54 | 310.541 | 239.81 | 239.59 | 239.92 | 332.9 | 331.33 | 332.88 |
| UL UPT [Mbps] | Mean | 43.37 | 29.79 | 43.58 | 46.27 | 15.10 | 23.18 | 26.33 | 40.74 | 56.37 | 58.92 | 19.12 | 30.11 | 31.85 |
| DL Latency [s] | Mean | 0.027 | 0.040 | 0.041 | 0.041 | 0.025 | 0.024 | 0.025 | 0.036 | 0.036 | 0.036 | 0.023 | 0.023 | 0.023 |
| UL Latency [s] | Mean | 0.970 | 1.084 | 0.944 | 0.899 | 1.516 | 1.213 | 1.144 | 0.901 | 0.758 | 0.746 | 1.298 | 1.005 | 0.930 |
| RU (%) |  | 52 | 67.8 | 67.8 | 67.8 | 50.2 | 49.7 | 50.2 | 65.8 | 65.7 | 65.6 | 48.1 | 48.3 | 48.2 |
| λd/λu |  | 2.4/2.4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Additional comments: DL:UL ratio 1:1 Legacy TDD Slot config: {DDDSU}, with S = [12D, 2G, 0U] SBFD slot config: Two options considered Alt. 2: {XXXXU}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Alt. 4: {XXXXX}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) SBFD Antenna configuration: Two options considered Option 1: (same # of antenna elements as TDD) Option 2: (# of antenna elements for SBFD is two times that of TDD) |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4. legacy TDD vs. SBFD for 2:1 (DL:UL) ratio – low load

| Reported parameters |  | DL/UL ratio 2/1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Legacy TDD | SBFD Opt.1 |  |  |  |  |  | SBFD Opt. 2 |  |  |  |  |  |
|  |  |  | Alt. 2 |  |  | Alt. 4 |  |  | Alt. 2 |  |  | Alt. 4 |  |  |
|  |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  |
|  |  |  | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 |
| DL UPT [Mbps] | Mean | 466.69 | 381.31 | 382.89 | 382.59 | 485.04 | 485.49 | 485.44 | 402.05 | 399.73 | 399.77 | 505.57 | 506.44 | 505.99 |
| UL UPT [Mbps] | Mean | 54.61 | 51.44 | 52.24 | 53.51 | 39.77 | 40.60 | 40.93 | 61.59 | 63.13 | 63.55 | 44.80 | 45.69 | 45.73 |
| DL Latency [s] | Mean | 0.013 | 0.016 | 0.016 | 0.016 | 0.013 | 0.013 | 0.013 | 0.015 | 0.015 | 0.015 | 0.012 | 0.012 | 0.012 |
| UL Latency [s] | Mean | 0.495 | 0.541 | 0.529 | 0.513 | 0.563 | 0.538 | 0.533 | 0.466 | 0.431 | 0.424 | 0.527 | 0.526 | 0.516 |
| RU |  | 18.4% | 26.9% | 23.7 | 26.4 | 19.5% | 19.4 | 19.4 | 25.0% | 24.7 | 24.6 | 18.4% | 18.4 | 18.4 |
| λd/λu |  | 0.48/0.24 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Additional comments: DL:UL ratio 2:1 Legacy TDD Slot config: {DDDSU}, with S = [12D, 2G, 0U] SBFD slot config: Two options considered Alt. 2: {XXXXU}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Alt. 4: {XXXXX}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) SBFD Antenna configuration: Two options considered Option 1: (same # of antenna elements as TDD) Option 2: (# of antenna elements for SBFD is two times that of TDD) |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5. legacy TDD vs. SBFD for 2:1 (DL:UL) ratio – medium load

| Reported parameters |  | DL/UL ratio 2/1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Legacy TDD | SBFD Opt.1 |  |  |  |  |  | SBFD Opt. 2 |  |  |  |  |  |
|  |  |  | Alt. 2 |  |  | Alt. 4 |  |  | Alt. 2 |  |  | Alt. 4 |  |  |
|  |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  |
|  |  |  | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 |
| DL UPT [Mbps] | Mean | 359.10 | 284.18 | 284.75 | 284.71 | 381.31 | 377.56 | 380.84 | 302.38 | 303.49 | 303.29 | 400.02 | 401.63 | 401.07 |
| UL UPT [Mbps] | Mean | 56.39 | 49.77 | 55.24 | 56.35 | 38.53 | 42.94 | 44.69 | 58.01 | 63.37 | 65.61 | 42.89 | 46.69 | 48.03 |
| DL Latency [s] | Mean | 0.019 | 0.025 | 0.025 | 0.025 | 0.018 | 0.018 | 0.018 | 0.023 | 0.023 | 0.023 | 0.017 | 0.017 | 0.017 |
| UL Latency [s] | Mean | 0.615 | 0.633 | 0.549 | 0.529 | 0.685 | 0.585 | 0.565 | 0.483 | 0.474 | 0.452 | 0.636 | 0.590 | 0.578 |
| RU |  | 35.8 | 49.6 | 49.1 | 48.9 | 35.0 | 35.0 | 34.8 | 47.5 | 46.7 | 46.6 | 34.2 | 34.0 | 33.8 |
| λd/λu |  | 1.43/0.71 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Additional comments: DL:UL ratio 2:1 Legacy TDD Slot config: {DDDSU}, with S = [12D, 2G, 0U] SBFD slot config: Two options considered Alt. 2: {XXXXU}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Alt. 4: {XXXXX}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) SBFD Antenna configuration: Two options considered Option 1: (same # of antenna elements as TDD) Option 2: (# of antenna elements for SBFD is two times that of TDD) |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6. legacy TDD vs. SBFD for 2:1 (DL:UL) ratio – high load

| Reported parameters |  | DL/UL ratio 2/1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Legacy TDD | SBFD Opt.1 |  |  |  |  |  | SBFD Opt. 2 |  |  |  |  |  |
|  |  |  | Alt. 2 |  |  | Alt. 4 |  |  | Alt. 2 |  |  | Alt. 4 |  |  |
|  |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  | RSI (dB) |  |  |
|  |  |  | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 | 135 | 145 | 165 |
| DL UPT [Mbps] | Mean | 260.56 | 198.55 | 198.77 | 199.06 | 280.61 | 280.28 | 280.16 | 214.17 | 213.66 | 215.052 | 303.82 | 305.91 | 304.72 |
| UL UPT [Mbps] | Mean | 52.12 | 37.16 | 51.27 | 55.15 | 23.93 | 35.17 | 39.544 | 45.38 | 57.00 | 61.50 | 29.32 | 41.332 | 43.86 |
| DL Latency [s] | Mean | 0.034 | 0.069 | 0.069 | 0.069 | 0.032 | 0.032 | 0.032 | 0.052 | 0.052 | 0.051 | 0.028 | 0.028 | 0.028 |
| UL Latency [s] | Mean | 0.763 | 0.844 | 0.755 | 0.709 | 1.012 | 0.800 | 0.733 | 0.835 | 0.699 | 0.627 | 0.944 | 0.757 | 0.735 |
| RU |  | 60.3 | 75.8 | 75.4 | 75.2 | 58.7 | 58.8 | 58.8 | 73.6 | 73.1 | 73.1 | 56.4 | 56.4 | 56.3 |
| λd/λu |  | 2.86/1.43 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Additional comments: DL:UL ratio 2:1 Legacy TDD Slot config: {DDDSU}, with S = [12D, 2G, 0U] SBFD slot config: Two options considered Alt. 2: {XXXXU}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Alt. 4: {XXXXX}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split, and U is uplink only slot Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) SBFD Antenna configuration: Two options considered Option 1: (same # of antenna elements as TDD) Option 2: (# of antenna elements for SBFD is two times that of TDD) |  |  |  |  |  |  |  |  |  |  |  |  |

We observe that Alt. 4 yields better downlink performance than Alt. 2, due to the fact that there is additional DL slot (and hence RBs) available when compared to Alt. 2 ({XXXXX} vs. {XXXXU}). Additionally, we observe a drop in downlink UPT for SBFD Alt. 2 when compared with legacy TDD, since each of the SBFD (X) slots only provide 80% of the bandwidth for downlink transmissions, whereas TDD offers four (full bandwidth) slots out of every five for downlink transmissions. For this reason, we also observe that UL performance for Alt. 2 is better than that of Alt. 4, also better than legacy TDD (for RSI greater than 145 dB under SBFD Opt. 1), and better than legacy TDD irrespective of RSI value for SBFD Opt. 2.

Another observation is that SBFD Option 2 provides better performance than SBFD Option 1. This directly results from the beamforming gain Option 2 offers (two elements per TxRU, compared to a 1-1 mapping between antenna element and TxRU for Option 1).

With regards to the impact of RSI, we observe that as RSI increases from 135 dB to 165 dB, UL performance improves. This results from the fact that total residual interference on the UL subband decreases. Since RSI has no impact on DL, DL performance is unaffected.

An important observation here is that, as load increases, low RSI at gNB receiver has a significant impact on uplink performance. The degradation gets worse when DL/UL ratio is 2/1 vs. 1/1, due to the fact that there is a larger proportion of DL traffic in the system.

Observation 12. Restricting DL subband transmissions on slots that correspond to UL slots in legacy TDD can improve uplink performance but negatively impacts downlink performance.

Observation 13. The static/fixed subband partitioning, e.g., [DUD] = [40 20 40] RB split all the time, results in worse performance for SBFD compared with legacy TDD in downlink. Thus, flexible/versatile subband partitioning and its dynamic indication mechanisms should be considered to cope with varying traffic/channel conditions.

Observation 14. Utilizing SBFD option 2 (total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD) improves SBFD performance.

Proposal 9. Analysis on various downlink performance degradation aspects due to the SBFD operations compared with legacy TDD systems should also be an important part of the NR-Duplex study.

Proposal 10. To overcome the degraded downlink performance due to the static/fixed subband partitioning, flexible/versatile subband partitioning and its dynamic indication mechanisms should be further discussed to cope with varying traffic/channel conditions.

## On CLI Mitigation aspects

A granularity of a subband in the SBFD scenario can be a group of RBs. Since the gNB can flexibly schedule an UL transmission over a set of RBs on a subband for a UE, this may be adjacent to another subband being allocated for DL in SBFD. As observed by Figures 3, 4, 5, and 6 as the LLS results, the UL transmission may cause a severe UE-to-UE CLI leakage on the adjacent DL subband, which depends on the frequency gap between the UL RBs and the DL RBs.

In line with these aspects as well, in RAN1#110 it was agreed to study the potential solutions of UE-to-UE CLI measurement and reporting for SBFD scheme.

### Dynamic Power Management

One way of handling this CLI issue is to consider dynamic UL power control depending on some factors such as the frequency gap, beam/spatial-domain parameter, a priority indication on the UL, and so on. Dynamic link adaptation mechanism can also be considered similarly. For example, MCS adjustment depending on the factors can be beneficial to mitigate the effects of the CLI dynamically.

Observation 15. In the SBFD scenario, an UL transmission over a subband causes significant UE-to-UE CLI leakage on the adjacent DL subband depending on a frequency gap between the UL RBs and DL RBs of each subband.

Proposal 11. Study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically.

On the other hand, gNB may apply a downlink power backoff on some SBFD slots or symbols to deal with self-interference caused by the FD operation as a part of gNB implementation. This can have some impacts to UE behaviours unless the amount of power backoff is negligible on a given slot.

Since such a power management behaviour can be in a dynamic manner in the SBFD scenario, it is beneficial to consider informing UE of the dynamic power adjustment and related operations. For example, UE can apply the information to a CSI calculation by adjusting a channel power-level for compensation to improve CSI accuracy and depending on the information the UE may be better skipping some measurements on the SBFD slots/symbols. Details on when and how to trigger these behaviours are to be further studied to deal with the UE-to-UE CLI.

Observation 16. As a part of gNB implementation, the gNB may apply a downlink power backoff on some SBFD slots or symbols to deal with self-interference caused by the FD operation, which can impact to UE behaviours depending on the amount of power backoff.

Proposal 12. Consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario.

### Subband-edge-specific CLI Measurement

The UE-to-UE CLI leakage on the adjacent subbands in SBFD can be measured and reported as part of UE-to-UE CLI mitigation techniques. The inter-subband CLI in SBFD may impose higher interference on subband-edge RBs and resources, whereas the resources in the middle of the SBFD subband may not experience CLI as much. As such, measuring and averaging the CLI over the whole SBFD subband may result in down estimation of the CLI.

For example, the UE can measure a delta parameter that is based on measuring and calculating the difference between a first CLI-RSSI measured from the resources in the edge of the configured RBs and a second CLI-RSSI measured from the resources located in the middle of the configured RBs. As such, the difference between the CSI-RSSI measured in the edge and CLI-RSSI measured in the middle can be used as an indication of inter-subband CLI.

Observation 17. Inter-subband UE-to-UE CLI measurement in SBFD slots based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband.

Proposal 13. Study measurement resources and reporting configurations for subband-edge CLI measurement.

### CLI mitigation via monitoring beams

In general, DL/UL resource assignment for a UE can be based on CSI/beam measurement and reporting procedures supported in NR. Based on the CSI/beam measurements performed on a configured DL RS (e.g., CSI-RS) resource, the UE can report one or more preferred beam/RS indexes along with corresponding quality metrics such as CQI, L1-RSRP, or L1-SINR. The reported contents can be subband-wise, depending on gNB’s configuration, and those can be used for gNB’s scheduling on the DL/UL resource assignment for the UE. The beam information for the DL/UL resource assignment can be based on an indicated transmission configuration indicator (TCI) for a DL/UL channel/signal. When the UE receives a grant (DCI), the UE can apply a spatial filter determined by the indicated beam information to perform the DL reception or the UL transmission. These general procedures are not based on any dynamic information related to CLI (e.g., UE-to-UE CLI) as the UE’s measurement and reporting is based on a configured RS resource which is relevant only with the gNB.

A CLI due to a signal transmitted by other (aggressor) UE can be present and severely degrade a reception performance of a DL signal by the grant at the UE, where the presence of the interference may not be known prior to the grant, e.g., in case when the aggressor UE is associated with a different serving-cell/TRP. Due to the CLI which is not captured in the general CSI/beam reporting, the UE may fail to receive the DL signal, which degrades the DL performance. The failure in receiving the DL signal can continue to happen, in case when such unexpected UE-to-UE CLIs may exist for a duration of time.

Since such a UE-to-UE CLI can happen unexpectedly, it is beneficial to consider a conditional CLI handling behavior based on monitoring the beams at the (victim) UE side, where the monitoring can be conducted per subband which can be configured to the UE as performing subband-wise CLI measurement and reporting. Then, the victim UE can report the subband-wise CLI monitoring results, which can be used at the gNB to determine whether the assigned DL/UL resource to the UE should be changed to a different subband to avoid the CLI on a subband. The conditional behavior on CLI monitoring done at the victim UE may greatly simplify the network and UE implementation for CLI handling, as this procedure is transparent to the aggressor UE side. The condition can at least include a case when the victim UE detects a PDSCH reception failure, e.g., along with NACK transmission in general.

Observation 18. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP.

Proposal 14. Study a conditional CLI handling behavior based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

## UL/DL timing alignment

In conventional TDD systems, the UL and DL slots are considered separately in time domain. Therefore, subsequent slots are split in DL-only, UL-only, and flexible slots. The symbols in flexible slots can be scheduled to be used as DL or UL based on received configurations. The flexible symbols can also be used as guard period for timing alignment requirements (e.g., DL/UL switching, UL timing advance (TA)).

In SBFD schemes with one or more subbands allocated for UL transmission in DL slots, a non-zero timing advance or switching time in SBFD could result in inter-slot interference. As an example, shown in Figure 6, where the UL signals in UL SBs and DL signals in DL SBs/slots are interfered by each other due to timing advance. Also, inter-slot interference is shown in Figure 7 that is affecting the SBFD slots due to required switching time for UE between DL and UL that is also a factor for the timing alignment requirements.

For example, especially for a semi-statically scheduled resource (e.g., configured grant PUSCH), every time when UE receives a TA command which affects all related UL transmission including the CG-PUSCH, the UE has to check whether the adjusted/accumulated TA and the switching time requirement result in overlap with DL symbol in a prior DL slot, if scheduled, for SBFD operation. This does not happen in legacy TDD system as there is the flexible slot for this reason at least, while in SBFD operation, it is always possible to face a back-to-back scheduling between DL and UL for a UE.

Figure 6. Inter-slot interference due to timing advance in SBFD UL SBs

Figure 7. Inter-slot interference due to switching time between DL and UL

While the gNB might address the required timing advance/switching time based on resource allocations and scheduling (e.g., as part of gNB implementation), this requires significantly increased scheduling complexity at the gNB side and also increases the signaling overhead, especially for taking such semi-statically scheduled resources for both DL and UL into account. In another aspect, in case a scheduled (dynamic) grant for UL or DL with high priority is received and if the timing alignment requirement including required Rx/Tx or Tx/Rx switching time is not met, e.g., resulting in some DL/UL overlapped symbols being faced at the UE, this could again result in a severe performance degradation or dropping corresponding slots.

Observation 19. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in inter-slot interference and dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

Therefore, there should be a means for UE to detect if the allocated UL/DL timing alignment is accurate and enough, especially for such back-to-back scheduling between DL and UL given to the UE. In case UE has detected timing discrepancies, the UE could report the issues along with requesting for adjustment or at least should be able to determine which overlapped portion of either DL symbol(s) or UL symbol(s) can be dropped or punctured. For example, the UE could also determine to include the time required for the timing alignment inside respective UL/DL SB in SBFD slot, where the scheduled UL transmission on the SBFD slot can have a rate-matched or punctured symbol(s) in the front symbol positions of the UL transmission, or alternatively DL symbol(s) can be punctured, which should depend on gNB’s flexible configurations for such UE behaviours.

Proposal 15. Study UL/DL timing alignment issues in subband non-overlapping full duplex systems.

## PUCCH/PUSCH Resource Allocation in SBFD Symbols

It is important to properly support UL resource allocations in SBFD symbols with UL subbands, which is one of main goals for this NR-Duplex feature. In a scenario, a HARQ-ACK/PUCCH transmission that is configured for a SPS PDSCH or by a dynamic-grant based PDSCH could be scheduled in an SBFD slot, where the respective PUCCH resource indicator (PRI) can point to a frequency location that is outside of the SBFD UL subband boundary. As such, the PRI that was originally configured to span the whole BWP in a legacy UL-only slot, may point to a frequency position outside of the SBFD UL subband. This could result in dropping the HARQ-ACK/PUCCH transmission that could lead to increase the latency, which is also one of the main objectives of employing SBFD in NR-Duplex. Alternatively, the UE could be configured with supplementary configurations so that the UE could reinterpret the original PRI to be mapped to different frequency resources inside the SBFD UL subband boundaries.

In another scenario, repetition-based PUSCH transmissions may be scheduled for a UE, where one or more of the repetitions can be occurred in SBFD slots. Since the SBFD slots include UL subbands, it needs to be addressed whether they could be counted as available slots or not. For example, the UE could decide whether to count the SBFD slots as well, or only count the legacy UL slots as available, based on configurations from gNB. In case the scheduled frequency resources are within the SBFD UL subbands, this should be straightforward for UE to be able to transmit PUSCH repetitions in respective SBFD slots. However, in case the scheduled frequency resources are outside of the SBFD UL subbands’ boundaries, the UE may need further configurations on how to reinterpret the scheduled frequency resources to be mapped within the SBFD boundaries.

Observation 20. In case the scheduled frequency resources for an UL transmission are located outside of the boundaries of the SBFD UL subbands, the UE may need further configurations to reinterpret the frequency resources to be mapped within the SBFD UL boundaries.

Proposal 16. Study the feasibility and potential benefits of providing supplementary configurations to the UE to be used in case PUCCH/PUSCH frequency resources are mapped to outside of the SBFD UL boundaries.

# Conclusion

In this contribution, we discussed subband non-overlapping full duplex (SBFD) operations, based on the analysis on effects of intra-/inter-subband CLI, and discussed issues on power management, sensing-based CLI mitigation, UL/DL timing alignment, UE-to-UE CLI mitigation, resource allocations in SBFD, and benefits of SBFD operating through multiple BWPs. From the discussions, we made following observations and proposals:

Observation 1. Mixed D/U regions informed to a UE per symbol/slot in a cell can be used for a subband-wise UL transmission or DL reception, which results in UL coverage/capacity enhancement while achieving parallel DL transmissions over non-overlapped RBs.

Observation 2. In cases with dynamic RB gap indication, the DL subbands can be determined based on the indicated UL subbands and the RB gap.

Observation 3. Using SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration could result in improving scheduling flexibility, optimal resource allocations, lower latency, and so forth.

Observation 4. In case the UE is configured with physical channel priorities, the UE could use them to handle collisions in SBFD symbols.

Observation 5. In case the SBFD operation is not supported in SSB symbols, the SBFD objectives such as UL coverage enhancement and latency reduction may be affected and degraded.

Observation 6. In case the beam direction corresponding to a UE with UL transmission scheduled in the same symbol as an SSB transmission is far enough from the beam direction of the respective SSB, the simultaneous UL transmission in SSB symbols can be possible.

Observation 7. In cell (re)selection for SBFD cells with reduced EPRE due to SSB transmission in SBFD symbols, in case the UE knows the ratio by which the EPRE is reduced, the UE can use scaling parameters for compensating the reduced EPRE in evaluating the RSRP or RSRQ during cell ranking.

Observation 8. DL throughput performance suffers considerably at high MCSs and approaches to almost zero when there is any degree of intra-subband CLI overlap with DL signal.

Observation 9. Inter-carrier interference, resulting from time advance on inter-subband CLI may impact DL throughput performance significantly, especially at high MCS indices, when there is no adjacent inter-subband distance between the DL signal and the CLI, i.e., 0-RB gap.

Observation 10. At high MCS indices, DL throughput performance recovers to 90% of the maximum throughput when the inter-subband distance between the DL signal and the CLI signal is at least 2-RB in this example scenario.

Observation 11. In addition to the frequency gap between the DL signal and the CLI (UL) signal, the amount of frequency resources (PRBs) allocated to the DL signal relative to that for the CLI signal also impacts the normalized throughput performance significantly.

Observation 12. Restricting DL subband transmissions on slots that correspond to UL slots in legacy TDD can improve uplink performance but negatively impacts downlink performance.

Observation 13. The static/fixed subband partitioning, e.g., [DUD] = [40 20 40] RB split all the time, results in worse performance for SBFD compared with legacy TDD in downlink. Thus, flexible/versatile subband partitioning and its dynamic indication mechanisms should be considered to cope with varying traffic/channel conditions.

Observation 14. Utilizing SBFD option 2 (total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD) improves SBFD performance.

Observation 15. In the SBFD scenario, an UL transmission over a subband causes significant UE-to-UE CLI leakage on the adjacent DL subband depending on a frequency gap between the UL RBs and DL RBs of each subband.

Observation 16. As a part of gNB implementation, the gNB may apply a downlink power backoff on some SBFD slots or symbols to deal with self-interference caused by the FD operation, which can impact to UE behaviours depending on the amount of power backoff.

Observation 17. Inter-subband UE-to-UE CLI measurement in SBFD slots based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband.

Observation 18. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP.

Observation 19. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in inter-slot interference and dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

Observation 20. In case the scheduled frequency resources for an UL transmission are located outside of the boundaries of the SBFD UL subbands, the UE may need further configurations to reinterpret the frequency resources to be mapped within the SBFD UL boundaries.

Proposal 1. Study mechanisms on how to inform UE of mixed D/U regions per symbol/slot to enable subband non-overlapping FD (SBFD) operations.

Proposal 2. Consider implicit determination on the subband frequency locations of the DL subbands based on the indicated UL subband at least, where a parameter on RB gaps can instead be indicated when needed.

Proposal 3. Study the aspects of BWP switching for SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration.

Proposal 4. Consider configuration of physical channel priorities for handling the collisions in SBFD symbols, including uplink, downlink, control channels, data channels, dynamic grant, configured grant, and so forth.

Proposal 5. Support SBFD operation in SSB symbols.

Proposal 6. Support the simultaneous UL transmission in SSB symbols, where the spatial collision avoidance can be performed via gNB implementation.

Proposal 7. Support enhancements on random-access procedures and cell ranking for cell (re)selection including cells operating SBFD, to avoid latency and ping-ponging effects.

Proposal 8. Study performance of applying a frequency gap or guard RBs for a UL transmission in an SBFD framework for interference mitigation with regards to adjacent DL subbands.

Proposal 9. Analysis on various downlink performance degradation aspects due to the SBFD operations compared with legacy TDD systems should also be an important part of the NR-Duplex study.

Proposal 10. To overcome the degraded downlink performance due to the static/fixed subband partitioning, flexible/versatile subband partitioning and its dynamic indication mechanisms should be further discussed to cope with varying traffic/channel conditions.

Proposal 11. Study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically.

Proposal 12. Consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario.

Proposal 13. Study measurement resources and reporting configurations for subband-edge CLI measurement.

Proposal 14. Study a conditional CLI handling behavior based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

Proposal 15. Study UL/DL timing alignment issues in subband non-overlapping full duplex systems.

Proposal 16. Study the feasibility and potential benefits of providing supplementary configurations to the UE to be used in case PUCCH/PUSCH frequency resources are mapped to outside of the SBFD UL boundaries.

# References

RP-220633, “Revised SID: Study on evolution of NR duplex operation”, CMCC.

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109-e, May 2022

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111, November 2022

# Appendix – Simulation assumptions

Table 7. LLS simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 |
| Antenna Configuration | 1Tx-2Rx |
| Carrier Frequency | 4GHz |
| SCS | 15kHz |
| MCS | 4, 10, 19 and 27 |
| System BW | 20MHz |
| Allocated no. of RBs for UL and DL signals | DL (Victim): 1RB, 2RBs, 5RBs, 10 RBs, 25RBs, 50RBs |
|  | UL (Aggressor): 25RBs |
| DL to UL power ratio (dB) | -6dB, 0dB, 6dB, 18dB |
| UL timing advance over DL | 0 , 1/2 of a symbol, 1/4 of a symbol |
| Propagation condition | TDL-A |
| Delay Spread | 30ns |
| UE Velocity | 3km/h |
| DMRS | DM-RS type-1, # of DMRS 1 + 1 |
| Channel estimation | Realistic |
| Receiver Type | MMSE |
| PA nonlinearity | None |

Table 8. SLS simulation assumptions

| Parameter | Deployment Scenario |
| --- | --- |
|  | Urban Macro (UMa) (from 38.913) |
| Layout | 21 cells with wraparound ISD: 500m |
| Channel Model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor |
| UE Mobility | 3 Km/hr |
| Carrier frequency | 3.5 GHz |
| System bandwidth | 100 MHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25 m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |
| Open-loop power control | Default: P_0 = -92 dBm, alpha=1.0 |
| BS/UE TX power | BS: 49dBm, UE: 23dBm |
| BS antenna configuration | Baseline TDD: (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 1, 4, 4, 0.8, 0.5). Shared Tx/Rx antenna array with 32 TxRUs SBFD: Separate antenna array for Tx/Rx Option 1 (same # of antenna elements as TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 2, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. Option 2 (# of antenna elements for SBFD is two times that of TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. |
| Slot structure | Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] SBFD Alt. 2 - Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. SBFD Alt. 4 - Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. SBFD Subband configuration#1 with {DUD} pattern, one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. All cells use the same SBFD slot configuration |
| gNB self-interference | RSI : 135 dB, 145 dB, 165 dB |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports 4 Tx, 2 Rx |
| Channel estimation | Realistic |
| Transmission scheme | 16/32 Tx Type 1 Codebook |
| Scheduler | SU-MIMO (with PF) |
| Target BLER | 10% first transmission BLER |
| HARQ/repetition | 3 HARQ retransmission |
| Metric | DL/UL User Perceived Throughput |
| Traffic model | FTP3 (0.5MB as packet size) for DL/UL Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) DL:UL traffic ratio 1:1 and 2:1 are considered |