3GPP TSG RAN WG1 #112		     R1-2300429

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:	Discussion on more than 2bits HARQ-ACK for SPS PDSCHs with NACK only feedback mode

Agenda Item:	8.12

Document for:	Discussion and Decision

- Introduction

In this contribution, we share our view on more than 2bits HARQ-ACK for SPS PDSCHs with NACK only feedback mode.

- More than 2bits HARQ-ACK for SPS PDSCHs with NACK only feedback mode

In previous RAN1 meeting, FG#33-5-1j was agreed for NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-common PDSCH for multicast, which includes a component of support of separate SPS-PUCCH-AN-List from unicast [1].

| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-commmon PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Multiple TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast | 33-5-1f |
| --- | --- | --- | --- | --- |

That means, sps-PUCCH-AN-ListMulticast in pucch-ConfigurationListMulticast2 should be provided for SPS group-common PDSCH for multicast with NACK-only feedback when the UE is provided moreThanOneNackOnlyMode. The PUCCH for NACK-only based HARQ-ACK feedback for only SPS group-common PDSCH for multicast should be determined from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast2. In last RAN1 meeting, the following CR in [2] was agreed to capture this agreement.

# 18 Multicast Broadcast Services < Unchanged parts are omitted > For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. The UE determines a PUCCH to provide the HARQ-ACK information as described in clause 9.2.1 when UE is not provided moreThanOneNackOnlyMode, or as the first PUCCH in Table 18-1 when UE is provided moreThanOneNackOnlyMode. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. A UE that is indicated the second HARQ-ACK reporting mode, and for the case when the UE reports more than one HARQ-ACK information bits, the UE can be indicated to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or, for only one G-RNTI or only one G-CS-RNTI, according to the second HARQ-ACK reporting mode by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when the UE is provided moreThanOneNackOnlyMode. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. For a UE that is indicated the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode, all PUCCH resources associated with the second HARQ-ACK reporting mode have same starting symbol and same number of symbols. Table 18-1: Mapping of values of HARQ-ACK information bits to PUCCH resources for the second HARQ-ACK reporting mode Value of HARQ-ACK information bits; PUCCH resource. {0}; {0,0}; {0,0,0}; {0,0,0,0}; 1st PUCCH resource from resourceList/sps-PUCCH-AN-ListMulticast. {1,0}; {1,0,0}; {1,0,0,0}; 2nd PUCCH resource from resourceList/sps-PUCCH-AN-ListMulticast. {0,1}; {0,1,0}; {0,1,0,0}; 3rd PUCCH resource from resourceList/sps-PUCCH-AN-ListMulticast. {1,1,0}; {1,1,0,0}; 4th PUCCH resource from resourceList. {0,0,1}; {0,0,1,0}; 5th PUCCH resource from resourceList. {1,0,1}; {1,0,1,0}; 6th PUCCH resource from resourceList. {0,1,1}; {0,1,1,0}; 7th PUCCH resource from resourceList. {1,1,1,0}; 8th PUCCH resource from resourceList. {0,0,0,1}; 9th PUCCH resource from resourceList. {1,0,0,1}; 10th PUCCH resource from resourceList. {0,1,0,1}; 11th PUCCH resource from resourceList. {1,1,0,1}; 12th PUCCH resource from resourceList. {0,0,1,1}; 13th PUCCH resource from resourceList. {1,0,1,1}; 14th PUCCH resource from resourceList. {0,1,1,1}; 15th PUCCH resource from resourceList. < Unchanged parts are omitted >

One remaining issue is that sps-PUCCH-AN-ListMulticast can only configured at most 4 PUCCH resources which can’t support the case with more than 2 HARQ-ACK information bits in a slot. Thus, whether/how to support of more than 2 TBs in a slot for only multicast SPS PDSCHs with the second HARQ-ACK reporting mode needs further consideration. One simple option is not to support the case to provide more than 2 HARQ-ACK information bits for multicast SPS PDSCHs with second HARQ-ACK reporting mode when moreThanOneNackOnlyMode is provided. For this option, FG#33-5-1j can be modified as following.

| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-commmon PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Up to 2 TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast | 33-5-1f |
| --- | --- | --- | --- | --- |

Another option is if there are more than 2 TBs for multicast SPS PDSCHs with second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided, UE provides the HARQ-ACK information according to the first reporting mode and determines a PUCCH resource to provide the HARQ-ACK information from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast1 or sps-PUCCH-AN-List configured in pucch-ConfigurationList. For example, UE determines the PUCCH resource to provide the HARQ-ACK information from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast1 if provided; otherwise, the UE determines the PUCCH from sps-PUCCH-AN-List configured in pucch-ConfigurationList. For this option, the specification can be modified as following. However, for this option, UE behavior violates that when moreThanOneNackOnlyMode is provided and is not preferred.

For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. The UE determines a PUCCH to provide the HARQ-ACK information as described in clause 9.2.1 when UE is not provided moreThanOneNackOnlyMode, or as the first PUCCH in Table 18-1 when UE is provided moreThanOneNackOnlyMode. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. A UE that is indicated the second HARQ-ACK reporting mode, and for the case when the UE reports more than one HARQ-ACK information bits, the UE can be indicated to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or, for only one G-RNTI or only one G-CS-RNTI with up to 2bits HARQ-ACK information, according to the second HARQ-ACK reporting mode by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when the UE is provided moreThanOneNackOnlyMode. For only one G-CS-RNTI with more than 2 bits HARQ-ACK information, when the UE is provided moreThanOneNackOnlyMode , the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0.

Another option is if there are more than 2 TBs for multicast SPS PDSCHs with second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided, UE provides the HARQ-ACK information according to the second reporting mode and determines a PUCCH resource to provide the HARQ-ACK information from resourceList configured in pucch-ConfigurationListMulticast2. That is, same PUCCH resources are used for DG PDSCH and SPS PDSCH HARQ-ACK feedback. For this option, the specification can be modified as following.

For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. The UE determines a PUCCH to provide the HARQ-ACK information as described in clause 9.2.1 when UE is not provided moreThanOneNackOnlyMode, or as the first PUCCH in Table 18-1 when UE is provided moreThanOneNackOnlyMode. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. A UE that is indicated the second HARQ-ACK reporting mode, and for the case when the UE reports more than one HARQ-ACK information bits, the UE can be indicated to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or, for only one G-RNTI or only one G-CS-RNTI, according to the second HARQ-ACK reporting mode by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when the UE is provided moreThanOneNackOnlyMode wherefor the HARQ-ACK only in response to a G-CS-RNTI, when the number of HARQ-ACK is no more than 2 bits, the UE selects a PUCCH resource from a set of PUCCH resources provided by sps-PUCCH-AN-ListMulticast. When the number of HARQ-ACK is more than 2 bits, the UE selects a PUCCH resource from a set of PUCCH resources provided by resourceList. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. For a UE that is indicated the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode, all PUCCH resources associated with the second HARQ-ACK reporting mode have same starting symbol and same number of symbols.

Proposal: When a UE provides only HARQ-ACK information bits for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided,

- Option 1: UE does not expect to provide more than two HARQ-ACK information bits only for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode.

- Option 2: When the number of HARQ-ACK information bits is more than two, the UE provides HARQ-ACK information according to the second HARQ-ACK reporting mode and determines a PUCCH to provide the HARQ-ACK information bits from resourceList configured in pucch-ConfigurationListMulticast2.

Considering at the late stage, the above first option is slightly preferred.

- Conclusion

In this contribution, we discuss for more than 2bits HARQ-ACK for SPS PDSCHs with NACK only feedback mode, and the following proposal is made.

Proposal: When a UE provides only HARQ-ACK information bits for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided,

- Option 1: UE does not expect to provide more than two HARQ-ACK information bits only for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode.

- Option 2: When the number of HARQ-ACK information bits is more than two, the UE provides HARQ-ACK information according to the second HARQ-ACK reporting mode and determines a PUCCH to provide the HARQ-ACK information bits from resourceList configured in pucch-ConfigurationListMulticast2.

References

[1]	RAN WG1 #110bis e-meeting Chairman’s Notes

[2] R1-2212974, CR on interpretation of moreThanOneNackOnlyMode, Moderator (Huawei), HiSilicon, CBN, Qualcomm, vivo, ZTE