3GPP TSG RAN WG1 #112                                              R1- 2300455

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:	Co-channel coexistence for LTE sidelink and NR sidelink

Agenda Item:	9.4.2

Document for:	Discussion and Decision

- Introduction

The NR Sidelink evaluation WID has been revised to focus on Type A devices and operating combination A for studying the mechanism of co-channel coexistence for LTE sidelink and NR sidelink[1].

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A

In this contribution, we provide our investigation on the co-channel coexistence for LTE sidelink and NR sidelink, including the scenarios and potential issues, as well as some preliminary simulation results.

- Resource collision handling

In the previous meetings, the following agreements were achieved.

Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms. Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.

It has been agreed that the PHY layer carries out the resource determination for dynamic resource pool sharing of NR and LTE SL. Then, what information should be shared by LTE SL module and how the NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure should be discussed.

Firstly, to help the NR SL module avoid resource collision, the LTE SL module should share the sensing results with NR SL module. The reservation and measurement-related information, such as the time and frequency locations of reserved resources by other LTE UEs, SL RSRP measurement results, resource reservation period, and priority based on decoded SCI, should be shared by the LTE module. Besides, the LTE logical subframe-related information is also needed to determine the resource location of reserved resources by other LTE UEs, in case the resource pool of NR and LTE SL are not fully aligned.

Proposal 1: For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module includes one or more of the following parameters:

- Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCI and for own LTE SL transmissions

- Priority based on decoded SCI and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

- LTE logical subframe related information

- Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE

Once the NR SL module acquires the information, it can exclude the resources reserved by LTE RAT with a RSRP higher than the corresponding NR RSRP threshold in the resource selection procedure. If necessary, the NR SL module could further adopt a dedicated RSRP threshold for evaluating whether the resources reserved by LTE RAT should be excluded to protect LTE transmission. Alternatively, the LTE SL module could just indicate the time and frequency locations of reserved resources of other LTE UEs to the NR SL module. In this case, the NR SL module would exclude these indicated resources regardless of the priority and RSRP similar to the Inter-UE Coordination mechanism, thus the information that needs to be shared from the LTE SL module is the resources reserved by LTE RAT. NR SL module may even drop its transmission to avoid resource conflict if RSRP measurement or priority of LTE transmission is higher than the threshold.

Proposal 2: NR SL module performs resource allocation similar to Rel-16 or Rel-17 to exclude the resources reserved by LTE RAT, or drop its transmission to avoid the resource conflict if RSRP or priority of LTE transmission is higher than the corresponding threshold.

- Mix SCS

In RAN1#110 meeting, the configuration of 15kHz in NR SL resource pool was agreed, but whether support the higher SCSs has not been decided yet.

Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools Proposal 1-5 (II): - For dynamic resource pool sharing, the following options are studied to resolve the AGC issue in LTE SL UEs which is caused by NR SL PSCCH/PSSCH transmissions if higher SCSs are supported: - Option 1: The NR SL transmissions of higher SCSs are transmitted on all slots within a LTE SL subframe of 15 kHz - FFS: Whether this takes place in all slots configured within the LTE SL resource pool or only when the NR SL transmission overlaps an LTE SL transmission based on information shared by the LTE SL module. - Option 3: NR SL UE uses the information shared by the LTE SL module in its own resource selection procedure to exclude slots overlapping with LTE SL transmissions. - FFS: Exclude only those slots where the first symbol of the NR SL transmission is not overlapping in time with the first symbol of the LTE subframe. - Note: This study does not imply RAN1 supporting higher SCS

If higher SCS is supported for the dynamic sharing resource pool, it would cause AGC setting issues for the LTE SL UEs that are using the shared resource pool. There are two options discussed in the previous meeting. Option 1 is that the NR SL transmissions of higher SCSs are transmitted on all slots within an LTE SL subframe of 15 kHz, while Option 3 is that NR SL UE uses the information shared by the LTE SL module in its own resource selection procedure to exclude slots overlapping with LTE SL transmissions.

In Option 1, NR SL UE needs to transmit on multiple consecutive slots, which is different from the legacy single-slot-based framework. Even if the design of consecutive slots in SLU is reused, there are still some issues specific to this topic. For example, the boundary of these consecutive slots should align with the boundary of LTE subframe. Otherwise, the AGC setting issue still exists. As shown in Figure 1, if NR SL UE selects NR slot 0 and NR slot 1 to transmit, the boundary between this transmission and LTE SL subframe is not aligned. Thus the AGC adjusted in LTE subframe 0 is not applicable in its second-half subframe.

Figure 1: Misalignment between NR transmission and LTE SL subframe

Observation 1: Using multiple consecutive slots for mix-SCS cocexistence needs additional efferts to ensure the NR SL transmissions of higher SCSs transmitting on all slots within a LTE SL subframe, including the support of resource (re-)selection of MCSt, and ensuring full overlap between any overlapping LTE logical subframe and NR logical slot, etc.

While in Option 3, NR SL UE would exclude all the slots overlapping with LTE SL transmissions to avoid the AGC issue brought by NR SL transmission. Nevertheless, it requires a more rigorous RSRP threshold than avoiding resource collision. Thus, Option 3 would lead to a significant capacity reduction on NR SL UE due to excessive exclusion.

In addition, to solve the excessive exclusion problem, an approach was proposed that NR module excludes only those slots where the first symbol of the NR SL transmission is not overlapping in time with the first symbol of the LTE subframe. However, this approach does not fully solve the AGC issue, even though the excessive exclusion is alleviated. As shown in Figure 2, if NR SL UE selects the slot in which the first symbol is overlapping in time with the first symbol of the LTE subframe, while the next slot is not used for transmission, the AGC gain adjusted based on the first half of the LTE subframe is no longer applicable as the experienced power level decreases significantly in the second half, which may still result in distortion due to inaccurate power compensation.

Figure 2: Impact of NR transmissions on the AGC setting of LTE SL UE

Observation 2: Option 3 would lead to a significant capacity reduction on NR SL UE due to excessive exclusion, and the approach that excluding only those slots where the first symbol of the NR SL transmission is not overlapping in time with the first symbol of the LTE subframe may not fully resove the AGC issue.

Considering both options need to be further enhanced, we prefer to limit the SCS to 15KHz for now and focus on other topics for the sake of progress.

Proposal 3: Focus study of coexistence on the same SCS case, and continue the study of mix-SCS when the coexistence of 15KHz is completed.

- PSFCH

In last meeting, the following proposal was proposed.

Proposal 1-1(VII): - For dynamic resource pool sharing, in NR SL resource pools with PSFCH configured and when HARQ-ACK is enabled, based on (pre-)configuration, when PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain, the NR SL UE - Always avoids transmissions on the PSFCH resources (Alt 1), or - FFS details including whether the TX UE avoid selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and/or RX UE does not transmit on the overlapping PSFCH resources. - Does not avoid transmission on the PSFCH resources (Alt 2), or - Conditionally avoids transmissions on a subset of the PSFCH resources. - FFS details of conditions including - a (pre-)configured subset, - the consideration of the LTE RSRP and LTE and/or NR priority, - presence of PSCCH/PSSCH transmission in the same time slot LTE subframe as PSFCH transmission with the same power by the same UE. - FFS for the case when there is an overlapping of time and frequency resources between PSFCH and LTE SL transmission - Introduce additional PSFCH periodicity of [5, 8 and] 10. - Note: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration. - FFS: Whether to confine the PSFCH transmission, in the time domain, within the guard symbol of the LTE SL subframe. FFS details including the conditions.

To avoid the interference between PSFCH transmission of NR devices and LTE transmission, there are several alternatives proposed in the last several meetings.

In the first alternative, NR SL UE always avoids transmissions on the PSFCH resources when PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain. One simple way is that the NR UE does not transmit PSFCH when the PSFCH transmission in time slots would overlap with subframes used for LTE SL transmissions. However, this alternative may cause the number of (unnecessary) NR PSSCH retransmissions to increase heavily. Consequently, not only the NR SL performance is degraded (e.g., larger latency, severer congestion, and higher packet loss rate, etc.), but also the LTE SL performance would be degraded due to higher inter-system interference.

Observation 3: Only RX UE not transmitting PSFCH on the overlapping PSFCH resources would result in performance degradation of both NR SL UE and LTE SL UE.

To alleviate the above problems, NR devices can select the PSSCH resource that both the PSSCH resource and its corresponding PSFCH resource have not been reserved by LTE devices. This enables the UE transmitting PSSCH to avoid the PSFCH collision with LTE transmission, then the PSSCH transmission time and latency would be affected less.

Proposal 4: When PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain, TX UE avoids selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and RX UE does not transmit on the overlapping PSFCH resources.

Additionally, if NR SL UE conditionally avoids transmissions on a subset of the PSFCH resources, the above problems can also be alleviated. For example, NR SL UE is allowed to transmit PSFCH when the measured LTE RSRP is lower than the threshold, which implies that the interference to LTE is low, or when the priority of PSFCH is higher than the threshold, to protect high-priority NR SL transmission. However, this alternative would permit too many PSFCH transmissions, and the interference between PSFCH transmission of NR devices and LTE transmission still exists in some PSFCH resources. In order to allow NR SL UE to transmit some high-priority PSFCH and reduce the interference as much as possible, NR SL UE could conditionally transmit PSFCH on a subset of the PSFCH resources. For example, if the priority of PSFCH is higher than a threshold or the RSRP measurement result is lower than a threshold, NR SL UE is allowed to transmit PSFCH on some PSFCH resources.

Observation 4: The alternative that NR SL UE conditionally avoids transmissions on a subset of the PSFCH resources cannot solve the interference problem between PSFCH transmission of NR devices and LTE transmission.

Proposal 5: NR SL UE conditionally transmits PSFCH on a subset of the PSFCH resources. For example, high-priority PSFCH or low-interference PSFCH is allowed to transmit on some PSFCH resources.

Nevertheless, there is another alternative proposed to resolve this issue, i.e., introducing additional longer PSFCH periodicity. In this approach, as more PSFCH transmission are stacked in one PSFCH occasion, the RSSI of the PSFCH slot would rise. Hence, when LTE SL selects transmission resources, the probability of selecting slots with PSFCH occasion into the set SB goes down after candidate resource reordering by S-RSSI. However, such resource exclusion may be problematic, because the S-RSSI period does not match the PSFCH period. Moreover, this alternative may cause backward compatibility issues with legacy UE.

| Figure 3: Comparison of average PRR of LTE-UE | Figure 4: Comparison of average PRR of NR-UE |
| --- | --- |

Some simulations were executed to evaluate the performance of the following alternatives:

Alt 1: NR UE does not transmit PSFCH when the PSFCH transmission in time slots would overlap with subframes used for LTE SL transmissions.

Alt 2: NR UE adopt a longer PSFCH period, so LTE UE could avoid PSFCH transmission via S-RSSI resource reordering.

The general simulation parameters can be found in Annex A. It can be observed that Alt 2 suffers an obvious PRR loss (about 4%) in NR RAT comparing with Alt 1, while the performance gain in LTE RAT is negligible. The PRR loss comes from the situation that the transmission interval in NR RAT is prolonged due to a longer PSFCH period, which results in less transmission opportunities.

Observation 5: Alt 1 obtains around 4% PRR gain than Alt 2 in NR RAT, while the PRR loss in LTE RAT is not obvious.

Observation 6: If introducing additional longer PSFCH periodicity, how to address the prolonged transmission latency and less transmission opportunities problems should be discussed, for example, by allowing the combination of blind transmission and HARQ-based transmission to reduce transmission latency.

- S-SSB Transmission

In both LTE SL and NR SL, the resource pool is determined by excluding the synchronization signal resources. However, the design of the synchronization signal in LTE SL and NR SL is different, thus, there might be a collision between the synchronization signal of one RAT and the PSSCH transmission of the other RAT. Thus, an additional mechanism would be required to ensure that synchronization signal resources of both RATs are excluded from the resource pools. For example, the synchronization signals in both LTE SL and NR SL could be aligned in the time domain. Additionally, the NR SL module reuses the LTE SL synchronization to avoid the collision between the synchronization signal and the resource pools of different RATs due to the timing difference between LTE SL and NR SL.

Observation 7: The design of the synchronization signal is different between LTE SL and NR SL, so there might be a collision between the synchronization signal of one RAT and PSSCH transmission of the other RAT.

Proposal 6: Dynamic resource sharing should resolve the issue of collision between the synchronization signal and the resource pools of different RATs, e.g., by aligning LTE SL and NR SL synchronization signals in time domain, or adopting LTE SL synchronization signals for NR SL, etc.

- Conclusion

This contribution discuss the co-channel coexistence for LTE sidelink and NR sidelink, with the following observations and proposals:

Observation 1: Using multiple consecutive slots for mix-SCS cocexistence needs additional efferts to ensure the NR SL transmissions of higher SCSs transmitting on all slots within a LTE SL subframe, including the support of resource (re-)selection of MCSt, and ensuring full overlap between any overlapping LTE logical subframe and NR logical slot, etc.

Observation 2: Option 3 would lead to a significant capacity reduction on NR SL UE due to excessive exclusion, and the approach that excluding only those slots where the first symbol of the NR SL transmission is not overlapping in time with the first symbol of the LTE subframe may not fully resove the AGC issue.

Observation 3: Only RX UE not transmitting PSFCH on the overlapping PSFCH resources would result in performance degradation of both NR SL UE and LTE SL UE.

Observation 4: The alternative that NR SL UE conditionally avoids transmissions on a subset of the PSFCH resources cannot solve the interference problem between PSFCH transmission of NR devices and LTE transmission.

Observation 5: Alt 1 obtains around 4% PRR gain than Alt 2 in NR RAT, while the PRR loss in LTE RAT is not obvious.

Observation 6: If introducing additional longer PSFCH periodicity, how to address the prolonged transmission latency and less transmission opportunities problems should be discussed, for example, by allowing the combination of blind transmission and HARQ-based transmission to reduce transmission latency.

Observation 7: The design of the synchronization signal is different between LTE SL and NR SL, so there might be a collision between the synchronization signal of one RAT and PSSCH transmission of the other RAT.

Proposal 1: For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module includes one or more of the following parameters:

- Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCI and for own LTE SL transmissions

- Priority based on decoded SCI and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

- LTE logical subframe related information

- Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE

Proposal 2: NR SL module performs resource allocation similar to Rel-16 or Rel-17 to exclude the resources reserved by LTE RAT, or drop its transmission to avoid the resource conflict if RSRP or priority of LTE transmission is higher than the corresponding threshold.

Proposal 3: Focus study of coexistence on the same SCS case, and continue the study of mix-SCS when the coexistence of 15KHz is completed.

Proposal 4: When PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain, TX UE avoids selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and RX UE does not transmit on the overlapping PSFCH resources.

Proposal 5: NR SL UE conditionally transmits PSFCH on a subset of the PSFCH resources. For example, high-priority PSFCH or low-interference PSFCH is allowed to transmit on some PSFCH resources.

Proposal 6: Dynamic resource sharing should resolve the issue of collision between the synchronization signal and the resource pools of different RATs, e.g., by aligning LTE SL and NR SL synchronization signals in time domain, or adopting LTE SL synchronization signals for NR SL, etc.

- Reference

- RP-221938, “WID revision: NR sidelink evolution” .

- R1-2007687, “Discussion on sidelink evaluation methodology”.

# Annex A

Table 1 System level simulation assumption

| Parameter | value |
| --- | --- |
| Deployment | Urban scenario |
| Link type | V2V |
| UE type | NR UE : LTE UE = 1:1, the number of each type of UE is 150 |
| PSFCH period | 4 slots in Alt 1, 10 slots in Alt 2 |
| Communication type | Broadcast in LTE module, multicast in NR module |
| Carrier frequency | 6GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15KHz |
| Traffic parameter for LTE and NR | Traffic type: Periodic traffic Packet arrival interval of periodic traffic: 50ms Packet latency requirement of periodic traffic: 50ms Packet size of periodic traffic: 800 or 1200byte |
| Power model | Follow TR 38.840 with modifications discussed in [2] |
| Max transmission time | Twice for LTE and six times for NR |
| TX power | 23dBm |