3GPP TSG RAN WG1 #112		 	R1-2300567

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.2.2.1

Source:	           Xiaomi

Title:	Discussion on evaluation on AI/ML for CSI feedback enhancement

Document for:	Decision

- Introduction

In this contribution, we present our views on evaluation methodology and preliminary evaluation results on AI/ML for CSI feedback enhancement, including AI/ML based CSI compression and AI/ML based CSI prediction in time domain.

- Discussion on AI/ML based CSI compression

  - Template for evaluation results reporting of AI CSI compression

In previous meetings [1][2], the following working assumption and agreement were achieved:

Working Assumption The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression without generalization/scalability verification - FFS the description and results for generalization/scalability may need a separate table - FFS the value or range of payload size X/Y/Z - FFS the description and results for different training types/cases may need a separate table - FFS: training related overhead Table X. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [training type/case] Source 1; …. CSI generation part; AL/ML model backbone. Pre-processing. Post-processing. FLOPs/M. Number of parameters/M. [Storage /Mbytes]. CSI reconstruction part; AL/ML model backbone. [Pre-processing]. [Post-processing]. FLOPs/M. Number of parameters/M. [Storage /Mbytes]. Common description; Input type. Output type. Quantization /dequantization method. Dataset description; Train/k. Test/k. Ground-truth CSI quantization method. [Other assumptions/settings agreed to be reported]. Benchmark. Intermediate KPI I#1 of benchmark, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Intermediate KPI I#1 of benchmark, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for intermediate KPI I#1, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for intermediate KPI#1, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. …. Intermediate KPI I#2 of benchmark, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Intermediate KPI I#2 of benchmark, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for intermediate KPI I#2, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for intermediate KPI#2, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. …. Gain for Mean UPT; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. Gain for 5% UPT; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. …. FFS others. Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations: - Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side - Case 2: Not aligned AI/ML model structures between NW side and UE side - Companies to report the AI/ML structures for the UE part model and the NW part model, e.g., different backbone (e.g., CNN, Transformer, etc.), or same backbone but different structure (e.g., number of layers) - FFS different sizes of datasets between NW side and UE side - FFS aligned/different quantization/dequantization methods between NW side and UE side - FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

The Table in the working assumption summarized the required information and metrics of AI model inference, and the training related overhead is for further study. In our understanding, training related overhead as a metric for the complexity of AI model training should be captured in the table. And the other parameters which would impact the complexity of AI model training e.g., the required dataset size for convergence, number of epoch times, batch size and learning rate should be captured in the table, too.

In last meeting, whether separate table is needed for Type 1 and Type 2 with one UE and one NW training was heated discussed, and whether the description and results for different training types/cases need a separate table is for further study. From our understanding, the only difference between type 1 and type 2 with one UE and one NW is whether the back propagation information quantized. If the gradient is quantized before delivery from NW to UE, there might be performance loss compared with Type 1 due to the quantization. Considering that type 2 training is performed in an offline way and the gradient is delivered without loss, it is not necessary to provide separate table for Type 1 and Type 2 with one UE and one NW. For Type 3 separate training, when not aligned AI/ML model structures between NW side and UE side are used, it was agreed that companies to report the AI/ML structures for the UE part model and NW part model, e.g., different backbone, or same backbone but different structure (e.g., number of layers). However, there is only AI/ML backbone listed in the table and more details about the AI/ML structure would be beneficial for companies to reproduce the evaluation, which is important for the calibration phase. To simplify the results collection, it is proposed to use separate tables for different training types/cases.

Based on the discussion above, we suggest to update the template table as following at least for type 3 case 2 not aligned AI/ML model between NW side and UE side:

Table 1. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [Type 3/case 2]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |  |
|  | AI/ML model structure e.g., number of layers |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
|  | Convergence time of AI model training, e.g., the number of epoch times, batch size, learning rate |  |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |  |
|  | AI/ML model structure e.g., number of layers |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
|  | Convergence time of AI model training, e.g., the number of epoch times, batch size, learning rate |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
| Dataset description | Train/k Note: Various dataset size can be considered. |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method |  |  |  |
|  | Training related overhead |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

Proposal 1: The parameter and metric for complexity of AI model training e.g., the training related overhead, the required dataset size for convergence, number of epoch times, batch size and learning rate are captured in the template table.

Proposal 2: AI model structure e.g., the number of layers is provided in the template table at least for case 2 of Type 3 separate training.

Proposal 3: Using separate tables for different training types/cases is supported.

Proposal 4: Adopt Table 1 as template for simulation results collection at least for case 2 of Type 3 separate training.

  - Scalability for CSI compression

In last meeting, the following agreements were achieved on the evaluation of scalability for CSI compression:

Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1. - Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn. - Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1. - Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn. - Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization. - Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) in the following are evaluated - Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions. - Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions - Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions

Scalability over different output dimensions of CSI generation part

The generalization evaluation of different size of CSI feedback payloads is conducted by parallel training of one encoder part and three decoder parts. The basic AI model for feedback payloads generalization is as Fig.1 shows. Setting the maximum feedback bit number as 240 bit, different feedback payloads are obtained by cutting off the tail of the maximum 240 bit. The output of encoder is always equal to the maximum bit number, while the input of the decoder is the truncated bit of the output of encoder. The loss function is the average results of different decoder parts. In this two-sided model, the UE side only need to deploy one encoder part, and the output of encoder can be various based on gNB’s configuration and/or indication.

The evaluation result is shown in Table 2, where the baseline results is achieved by training one encoder part and one decoder part with different feedback payloads separately, the generalization is results of generalized AI model, which is a single AI model with one encoder part and three decoder parts. As Table 2 shows, the generalized AI model has worse performance than the baseline scheme, the less the feedback payloads, the lager the gap. In that sense, it is proposed to study the pre-processing mechanisms for input of decoder to improve the AI model generalization performance on various feedback payloads.

Figure 1, The basic structure of AI model for feedback payloads generalization

Table 2, SGCS results of feedback payload generalization

|  | 120 bit | 180 bit | 240 bit |
| --- | --- | --- | --- |
| Baseline | 0.70 | 0.78 | 0.83 |
| Generalization | 0.60 | 0.72 | 0.81 |

Scalability over different input dimensions of CSI generation part

For evaluating the scalability over different input dimensions of CSI generation part, we conducted simulation by using different size of subband. Keeping totally 48 RB unchanged, two options are considered in the simulation. Both the dataset of Option 1 and Option 2 are used for training, and 0 is padding to the end of vector for option 2.

- Option 1: 2 RB per subband

- Option 2: 4 RB per subband

The evaluation result is obtained based on the feedback payload equals to 120 bits and is shown in Table 3, where the baseline results is achieved by training AI model with fixed subband number and testing AI model with the same subband number. And the generalization results is achieved by training AI model with mixed subband number and testing AI model with different subband number separately. As Table 3 shows, the generalization model can achieve similar SGCS performance with the baseline. It can be observed that the AI/ML model shows good generalization performance in various subband number.

Table 3, SGCS results of subband number generalization

|  | 2RB per subband | 4RB per subband |
| --- | --- | --- |
| Baseline (case 1) | 0.65 | 0.70 |
| Generalization (case 3) | 0.63 | 0.69 |

Observation 1: The generalized AI model does not work well on various CSI feedback payloads, especially for a small number of feedback payloads.

Observation 2: AI/ML model shows good generalization performance in different sizes of subband and subband number.

Proposal 5: RAN1 study pre-processing mechanisms for the input of decoder to improve the AI model generalization performance on various feedback payloads.

  - Scenario generalization for CSI compression

We present our preliminary evaluation results for different scenarios in this section, and all three cases with different training dataset and testing dataset are considered in the evaluation.

Four kinds of training dataset as Uma only, Umi only and mixed Uma and Umi with different ratios are considered, and the AI/ML model is tested by Uma and Umi dataset separately. The evaluation result is obtained based on feedback payload equals to 120 bits and shown in Table 4. As table 4 shows, for Umi testing dataset, the best performance is achieved when the AI/ML model is trained by Umi dataset only, and the worst performance is achieved when the AI/ML model is trained by Uma dataset only. When the training dataset is mixed dataset, the dataset with more Umi data has higher SCGS value. While for Uma testing dataset, the SGCS performance are very close for different training datasets. It can be observed that the AI/ML model shows good generalization performance in various scenarios.

Table 4, SGCS results of scenarios generalization

|  | Testing dataset |  |
| --- | --- | --- |
| Training dataset | Uma | Umi |
| Uma | 0.70 | 0.66 |
| Umi | 0.69 | 0.71 |
| Uma : Umi=5 : 5 | 0.71 | 0.70 |
| Uma : Umi=8 : 2 | 0.70 | 0.68 |

Observation 3: AI/ML model shows good generalization performance in different scenarios.

  - Different training types

In last meeting[1], the following agreements were achieved on Type 3 (Separate training at NW side and UE side):

Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors - Case 1 (baseline): Type 3 training between one NW part model and one UE part model - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different - FFS: different quantization methods between NW side and UE side - Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models - Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models - FFS: whether/how to report overhead of dataset

We present our preliminary evaluation results on Type 3 separate training in this section. Based on the example of Type 3, the following procedure is considered in our evaluation for the sequential training starting with NW side training:

Step 1: NW trains a two-sided AI model with both encoder part and decoder part based on dataset#1 of original CSI, Vin.

Step 2: NW shares a dataset#2 to UE for UE training encoder part, dataset#2 includes Vin and Vq, where Vq is the output of encoder at NW side. Note that Vin is not the output during the NW training, but the output results after NW side training is finished.

Step 3: UE trains encoder based on dataset#2 with the input as Vin and the label as Vq.

Step 4: Joint inference is performed by the decoder at NW side and encoder at UE side trained separately.

Three cases for Type 3 training was evaluated:

- Case 1: Aligned AI/ML model structure between NW side and UE side by using joint training

- Case 2: Aligned AI/ML model structure between NW side and UE side by using separate training

  - Step 1: NW first training by using TF structure in both CSI generation and CSI reconstruction part.

  - Step 2: NW shares dataset (Vin, Vq) to UE

  - Step 3: UE reuses the TF structure in step 1 to perform CSI generation part training

- Case 3: Not aligned AI/ML model structures between NW side and UE side by using separate training

  - Step 1: NW first training by using TF structure in both CSI generation and CSI reconstruction part

  - Step 2: NW shares dataset (Vin, Vq) to UE

  - Step 3: UE use the CNN structure to perform CSI generation part training

We firstly evaluated the baseline scheme of Type 1 joint training by using TF structure in both CSI generation and CSI reconstruction part. The backbone and structure of CSI generation part in case 2 is same with case 1, only the parameters of the model is retrained. By comparing the SGCS results of case 1 and case 2 in Table 5, it can be observed that there is small degradation of Type 3 separate training compared with Type 1 joint training on SGCS. Then we further evaluated case 3 by using CNN structure for CSI generation part training. As Table 5 shows, there is large degradation between case 2 and case 3. By comparing the SGCS results of case 1, case 2 and case 3, it can be observed that when AI model structure is known, separate training can achieve similar performance with joint training, and when different AI backbone is used for CSI generation part and CSI reconstruction part, performance of separate training decreased compared with joint training.

Table 5：Evaluation results comparison of different training types

| Case | SGCS |
| --- | --- |
| Case 1 | 0.692 |
| Case 2 | 0.690 |
| Case 3 | 0.493 |

Observations 4: When AI model structure is known, separate training can achieve similar performance with joint training.

Observations 5: When different AI backbone is used for CSI generation part and CSI reconstruction part, performance of separate training decreased compared with joint training.

  - Quantization of compressed CSI

In last meeting, the following agreements were achieved for quantization of compressed CSI

Agreement For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies: - Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training - Fixed/pre-configured quantization method/parameters is applied for the inference phase - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2: Quantization aware training, where quantization/dequantization is involved in the training process - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase - Companies to report how to update the quantization method/parameters during the training - Note: the above cases apply for training Type 1/2/3 - Others are not precluded.

We consider the following cases to study the impact of quantization in AI/ML model based CSI compression.

- Case 1: Training without quantization, inference without quantization

- Case 2: Training with quantization, inference with quantization

  - Case 2-1: Scalar quantization

  - Case 2-2: Vector quantization.

Wherein, case 1 is the ideal case without any impact of quantization, which can be assumed to the upper band of AI/ML with quantization. Case 2-1 and case 2-2 is more in line with the real system where the testing phase is with quantization. Note that 2-bit scale quantization is used in case 2-1. In case 2-2, the quantization parameters are updated in together with the AI/ML models during the training phase, after training is finished, the final quantization codebook is applied for the inference phase The SGCS results is shown in Table 6. It can be observed that there is around 7% loss caused by quantization operation by comparing the SGCS results of case 1 and case 2/3, more suitable quantization method can be considered to improve the performance. There is minor performance gap between case 2 and case 3, which implies that vector quantization and scalar quantization can achieve similar performance.

Table 6: SGCS performance of different quantization cases

| Case | SGCS |
| --- | --- |
| Case 1, Training without quantization, inference without quantization | 0.807 |
| Case 2-1, Training with scalar quantization, inference with scalar quantization | 0.753 |
| Case 2-2, Training with vector quantization, inference with vector quantization | 0.759 |

Observation 6: There is around 7% performance loss caused by quantization.

Observation 7: Vector quantization and scalar quantization can achieve similar performance.

Proposal 6: The case of training without quantization and inference without quantization should be evaluated as the upper bound.

  - Fine-tuning performance

In the previous meeting, the methodology of model fine-tuning was agreed and performance improvement is expected with model fine-tuning. We conduct the evaluation of model fine-tuning in this section. The AI/ML model is trained based on draining dataset#1 from Uma, then the trained model is updated based on fine-tuning dataset #2 from Umi. After that, the AI/ML model is tested on dataset from Umi. The amount of data in dataset#2 is around fifteenth of dataset#2.

As Table 7 shows, the performance improved to 0.673 from 0.655. Setting 0.705 as the upper band of testing results of AI/ML model in Umi scenario, and 0.655 as the lower band of testing results of AI/ML model in Umi scenario. The fine-tuning procedure can make up around half of the gap between the lower band and the upper band with much less dataset size.

Table 7: SGCS performance of AI/ML fine-tuning

| Training dataset | Uma | Umi | Uma + Umi fine-tuning |
| --- | --- | --- | --- |
| Testing dataset | Umi | Umi | Umi |
| SGCS results | 0.655 | 0.705 | 0.673 |

Observation 8: The fine-tuning procedure can be used to improve the performance.

  - AI model structure and complexity

A Transformer-based AI model is used in the evaluation for the baseline scheme of AI based CSI compression. As Figure 2 shows, the AI model including two parts as encoder part and decoder part. The input data is eigenvector wk based on SVD of channel matrix, after linear embedding and positional embedding, the input data are encoded in the encoder. The encoded data then input to the decoder to get recovered eigenvector wk'. In general, the encoding procedure is performed at UE side and UE feedback quantized data to gNB, then the decoding procedure is performed at gNB side.

The simulation assumption for intermediate results calculation is as Table 8 shows, and the AI model training parameters are as Table 9 shows. The FLOPs and number of AI/ML parameters for the CSI generation part and the CSI reconstruction part are provided separately in Table 10.

Figure 2, The basic structure of Transformer model

Table 8, Simulation assumption for intermediate result calculation

| Parameter | Value |
| --- | --- |
| Scenario | Uma and/or Umi |
| Frequency Range | 2GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| AI content | Eigenvector |
| Channel estimation | Ideal |

Table 9, Parameter of AI model training

| AI training parameter | Value |
| --- | --- |
| Quantization | Scalar quantization |
| Loss function | SGCS |
| Learning rate | 0.001 |
| Optimizer | Adam |
| Epoch | 100 |
| Batchsize | 128 |
| Dataset construction | 210,000 200 drop*21 cell*50 UE*1 samples per UE |
| Sample of training set | 199,500 |
| Sample of validation set | 10000 |
| Sample of test set | 10500 |

Table 10, Complexity and memory storage of AI model

|  | CSI generation part | CSI reconstruction part |
| --- | --- | --- |
|  | 240 bit | 240 bit |
| FLOPs | 257.97*106 | 257.97*106 |
| Number of AI/ML model parameters | 10.79*106 | 10.79*106 |

  - Throughput results

We select Rel-16 TypeII codebook as the baseline, and comparing the throughput results of AI CSI with baseline with rank adaptation. As Table 11 shows, the AI based CSI compression outperforms eType2 codebook in average SE, 5% SE and 95% SE. Setting ideal CSI based on SVD as the upper band, AI based CSI enhancement has about 5% performance loss while eType2 codebook has around 23% performance loss in average SE. Setting eType2 codebook as benchmark and AI based CSI enhancement shows around 23% performance gain on average SE.

Table 11, Performance simulation results

| Case | Average SE | 5% SE | 95% SE |
| --- | --- | --- | --- |
| Ideal | 5.920 | 1.310 | 4.268 |
| AI (120bit) | 5.602 | 1.283 | 3.154 |
| eType2 (Param4, 168bit) | 4.545 | 1.105 | 3.100 |

Observation 9: AI based CSI compression shows around 23% performance gain on average SE compared with eType2 codebook.

- Discussion on AI/ML based CSI prediction

In last meeting, the following working assumption were achieved on AI/ML based CSI prediction:

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction Working Assumption For the AI/ML based CSI prediction sub use case, the following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction for the case without generalization/scalability verification - FFS the description and results for generalization/scalability may need a separate table - FFS whether/how to capture the muliptle predicted CSI instances and their mapping to slots Table X. Evaluation results for CSI prediction without model generalization/scalability, [traffic type], [Max rank value], [RU] Source 1; …. AI/ML model description; AL/ML model backbone. [Pre-processing]. [Post-processing]. FLOPs/M. Parameters/M. [Storage /Mbytes]. Input type. Output type. Assumption; UE speed. CSI feedback periodicity. Observation window (number/distance). Prediction window (number/distance). Whether/how to adopt spatial consistency. Dataset size; Train/k. Test/k. Benchmark 1. Intermediate KPI #1 of Benchmark 1. Gain for intermediate KPI#1 over Benchmark 1. Intermediate KPI #2 of Benchmark 1. Gain for intermediate KPI#2 over Benchmark 1. Gain for eventual KPI (Benchmark 1); Mean UPT. 5% UPT. Benchmark 2. Intermediate KPI #1 of Benchmark 2. Gain for intermediate KPI#1 over Benchmark 2. Intermediate KPI #2 of Benchmark 2. Gain for intermediate KPI#2 over Benchmark 2. Gain for eventual KPI (Benchmark 2); Mean UPT. 5% UPT. FFS others.

To study the impact of input sample number and prediction window on prediction accuracy, we conducted the following evaluation cases as listed in Table 12. In case 1, the input sample number is 8 and the prediction number is 1. In case 2, the input sample number is 4 and the prediction number is 1. And for case 3, the input sample number is 8 and the prediction number is 4, the 4 prediction samples are predicted one by one, for example, sample 1~8 are used to predict sample 9, and sample 2~8 with the predicted sample 9 are used to predict sample 10.

The input CSI types is raw channel matrix, and the nearest historical CSI without prediction is selected as benchmark. The evaluation results of AI based raw channel matrix prediction are listed in Table 13. As Table 13 shows, the AI based CSI prediction outperforms the benchmark scheme, the NMSE of benchmark scheme is around 0.55 for case 1 and case 2, the NMSE decreased from -7.8 to -9.4 when the observation window increased from 4 to 8. The prediction accuracy increased with larger observation window and decreased with larger prediction window.

Observation 10: AI based CSI prediction outperforms the benchmark scheme.

Observation 11: The prediction accuracy increased with larger observation window.

Observation 12: The prediction accuracy decreased with larger prediction window.

Table 12: Evaluation cases of AI based CSI prediction

| Case | Observation window |  | Prediction window |  |
| --- | --- | --- | --- | --- |
|  | Number | Distance | Number | Distance |
| 1 | 8 | 5 | 1 | 5 |
| 2 | 4 | 5 | 1 | 5 |
| 3 | 8 | 5 | 4 | 5 |

Table 13: Evaluation results of AI based raw channel matrix prediction

| Case | Benchmark [NMSE(dB)] |  |  |  | AI [NMSE(dB)] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.549 |  |  |  | -9.407 |  |  |  |
| 2 | 0.550 |  |  |  | -7.814 |  |  |  |
| 3 | 1st | 2nd | 3rd | 4th | 1st | 2nd | 3rd | 4th |
|  | 0.550 | 4.269 | 3.874 | 2.024 | -9.213 | -5.097 | -2.701 | -2.049 |

- Conclusions

In this contribution, we present views on evaluation methodology, evaluation metrics and initial evaluation results on AI/ML for CSI feedback enhancement, we have the following observations and proposals:

Observation 1: The generalized AI model does not work well on various CSI feedback payloads, especially for a small number of feedback payloads.

Observation 2: AI/ML model shows good generalization performance in different sizes of subband and subband number.

Observation 3: AI/ML model shows good generalization performance in different scenarios.

Observations 4: When AI model structure is known, separate training can achieve similar performance with joint training.

Observations 5: When different AI backbone is used for CSI generation part and CSI reconstruction part, performance of separate training decreased compared with joint training.

Observation 6: There is around 7% performance loss caused by quantization.

Observation 7: Vector quantization and scalar quantization can achieve similar performance.

Observation 8: The fine-tuning procedure can be used to improve the performance.

Observation 9: AI based CSI compression shows around 23% performance gain on average SE compared with eType2 codebook.

Observation 10: AI based CSI prediction outperforms the benchmark scheme.

Observation 11: The prediction accuracy increased with larger observation window.

Observation 12: The prediction accuracy decreased with larger prediction window.

Proposal 1: The parameter and metric for complexity of AI model training e.g., the training related overhead, the required dataset size for convergence, number of epoch times, batch size and learning rate are captured in the template table.

Proposal 2: AI model structure e.g., the number of layers is provided in the template table at least for case 2 of Type 3 separate training.

Proposal 3: Using separate tables for different training types/cases is supported.

Proposal 4: Adopt Table 1 as template for simulation results collection at least for case 2 of Type 3 separate training.

Proposal 5: RAN1 study pre-processing mechanisms for the input of decoder to improve the AI model generalization performance on various feedback payloads.

Proposal 6: The case of training without quantization and inference without quantization should be evaluated as the upper bound.

- References

[1] Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0, Toulouse, France, 14th – 18th November 2022

[2] Draft Report of 3GPP TSG RAN WG1 #110bis-e v0.1.0, Online, 10th – 19th October 2022