3GPP TSG RAN WG1 #112                                       	       R1-2300570

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.2.3.2

Source:	Xiaomi

Title:                     Potential specification impact on AI/ML for beam management

Document for:	Discussion and Decision

Introduction

In RAN-94 e-meeting, the SI of Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1] was approved. The objective can be seen as below:

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

- Characterize lifecycle management of AI/ML model: e.g.,  model training, model deployment , model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved.

Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This contribution focus on the discussion of sub use case and potential specification impacts of AI/ML for beam management.

Sub use case priority

According to the SID, the sub use case for beam management include beam prediction in spatial domain and beam prediction in time domain. And the agreement archived in RAN1-109 e-meeting [2] can be seen as below.

Agreement

For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

- FFS: details of BM-Case1 and BM-Case2

- FFS: other sub use cases

Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

The following conclusion is archived in RAN1-110b e-meeting [4]

Conclusion

For AI/ML based beam management, RAN1 has no consensus to support on studying any other sub use case in addition to BM-Case1 and BM-Case2

Note: this conclusion is independent of the discussion on the alternatives of AI/ML model inputs for BM-Case1 and BM-Case2

While in spatial domain, the principle can be seen in Figure 1. It means that UE only measures a subset of beam pairs for input to the AI model, and obtain quality of all beam pairs or only the Top-K beam pairs by the output of the AI model. Thus the reference signal overhead, the measurement complexity and latency can be reduced.

Figure 1, Principle of AI based beam prediction in spatial domain

While in time domain, the principle can be seen in Figure 2. It means that the best beam at time T+m can be predicted by AI based on the history information. And the history information may include the beam information in last N periods. And the beam information may contain the quality of the all Tx beams in each period. But this sub use case also includes two schemes. The scheme 1 (Figure 3) is that the periodicity for history measurement instance is same as that of future time instance. For example, there are N+M short periods in a large period, and in each large period, UE performs beam measurement and reports in the first N short periods and predicts best beams in other M periods. The reference signal overhead and UE side measurement complexity can be reduced in the other M periods. The scheme 2 (Figure 4) is that the periodicity for history measurement instance is larger than that of future time instance. For example, UE only perform beam measurement and report with a long history period, and based on the beam information of N long history periods, the best K beams can be predicted for the (N+1)th long period, and the  (N+1)th long period can be divided into M+1 short periods. The beam selection accuracy can be improved in the (N+1)th long period. Based on the analysis above, reference signal overhead and beam selection latency can be reduced by AI based beam prediction in time domain.

Proposal 1: For BM-Case2, support the periodicity of future time instance is same or 1/N of measurement/report instance.

Figure 2, Principle of AI based beam prediction in time domain

Figure 3, Scheme 1 of AI based beam prediction in time domain

Figure 4, Scheme 2 of AI based beam prediction in time domain

Specification impact on AI model input/output

Spatial domain beam prediction

According to RAN1-109 e-meeting [2], RAN1-110 meeting [3] and RAN1-111 meeting [5], the following agreements on spatial domain beam prediction are archived.

Agreement

For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

For the sub use case BM-Case1, support the following alternatives for further study:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The beam patterns of Set A and Set B can be clarified by the companies.

Agreement

Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and  other information

  - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure)

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

  - FFS: details of Beam angle(s)

- FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s))

- Note1: It is up to companies to provide other alternative(s)

- Note2: Beam ID is only used for discussion purpose

- Note3: All the outputs are “nominal” and only for discussion purpose

- Note4: Values of N is up to each company.

- Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side.

- Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

Agreement

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on the following L1 reporting enhancement for AI/ML model inference

- UE to report the measurement results of more than 4 beams in one reporting instance

- Other L1 reporting enhancements can be considered

For inference, we support either at gNB side or at UE side. As for the set A and set B, we prefer to consider set B is a subset of set A with high priority. First, set B is a subset of set A, thus there will be some measured beam pairs with only a part of predicted beam pairs. If set B and set A is different, all beam pairs need to be predicted. In this case, the beam prediction accuracy will be reduced. Second, as for data set collection, only one beam pattern is needed, i.e., narrow beam with set B is a subset of set A. if set B and set A is different, both wide beam pattern and narrow beam pattern will be needed. Which will increase reference signal overhead and overhead for data set collection.

Proposal 2: For spatial domain beam prediction, consider set B is a subset of set A with high priority.

While for the AI model input, we evaluated the performance of Alt 1 and Alt 4 in [3] for spatial domain beam prediction. Based on our evaluation results, we can see scheme 2 and scheme 3 can obtain more performance gain than that of scheme 1. The detail of each scheme can be seen as below:

- Scheme 1: Random selection of beam pairs for L1-RSRP input;

  - Select 64 beam pairs randomly and input their L1-RSRP to AI model for beam prediction.

- Scheme 2: Random selection of beam pairs for L1-RSRP and beam ID input;

  - Select 64 beam pairs randomly and input their L1-RSRP and beam ID to AI model for beam prediction.

- Scheme 3: Always select 64 beam pairs with same beam pair IDs for L1-RSRP input;

  - Select 64 beam pairs with same beam pair IDs and input their L1-RSRP to AI model for beam prediction.

The more input information of scheme 2 compared to scheme 1 is the beam pair ID, based on which the beam prediction accuracy is improved. It means that if scheme 2 is adopted, beam pair ID of each L1-RSRP should be input to the AI model. Thus, if AI model inference is conducted at UE side, gNB need to indicate the Tx beam information of each RS to UE. If AI model inference is conducted at gNB side, UE need to indicate the Rx beam information for each RS ID and L1-RSRP pair. In addition to beam ID, we also support assistance information such as Tx and/or Rx beam shape information in Alt 2.

Proposal 3: Support L1-RSRP and beam (pair) ID as AI/ML model input with high priority for variable set B.

Proposal 4: For spatial domain beam prediction, study how to indicate the Tx beam information, including Tx beam ID/Tx beam shape information of gNB to UE for UE side inference.

Proposal 5: For spatial domain beam prediction, study to report Rx beam information, including Rx beam ID/Rx beam shape information of UE to gNB for gNB side inference.

But for Alt 3, it needs to define a new measurement quantity which will introduce much more additional workload compared to exist L1-RSRP. Thus we prefer to consider it with low priority.

For Tx/Rx beam pair prediction at gNB side, it is possible that some beams of the Top-K predicted beams are not measured by UE for AI model input. Thus UE doesn’t know the Tx beam ID since it is not included in set B. In this case, how does gNB to indicate the Tx beam ID to UE? In fact, the target that gNB indicates the Tx beam ID to UE is to let UE know which Rx beam will be used to perform reception. So if it is Tx/Rx beam pair prediction, it means gNB can predict the Rx beam, gNB can indicate the Rx beam ID instead of Tx beam ID to UE.

Proposal 6: For beam indication of Tx beam being not measured by UE, gNB can indicate the Rx beam ID instead of Tx beam ID to UE in the case of Tx/ Rx beam pair prediction at gNB side.

In addition, the following agreement were archived in RAN1-110b e-meeting:

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam considers the following options

  - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

Agreement

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair considers the following options:

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

From the evaluation results, we also can find that the scheme 3 provides the best performance since the input of schemes 3 is the L1-RSRP with a group of fixed beam pairs. It means that if scheme 3 is adopted, the beam pair ID of the L1-RSRP for input to the AI model should be fixed. While for the gNB Tx beam, it is easy for fixed Tx beam since gNB can only transmit RS for beam measurement with only fixed 2 or 4 Tx beams. But for Rx beam at UE side, the Rx beam information should be indicated to realize the fixed beam pair ID.

Proposal 7: For the case of Tx beam or TxRx beam pair inference with specific Rx, support to indicate Rx beam information to UE for obtaining L1-RSRP input to AI/ML model.

In addition, since different UE may have different capability on the number of Rx beam, it is needed to discuss that whether a common AI model or separate AI models will be trained for different number of Rx beam at UE side. If different AI model is trained for different number of Rx beam, whether the AI model for Rx beam number M can be used for UE with Rx beam number N or not when M < N? It also need to be studied and some potential specification impact on indication of Rx beam number of AI model will be introduced. As evaluated by our contribution [6], we can find that the AI model trained by a large number of Rx beam can provide good generalization capability for inference with less Rx beams

Proposal 8: Support a common AI model for UE with different number of Rx beam.

Both RS ID and L1-RSRP can be reported by L1 beam report in legacy system. And for UE side AI/ML model, it also provides benefits such as RS overhead reduction with additional Rx beam prediction. Thus, we prefer to support Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams as the AI/ML model output with high priority.

Proposal 9: Support Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams as the AI/ML model output with high priority.

In last RAN-1-110b e-meeting, the enhancement on L1-beam report for BM case 1 was discussed and the following agreement was archived.

Agreement

For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW

- The beam(s) that is based on the output of AI/ML model inference

- FFS: Predicted L1-RSRP corresponding to the beam(s)

- FFS: other information

The controversial point is whether to report the predicted L1-RSRP or not. From our point of view, we support to consider L1-RSRP as AI/ML model output based on our evaluation results in [6] since the difference between the ideal L1-RSRP and the predicted L1-RSRP is lower than 0.5 dB. Thus we support to report the predicted L1-RSRP to gNB for reference.

But it is possible that the L1-RSRP for some of the Top-K beams are measured L1-RSRP, if some of them are beams in set B. in order to provide more information to gNB, UE can indicate which L1-RSRP is obtained by measurement and which L1-RSRP is obtained by AI/ML model output.

Proposal 10: Support to report predicted L1-RSRP in the L1-beam report with an indication to let gNB know which L1-RSRP is a predicted L1-RSRP.

Temporal beam prediction

According to RAN1-109 e-meeting [2] and RAN1-110 meeting [3], the following agreements on temporal beam prediction are archived.

Agreement

For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input:

- The value of K is up to companies

Agreement

Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance.

- At least F = 1

- The other value(s) of F is up to companies

Agreement

For the sub use case BM-Case2, further study the following alternatives:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

- Alt.3: Set A and Set B are the same

- Note1: The beam pattern of Set A and Set B can be clarified by the companies.

Agreement

For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact:

- Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals

- Content/type of the collected data

Agreement

In order to facilitate the AI/ML model inference, study the following aspects as a starting point:

- Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting

- Enhanced or new signaling for measurement configuration/triggering

- Signaling of assistance information (if applicable)

- Other aspect(s) is not precluded

The following agreement were archived in RAN1-110b e-meeting:

Agreement

For BM-Case2 with a UE-side AI/ML model, study the potential specification impact   of L1 signaling to report the following information of AI/ML model inference to NW

- The beam(s) of N future time instance(s) that is based on the output of AI/ML model inference

  - FFS: value of N

- FFS: Predicted L1-RSRP corresponding to the beam(s)

- Information about the timestamp corresponding the reported beam(s)

  - FFS: explicit or implicit

- FFS: other information

For inference, we support either at gNB side or at UE side. As for the set A and set B, we prefer to consider set B and set A are the same set, in this case, the beam prediction accuracy will be high. But there will be problem for gNB side inference. Since in existed beam measurement report, without group based beam report, at most 4 beams are reported at each instance. But if only 4 beams are input to the AI model, the beam prediction accuracy will be not acceptable. Thus it is necessary to increase the maximum number of beams in each beam report.

Proposal 11: For temporal beam prediction, consider set B is same as set A with high priority.

In addition, for temporal beam prediction, it is necessary to discuss how to report the predicted beam in future time instance for UE side inference. There are two alternatives. Alt 1 is to report the predicted beam in each future time instance independently. Alt 2 is to report the predicted beam in future time instance(s) and the measurement beams in the last measurement instance together. With Alt 2, the latency can be reduced but the enhancement on beam measurement report should be considered to include beam results of more than one time instance. In addition, for L1-RSRP report of more than one time instance, it is also need to discuss whether to report one absolute value for each time instance or only report one absolute value for all time instances.

Proposal 12: Consider one absolute L1-RSRP for each time instance or one absolute L1-RSRP for all time instance in one beam report including beam reports of more than one time instance for BM-case 2.

In addition, if beam information for more than one time instance can be reported by one beam report, which side decide the number/ periodicity of the time instance?  If the AI/ML model is trained by UE, it is much likely that it is UE to report the number/ periodicity of the time instance. Else, it is gNB to indicate the number/ periodicity of the time instance together with the configuration of parameters of the AI/ML model.

Proposal 13: Consider UE to report the number/ periodicity of the time instance in beam report for BM-case 2.

AI model performance monitoring

According to RAN1-110 meeting [3] and RAN1-111 meeting [5], the following agreements on model monitoring are archived.

Agreement

Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects

- Performance metric(s)

- Benchmark/reference for the performance comparison

- Signaling/configuration/measurement/report for model monitoring, e.g., signaling aspects related to assistance information (if supported), Reference signals

- Other aspect(s) is not precluded

Agreement

Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the necessity and the potential specification impacts from the following aspects:

- UE reporting of beam measurement(s) based on a set of beams indicated by gNB

- Signaling, e.g., RRC-based, L1-based

- Note: Performance and UE complexity, power consumption should be considered

As for AI model performance monitoring, the simplest method is that compare the predicted best K beams with the real best K beams. And in order to obtain the real best K beams, take spatial domain beam prediction as an example, gNB need to transmit reference signals with all Tx beams and UE need to measure all reference signals with all Rx beams. It means gNB need to transmit all beams in set A periodically, semi-persistently or non-periodically for performance monitoring.

Proposal 14: gNB to transmit all beams in set A periodically/semi-persistently/ a-periodically for performance monitoring.

UE side inference

For UE-side AI/ML model, the following agreement was archived in RAN1-110b e-meeting.

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side Model monitoring

  - UE monitors the performance metric(s)

  - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation

- Atl2. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

- Alt3. Hybrid model monitoring

  - UE monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Alt 1, UE makes decision based on performance metric monitoring. But after the decision, it is necessary to report to the gNB since there will be some beam measurement reconfiguration with model selection/activation/ deactivation/switching/fallback operation.

Proposal 15: For UE-side AI/ML model with UE-side model monitoring, support UE to indicate the decision to NW.

Alt 2, based on NW monitors the performance metric, there will be two alternatives for performance metric calculation. Option 1 is UE calculates the performance metric and report to gNB. Option 2 is gNB calculates the performance metric based on UE reporting the beams of predicted Top-K beams and the beams of genie-aided Top-K beams to gNB. But the signaling overhead is larger than Option 1.

Proposal 16: For UE-side AI/ML model with NW-side model monitoring, consider UE to calculate the performance metric and report to NW, or NW to calculate the performance metric based on UE report.

Alt 3, based on UE monitors performance metrics, but when to report the metric to gNB is an important issue. One straightforward way is to support an event-triggered report. It means that if the performance metric is lower than a threshold, UE will report the metric to the gNB. In this case, the threshold can be configured by gNB. Or support periodical report triggered and stopped by gNB.

Proposal 17: For UE-side AI/ML model with hybrid model monitoring, support an event-triggered report of performance metric from UE based on a threshold configured by gNB.

The metrics for evaluate the performance of AI model can be same as the KPI for AI model. i.e., the KPIs listed in the agreement in RAN1-109 e-meeting. For example, take ‘Beam prediction accuracy (%) for Top-1 and/or Top-K beams’ as an example, if it is lower than a threshold, it means the performance of AI model is not acceptable.

Figure 5, performance monitoring for spatial domain beam prediction at UE side

Agreement

- To evaluate the performance of AI/ML in beam management, further study the following KPI options:

  - Beam prediction accuracy related KPIs, may include the following options:

    - Average L1-RSRP difference of Top-1 predicted beam

    - Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

      - Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

    - CDF of L1-RSRP difference for Top-1 predicted beam

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

    - the definition of L1-RSRP difference of Top-1 predicted beam:

      - the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

    - Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

Proposal 18: Threshold of beam prediction accuracy related KPIs can be used for performance monitoring.

While for UE-side AI/ML model, both UE initiated and gNB initiated performance monitoring can be further studied. For UE initiated case, UE can send request to gNB for measurement configuration for performance monitoring. And the request can be indicated through RA procedure or scheduling request. For gNB initiated case, gNB can trigger the performance monitoring by explicit or implicit signaling. For implicit signaling, it can be the measurement configuration for performance monitoring. For explicit signaling, it can be DL MAC CE or DCI.

Proposal 19: Study potential signaling for both NW-side initiated and UE-side initiated performance monitoring for UE-side AI/ML model.

gNB side inference

For NW-side AI/ML model, the following agreement was archived in RAN1-110b e-meeting.

Agreement

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring:

- NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Agreement

Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects

- Beam measurement and report for model monitoring

- Note: This may or may not have specification impact.

While for gNB side inference, when gNB transit all beams in set A, UE need to measure all beam pairs. In addition to report the measurement results of set B for gNB side inference, the real best K beams also need to be reported to gNB for AI model performance monitoring. Then gNB evaluate the performance of AI model by comparing the predicted best K beams and reported real best K beams based on the threshold of beam prediction accuracy related KPIs noted above. If gNB detects the performance of AI model is not acceptable, it can deactivate the AI model or switch to a new AI model.

Proposal 20: For NW-side model monitoring for network-side AI/ML model, support to report both set B and set C, where set B will be used as network-side AI/ML model input, and set C consists of Top-K beams by UE’s measurement of set A.

Since it is NW-side AI/ML model, when to trigger the performance monitoring will be triggered by gNB. How to trigger the performance monitoring and request UE to report the measurements for performance monitoring? It can be triggered explicitly or implicitly. For example, gNB can configure the beam measurement configuration for performance monitoring to trigger it implicitly. Or gNB can configure the beam measurement configuration first and then trigger the performance monitoring by MAC CE or DCI.

Proposal 21: Study potential signaling for NW-side initiated performance monitoring for NW-side AI/ML model.

Figure 5, performance monitoring for spatial domain beam prediction at gNB side

Data collection for AI model training

For data collection for AI/ML model training at UE side, the following agreement was archived in RAN1-111 meeting.

Agreement

Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects.

- Whether and how to initiate data collection

- Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B

- Assistance information from Network to UE (If supported)

- Other aspect(s) is not precluded

For UE side model training, there is no related report. the data collection can be requested by UE and initiated by gNB’s configuration of set A and/or set B. UE can indicate the requested data size and the relationship between set B and set A in the request. If set B is a subset of set A or set B is equal to set A, gNB can configure set A only and indicate the relationship of set B and set A. for example, indicate which RSs in set A belong to set B. If set B is different from set A, gNB need to configure both set A and set B. And also the relationship on beam shapes between set A and set B can be included in the assistance information.

Proposal 22: For data collection for AI/ML model training at UE side, support UE request for gNB’s configuration with indicating the data size and the preferred relationship between set B and set A.

Proposal 23: For data collection for AI/ML model training at UE side, support gNB indicating the relationship between set B and set A, the beam shapes of set B and set A.

While for NW side model training, since there are different relationship between set B and set A. And also there are different alternatives for AI model output, e.g., L1-RSRP of all beams(pairs) or Top-K beams(pairs) ID. The possible combinations are listed in the following 4 cases. And for different combinations the content of the collected data will be different, which can be seen in the Table 1.

- Case 1: set B is different from set A & output is L1-RSRP of all beam (pairs)

- Case 2: set B is a subset of set A (or set B= set A) & output is L1-RSRP of all beam (pairs)

- Case 3: set B is different from set A & output is Top K beam (pairs) ID

- Case 4: set B is a subset of set A (or set B= set A) & output is Top K beam (pairs) ID

Table 1, model input and Label for different cases

| Case | Model input | Label |
| --- | --- | --- |
| Case 1 | L1-RSRP of set B (Rx beam/beam pair ID) | L1-RSRP of set A (Rx beam/beam pair ID) |
| Case 2 | Reuse Label |  |
| Case 3 | L1-RSRP of set B (Rx beam/beam pair ID) | Top K beam (pairs) ID |
| Case 4 |  |  |

According to Table 1, we can see that for case 1, the content of the collected data needs to contain L1-RSRP of set B and L1-RSRP of set A. For case 2, only L1-RSRP of set A is needed and the model input can be decided by gNB based on L1-RSRP of set A. For case 3 and case 4, L1-RSRP of set B should be reported as model input and Top-K beams should be reported as Label.

Proposal 24: Support following options for the content of the collected data for model training for different set B and set A, and different model output.

- Option 1: L1-RSRP of set B and L1-RSRP of set A

- Option 2: L1-RSRP of set A

- Option 3: L1-RSRP of set B and Top-K beams ID

Since the data size for model training will be large than beam report. it is better to report it by RRC or CG grant PUSCH. And also it is better to support more than one data sample in one report, e.g., define a time window for each report or determine the time window for each report based on the time domain resource for measurement report. as for the trigger of the start of the data collection and the stop of it, it can also be indicated by gNB explicitly or implicitly. For example, gNB can trigger the start of data collection by measurement configuration. While for the mechanism of stop the data collection, the possible one is to configure a number of semi-persistent report.

Proposal 25: Support to define a time window for each report to include more than one data sample and configure a number of report to stop the data collection.

Conclusion

In this contribution, we discuss about the sub use case and potential specification impacts of AI/ML for beam management. Based on above discusses, we provide the following proposals.

Proposal 1: For BM-Case2, support the periodicity of future time instance is same or 1/N of measurement/report instance.

Proposal 2: For spatial domain beam prediction, consider set B is a subset of set A with high priority.

Proposal 3: Support L1-RSRP and beam (pair) ID as AI/ML model input with high priority for variable set B.

Proposal 4: For spatial domain beam prediction, study how to indicate the Tx beam information, including Tx beam ID/Tx beam shape information of gNB to UE for UE side inference.

Proposal 5: For spatial domain beam prediction, study to report Rx beam information, including Rx beam ID/Rx beam shape information of UE to gNB for gNB side inference.

Proposal 6: For beam indication of Tx beam being not measured by UE, gNB can indicate the Rx beam ID instead of Tx beam ID to UE in the case of Tx/ Rx beam pair prediction at gNB side.

Proposal 7: For the case of Tx beam or TxRx beam pair inference with specific Rx, support to indicate Rx beam information to UE for obtaining L1-RSRP input to AI/ML model.

Proposal 8: Support a common AI model for UE with different number of Rx beam.

Proposal 9: Support Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams as the AI/ML model output with high priority.

Proposal 10: Support to report predicted L1-RSRP in the L1-beam report with an indication to let gNB know which L1-RSRP is a predicted L1-RSRP.

Proposal 11: For temporal beam prediction, consider set B is same as set A with high priority.

Proposal 12: Consider one absolute L1-RSRP for each time instance or one absolute L1-RSRP for all time instance in one beam report including beam reports of more than one time instance for BM-case 2.

Proposal 13: Consider UE to report the number/ periodicity of the time instance in beam report for BM-case 2.

Proposal 14: gNB to transmit all beams in set A periodically/semi-persistently/ a-periodically for performance monitoring.

Proposal 15: For UE-side AI/ML model with UE-side model monitoring, support UE to indicate the decision to NW.

Proposal 16: For UE-side AI/ML model with NW-side model monitoring, support UE to calculate the performance metric and report to NW.

Proposal 17: For UE-side AI/ML model with hybrid model monitoring, support an event-triggered report of performance metric from UE based on a threshold configured by gNB.

Proposal 18: Threshold of beam prediction accuracy related KPIs can be used for performance monitoring.

Proposal 19: Study potential signaling for both NW-side initiated and UE-side initiated performance monitoring for UE-side AI/ML model.

Proposal 20: For NW-side model monitoring for network-side AI/ML model, support to report both set B and set C, where set B will be used as network-side AI/ML model input, and set C consists of Top-K beams by UE measurement of set A.

Proposal 21: Study potential signaling for NW-side initiated performance monitoring for NW-side AI/ML model.

Proposal 22: For data collection for AI/ML model training at UE side, support UE request for gNB’s configuration with indicating the data size and the preferred relationship between set B and set A.

Proposal 23: For data collection for AI/ML model training at UE side, support gNB indicating the relationship between set B and set A, the beam shapes of set B and set A.

Proposal 24: Support following options for the content of the collected data for model training for different set B and set A, and different model output.

- Option 1: L1-RSRP of set B and L1-RSRP of set A

- Option 2: L1-RSRP of set A

- Option 3: L1-RSRP of set B and Top-K beams ID

Proposal 25: Support to define a time window for each report to include more than one data sample and configure a number of report to stop the data collection.

References

- RP-213599 New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RAN1-109 e-meeting chairman’s notes.

- RAN1-110 meeting chairman’s notes.

- RAN1-110b e-meeting chairman’s notes.

- RAN1-111 meeting chairman’s notes.

- R1-2300569, Evaluation on AI/ML for beam management