3GPP TSG RAN WG1 #112		                                                          R1-2300631

Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on format 4_2 function description. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In section 7.3.1.5, for DCI format 4_0, it describes that DCI format 4_0 is used for the scheduling of PDSCH for broadcast in DL cell, and for DCI format 4_1, it describes that DCI format 4_1 is used for the scheduling of PDSCH for multicast in DL cell. Although DCI format 4_2 is designed for multicast scheduling, in the description of DCI format 4_2, it does not specify its used for scheduling of PDSCH for multicast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that DCI format 4_2 is used for the scheduling of PDSCH for multicast in DL cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause ambiguity that DCI format 4_2 can be used for broadcast of PDSCH scheduling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.3.1.5.2	Format 4_1

DCI format 4_1 is used for the scheduling of PDSCH for multicast in DL cell.

The following information is transmitted by means of the DCI format 4_1 with CRC scrambled by G-RNTI for multicast or G-CS-RNTI configured by MBS-RNTI-SpecificConfig:

-	Frequency domain resource assignment – log2(NRBDL,CFR(NRBDL,CFR+1)2) bits where NRBDL,CFR equals to

-	the size of CORESET 0 if CORESET 0 is configured for the cell; and

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	Reserved bits – 3 bits

7.3.1.5.3	Format 4_2

DCI format 4_2 is used for the scheduling of PDSCH for multicast in DL cell.

The following information is transmitted by means of the DCI format 4_2 with CRC scrambled by G-RNTI for multicast or G-CS-RNTI configured by MBS-RNTI-SpecificConfig:

-	Frequency domain resource assignment – number of bits determined by the following, where NRBDL,CFR is the size of the common frequency resource as configured by higher layer parameter locationAndBandwidthMulticast:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS38.214],

-	log2(NRBDL,CFR(NRBDL,CFR+1)2) bits if only resource allocation type 1 is configured, or

-	maxlog2(NRBDL,CFR(NRBDL,CFR+1)2),NRBG+1 bits if resourceAllocation in pdsch-ConfigMulticast is configured as 'dynamicSwitch'.

-	If resourceAllocation in pdsch-ConfigMulticast is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2(NRBDL,CFR(NRBDL,CFR+1)2) LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter is configured; otherwise I is the number of entries in the default table.