3GPP TSG RAN WG1 #112                                                   R1-2300650

Athens, Greece, February 27th – March 3rd, 2023

Source:	CATT

Title:		Further discussions on unified TCI framework extension for multi-TRP operation

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

Introduction

In this contribution, our views on extending Rel-17 unified TCI to multi-TRP operation as well as multi-panel UL transmission are analyzed, including power control related issues.

Extension for indication of multiple DL/UL TCI states

In RAN1 #111 meeting, the following agreements on unified TCI for multi-TRP have been achieved [1]:

Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH. Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception FFS: Detail design of the new indicator field

In this section, CJT-based PDSCH transmission, TCI state association, TCI state update, beam indication signaling and TRP-specific BFR enhancement are discussed.

CJT-based PDSCH transmission

Based on previous agreement, up to two joint TCI states are supported for CJT-based PDSCH reception. Whether one or two TCI states are applied depends on UE capability. If UE supports two indicated TCI states, three alternatives on QCL assumptions are proposed for down-selection in the last meeting. From the views of some companies, gNB pre-compensation has to be used for CJT, otherwise the delay or frequency offset across TRPs could not be handled by UE. Then some parameters of the QCL assumption for the second TCI state could be removed. Taking Alt2 (i.e. PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state) as an example, frequency pre-compensation is used. TRS in the first indicated joint TCI state is used for Doppler estimation, and TRS in each joint TCI state is used for delay estimation, respectively. This is similar as PDSCH-SFN schemeB. However, for PDSCH-SFN schemeA in Rel-17, pre-compensation is not needed for PDSCH, and two TRP specific TRS are transmitted. With Doppler and delay estimated for each TRP, UE is feasible to compensate the offset across TRPs. In our opinion, CJT-based PDSCH is similar as that of PDSCH-SFN. Therefore, gNB pre-compensation may not be necessary for CJT-based PDSCH transmission. Consequently, QCL-Type A should be configured in each TCI state.

Proposal 1: For CJT-based PDSCH transmission, when two joint TCI states are indicated, PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA (Alt1).

TCI state association

Within the unified TCI framework, when single or multiple TCI states are indicated, UE has to correctly associate each TCI state to corresponding channel resource(s) or transmission repetition(s). Therefore, Rel-18 has to specify the TCI state association with resource and/or time domain repetition. TCI state association for PDCCH has been agreed in the last meeting. The association schemes for the other channels are discussed in the following.

TCI state update for S-DCI based multi-TRP

In Rel-17 multi-TRP, TCI state ID(s) is directly configured for PDCCH/PUCCH. In addition, in Rel-15/16, TCI state ID and SpatialRelationInfo are configured for CSI-RS and SRS, respectively. However, such configuration is not applicable to Rel-18 unified TCI, where association with the indicated TCI states would be configured instead for these channels or reference signals. To define this association, TCI state indicators (the order index of the indicated TCI states, e.g. TCI_ind_0 denotes the first TCI state, TCI_ind_1 denotes the second TCI state,…), which refer to the indicated TCI states, should be introduced in Rel-18.

Proposal 2: To define the association, TCI state indicators referring to the indicated TCI states should be introduced in Rel-18.

In the last meeting, a new indicator field has been introduced in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception. Whether this field is present or not in DCI format 1_1/1_2 is configured by RRC. Namely, this field is used to indicate single TRP or multi-TRP transmission for the scheduled PDSCH, which could be composed of two bits. “codepoint = 00” means single TRP transmission and PDSCH is associated with the first indicated TCI state. “codepoint = 01” means single TRP transmission and PDSCH is associated with the second indicated TCI state. “codepoint = 10” means multi-TRP transmission and PDSCH is associated with both indicated TCI states. For multi-TRP transmission case (i,e. “codepoint = 10”), the association rules have been specified in Rel-16. For SDM-1a, TCI state is associated with DMRS CDM group. For FDM-2a/FDM-2b, there is association between TCI state and each allocated frequency resource. In addition, for TDM-3/TDM-4, the association between TCI state and each time domain repetition is specified. These association rules could be reused in Rel-18.

Proposal 3: For PDSCH reception in S-DCI based multi-TRP, the RRC configured indicator field is composed of two bits. “codepoint = 00” means single TRP transmission and PDSCH is associated with the first indicated TCI state. “codepoint = 01” means single TRP transmission and PDSCH is associated with the second indicated TCI state. “codepoint = 10” means multi-TRP transmission and PDSCH is associated with both indicated TCI states.

Proposal 4: For multi-TRP PDSCH transmission (indicated by the new indicator field), the Rel-16 association rules （i.e. TCI state is associated with CDM group/frequency resource/time domain repetition）could be reused.

If PDSCH is scheduled by DCI format 1_0, single TRP transmission is expected. Since there is no TCI field in DCI format 1_0, a default TCI state should be defined for PDSCH reception. In our opinion, the first indicated TCI state could be defined as the default TCI state. When the indicator field is not present in DCI format 1_1/1_2, according to the agreement, default indicated TCI state(s) are also needed for PDSCH reception. To avoid spec complexity, a common default TCI state, i.e. the first indicated TCI state, is preferred for both cases.

Proposal 5: For PDSCH reception scheduled by DCI format 1_1/1_2 without the new indicator field or by DCI format 1_0, the first indicated TCI state is used as the default TCI state for PDSCH reception.

It has been agreed that for PUSCH transmission scheduled by DCI format 0_1/0_2, an indicator field in DCI format 0_1/0_2 is used to inform which TCI state(s) shall be applied to PUSCH transmission. In Rel-17 PUSCH multi-TRP, an SRS resource set indicator with 2bits is employed for single TRP and multi-TRP switching, which is shown in Table 1. This field could be re-defined as the TCI state association. Following the same principle in Table 1, “codepoint = 00” means single TRP transmission and PUSCH is associated with the first indicated TCI state. “codepoint = 01” means single TRP transmission and PUSCH is associated with the second indicated TCI state. “codepoint = 10” and “codepoint = 11” mean multi-TRP transmission and PUSCH repetition with the first indicated TCI state and the second TCI state order. Such association is applicable to DG-PUSCH and Type 2 CG PUSCH. For Type 1 CG PUSCH, similar as PDCCH, TCI state association is defined for each Type 1 CG PUSCH configuration.

Table 1: SRS resource set indication

| Bit field mapped to index | SRS resource set indication |
| --- | --- |
| 0 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved. |
| 1 | SRS resource indicator field and Precoding information and number of layers field are associated with the second SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved. |
| 2 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set. |
| 3 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set. |
| NOTE 1: The first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'nonCodeBook' if txConfig=nonCodebook or 'codebook' if txConfig=codebook. When only one SRS resource set is configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codebook' or 'nonCodeBook' respectively, the first SRS resource set is the SRS resource set. The association of the first and second SRS resource sets to PUSCH repetitions for each bit field index value is as defined in Clause 6.1.2.1 of [6, TS 38.214]. NOTE 2: For DCI format 0_2, the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 are composed of the first NSRS, 0_2 SRS resources together with other configurations in the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codebook' or 'nonCodeBook', respectively, except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList'. |  |

Proposal 6: For PUSCH transmission scheduled by DCI format 0_1/0_2, the SRS resource set indicator with 2bits field in DCI format 0_1/0_2 could be re-defined as the TCI state association.

Proposal 7: For Type 1 CG PUSCH, TCI state association is configured for each Type 1 CG PUSCH configuration.

TCI state update for M-DCI based multi-TRP

It has been agreed that the indicated TCI state specific to a CORESETPoolIndex value is applied to PDCCH and its scheduled/activated PDSCH on a CORESET that is associated with the same CORESETPoolIndex value. For PUCCH, there is a case that PUCCH transmission is scheduled by a DCI with DL assignment. For the other cases, there is no explicit or implicit association with CORESETPoolIndex. For the former case, PUCCH transmission could follow the TCI state of the scheduling DCI. For the other cases, referring to the TCI association mechanism mentioned above for S-DCI based PDCCH/PUCCH, CORESETPoolIndex could be configured for each resource. In our opinion, it seems not necessary to introduce two different association rules for PUCCH transmission. Therefore, CORESETPoolIndex configured for each PUCCH resource is preferred.

Proposal 8: For M-DCI based multi-TRP, for PUCCH, CORESETPoolIndex could be configured for each resource.

For Type 1 CG PUSCH, according to the agreement of the last meeting, when two SRS resource sets are configured，SRS_resource_set_index would be configured in the Type 1 CG configuration. SRS_resource_set_index indicates one SRS resource set, which is associated with a CORESETPoolIndex value. Therefore, in this case, Type 1 CG PUSCH is actually associated with CORESETPoolIndex. The indicated TCI state associated with the same CORESETPoolIndex should be applied to PUSCH transmission. When only one SRS resource set is configured, there is no explicit or implicit association with CORESETPoolIndex. CORESETPoolIndex could be configured in the SRS resource set.

Proposal 9: For M-DCI based multi-TRP, for Type 1 CG PUSCH, when two SRS resource sets are configured, the indicated TCI state specific to a CORESETPoolIndex value that is indirectly associated with the Type 1 CG PUSCH would be applied to PUSCH transmission; when only one SRS resource set is configured, CORESETPoolIndex could be configured in the SRS resource set.

TCI state update for reference signals

In Rel-17 unified TCI framework, reference signals (e.g. CSI-RS, SRS) could follow the indicated TCI state(s) or not depending on the configuration. If not, Rel-15/16 TCI state update signaling/configuration design(s) are reused. The same rule should be extended to Rel-18. If RS is configured to share the indicated TCI state(s), the association of the TCI state(s) to the RS has to be designed.

Same as Rel-17, P/SP CSI-RS can never share the indicated TCI states in Rel-18, and the legacy RRC/MAC-CE signalling mechanism is always used for TCI state update. For aperiodic CSI-RS, the association could be achieved by its triggering DCI. Namely, aperiodic CSI-RS could follow the TCI state of the CORESET carrying the triggering DCI. This mechanism is also applied to aperiodic SRS. There may be the case that the triggering DCI is transmitted by SFN, where two TCI states are associated with the CORESET. Then one of the TCI states should be specified for aperiodic CSI-RS/SRS. However, this scheme has some limitation on the triggering DCI. For example, if two SRS resource sets aiming at different TRPs are triggered, different DCIs with different associated TCI states have to be used. Another alternative for the association between TCI state and aperiodic CSI-RS/SRS is to configure a TCI state indicator for each resource/resource set. In our opinion, either alternative could be considered in Rel-18. If P/SP SRS is configured to follow the indicated TCI state(s), each SRS resource could be associated with one indicated TCI state. The association is implemented by TCI state indicators.

Proposal 10: For aperiodic CSI-RS/SRS, if configured to follow the indicated TCI state, the following two alternatives could be considerred:

Alt-1: Aperiodic CSI-RS/SRS could follow the TCI state of the CORESET carrying the triggering DCI. If the triggering DCI is transmitted by SFN, one of the TCI states should be specified.

Alt-2: A TCI state indicator is configured by RRC for each CSI-RS/SRS resource/resource set.

Besides, as mentioned above, for M-DCI based multi-TRP, when two SRS resource sets are configured for CB/NCB, a SRS resource set is associated with a CORESETPoolIndex value. In this case, the indicated TCI state specific to the CORESETPoolIndex value should be applied to SRS transmission.

Proposal 11: For M-DCI based multi-TRP, when two SRS resource sets are configured for CB/NCB, the indicated TCI state specific to the CORESETPoolIndex value, which is associated with the SRS resource set, should be applied to SRS transmission.

TCI state update and activation

Based on the agreement of the last meeting, for S-DCI based multi-TRP, the existing TCI field in DCI format 1_1/1_2 can indicate TCI state(s) for one or both of the two TRPs. The switching between single TRP and multi-TRP does not depend on the number of TCI states indicated in DCI format 1_1/1_2. As discussed above, when two joint/DL TCI states are indicated, PDSCH transmission could either be single TRP or multi-TRP, depending on the value of the new TCI indicator field in the DCI. Similarly, for PUSCH, dynamic switching is also supported according to the “SRS resource set” field in DCI format 0_1/0_2. Therefore, when TCI states for both of the two TRPs are indicated, each channel could determine its single TRP or multi-TRP transmission separately. However, there is an issue when TCI state(s) for only one TRP is indicated. UE needs to discriminate the TCI state(s) of which TRP should be updated. As a candidate scheme, an additional field could be introduced in the activation MAC-CE. For example, additional flag is introduced for each codepoint in the activation MAC-CE to indicate TCI state update. If Rel-17 MAC-CE is used as shown in Figure 1, 1 bit flag is added to Pi.

Figure 1：Rel-17 TCI States Activation/Deactivation for UE-specific PDSCH MAC CE

Proposal 12: For S-DCI based multi-TRP, an additional field could be introduced in the activation MAC-CE to indicate partial TCI state update.

Enhancement to TRP-specific BFR

TRP-specific BFR has been supported in Rel-17, which should be extended to unified TCI. In Rel-17, both explicit BFD RS and implicit BFD RS have been specified. For explicit BFD RS, two BFD RS sets (each corresponding to one TRP) could be configured for either S-DCI or M-DCI. However, for implicit BFD RS, two BFD RS sets are only applicable to M-DCI. Since for M-DCI, each CORESETPoolIndex configured by RRC corresponds to one TRP. Then, the RS within the TCI states for those CORESETs associated with the same CORESETPoolIndex could be used as BFD RS for one TRP. But there is no such TRP-ID like parameter for S-DCI. Therefore it is difficult for UE to discriminate two BFD RS sets. Considering unified TCI framework, as mentioned above, TCI state indicators may be introduced for S-DCI. In this way, implicit TRP-specific BFD RS is also feasible for S-DCI in Rel-18.

Proposal 13: By configuring TCI state indicator for each CORESET, implicit TRP-specific BFD RS could be supported for S-DCI in Rel-18.

In Rel-17 unified TCI framework, after X symbols from the UE receives the BFRR from NW, the UE assumes the same QCL parameter as the ones associated with the index q new for DL receptions. And the UE uses the same UL spatial filter as the one associated with the index q new or the last PRACH transmission for UL transmissions. Considering Rel-17 multi-TRP, TRP-specific NBI RS set is configured and q new is reported using BFR MAC-CE associated witha BFD-RS set (i.e. a TRP). For Rel-18 UTCI, the beam resetting mechanism of Rel-17 could be extended to TRP-specific beam resetting. Namely, if UE receives the BFRR corresponding to a BFR MAC-CE associated with a BFD-RS set, the DL and UL channels/RSs of the TRP sharing the indicated TCI state(s) are all reset to the corresponding q new , if reported, in the BFR MAC-CE.

Proposal 14: If UE receives the BFRR corresponding to a BFR MAC-CE associated with a BFD-RS set, the DL and UL channels/RSs of the TRP sharing the indicated TCI state(s) are all reset to the corresponding

q new, if reported, in the BFR MAC-CE.

Simultaneous multi-panel UL transmission

Association of panel and TCI state

In Rel-17, UE-initiated UL panel activation and selection is supported for fast uplink panel selection, via UE reporting a list of UE capability values along with the pairs of SSBRI/CRI and L1 RSRP/L1 SINR in a beam reporting instant. Each UE capability value comprises the maximum supported number of SRS antenna ports, and the maximum supported numbers of SRS antenna ports of any two UE capability values are different.

For STxMP UL transmission in Rel-18, up to 2 TRPs and up to 2 panels is considered. When two TCI states are selected, the two TCI states may correspond to same or different panels. In terms of RF functionality, in most cases, a UE panel is not able to generate two analog beams simultaneously. Therefore indicating two beams corresponding to the same panel for simultaneous UL transmission should be avoided. That means that for the configuration of unified TCI for simultaneous UL transmission, gNB has to know the correspondence between panels and TCI states.

Proposal 15: For STxMP UL transmission, how to facilitate gNB to know the correspondence between panels and TCI states should be studied.

There are various approaches to identify the correspondence between panels and TCI states.

One approach is to use UE capability value to indicate the association between SSB resources/CSI-RS resources and panels as that for Rel-17 fast panel selection. Note that in Rel-17, two different UE capability values must correspond to two different maximum supported numbers of SRS antenna ports. That is, if a UE receives two CSI-RS resources with two panels, respectively, and the two panels correspond to same maximum supported number of SRS antenna ports, same UE capability value would be reported. Then gNB does not know whether the two CSI-RS resources correspond to same panel or different panels. The two UL panels deployed by UE may correspond to same or different maximum number of SRS antenna ports, and same or different maximum number of MIMO layers. Therefore allowing UE capability values for two panels correspond to same or different maximum number of SRS antenna ports, and same or different maximum supported number of MIMO layers should be supported.

Proposal 16: For STxMP UL transmission, consider enhancing UE capability values to represent panels. The following two enhancements can be considered:

- Two different UE capability values correspond to same or different maximum supported number of SRS antenna ports are supported;

- Using UE capability value to indicate maximum supported number of MIMO layers.

Another approach is using group-based beam reporting to faciliate gNB to know which beams can be used for UL transmission simultaneously. UE can report two groups of beams, with each group corresponds to a panel. Then any two SSB resources/CSI-RS resources in different beam groups can be used for UL transmission simultaneously.

Proposal 17: For STxMP UL transmission, group-based beam reporting is considered, with each group corresponds to a panel.

TCI state association for PUSCH

Based on the agreements on Rel-18 S-DCI based STxMP PUSCH in previous meetings, Rel-18 DCI-based STxMP PUSCH transmission schemes and Rel-17 S-DCI based multi-TRP TDM scheme have a lot in common:

- 2 panels/TRPs are considered;

- Two SRS resource sets for CB or NCB are configured;

- For codebook-based PUSCH, DCI indicates two TPMI fields;

- For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each field indicates SRS resource(s) for each SRS resource set separately;

- The DCI field “SRS resource set indicator” is used to indicate the switching between the STxMP/multi-TRP scheme and sTRP transmission.

In RAN1 #111 meeting, the following agreement on unified TCI framework extension for S-DCI based MTRP was achieved[1]:

Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

On TCI state indication for PUSCH, it is our view that same scheme can be used for Rel-18 DCI-based STxMP PUSCH transmission schemes and Rel-17 S-DCI based multi-TRP TDM scheme, i.e., the conclusions on unified TCI framework extension for S-DCI based MTRP shall be extended to Rel-18 S-DCI based STxMP PUSCH.

Proposal 18: On indicating the associated joint/UL TCI state(s) of an S-DCI based STxMP PUSCH transmission, the same mechanism as that for Rel-17 S-DCI based multi-TRP TDM scheme is used.

In RAN1#111 meeting, the following agreement was achieved on M-DCI based STxMP PUSCH[1]:

Agreement - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - Regarding how to interpret the SRI/TPMI field in DCI: - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set

According to the agreement, for M-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, the first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1, and the PUSCH is associated with SRS resource set with the same value of coresetPoolIndex. On interpreting SRI/TPMI for M-DCI based STxMP, for DG-PUSCH and Type 2 CG PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling/activation DCI format 0_1 or 0_2 is received, and for Type 1 CG PUSCH, the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set associated with SRS_resource_set_index value configured in RRC in ConfiguredGrantConfig. For a PUSCH for M-DCI based STxMP, it is natural to apply the indicated joint/DL TCI state specific to the coresetPoolIndex value associated with the SRS resource set for determining its SRI/TPMI.

Proposal 19: For an M-DCI based STxMP PUSCH transmission, the UE applies the indicated joint/DL TCI state specific to the coresetPoolIndex value associated with the SRS resource set for determining its SRI/TPMI.

TCI state association for PUCCH

In RAN1#111 meeting, the following agreements have been achieved on S-DCI based mutli-TRP PUCCH transmission:

Agreement Support the SFN scheme for single-DCI based STxMP PUCCH transmission Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design

Similar as S-DCI based multi-TRP TDM scheme, for Rel-18 S-DCI based STxMP PUCCH transmission, a PUCCH resource can be indicated with one or two joint/UL TCI state(s). On informing the associated joint/UL TCI state(s) of a PUCCH resource, we prefer to use a common solution in Rel-18 for S-DCI based PUCCH transmission schemes, i.e., same mechanism as that for multi-TRP PUCCH repetition schemes is used for Rel-18 S-DCI based STxMP PUCCH transmission.

Proposal 20: On indicating the associated joint/UL TCI state(s) of an S-DCI based STxMP PUCCH trasnsmission, the same mechanism as that for S-DCI based MTRP PUCCH transmission is used.

Power control enhancement for Multi-TRP transmission

In RAN1 #109 e-meeting, the following agreement on power control enhancement for Multi-TRP transmission were achieved [2]:

Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

In RAN1 #109 e-meeting, it was agreed that the power control parameter settings for NR Rel-17 TDM PUSCH/PUCCH repetition can be determined by the indicated joint or UL TCI state associated to the corresponding PUSCH/PUCCH transmission occasion. One issue to be further discussed is how to extend it to NR Rel-18 MTRP scheme(s) with STxMP, including S-DCI and M-DCI based MTRP operation. Regarding this issue, there is the following proposal in the discussion after RAN1 #111 meeting,

Proposal 4.1: On unified TCI framework extension, if an indicated joint/UL TCI state applies to a PUSCH/PUCCH/SRS transmission occasion(s)/port(s), the UE should apply the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS associated the indicated joint/UL TCI state to the PUSCH/PUCCH/SRS transmission occasion(s)/port(s)

For STxMP transmission, since the transmission power of each panel might be different and the maximum transmission power for each panel is limited, panel-specific power control should be considered. Suppose UE has two panels, for once transmission, when two TCI states are indicated to UE to indicate two PUSCH/PUCCH transmission occasions to the same/different TRPs, UE can determine two individual Tx PC parameter settings as well as PL-RS accordingly and use it to perform open-loop power control of each transmission.

Proposal 21: For Rel-18 STxMP, if an indicated joint/UL TCI state applies to a PUSCH/PUCCH/SRS transmission occasion(s)/port(s), the UE should apply the UL PC parameter setting and the PL-RS associated the indicated joint/UL TCI state to the PUSCH/PUCCH/SRS transmission occasions(s)/port(s).

For NR Rel-18 STxMP transmission, considering each panel has its own maximum transmission power, and the total transmission power of a UE is also limited. Power control mechanisms need to be considered for these limitations. Suppose UE has two panels, the maximum transmission power for each panel is Pcmax1, Pcmax2. The maximum total transmission power of the UE is Pcmax which is configured per CC[3]. Basically, power control mechanism of the following cases should be studied:

Case1: P1 + P2 <= Pcmax

Case2: P1 + P2  > Pcmax

where P1 and P2 represents the transmission power of panel 1 and panel 2 determined from equation (1) provided by [3]  respectively, with Pcmax replaced by Pcmax1 and Pcmax2 in the calculation.

(1)

For Case 2, how to determine actual transmission power of panel 1 and panel 2 on the basis of P1 and P2 to meet the requirement of total transmission power limitation need to be further studied.

Figure 3: Power restriction cases in NR Rel-18 uplink simultaneous transmission

For Case1, only power control of per-panel power limitation need to be considered, whether the current framework(i.e. equation (1) to determine transmission power of PUSCH transmission) can be extended to the case of per-panel power limitation needs to be further studied. In detail, the definition of Pcmax in equation (1) is a cell- specific parameter, it is determined from SIB parameter and UE power class limitation. If per-panel power limitation is considered, Pcmax,i for panel i shall be used. Pcmax,i  can be obtained by UE capability reporting and RRC configuration.

For Case 2, power control schemes considering both per-panel power limitation and total transmission power limitation need to be considered. Suppose UE has two panels, the maximum transmission power for panel 1 and panel is Pcmax1 = 23dBm, Pcmax2 = 23dBm. The maximum total transmission power of the UE Pcmax = 20dBm. For once transmission, UE determines its real PUSCH transmission power P1 = 21dBm and P2 = 22dBm individually according to equation (1) with Pcmax replaced by Pcmax1 and Pcmax2 respectively. In addition, considering Pcmax is limited by 20dBm and P1 + P2  > Pcmax, power control mechanism, e.g. power back-off mechanism can be used to reduce the transmission power of both TRPs or one of the TRPs, detailed power back-off mechanisms can be further studied.

Proposal 22: In order to support TRP-specific UL power control in sDCI-mTRP operation. The case of Per-panel power limitation and/or total power limitation should be considered. For the case of total power limitation, some power back-off mechanisms can be further studied. For the case of per-panel power limitation, the formula in TS38.213 to determine PUSCH transmission power needs to be further refined.

Conclusions

In this contribution, we discuss extending Rel-17 unified TCI to multi-TRP use cases and multi-panel UL transmission. The power control related issues are also analyzed. We have the following proposals:

Proposal 1: For CJT-based PDSCH transmission, when two joint TCI states are indicated, PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA (Alt1).

Proposal 2: To define the association, TCI state indicators referring to the indicated TCI states should be introduced in Rel-18.

Proposal 3: For PDSCH reception in S-DCI based multi-TRP, the RRC configured indicator field is composed of two bits. “codepoint = 00” means single TRP transmission and PDSCH is associated with the first indicated TCI state. “codepoint = 01” means single TRP transmission and PDSCH is associated with the second indicated TCI state. “codepoint = 10” means multi-TRP transmission and PDSCH is associated with both indicated TCI states.

Proposal 4: For multi-TRP PDSCH transmission (indicated by the new indicator field), the Rel-16 association rules （i.e. TCI state is associated with CDM group/frequency resource/time domain repetition）could be reused.

Proposal 5: For PDSCH reception scheduled by DCI format 1_1/1_2 without the new indicator field or by DCI format 1_0, the first indicated TCI state is used as the default TCI state for PDSCH reception.

Proposal 6: For PUSCH transmission scheduled by DCI format 0_1/0_2, the SRS resource set indicator with 2bits field in DCI format 0_1/0_2 could be re-defined as the TCI state association.

Proposal 7: For Type 1 CG PUSCH, TCI state association is configured for each Type 1 CG PUSCH configuration.

Proposal 8: For M-DCI based multi-TRP, for PUCCH, CORESETPoolIndex could be configured for each resource.

Proposal 9: For M-DCI based multi-TRP, for Type 1 CG PUSCH, when two SRS resource sets are configured, the indicated TCI state specific to a CORESETPoolIndex value that is indirectly associated with the Type 1 CG PUSCH would be applied to PUSCH transmission; when only one SRS resource set is configured, CORESETPoolIndex could be configured in the SRS resource set.

Proposal 10: For aperiodic CSI-RS/SRS, if configured to follow the indicated TCI state, the following two alternatives could be considerred:

Alt-1: Aperiodic CSI-RS/SRS could follow the TCI state of the CORESET carrying the triggering DCI. If the triggering DCI is transmitted by SFN, one of the TCI states should be specified.

Alt-2: A TCI state indicator is configured by RRC for each CSI-RS/SRS resource/resource set.

Proposal 11: For M-DCI based multi-TRP, when two SRS resource sets are configured for CB/NCB, the indicated TCI state specific to the CORESETPoolIndex value, which is associated with the SRS resource set, should be applied to SRS transmission.

Proposal 12: For S-DCI based multi-TRP, an additional field could be introduced in the activation MAC-CE to indicate partial TCI state update.

Proposal 13: By configuring TCI state indicator for each CORESET, implicit TRP-specific BFD RS could be supported for S-DCI in Rel-18.

Proposal 14: If UE receives the BFRR corresponding to a BFR MAC-CE associated with a BFD-RS set, the DL and UL channels/RSs of the TRP sharing the indicated TCI state(s) are all reset to the corresponding

q new, if reported, in the BFR MAC-CE.

Proposal 15: For STxMP UL transmission, how to facilitate gNB to know the correspondence between panels and TCI states should be studied.

Proposal 16: For STxMP UL transmission, consider enhancing UE capability values to represent panels. The following two enhancements can be considered:

- Two different UE capability values correspond to same or different maximum supported number of SRS antenna ports are supported;

- Using UE capability value to indicate maximum supported number of MIMO layers.

Proposal 17: For STxMP UL transmission, group-based beam reporting is considered, with each group corresponds to a panel.

Proposal 18: On indicating the associated joint/UL TCI state(s) of an S-DCI based STxMP PUSCH transmission, the same mechanism as that for Rel-17 S-DCI based multi-TRP TDM scheme is used.

Proposal 19: For an M-DCI based STxMP PUSCH transmission, the UE applies the indicated joint/DL TCI state specific to the coresetPoolIndex value associated with the SRS resource set for determining its SRI/TPMI.

Proposal 20: On indicating the associated joint/UL TCI state(s) of an S-DCI based STxMP PUCCH trasnsmission, the same mechanism as that for S-DCI based MTRP PUCCH transmission is used.

Proposal 21: For Rel-18 STxMP, if an indicated joint/UL TCI state applies to a PUSCH/PUCCH/SRS transmission occasion(s)/port(s), the UE should apply the UL PC parameter setting and the PL-RS associated the indicated joint/UL TCI state to the PUSCH/PUCCH/SRS transmission occasions(s)/port(s).

Proposal 22: In order to support TRP-specific UL power control in sDCI-mTRP operation. The case of Per-panel power limitation and/or total power limitation should be considered. For the case of total power limitation, some power back-off mechanisms can be further studied. For the case of per-panel power limitation, the formula in TS38.213 to determine PUSCH transmission power needs to be further refined.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109.

- 3GPP RAN1, TS 38.213 V17.0.