3GPP TSG-RAN WG1 #112                                              R1-2300652

Athens, Greece, February 27th – March 3rd, 2023

Source: 	CATT

Title:	Discussion on CSI enhancement

Agenda Item:	9.1.2

Document for:	Discussion and Decision

Introduction

In RAN1#109 e-meeting, it was agreed to specify the following three features of CSI enhancement for medium/high mobility and coherent-JT[1]:

Agreement For Rel-18 CSI enhancements, proceed to support and specify the following features (the previously agreed work scopes apply): - Type-II codebook refinement for CJT mTRP - Type-II codebook refinement for high/medium UE velocities exploiting time-domain correlation/Doppler-domain information - UE reporting of time-domain channel properties (TDCP) measured via CSI-RS for tracking - The use case of aiding gNB-side CSI prediction is to be confirmed in RAN1#110

In this contribution, further details on CSI enhancement for high/medium mobility and coherent-JT will be discussed.

CSI reporting enhancement for high/medium UE velocities

Refinement to Rel-16/17 Type II codebook

Bitmap for NZC locations

Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support the following: - Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector - The number of selected DD basis vectors is denoted as Q - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors. FFS: Further overhead reduction on bitmap(s) FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector Proposal 2.E.2: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt2. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design and further compressed using source-coding (e.g Huffman code) - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt3B: A single 2-dimensional bitmap of size 2LQ to report the selected S pairs of SD components and DD basis vector and a single 2-dimensional bitmap of size MS for indicating the location of the NZCs, where each row corresponds to a selected FD basis vector and each column corresponds to one of the selected S pairs of SD component and DD basis vector. - Alt3C: A single 2-dimensional bitmap of size 2LM to report the selected S pairs of SD component and FD basis vector and a single 2-dimensional bitmap of size QS for indicating the location of the NZCs, where each row corresponds to a selected DD basis vector and each column corresponds to one of the selected S pairs of SD component and FD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L - s1-s0+ mint1-t0, Q - t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively.

As agreed in RAN1#111 meeting[2], Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs. Several alternatives for the bitmap design were identified and discussed. As there is no restrictions on the selection of NZCs, Alt1 can achieve the best performance, but the bitmap overhead is large with 2LMQ bits for each layer. The performance and overhead tradeoff should be considered while designing the bitmap.

Three schemes (Alt2/Alt3/Alt4) are proposed to reduce bitmap overhead in the previous meetings. Alt3 is similar to legacy Rel-16 bitmap for indicating the locations of the NZCs. We evaluate the performance of three sub-alternatives of Alt3 compared with Alt1. System level evaluations are performed under two scenarios. One is at 10km/h UE velocities with N4=8 and Q=3, the other is at 30km/h UE velocities with N4=4 and Q=2. The parameter  L=4 and M=4. Other simulation assumptions are shown in Table A-1 in Appendix.

Figure 1 Performance of Alt3A/Alt3B/Alt3C compared with Alt1

The average throughput and bitmap overhead are shown in Figure 1. Based on the simulation results, it is observed that Alt3A has best performance compared with Alt3B and Alt3C under same bitmap overhead. Therefore, Alt3A is preferred in Alt3X families.

Proposal-1:

- Support Alt3A for indicating the locations of the NZCs.

The value of S needs to be carefully determined considering the tradeoff between performance and overhead. It can be fixed or RRC configured value. Larger S value can achieve better performance but introduce higher bitmap overhead. System level simulation is performed to compare the average throughput of Alt3A with Alt1 under different S value. The simulation results are shown in Table 1. When Q=2, the bitmap overhead of Alt1 is 64bits. The bitmap overhead of Alt3A with S=4 is 40bits which incurs 37.5% overhead reduction, and the average throughput has only 0.58% loss compared with Alt1. Therefore, S=4 is enough for Q=2. When Q=3, the bitmap overhead of Alt1 is 96bits. The bitmap overhead of Alt3A with S=6 is 60bits which incurs 37.5% overhead reduction, and the average throughput has only 1.3% loss compared with Alt1. Therefore, S=6 can be used when Q=3. As different S value is needed for different Q and M value under different scenarios, we prefer that S value is configured by gNB.

Table 1 Performance of Alt3A compared with Alt1 and different S

| L=4 M=4 | Q=3,N4=8,10km/h |  | Q=2,N4=4,30km/h |  |
| --- | --- | --- | --- | --- |
|  | Bitmap overhead MQ+S*2L | Average throughput gain of Alt3A compared with Alt1 | Bitmap overhead MQ+S*2L | Average throughput gain of Alt3A compared with Alt1 |
| S=2 | - | - | 24 | -2.49% |
| S=3 | 36 | -3.48% | 32 | -1.24% |
| S=4 | 44 | -2.45% | 40 | -0.58% |
| S=5 | 52 | -1.82% | 48 | -0.42% |
| S=6 | 60 | -1.30% | 56 | -0.28% |
| S=7 | 68 | -0.97% | - | - |
| S=8 | 78 | -0.67% | - | - |

Proposal-2:

- Value S is configured by gNB for Alt3A.

CQI reporting

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives: - Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices - Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Alt2B. The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported - If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots)

In RAN1#111 meeting, the time instance and/or PMI(s) in which a CQI is associated with and the number of CQI(s) included in a CSI report were discussed and there was no consensus. Regarding the time instance and/or PMI(s) in which a CQI is associated with, TS 38.214 has the following statement: “CQI shall be calculated conditioned on the reported PMI, RI”. This implies that the CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices. Therefore, Alt2A and Alt2B should be precluded first.

Figure 2 Three CQI reporting schemes

System level simulation is performed to compare the performance among the three CQI schemes shown in Figure 2. AR (autoregressive) model is used for UE-side channel prediction.

- Alt1: One CQI is included in a CSI report, and it is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices.

- Alt1B: Two CQIs are included in a CSI report, and each CQI is associated with WCSI/2 and N4/2 W2 matrices.

- Alt1C: Four CQIs are included in a CSI report, and each CQI is associated with WCSI/4 and N4/4 W2 matrices.

Figure 3 SLS results for three CQI schemes

The SLS results are shown in Figure 3. It is evaluated under two scenarios. One is at 10km/h UE velocities with N4=8, the other is at 30km/h UE velocities with N4=4. Other simulation assumptions for the results are shown in Table A-1 in Appendix. Based on the simulation results, there is almost no performance gain of Alt1C (4 CQIs) over Alt1B (2 CQIs). Comparing Alt1B (2 CQIs) and Alt1 (1 CQI), it is observed that Alt1B (2 CQIs) has marginal at 10km/h UE velocity and some gain at 30km/h UE velocity. Therefore, we prefer one CQI or two CQI for each sub-band in a CSI report.

Observation-1:

- 2 CQIs in a CSI report have some performance gain compared with 1 CQI.

- More than 2 CQIs in a CSI report have marginal performance gain compared with 2 CQIs.

Proposal-3:

- Support at most 2 CQIs for each sub-band in a CSI report which is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices.

Codebook Parameters

Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, at least Q=2 is supported. - FFS: Whether Q=3 and/or Q=4 are also supported as other candidate value(s), as well as the supported Parameter Combination(s) Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding parameter Q, decide in RAN1#112 whether to support the additional values of 3 and/or 4

In RAN1#111 meeting, the parameter Q=2 is supported. Further study is needed to determine whether to support the additional values of 3 and/or 4. System level simulation is performed to compare the performance of Q=3 and Q=4 value with Q=2.

Figure 4 SLS results for three Q value

The SLS results are shown in Figure 4. It is evaluated at 10km/h and 30km/h UE velocities using 32Tx ports and 2 Rx ports in UMa scenario. The PMIs before compression in one CSI report, i.e., N4 is 8. Other simulation assumptions for the results are shown in Table A-1 in Appendix. Based on the simulation results, there is almost no throughput gain of Q=4 over Q=3. Comparing Q=3 and Q=2, it is observed that Q=3 has 2.07% gain at 10km/h UE velocity. Therefore, we prefer to support the additional Q values of 3.

Proposal-4:

- Support Q=3.

Agreement For the Type-II codebook refinement for high/medium velocities, the parameter N4 (length of DFT vector, unit-less) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {1, 2, 4} - FFS: If additional candidate value(s) of N4 are supported, e.g. 3, 5, 6, 8, 10, 16, 32, as well as the supported Parameter Combination(s) Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), support 8 as an additional candidate value - FFS (by RAN1#112): Whether any of the following additional candidate values are supported: 3, 5, 16, 32 - The candidate values supported by UE are reported via UE capability (details can be discussed in UE feature).

In RAN1#111 meeting, it was agreed that the parameter N4 support the following value: {1, 2, 4, 8}. For the candidate values {3, 5, 16, 32}, whether the values are supported is to be decided. In our view, {3, 5} is close to {2, 4}, the performance would be similar. Therefore {3, 5} should not be supported. For {16, 32}, the large values are more suitable for low speed UE than high speed UE. System level evaluation is performed at 10km/h UE velocities. Other simulation assumptions are shown in Table A-1 in Appendix.

Figure 5 Gain of enhanced codebook with different N4 comparing with Rel-16 codebook

The SLS results are shown in Figure 5. Comparing the average throughput of different N4 with Rel-16 under same overhead, it is observed that N4=8 can achieve best throughput gain. There is no throughput gain when N4=16, because the accuracy of channel prediction deteriorates when N4 becomes large. Therefore, it is not necessary to support large N4 value: {16, 32}.

Proposal-5:

- Don’t support additional value of N4.

Agreement For the Type-II codebook refinement for high/medium velocities, the parameter δ (in slots) is gNB-configured via higher-layer (RRC) signalling from a set of the following candidate values: - First candidate value: δ=0, - 2 additional non-zero values of parameter δ are supported - FFS: the non-zero value(s), to be selected from 1, 2, 3, 4, 5, 6, and 8 Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), support the additional value of 2 - FFS (by RAN1#112): For the last supported additional value, down select between 1, 3, 4, and 5

In RAN1#111 meeting, it was agreed that the parameter δ support the following value: {0, 2}. For the following candidate values: {1, 3, 4, 5}, whether the values are supported is to be decided. In our view, {1, 3} is close to {0, 2}, the performance would be similar. Therefore {1, 3} are not necessary. For {4, 5}, δ=4 is more useful considering processing and scheduling delay after gNB receiving the reported CSI, as the typical delay is around 4 slots. Therefore, we prefer that the last supported additional value is 4.

Proposal-6:

- Support δ=4.

UE reporting of time-domain channel properties by TRS

In RAN1#109-e meeting, it was agreed that the feature of UE reporting of time-domain channel properties (TDCP) measured via CSI-RS for tracking is supported. There are two use cases for this feature. The first use case is aiding gNB to determine CSI reporting configuration and CSI-RS resource configuration parameters. The second use case is aiding gNB implementation of CSI prediction for TDD. In RAN1#111 meeting, the following 3 alternatives were identified for further down selection.

Agreement For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112: - AltA.1 (Doppler spread) as described in R1-2210523 - AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP) - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,… - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by t,τ=ct,τct,0, where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. - AltB (TD correlation profile) as described in R1-2210523 Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming: - Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports. - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric. - Other scenarios of the agreed use cases can optionally be simulated - Based on the agreed EVM for sTRP and mTRP Note: Different alternatives may or may not apply to different use cases FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high FFS: TDCP parameter(s) signalled with respect to each alternative

Doppler spread or Doppler shift are straightforward to reflect medium/high channel properties. TD correlation profile is intermediate parameter for determining Doppler spread instead of direct channel properties. In addition, there is no advantage of AltB in terms of reporting overhead over AltA.2, as a number of auto-correlation(s) for multiple lags in time domain should be reported in the CSI reporting.

Observation-2:

- For TDCP via TRS, AltA.1/A.2 based on Doppler profile is straightforward to reflect medium/high channel properties.

Based on AltA.1, the Doppler spread, i.e., fd,max-fd,min is reported by UE. However, only Doppler spread is not sufficient and absolute Doppler shift is also needed. Considering the definition of QCL-Type A, both Doppler shift and Doppler spread are included for UE to mitigate the impact of channel variation in time domain. Therefore, for use case of aiding CSI prediction at gNB, Doppler spread reporting alone is not sufficient.

For TDD system, gNB can estimate the DL channel from UL RS transmitted by UE utilizing the channel reciprocity, and then gNB can calculate the DL precoding matrix to match the estimated channel. To overcome fast channel aging, if gNB obtains multiple Doppler shifts of multiple delay paths and H at time instant t, it can predict channel H at a future time instant t+∆t. Then, gNB can calculate the precoder accurately that matches the channel at time instant t+∆t. The key point of channel prediction for TDD is the acquisition of Doppler information.

Observation-3:

- For TDD system, gNB can predict future channel and precoders if gNB obtains current channel by SRS and Doppler shifts of multiple delay paths.

We provide link simulation results for the use case of gNB CSI prediction in TDD system in Figure 6. The detailed simulation assumption can be found in Table A-2 in the Appendix. It can be observed from the simulation results that compared with baseline (without gNB-side CSI prediction), single Doppler reporting provides slight performance gain, and obvious performance gain can be achieved with multiple Doppler reporting.

Figure 6 Performance comparisons of alternatives, TDD, 60Km/h, MCS 4

Proposal-7:

- For TDCP via TRS, AltA.2 is supported.

CSI enhancements for coherent-JT

The configuration and indication of SD basis

In Rel-16, the number of SD basis is configured by RRC signaling as parameter L. In the last meeting, the following agreements were reached on SD basis selection and configuration.

Agreement On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling - When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations - NL =1 is one of the supported candidate values - FFS: Other supported value(s) of NL, and its respective UE capability - FFS: The supported combinations of values for {L1, ..., LNTRP} - Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of PCSI-RS/2Ln codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2. - The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and - for Rel-17-based refinement, the gNB configures a set of N_L combinations for {alpha1, ..., alphaNTRP} Ln=αnPCSI-RS/2 where αn=12,34, 1 FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints.

A set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signalling. The supported candidate values include the legacy candidate values, i.e., {2, 4, 6} for Rel-16-based refinement, and Ln=αnPCSI-RS/2 where αn=12,34, 1 for Rel-17-based refinement. An example of the configuration is shown in Table 2. When NTRP=4, there are 81 possible combinations. Searching over all 81 combinations incurs heavy UE computational burden and feedback overhead. The maximum number of configured combinations shall be decided. With the limitation of the maximum number, all combinations of values for {L1, ..., LNTRP} shall be allowed. That is, there is no further selection of possible combinations. It is up to gNB implementation.

Table 2: Candidate of SD basis configuration

|  | # The parameters of SD basis |  |  |  |
| --- | --- | --- | --- | --- |
|  | # L1 | # L2 | # L3 | # L4 |
| # 1 | # 2 | # 2 | # 2 | # 2 |
| # 2 | # 2 | # 2 | # 2 | # 4 |
| # 3 | # 2 | # 2 | # 2 | # 6 |
| # 4 | # 2 | # 2 | # 4 | # 2 |
| # 5 | # 2 | # 2 | # 4 | # 4 |
| # … |  |  |  |  |

Proposal-8:

- RAN1 decides the maximum number of configured combinations NL, and candidate values include 8, 16.

- There is no further down selection on the combinations, i.e., all combinations of values for {L1, ..., LNTRP} shall be allowed.

W2 quantization and SCI

In Rel-16, the coefficient for SCI is assuming to 1, hence W2 quantization can be achieved by differential quantization for the strongest coefficient in a single polarization direction and differential quantization across different polarizations. For Rel-18 C-JT transmission hypothesis, the following agreement and work assumption were achieved in RAN1#110be meeting.

Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Since the co-located and distributed layouts are both supported for C-JT, the amplitude/power of different TRPs will vary greatly, especially for distributed scenarios. In addition, only one common SCI among multiple TRPs was supported. UE can perform differentiation with respect to reference amplitude within each of the 2N groups, and then perform differentiation with respect to one common SCI. Hence Alt 3 results in more precise quantization. That’s because if one TRP is much weaker than other TRPs, the weak coefficients of this TRP can also be accurately quantized with Alt 3. Therefore, the working assumption on Alt 3 should be confirmed.

Proposal-9:

- For W2 quantization group, the work assumption on Alt 3 is confirmed.

FD basis reporting for mode 1

Regarding FD basis reporting, the following three alternatives were agreed for further down-selection in RAN1#111 meeting.

Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

A conclusion was also made that there is no consensus to introduce explicit per-CSI-RS resource amplitude and/or co-phasing.

Conclusion On the Type-II codebook refinement for CJT mTRP, for mode-1 and mode-2, there is no consensus on introducing additional/explicit per-CSI-RS-resource amplitude scaling and/or co-phase (with separate alphabet set(s)) as additional PMI component(s). - Note: This conclusion has no impact on the Working Assumption reached in RAN1#110bis-e regarding W2 quantization group

Considering inter-site CJT operation, the propagation delay among different TRPs may be large enough to create remarkable difference among FD basis selection for different TRPs. This is because the selected FD basis is a reflection of delay path information. Since there is no explicit/additional co-phasing information per the conclusion, the difference shall be reflected in some way. The three alternatives are analyzed in the following.

- Alt1: A per-TRP FD basis selection offset is applied to a FD basis common to all TRPs. The problem is that the FD basis is common to all TRPs. This imposes restriction on the FD basis selection. Only when all TRPs have the same distribution of propagation delay, the selection can work. Otherwise, the common FD basis shall contain FD basis of all TRPs, i.e., it is the union of FD basis of all TRPs. The result is much larger number of FD basis and increased feedback overhead due to large number of FD basis. If the configured number of FD basis is not large enough, some FD basis may have to be dropped leading to performance degradation. Furthermore, the FD basis selection by Alt1 is not independent across TRPs, and this contradicts the agreed definition of mode 1.

- Alt2: No FD basis offset is applied, and FD basis are independently selected across N TRPs. According to Rel-16 and Rel-17 design, a cyclic shift is applied for each TRP before reporting. That is, reported FD basis for each TRP start with #0. Consider an example with two TRPs. The independently selected FD basis for the two TRPs before shifting are {2, 3, 4, 6} and {10, 12, 14, 15} respectively. After the shifting, the FD basis become {0, 1, 2, 4} and {0, 2, 4, 5}. The co-phasing information is completely lost and system performance degradation is expected. To avoid such situation, the per-TRP FD basis indications shall be redesigned. At least, the cyclic shifting applied to all TRPs shall be common as shown in Figure 7. For the TRPs not containing SCI, the first FD basis is not #0. The number of reported FD basis and the range of FD basis selection for other TRPs would increase leading to additional feedback overhead.

Figure 7 Independent FD basis selection with common cyclic shift

- Alt3: A per-TRP FD basis offset is applied to per-TRP independently selected FD basis as shown in Figure 8. As the FD basis is independently selected for each TRP, the restriction imposed by Alt1 is removed. The FD basis offset reflects the propagation difference among different TRPs and provides necessary co-phasing information to gNB. The per-TRP FD basis indication can still be performed according to current spec. We do not have to reopen the discussion of cyclic shifting among different TRPs which is very complex. The specification impact is much less than Alt2. Take an example with N3 = 13 and M = 7. For a TRP, the overhead of the additional FD basis offset reporting is log2(N3)=4 bits. The overhead of reporting all FD basis according to Rel-16 specification is log2(N3-1M-1)=10 bits. However, if FD basis cyclic shift is not applied to the TRP, the reporting overhead would be log2(N3M)=12 bits. Take another example with N3 = 13 and M = 4. For a TRP, the overhead of the additional FD basis offset reporting is log2(N3)=4 bits. The overhead of reporting all FD basis according to Rel-16 specification is log2(N3-1M-1)=8 bits. The reporting overhead when FD basis cyclic shift is not applied to the TRP, would be log2(N3M)=10 bits. The additional overhead is 2 bits for the FD basis offset reporting. But in return, the specification impact is much less that Alt2.

Figure 8 Independent FD basis selection with explicit offset reporting

Proposal-10:

- On the Type-II codebook refinement for CJT mTRP, for mode-1, Alt3 is supported.

- Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

Other enhancements for mode 1

For mode 1, independent FD basis are selected. If explicit co-scaling is not introduced for mode 1, the only difference between mode 1 and mode 2 is independent or common FD basis selection. However, even if mode 1 is configured by RRC signaling, the delay paths/FD basis of some TRPs may be aligned. In this case, reporting independent FD basis resulting in unnecessary waste of UL resources. Therefore, enhancement to mode 1 can be considered if the FD basis of some TRP/TRP groups is aligned. For example in Figure 9, the UE can omit FD basis of some TRP/TRP groups if the FD basis of some TRP/TRP groups is aligned to reduce the feedback overhead, e.g., similar as mode 2.

Figure 9  Omission of FD basis reporting for mode 1

Proposal-11:

- FD basis for a TRP can be omitted if the FD basis of multiple TRPs are aligned.

Conclusions

In this contribution, we provided our views on the enhancements for DL CSI enhancements. We have the following observations and proposals:

Observation-1:

- 2 CQIs in a CSI report have some performance gain compared with 1 CQI.

- More than 2 CQIs in a CSI report have marginal performance gain compared with 2 CQIs.

Observation-2:

- For TDCP via TRS, AltA.1/A.2 based on Doppler profile is straightforward to reflect medium/high channel properties.

Observation-3:

- For TDD system, gNB can predict future channel and precoders if gNB obtains current channel by SRS and Doppler shifts of multiple delay paths.

Proposal-1:

- Support Alt3A for indicating the locations of the NZCs.

Proposal-2:

- Value S is configured by gNB for Alt3A.

Proposal-3:

- Support at most 2 CQIs for each sub-band in a CSI report which is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices.

Proposal-4:

- Support Q=3.

Proposal-5:

- Don’t support additional value of N4.

Proposal-6:

- Support δ=4.

Proposal-7:

- For TDCP via TRS, AltA.2 is supported.

Proposal-8:

- RAN1 decides the maximum number of configured combinations NL, and candidate values include 8, 16.

- There is no further down selection on the combinations, i.e., all combinations of values for {L1, ..., LNTRP} shall be allowed.

Proposal-9:

- For W2 quantization group, the work assumption on Alt 3 is confirmed.

Proposal-10:

- On the Type-II codebook refinement for CJT mTRP, for mode-1, Alt3 is supported.

- Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

Proposal-11:

- FD basis for a TRP can be omitted if the FD basis of multiple TRPs are aligned.

References

- 3GPP RAN1#109 e-meeting, Chairman’s notes.

- 3GPP RAN1#111 meeting, Chairman’s notes.

Appendix

System level simulation assumptions for high/medium UE velocities

Table A-1: System level simulation assumptions for high/medium UE velocities

| Parameter | Value |
| --- | --- |
| Duplex | FDD |
| Multiple access | OFDMA |
| Scenario | Dense Urban (Uma) |
| Carrier frequency | 2GHz |
| Inter-Macro BS distance | 500m |
| Antenna setup at gNB | 32Tx: (1,1,2,8,2), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup at UE | 2Rx: (1,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 41 dBm for 10MHz |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| Simulation bandwidth | 10 MHz with 15KHz |
| Maximum MU layers | 12 |
| CSI-RS period | 5 slots |
| Predicted CSI report | Periodic predicted CSI feedback with period 20slots/40slots; Number of PMIs included in a report: 4/8; TD/DD unit size d=5slots; Number of CQIs included in a report: 1, 2, 4. |
| Traffic model | Full Buffer |
| UE distribution | 100% outdoor |
| UE receiver | MMSE-IRC |

Link level simulation assumptions for TDCP

Table A-2: Link level simulation assumptions for TDCP

| Parameter | Value |
| --- | --- |
| Channel model | TDL-A |
| Duplex | TDD |
| Delay spread | 300 ns |
| Carrier frequency | 3.5 GHz |
| Speed | 60 km/h |
| BW | 20 MHz |
| SCS | 30 KHz |
| Duplexing | TDD |
| Antenna setup at gNB | 4 Tx |
| Antenna setup at UE | 2 Tx |
| TRS burst configuration | Periodic of 10ms, 2-slot pattern |
| TRS | Full bandwidth |
| SRS configuration | Periodic of 10ms |
| RBs of SRS | Full bandwidth |
| MCS | MCS 4 based on 64QAM table |
| Rank | Rank 1 |