3GPP TSG RAN WG1 #112	R1-2300780

Athens, Greece, February 27th - March 3rd, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Views on Positioning for RedCap UEs

Agenda item:	9.5.5

Document for:	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized and in Rel-17 enhancements were made. At RAN#98, a new work item “Expanded and improved NR positioning” was approved [1]. This contribution discussed our views related to RedCap UE Positioning. Our companion contributions discuss our other views [3-8]. The objective in the WID is:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs)

  - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2].

    - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs.

  - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

# Discussion

## Frequency Hopping for reception of DL PRS

As part of PRS frequency stitching/hopping the UE may need to align the phase of multiple frequency “chunks” in order to remove errors due to phase offsets between the chunks. This procedure can be done by having overlapping frequency part for concurrent chunks (or hops). Which RB/REs should be used for this frequency alignment should be discussed. Figure 3 shows an example of how this might look. If the UE has a deep fade on the RB/REs which are used for the phase alignment it may cause poor performance of the PRS measurement.

Figure 1. Example of PRS frequency hopping with overlapping frequency hops.

Proposal 1: RAN1 should discuss how to perform phase alignment between frequency chunks in PRS frequency hopping/stitching including the impacts of a poor channel on the overlapping RB/REs.

The UE can use these overlapping REs to potentially remove the phase offset between the hops. However, RAN1 should investigate the size of the overlap that is needed for the phase offset to be corrected sufficiently. On top of the overlap needed for PRS frequency hopping RAN1 should also consider how this procedure may impact both UL-TDOA and multi-RTT. In particular in the case of multi-RTT it should be studied if the phase alignment between hops is needed in both UL and DL directions and if so how to enable it.

Proposal 2: RAN1 should support phase alignment for Multi-RTT and determine if phase alignment is needed for both UL at the gNB and DL at the UE.

DL PRS frequency hopping should be supported for both within and without a measurement gap configuration. To support PRS measurement outside of a measurement gap, the UE may be configured with PRS processing windows across the DL BWPs for PRS frequency hopping, and each PRS processing window configuration can indicate the priority of PRS reception. According to the priority, it would be possible for the UE not to measure a certain PRS frequency hop. If the UE misses a PRS frequency hop which is located between two PRS frequency hops, it might be difficult to aggregate the PRS resources to obtain a positioning measurement by CA. An intuitive solution would be to configure PRS reception as always high priority, but it is too restrictive for the gNB configuration. Similarly, for the UL SRS transmission, the UE can miss a chance to transmit an SRS frequency hop according to the defined priority. Thus, RAN1 needs to discuss this issue to support PRS/SRS frequency hopping.

Proposal 3: RAN1 should specify solutions to effectively support DL PRS frequency hopping within PPW configurations across multiple DL BWPs.

## Frequency Hopping for transmission of UL SRS for positioning

In the case of DL PRS frequency hopping/stitching, the UE may receive a wideband PRS resource by frequency hopping, but UL SRS transmission is different from DL PRS reception. The UE may transmit a single SRS resource at a particular SRS frequency hop and another SRS resource at a different SRS frequency hop. That is, gNBs/TRPs receive the different SRS resources by different SRS frequency hops, and when the gNB combines the multiple SRS resources, it cannot see a single ZC sequence. This might not be an optimal configuration for SRS frequency hopping.

Observation 1: The gNB does not see a single ZC sequence from the combined SRS resources across multiple SRS frequency hops if each hop is an independent sequence.

RAN1 needs to consider support reception of a single longer ZC sequence after combining the SRS resources. One option is to introduce a “virtual” SRS configuration where multiple SRS for positioning resources/resource sets are tied to the “virtual” SRS configuration. Each SRS resource/resource set within the virtual configuration would correspond to one SRS hop. In practice this could mean that the SRS sequence generation for a given hop is based on the overall virtual SRS configuration. In this way a single long SRS sequence can be seen at the gNB after receiving all the SRS frequency hops. In addition, it does not break the long standing spec assumption that the SRS is configured per BWP as the virtual SRS configuration would be a new configuration. The configuration details may have higher layer impact.

Proposal 4: RAN1 should support a single ZC sequence configuration across multiple SRS frequency hops (e.g., a virtual SRS configuration for positioning frequency hopping).

## Common Aspects

At RAN1#110-bis the following agreement was reached about modelling the time gap between hops in frequency hopping:

Agreement

For the evaluation of TX/RX frequency hopping for positioning of redcap UEs, the value of the gap between two consecutive hops includes at least from 100us to 5ms.

- Companies should indicate if other smaller values are used in their evaluations, and justify the feasibility of smaller values

In our understanding the specific values between hops should be one of the major discussion points during the WI and may need to involve RAN4. The reason for this is that in TS 38.133 it is specified that DCI and timer based BWP switching requirements are as in the following table

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

As we can see this means that more than a few milliseconds could be needed for a UE to complete SRS frequency hopping. E.g., a UE with 15 kHz SCS could need over 12 ms just to transmit 5 20 MHz hops (if type 2). This is clearly too large of a delay between the start and end of FH for positioning. It may lead to issues with UE speeds as investigated during the SI.

In our understanding the values in the table above take into account a number of components which may not be necessary for frequency hopping for RedCap. For example, by introducing a new fast RF switching for RedCap positioning we could limit the scenario to each hop having the same bandwidth and only the center frequency needs to change. We could also consider other ways to minimize the time gap and unnecessary signalling overhead (e.g., using only 1 DCI up front to trigger all the switching).

RAN4 has previously discussed some fast RF switching times for RedCap UEs (e.g., [2]). It would be good to start early with discussion in RAN1 and in RAN4 to ensure that we have sufficient time to converge on this important topic.

Observation 2: The currently specified BWP switching times may not be acceptable for RedCap FH for positioning.

Observation 3: Some of the component delays which make up the values in the above table from TS 38.133 may not apply to RedCap FH for positioning.

Proposal 5: RAN1 should discuss shorter switching times for RedCap FH for positioning and involve RAN4 if/when necessary.

Proposal 6: RAN1 should discuss the way to reduce the time gap and unnecessary signalling overhead of RedCap FH for positioning (e.g., support a single DCI triggering all the switching).

In addition to the discussion on the switching time, RAN1 can also begin to discuss UE behavior during FH. For example, in UL there is no concept of measurement gap. This may be problematic as the UE needs to switch between hops but a full BWP switch at each hop is not acceptable in terms of delay and complexity. One potential solution to this problem is to define a window in time where the UE can transmit FH SRS for positioning and is not expected to receive/transmit other signals and channels during the window.

Proposal 7: Support an UL time window where the UE is not expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning.

# Conclusions

In this contribution we make the following observations and proposals:

Proposal 1: RAN1 should discuss how to perform phase alignment between frequency chunks in PRS frequency hopping/stitching including the impacts of a poor channel on the overlapping RB/REs.

Proposal 2: RAN1 should support phase alignment for Multi-RTT and determine if phase alignment is needed for both UL at the gNB and DL at the UE.

Proposal 3: RAN1 should specify solutions to effectively support DL PRS frequency hopping within PPW configurations across multiple DL BWPs.

Observation 1: The gNB does not see a single ZC sequence from the combined SRS resources across multiple SRS frequency hops if each hop is an independent sequence.

Proposal 4: RAN1 should support a single ZC sequence configuration across multiple SRS frequency hops (e.g., a virtual SRS configuration for positioning frequency hopping).

Observation 2: The currently specified BWP switching times may not be acceptable for RedCap FH for positioning.

Observation 3: Some of the component delays which make up the values in the above table from TS 38.133 may not apply to RedCap FH for positioning.

Proposal 5: RAN1 should discuss shorter switching times for RedCap FH for positioning and involve RAN4 if/when necessary.

Proposal 6: RAN1 should discuss the way to reduce the time gap and unnecessary signalling overhead of RedCap FH for positioning (e.g., support a single DCI triggering all the switching).

Proposal 7: Support an UL time window where the UE is not expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning.

# References

- RP-223549, “Expanded and Improved NR Positioning”

- R4-2120418, “WF on RedCap RRM requirements”

- R1-2300779, “Views on Bandwidth aggregation for positioning measurements”, Nokia, Nokia Shanghai Bell

- R1-2300778, “Views on NR DL and UL Carrier phase positioning”, Nokia, Nokia Shanghai Bell

- R1-2300777, “Views on LPHAP”, Nokia, Nokia Shanghai Bell

- R1-2300040, “Design of SL positioning reference signal SL-PRS”, Nokia, Nokia Shanghai Bell

- R1-2300041, “On measurements and reporting for SL positioning”, Nokia, Nokia Shanghai Bell

- R1-2300042, “On resource allocation for SL positioning reference signal”, Nokia, Nokia Shanghai Bell