3GPP TSG RAN WG1#112	        	R1-2300867

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:					    9.1.4.2

Source:	Sony

Title:	Considerations on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion

# Introduction

Objective 5: Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

-	Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

According to WID for Rel-18 MIMO enhancement [1], enhancement for SRI/TPMI is required to enable 8 Tx UE transmission.

In this contribution, dual codeword (CW) transmission in PUSCH with rank>4 and antenna port group assumption in UL 8 Tx are discussed.

# Discussion

## Dual CW transmission in PUSCH with rank>4

In RAN1#110bis meeting [2], the following working assumption was made.

Working Assumption

For uplink transmission with rank>4, support dual CW transmission.

For NR DL, dual CW transmission is supported in the case of rank>4. In higher rank, since different layer tends to be different channel quality, dual CW transmission would be beneficial. By dual CW transmission, different MCS for each CW transmission can be used depending on channel quality on each layer. Therefore, also in NR UL with rank > 4, dual CW transmission should be supported.

Proposal 1: RAN1 should confirm the working assumption

- For uplink transmission with rank>4, support dual CW transmission.

In RAN1#111 meeting [3], the following agreement regarding UCI multiplexing on PUSCH was made.

Agreement

For PUSCH transmission with rank>4 by an 8TX UE, to support UCI multiplexing on PUSCH, down-select at least one of the following options in RAN1#112,

- Option1: UCI is always multiplexed on one of the CWs

- Option2: UCI is multiplexed on both CWs

- Option3: Based on UCI (e.g., type, payload size, etc.) UCI is multiplexed on one or both CWs

- Option4: UCI is multiplexed only when single CW is enabled

- Option5: UCI is repeated across the two CWs

- Other options are not precluded

Option 1 is less specification impact as it is similar as single CW. Regardless of the number of CWs, UCI is always mapped to one of the CWs in option 1. For option 2, it has benefit in terms of data utilization in PUSCH. However, in the case of dual CW transmission, channel quality on different layers tends to be different. In addition, MCS on UCI is not changed depending on which CW UCI is mapped to. Therefore, transmission of UCI which is mapped to a CW with lower channel quality tends to be failed. For option 3, although it is attractive solution as UCI multiplexing could be optimized depending on condition such as UCI type or payload size, it would bring large standardization effort as well as much specification impact since RAN1 would need to further discuss how to multiplex UCI for each UCI type or payload size if this option 3 would be agreed. For option 4, although it is also less specification impact like option 1, simultaneous PUSCH and PUCCH transmission is mandatory when PUSCH with dual CWs transmission and PUCCH transmission occurs at the same slot, which would lead high UE power consumption. For option 5, benefit of repetition in two CWs would be doubtable since two CWs are transmitted by using different layer with different channel quality.

Based on the above our analysis, option 1 is preferable for UCI multiplexing on PUSCH with dual CWs.

Proposal 2: support option 1: UCI is always multiplexed on one of the CWs

## Antenna port on 8 Tx partial-coherent UE

In RAN1#110bis meeting [2], the following agreement regarding assumption of port coherency was made.

Agreement

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7}

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

  - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7}

- For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

  - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7}

  - Alt3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5, 7}

- Note: Other alternatives which are not foreseen are not precluded

By following the legacy rule of coherent group in DL 8 Tx and UL 4 Tx, a coherent group should contain 2 or 4 closest antennas with different polarizations. In that sense, alt 2 for when Ng=2 and alt 1 for when Ng=4 should be supported.

Proposal 3: For codebook design of an 8TX partial-coherent UE, the following alternative for coherent groups should be supported

- For when Ng=2, the following convention for assumption of port coherency scheme is used

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

- For when Ng=4, the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

# Conclusions

The following Proposals were made in this contribution.

Proposal 1: RAN1 should confirm the working assumption

- For uplink transmission with rank>4, support dual CW transmission.

Proposal 2: support option 1: UCI is always multiplexed on one of the CWs

Proposal 3: For codebook design of an 8TX partial-coherent UE, the following alternative for coherent groups should be supported

- For when Ng=2, the following convention for assumption of port coherency scheme is used

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

- For when Ng=4, the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

# References

[1]			 RP-223276	, “WID Update: MIMO Evolution for Downlink and Uplink,” 3GPP TSG RAN Meeting #98-e, December 2022.

[2] RAN1#110bis meeting chairman’s note, October 2022

[3] RAN1#111 meeting chairman’s note, November 2022