3GPP TSG RAN WG1 #112								             R1-2300869

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item 	:	9.2.2.2

Source 	:	Sony

Title 	:	Considerations on CSI measurement enhancements via AI/ML

Document for 	:	Discussion and decision

# Introduction

According to the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface SID [1], the SI contains the following objectives:

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

In RAN1#109-e, the following agreement was made [2]:

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

- Note: Study of other sub use cases is not precluded.

- Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

In RAN1#111, the following agreement was made [3]:

Agreement

Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement.

Note: Continue evaluation discussion in 9.2.2.1.

Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer further till the end of R18 AI/ML SI.

Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

In this contribution, we provide our views on other CSI measurement enhancements via AI/ML sub-use cases.

# CSI Measurement and Feedback

In this section we analyse two sources of inaccuracy for CSI feedback

  - Timeliness of channel measurements

In usual operation, the UE carries out CSI measurements in one subframe or slot, the results of these measurements are sent to the network in the form of CSI reports during a later UL slot/subframe and the gNB uses these measurements to select and schedule resources for future transmissions to and from the UE in another slot/subframe. The gNB does this on the implied assumption that the channel conditions between the time the CSI measurements were done and when the transmission takes place are similar. The degree to which the channel quality measurements taken from a time before the actual transmission time reflects the actual channel quality during the time the physical channel transmission occurs depends on how far apart the two times are but also on the relative mobility of the UE. For a high mobility UE, radio propagation conditions can change significantly between the two times concerned. It is therefore desirable to take the channel measurement as close as possible to the actual physical channel transmission slot or subframe. Failing this, the CSI measurements taken some time in the past can also be forward propagated to the likely physical channel transmission time and then used to determine the allocation of transmission resources. AI/ML can help in finding a good model for this extrapolation or prediction of channel measurements from previous and likely coarse measurements. This can be done in many possible ways two of which are discussed here.

    - CSI Prediction at the gNB

Scheduling decisions are taken by the gNB. The gNB can therefore predict the CSI for a number of possible transmission time intervals and then schedule the UE at the time interval corresponding to the best predicted CSI. For this, the gNB would need accurate CSI measurements at the current time from which to predict. For DL scheduling, the resources required to transmit such CSI measurements from the UE can be minimised by CSI compression.

Proposal 1: For the CSI prediction sub-use case, RAN1 should consider CSI prediction at the gNB and the implication for accurate CSI measurement reporting to support this by the UE.

    - CSI Prediction at the UE

At RAN1#111, it was agreed to select CSI prediction at the UE as a sub-use case for study. As scheduling decisions are taken by the gNB, the UE would need additional information regarding the likely time interval the gNB will schedule UE transmissions on so that it can send the gNB the predicted CSI for that time interval. Another option is for the UE to predict the CSI at more than one possible transmission time interval and deliver all the predicted CSIs to the gNB so it can make the scheduling decision. Either way, as a follow up to the agreement reached at RAN1#111 with respect to CSI prediction at the UE, further consideration has to be given to how the UE will determine the time interval(s) at which to predict the CSI.

Proposal 2: For the CSI prediction sub-use case, RAN1 should consider how the UE determines the appropriate scheduling time interval(s) at which to predict the CSI.

  - Frequency and time granularity of channel measurements

CSI measurements are taken on configured CSI resources. Resources configured for CSI represent an overhead as data cannot be transmitted on such resources. For this reason, in Rel17, the maximum configurable density for CSI resources is 3 resource elements per PRB. This maximum density determines the maximum amount of information that can be transmitted in a measurement report. It is desirable to increase CSI resource density as this gives better channel measurement accuracy and frequency granularity to help drive a more accurate adaptive modulation and coding process. However, an increase in density, as well as depleting transmission resources will also require more CSI measurement reporting resources in the UL. By post-processing sparse CSI measurements by for example interpolation, the frequency (and time) granularity of CSI measurements can be improved. This improved granularity can be used to make better resource allocation decisions. Post-processed CSI measurements, in the limit, could generate a CQI value for every resource block. To transmit this to the gNB, further compression is required to reduce the number of bits needed.

The post-processing of CSI measurements can be subjected to AI/ML to derive a good model that can be applied to extend the coarse channel measurements into a full band measurement with granularity approaching a channel measurement per resource block. Compressing this full-band channel measurement can also be achieved by AI/ML to produce a concise transmissible format that when exploded at the gNB will provide higher granularity CSI measurements for better resource allocation and choice of MCS etc. CSI measurement compression in both spatial and frequency domain was agreed in RAN1#109e to be under study in this SI.

Proposal 3: RAN1 should study paradigms for increasing CSI measurement granularity in both time and frequency for more accurate resource and MCS allocation.

With denser CSI measurements and their efficient reporting, there may be a need for new methods of resource allocation and associated transport channel processing that exploit the denser channel measurements.

Proposal 4: RAN1 should study new methods of resource allocation and transport channel processing based on higher CSI measurement granularity.

# Use case of compressed CSI feedback

In the RAN1#109-e meeting, we agreed to study spatial-frequency domain CSI compression using two-sided AI model as one representative sub use case. In this section, we discuss about the compression of CSI feedback using two-sided AI model. Considering use case of compression of channel information (e.g. compressed raw channel or eigenvectors), UE does not need to feedback conventional PMI information to gNB because gNB can calculate precoder of downlink transmission from decoded CSI feedback. One issue is whether the compressed channel information is treated as a new PMI type or new CSI feedback. We think this would have a specification impact. If the compressed channel information can be treated as a new PMI type, the specification impact could be smaller than treating it as new CSI feedback. However, if it is difficult to treat the compressed channel information as a new PMI type, introduction of new CSI feedback information needs to be considered. We think RAN1 should study whether the compressed channel information is treated as a new PMI type or new CSI feedback information.

Proposal 4: RAN1 should study whether the compressed channel information is treated as a new PMI type or new CSI feedback information.

If new CSI feedback information is introduced for AI/ML based CSI compression, this could have a large specification impact on channel state information framework in section 5.2.1 in TS38.214. We think this specification impact is too large, so introducing a new PMI type in section 5.2.2.2 of TS38.214 is better to save workload. We cannot find any reason to introduce new CSI feedback information for AI/ML based CSI compression so far. So, we propose to study specification impact of new PMI type for the CSI compression using two-sided model use case.

Proposal 5: RAN1 should study the specification impact of new PMI type for the CSI compression using two-sided model use case.

# Conclusion

In this contribution, we discussed our views on CSI measurement enhancements via AI/ML. We observed and proposed as follows:

Proposal 1: For the CSI prediction sub-use case, RAN1 should consider CSI prediction at the gNB and the implication for accurate CSI measurement reporting to support this by the UE.

Proposal 2: For the CSI prediction sub-use case, RAN1 should consider how the UE determines the appropriate scheduling time interval(s) at which to predict the CSI.

Proposal 3: RAN1 should study paradigms for increasing CSI measurement granularity in both time and frequency for more accurate resource and MCS allocation.

Proposal 4: RAN1 should study new methods of resource allocation and transport channel processing based on higher CSI measurement granularity.

Proposal 4: RAN1 should study whether the compressed channel information is treated as a new PMI type or new CSI feedback information.

Proposal 6: RAN1 should study specification impact of new PMI type for the CSI compression using two-sided model use case.

References

[1]	RP-213599, Qualcomm (Moderator), “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” RAN #94e, December, 2021.

[2]	Chairman's Notes RAN1 #109-e, May 2022.

[3]	Chairman's Notes RAN1 #111, November 2022.

[4]	R1-2212909, Moderator (Apple), “Summary #4 on other aspects of AI/ML for CSI enhancement,” RAN1 #111,  November, 2022.