3GPP TSG RAN WG1 #112			R1-2300871

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item: 	9.2.4.2

Source:		Sony

Title:			On Other Aspects on AI/ML for Positioning Accuracy Enhancement

Document for:	Discussion & Decision

# Introduction

A new study item on Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RANP#94e [1]. Some of the objectives of the study item is shown below:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

During RAN1#111, the following agreements were made [2]:

Agreement For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI. Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement - Types of measurement as model inference input - new measurement - existing measurement - UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b) - For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a) - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - New and/or enhancement to existing assistance signaling - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed Regarding data collection for AI/ML model training for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified for further study - For direct AI/ML positioning, ground truth label is UE location - PRU with known location - UE generates location based on non-NR and/or NR RAT-dependent positioning methods - LMF generates UE location based on positioning methods - LMF with known PRU location - Note: user data privacy needs to be preserved - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output - PRU generates label directly or calculates based on measurement/location - UE generates label directly or calculates based on measurement/location - Network entity generates label directly or calculates based on measurement/location - The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: other options of entity to generate other training data are not precluded - Note: Existing PRU definition is in 38.305 Regarding data collection for AI/ML model training for AI/ML based positioning, study benefits, feasibility and potential specification impact (including necessity) for the following aspects - Request/report of training data - Ground truth label - Measurement corresponding to model input - Associated information of ground truth label and/or measurement corresponding to model input - Assistance signaling and procedure to facilitate generating training data - Reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier - Assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc. - Note1: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed - Note2: Study may consider different entity to generate training data as well as different types of training data when applicable - Note3: study considers both of the following cases when applicable - when the training entity is the same entity to generate training data - when the training entity is not the same entity to generate training data Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network For AI/ML based positioning accuracy enhancement, direct AI/ML positioning and AI/ML assisted positioning are selected as representative sub-use cases.

This contribution provides our views on AI/ML for positioning, particularly on AI/ML assisted positioning to improve positioning accuracy.

# Discussions on AI/ML Assisted Positioning

The positioning sub use-cases where AI/ML approaches can be applied were identified. AI/ML assisted positioning as one of the sub use-cases was agreed and selected for characterization and performance evaluations of AI/ML based positioning accuracy enhancement. In AI/ML assisted positioning, the output of AI/ML model inference can be a new measurement and/or an enhancement of the existing measurement. We consider this sub use-case is important for AI/ML approaches to enhance positioning accuracy.

AI/ML assisted positioning can be performed by utilizing multipath reporting that was introduced in Rel-17, particularly the multiple paths reporting mechanism. For DL-TDOA and Multi-RTT positioning, UE is capable of measuring and reporting a maximum of 8 additional paths. In the measurement report, the indices of the additional paths and the associated relative path power measurement (PRS-RSRPP) for each path could be reported. Likewise in UL case, the gNB could also report multipath measurements (i.e, UL-RSRPP) to LMF.

By allowing UE/TRP to report the received power of the multipath components, the LMF has the chance to process and measure in the channel delay domain. Based on the outcome of positioning in Rel-17, it is up to LMF on how to utilize this information (i.e., implementation aspect of LMF). We consider the usage of AI/ML to process such information, including the collected path measurements from UE/TRP, would be beneficial in AI/ML assisted positioning sub use-case, especially to improve the positioning accuracy.

Multipath measurement and reporting open up a new opportunity for AI/ML deployment, since LMF can collect more detailed channel information from UE/TRP. For example, in the legacy, LMF would only obtain one timing measurement per measurement report. The accuracy of identifying LOS/NLOS mainly relies on the UE. But now the LMF is capable of acquiring multiple path information. Having the path information including relative signal power (PRS-RSRPP) and relative delay of multiple propagation path, LMF can make its own decision on the LOS path identification.

Observation 1: The multiple paths reporting from UE/TRP to LMF as a feature in rel-17 could assist network-side (e.g., LMF) to make its own decision on LOS path selection.

In principle, the network (specifically, LMF) can collect a huge amount of positioning measurement data which could be from different UEs/gNBs. It enables LMF to further create and train an AI/ML model using the set of collected data.  This method can be particularly used for NLOS mitigation and can achieve an improved detection accuracy.

The usage of AI/ML can be briefly divided into three phases:

- Data collection phase

- Model training and updating phase

- Model deployment phase (Inference)

These three phases and the involved communication units are illustrated in Figure 2. In the data collection phase, channel quality and multipath information are measured and collected for further analysis. This procedure is performed by UE or gNB, particularly for the UE with a fixed-location that can provide an exact location. The sources of the data can differ in DL and UL cases. In DL, UE receives the reference signal and processes the data, while in UL, the gNB performs this procedure. After the measurement is performed, LMF collects the results, such as the path information, quality matrix and possible its own location. It should be noted that in this phase, LMF also needs to process and validate the data to further transfer it to a training set as an input to the model training.

Once LMF collects a certain amount of data, LMF can perform ML model training. Model training step may include model selection, training, validation and testing. Lastly, at the output from the training phase, an inference model will be deployed in UE or gNB or LMF to handle specific inference operation, e.g., NLOS detection.

Data Collection with processing and validation

(LMF or reference UE/gNB)

Model Training and Updating

(LMF)

Model deployment

(UE/gNB/LMF)

Figure 2: Procedure of Machine learning utilization for positioning.

Observation 2: The procedure of AI/ML for positioning can be at least divided in three phases:

- Data collection with data processing and validation,

- Model Training and updating,

- Model deployment.

Proposal 1: Support AI/ML with model transfer, such as LMF to create and train AI/ML model (e.g., for NLOS mitigation) and inference model is in another entity such as in UE, gNB.

It was agreed to study and provide inputs on benefit(s) and potential specification impact at least for some cases of AI/ML based positioning. Among various cases, Case 2b was highlighted as the main focus which defined UE-assisted/LMF-based positioning with LMF-side model can be deployed for direct AI/ML positioning. In addition to this case, we also considers the other possible use case which is not covered by the existing case list, such as UE-assisted with LMF-side Model but for assisted AI/ML positioning. In such case, the AI/ML model training and the inference model are operated separately in two different units, such as model training in LMF but inference in UE/gNB. The operation can be similar to direct AI/ML. Hence, we consider to slightly modify case 2b to support assisted AI/ML.

Proposal 2 – Modify Case 2b to also support assisted AI/ML positioning. Hence, Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning or assisted AI/ML positioning.

In terms of specification impact, two aspects may typically be relevant. The first aspect is the signalling for training data collection. ML model training relies on the known properties learnt from the training data, which contains measurements from the UE or gNB (as the input to the ML model) and the ground truth value (as the expected output from the ML model given specific input set). Only by feeding the ML model with a sufficiently large input set and the expected output set, LMF can obtain a robust inference model. To fulfil the above requirement, there should be some specific reporting from UE/gNB to LMF to allow the LMF to collect the right information for model training. Therefore, potentially there could be some new specification impact on the reported content dedicated for model training and which has not been supported in Rel.17.

For example, to train an AI/ML model for NLOS identification, LMF needs to be aware of the true LOS/NLOS labels of the measurements in the training set. These labels should be informed by the UE, preferably a reference UE or reference gNB which knows its own location. The UE/gNB reports channel observation information to the LMF. The channel observation information may consist of the observed power delay profiles (i.e, CIR), power metrics, SNR, other channel characteristics, etc. that may be used as the input for AI/ML assisted positioning, particularly to identify the LOS path and improve the positioning accuracy.

Observation 3: The channel observation (e.g., in a form of CIR, SNR, RSRP) is used as part of the data collection in the creation of training model

Proposal 3: Support channel observation as part of the data collection from UE and gNB for downlink and uplink-based positioning, respectively.

Reference UE for positioning has been discussed in Rel-17 and known as Positioning Reference Unit (PRU). The usage of PRU could be beneficial for AI/ML for positioning, especially in providing the channel observation around the PRU.

Proposal 4: Support PRU to provide the channel observation to LMF for AI/ML Positioning.

Firstly, the network (e,g., LMF) trains and create AI/ML model or here, we call it prediction models (PM) given the information from the UE/gNB. Note, we consider the details of the PM creation using AI/ML is up to the implementation. The second aspect is about signalling the prediction model in the model deployment phase (inference). After the model training, the LMF can utilize the model and apply it on the new measurements. But the model may not necessarily be used only in LMF. The UE can also perform the PM deployment or also known as inference. For example, LMF can also report the prediction model to UE and allow them to perform ML inference. This can avoid heavy computation in LMF.

Proposal 5: Support AI/ML Positioning with UE-side inference.

This process require the transmission of PM to the UE. Hence, we should also consider the inference model structure and size. It is beneficial to provide the inference model in smaller size (without compromising the performance), particularly to the UEs given the environment to the UE operates in. The inference model from LMF to the UE can consider the following aspects:

- The inference model can be partially or fully provided.

- The time validity of the inference model.

- The structure (i.e., number and types of stages, interconnections, etc.) of the predictive neural network as part of the inference model.

- The measurement that the UE should perform associated with the provided inference model.

Proposal 6: On AI/ML model indication, define the inference model (e.g., contents, structure, size) to be provided from LMF to UE/gNB.

To reduce the complexity of data collection and overhead while improving the positioning accuracy, we can consider adopting the distributed deep learning model. For example, multi models are deployed at specified gNBs separately and the number of specified gNBs may be different when some states of channel are changed, but it can be determined by UE based on a pre-threshold, like, choosing reliable gNBs according to certain criteria such as a higher SNR. As an example, it is illustrated in the Figure 3. Here, when the UE moves, the channel environment between each gNB and UE may change, leading to the changes of the number of reliable gNBs.

Figure 3: the number of positioning gNBs varies with the channel environment

Every model is trained independently by unique propagation channel environment between the deployed gNB and UE. Channel matrix (i. e. complex H matrix) can be expressed as the amplitude and phase information of channel in REs. Hence, it can be the model input for training/inference which can be obtained by estimating SRS when performing UL positioning. There will be multi outputs from different gNBs model, where the outputs are the position feature vector. Another deep learning model is deployed at LMF to fuse the positioning feature from gNBs model for enhancing the positioning accuracy. The information transmitted between UE, gNBs and LMF in positioning with distributed learning model are shown in Figure 4.

Figure 4: procedure of distributed learning model in positioning

As described above, we have evaluated the performance of distributed model and the results are shown in Figure 5. There is a parameter  J which express the average number of positioning gNBs. Conventional fingerprint positioning as the baseline is compared with our proposed method with the same simulation environment. It can be observed that the achieved positioning accuracy based on distributed learning model is similar with conventional fingerprint positioning and with the advantage of using less gNBs. We consider the distributed learning model is beneficial and it can be further studied because it can provides good positioning accuracy (i.e., comparable as the finger printing method) and reduces the measurement overhead.

Figure 5: comparison of CDF of positioning errors for various methods

Observation 4: Distributed learning model can achieve a better positioning accuracy based on training/inference by specific propagation channel environment.

Proposal 7: Distributed learning model for positioning accuracy improvements can be considered.

# Summary

In this contribution, we have discussed our view on usage of AI/ML for positioning. Our observation are listed below:

Observation 1: The multiple paths reporting from UE/TRP to LMF as a feature in rel-17 could assist network-side (e.g., LMF) to make its own decision on LOS path selection.

Observation 2: The procedure of AI/ML for positioning can be at least divided in three phases:

Observation 3: The channel observation (e.g., in a form of CIR, SNR, RSRP) is used as part of the data collection in the creation of training model

Observation 4: Distributed learning model can achieve a better positioning accuracy based on training/inference by specific propagation channel environment.

We have also the following proposals:

Proposal 1: Support AI/ML with model transfer, such as LMF to create and train AI/ML model (e.g., for NLOS mitigation) and inference model is in another entity such as in UE, gNB.

Proposal 2 – Modify Case 2b to also support assisted AI/ML positioning. Hence, Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning or assisted AI/ML positioning.

Proposal 3: Support channel observation as part of the data collection from UE and gNB for downlink and uplink-based positioning, respectively.

Proposal 4: Support PRU to provide the channel observation to LMF for AI/ML Positioning.

Proposal 5: Support AI/ML Positioning with UE-side inference.

Proposal 6: On AI/ML model indication, define the inference model (e.g., contents, structure, size) to be provided from LMF to UE/gNB.

Proposal 7: Distributed learning model for positioning accuracy improvements can be considered.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #111, November 2022