3GPP TSG RAN WG1 #112			R1-2300879

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item: 	9.5.1.1

Source: 	Sony

Title: 	Considerations on SL Positioning Reference Signal

Document for: 	Discussion & Decision

# Introduction

A new work item on expanded and Improved NR Positioning was approved in RANP#98e [1]. The work item is mostly based on the outcome of the study item and summarized in [2]. Sidelink positioning has been identified as one of the new positioning features in rel-18. The work item objective related to sidelink positioning is given below.

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

In this contribution, we provide our views on the characteristics of sidelink positioning reference signal.

# Discussion

Sidelink Positioning and ranging requires the transmission of positioning reference signal. During the study item phase, it was concluded to use a comb-based frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS.

The usage of comb-based frequency domain is essential as this would enable more UEs transmit SL-PRS within the same symbol. This would enable sidelink positioning measurement from multiple UE transmitting SL-PRS within at least one symbol. The comb-structure as in the legacy NR positioning can be used as the starting point.

Proposal 1: Support SL-PRS frequency domain with comb structure. The legacy comb-N structure in legacy NR positioning can be used as the baseline.

On the time domain structure of SL-PRS, we prefer to have time-gap on before and after SL-PRS transmission. The time-gap can be used to minimize the interference from/to the adjacent symbol. Furthermore, the time-gap prior to and after the SL-PRS can be used for AGC purpose and possible Rx/Tx retuning, respectively. This is illustrated in Figure 1. We consider one symbol for each gap is sufficient.

Figure 1: Time domain structure of SL-PRS transmission

Proposal 2: Support the time-gap prior to and after the SL-PRS transmission. The first and second gap are used for AGC and Rx/Tx retuning.

The SL-PRS is based on the legacy DL-PRS where a pseudo-random generator is required at the UE transmitting SL-PRS to generate the SL-PRS. There are several parameters which affect the initialization of generating pseudorandom sequence. We consider one of the parameters is the SL-PRS ID, which associates with the UE Tx ID. Hence, the UE Rx who perform positioning measurement can regenerate the SL-PRS and also identify the UE Tx.

Proposal 3: Support the introduction of SL-PRS ID and the ID is associated with the UE Tx ID.

The transmission of SL-PRS is expected to be in two type of SL resource pool, namely dedicated resource pool and shared resource pool. The bandwidth of the SL-PRS should be reconfigurable and it is up to the gNB to allocate the bandwidth of SL-PRS within a resource pool, regardless of the resource pool type. However, the maximum bandwidth of SL-PRS should be equivalent to the resource pool bandwidth.

Proposal 4: SL-PRS bandwidth within a resource pool is reconfigurable up-to the maximum resource pool bandwidth.

# Conclusion

In this contribution we provide our view on sidelink positioning reference signal. We made the following proposals:

Proposal 1: Support SL-PRS frequency domain with comb structure. The legacy comb-N structure in legacy NR positioning can be used as the baseline.

Proposal 2: Support the time-gap prior to and after the SL-PRS transmission. The first and second gap are used for AGC and Rx/Tx retuning.

Proposal 3: Support the introduction of SL-PRS ID and the ID is associated with the UE Tx ID.

Proposal 4: SL-PRS bandwidth within a resource pool is reconfigurable up-to the maximum resource pool bandwidth .

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021

- RAN1 Chairman’s notes, RAN1#111, November 2022