3GPP TSG RAN WG1 #112			R1-2300897

Athens, Greece, 27th Feb – 3rd Mar 2023

Source:	Panasonic

Title: 	Discussion on Sidelink Co-channel Coexistence

Agenda Item:		9.4.2

Document for:	Discussion, Decision

# Introduction

This contribution is to discuss co-channel coexistence for LTE sidelink and NR sidelink on semi-static resource pool partitioning and dynamic resource sharing. It also covers the mechanism(s) for resource pool configuration, resource sensing and reservation, SCS and PSFCH handling.

# Discussion

As per RAN1#109-e meeting agreement, the solution of co-channel coexistence will be discussed under the scope of resource pools. The solutions would be roughly categorized as semi-static resource pool partitioning (SRPP) and dynamic resource pool sharing (DRPS). This contribution will mainly focus on the remaining issues for DRPS as the issues for SRPP have been mostly concluded.

## Device Types

The dynamic resource sharing would be for resource pools accessible for Type A devices and also other types of devices (“device type” is used in R18 to distinguish from R17 “UE type”). The following agreement and proposals are from the RAN1#110 and RAN1#109-e meeting for defining different types and also on the coexistence of different device types.

Working assumption (RAN1#110) Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module. Proposal 1-2a (II): (RAN1#110 Issue 1-2: Type B Devices) - For co-channel coexistence solutions in Rel-18, RAN1 is to study device type B which contains: - Alt 1: Only an NR SL module. - Alt 2: a co-located LTE SL and NR SL module, but the LTE SL module does not share any LTE sensing and resource reservation information. Proposal 1-3 (II): (RAN1#110 Issue 1-3: Device Type Coexistence) - For co-channel coexistence solutions in Rel-18, the supported device type(s) Type A (and Type B, if agreed to be supported) (type A and/or type B) has to at least coexist at least with LTE SL devices. - RAN1 does not pursue any further potential enhancements for device type other than A and/or B (and B if agreed). Agreement: (RAN1#109-e) For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported. Proposal 1-1 (IV) (RAN1#109-e) - For the study of co-channel coexistence solutions in Rel-18, at least device type A is considered. - FFS: Whether type B devices are considered. - For the study of co-channel coexistence solutions in Rel-18, the supported considered device type(s) coexist with type C devices in the same channel, type D and type E devices. - Note: - Type A devices are Rel-18 devices that contain both LTE SL and NR SL modules - Type B devices are Rel-18 devices that contain only NR SL modules - Type C devices are Rel-14/Rel-15 devices that contain only LTE SL modules - Type D devices are Rel-16/17 devices that contain only NR SL modules Type E devices are Rel-16 devices that contain both LTE SL and NR SL modules based on in-device coexistence framework

It has also been agreed in the revised WID from RAN#97-e meeting that “RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A”.

Regarding to the device types, we think that, for V2X usage perspective, co-existence with type A and type C are sufficient and with high priority as it is the use case where NR SL is introduced to the deployment of LTE SL. The co-existence among type A, B and C can be considered for other than V2X use case but lower priority.

Proposal 1: For V2X usage perspective, co-existence of type A and type C are sufficient (with the high priority). The co-existence among type A, B and C can be considered for SL use cases other than V2X (with lower priority).

For the intra UE LTE/NR overlap scenarios of [LTE TX, NR RX] and [LTE RX, NR TX], no concurrent Tx/RX could be performed as the transmission signal interferes to receiver side similar to single SL operation. For the overlap scenario of [LTE RX, NR RX], we think at least type A SL devices should be capable to receive both LTE SL and NR SL simultaneously, as it has similar complexity with separate FDMed LTE and NR spectrums. Then for the overlap scenario of [LTE TX, NR TX], for SL UEs capable to perform simultaneous TX of LTE and NR SLs, some optimization on dynamic power sharing between [LTE TX, NR TX] may also be considered.

Proposal 2: Type A devices should be capable to receive both LTE SL and NR SL simultaneously.

Proposal 3: For SL UEs capable to perform simultaneous TX of LTE and NR SLs, some optimization on dynamic power sharing may also be considered.

## Resource Pools Configurations

As discussed in last RAN1#109-e meeting, how to configure/update such resource pool would be a critical issue, as no spec change is allowed for LTE. Then the up to 16 resource pools (per carrier) for LTE can only be updated in legacy manners (Uu: RRC, SIB; PC5: as per section 5.1 of TS23.287). This may be out of RAN1 scope.

For the configuration of the dynamic sharing resource pools. We think there could be two possibilities: namely – 1) the same resource pool configuration is applied for LTE SL and NR SL, and 2) LTE resource pools are overlapped with NR resource pools and the LTE/NR pools are configured separately. Considering the two possibilities, we think it needs to be clarified how the resource pools are configured to LTE SL and NR SL. If to support partial overlapping between LTE and NR resource pools are necessary, the latter configuration should be taken.

Proposal 4: The configuration of the dynamic sharing resource pools may have two possibilities: – 1) same resource pool for LTE SL and NR SL, and 2) overlapped but separated configuration for LTE and NR SL. The kind of resource pool configuration needs to be clarified.

The following proposal was discussed in the last RAN1#111 meeting but not concluded on the timeline of dynamic resource pool sharing.

Proposal 1-4 (II): (Issue 1-4: Timeline of Information Shared by LTE SL Module) - For dynamic resource pool sharing, for details regarding the timeline on how the NR SL module uses the information shared by the LTE SL module, - Alt 1: The latest LTE SL subframe used to determine the information shared with the NR SL module is at most Tvalid ms prior to the time of sharing the information. - Alt 2: Further time-related aspects are up to UE implementation.

During the discussion for Alt.1 in the last meeting, there was two different interpretations that whether Tvalid contain T explicitly or not. In our understanding, "NR module processing time" + "NR module and LTE module communication delay" + "LTE module processing time" are not separately specified. Then it looks the sum of them is called as Tvalid i.e. [n, n+Tvalid] would be more reasonable. Therefore, we support the latter interpretation in Alt.1.

Proposal 5: We support Alt 1 to have at most Tvalid ms prior to the time of sharing the information 2 for the latest LTE SL subframe used to determine the information shared with the NR SL module. The Tvalid includes T.

## Resource Sensing and Reservation

The following was discussed and agreed in RAN1#110bis-e and RAN1#111 meeting that the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

Agreement (RAN1#110b, Issue 1-2: DRPS – Details of LTE Sensing Information Shared by LTE SL Module) For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE Agreement (RAN1#110b, Issue 1-3: DRPS – Use of LTE Sensing Information) For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission. - FFS: which layer carries out the resource determination: PHY layer or MAC layer. Agreement (RAN1#111, Issue 1-2: Use of Information Shared by LTE SL Module) For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.

How the NR SL module uses the information shared by the LTE module has been agreed in RAN1#111 that the procedures will be in PHY layer. Also, we are in the opinion that all the LTE PHY layer sensing information should be transparent to NR module. Then the information from LTE sensing procedures would be treated with equal priority during NR sensing procedures, i.e., non-proper resources from LTE sensing (by priority, SCI, etc.) are excluded to each X%.

Proposal 6: Assuming all LTE PHY layer sensing information are transparent to NR module, then the information would be treated same as NR sensing, i.e., non-proper resources from LTE sensing (by priority, SCI, etc.) are excluded to each X%.

For whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL, we think Inter-UE coordination can be used for there are both Type B UE and Type A UE in the resource pool (e.g., If RSU is Type A UE and others are Type B UE in rel.17 manner). Other Type A devices may broadcast its LTE sensing results (from other LTE UEs) as “non-preferred resource” as inter-UE coordination so that at least Type B devices would try to avoid such resources. Type B devices may have lower priority compared with type A devices.

Proposal 7: Inter-UE coordination can be used for there are both Type B UE and Type A UE in the resource pool (e.g., if RSU is Type A device and others are Type B devices in rel.17). Type A devices may broadcast its LTE sensing results (from other LTE UEs) as “non-preferred resource” as inter-UE coordination so that other Type A devices and Type B devices would try to avoid such resources. Type B devices may have lower priority compared with type A devices.

Also, for Type A devices, it may indicate their own reservation with both LTE and NR SCIs. When NR Tx/Rx is performed, one possible solution is that a reservation/indication by LTE, so that LTE UE will be able to skip the resources used by NR in this kind of pool. The LTE reservation/indication could be either prior to or together with the NR transmission and/or NR reservation. Alternatively, for in-coverage UEs, they can use gNB to relay the information via UL and DL.

Proposal 8: For Type A devices, it may indicate their own reservation with both LTE and NR SCIs (at least for type C devices). Alternatively, for in-coverage UEs, they can use gNB to relay the information via UL and DL.

Proposal 9: For in-coverage UEs, they can use gNB to relay the sensing information via UL and then DL to targeted UEs.

In another possibility, for an LTE/NR shared resource pool, it could be specified that periodic reservation of LTE V2X is used for LTE V2X, and the remaining resource is used for NR V2X. Considering dynamic scheduled NR SL are mostly aperiodic transmissions and may be with more time-sensitive latency requirement, the dynamically scheduled NR SL transmissions may be prioritized over LTE even with lower priority.

Proposal 10: For an LTE/NR shared resource pool, it could be specified that periodic reservation of LTE V2X is used for LTE V2X, and the remaining resource is used for NR V2X. The dynamically scheduled NR SL transmissions may be prioritized over LTE even with lower priority.

## PSFCH Handling

The constraints for SCS and PSFCH has been discussed in the RAN1#110 and the last RAN1 #110bis-e meeting. The following agreement is from RAN1#110 and the proposal is from RAN1#111.

Agreement (RAN1#110, Issue 2-3: Dynamic Resource Pool Sharing – SCS and PSFCH) For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools - For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set. Proposal 1-1(VII): (RAN1#111, Issue 1-1: PSFCH Overlapping): - For dynamic resource pool sharing, in NR SL resource pools with PSFCH configured and when HARQ-ACK is enabled, based on (pre-)configuration, when PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain, the NR SL UE - Always avoids transmissions on the PSFCH resources (Alt 1), or - FFS details including whether the TX UE avoid selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and/or RX UE does not transmit on the overlapping PSFCH resources. - Does not avoid transmission on the PSFCH resources (Alt 2), or - Conditionally avoids transmissions on a subset of the PSFCH resources. - FFS details of conditions including - a (pre-)configured subset, - the consideration of the LTE RSRP and LTE and/or NR priority, - presence of PSCCH/PSSCH transmission in the same time slot LTE subframe as PSFCH transmission with the same power by the same UE. - FFS for the case when there is an overlapping of time and frequency resources between PSFCH and LTE SL transmission - Introduce additional PSFCH periodicity of [5, 8 and] 10. - Note: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration. - FFS: Whether to confine the PSFCH transmission, in the time domain, within the guard symbol of the LTE SL subframe. - FFS details including the conditions.

Regarding to PSFCH, it was proposed to support (pre-)configuration among three operation i.e, always avoid PSFCH transmission that overlaps with LTE SL Tx, not avoid PSFCH transmission that overlaps with LTE SL Tx, conditionally avoid PSFCH transmission that overlaps with LTE SL Tx. At first, our preference is not to take conditionally avoid PSFCH transmission that overlap with LTE SL because it requires more discussion on what condition should be used for the decision and it would make the design more complex.  Between always avoid and not avoid, our preference is always avoid PSFCH transmission that overlaps with LTE SL Tx, although we can compromise to not avoid PSFCH transmission that overlaps with LTE SL Tx . We prefer to select either of the behaviour and not to have (pre-)configuration. It can be based on the majority based decision if technical discussion does not reach the conclusion. For always avoid PSFCH transmission that overlaps with LTE SL Tx, we think both TX UE and RX UE should avoid the overlap. i.e. the PSCCH/PSSCH TX UE avoids selecting PSFCH resources, and the PSCCH/PSSCH RX UE does not transmit on PSFCH resources, that overlap with LTE SL transmissions.

Proposal 11: For PSFCH handling, to avoid PSFCH transmission that overlaps with LTE SL Tx.

Proposal 12: PSCCH/PSSCH TX UE should avoid selecting PSFCH resources, and the PSCCH/PSSCH RX UE should not transmit on PSFCH resources, that overlap with LTE SL transmissions.

# Conclusion

In this contribution, followings proposals are made: (To be updated later)

Proposal 1: For V2X usage perspective, co-existence of type A and type C are sufficient (with the high priority). The co-existence among type A, B and C can be considered for SL use cases other than V2X (with lower priority).

Proposal 2: Type A devices should be capable to receive both LTE SL and NR SL simultaneously.

Proposal 3: For SL UEs capable to perform simultaneous TX of LTE and NR SLs, some optimization on dynamic power sharing may also be considered.

Proposal 4: The configuration of the dynamic sharing resource pools may have two possibilities: – 1) same resource pool for LTE SL and NR SL, and 2) overlapped but separated configuration for LTE and NR SL. The kind of resource pool configuration needs to be clarified.

Proposal 5: We support Alt 1 to have at most Tvalid ms prior to the time of sharing the information 2 for the latest LTE SL subframe used to determine the information shared with the NR SL module. The Tvalid includes T.

Proposal 6: Assuming all LTE PHY layer sensing information are transparent to NR module, then the information would be treated same as NR sensing, i.e., non-proper resources from LTE sensing (by priority, SCI, etc.) are excluded to each X%.

Proposal 7: Inter-UE coordination can be used for there are both Type B UE and Type A UE in the resource pool (e.g., if RSU is Type A device and others are Type B devices in rel.17). Type A devices may broadcast its LTE sensing results (from other LTE UEs) as “non-preferred resource” as inter-UE coordination so that other Type A devices and Type B devices would try to avoid such resources. Type B devices may have lower priority compared with type A devices.

Proposal 8: For Type A devices, it may indicate their own reservation with both LTE and NR SCIs (at least for type C devices). Alternatively, for in-coverage UEs, they can use gNB to relay the information via UL and DL.

Proposal 9: For in-coverage UEs, they can use gNB to relay the sensing information via UL and then DL to targeted UEs.

Proposal 10: For an LTE/NR shared resource pool, it could be specified that periodic reservation of LTE V2X is used for LTE V2X, and the remaining resource is used for NR V2X. The dynamically scheduled NR SL transmissions may be prioritized over LTE even with lower priority.

Proposal 11: For PSFCH handling, to avoid PSFCH transmission that overlaps with LTE SL Tx.

Proposal 12: PSCCH/PSSCH TX UE should avoid selecting PSFCH resources, and the PSCCH/PSSCH RX UE should not transmit on PSFCH resources, that overlap with LTE SL transmissions.

# Reference

[1] RP-222806, Revised “Work Item Description: NR sidelink evolution,” OPPO, RAN#98-e

[2] TR38.885: Study on NR Vehicle-to-Everything (V2X)

[3] Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0, Online meeting, 9th – 20th May 2022

[4] Final Report of 3GPP TSG RAN WG1 #110 v1.0.0, Toulouse, France, 22nd – 26th August 2022

[5] Final Report of 3GPP TSG RAN WG1 #110bis v1.0.0, Online meeting, 10th – 19th October 2022

[6] Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0, Toulouse, France, 14th – 18th November 2022

[7] R1- 2205588, FL Summary for AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink, Moderator (Fraunhofer HHI), RAN1#109-e

[8] R1-2208096, FL Summary EoM of AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink, Moderator (Fraunhofer HHI), RAN1#110

[9] R1- 2210298, FL Summary EoM for AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink, Moderator (Fraunhofer HHI), RAN1#110bis-e

[10] R1- 2212540, FL Summary EoM for AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink, Moderator (Fraunhofer HHI), RAN1#111