3GPP TSG-RAN WG1 Meeting #112	R1-2300917

Athens, Greece, February 27th – March 3rd, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TDRA for multiple PUSCHs scheduled by single DCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Table 6.1.2.1.1-1A, reference is made to the non-existent RRC parameter pusch-TimeDomainAllocationListForMultiPUSCH-17 for the case of multiple PUSCHs scheduled by a single DCI. This is due to the fact that RAN2 decided to reuse the parameter pusch-TimeDomainAllocationListForMultiPUSCH-r16 for multi-PUSCH scheduling with single DCI for both Rel-16 and Rel-17 functionality instead of introducing a new Rel-17 parameter. In addition in the field descriptions for the PUSCH-Config IE, 38.331 states that the parameter pusch-TimeDomainAllocationList for single-PUSCH scheduling shall not be configured simultaneously with pusch-TimeDomainAllocationListForMultiPUSCH. This conflicts with the final row of Table 6.1.2.1.1-1A which indicates “No/Yes” for the configuration of pusch-TimeDomainAllocationList. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removal of the RRC parameter name pusch- TimeDomainAllocationListForMultiPUSCH-17 from Table 6.1.2.1.1-1A in 38.214. - Change “No/Yes” to “No” in final row of Table 6.1.2.1.1-1A to reflect that pusch-TimeDomainAllocationList shall not be configured simultaneously with pusch-TimeDomainAllocationListForMultiPUSCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior for determing applicable TDRA table in the case of multiple-PUSCHs scheduled by single DCI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text omitted ***

##### 6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

*** Unchanged text omitted ***

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1 | pusch-Config includes pusch-TimeDomainAllocationList-ForMultiPUSCH | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No | Yes | - | pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config |
| No/Yes | No | - | Yes | pusch-TimeDomainAllocationList-ForMultiPUSCH provided in pusch-Config |

*** Unchanged text omitted ***