3GPP TSG RAN WG1 #112			   R1-2300992

Athens, Greece, February 27th – March 3rd, 2023

Source:	CMCC

Title:	Discussion on evaluation on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for:	Discussion and Decision

Introduction

In RAN1#111 meeting [1], the following working assumptions and agreements for evaluation methodology and KPIs have been approved.

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- The following case for generalization verification, can be optionally considered by companies:

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

    - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

    - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side

Agreement

- For the evaluation of the overhead for BM-Case1, adoption the following metrics:

  - RS overhead reduction,

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Companies report the assumption on additional measurements

Agreement

- Companies report the pattern of Set B.

- Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B).

Agreement

For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering

- Various Set B of beam(pairs)

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

- Option 2: Measurements of specific Rx beam(s)

  - Option 2a: Measurements of specific Rx beam(s) per model input sample

  - Option 2b: Measurements of specific Rx beam(s) for all model input sample

  - FFS how to select the specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

- Other options are not precluded and can be reported by companies.

Agreement

- For generalization performance verification, consider the following

  - Scenarios

    - Various deployment scenarios,

      - e.g., UMa, UMi and others,

      - e.g., 200m ISD or 500m ISD and others

      - e.g., same deployment, different cells with different configuration/assumption

      - e.g., gNB height and UE height

      - FFS: e.g., Carrier frequencies

    - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others

    - Various UE mobility,

      - e.g., 3km/h, 30km/h, 60km/h and others

  - Configurations (parameters and settings)

    - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions)

    - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions)

    - Various Set B of beam (pairs)

    - T1 for measurement /T2 for prediction for BM-Case2

  - Other scenarios/configurations(parameters and settings) are not precluded and can be reported by companies.

Agreement

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on additional measurements

    - FFS: Option 3: RS OH reduction%=1-NLM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance

      - where M is the total number of beams (pairs) to be predicted for each time instance

      - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

    - Companies report the assumption on T1 and T2 patterns

    - Other options are not precluded and can be reported by companies.

In this contribution, we concentrate on evaluation methodology and performance results of spatial-domain beam prediction (BM-Case1).

Evaluation methodology

In RAN1#110bis-e meeting, two different options on the selection of Set B of beams (pairs) have been agreed. One option is that Set B is fixed across training and inference, another option is Set B is variable. For how to change the beams (pairs) in Set B, the following options have been provided in RAN1#111 meeting.

    - Opt A: Set B is changed following a set of pre-configured/pre-known candidates of Set B in pre-configured / pre-known order

    - Opt B: Set B is changed among pre-configured/ pre-known candidates of Set B

    - Opt C: Set B is changed among candidates of Set B which is subsets of Set A beams (pairs)

    - Opt D: Set B is changed among candidates of Set B which is subsets of Set C (a set of beams (pairs) for measurement)

In our view, both Opt B and Opt D can be considered. In practical, the Set B can be configured by the network, and the network can select or change the beams (pairs) in set B according to UE speed or history beam reporting. Opt D can be treated as a special case of Opt B when the pre-configured/pre-known patterns of Set B are limited by the beams of Set C.

Proposal 1: On selection of Set B of beams (pairs), study the following options when Set B is variable

-  Set B is changed among pre-configured/pre-known candidates of Set B

-  Set B is changed among Set A beams (pairs) candidates of Set B which is subsets of Set C (a set of beams (pairs) for measurement)

For the evaluation assumption for generalization performance, various scenarios and configurations have been agreed as the assumptions. One option is various Set B of beam (pairs). We think if Set B is fixed, various Set B should be considered to evaluate whether the AI/ML model can be used for other patterns of Set B. If the Set B is variable, the generalization for different patterns is naturally considered.

KPI discussion

In RAN1#109-e meeting [2], the beam prediction accuracy (%) with 1dB margin for Top-1 beam has been defined as the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”. In RAN1#110bis e-meeting, the beam prediction accuracy (%) of Top-1, Top-K/1, Top-1/K were discussed. In our view, if the Top-1 genie-aided beam (pair) is not included in the Top-K predicted beam (pair), but the ideal highest L1-RSRP of the predicted Top-K beams (pairs) is within 1 dB of the L1-RSRP of the Top-1 genie-aided beam, the performance loss can be negligible. Therefore, we propose to define the KPI “Beam prediction accuracy (%) with 1dB margin for Top-K beam” as follows.

Proposal 2: The definition of beam prediction accuracy (%) with 1 dB margin for Top-K beams is:

- The percentage of “the ideal highest L1-RSRP of the predicted Top-K beams is within 1 dB of the L1-RSRP of the Top-1 genie-aided beam”

Evaluation results of BM-Case1

In this section, we provide the evaluation results of BM-Case1. In our simulation, both Tx-Rx beam pair prediction and DL Tx beam prediction are considered.

Evaluation assumption

The evaluation assumption in our simulations is given as follows.

Table 1. Evaluation assumption for BM-Case1

| Parameters | Values |
| --- | --- |
| UE distribution | - 20 UEs per cell for dataset generation - 80% indoor ,20% outdoor as in TR 38.901 |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ TXRU weights mapping: (Mp, Np, P, Mg, Ng) = (1,1,2,1,1) 64 Tx beams with: Azimuth angle φi = [-7*pi/16, -5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16, 7*pi/16] Zenith angle θj = [8*pi/16, 9*pi/16, 10*pi/16, 11*pi/16, 12*pi/16, 13*pi/16, 14*pi/16, 15*pi/16] |
| UE Antenna Configuration | (M,N,P) = (1,4,2)], (Mg, Ng) = (1, 1) TXRU weights mapping: (Mp, Np, P, Mg, Ng) = (1,1,2,1,1) 4 Rx beams with: Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8] Zenith angle θj = pi/2 |
| Set B selection | (NTx* NRx)= (4*4),(6,4), (8*4),(10,4), (12*4) |

Evaluation results of Tx-Rx beam pair prediction

Option 1: Set B is fixed

In this subsection, we show the beam prediction accuracy and RS overhead reduction (Option2) of Tx-Rx beam pair prediction when Set B is fixed. We consider two AI/ML models in the simulation. For one model, the model output is Top-K beam pair ID and corresponding L1-RSRP, for the other model, the model output is only the Top-K beam pair ID.

Three different beam pair patterns in Set B are considered: (NTx* NRx)= (4*4), (8*4), (12*4), where NTx is the number of Tx beams for measurement in Set B and NRx is the number of Rx beams used in Set B. Set A includes (MTx* MRx)=(64*4) beam pairs. Option 1 of exhaustive beam sweeping in Set A is considered as the baseline. The evaluation results of the two models are shown in Table 2.

Table 2. Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction

| Assumptions |  | Number of beam pairs in Set A | (MTx* MRx)=64*4 | (MTx* MRx)=64*4 |
| --- | --- | --- | --- | --- |
|  |  | Number of beam pairs in Set B | Config1: (NTx* NRx)= (4*4) Config2: (NTx* NRx)= (8*4) Config3: (NTx* NRx)= (12*4) | Config1: (NTx* NRx)= (4*4) Config2: (NTx* NRx)= (8*4) Config3: (NTx* NRx)= (12*4) |
|  |  | Baseline scheme | Option1 | Option1 |
| AI/ML model input/output |  | Model input | L1-RSRP of beam pairs in Set B | L1-RSRP of beam pairs in Set B |
|  |  | Model output | Top-K beam pair ID and corresponding L1-RSRP | Top-K beam pair ID |
| Data Size |  | Training | 39600 | 39600 |
|  |  | Testing | 3960 | 3960 |
| AI/ML model |  | Short model description | CNN | CNN |
|  |  | Model complexity | Config1 (4*4): 2.32×106 Config2 (8*4): 2.58×106 Config3 (12*4): 2.85×106 | Config1 (4*4): 1.35×106 Config2 (8*4): 1.61×106 Config3 (12*4): 1.87×106 |
|  |  | Computational complexity | Config1 (4*4): 1.93×106 Config2 (8*4): 3.84×106 Config3 (12*4): 3.75×106 | Config1 (4*4): 2.69×106 Config2 (8*4): 3.21×106 Config3 (12*4): 3.74×106 |
| Evaluation results [With AI/ML / baseline] | Beam prediction accuracy (%) | Top-1 (%) | Config1 (4*4): 57.6% Config2 (8*4): 73.9% Config3 (12*4): 82.3% baseline:100% | Config1 (4*4): 61.1% Config2 (8*4): 79.6% Config3 (12*4): 86% baseline:100% |
|  |  | Top-2/1 (%) | Config1 (4*4): 75.6% Config2 (8*4): 90.5% Config3 (12*4): 94.5% baseline:100% | Config1 (4*4): 79.1% Config2 (8*4): 93.1% Config3 (12*4): 96.1% baseline:100% |
|  |  | Top-3/1 (%) | Config1 (4*4): 83% Config2 (8*4): 94.5% Config3 (12*4): 96.9% baseline:100% | Config1 (4*4): 87.1% Config2 (8*4): 96.1% Config3 (12*4): 97.7% baseline:100% |
|  | RS overhead Reduction (%): 1-N/M with Option2 | Top-1 | Config1 (4*4): 93.4% Config2 (8*4): 87.1% Config3 (12*4): 80.9% |  |
|  |  | Top-2 | Config1 (4*4): 93% Config2 (8*4): 86.7% Config3 (12*4): 80.5% |  |
|  |  | Top-3 | Config1 (4*4): 92.6% Config2 (8*4): 86.3% Config3 (12*4): 80% |  |

From the results, it can be observed that with the increasing of K (from Top-1 to Top-3), the prediction accuracy of AI model improves significantly, while the increased beam sweeping overhead is small. Selecting an appropriate value of K can achieve a trade-off between prediction accuracy and beam sweeping overhead.

For beam measurement pattern of 8*4 and 12*4, compared with baseline option 1, when selecting Top-3 beam pairs from the AI model, the prediction accuracy of AI model is close to the result of exhaustive beam sweeping, but the beam sweeping overhead is much smaller. For example, for 12*4 measurement pattern, the two AI models respectively have 3.1% and 2.3% prediction accuracy loss compared to option 1, but can save 80% beam sweeping overhead.

Option 1: Set B is variable

In this subsection, we show the beam prediction accuracy and RS overhead reduction of Tx-Rx beam pair prediction when Set B is variable. We assume Set B can be selected from the five beam pair patterns: (NTx* NRx)= (4*4),(6,4), (8*4),(10,4), (12*4). The data samples of the five beam patterns are used for both training and testing. The input of the AI/ML model is L1-RSRP of all beam pairs in Set A, but only the L1-RSRP values of beam pairs in Set B are measured, L1-RSRP of other beam pairs are set as a particular value, e.g., 0. The evaluation results are shown in Table 3. The beam prediction accuracy/RS overhead in the table is the average value over the five beam pair patterns of Set B.

Table 3. Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction

| Assumptions |  | Number of beam pairs in Set A | (MTx* MRx)=64*4 |
| --- | --- | --- | --- |
|  |  | Number of beam pairs in Set B | Selected from the follow beam pair patterns: (NTx* NRx)= (4*4),(6,4),(8*4),(10,4), (12*4). |
|  |  | Baseline scheme | Option1 |
| AI/ML model input/output |  | Model input | L1-RSRP of all beam pairs in Set A (but only L1-RSRP of beam pairs in Set B is useful) |
|  |  | Model output | Top-K beam pair ID and corresponding L1-RSRP |
| Data Size |  | Training | 200000 |
|  |  | Testing | 20000 |
| AI/ML model |  | Short model description | CNN |
|  |  | Model complexity | 3.1×106 |
|  |  | Computational complexity | 29.9×106 |
| Evaluation results [With AI/ML / baseline] | Beam prediction accuracy (%) | Top-1 (%) | 71.7% baseline:100% |
|  |  | Top-2/1 (%) | 89% baseline:100% |
|  |  | Top-3/1 (%) | 93.5% baseline:100% |
|  | RS overhead Reduction (%): 1-N/M with Option 2 | Top-1 | 87.1% |
|  |  | Top-2 | 86.7% |
|  |  | Top-3 | 86.3% |

It can be seen that when Set B is variable, the AI/ML model can achieve satisfactory performance with the cost of higher model complexity and computational complexity. For Top-3 beam pair prediction, the AI model can achieve average 93.5% beam pair prediction accuracy and reduce 86.3% RS overhead.

Above all, for BM-Case1, AI based spatial beam prediction can largely reduce beam sweeping overhead with minor loss of prediction accuracy for Top-K beam pair.

Observation 1: Compared with baseline option 1, BM-Case 1 with Tx-Rx beam pair prediction has minor loss of prediction accuracy for Top-3 beam pair but has large beam sweeping overhead reduction.

Evaluation results of DL Tx beam prediction

In this subsection, we show the beam prediction accuracy and RS overhead reduction of DL Tx beam prediction when Set B is fixed. We consider two AI/ML models in the simulation. For one model, the model output is Top-K DL Tx beam ID and corresponding L1-RSRP, for the other model, the model output is Top-K DL Tx beam ID. Three different beam patterns in Set B are considered: NTx= 4,8,12. For Rx beam assumption, we consider the following three options.

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

- Option 2a: Measurements of specific Rx beam(s) per model input sample

- Option 2b: Measurements of specific Rx beam(s) for all model input sample

For Option2a, the specific Rx beam per model input sample is selected by a pre-defined order. For option 2b, the specific Rx beam for all model input sample is Rx beam #1.

Exhaustive beam sweeping in Set A is considered as the baseline. The Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams. The evaluation results with Rx beam assumption Option 1 are shown in Table 4.

Table 4. Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction with Rx beam assumption of Option 1

| Assumptions |  | Number of beams in Set A | 64 | 64 |
| --- | --- | --- | --- | --- |
|  |  | Number of beams in Set B | Config1: NTx= 4 Config2: NTx= 8 Config3: NTx= 12 | Config1: NTx= 4 Config2: NTx= 8 Config3: NTx= 12 |
|  |  | Baseline scheme | Option1 | Option1 |
| AI/ML model input/output |  | Model input | L1-RSRP measurement based on Set B | L1-RSRP measurement based on Set B |
|  |  | Model output | Top-K DL Tx beam ID and corresponding L1-RSRP | Top-K DL Tx beam ID |
| Data Size |  | Training | 39600 | 39600 |
|  |  | Testing | 3960 | 3960 |
| AI/ML model |  | Short model description | CNN | CNN |
|  |  | Model complexity | Config1 (NTx= 4): 1.30×106 Config2 (NTx= 8): 1.56×106 Config3 (NTx= 12): 1.82×106 | Config1 (NTx= 4): 1.30×106 Config2 (NTx= 8): 1.56×106 Config3 (NTx= 12): 1.82×106 |
|  |  | Computational complexity | Config1 (NTx= 4): 2.59×106 Config2 (NTx= 8): 3.12×106 Config3 (NTx= 12): 3.64×106 | Config1 (NTx= 4): 2.59×106 Config2 (NTx= 8): 3.12×106 Config3 (NTx= 12): 3.64×106 |
| Evaluation results [With AI/ML / baseline] | Beam prediction accuracy (%) | Top-1 (%) | Config1 (NTx= 4): 56.8% Config2 (NTx= 8): 86.5% Config3 (NTx= 12): 92.0% baseline:100% | Config1 (NTx= 4): 66.8% Config2 (NTx= 8): 83.5% Config3 (NTx= 12): 91% baseline:100% |
|  |  | Top-2/1 (%) | Config1 (NTx= 4): 75.2% Config2 (NTx= 8): 95.7% Config3 (NTx= 12): 98.5% baseline:100% | Config1 (NTx= 4): 81.2% Config2 (NTx= 8): 94.2% Config3 (NTx= 12): 97.9% baseline:100% |
|  |  | Top-3/1 (%) | Config1 (NTx= 4): 84.8% Config2 (NTx= 8): 97.6% Config3 (NTx= 12): 99.4% baseline:100% | Config1 (NTx= 4): 87.3% Config2 (NTx= 8): 97.0% Config3 (NTx= 12): 98.9% baseline:100% |
|  | RS overhead Reduction (%): 1-N /M with Option 2 | Top-1 | Config1 (4*4): 93.4% Config2 (8*4): 87.1% Config3 (12*4): 80.9% |  |
|  |  | Top-2 | Config1 (4*4): 93% Config2 (8*4): 86.7% Config3 (12*4): 80.5% |  |
|  |  | Top-3 | Config1 (4*4): 92.6% Config2 (8*4): 86.3% Config3 (12*4): 80% |  |

From the results in Table 4, it can be observed that with the increasing of K (from Top-1 to Top-3), the prediction accuracy of DL Tx beam increases significantly. For Top-2 and Top-3 DL Tx beam prediction of NTx= 8 and 12, the two AI models can achieve almost 95% and above beam pair prediction accuracy while reducing 80% and above RS overhead.

Observation 2: Compared with baseline option 1, BM-Case 1 with DL Tx beam prediction has minor loss of prediction accuracy for Top-2 and Top-3 beam prediction but has large beam sweeping overhead reduction.

The evaluation results of DL Tx beam prediction with Rx beam assumption of Option 2a and Option 2b are shown in Table 5 and Table 6, respectively.

Table 5. Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction with Rx beam assumption of Option 2a

| Assumptions |  | Number of beams in Set A | 64 | 64 |
| --- | --- | --- | --- | --- |
|  |  | Number of beams in Set B | Config1: NTx= 4 Config2: NTx= 8 Config3: NTx= 12 | Config1: NTx= 4 Config2: NTx= 8 Config3: NTx= 12 |
|  |  | Baseline scheme | Option1 | Option1 |
| AI/ML model input/output |  | Model input | L1-RSRP measurement based on Set B | L1-RSRP measurement based on Set B |
|  |  | Model output | Top-K DL Tx beam ID and corresponding L1-RSRP | Top-K DL Tx beam ID |
| Data Size |  | Training | 39600 | 39600 |
|  |  | Testing | 3960 | 3960 |
| AI/ML model |  | Short model description | CNN | CNN |
|  |  | Model complexity | Config1 (NTx= 4): 1.30×106 Config2 (NTx= 8): 1.56×106 Config3 (NTx= 12): 1.82×106 | Config1 (NTx= 4): 1.30×106 Config2 (NTx= 8): 1.56×106 Config3 (NTx= 12): 1.82×106 |
|  |  | Computational complexity | Config1 (NTx= 4): 2.59×106 Config2 (NTx= 8): 3.12×106 Config3 (NTx= 12): 3.64×106 | Config1 (NTx= 4): 2.59×106 Config2 (NTx= 8): 3.12×106 Config3 (NTx= 12): 3.64×106 |
| Evaluation results [With AI/ML / baseline] | Beam prediction accuracy (%) | Top-1 (%) | Config1 (NTx= 4): 39.6% Config2 (NTx= 8): 71.8% Config3 (NTx= 12): 85.3% baseline:100% | Config1 (NTx= 4): 55.1% Config2 (NTx= 8): 76.1% Config3 (NTx= 12): 85.9% baseline:100% |
|  |  | Top-2/1 (%) | Config1 (NTx= 4): 57.3% Config2 (NTx= 8): 88% Config3 (NTx= 12): 94.2% baseline:100% | Config1 (NTx= 4): 70.5% Config2 (NTx= 8): 89.5% Config3 (NTx= 12): 94.6% baseline:100% |
|  |  | Top-3/1 (%) | Config1 (NTx= 4): 68.2% Config2 (NTx= 8): 92.4% Config3 (NTx= 12): 96.4% baseline:100% | Config1 (NTx= 4): 78.1% Config2 (NTx= 8): 93.0% Config3 (NTx= 12): 96.6% baseline:100% |
|  | RS overhead Reduction (%): 1-N /M with Option 2 | Top-1 | Config1 (4*4): 93.4% Config2 (8*4): 87.5% Config3 (12*4): 81.3% |  |
|  |  | Top-2 | Config1 (4*4): 90.6% Config2 (8*4): 84.4% Config3 (12*4): 78.1% |  |
|  |  | Top-3 | Config1 (4*4): 89.1% Config2 (8*4): 82.9% Config3 (12*4): 76.6% |  |

Table 6. Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction with Rx beam assumption of Option 2b

| Assumptions |  | Number of beams in Set A | 64 | 64 |
| --- | --- | --- | --- | --- |
|  |  | Number of beams in Set B | Config1: NTx= 4 Config2: NTx= 8 Config3: NTx= 12 | Config1: NTx= 4 Config2: NTx= 8 Config3: NTx= 12 |
|  |  | Baseline scheme | Option1 | Option1 |
| AI/ML model input/output |  | Model input | L1-RSRP measurement based on Set B | L1-RSRP measurement based on Set B |
|  |  | Model output | Top-K DL Tx beam ID and corresponding L1-RSRP | Top-K DL Tx beam ID |
| Data Size |  | Training | 39600 | 39600 |
|  |  | Testing | 3960 | 3960 |
| AI/ML model |  | Short model description | CNN | CNN |
|  |  | Model complexity | Config1 (NTx= 4): 1.30×106 Config2 (NTx= 8): 1.56×106 Config3 (NTx= 12): 1.82×106 | Config1 (NTx= 4): 1.30×106 Config2 (NTx= 8): 1.56×106 Config3 (NTx= 12): 1.82×106 |
|  |  | Computational complexity | Config1 (NTx= 4): 2.59×106 Config2 (NTx= 8): 3.12×106 Config3 (NTx= 12): 3.64×106 | Config1 (NTx= 4): 2.59×106 Config2 (NTx= 8): 3.12×106 Config3 (NTx= 12): 3.64×106 |
| Evaluation results [With AI/ML / baseline] | Beam prediction accuracy (%) | Top-1 (%) | Config1 (NTx= 4): 43.8% Config2 (NTx= 8): 72.1% Config3 (NTx= 12): 84.8% baseline:100% | Config1 (NTx= 4): 53.4% Config2 (NTx= 8): 75.5% Config3 (NTx= 12): 86.1% baseline:100% |
|  |  | Top-2/1 (%) | Config1 (NTx= 4): 58.9% Config2 (NTx= 8): 89% Config3 (NTx= 12): 94.1% baseline:100% | Config1 (NTx= 4): 69.3% Config2 (NTx= 8): 89.0% Config3 (NTx= 12): 94.8% baseline:100% |
|  |  | Top-3/1 (%) | Config1 (NTx= 4): 68.8% Config2 (NTx= 8): 93.5% Config3 (NTx= 12): 96.7% baseline:100% | Config1 (NTx= 4): 77.7% Config2 (NTx= 8): 93.3% Config3 (NTx= 12): 97.0% baseline:100% |

Compared with the results in Table 4-6, it can be observed that DL Tx beam prediction with the “best” Rx beam achieves higher prediction accuracy than that of DL Tx beam prediction with specific Rx beam. However, if a proper number of beams in Set B (e.g., 8 or 12) is selected, the beam prediction accuracy for Top-3 DL Tx beam prediction can also be greater than 90%.

Observation 3: For BM-Case1, DL Tx beam prediction with the “best” Rx beam achieves higher prediction accuracy than that of DL Tx beam prediction with specific Rx beam.

Conclusion

In this contribution, we have presented our views on evaluation methodology and performance results of BM-Case1. The following observations and proposals are made.

Observation 1: Compared with baseline option 1, BM-Case 1 with Tx-Rx beam pair prediction has minor loss of prediction accuracy for Top-3 beam pair but has large beam sweeping overhead reduction.

Observation 2: Compared with baseline option 1, BM-Case 1 with DL Tx beam prediction has minor loss of prediction accuracy for Top-2 and Top-3 beam prediction but has large beam sweeping overhead reduction.

Observation 3: For BM-Case1, DL Tx beam prediction with the “best” Rx beam achieves higher prediction accuracy than that of DL Tx beam prediction with specific Rx beam.

Proposal 1: On selection of Set B of beams (pairs), study the following options when Set B is variable

-  Set B is changed among pre-configured/pre-known candidates of Set B

-  Set B is changed among Set A beams (pairs) candidates of Set B which is subsets of Set C (a set of beams (pairs) for measurement)

Proposal 2: The definition of beam prediction accuracy (%) with 1 dB margin for Top-K beams is:

- The percentage of “the ideal highest L1-RSRP of the predicted Top-K beams is within 1 dB of the L1-RSRP of the Top-1 genie-aided beam”

References

- 3GPP RAN1#111, RAN1 Chairman’s Notes, Toulouse, France, November 2022.

- 3GPP RAN1#109-e, RAN1 Chairman’s Notes, May 2022.