3GPP TSG RAN WG1 Meeting #112			R1-2301097

Athens, Greece, February 27 – March 03, 2023

Agenda item:	9.2.2.1

Title:	Evaluation of joint CSI estimation and compression with AI/ML

Source:	Beijing Jiaotong University

Document for:	Discussion

1. Introduction

In RAN 94-e meeting, new study item on AI/ML for NR Air Interface was approved [1]. In the study item scope, the initial set of use cases were identified:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this paper, we provide preliminary evaluation result for joint CSI estimation and compression with AI/ML in another paper [2].

2. Preliminary results

2.1 Simulation assumptions

To simulate the proposed method in another paper [2], we follow the proposal [3] and adopt CDL-C to generate the uplink CSI and downlink CSI. Table 1 describes the detailed parameter configuration of our simulation.

The uplink CSI and downlink CSI are generated by Matlab in which the downlink center frequency is 2 GHz and the uplink center frequency is 1.9 GHz. The number of the antennas at the gNB is  Nt Nt = 32 and the number of the antennas at the UE is Nr =1. The uplink and the downlink both have Nc = 256 subcarriers. The training, validation, and testing sets contain 100,000, 30,000, and 20,000 samples, respectively. A sample pair contains a downlink CSI sample and an uplink CSI sample.

Table 1: Simulation assumptions

| Parameter | Values |
| --- | --- |
| Dulex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz for downlink, 1.9GHz for uplink |
| Bandwidth | 10MHz |
| Nt | 32: (8,8,2, 1, 1,2,8), (dH.dV) = (0.5, 0.8)λ |
| Nr | 1: (1,1,1,1, 1, 1, 1), (dH.dV) = (0.5, 0.5) λ |
| Channel model | CDL-C |
| UE speed | 0 km/h |
| Delay spread | 30 ns |

Besides, the pilot length in time domain is L=16, the pilot interval in frequency domain is Ni=4, and the number of subcarriers used in the uplink Nc'=32.

2.2 Simulation results

Fig.1 shows the performance of the DL-based joint CSI estimation and compression in the CDL-C scenario with NMSE. The “joint” label represents the proposed method. The “separate” label represents the separation based channel estimation and CSI feedback method, consisting of the channel estimation method proposed in [4] and the symbol-level CSI feedback method proposed in [5]. The performance of both the joint method and the separate method are increased with the increase of SNRdl. Comparing the joint method with the separate method at the same SNRdl, the performance of the joint method is better than that of the separate method. With the increase of SNRul, the performance gap is enlarged. Specifically, the NMSE of “joint (SNRdl=-10 dB)” is 0.9 dB better than that of “separate (SNRdl=-10 dB)” when SNRul=-10 dB, and the NMSE of “joint (SNRdl=-10 dB)” is 6.2 dB better than that of “separate (SNRdl=-10 dB)” when SNRul=10 dB.

Figure 1: Evaluation for the DL based joint CSI estimation and compression method. The label “joint(SNRdl=-10 dB)” means the method with downlink SNR=-10 dB, i.e., the channel estimation SNR. The label “separate(SNRdl=-10 dB)” means that the method proposed in [4] is adopted as channel estimation method and the method proposed in [5] is adopted as CSI feedback method. SNRul is the uplink SNR, i.e., the CSI compression SNR.

The FLOPs and parameter size of the joint method and the separate method is shown in Table 2. Both the FLOPs and the parameters of the joint method are smaller than that of the separate method. Specifically, the FLOPs of the joint method is 4 orders of magnitude smaller than that of the at the UE side, while the FLOPs of the joint method is only 0.1M smaller than that that of the at the BS side. Considering about the parameter size, the parameters of the joint method are almost 1/2 and 2/5 of that of the separate method at the UE and the BS, respectively.

| Type | devices | Joint Method | Separate Method |
| --- | --- | --- | --- |
| FLOPs | UE | 1.08 M | 9,233 M |
|  | BS | 57.82 M | 57.92 M |
| parameters | UE | 131,548 | 870,792 |
|  | BS | 158,410 | 256,682 |

Based on the above simulation results, we have the following proposal:

Proposal #1: The performance, FLOPs and parameters of the joint channel estimation and CSI compression method are better than the separated channel estimation and CSI compression method.

3. Conclusion

This paper provides preliminary evaluation result in another paper [2] for joint CSI estimation and compression with AI/ML.

Proposal #1: The performance, FLOPs and parameters of the joint channel estimation and CSI compression method are better than the separated channel estimation and CSI compression method.

4. References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] R1-220XXX (申请之后填写), joint CSI estimation and compression with AI/ML

[3] R1-2205491, Summary#4 of [109-e-R18-AI/ML-03] Moderator (Huawei)

[4] M. B. Mashhadi and D. Gündüz, "Pruning the Pilots: Deep Learning-Based Pilot Design and Channel Estimation for MIMO-OFDM Systems," IEEE Transactions on Wireless Communications, vol. 20, no. 10, pp. 6315-6328, 2021.

[5] J. Xu, B. Ai, N. Wang N and W. Chen, “Deep Joint Source-Channel Coding for CSI Feedback: An End-to-End Approach”, IEEE Journal on Selected Areas in Communications, vol. 41, no. 1, pp. 260-273, 2023.