3GPP TSG RAN WG1 #112							            R1-2301219

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.2

Source:	AT&T

Title:	Views on CSI Enhancements for CJT

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on MIMO Evolution for Downlink and Uplink [1], 3GPP has agreed on a list of goals related to CSI enhancements. In particular, the 4th objective is devoted to CSI enhancements for coherent JT. This contribution focuses on CSI enhancements devoted to CJT.

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

In this contribution, we continue to discuss our views on CSI enhancements for coherent JT.

# CSI Enhancement for CJT-mTRP

## W2 quantization

In RAN1#110bis-e [2], it was agreed to down select from the two alternatives of W2 quantization and SCI design:

Alt1 provides the highest overhead reduction since it comprises a single amplitude group per polarization (Cgroup,amp=2). Alt3 provides higher resolution as it comprises one amplitude group per polarization for each CSI-RS resource group. Alt3 might be more flexible and can accommodate the distributed scenarios (non-colocated) of CJT-mTRP because the amplitude groups can be used to convey the receive power difference between the distributed TRPs. However, the SLS in [2] showed a minor discrepancy gain between Alt1 and Alt3. Our current position is to support Alt1 and will study the new SLS results to see whether there is a change in the gain behavior.

Proposal 1: Support Alt1 for the design W2 quantization group and Strongest Coefficient Indicator (SCI) design

## Offset Selection for FD Basis

In Rel-18 Type-II codebook Mode-1, the FD basis for each CSI resource is selected independently. Based on the agreement in RAN1#111 [3], it was agreed to down select only one from three alternatives:

- Alt1 gives an optimized version by having a common FD basis selection across all the CSI-RS resources scaled with per CSI-RS resource FD offset. This is expected to have the highest overhead reduction due to the common FD basis.

- In Alt2, FD basis are independently selected across the N CSI-RS resources without any per-CSI-RS-resource FD basis selection offset.

- Alt3 provides an individual offset relative to the per CSI resource FD basis that are independently selected across N CSI-RS resources.

Alt1 gives an optimized scheme (overhead) by having a common FD basis selection across all the CSI-RS resources scaled with the per CSI-RS resource FD offset. Alt1 better suits the scenarios of co-located CJT-mTRPs where channel (for each CSI-RS resource) is likely to have similar mutual frequency/delay characteristics with the channels of the rest of the TRPs.

Alt2 & Alt3 can be viewed as functionally equivalent since the FD basis are independently selected across N CSI-RS resources. The essence of the R18 Mode-1 Type II codebook is the independent selection of the FD basis across N CSI-RS resources and it is devoted to the distributed or non-co-located CJT-mTRP scenarios since the channels may not have similar frequency\delay characteristics due the different locations of the cooperating TRPs. So Alt2 & Alt3 offers more flexibility with higher resolution of FD basis and offset.

During the offline discussion, Alt2 & Alt3 were merged into Alt2 in Offline proposal 1.B.1.

Observation 1: Alt1 (Offline proposal 1.B.1) provides an optimized scheme for the scenarios of co-located CJT-mTRPs where channel for each CSI-RS resource is likely to have common mutual frequency/delay characteristics.

Observation 2: Alt2 (Offline proposal 1.B.1) provides a higher resolution since the FD basis are independently selected across N CSI-RS resources that is needed in the distributed or non-co-located CJT-mTRP scenario as the channels may have diverse frequency/delay characteristics due the different locations of the cooperating TRPs.

Proposal 2: Support both Alt1 & Alt2 from Offline proposal 1.B.1 for the FD Basis and Offset Selections in RAN1#112

## Parameter Combination for Rel-18 Type II

### SD basis (NL, L1, ..., LNTRP)

In RAN1#111 [3], it was agreed that the number of NL combinations for values of the SD basis {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling and when NL>1, the UE reports the selected combinations of {L1, ..., LNTRP} in CSI part 1. If an index is used to link between the parameters of the SD basis (NL, L1, ..., LNTRP) and the compression parameters ( & pv), it could lead to an overlong table with potentially unnecessary redundancy since each index value needs to be associated to multiple combinations of  & pv (for example see the snapshot below for Rel-16 Table 5.2.2.2.6-1). In Figure 1, the permitted combinations of (NL, L1, ..., LNTRP) are grouped with an associated index (Part A), then the index is associated to chosen values of ( & pv).

An alternative approach is to follow the legacy of Rel-16 and use a linkage between the parameters of the SD basis (NL, L1, ..., LNTRP) and compression parameters ( & pv). The linkage parameter can be the Lmax n=1NLn≤Lmax which can be used to group more combinations of {L1, ..., LNTRP} such that n=1NLn≤Lmax, then construct the Rel-18 codebook parameter configurations for Lmax,pv, & β as shown in Figure 2. This can reduce the size of the mapping table, redundancy, and complexity; thus optimize the overhead.

Another important aspect related to NL is that it should not should not exceed a predefined maximum value (NL,max) to avoid complexity at the UE end.

Figure 1: Mapping between (NL, L1, ..., LNTRP) & (pv, β) using the Index

Figure 2: Mapping between (NL, L1, ..., LNTRP) & (pv, β) using the Lmax

Observation 3: Using the Index to link between the parameters of the SD basis (NL, L1, ..., LNTRP) and compression parameters ( & pv) could lead to an overlong table with unnecessary redundancy

Proposal 3: Support adding a linkage parameter Lmax such that n=1NLn≤Lmax between the parameters of the SD basis (NL, L1, ..., LNTRP) and the compression parameters ( & pv) to construct the Rel-18 codebook parameter configurations for Lmax,pv, & β.

Proposal 4: Support defining a maximum value (NL,max) for the number of NL of the combinations of the SD basis {L1, ..., LNTRP} to avoid higher complexity at the UE end.

### The Compression parameters β & pv

The parameters  & pv are used to optimize the overhead where the parameter pv is used to control the amount of FD compression and the parameter is used to control the size of the NZC selected by the UE. Therefore, the trade-off between performance & overhead can vary as a function of these two parameters  & pv. We are open to add additional values of ( & pv) to improve the performance of the Rel-18 Type-II codebook and reduce the overhead

Proposal 5: Support adding new values of ( & pv) to further optimize the performance of Rel-18 Type-II codebook and reduce overhead

# Conclusion

In this contribution, we propose and observe the following:

Proposal 1: Support Alt1 for the design W2 quantization group and Strongest Coefficient Indicator (SCI) design

Proposal 2: Support both Alt1 & Alt2 from Offline proposal 1.B.1 for the FD Basis and Offset Selections in RAN1#112

Proposal 3: Support adding a linkage parameter Lmax such that n=1NLn≤Lmax between the parameters of the SD basis (NL, L1, ..., LNTRP) and the compression parameters ( & pv) to construct the Rel-18 codebook parameter configurations for Lmax,pv, & β.

Proposal 4: Support defining a maximum value (NL,max) for the number of NL of the combinations of the SD basis {L1, ..., LNTRP} to avoid higher complexity at the UE end.

Proposal 5: Support adding new values of ( & pv) to further optimize the performance of Rel-18 Type-II codebook and reduce overhead

Observation 1: Alt1 (Offline proposal 1.B.1) provides an optimized scheme for the scenarios of co-located CJT-mTRPs where channel for each CSI-RS resource is likely to have common mutual frequency/delay characteristics.

Observation 2: Alt2 (Offline proposal 1.B.1) provides a higher resolution since the FD basis are independently selected across N CSI-RS resources that is needed in the distributed or non-co-located CJT-mTRP scenario as the channels may have diverse frequency/delay characteristics due the different locations of the cooperating TRPs.

Observation 3: Using the Index to link between the parameters of the SD basis (NL, L1, ..., LNTRP) and compression parameters ( & pv) could lead to an overlong table with unnecessary redundancy

# Reference

- RP-213598 WID-MIMO Evolution for Downlink and Uplink.

- Chairman’s notes, RAN1#110bis-e, e-Meeting, October 10th – 19th, 2022

- Chairman’s notes, RAN WG1 #111, Toulouse, France, November 14th – 18th, 2022

- Chairman’s notes, RAN1#109e, May 9th – 20th.

- Chairman’s notes, RAN1#110, Toulouse, France, August 22nd – 26th, 2022