3GPP TSG RAN WG1 #112	        		               R1-2301228

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	5

Source: 	Samsung

Title: 	Discussion on LS related to reduced 1024 QAM capability

Document for:	Discussion and Decision

Introduction

In [1], RAN2 sent LS to RAN1 to receive the feedback regarding the impact due to the introduction of reduced 1024QAM capability (pdsch-1024QAM-2MIMO-FR1-r17), that is, 1024QAM UE capability with supporting up to 2 MIMO layers. Followings are agreements and assumptions that were made in RAN2, and some of them need RAN1 confirmation.

- [1] Clarify that UE shall at most report one of pdsch-1024QAM-2MIMO-FR1-r17and pdsch-1024QAM-FR1-r17. - [2] Assume to Extend supportedModulationOrderDL to include 1024 QAM (confirm with R1). - [3] The MIMO layer for 1024 QAM is Min (2, maxNumberMIMO-LayersPDSCH) for the CC where 1024 QAM is reported - [4] Assume that Max data rate shall be derived from the higher data rate between 1024 QAM or 256 QAM for CC where 1024 QAM is indicated and the UE support reduced 1024 capability (confirm with R1) - [5] Assume to Clarify that both scalingFactor and scalingFactor-1024QAM-FR1-r17 can be included for in one per CC capability and legacy scalingFactor is used when non-1024 QAM is scheduled (confirm with R1)

In this contribution, it will discuss bullets 2, 4 and 5.

Discussion

For bullet 2, the main motivation of extending the field to include 1024QAM is to provide forward compatibility when higher modulation order than 1024QAM is supported in future. However, it is note that current specification doesn’t have any issue, and the time of supporting higher modulation than 1024QAM is unknown. The issue is not critical issue at this stage. In this sense, bullet 2 can be discussed later with other candidate solutions when higher modulation than 1024QAM is discussed in future.

Proposal 1: It is premature to confirm bullet 2 from RAN1 understanding.

For bullet 4, it is understood that RAN2 agreed to avoid the situation where 1024QAM with 2 MIMO layers should not be considered for calculating supported max data rate since 256QAM with 4 MIMO layers provide higher data rate. However, the concern situation does not seem to happen in current specification. According to the definition of supportedModulationOrderDL shown in table below, UE would assume 256QAM for the max data rate calculation even though the UE reports pdsch-1024QAM-2MIMO-FR1-r17.

supportedModulationOrderDL Indicates the maximum supported modulation order to be applied for downlink in the carrier in the max data rate calculation as defined in 4.1.2. If included, the network may use a modulation order on this serving cell which is higher than the value indicated in this field as long as UE supports the modulation of higher value for downlink. If not included: - for FR1, the network uses the modulation order signalled per band i.e. [pdsch-1024QAM-FR1] when [pdsch-1024QAM-FR1] is signalled for the band, otherwise the network uses the modulation order signalled in pdsch-256QAM-FR1. - for FR2, the network uses the modulation order signalled per band i.e. pdsch-256QAM-FR2 if signalled. If not signalled in a given band, the network shall use the modulation order 64QAM. In all the cases, it shall be ensured that the data rate does not exceed the max data rate (DataRate) and max data rate per CC (DataRateCC) according to TS 38.214 [12].

Since 4 MIMO layers is mandatory capability in FR1 for single CC standalone NR, there is no ambiguity issue. 256QAM with 4 MIMO layers is considered to calculate the max data rate even though the UE reports pdsch-1024QAM-2MIMO-FR1-r17. For multiple CC, it is understood that maxNumberMIMO-LayersPDSCH would be applicable for both 256QAM and 1024QAM. The main motivation of introducing pdsch-1024QAM-2MIMO-FR1-r17 is that it is likely that the UE couldn’t support 1024QAM with 4MIMO layers in single CC standalone NR. Therefore, the case where a UE simultaneously supports multiple CCs and pdsch-1024QAM-2MIMO-FR1-r17 needs further discussion in RAN1.

Proposal 2:  Current specification doesn’t have any problems for calculating max data rate. Thus, it is unnecessary to confirm bullet 4. However, it needs to discuss whether pdsch-1024QAM-2MIMO-FR1-r17 can be considered or not in case of multiple CC.

For bullet 5, it is shown that scalingFactor-1024QAM-FR1-r17 is considered for max data rate calculation when 1024QAM for PDSCH is signaled in the band in the table below. scalingFactor is used for max data rate calculation. It needs further clarification on “legacy scalingFactor is used when non-1024 QAM is scheduled” explained in RAN2 agreements. Based on the current specification, scalingFactor is applied when Q value in the max data rate equation is modulation order other than 1024QAM, while scalingFactor-1024QAM-FR1-r17 is applied when Q value is 1024QAM. Thus, it should be discussed first before deciding whether to confirm bullet 5 or not.

scalingFactor-1024QAM-FR1-r17 Indicates the scaling factor to be applied to the band in the max data rate calculation for 1024-QAM as defined in 4.1.2 when support of 1024-QAM for PDSCH is signalled for the band. Value f0p4 indicates the scaling factor 0.4, f0p75 indicates 0.75, and so on. If absent, the scaling factor 1 is applied to the band in the max data rate calculation for 1024-QAM. UE indicating support of this feature shall also indicate support of pdsch-1024QAM-FR1-r17 or pdsch-1024QAM-2MIMO-FR1-r17 to the band.

Proposal 3:  RAN1 needs to discuss whether two different scaling factors can be used in the same band for the max data rate calculation.

Conclusions

The followings are proposed in this contribution in order that RAN1 sends reply LS to RAN2.

Proposal 1: It is premature to confirm bullet 2 from RAN1 understanding.

Proposal 2:  Current specification doesn’t have any issue for calculating max data rate. Thus, it is unnecessary to confirm bullet 4. However, it needs to discuss whether pdsch-1024QAM-2MIMO-FR1-r17 can be considered in case of multiple CC.

Proposal 3:  RAN1 needs to discuss whether two different scaling factors can be used in the same band for the max data rate calculation.

- References

[1] R1-2300019, “LS on reduced 1024 QAM capability”