3GPP TSG RAN WG1 #112	        		  R1-2301229

# Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | Draft | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Beam Failure Recovery related to unified TCI state framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#107-e, it was agreed only for intra-cell beam management to support Rel-16 SCell BFR, but the spec doesn’t limit the scope of SCell BFR to intra cell beam management. Agreement On Rel-17 unified TCI framework, for intra-cell beam management, after X symbols from the UE receives the BFRR from NW, the UE assumes the same QCL parameter as the ones associated with the index q new for all PDSCH /PDCCH receptions in a CC, as well as other signals/channels configured to sharing the same indicated Rel-17 TCI state as PDSCH /PDCCH reception. - The above applies to Rel-15 SpCell BFR , Rel-16 CBRA based SpCell BFR , and Rel-16 SCell BFR - Note: q new is a candidate beam identified by the UE in set q1. q1 is the set of candidate beams Clarify that For SCell BFR, PDCCHs are monitored on the SCell(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add phrase “if SSB -MTC-AdditionalPCI is not provided” to limit the scope of SCell BFR to intra-cell beam management. Add phrase “on the SCell (s) indicated by the MAC CE” to indicate that PDCCHs monitoring is on the SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is not aligned with RAN1 agreements. Spec is ambiguos for the monitoring of PDCCHes in case of SCell BFR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

# 6	Link recovery procedures

## *** Unchanged text is omitted ***

If a UE is provided dl-OrJoint-TCIStateList or TCI-UL-State indicating a unified TCI state, after 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	if SSB -MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, on the SCell (s) indicated by the MAC CE, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as the one corresponding to qnew, if any, and using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate