3GPP TSG RAN WG1 #112    	                                              R1-2301231

Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on power control parameters for multiple aperiodic SRS resource sets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the case of more than one aperiodic SRS resource set used for antenna switching, the more than one SRS resource set should be configured with the same power control parameters to appropriately acquire DL CSI in the gNB side. In the current specification, this has been captured only for 1T4R, but it is omitted in other possible cases having multiple aperiodic SRS resource sets (e.g., 1T2R, 1T6R, 1T8R, 2T4R, 2T6R, 2T8R, or 4T8R). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuring the same power control parameters for more than one aperiodic SRS set of antenna switching shall be captured in the missed cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The more than one aperiodic SRS set of antenna switching with certain xTyR cases could be transmitted with different power control parameters, and then different parts of DL channel are estimated in the gNB side with different transmission power in the UE side, hence the performance of DL CSI acquisition may be degraded. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged parts are omitted>

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r6' for 1T6R, 't1r8' for 1T8R, 't2r6' for 2T6R, 't2r8' for 2T8R, 't4r8' for 4T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R):

-	For 1T2R, if the UE is indicating srs-AntennaSwitching2SP-1Periodic and/or srs-ExtensionAperiodicSRS:

-	when the UE is indicating srs-AntennaSwitching2SP-1Periodic only, then up to two SRS resource sets with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set with resourceType in SRS-ResourceSet set to 'periodic' can be configured, or up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet can be configured, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set.

-	when the UE is indicating srs-ExtensionAperiodicSRS only, then up to two SRS resource sets with resourceType in SRS-ResourceSet set to 'aperiodic' and up to one SRS resource set with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' can be configured, or up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet can be configured. In the case of two resource sets with resourceType in SRS-ResourceSet set to 'aperiodic', a total of two SRS resources are transmitted in different symbols of two different slots, the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port pair and the two sets are each configured with one SRS resource. In the case of the one resource set with resourceType in SRS-ResourceSet set to 'aperiodic' is configured, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. Each SRS resource set with 'resourceType' in SRS-ResourceSet set to 'periodic' or 'semi-persistent' has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set.

-	zero or one or two SRS resource sets configured with different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. An SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, and

-	zero or one SRS resource set with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating srs-ExtensionAperiodicSRS, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS, where in the case of one resource set, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. In the case of two resource sets, a total of two SRS resources are transmitted in different symbols of two different slots, the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with one SRS resource, or

-	otherwise, for 1T2R, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or

-	For 2T4R, if the UE is indicating srs-AntennaSwitching2SP-1Periodic and/or srs-ExtensionAperiodicSRS:

-	when the UE is indicating srs-AntennaSwitching2SP-1Periodic only, then up to two SRS resource sets with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set with resourceType in SRS-ResourceSet set to 'periodic' can be configured, or up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet can be configured, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource.

-	when the UE is indicating srs-ExtensionAperiodicSRS only, then up to two SRS resource sets with resourceType in SRS-ResourceSet set to 'aperiodic' and up to one SRS resource set with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' can be configured, or up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet can be configured. In the case of two resource sets with resourceType in SRS-ResourceSet set to 'aperiodic', a total of two SRS resources are transmitted in different symbols of two different slots, the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair and the two sets are each configured with one SRS resource. In the case of one resource set with resourceType in SRS-ResourceSet set to 'aperiodic' is configured, a total of two SRS resources transmitted in different symbols in the same slot, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource. Each SRS resource set with 'resourceType' in SRS-ResourceSet set to 'periodic' or 'semi-persistent' has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource.

-	zero or one or two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, and,

-	zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating srs-ExtensionAperiodicSRS, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS, where in the case of one resource set, a total of two SRS resources transmitted in different symbols in the same slot, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource. In the case of two resource sets, a total of two SRS resources are transmitted in different symbols of two different slots, the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. The two sets are each configured with one SRS resource, or

-	otherwise, up to two SRS resource sets configured with a different value for the higher layer parameter  resourceType in SRS-ResourceSet, where each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, or

-	For 1T4R, if the UE is indicating srs-AntennaSwitching2SP-1Periodic and/or srs-ExtensionAperiodicSRS and/or srs-OneAP-SRS,

-	zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource is associated with a different UE antenna port, and

-	zero or two SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic', if the UE is not indicating srs-ExtensionAperiodicSRS or srs-OneAP-SRS, or zero or one or two or four SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS and srs-OneAP-SRS, or zero or two or four SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS only, or zero or one or two SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-OneAP-SRS only. In the case of one resource set, a total of four SRS resources are transmitted in different symbols in the same slot, and the SRS port of each resource in the given set is associated with a different UE antenna port. In the case of two resource sets, a total of four SRS resources are transmitted in different symbols of two different slots, and the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. In the case of four resource sets, if UE is capable if supporting four sets, a total of four SRS resources are transmitted in different symbols of four different slots, and the SRS port of each SRS resource in the given four sets is associated with a different UE antenna port. The four sets are each configured with one SRS resource. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different. Or,

-	otherwise, for 1T4R,

-	zero or one SRS resource set configured with higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' with four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource is associated with a different UE antenna port, and

-	zero or two SRS resource sets each configured with higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic' and with a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different. Or,

-	For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, or 4 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4, or

-	For 1T6R, zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic',  where in the case of one resource set has six SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the resource in the set is associated with a different UE antenna port, and

-	For 1T6R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has six SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the resource in the set is associated with a different UE antenna port, and

-	For 1T6R, zero or one or two or three SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set a total of six SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource in the set is associated with a different UE antenna port. In the case of two resource sets a total of six SRS resources are transmitted in different symbols of two different slots, and the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. In the case of three resource sets, a total of six SRS resources are transmitted in different symbols of three different slots, and the SRS port of each SRS resource in the given three sets is associated with a different UE antenna port, or

-	For 1T8R, zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has eight SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the resource in the set is associated with a different UE antenna port, and

-	For 1T8R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has eight SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the resource in the set is associated with a different UE antenna port, and

-	For 1T8R, zero or two or three or four SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of two resource sets a total of eight SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. In the case of three resource sets a total of eight SRS resources are transmitted in different symbols of three different slots, and the SRS port of each SRS resource in the given three sets is associated with a different UE antenna port. In the case of four resource sets a total of eight SRS resources are transmitted in different symbols of four different slots, and the SRS port of each SRS resource in the given four sets is associated with a different UE antenna port, or

-	For 2T6R, zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has three SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, and

-	For 2T6R, zero or one SRS resource sets configured resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time.  Each SRS resource set has three SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, and

-	For 2T6R, zero or one or two or three SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic, where in the case of one resource set has three SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair. In the case of two resource sets a total of three SRS resources are transmitted in different symbols of two different slots, and the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. One set is configured with two SRS resources and another set with one resource. In the case of three resource sets a total of three SRS resources are transmitted in different symbols of three different slots, and the SRS port pair of each SRS resource in the given three sets is associated with a different UE antenna port pair, or

-	For 2T8R, zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, and

-	For 2T8R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, and

-	For 2T8R, zero or one or two or three or four SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the  resource in the set is associated with a different UE antenna port pair. In the case of two resource sets a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. In the case of three resource sets a total of four SRS resources transmitted in different symbols of three different slots, and where the SRS port pair of each SRS resource in the given three sets is associated with a different UE antenna port pair. Two sets are configured with one SRS resource in each set and one resource set is configured with two resources. In the case of four resource sets a total of four SRS resources transmitted in different symbols of four different slots, and where the SRS port pair of each SRS resource in the given four sets is associated with a different UE antenna port pair. Four sets are configured with one SRS resource in each set, or

-	For 4T8R, zero or one or two SRS resource set configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or

-	For 4T8R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports.

The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot.

For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols.

-	When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission.

The UE shall expect to be configured with the same number of SRS ports for all SRS resources in the SRS resource set(s) with higher layer parameter usage set as 'antennaSwitching'.

In the case that more than one SRS resource set configured with resourceType in SRS-ResourceSet set to ‘aperiodic’, the UE shall expect that the more than one set are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet.

For 1T2R, 1T4R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, or 4T8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R or 4T=4R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

The value of Y is defined by Table 6.2.1.2-1.

<Unchanged parts are omitted>