3GPP TSG RAN WG1 #112			R1-2301248

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.1.2

Source: 	Moderator (Samsung)

Title: 	Summary of OFFLINE discussion on Rel-18 MIMO CSI

Document for:	Discussion and Decision

Priority for RAN1#112

|  | Issue | Topic |
| --- | --- | --- |
| 1 | Type-II CJT | W2 quantization: working assumption on Alt3 |
| 2 |  | Finalize details for mode-1: Alt1 vs 2 vs 3 |
| 3 |  | Finalize Parameter Combination (including supported NL values) |
| 4 |  | Finalize UCI parameter design |
| 5 |  | Initial discussion on CBSR |
| 6 |  | Initial discussion on CSI omission |
| 6 | Type-II Doppler | FFS: Whether to prioritize/down-select refinement of Rel-16 vs Rel-17 Type-II |
| 7 |  | Finalize CQI: if X>1 is supported, CQI/PMI/slot correspondence |
| 8 |  | Finalize remaining parameter issues: last  value, need for additional Q values, d values, need for additional N4 values, need for additional K values |
| 9 |  | Finalize W2_tilde quantization (N4>1) |
| 10 |  | Finalize NNZC bitmap design |
| 11 |  | Initial discussion on Parameter Combination |
| 11 | TDCP | Finalize down-selection of parameter type (Alt A1 vs A2 vs B) |

# Type-II CJT

|  | Issue | Topic |
| --- | --- | --- |
| 1 | Type-II CJT | W2 quantization: working assumption on Alt3 |
| 2 |  | Finalize details for mode-1: Alt1 vs 2 vs 3 |
| 3 |  | Finalize Parameter Combination (including supported NL values) |
| 4 |  | Finalize UCI parameter design |
| 5 |  | Initial discussion on CBSR |
| 6 |  | Initial discussion on CSI omission |

Table 1A Type-II CJT: issues

| Topic | Moderator comments and proposals |
| --- | --- |
| 2 | [111] Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources Early temp check: - Alt1: Nokia/NSB, AT&T, Samsung, DOCOMO (1st), OPPO (2nd pref), MediaTek, CMCC (2nd pref) - Alt2: vivo, Qualcomm, Lenovo, LG, Apple, OPPO, CMCC, - Alt3: Huawei/HiSi, ZTE, CATT, Spreadtrum, Fraunhofer IIS/HHI, DOCOMO FL assessment: Failure to converge would result in no support for (down-scoping of) mode-1 in Rel-18, despite the previous agreement to support mode-1 - Mode-2-only for Type-II CJT is perhaps a non-issue but unfortunate since we had a previous agreement to support mode-1 Comparing Alt2 and Alt3, since the legacy FD basis selection is used on both, the only difference lies in the use of per-resource FD basis selection offset for Alt3. Since per-resource FD basis selection offset and per-resource Minit are functionally equivalent, Alt2 and Alt3 are expected to yield the same UPT (with similar overhead). - Q2.1: Do you agree? If so, what would be the advantage of Alt3 over Alt2? If not, explain your rationale. Comparing Alt1 and Alt3, the only difference lies in the use of resource-common vs. per-resource FD basis selection. Therefore, if per-resource FD basis selection doesn‘t offer any UPT gain relative to resource-common FD basis selection, Alt1 is expected to slightly outperform Alt3 (and Alt2) since the resulting PMI overhead from Alt1 is lower. - Q2.2: Do you agree? If not, explain your rationale. Q2.3: Share your view (based on factors such as performance in terms of UPT vs overhead, spec and UE complexity) on the comparison between Alt1 and Alt2. Offline proposal 1.B.1: On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources |
| 3 | FL assessment on baseline: Rel-16/17 supported Parameter Combinations are fully reused for the Rel-16-based and Rel-17-based Type-II CJT codebook refinements - Additional codebook parameters (such as sets of {Ln}, NL, NTOT) will be higher-layer configured via RRC parameters, i.e. not included as a part of Parameter Combination framework Note that adding a new codebook parameter into the Parameter Combination framework results in pros and cons: - Pros: Reduced number of overall combinations required to be supported by UEs. This is relevant when, e.g. some combinations of parameter values are clearly useless and/or redundant - Cons: Potentially lengthy debate on expanding combinations with the new parameter(s) while removing some existing ones (to minimize the total number) Any departure from the above baseline (i.e. adding a parameter as a part of Parameter Conbination framework) will require strong justification. - Q3.1: Share your view on this issue, i.e. whether any of the new codebook parameters needs to be included as a part of the Parameter Combination framework along with its benefit(s) Offline conclusion 1.C.1: On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) codebook parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv,} combinations - FFS: Whether/how the list of supported {Ln} combinations can be linked with the list of supported {pv,} combinations without introducing a new (not previously agreed) codebook parameter, e.g. via some UE capability |

Table 2 Type-II CJT: inputs from companies

| Company | Input |
| --- | --- |
| Mod V0 | Please share your views on the offline questions in TABLE 1A/B |
| Samsung | Re Q3.1: a few comments: - The down-selection of parameter combinations should be based on the UPT vs overhead results for up to rank 4 (just like legacy Rel.16) - The total number of candidate parameter combinations based on the agreed candidate values of {Ln}, p_v, and beta is much larger (a few hundreds) than Rel.16/17. Following legacy (which supports 8 combinations), from this large set of candidates, we need to select up to 8 combinations for (L, pv, beta) for CJT codebook. In Rel.16 (R1-1908499), we agreed to a criterion (copied below) for the selection of the supported parameter combinations. We did the same in Rel.17. We should use a similar criterion for CJT codebook also. The only difference here is that we need to select a set of values {Ln} together with a set of values for (L, pv, beta) from legacy. A possible starting point for the criterion for CJT codebook can be as shown in red. From R1-1908499 Offline agreement: Use the following criteria for down-selecting the supported parameter combinations: - Avoid overly complex down selection by reducing the number of combinations for {Ln} and (L,p,beta) only - Alpha has not been decided and making it N3 dependent is unnecessarily convoluted - For #1 - Remove combinations with: - the total overhead exceeding the max of Rel.15 Rel.16/17 Type II - the UPT lower than Rel.15 Rel.16/17 Type II for the same overhead - When several combinations appear redundant in terms of overhead (sharing similar overhead – or vice versa), choose the one with the best UPT, unless the best combo is shown to be scenario-dependent. Then it is justified to support several combos with more or less the same overhead Use the following format for a proposed supported parameter combination: L; p=y0 (rank 1-2); p=v0 (rank 3-4); β; Restriction (if any). And for a proposed supported {Ln} L1; L2; L3; L4. |
| MediaTek | Regarding Q2.3 We think that Alt 1 reporting achieves commonality between Mode 1 and Mode 2 codebook in terms of UE implementation and spec effort. The UE may compute and report a common FD basis Wf in either mode, with the addition of per layer FD bases shift in Mode 1. In terms of performance, we did not observe consistent gain for selecting per TRP FD bases as compared to TRP common FD bases, when the FD bases are selected per layer post SVD operation as in legacy Rel-16. Regarding Q3.1 In general, we are supportive of reusing the combinations of (pv,β) from Rel-16. However, since the CJT codebook has a combination of (L1,…LN) rather than a single L parameter as in Rel-16, we need to decide a single parameter which can be a representative of the combination (L1,…LN). This parameter could naturally be Lmax. In this case, the param combination triplet is (Lmax,pv,β). Further, we think that exhaustively listing the combinations of (L1,…LN) for all possible number of TRPs N would be very large. Instead, we may have that - NL combinations of L1,…LN satisfying Lmax≥n=1NLn configured by gNB via DL signaling - Prefer NL≤4 to minimize UE complexity and DL signaling overhead of indicating the combinations |
| Samsung2 | Re Q2.1, Q2.2: agree Re Q2.3: the main argument in support of Alt2/3 is different delay profiles across TRPs in some scenarios (e.g. inter-cell), however, - If delay profiles are very different (and spread out), the overall delay profile will be outside the CP length, and hence CJT doesn’t make any sense. - So, for CJT to work, the delay profiles of different TRPs must be ‘short’ enough so that they are within the CP. This should be assumed/ensured for relevant scenarios. - Assuming the overall delay profile is within the CP, there is no significant difference between TRP-common Wf (Alt1) or per-TRP Wf (Alt2/3), as observed in our simulations Re Q3.1 - There is a typo in our previous comment, i.e., the parameter L from legacy table is not needed since we will have a set of {Ln} combinations configured by the gNB (as agreed last meeting). We fixed it below. - Re Lmax, as discussed at length during last meeting, this parameter is redundant (can be derived from configured N_L combinations, hence not needed From R1-1908499 Offline agreement: Use the following criteria for down-selecting the supported parameter combinations: - Avoid overly complex down selection by reducing the number of combinations for {Ln} and (L,p,beta) only - Alpha has not been decided and making it N3 dependent is unnecessarily convoluted - For #1 - Remove combinations with: - the total overhead exceeding the max of Rel.15 Rel.16/17 Type II - the UPT lower than Rel.15 Rel.16/17 Type II for the same overhead - When several combinations appear redundant in terms of overhead (sharing similar overhead – or vice versa), choose the one with the best UPT, unless the best combo is shown to be scenario-dependent. Then it is justified to support several combos with more or less the same overhead Use the following format for a proposed supported parameter combination: L; p=y0 (rank 1-2); p=v0 (rank 3-4); β; Restriction (if any). And for a proposed supported {Ln} L1; L2; L3; L4. |
| ZTE | Q2.1: In our views, the Alt3 may be assumed as a detailed solution based on the principle of Alt2. Q2.2/3: Due to the channel variation for each of serving TRPs, per-resource FD basis selection seems much reasonable. It is clear that we may have different/individual delay profiles corresponds to respective TRPs. If going with Alt1, we may have to consider more FD bases (i.e., a larger Mv) to be selected, and the report overhead increases significantly. From the transmission performance perspective, we observe the average and cell-edge UPT gains of Alt2 (and Alt3) over Alt3 while considering report overhead. Q3.1: In CJT, for a given of L_total of summing a given set of {Ln}, we may combine the impact of the number of TRPs and number of {Ln} for each TRP together. Based on our evaluation, a larger value {Pv, Beta} may be suitable for a larger {Ln}, but considering the range of number of TRPs (from 1 to 4), it may introduce the combination of {L_total, Pv, Beta} are quite complicated. Since that, we suggest to provide the individual configuration of {Ln} and {Pv, Beta}. The linking between {Ln} and {Pv, Beta} is up to gNB configuration. For the latest suggestion from Samsung, we share the similar views of individual tables for {Pv, Beta} and {Ln}, but further consideration of restriction may not be needed and can be left to gNB implementation. Otherwise, there may be too many error/worse cases like having pv = 1/8 for v=1,2 (hence 1/16 for v=3,4) Furthermore, we have some concerns for having a rule of “Remove combinations with the total overhead exceeding the max of Rel.15 Rel.16/17 Type II”, due to the fact that we have up to 4 TRPs, extending the upper bound of R16/17 Type II may be possible with higher probability. If having this rule, I think twice (or more) of the max should be considered as a baseline, and upper bound of UPT performance should be considered together. The current situation is different from Rel-16 enhancement on overhead reduction in eType CSI. |
| OPPO | Q2.1: Yes. We think independent FD basis selection across N CSI-RS resources with per-CSI-RS-resource FD basis selection offset is equivalent to independent window (M_init) for each CSI-RS resource. Since FD basis selection offset needs additional overhead, Alt.3 cannot provide benefits over Alt.2 with the same overhead. Q2.2: We don’t think per-resource FD basis selection cannot offer any UPT gain relative to resource-common FD basis selection. Furthermore, according to the previous agreement, Mode 1 was defined as per-TRP/TRP-group SD/FD basis selection which allows independent FD basis selection across N TRPs /TRP groups. Alt.1 may not be consistent with the definition. Q3.1: We intend to agree with MTK that Lmax can be used instead of L in the parameter combination, e.g. Lmax,pv,β can be configured by gNB. The total number of SD basis should be restricted considering the UE complexity and actual deployment. The candidate NL combination can be derived based on Lmax. For example, the candidate NL combination for each value of Lmax can be pre-defined between gNB and UE, and UE can report the target combination based on gNB configured Lmax. |
| AT&T | Q2.3 Our view is that Alt 1 gives an optimized scheme (overhead & UE complexity) by having a common FD basis selection across all the CSI-RS resources scaled with per CSI-RS resource FD offset. We will wait the SLS performance evaluations from the companies and decide our final position. Q3.1 Our position from the beginning was to support Lmax such that n=1NLn≤Lmax as it could reduce the overhead and complexity by restricting the UE to the number of combinations NL associated with the gNB configuration parameter Lmax. Therefore, we agree with MTK & OPPO and support the param combination triplet (Lmax,pv,β). |
| Intel | Q2.1: Alt 2 and Alt 3 are different and have different UPT if 0 ≤ φn <1. In this case basis vectors in Wf,n1 and Wf,n2 are not mutually orthogonal. It can be viewed as different rotation factor for oversampled basis. Q2.3: Considering that common selection of FD vectors is already supported (mode-2), we have slight preference on Alt 2 for mode 1. This solution gives larger difference in the flexibility of FD vectors selection. If Alt 1 has significant support, we can consider merging of mode-1 and mode-2 as common selection with integer shift of FD vectors since the overhead for mode-1 with Alt1 and mode-2 is very close (up to 15 bits). Q3.1: We can consider including Lmax as part of parameter combination. However, since the supported value range for Lmax is not clear yet we prefer to defer discussion on parameter combinations (on whether to include Lmax as part of combination or not). So, we can focus on the value range for Lmax. |
| Fujitsu | Q2.2/3: Since both co-located and distributed layouts should be considered and agreed in EVM phase, per-resource FD basis selection is more suitable for scenarios with greater delay difference. However, considering the reporting overhead and commonality with mode 1&2, Alt 1 might be a compromise scheme to achieve the acquisition of the delay difference across N TRPs. Q3.1: Considering the simpler design on combination value across N TRPs, we prefer individual {Ln} configuration. |
| Ericsson | Q2.1 Agree. For reasons mentioned by the FL, we expect Alt 2 and Alt 3 to perform similarly. Q2.3 We think Alt 1 for mode 1 will be simpler from specification perspective compared to Alt 2 and Alt 3. In our view, the FD basis shift is layer-common (i.e., one FD basis shift per TRP). Our positoin is to support Alt 1, and we hope to bring results to the meeting on this topic. Q3.1 Since we have already agreed that the gNB can configure the UE with NL combinations of {Ln}, the total number of SD basis vectors Ltot=n=1NLn may be different among the NL combinations of {Ln}. One approach is to define the parameter combinations for {Ltot, pv, β}. We are also fine with leaving the linking between Ltot=n=1NLn and {pv, β} to gNB configuration which may be a simpler approach. In this case, for each of NL combinations of {Ln}, the gNB will configure a corresponding {pv, β}. |
| Apple | Q2.1 We has slightly different opinion If φn is only allowed to be integer, we agree. If φn is allowed to be fraction similar as oversampling which was not adopted for FD basis selection, then, compared to Alt 2, Alt 3 provides finer delay compensation/alignment between different TRPs Q2.2 We has slightly different opinion There are two things in terms of the channels from two TRPs - Average delay of the channel, e.g., first arrival path - Delay spread profile of the channel (frequency selectivity) Alt 3 provides more flexible FD basis selection compared to Alt 1 at expense of higher CSI overhead. This can be useful to handle the second bullet, i.e., different TRP has different dela spread due to different reflections etc Q2.3 We first need to discuss whether φn is allowed to be fraction If it is not allowed, Alt 2 can handle channel with different delay spread better than Alt 1 If it is allowed, Alt 1 could be better to perform finer delay compensation/alignment between different TRP Q3.1 We agree with the assessment. We prefer not to add a new parameter as a part of Parameter Combination framework |
| LG | Q2.1 We agree with FL’s assessment. Q2.3 A main target scenario of Mode-1 is inter-cell, in which DL channel of each TRP has a large difference in delay profile. Alt 1 and Alt 2 may have a similar performance in inter-cell scenario with small ISD but Alt 2 will outperform in large ISD scenario. Also, as OPPO mentioned, we already agreed that Mode 1 allows independent FD basis selection across N TRPs / TRP groups in 110 meeting. Alt 1 allows different FD basis selection across TRPs but it is not independent selection, which is not aligned with the agreement. Q3.1 This issue is also related to TRP selection of UE, which was agreed as a new bitmap reporting with CSI. In other words, UE side TRP selection has an impact on {pv, β}. For example, if UE selects only k-th TRP among 4 TRPs (i.e., STRP transmission) it makes sense to determine {pv, β} only based on Lk. To this end, we prefer to define the parameter combinations for {Ltot, pv, β} and Ltot=n=1NLn where Ln=0 for non-selected TRP. |
| Qualcomm | Q2.1 Agree with FL’s assessment, same UPT for Alt2 and Alt3 Q2.2/2.3 In our simulation of Alt1 with per-layer FD offset after SVD, a tricky thing is, Alt1 does not show as a “compromise” b/w mode2 FD-joint CB and mode1-Alt2/3 FD-independent CB. Instead, Alt1 shows loss over both mode2 and mode1-Alt2/3 at smaller p_v with e.g. M=2 The analysis is, either mode2 or mode1-Alt2/3 can guarantee a summation-power-based W2, while for Alt1, our algorithm is, first to find the FD index “pattern” of a per-layer strongest TRP, then use this “pattern” to slide for other TRPs to find the per-layer&per-TRP offset We are open if there are some other Alt1-specific algorithms, which has physical meaning to exploit TRP-relative delay offset (which is the motivation of Alt1 in my understanding) Q3.1 We think the simplest way to reuse Rel-16 more is, just replace the “L” with Ltot, where Ltot=n=1NLn – thus ParamCombo Ltot, pv, β works similar as Rel-16 – similar view as MediaTek, OPPO, AT&T, Ericsson, LG (if I didn’t miss, and Lmax is considered similar as Ltot) As for the table of NL rows of Ln value combinations, we agree that a standard-defined table would work, no need for RRC config – similar view as Samsung, OPPO |
| vivo | Q2.1: We think Alt 2 and Alt 3 should have same performance and overhead. There is no benefit of Alt 3 over Alt 2. Q2.2: Based on our evaluation, Alt 1 does not outperform Alt 2. Although Alt 1 has slightly lower overhead, its performance is also a bit lower especially in high-payload region. The final curves are quite close, but Alt 2 curve is a bit superior. Based on the above, we don’t think there is any reason to have a special design on Wf as in Alt 1. Totally independent selection as in Alt 2 is sufficient. Q3.1: - We are okay not to include any new parameters into codebook parameter combinations. - We are okay to have separate tables for N_L and p/beta. |
| CATT | Q2.1. Minit is reported only when N3 > 19. Alt2 and Alt3 do not perform the same. With legacy design, there is cyclic shifting that shifts the FD basis containing SCI to FD basis#0. Legacy FD basis selection indication assumes FD basis#0 is always reported, and the other FD basis is reported with reference to #0. Without modification to legacy FD basis selection indication scheme, Alt2 does not work. With Alt1, an FD basis selection offset is applied to FD basis common across N CSI-RS resources. Though different FD basis can be selected for different TRP, they are not independent selection. This contradicts the definition of mode 1. Therefore, Alt1 shall be precluded first. Our preference is Alt 3, but can live with Alt2 as long as necessary modification to FD basis selection indication is made. Q3.1 We prefer not to introduce additional codebook parameters in the parameter combination, as too many parameters make the selection of combination a very complex task. We share similar view as ZTE that the set of {Ln} can be configured independently from {Pv, Beta}. |
| Spreadtrum | Q2.1 We have similar view with FL that per-resource FD basis selection offset for Alt3 and per-resource Minit for Alt2 are functionally equivalent since only a resource-common Minit is needed for Alt3. However, for Alt3, before FD basis selection, per-resource FD basis selection offset can be determined by remapping the strongest FD basis for each resource to FD basis 0. The FD basis remapping operation in Alt3 will increase the possibility that the reference amplitude is the strongest amplitude within each group for both W2 grouping methods. Besides, the overhead of SCI can be reduced as well. Q2.2/Q2.3 We have different opinion. When the distribution of strong FD basises corresponds to different resources will be different, without increasing the number of selected FD basises, some of the strong FD basises may not be selected. Also, in order to cover the strong FD basises for different resources, the per-resource FD basis selection offset should be used to remap the first selected FD basis to FD basis 0 (rather than remapping the strongest FD basis for each resource to FD basis 0 in Alt3). Compared with Alt3, the remapping operation in Alt1 cannot reduce the overhead of SCI especially when W2 coefficients are grouped per polarization per resource. Q3.1 We have similar view to consider L_max as part of parameter combination. |
| Xiaomi | Q2.1 Agree that Alt 2 and Alt 3 may provide same UPT, and we prefer Alt 2 for low complexity. Q2.2 Previous agreement was archived to support FD basis selection per TRP/TRP group, Alt 1 may not align with the agreement. And Alt 2 may provide good performance in the case of large delay difference between M-TRPs. Q3.1 We also think that the total number of L for M-TRP should be used instead of single L. |
| Mod V18 | Current summary: CJT Mode 1: - Alt1: MediaTek, Samsung, Intel (2nd), Fujitsu, Ericsson, Nokia/NSB, Apple (if fractional φn) - Alt2: ZTE, OPPO, vivo, Intel, Apple (if no fractional φn), Qualcomm, LG, CATT (2nd), Xiaomi, - Alt3: ZTE, Huawei/HiSi, Qualcomm, CATT, Spreadtrum, To make our discussion more efficient, we will focus the down selection on Alt1 and Alt2 (I assume companies preferring Alt3 are fine with Alt2) Offline proposal 1.B.1: On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources CJT Parameter combination: Separate tables for {Ln} and {pv,beta}, not adding a new parameter not previously agreed - Support: Samsung, MediaTek (if resulting in fewer combos), ZTE, Intel (defer), Fujitsu, Ericsson (2nd, with linkage), Apple, vivo, CATT, - Concern: Replace L with Lmax in a single table (hence adding a new parameter not previously agreed) - Support: MediaTek, OPPO, AT&T, Spreadtrum, - Concern: vivo, Replace L with Ltot in a single table (hence adding a new parameter not previously agreed) - Support: Ericsson, LG, Qualcomm, Xiaomi, - Concern: vivo, Based on the above summary, the following offline proposals are made Offline conclusion 1.C.1: On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv,} combinations |
| NTT DOCOMO | Sorry for being late. Q2.1: After further thinking, we tend to agree that per-resource FD basis selection offset and per-resource Minit are functionally equivalent. Thus, we revise our position in the table above (delete from Alt3 and add to Alt2). We are supportive for Offline proposal 1.B.1. Q2.2: Independent per-CSI-RS-resource FD basis selection will have benefits for some scenarios in our understanding. Hence, we’re open to down select from Alt1 and Alt2. Q3.1: We share similar view as SS and ZTE on introducing individual tables for {Pv, Beta} and {Ln}. For {Ln}, we have agreed to configure NL combinations of values for {L1, ..., LNTRP}. Regarding the linkage between {Pv, Beta} and {Ln}, considering overhead control, it is beneficial to define parameter combination for (Lmax, Pv, Beta). On the other hand, we also think it is beneficial to define some restrictions between {Pv, Beta} and {Ln} to reduce the supported combinations. For offline conclusion, 1.C.1, it might be a bit premature to draw this sort of “non-consensus” conclusion. As discussed above, at least there should be some restrictions between configured {Ln} and {Pv, Beta}. [Mod: Since Parameter Combination needs to be completed by the end of RAN1#112 to be on track (I already announced the priority items before Christmas 2022 , this must be concluded on day 1 of RAN1#111. Hence it is not premature. But I added an FFS to accommodate your point at least partially] |
| NEC | Q2.1: Our preference added in above table. We think Alt 2 and Alt 3 are equivalent. As listed in following tables: Table 1. FD basis selection for mode-1 codebook based on Rel-16 eTypeII codebook N3; Alt 2; Alt 3. N3≤19 (without FD window); For reference TRP, Mv FD bases selected from N3 bases (log2N3-1Mv-1) (with FD basis remapping); For reference TRP, Mv FD bases selected from N3 bases (log2N3-1Mv-1) (with FD basis remapping). For other TRPs, Mv FD bases selected from N3 bases (log2N3Mv); For other TRPs, FD basis offset indication (fixed as 0), and Mv FD bases selected from N3 bases. Note: As the Mv FD bases are freely selected from N3 bases without FD window, there is no need of FD basis offset indication, which will cause unnecessary overhead, so in this case, FD basis offset can be assumed as 0 and no need of report.. N3>19 (with FD window); For reference TRP, Minitial indication (log22Mv) and Mv FD bases selected from 2Mv bases (log22Mv-1Mv-1) (with FD basis remapping); For reference TRP, Minitial indication (log22Mv) and Mv FD bases selected from 2Mv bases (log22Mv-1Mv-1) (with FD basis remapping). For other TRPs, Minitial indication and Mv FD bases selected from 2Mv bases Note: Minitial for the other TRPs serves same function as FD basis offset, and the range should be different from that for reference TRP.; For other TRPs, FD basis offset indication, and Mv FD bases selected from 2Mv bases Note: FD basis offset indication for the other TRPs is actually same as Minitial for the other TRPs in Alt 2.. Table 2. FD basis selection for mode-1 codebook based on Rel-17 port selection codebook Number of FD basis; Alt 2; Alt 3. M=1; For reference TRP, no need of report; For reference TRP, no need of report. For other TRPs, one FD basis selected from N3 bases; For other TRPs, FD basis offset indication Note: Actually, the FD basis offset indication in this case is same as the one FD basis selection in Alt 2.. M=2 and window N=2; For reference TRP, no need of report; For reference TRP, no need of report. For other TRPs, starting of the window N indication is needed and no need of report for M FD basis selected from window N; For other TRPs, FD basis offset indication, and no need of report for M FD basis selected from window N Note: FD basis offset indication for the other TRPs is actually same as the starting of the window N for the other TRPs in Alt 2.. M=2 and window N=4; For reference TRP, second FD basis indication; For reference TRP, second FD basis indication. For other TRPs, starting of the window N indication is needed and second FD basis indication; For other TRPs, FD basis offset indication, and second FD basis indication Note: FD basis offset indication for the other TRPs is actually same as the starting of the window N for the other TRPs in Alt 2.. Offline proposal 1.B.1: We are fine with the proposal. And compared with Alt 1, we prefer Alt 2, as the offset between delays should be independent for different TRPs. Offline conclusion 1.C.1: We are fine with the proposal, and we share similar view with Ericsson that different combinations of NL combinations can be associated with different values of {pv,}, as the number of TRPs and the values of Ln can be dynamically selected by UE. |
| Huawei, HiSilicon | Re Q2.1: We think Alt3 can be equivalent to Alt2, if the Minitial reporting scheme of Alt2 is modified. Re Q2.2/3: - Alt1 is too restrictive since the delay profile may be still different even if FD basis offset are reported to represent the delay differences roughly. Although common Wf can lead to overhead reduction, it may also results in performance degradation. Based on our SLS results, we observe Alt2/3 outperforms Alt1 at both mean and 5% UPT in terms of UPT v.s. overhead. - From the UE complexity perspective, per-TRP independently FD basis selection (Alt2/3) has less complexity for UE processing without searching over different set of TRP-common Wf. Re Q3.1: - For parameter combination configuration, we share similar opinion with MTK and OPPO. Since joint configuration of {L1, ..., LNTRP} and (pv,β) may lead to a lengthy table. {L1, ..., LNTRP} and (pv,β) can be configured separately with a linkage parameter Lmax. And the gNB-configured set of NL {L1, ..., LNTRP} combinations are corresponding to one Lmax, where Lmax≥n=1NTRPLn. In this way, Lmax,pv,β combination design can help to reduce redundant combinations with similar overhead and poor performance. And for given Lmax, NL combinations of {L1, ..., LNTRP} can be freely configured or configured based on a pre-defined candidate combination table. [Mod: I understand your preference but a few companies have strong concern on defining a new codebook parameter such as Ltot/Lmax, which went against the compromise proposal we had in RAN1#111 where Lmax/Ltot is ruled out. Given these two facts I will not discuss Lmax/Ltot at least as a codebook parameter.] - For the value NL, we support to restrict the upper bound of NL for {Ln} configuration, e.g. 4, so as to reduce the UE complexity. [Mod: This will be decided n RAN1#112 – thanks for the good starting point proposal] - As for the criteria for parameter combination selection, we have similar concern as ZTE for the maximum overhead limitation. Since we intend to enable CJT of up to 4 TRPs, the maximum overhead should be extended to achieve a better CJT performance over sTRP transmission. We agree with ZTE that at least twice or three times of the max of Rel.16/17 Type II should be a baseline. |
| Google | Offline proposal 1.B.1: Support. We prefer Alt2. Offline conclusion 1.C.1: Support |
| vivo | Proposal 1.B.1 OK Conclusion 1.C.1 OK |
| Fraunhofer IIS/HHI | Offline Proposal 1.B.1: We support Alt 3. But we can be open to other alternatives based on the evaluation results. Offline conclusion 1.C.1: OK |
| Lenovo | Offline Proposal 1.B.1: Support Alt2, agree with FL assessment regarding similarity with Alt3 Offline conclusion 1.C.1: We are fine with the conclusion, however we would like to point out one concern which is also implied from comments of other companies, mainly regarding the homogeneity between two agreements: - UE reporting a selection of N TRPs (from a set of NTRP TRPs) - The RRC configuration of the NL beam combinations corresponding to the NTRP TRPs. This needs further discussion in the meeting since the UE would now need to test up to 2NTRP.NL beam combination hypotheses and TRP hypotheses to select the beams. Further discussion is needed in our opinion [Mod: This is a separate issue and IMO a UE implementation issue. The above assumes that the UE performs exhaustive search The max value of NL will be separately discussed and finalized in RAN1#112] |
| Samsung | Offline Proposal 1.B.1: OK and support Alt1. Based on our results and analysis, Alt1 is better or no worse than Alt2/3, has lower overhead, and lower complexity (common vs per-TRP Wf). Hence, Alt1 is superior than Alt2/3 based on all three metrics (according to EVM). Offline conclusion 1.C.1: OK. We have the same view with vivo. Again, we had already intensively discussed during the last RAN1 #111 on Lmax/Ltot and we agreed not to introduce those new parameters but to use {L1,…,LNTRP} to configure Ln by gNB as in the compromised agreement, and we don’t need a linkage parameter between two tables using a redundant parameter which just provides a complicated framework without technical justification. Separate tables on {Ln} and (pv,beta) with some restriction (if needed) are sufficient without a complicated linkage scheme. Based on this, we cannot accept to introduce a new parameter such as Lmax/Ltot. At most, we can be OK with a new parameter for UE capability on NL . |
| Mod V28 | Current summary: CJT Mode 1: - Alt1: MediaTek, Samsung, Intel (2nd), Fujitsu, Ericsson, Nokia/NSB, Apple (if fractional φn) - Alt2: ZTE, OPPO, vivo, Intel, Apple (if no fractional φn), Qualcomm, LG, CATT (2nd), Xiaomi, NEC, Google, Lenovo, - Alt3: ZTE, Huawei/HiSi, Qualcomm, CATT, Spreadtrum, Fraunhofer IIS/HHI, Proposal 1.B.1: - Support/fine: MediaTek, Samsung, Fujitsu, Ericsson, Nokia/NSB, Apple, ZTE, OPPO, vivo, Intel, Qualcomm, LG, CATT, Xiaomi, Huawei/HiSi, Spreadtrum, NTT DOCOMO, NEC, Google Given the above proposal 1.B.1 is unchanged CJT Parameter combination: Separate tables for {Ln} and {pv,beta}, not adding a new parameter not previously agreed - Support: Samsung, MediaTek (if resulting in fewer combos), ZTE, Intel (defer), Fujitsu, Ericsson (2nd, with linkage), Apple, vivo, CATT, - Concern: Replace L with Lmax in a single table (hence adding a new parameter not previously agreed) - Support: MediaTek, OPPO, AT&T, Spreadtrum, NTT DOCOMO, Huawei/HiSi, - Concern: vivo, Samsung Replace L with Ltot in a single table (hence adding a new parameter not previously agreed) - Support: Ericsson, LG, Qualcomm, Xiaomi, - Concern: vivo, Samsung, Based on the above summary, offline conclusion 1.C.1 is revised as follows Offline conclusion 1.C.1: On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) codebook parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv,} combinations - FFS: Whether/how the list of supported {Ln} combinations can be linked with the list of supported {pv,} combinations without introducing a new (not previously agreed) codebook parameter, e.g. via some UE capability |
| CMCC | Offline Proposal 1.B.1: We support this proposal and we prefer Alt2 unless Alt1 does show good trade-off over UPT vs PMI overhead. Offline conclusion 1.C.1: Support. We have made a comprise agreement in last meeting that we only agreed to let UE report Ln combination and the new parameters Lmax/Ltot is not introduced. So, when we are discussing the parameters combination, these not agreed parameters should not be involved in. |
| InterDigital | Q2.1 One Wf captures most of the compression of the aggregated CJT channel. We support Alt1. Q2.2 Alt 2 and Alt 3 may not offer substantial UPT gain over Alt 1, as most of the compression of the CJT channel can be captured by the resource-common FD compression matrix Wf. The feedback overhead of reporting FD basis in Alt 1 is less than Alt 2 and Alt 3. The higher overhead of Alt 2 and Alt 3 and their limited UPT performance enhancements over Alt 1 may not justify their use over Alt 1. Q3.1 We are okay with adding new parameters (Ltot and/or Lmax) to parameter combinations. |
| Mod V31 | Current summary: CJT Mode 1: - Alt1: MediaTek, Samsung, Intel (2nd), Fujitsu, Ericsson, Nokia/NSB, Apple (if fractional φn), IDC - Alt2: ZTE, OPPO, vivo, Intel, Apple (if no fractional φn), Qualcomm, LG, CATT (2nd), Xiaomi, NEC, Google, Lenovo, CMCC - Alt3: ZTE, Huawei/HiSi, Qualcomm, CATT, Spreadtrum, Fraunhofer IIS/HHI, Proposal 1.B.1: - Support/fine: MediaTek, Samsung, Fujitsu, Ericsson, Nokia/NSB, Apple, ZTE, OPPO, vivo, Intel, Qualcomm, LG, CATT, Xiaomi, Huawei/HiSi, Spreadtrum, NTT DOCOMO, NEC, Google, CMCC CJT Parameter combination: Separate tables for {Ln} and {pv,beta}, not adding a new parameter not previously agreed - Support/fine: Samsung, MediaTek (if resulting in fewer combos), ZTE, Intel (defer), Fujitsu, Ericsson (2nd, with linkage), Apple, vivo, CATT, CMCC - Concern: Replace L with Lmax in a single table (hence adding a new parameter not previously agreed) - Support/fine: MediaTek, OPPO, AT&T, Spreadtrum, NTT DOCOMO, Huawei/HiSi, IDC - Concern (against compromise in RAN1#111): vivo, Samsung, ZTE, Nokia/NSB, CMCC Replace L with Ltot in a single table (hence adding a new parameter not previously agreed) - Support/fine: Ericsson, LG, Qualcomm, Xiaomi, IDC - Concern (against compromise in RAN1#111): vivo, Samsung, ZTE, Nokia/NSB, CMCC Based on the above summary, proposal 1.B.1 and conclusion 1.C.1 remain unchanged. |

# Type-II Doppler

|  | Issue | Topic |
| --- | --- | --- |
| 6 | Type-II Doppler | FFS: Whether to prioritize/down-select refinement of Rel-16 vs Rel-17 Type-II |
| 7 |  | Finalize CQI: if X>1 is supported, CQI/PMI/slot correspondence |
| 8 |  | Finalize remaining parameter issues: last  value, need for additional Q values, d values, need for additional N4 values, need for additional K values |
| 9 |  | Finalize W2_tilde quantization (N4>1) |
| 10 |  | Finalize NNZC bitmap design |
| 11 |  | Initial discussion on Parameter Combination |

Table 3A Type-II Doppler: issues

| Topic | Moderator comments and proposals |
| --- | --- |
| 7 | [111] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives: - Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices - Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Alt2B. The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB [111] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported - If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots) FL assessment: - Two related issues: a) whether to support X=2 in addition to the agreed X=1 (X>2 is perhaps out of question); b) reference slot(s) and W2(s) for CQI calculation - Issue a) depends on whether X=2 offers significant gain over X=1 (with the additional CQI overhead accounted) - Issue b) Alt1 minimizes mismatch between CQI and PMI, Alt2A is perhaps the “simplest” - Failure to converge on issue b) (for X=1) would result incomplete Type-II Doppler CSI, i.e. the feature cannot be implemented in real-life NW (CQI calculation is unspecified) - Likewise, if we choose the “simplest” scheme but resulting in large CQI-PMI mismatch, the gain due to Doppler compression at high speed will disappear no incentive for operators and vendors to implement this in real-life NW Q7.1: Please share your view on the above two issues and FL assessment (preferably with empirical evidence, e.g. SLS results). Offline proposal 2.B.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrices - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the [middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrices][last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrices] |
| 9 | Proposal 2.I.1 from RAN1#111: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Proposal 2.I.2 from RAN1#111: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported In RAN1#111, the above two proposals were acceptable to most, but a few companies (e.g. Fraunhofer, Intel, Lenovo) requested more time to evaluate. Q9.1: Can all companies support/accept the above two proposals? - 2.I.1: No clear reason that Doppler codebook needs different amplitude/phase alphabet from N4=1 - 2.I.2: The analogy of the so-called “RX power difference across TRPs” across DD basis vectors doesn’t exist. Hence the analogy of “Alt1” solution for CJT should not be contentious for Doppler codebook. Offline proposal 2.D.1 (from RAN1#111): For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Offline proposal 2.D.2 (from RAN1#111): For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported |
| 10 | Latest status from RAN1#111: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt2. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design and further compressed using source-coding (e.g Huffman code) - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt3B: A single 2-dimensional bitmap of size 2LQ to report the selected S pairs of SD components and DD basis vector and a single 2-dimensional bitmap of size MS for indicating the location of the NZCs, where each row corresponds to a selected FD basis vector and each column corresponds to one of the selected S pairs of SD component and DD basis vector. - Alt3C: A single 2-dimensional bitmap of size 2LM to report the selected S pairs of SD component and FD basis vector and a single 2-dimensional bitmap of size QS for indicating the location of the NZCs, where each row corresponds to a selected DD basis vector and each column corresponds to one of the selected S pairs of SD component and FD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L – s1-s0+ mint1-t0, Q – t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively. Early temperature check: - Alt1: OPPO, Samsung, Apple, Lenovo, Spreadtrum, NEC, Intel, Nokia/NSB, Google - Alt2: Intel - Alt3A/B/C: Fraunhofer IIS/HHI, Qualcomm (study), Lenovo - Alt4: vivo The number of alternatives is excessive to have a constructive down-selection discussion. - Q10.1: @Intel: would it be fine to remove Alt2 and keep Intel in Alt1 given the situation? - Q10.2: @Fraunhofer, Qualcomm, Lenovo, and other potential Alt3x proponents: could you please reduce the number of alternatives by removing/merging among Alt3A/B/C? Offline proposal 2.E.1: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. |

Table 4 Type-II Doppler: views from companies

| Company | Input |
| --- | --- |
| Mod V0 | Please share your views on the 3 offline questions in TABLE 3A |
| Samsung | Q9.1: support both proposals Q10.1/2: Alt1 (extension of legacy should be baseline), and other alternatives should be compared with the baseline based on UPT vs overhead tradeoff |
| MediaTek | Regarding Q9.1 Agree with both proposals Regarding Issue 10 Our simulation results show that Alt 3A/3B can achieve better UPT overhead tradeoff compared to Alt 1, so we are open to Alt 3A/3B. We are also open to having Alt 1, since it’s simple extension of the legacy deign. |
| Samsung | Q7.1: Re issue a), X=2 offers additional avg. UPT gain over X=1 with slight increase in overhead, hence achieves a better UPT vs overhead tradeoff. This is expected since the channel changes at a fast enough rate during Wcsi. Re issue b), Alt2A and 2B are not aligned with the current spec, since the CQI has to be conditional on PMI, which is for the entire Wcsi window. Besides, CQI calculated based only on 1 slot can’t be accurate for the whole Wcsi, which will lead to CQI mismatch, hence UPT loss (when compared with Alt1). This is what we observe via simulations |
| Fraunhofer IIS/Fraunhofer HHI | Q10: In our evaluations, Alt 3A/3B/3C offers significant overhead reduction compared to Alt 1 only with a negligible performance loss. Therefore, we do not support Alt 1. Considering the codebook structure, i.e., W1W2(Wf⨂Wd)H, Alt 3A is the most natural and straight forward way to reduce the feedback overhead as the bitmap size (2LS bits) and design are similar to Rel. 16/17 CBs bitmaps. For Rel. 16/17 CBs, S = M is the number of delays, whereas for Rel. 18, S is the number of delay-Doppler component pairs. Moreover, among alternatives 3A/3B/3C, the performance-overhead trade-off of Alt 3A is found to be better compared to Alt 3B/3C. Therefore, we prefer ALT 3A. Re complexity, we do not think any of the alternatives 3A/3B/3C are complex compared to Alt 1. |
| ZTE | Q 7.1: We support Alt2A and X=1 as a baseline. If considering CQI based on more than one slot, our concern is relevant to any necessity considering non-prediction of interference, OLLA and CQI refinement in MU-MIMO. If having X=2, handling the increase of report overhead, especially for X=2, should be considered. Q 9.1: We support those two proposals. Q 10: Support Alt1. For Alt3A/B/C, addition bitmap or combinatorial coefficients C(MQ, S)/C(2LQ, S)/C(2LM, S) shall be provided to indicate the selected S pair. Currently we only have Q=2, and then benefit that Alt 3A/3B/3C can achieve less PMI overhead compared to Alt 1 under Q=2 becomes unclear. Meanwhile, if having Alt3A/B/C, the solution of CSI omission may be complicated. |
| OPPO | Q 7.1: prefer Alt1, gNB needs to know the quantity of succeeding PMI. Q 9.1: support both proposals. Q 10: Alt 3A can save 10% overhead without UPT loss for Q=2 based on our evaluation, we are supportive of Alt 3A. We are also OK with Alt1 since it’s simple and clean. |
| Intel | Q7.1: We do not observe significant performance gains for X>1 comparing to X=1 if practical prediction algorithm is applied at the UE side (non-ideal or non-genie aided prediction). Thus, we support X=1. For the CQI calculation assumption we support Alt2A or Alt2B. The difference between Alt2A and Alt2B is small, it may not be visible by RAN4 tests. Considering that the prediction algorithm is up to UE, UE can implement Alt2B while specification is written according to Alt2A. Q9.1: We support both proposals. Q10: We are fine to remove Alt2. The performance degradation for Alt3A comparing to Alt1 is not significant with practical channel prediction algorithm at the UE. However, considering that the performance of Alt1 is still slightly better, performance-overhead tradeoff is not looking much better for Alt3A. So, we are OK to accept Alt 1 as a simplest solution. |
| Fujitsu | Q7.1: Considering the feedback overhead and the inaccuracy of CQI prediction itself, X>1 CQIs reporting is unnecessary. For the CQI calculation assumption, we have the similar views on ZTE and Intel that the CQI and PMI in Alt 2A are the best match. In addition, gNB can also calculate the correlation value of different W2 by its implement if gNB wants to obtain the future CQI(s) roughly. In that case, this can also prove that only one CQI reporting with best matching PMI is sufficient. Q9.1: Support both proposals. Q10: Considering less impact on current specs, Alt 1 is preferred. |
| Ericsson | Q7.1 We agree with the FL’s assessment. Our system level results show some notable performance gains for X=1 Alt 1 over Alt 2A (10-30% cell edge gains). Hence, we support Alt 1. Similarly, we see performance gains for X=2 (between 10-20% cell edge gains). We support specifying X=2 in addition to X=1. Q9.1 support both proposals Q10: We see some overhead savings with Alt 3A in our results compared to Alt 1. But, we agree Alt 1 is the simplest. So we are fine with either Alt 1 or Alt 3A. |
| Apple | Q7.1 We slight prefer to have only X=1 CQI. We prefer to start with loose CQI definition especially in RAN1 specification for CSI prediction and leave it to RAN4 to have more concrete test case. Q9.1 We are fine with both proposals. Q10 We still prefer Alt 1 |
| LG | Q7.1 Issue b) We support Alt 2A and agree with Intel’s assessment for Alt 2A/2B comparison. Regarding Alt 1, in our view, it cannot achieve potential gain of PMI prediction even though CQI window and PMI window is matched. This is because, CQI should satisfy target BLER in every single slot in PMI window. As a result, conservative CQI (more conservative as N4 increases) is reported, low MCS is applied and DL throughput is low. If companies want to achieve potential gain of PMI prediction by matching CQI and PMI window, Alt 1 enhancement based on X=1 is not reasonable and X=N4 should be supported. Q9.1 We are fine with both proposals. Q10 We still prefer Alt 1, which is simple and straight forward extension of legacy. |
| Qualcomm | Q7.1 In our simulation, we do not observe performance gain with entire-window-based CQI definition, over single-slot-based (Alt2A). Besides, UE complexity can be increased W_CSI or N4 times for capacity calculation, regardless of whether averaging is needed (X=1) or not (X>1) Therefore, we prefer Alt2A We can compromise to X=2, only if it is only relevant to 2 slots (still high cost with doubled complexity for CQI calculation), and optional UE feature Q9.1 OK with both proposals. Q10 Compared with Alt1, we observe similar UPT with Alt3A/3C, at a 6%-9% overall overhead reduction, still meaningful (we did not simulate Alt3B) We prefer either Alt3A or Alt3C |
| vivo | Q7.1: Based on our evaluation, to have multiple CQIs in time domain or have a large window (e.g., Alt 1) for CQI calculation does not being clear performance gain (<=1%) over Alt 2A and X=1 with OLLA on. Further, to have multiple (e.g., 2) CQIs will double UE’s complexity for CQI calculation. To have a large window (e.g., W-CSI window) for CQI calculation is even more complex, as UE has to ensure DL channel in any of these slots to meet the 10% BLER target. Therefore, we support X=1 and Alt 2A. Q9.1: We are okay with proposal 2.I.2. Q.10: We are okay to study these alternatives and perform down-selection based on performance-overhead evaluation. We prefer Alt 4. Our evaluation shows such non-rectangular bitmap has performance-overhead gain (>5% UPT gain for similar overhead) compared with legacy bitmap design in CJT case as seen in R1-2210992. Similar gain can be expected for Type II Doppler as well. |
| CATT | Q7.1 We prefer to have X = 2 based on our simulation results. X = 2 does provide some performance gain over X = 1. X > 2 provides only marginal gain over X = 2. Q9.1 Ok with the two proposals. Q10. Our first preference is Alt.1, but open to Alt 3X. Among the alternatives of Alt 3X, 3A performs the best according to our evaluation results. |
| Spreadtrum | Q9.1 We can support both proposals. Q10 We prefer Alt1. In our views, further overhead reduction may not be needed since the PMI overhead is mainly affected by the number of reported NZCs. |
| Xiaomi | Q9.1: Support Alt1 and more than one, i.e., X=2. For medium/high velocities scenario, both channel and PMI may be significantly changed at different compression unit. If the CQI is calculated via one slot or one compression unit and one CQI is reported, the calculated CQI may not match the variation of both channel and PMI, which degrades the system performance. Q10: Support Alt3A. According to our observation, Alt3A~Alt3C can save indication overhead of non-zero coefficients. We have same view with Fraunhofer. Alt3A is straight ward compared with Alt3B and Alt3C. |
| Mod V18 | This is the current summary Doppler CQI: Add X=2: - Support: Samsung, Ericsson, Qualcomm (only 2A, optional), CATT, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo - Not support: ZTE, MediaTek, Intel, Fujitsu, Apple, Qualcomm, vivo Slot/W2 ref: - Alt1: Samsung, OPPO, Ericsson, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo - Alt2A: ZTE, Intel, Fujitsu, LG, Qualcomm, vivo, - Alt2B: Intel Doppler W2 quantization (proposals 2.I.1/2 from RAN1#111): - Support: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, Qualcomm, vivo (2.I.2), CATT, Spreadtrum, Xiaomi, - Not support: Doppler bitmap: - Alt1: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, CATT, Spreadtrum, Nokia/NSB, Lenovo, Google - Alt2: - Alt3A: MediaTek, Fraunhofer IIS/HHI, OPPO, Ericsson, Qualcomm, CATT (2nd), Xiaomi, - Alt3B: MediaTek, - Alt3C: Qualcomm, - Alt4: vivo, Re Doppler CQI, as expected there are two main views: 1) Add X=2 + Alt1, 2) No X=2 + Alt2A. The main argument for view #1 is best performance of this combo due to minimizing CQI-PMI mismatch. The main argument from view #2 is the difficulty in ensuring 10% BLER across a wide window of W_CSI (when W_CSI is large) hence putting much burden to UE and testing. IMO these two arguments are valid. There are other arguments for views #1 and #2 although IMO they are speculative and highly debatable so I will not include them. A possible compromise on ref slot/W2 is to allow Alt1-like reference but only including the 1st and the last slots in W_CSI window. Also, since the gain of X=2 may not occur in some valid configurations, this can be an optional feature. So the following is my offline proposal: Offline proposal 2.B.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrices - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrices Re Doppler quantization, it seems the previous proposals are agreeable to the group: Offline proposal 2.D.1 (from RAN1#111): For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Offline proposal 2.D.2 (from RAN1#111): For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported Re Doppler bitmap, two main alternatives are Alt1 and Alt3A (although Alt1 seems to be the super-majority view). It may be beneficial to discuss this further to understand the potential overhead saving vs UPT trade-off for various parameter settings. For Monday in Athens, it suffices to narrow down the 6 alternatives to 2, with final selection before Athens ends. Offline proposal 2.E.1: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. |
| NTT DOCOMO | Sorry for being late. We are supportive for all of the following: Offline proposal 2.B.1 Offline proposal 2.D.1 (from RAN1#111): Offline proposal 2.D.2 (from RAN1#111) Offline proposal 2.E.1 |
| NEC | Offline proposal 2.B.1: Support the proposal. And we also support X=2, our position added above in Mod’s comment. Offline proposal 2.D.1/2.D.2: Support the proposal. Offline proposal 2.E.1: Regarding Alt 3A, we think the precondition should be sparse for {FD,DD} basis pair, while whether it’s sparse or not may depend on the configurations, e.g. when the value of Mv and Q is small, in such case, overhead of Alt 3A may be larger than Alt 1. And we haven’t finalize the value of Q (e.g. FFS on 3 and 4), and also we haven’t conclude whether to support enhancement based on Rel-17 port selection codebook for Doppler (if it’s supported, M is only 1 or 2 for Rel-17 codebook, which is not suitable for Alt 3A). So we are fine with Alt 1 or Alt 3A, while if Alt3A is supported, we think it should depend on the configuration, or in other words, in some configurations, Alt 3A can be supported for overhead reduction, and in some other configurations, Alt 1 should be supported. [Mod: Thanks, this is a very valid point. It heavily depends on Q for sure. We will finalize the Q values first before down-selecting between Alt1 and Alt3A] |
| Huawei, HiSilicon | Q 9.1: Fine with those two proposals. Q 10: Our simulation result shows Alt 3A/3B/3C has no obvious performance loss compared with Alt 1 and provide significant bitmap overhead reduction even if Q=2. Considering these three schemes have similar performance and Alt 3A has smallest bitmap overhead, we are fine to merging to Alt3A. |
| Google | Offline proposal 2.B.1: We also do not see the need for X=2 CQIs. If X=2 CQIs were supported, we think the second CQI should be associated with the last slot of the CSI reporting window, which is the same as the optional feature for X=1. Then the NW can determine whether any CQI compensation or interpolation is needed for the whole reporting window. [Mod: It is a valid point. We can check if this is acceptable to other. Else please be flexible ] Offline Proposal 2.D.1/2.D.2: Support. Offline proposal 2.E.1: Support. |
| Vivo | Proposal 2.D.1 OK Proposal 2.D.2 OK Proposal 2.E.1 We don’t think it is a good idea to make a down-selection before we have comprehensive study and evaluation. We should make the down-selection based on observations on the gains of these alternatives. For Alt3 series and Alt 4, careful study is needed to understand how different configurations of codebook parameters can impact the gain on overhead reduction and/or performance of these alternatives. For example, if we compare Alt 3A and Alt 1, as commented by NEC too, for small MQ values, it is expected S should be very close to MQ otherwise there will be performance loss. In this case, the overhead of the second bitmap of Alt 3A is similar as bitmap overhead in Alt 1, but Alt 3A still contains the first bitmap. Hence the bitmap overhead reduction benefit of Alt 3A is not clear in this case. Therefore, we don’t support this proposal. More study is needed. [Mod: I have announced the priority issues shortly after RAN1#111 and this is one of them. All the alternatives including Alt4 have been known since then. But as of today Feb 15th 2023, so far only vivo supports Alt4. Simply stating that “more study is needed” when your proposal garners no support and it is time to narrow things down for more focused discussion doesn’t seem to be progress-responsible. This would imply every single-company proposal would have to included That I cannot do. Anyway, you have until next week to convince other companies to support your proposed Alt4. If the situation changes by Monday the 27th, I’ll include Alt4 as one candidate. Else, proposal 2.E.1 remains unchanged and will be presented as is on the 27th. I am sure that, as a former FL, you understand]. |
| Fraunhofer IIS/HHI | Offline Proposal 2.B.1: We see significant gains when using X greater than or equal to 2. Offline Proposal 2.D.1/2.D.2: Support Offline proposal 2.E.1: Re comments for few companies, based on evaluation results, we did not observe S being close to MQ even for small values of M and Q. In our evaluations, we observed that S being in the range of M is sufficient to achieve performance close to Alt1. |
| Lenovo | Offline Proposal 2.B.1: Support in principle, however our preference is to leave the discussion on UE mandatory/optional for later. [Mod: Why? Lenovo has proposed “mandatory” (nothing is mandatory in Rel-18) and “optional” in a few occasions Anyway this is the best we can do to progress as a group’ Offline Proposal 2.D.1/2.D.2: Support Offline proposal 2.E.1: Support |
| Samsung | Offline Proposal 2.B.1: We see up to 2.5% avg. UPT gain with Alt1 over Alt2A, and additional up to 2% gain with X=2 over X=1. Based on these results, we support Alt1 for CSI reference and X=2 (in addition to X=1). Our concern is that if only X=1, and Alt2A are supported, the gain of the Type II Doppler codebook vanishes, implying it will not be implemented (which will be unfortunate). We understand the UE complexity concern from other companies. So, considering all of these, we can be OK with this proposal for progress since it addresses some of our concerns. Offline Proposal 2.D.1/2.D.2: Support Offline proposal 2.E.1: Support Alt1. For Q=2 (agreed value), we are not sure there is any overhead issue. We don’t need to overdesign by specifying scheme which is a small optimization. For Q>2, the overhead is large, and UPT gain is small (compared to Q=2), so the UPT vs overhead tradeoff is worse than that for Q=2. Alt3A can save some overhead, but this saving doesn’t make Q>2 better than Q=2 (in terms of UPT vs overhead). |
| Mod V28 | This is the current summary Doppler CQI: Add X=2: - Support: Samsung, Ericsson, Qualcomm (only 2A, optional), CATT, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo - Not support: ZTE, MediaTek, Intel, Fujitsu, Apple, Qualcomm, vivo Slot/W2 ref: - Alt1: Samsung, OPPO, Ericsson, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo - Alt2A: ZTE, Intel, Fujitsu, LG, Qualcomm, vivo, - Alt2B: Intel Proposal 2.B.1: - Support/fine: Samsung, Ericsson, Qualcomm, CATT, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo (optional TBD later), ZTE, MediaTek, Intel, Fujitsu, Apple, Qualcomm, vivo, NTT DOCOMO, Google (change X=2 2nd ref to last slot/W2), NEC, Fraunhofer IIS/HHI, Doppler W2 quantization (proposals 2.I.1/2 from RAN1#111): - Support/fine: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, Qualcomm, vivo (2.I.2), CATT, Spreadtrum, Xiaomi, NTT DOCOMO, NEC, Huawei/HiSi, Google, Fraunhofer IIS/HHI, Lenovo, - Not support: Doppler bitmap: - Alt1: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, CATT, Spreadtrum, Nokia/NSB, Lenovo, Google - Alt2: - Alt3A: MediaTek, Fraunhofer IIS/HHI, OPPO, Ericsson, Qualcomm, CATT (2nd), Xiaomi, - Alt3B: MediaTek, - Alt3C: Qualcomm, - Alt4: vivo, Proposal 2.E.1: - Support/fine: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, CATT, Spreadtrum, Nokia/NSB, Lenovo, Google, Fraunhofer IIS/HHI, Qualcomm, Xiaomi, NTT DOCOMO, NEC (after Q is finalized), Huawei/HiSi, Fraunhofer IIS/HHI, - Not support: vivo (must add Alt4) Proposal 2.B.1 remains as is, but I added brackets and bracketed text suggested by Google to check if it is agreeable to other companies. If not, we will use the previous version. Offline proposal 2.B.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrices - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the [middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrices][last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrices] Re Doppler quantization, it seems the previous proposals are agreeable to the group hence unchanged. Re Doppler bitmap, if vivo manages to convince some companies to support their Alt4, it will be added as one candidate. Else proposal 2.E.1 remains as is. |
| CMCC | Offline Proposal 2.B.1 We are OK with this proposal considering current situation, although we don’t like solutions with basic feature and optional feature merged. As for the bracketed text, we tend to agree with Google, the second CSI is associated with the last slot may be helpful for gNB’s CQI interpolation. Offline Proposal 2.D.1/2.D.2 Support. Offline proposal 2.E.1 Our first preference is Alt1 as simple extension of legacy design. Also, we are open for Alt3A if enough SLS results do show good UPT vs PMI overhead trade-off under sufficiently diverse configurations. |
| InterDigital | Q7.1 We support X=1 as a basic feature and X=2 as a UE optional feature. Q9.1 We are supportive of Proposal 2.I.1 and 2.I.2. Q10 We support Alt 1. |
| Mod V31 | This is the current summary Doppler CQI: Add X=2: - Support: Samsung, Ericsson, Qualcomm (only 2A, optional), CATT, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo - Not support: ZTE, MediaTek, Intel, Fujitsu, Apple, Qualcomm, vivo Slot/W2 ref: - Alt1: Samsung, OPPO, Ericsson, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo - Alt2A: ZTE, Intel, Fujitsu, LG, Qualcomm, vivo, - Alt2B: Intel Proposal 2.B.1: - Support/fine: Samsung, Ericsson, Qualcomm, CATT, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo (optional TBD later), ZTE, MediaTek, Intel, Fujitsu, Apple, Qualcomm, vivo, NTT DOCOMO, Google (change X=2 2nd ref to last slot/W2), NEC, Fraunhofer IIS/HHI, CMCC (tend to agree with Google), IDC, Doppler W2 quantization (proposals 2.I.1/2 from RAN1#111): - Support/fine: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, Qualcomm, vivo (2.I.2), CATT, Spreadtrum, Xiaomi, NTT DOCOMO, NEC, Huawei/HiSi, Google, Fraunhofer IIS/HHI, Lenovo, CMCC, IDC, - Not support: Doppler bitmap: - Alt1: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, CATT, Spreadtrum, Nokia/NSB, Lenovo, Google, CMCC, IDC, - Alt2: - Alt3A: MediaTek, Fraunhofer IIS/HHI, OPPO, Ericsson, Qualcomm, CATT (2nd), Xiaomi, - Alt3B: MediaTek, - Alt3C: Qualcomm, - Alt4: vivo, Proposal 2.E.1: - Support/fine: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, CATT, Spreadtrum, Nokia/NSB, Lenovo, Google, Fraunhofer IIS/HHI, Qualcomm, Xiaomi, NTT DOCOMO, NEC (after Q is finalized), Huawei/HiSi, Fraunhofer IIS/HHI, CMCC, IDC - Not support: vivo (must add Alt4) Given the above summary, the proposals remain unchanged. |