3GPP TSG-RAN WG1 Meeting #112	R1- 2301303

Athens, February 27th – March 3rd, 2023

Agenda Item:	9.11.4

Source:	Ericsson

Title:	On improved GNSS operation in IoT NTN

Document for:	Discussion

# 1	Introduction

One objective of the ongoing Release 18 WI on IoT NTN [1][2] is to improve the GNSS operation during long connections.

“Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1,RAN2]”

In this contribution, we provide our views on improved GNSS operation for IoT NTN.

# 2	Improved GNSS operation for IoT NTN

In the following subsections, we share our views on different GNSS-related aspects discussed in RAN1#110.

## 2.1	Closed loop time and frequency correction

In RAN1#109-e, the following agreement was made on closed loop time and frequency correction.

Agreement

Closed loop time and frequency correction, with potential enhancements, for IoT-NTN is considered to reduce the need for UE to update GNSS position fix in long connection time

In NTN, a UE can adjust its timing autonomously as long as it has a valid GNSS position. It is already possible to support closed loop timing correction in NTN as in terrestrial eMTC/NB-IoT. A key question is if the existing Timing advance (TA) maintenance procedure is sufficient to maintain a moving UE’s timing in IoT NTN. To address this, recall that the additional timing uncertainty in NTN stems from a) satellite’s motion and b) UE’s motion. It is expected that an IoT NTN UE will apply segmented uplink pre-compensation to account for the timing drift due to satellite’s motion. Therefore, for the sake of argument, let us assume a worst-case TA error due to satellite motion of 0.39 μs (i.e., 12*Ts where Ts=32.55 ns) for an eMTC UE. We note that this value is an upper bound on the maximum possible TA error due to satellite motion at the end of an uplink transmission segment assuming that the UE is stationary and the satellite contributes to a worst-case TA drift of 100 μs/s. We note that for NR NTN, RAN4 has relaxed the initial transmit timing error requirement from 12*Ts to 29*Ts to account for errors in the satellite/UE positions. If we assume a similar relaxation for eMTC NTN (which has a minimum transmit timing error requirement of 12*Ts in terrestrial networks), the net timing error due to satellite/UE motion needs to be within 17*Ts for eMTC NTN. Assuming a worst-case TA error of 12*Ts due to satellite motion at the end of a transmission segment, there remains 5*Ts (0.16 μs) to account for the TA error due to UE mobility. In Table 1, we provide the time duration until which the UE can meet the uplink transmit timing error budget for various UE speeds. The TA error rate due to UE mobility is estimated for LEO at 600 km and 30 degrees elevation angle based on the data provided in Table 1 in [3].

Table 1 Impact of UE mobility on timing error requirements for eMTC NTN.

| UE speed | TA error rate due to UE mobility only | TA error due to satellite’s motion only | Time to reach the timing error limit of 17*Ts |
| --- | --- | --- | --- |
| 3 km/h | 0.00467 μs/s | 0.39 μs | 34.3 s |
| 60 km/h | 0.0967 μs/s | 0.39 μs | 1.65 s |
| 120 km/h | 0.193 μs/s | 0.39 μs | 0.83 s |

Without closed loop timing correction, Table 1 shows that for 3 km/h, the UE may need to correct its timing to account for TA error due to UE mobility either via a TA command (TAC) or by refreshing its GNSS after 34 s. At 120 km/h, such an intervention will be needed every 0.83 s. Note that in this analysis, we have assumed that the UE already corrects its timing every segment to account for the TA error due to satellite motion. To conclude, the existing closed loop timing control mechanism is sufficient to address the uncertainty in UE’s position.

The existing TAC mechanism is sufficient to address the timing error due to incorrect UE position in IoT NTN.

However, minor enhancements to closed loop mechanism may be needed to enable an energy efficient operation that avoids GNSS position fix. For instance, the network should have the possibility to indicate to the UE if it can continue to transmit on the uplink even if the GNSS position validity duration has expired. This is essential because the role of closed-loop timing control is to reduce the need of GNSS reacquisition – so even if the GNSS position is invalid, the network may continue to send TACs to help the UE maintain its synchronization.

Network can potentially indicate to an IoT NTN UE in connected mode if it can continue its uplink transmission after GNSS validity duration has expired.

Moreover, if the network solely relies on TACs to correct UE’s timing before or after GNSS position validity expiry, the signalling overhead could be very high due to the large timing drift. A potential solution is to allow the UE to compute its TA values (until a new TAC is received) based on the service link and common TA drift parameters, where the parameters related to the service link drift can either be tracked by the UE or indicated by the network. This semi-autonomous approach may help reduce the signalling overhead as fewer TACs will be needed; and the UE need not perform a GNSS position fix to correct its timing.

For IoT NTN, it is possible for a network to broadcast drift parameters for common TA. If the network additionally indicates the UE-specific timing drift parameters for the service link to a UE in connected mode, it may enable the UE to update its TA values accordingly (until a new TAC is received) to compensate for its inaccurate GNSS position.

Network can optionally indicate UE-specific service link timing drift parameters to an IoT NTN UE in connected mode.

Upon GNSS position expiry in connected mode, an IoT NTN UE may use UE-specific service link drift information (in addition to common TA parameters) to calculate TA values before receiving the next TA command.

Unlike the closed loop TA mechanism, the closed loop frequency adjustment (FA) mechanism is not currently supported. Based on the results reported by the proponents of closed-loop FA, it seems that the frequency error in IoT NTN is comparable to that in terrestrial networks, barring some corner cases. Therefore, we do not see a strong need to design and introduce a new closed-loop FA signalling mechanism for IoT NTN.

Closed loop frequency correction mechanism should only be specified if absolutely necessary.

## 2.2	GNSS acquisition in connected mode

In RAN1#109-e, it was concluded that an IoT NTN UE may need to re-acquire a valid GNSS position fix during a long connection. In addition, the following related agreements were made in RAN1#110, RAN1#110-bis-e and RAN1#111.

Agreement

At least the following options can be considered on GNSS measurement in connected for potential enhancements for improved GNSS operations:

- Option 1: UE re-acquires GNSS position fix during RLF procedure

- Option 2: UE re-acquires GNSS position fix with a new gap

Note: this does not imply that a Rel-18 IoT NTN UE is mandated to support one or both of the options.

Agreement

Further study on whether there is a need for potential enhancements on the following for long connection time

- UE triggered GNSS measurement.

- Network triggered GNSS measurement.

Agreement

Support eNB to at least aperiodically trigger UE to make GNSS measurement.

Agreement

If eNB aperiodically triggers UE to make GNSS measurement, a MAC CE is used.

Agreement

For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap

  - FFS details of gap configuration

Agreement

The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement

  - FFS based on configured timing

If the UE must acquire GNSS position before losing the connection, it can do so using a measurement gap. Also, it will report GNSS assistance information to the network anyways, which is all the network needs to configure a measurement gap for GNSS reacquisition. Before accessing the network, the UE will already have a valid GNSS position. If the GNSS position fix is made at least every 4 hours, it will correspond to a hot start. As a result, we mainly need to focus on hot start when specifying the possible values for the GNSS gap duration. Nonetheless, due to UE mobility, there might be scenarios where a warm start will be needed. Therefore, we put forth the following proposal.

Network to configure the GNSS measurement gap duration from the following set of values: {1, 2, 5, X} seconds, where X is FFS.

RAN1 has already agreed to support network-triggered GNSS measurements, which can be realized using GNSS measurement gaps. Additionally, it may be possible for a UE to acquire a position fix during the inactive mode of connected mode discontinuous reception (C-DRX). We think that it is more appropriate to hold the discussion related to C-DRX in RAN2.

The discussion on supporting UE-triggered GNSS measurements during the inactive mode of C-DRX should be left to RAN2.

## 2.3	GNSS assistance information

In RAN1#109-e, RAN1#110 and RAN1#111, the following agreements were made for GNSS assistance information.

Agreement

GNSS assistance information that UE reports to eNB at least consists of:

- GNSS position fix time duration for measurement

- GNSS validity duration

Agreement

When eNB triggers UE to make GNSS measurements, UE re-acquires GNSS position fix.

- FFS details of signalling

- FFS how UE reports GNSS assistance information after eNB trigger and the detailed content

- Note: further discuss whether a UE is expected to handle all eNB triggers

Agreement

UE reports GNSS position fix time duration for measurement at least during the initial access stage.

    - which message carries this information is up to RAN2

Agreement

In connected mode, UE may report GNSS validation duration with MAC CE.

We need to address the case where the GNSS position expires in the connected mode. In this case, the UE may report GNSS assistance information in an RRC message after acquiring the GNSS position fix if configured by the network.

Upon expiry of GNSS position validity, a UE in connected mode should report the GNSS assistance information after GNSS reacquisition if configured by the network.

If a UE in connected mode has already reported its GNSS assistance information, the UE may not be required to report it again during the same connection if the contents of the GNSS assistance information report are identical to the previous GNSS report.

If a UE in connected mode has already reported its GNSS assistance information to the network, the UE may not be required to report it again during the same connection provided that the GNSS assistance information is expected to remain unchanged.

Finally, we also need to discuss the format of reporting the GNSS measurement time duration. Before accessing the network, the UE will already have a valid GNSS position. If the GNSS position fix is made at least every 4 hours, it corresponds to a hot start. As a result, we mainly need to focus on hot start when specifying the format of GNSS measurement time duration for IoT NTN. Nonetheless, due to UE mobility, there might be scenarios where a warm start will be needed. Therefore, we put forth the following proposal.

UE to report its GNSS measurement time duration in connected mode using a 2-bit field from the following set of values: {1, 2, 5, X} seconds, where X is FFS.

# 3	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	The existing TAC mechanism is sufficient to address the timing error due to incorrect UE position in IoT NTN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Network can potentially indicate to an IoT NTN UE in connected mode if it can continue its uplink transmission after GNSS validity duration has expired.

Proposal 2	Network can optionally indicate UE-specific service link timing drift parameters to an IoT NTN UE in connected mode.

Proposal 3	Upon GNSS position expiry in connected mode, an IoT NTN UE may use UE-specific service link drift information (in addition to common TA parameters) to calculate TA values before receiving the next TA command.

Proposal 4	Closed loop frequency correction mechanism should only be specified if absolutely necessary.

Proposal 5	Network to configure the GNSS measurement gap duration from the following set of values: {1, 2, 5, X} seconds, where X is FFS.

Proposal 6	The discussion on supporting UE-triggered GNSS measurements during the inactive mode of C-DRX should be left to RAN2.

Proposal 7	Upon expiry of GNSS position validity, a UE in connected mode should report the GNSS assistance information after GNSS reacquisition if configured by the network.

Proposal 8	If a UE in connected mode has already reported its GNSS assistance information to the network, the UE may not be required to report it again during the same connection provided that the GNSS assistance information is expected to remain unchanged.

Proposal 9	UE to report its GNSS measurement time duration in connected mode using a 2-bit field from the following set of values: {1, 2, 5, X} seconds, where X is FFS.

# References

RP 211601, “WID on IoT NTN enhancements”, RAN #95e, Electronic Meeting, March 17–23, 2022.

RP-223519, “Revised WID on IoT NTN enhancements”, MediaTek, RAN#98e, December 12-16, 2022.

R1-2207737, “Feature lead summary #2 of AI 9.12.4 on improved GNSS operations” RAN1#110, Toulouse, August 22-26, 2022.