3GPP TSG RAN WG1 #112		   		 	         	    	         R1-2301318

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:  9.1.1.1

Source:	ITRI

Title:	Discussion on unified TCI framework extension for multi-TRP

Document for:	Discussion and Decision

Introduction

In RAN1 #111, the following agreements were approved:

Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field

In this contribution, we provide our views on unified TCI framework extension for multi-TRP.

Unified TCI framework

S-DCI based MTRP

In Rel-17, two types of unified TCI framework on single-TRP operation are introduced, where one type is joint TCI state and another type is separate TCI state. The benefit of unified TCI framework is not only for single TRP operation, but also could be extended to multiple TRP operation. If UE has capability of beam correspondence (BC), two joint TCI states can be applied for unified TCI framework, where each joint TCI state is associated with one TRP. If UE has capability of no beam correspondence (NBC), separate TCI state type can be configured, where up to 2 UL TCI states and up to 2 DL TCI states can be applied for multi-TRP operation. However, if separate TCI state type is configured for multi-TRP operation, the number of MAC CE activated codepoints of TCI field may not be sufficient for all the possible combination(s) of DL/UL TCI states because there may be up to two DL TCI states and/or up to two UL TCI states indicated by a TCI codepoint. In this case, we can consider to increase the max number of MAE CE activated TCI codepoints and increase the max number of TCI field bits for flexible TCI state indication. For DCI overhead reduction, if the number of activated TCI codepoints is more than 8, the bit width for TCI field is determined as max3, log2I bits, where I is the number of the max number of MAE CE activated TCI codepoints.

Proposal 1: Increase the max number of MAE CE activated TCI codepoints and increase the max number of TCI field bits for flexible TCI state indication. If the number of activated TCI codepoints is more than 8, the bit width for TCI field is determined as max3, log2I bits, where I is the number of the max number of MAE CE activated TCI codepoints.

The new indicator filed for PDSCH reception(s)

In unified TCI framework for S-TRP operation, the QCL assumption of the DMRS port for the scheduled PDSCH is not indicated by the TCI filed in scheduling DCI with DL assignment but indicated by the applied joint/DL TCI state of previous DCI as shown in Figure 1. If the unified TCI framework was extended to M-TRP operation, the scheduling DCI with DL assignment cannot select one or two joint/DL TCI states from the applied joint/DL TCI states, which may loss the scheduling flexibility.

Figure 1 DM-RS ports are QCLed with applied TCI state(s) indicated by previous DCI

For the scheduling flexibility, we had agreed that a new indication field is supported to determine which one or both of the indicated joint/DL TCI states applying to the scheduled/activated PDSCH reception, where the detail is FFS. Considering all the possibility of indicated TCI state selection, a new 2-bit indicator field (as shown in Table 1 and Fig. 2) is sufficient for indicating TCI state selection for PDSCH reception, i.e., one codepoint for selecting the 1st applied joint/DL TCI state, one codepoint for selecting the 2nd applied joint/DL TCI state, and one codepoint for selecting all the applied joint/DL TCI states.

Proposal 2: Support a new 2-bit indicator field for indicating TCI state selection for PDSCH reception, i.e., one codepoint for selecting the 1st applied joint/DL TCI state, one codepoint for selecting the 2nd applied joint/DL TCI state, and one codepoint for selecting all the applied joint/DL TCI states.

Table 1 New 2-bit indicator field

Figure 2 Dynamic joint/DL TCI state(s) selection for PDSCH reception from applied joint/DL TCI state(s) according to the new indicator field

Besides, the presence of the new indication is configurable by RRC. When the new indicator field is not present in DCI format 1_1/1_2, it’s not clear how the UE applies which one or both indicated joint/DL TCI state(s) for PDSCH reception. In our opinion, the NW should still have flexibility to decide which one or both indicated joint/DL TCI state(s) as the default TCI state(s) for PDSCH reception based on DL traffic load or DL channel quality. Therefore, when the new indicator field is not present in DCI format 1_1/1_2, the default indicated joint/DL TCI state(s) is configured by RRC and the default indicated joint/DL TCI state(s) could be the 1st indicated joint/DL TCI state, the 2nd indicated joint/DL TCI state, or both of indicated joint/DL TCI states.

Proposal 3: When the new indicator field is not present in DCI format 1_1/1_2, the default indicated joint/DL TCI state(s) is configured by RRC and the default indicated joint/DL TCI state(s) could be the 1st indicated joint/DL TCI state, the 2nd indicated joint/DL TCI state, or both of indicated joint/DL TCI states.

Default TCI state(s) for potential PDSCH reception in unified TCI framework

In Rel-17 TCI framework for S-DCI based MTRP, if the time offset between the reception of the DL DCI and the corresponding PDSCH is less than a threshold, the default TCI states for potential PDSCH reception are indicated by the lowest codepoint among the TCI codepoints containing two different TCI states. However, in unified TCI framework for S-DCI based MTRP, if the default TCI states are the two different TCI states indicated by the lowest codepoint in the TCI field and the default TCI states are different from the indicated TCI states. In that case, the UE would need to dynamically switch between default TCI states for potential PDSCH reception and the indicated TCI states for other DL receptions. Hence, some modifications are required to ensure unified TCI state(s) for all DL receptions. As shown in Fig.2, We can set the default TCI state(s) for potential PDSCH reception as the indicated joint/DL TCI state(s) if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold.

Figure 3 Default TCI state(s) for S-DCI based MTRP in unified TCI framework

Proposal 4: For S-DCI based MTRP in unified TCI framework, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold, the indicated joint/DL TCI states are applied to PDSCH reception.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: Increase the max number of MAE CE activated TCI codepoints and increase the max number of TCI field bits for flexible TCI state indication. If the number of activated TCI codepoints is more than 8, the bit width for TCI field is determined as max3, log2I bits, where I is the number of the max number of MAE CE activated TCI codepoints.

Proposal 2: Support a new 2-bit indicator field for indicating TCI state selection for PDSCH reception, i.e., one codepoint for selecting 1st applied joint/DL TCI state, one codepoint for selecting 2nd applied joint/DL TCI state, and one codepoint for selecting all the applied joint/DL TCI states.

Proposal 3: When the new indicator field is not present in DCI format 1_1/1_2, the default indicated joint/DL TCI state(s) is configured by RRC and the default indicated joint/DL TCI state(s) could be the 1st indicated joint/DL TCI state, the 2nd indicated joint/DL TCI state, or both of indicated joint/DL TCI states.

Proposal 4: For S-DCI based MTRP in unified TCI framework, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold, the indicated joint/DL TCI states are applied to PDSCH reception.