3GPP TSG RAN WG1 #112		R1- 2301418

Athens, Feb. 27th – March 3rd, 2023

Agenda item:	9.5.1.2

Source: 	Qualcomm Incorporated

Title: 	Measurements and Reporting for SL Positioning

Document for:		Discussion and Decision

# Introduction

A work item has been agreed for positioning in RAN #98-e with the following objective for sidelink positioning [1]:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we discuss sidelink positioning measurements and associated signaling.

Specifying Unified or Separate Methods

RAN1 has left the issue of whether SL positioning methods are separate methods or one unified positioning method. In our view, specification as separate method provides more flexibility and forward compatibility. In sidelink, UEs can have a long lifetime, e.g. automobiles, and it is necessary to ensure that old devices and new devices can exist together in the same resource pool. The requirement for devices to operate in out of coverage scenarios means that per-UE configuration can be used to update the configuration for different UEs either.

Under those circumstances, it becomes infeasible to change the unified report format to accommodate new measurements or methods without impacting compatibility between devices supporting different releases.

Observation 1: A unified positioning method is limited in terms of forward compatibility and its ability to support devices implementing different releases.

By specifying the positioning methods separately, new methods and associated measurements can be introduced in future releases without impacting existing methods. Consequently, it is simple to support a mix of releases and device generations in the same resource pool: UEs can discard reports and requests for unsupported or unknown methods. Given, their forward compatibility, we propose to have separate sidelink positioning methods specified.

Proposal 1: Separate methods are specified for sidelink positioning.

Reporting Type

During the sidelink positioning study phase, RAN1 agreed to consider the time-domain behavior of the measurement report and the layer used to report it [2]:

Agreement With regards to the Sidelink Positioning measurement report, - Study the contents of the measurement report (e.g. time stamp(s), quality metric(s), ID(s), angular/timing/power measurements, etc) - Study the time domain behavior of the measurement report (e.g. one-shot, triggered, aperiodic, semi-persistent, periodic) - FFS whether the Sidelink Positioning measurement can be a high-layer report and/or a lower layer report.

A measurement report requires significant computations on the UE side and the measuring process itself has processing time that is comparable or longer than that required to process lower layer signals. Furthermore, the final destination of the report is higher layers. Hence, the low latency of lower layer triggering and signaling will not results in noticeable delay gains, which could be also masked by resource selection latency. On the other hand, lower layer signaling lacks security features present in higher-layer signaling and could limit forward compatibility and introduction of new features due to backward compatibility constraints arising from coexistence of UE supporting different releases in the same resource pool. Hence, we propose to have measurement report triggering be done by higher layers and to have the measurements reported in a higher-layer report. A higher-layer report would be transmitted as any other sidelink data in a resource pool that support sidelink communications and would use existing sidelink procedures.

Proposal 2: Measurement report is a higher-layer report in a resource pool that support sidelink communications.

Proposal 3: Measurement report is triggered by higher layers.

Similar to triggering, deactivation can also be by higher layers, enabling support for periodic/semi-persistent transmission of measurement reports, while minimizing control overhead. In some scenarios, it is desirable to have a single measurement report, e.g. for stationary objects. Here, supporting one-shot report triggering would be more efficient than activating, then deactivating, reporting. Both one-short (aperiodic) and periodic (semi-persistent) reporting are beneficial and we propose to support both.

Proposal 4: One-shot (aperiodic) and periodic (semi-persistent) reports are supported.

Proposal 5: Higher layers can deactivate a periodic (semi-persistent) reporting process.

Reported Measurements

RAN1 agreed to introduce a number of measurements for sidelink positioning and to consider reporting additional elements [2][3]:

Agreement With regards to the Positioning methods supported using SL-PRS measurements - at least the following measurements should be introduced: - SL-PRS based Rx-Tx measurement - SL-PRS based RSTD measurement - SL-PRS based RSRP measurement - SL-PRS based RSRPP measurement - SL-PRS based RTOA measurement - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement Agreement For the content of the sidelink positioning measurement report, potential elements may include at least the following: - One or more sidelink positioning measurement(s) - Timestamp(s) associated with a sidelink positioning measurement - Quality metric(s) associated with a sidelink positioning measurement - Identification Information for a sidelink positioning measurement - FFS any detail for the above

Rx-Tx time difference measurement is required for RTT calculations, and one question is whether the transmit time used in the calculation is the actual transmission time or the closest subframe time. In Uu, the Rx-Tx time difference is defined using the time of the subframe closest in time to the subframe in which PRS is received. In sidelink, a more robust solution is needed due to high UE mobility in some application, changes in synchronization reference between Rx and Tx, and changes in TA and differences in its value between UEs when uplink timing is used. Using the actual transmission time addresses these issues without increasing signaling or procedure complexity.

Observation 2: Using time of the subframe closest in time to the subframe in which SL-PRS is received would increase measurement uncertainty due to UE mobility, differences and changes in synchronization sources, and differences and changes in TA.

Proposal 6: Actual SL-PRS transmission time is used for the definition of the UE SL Rx-Tx time difference measurement.

In addition, to measurement, it is necessary for the report to contain identification information for the resource pool where SL-PRS is received, in addition to any source, destination, and session identifiers. Other than identification, the report should contain the measurement(s), and quality metrics as in the SI agreement.

Proposal 7: The measurement report contains the measurement, information to identify the resource pool where SL-PRS was received, source ID, destination ID, session ID, and quality metrics.

An angle of arrival can be provided in a global coordinate system (GCS) or a local coordinate system (LCS). Both systems are useful and relevant to different scenarios. For example, GCS can be used for absolute positioning. LCS is applicable to relative positioning scenarios. For example, if a vehicle determines that it is 0 degrees away from another vehicle’s direction of travel, it can deduce that a collision is likely and take proper action to avoid it.

Proposal 8: Both GCS and LCS are supported for reporting SL-AoA measurements.

Synchronization errors among anchor UEs is an issue that affects TDoA positioning in general. This was noted in the RAN1 agreement defining SL-TDoA and an observation:

Agreement With regards to SL-TDOA positioning method for SL-only positioning, - It corresponds to a method where SL-PRS is being transmitted from multiple anchor UEs to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor UEs (i.e. UL-TDOA-like operation) at least for the purpose of absolute positioning estimation of the target UE - Study the detailed procedure, necessary signalling for SL-TDOA, method(s) to mitigate impact of synchronization error between multiple anchor UEs including whether such method(s) are needed. Observation SL absolute positioning performance may be degraded due to uncertainty in the anchor UEs’ location coordinates and synchronization error (for SL-TDOA) between anchor UEs.

The same issue was also identified for Uu TDoA positioning, where no solution was specified. We propose to also apply the same to sidelink. For sidelink positioning, TDoA could be deployed in many different scenarios, e.g. anchors are RSUs connected by a wired link, anchors are RSUs connected wirelessly via sidelink or a network, anchors are in a factory, … Given the large number of different scenarios, it is preferrable to leave methods of mitigating synchronization error up to deployment. Synchronization quality metrics could be reported if helpful.

Proposal 9: Methods to mitigate impact of synchronization error between multiple anchor UEs are not specified and are left up to deployment. FFS whether a synchronization quality metric is reported.

Conclusions

Observation 1: A unified positioning method is limited in terms of forward compatibility and its ability to support devices implementing different releases.

Observation 2: Using time of the subframe closest in time to the subframe in which SL-PRS is received would increase measurement uncertainty due to UE mobility, differences and changes in synchronization sources, and differences and changes in TA.

Proposal 1: Separate methods are specified for sidelink positioning.

Proposal 2: Measurement report is a higher-layer report in a resource pool that support sidelink communications.

Proposal 3: Measurement report is triggered by higher layers.

Proposal 4: One-shot (aperiodic) and periodic (semi-persistent) reports are supported.

Proposal 5: Higher layers can deactivate a periodic (semi-persistent) reporting process.

Proposal 6: Actual SL-PRS transmission time is used for UE SL Rx-Tx time difference measurement.

Proposal 7: The measurement report contains the measurement, information to identify the resource pool where SL-PRS was received, source ID, destination ID, session ID, and quality metrics.

Proposal 8: Both GCS and LCS are supported for reporting SL-AoA measurements.

Proposal 9: Methods to mitigate impact of synchronization error between multiple anchor UEs are not specified and are left up to deployment. FFS whether a synchronization quality metric is reported.

- References

- RP-223549, “New WID on Expanded and Improved NR Positioning,” Intel Corporation, CATT, Ericsson. RAN #98-e.

- Chair’s notes, RAN1 #109, Toulouse.

- Chair’s notes, RAN1 #111, Toulouse.

- TS 38.211, “Physical channels and modulation (Release 17),” 3GPP.