3GPP TSG RAN WG1 #112			R1-2301487

Athens, Greece, February 27th – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on evaluation on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of the evaluation on use cases as following.

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution, we discuss the evaluation on AI/ML for beam management.

- Discussion on the evaluation on AI/ML for beam management

  - Sub use-cases description

At the RAN1#109-e meeting, the agreement supporting spatial domain beam prediction and temporal beam prediction for characterization and baseline performance evaluations was made as following [2].

Agreement

For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

- FFS: details of BM-Case1 and BM-Case2

- FFS: other sub use cases

Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

For both spatial domain and temporal-domain beam prediction, the agreement was made regarding the general simulation approach for dataset construction and performance evaluation, as well as the evaluation metric such as the complexity of AI/ML model and the performance comparison between AI-based method and baseline method. The simulation results in this contribution were conducted based on the agreed assumptions. In the subsequent sections, we discuss the evaluation methodology and simulation results of BM-Case 1 and BM-Case 2.

  - Evaluation methodology

2.2.1. Spatial domain beam prediction (BM-Case 1)

In this section, we provide our view and the assumption on the evaluation methodology of intermediate performance and generalization performance.

2.2.1.1. Rx beam determination

At the RAN1 #111 meeting, following agreement was achieved for the options for Rx beam assumption for DL Tx beam prediction.

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

- Option 2: Measurements of specific Rx beam(s)

  - Option 2a: Measurements of specific Rx beam(s) per model input sample

  - Option 2b: Measurements of specific Rx beam(s) for all model input sample

  - FFS how to select the specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

- Other options are not precluded and can be reported by companies.

In this contribution, we mainly focus on the NW side model for Tx beam prediction. When model inference is performed at NW side, Set B measurement results should be reported from UE to NW among available measurements. These available beam measurements that can be used for Set B is defined as Set C in this contribution. In the existing framework, the Rx beam determination is up to UE for the beam measurements. However, generally it is assumed that UE reports the beam measurements of the best Rx beam from all Rx beam or specific Rx beams, such as Rx beams at the panel which can be available at certain time. For those reasons, , we mainly pay attention to Option 1 (the best beam among all Rx beams) and Option 2a (the best beam among specific Rx beams) in this contribution. Moreover, we will provide two choices on how to select the ‘best’ Rx beam with exhaustive beam sweeping at UE side for Option 1. The detailed description of the simulation cases is as follows:

Option 1:  Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample. There are two options in selecting the “best” Rx beam: a) the ‘best’ Rx beam for the best Tx-Rx beam pair is selected after the exhaustive beam sweeping, which is represented by Simulation Case 0, as shown in Fig.1; b) the ‘best’ Rx beam for each Tx beam in the Set B is selected after exhaustive beam sweeping, which is represented by Simulation Case 1, as shown in Fig.2. Since all Rx beams are considered during the Set B measurement, there is no need for additional Rx beam sweeping after the Tx beam prediction except for top-K beam prediction.

Simulation Case 0: Tx beam prediction with best Rx beam for the best Tx beam within Set B.

Figure 1. Simulation Case 1’

Simulation Case 1: Tx beam prediction with the best Rx beam for each Tx beam within Set B

Figure 2. Simulation Case 1

Option 2a: Measurements of specific Rx beam(s) per model input sample. The specific Rx beams are assumed in Set C measurements. Among the measured results, the measurements with Rx beam achieving the largest RSRP for each Tx beam is used as inputs for model inference. In other word, the best beam among a fixed group of Rx beams for each Tx beam is used for Set B. Since the Rx beams for Set C measurement are confined, the additional Rx sweeping may be needed after Tx beam prediction even when top-1 beam prediction is applied.

Simulation Case 3: Tx beam prediction with the best Rx beam among specific Rx beams for each Tx beam within Set B

Figure 3. Simulation Case 3

Proposal 1: Study the Rx beam(s) determination methods for the Set B measurement for Tx beam prediction.

2.2.1.2. Generalization performance

At the RAN1#111 meeting, the following agreement was further made for verifying the generalization performance of an AI/ML model over various scenarios/configurations for beam management [3].

Agreement - For generalization performance verification, consider the following - Scenarios - Various deployment scenarios, - e.g., UMa, UMi and others, - e.g., 200m ISD or 500m ISD and others - e.g., same deployment, different cells with different configuration/assumption - e.g., gNB height and UE height - FFS: e.g., Carrier frequencies - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, - e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings) - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 - Other scenarios/configurations(parameters and settings) are not precluded and can be reported by companies.

In this contribution, we investigate the various configurations on UE parameters, i.e. different available Rx beams at UE side, for Tx beam prediction. In this generalization verification, we assume Simulation Case1 and Simulation Case 3 as the generalization case (GC) 1 where the single set of available Rx beams at UE side is assumed. If one model is specific to one Rx beam configuration, GC 1 can be considered. In addition to GC 1, we provide Simulation Case 2 and Simulation Case 4 where the multiple Rx beam configurations are mixed. GC 3 considers the scenario where one NW side model is used for multiple UEs having different Rx beam configurations.  The detail is as follows:

GC 1:  Simulation Case 1 and Case 3 as in Section 2.2.1.1.

GC 3:  Simulation Case 2 and Case 4 as depicted below. For both of the cases, there are two groups of available Rx beams at UE side. Per model inference occasions, only one group are used for beam measurements.

Simulation Case 2: Tx beam prediction with the best Rx beam for each Tx beam within Set C, where there are two Set C consisting of different available Rx beams.

Figure 4. Simulation Case 2

Simulation Case 4: Tx beam prediction with the best Rx beam among specific Rx beams for each Tx beam within Set C, where there are two Set C consisting of different available Rx beams. Set C measurements are performed with the fixed Rx beams in each Set C.

Figure 5. Simulation Case 4

As shown in Fig. 4 and Fig.5, two Set C consists of different Rx beams for Simulation Case 2 and Case 4. These cases are considered, where one NW side model is applied to multiple UEs with different Rx beam configuration.

In beam pair prediction, Rx beam information is essential. Since it is difficult for NW to obtain full DL Rx beam information due to the proprietary aspects, we put focus on UE side models for beam pair prediction. In case of UE side model, it is reasonable to assume that one model is specific to the same Rx beam configuration. On the other hand, if one UE side model is applied for multiple gNBs with different beam configuration, one model should be applicable to multiple Tx beam configurations. Hence, we investigate the various Set B for beam pair prediction, i.e. different Tx beams in Set B, for Tx-Rx beam prediction. In this generalization verification, we assume that the same Rx beams are always available at the UE side. Simulation Case 5 and Simulation Case 6 corresponding to generalization case 1 and case 3 are introduced as follows:

Simulation Case 5: Tx-Rx beam pair prediction with specific Tx beam configurations.

Figure 6. Simulation Case 5

Simulation Case 6: Tx-Rx beam pair prediction with mixed Tx beam configurations.

Figure 7. Simulation Case 6

2.2.1.3. Simulation assumption

In this contribution, the simulation assumption for the dataset generation in BM-Case1 follows the agreed evaluation methodology and the detailed information is listed in Table 1.

Table 1. Simulation parameters for dataset generation in BM-Case1

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | Uma with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | 3km/h |
| UE distribution | 80% indoor ,20% outdoor as in TR 38.901 |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | (M, N, P, Mg, Ng, Mp, Np), = (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ 64 downlink Tx beams(H(16)*V(4)) at NW side |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | (M, N, P, Mg, Ng, Mp, Np), = (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) 4 downlink Rx beams(H(4)) per UE panel at UE side |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | No beam correspondence |
| BS Tx Power | 40 dBm |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

2.2.2 Temporal beam prediction (BM-Case 2)

2.2.2.1. RS overhead reduction

At the RAN1#111meeting, the agreement on the evaluation of the overhead for BM-Case 2 was made as follows.

Agreement - For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on additional measurements - FFS: Option 3: RS OH reduction%=1-NLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction - Companies report the assumption on T1 and T2 patterns - Other options are not precluded and can be reported by companies.

Figure 8. T1 and T2 patterns in temporal beam prediction. (a) prediction of beam quality between each measurement/reporting (b) prediction of beam quality instead of measurement/reporting.

In temporal beam prediction, there are two patterns for T1 and T2 as shown in Fig.8: prediction of beam quality between each measurement/reporting (pattern A) and prediction of beam quality instead of measurement/reporting (pattern B). In case of pattern A, the time sequence of inputs for AI/ML model in T1 has different periodicity from one of outputs in T2. Since the beam prediction can be used to interpolate the beam quality between each measurement, the periodicity of beam measurement/reporting can be configured to be large compared to one without the beam prediction. On the other hand, the time sequence of inputs for AI/ML model has the same periodicity from one of outputs in pattern B. In this pattern, beam measurement/reporting can be skipped after certain number of measurements, because the beam prediction can compensate them based on the historical beam measurements. Thus, even though both patterns can reduce the overhead of measurement/reporting, one approach can lead to the large periodicity of measurement/reporting, while the other approach could enable skipping of measurement/reporting for a while. As both approaches bring the practical gain and the desired pattern can be different according to NW operation, it is beneficial to study both patterns for T1 and T2.

Proposal 2: Consider both Pattern A and Pattern B for temporal beam prediction.

2.2.2.2. Generalization performance

For the generalization performance of BM-Case2, we investigate the various scenarios on UE mobility, i.e., 30km/h,60km/h and 90km/h. For each Pattern A and Pattern B beam prediction, the generalization cases 1~3 are conducted under different assumptions of UE mobility. In this generalization performance, we adopted ‘average L1-RSRP difference of Top-1 predicted beam’ as the KPI for simplicity.

Table 2. Generalization cases (GC) for Pattern A/B

| Generalization case (GC) | Training dataset | Testing/inference dataset |  |  |
| --- | --- | --- | --- | --- |
|  |  | 30km/h | 30km/h | 30km/h |
| GC1/2 | 30km/h | GC1 | GC2 | GC2 |
|  | 60km/h | GC2 | GC1 | GC2 |
|  | 90km/h | GC2 | GC2 | GC1 |
| GC 3 | 30km/h:60km/h:90km/h=1:1:1 | GC3 | GC3 | GC3 |

2.2.2.3. Simulation assumption

On top of the simulation assumption in Section 2.2.1.3, the additional assumption for dataset generation in BM-Case 2 follows the agreed evaluation methodology and the detailed simulation parameters are listed in Table.3~5.

Table 3. Simulation parameters for dataset generation in BM-Case2

| Parameters | Values |
| --- | --- |
| UE Speed | 30Km/h, 60Km/h, 90Km/h |
| UE distribution | 100% outdoor as in TR 38.901 |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory | Random direction straight-line trajectories (Option 4) |
| UE orientation | Randomly per-UE chosen for UE orientation initially and is fixed |
| UE rotation speed | 0 |

Table 4. Time domain parameters for Pattern A

| Parameters | Values |  |  |
| --- | --- | --- | --- |
|  | 30km/h | 60km/h | 90km/h |
| Periodicity of time instances for each measurement in T1 | 960ms |  | 640ms |
| Number of time instances for measurement in T1 | 5 |  |  |
| Periodicity of time instances for prediction in T2 | 10ms |  |  |
| Number of time instances for prediction in T2 | 95 |  | 63 |

Table 5. Time domain parameters for Pattern B

| Parameters | Values |  |  |
| --- | --- | --- | --- |
|  | 30km/h | 60km/h | 90km/h |
| Periodicity of time instances for each measurement in T1 | 160ms |  |  |
| Number of time instances for measurement in T1 | 5 |  |  |
| Periodicity of time instances for prediction in T2 | 160ms |  |  |
| Number of time instances for prediction in T2 | 5 |  |  |

  - Performance evaluation results

    - Spatial domain beam prediction (BM-Case 1)

The simulation in this section follows the assumption described in Section 2.2.1. The agreed report format is adopted to capture the simulation results.

2.3.1.1 Performance with different Rx beam assumptions

In this sub-section, the comparison between different Rx beam determination methods of “best” Rx beam for Option 1, and the comparison between Option 1 and Option 2a are provided.

Table 6. Evaluation results for Different selection of “best” Rx beam for Option 1

|  |  |  | NTT DOCOMO Rx beam Option1: Simulation Case 0 | NTT DOCOMO Rx beam Option1: Simulation Case 1 |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 64 |  |
|  |  | Number of beam pairs in Set B | 8 |  |
|  |  | Baseline scheme | Option 1 and Option 2 |  |
| AI/ML model input/output |  | Model input | L1-RSRP values of the 8 beam pairs |  |
|  |  | Model output | L1-RSRP values of the 64 beam pairs |  |
| Data Size |  | Training | 320010 |  |
|  |  | Testing | 80040 |  |
| AI/ML model |  | Short model description | FNN |  |
|  |  | Model complexity [No. of para./ model size] | 266K/1.1MB |  |
|  |  | Computational complexity | 267K |  |
| Evaluation results [AI/ML /Baseline Option 1 /Baseline Option 2] | Beam prediction accuracy (%) | Top-1 (%) | 52.0 / 100 / 4.22 | 44.9 / 100 / 2.83 |
|  |  | Top-1/5 (%) | 90.6 / 100 / 41.2 | 87.6 / 100 / 28.3 |
|  |  | Top-5/1 (%) | 92.5 / 100 / 4.22 | 89.4 / 100 / 2.83 |
|  | L1-RSRP Diff. | Average L1-RSRP diff.(dB) | 1.30 / 0 / 6.83 | 1.60 / 0 / 7.37 |
|  | System performance | RS overhead Reduction (%) | 0.80 / 0 / 0.88 |  |
|  |  | RS overhead | 13 / 64 / 8 |  |

As shown in Table 6, in general, both cases have better performance in both average L1-RSRP difference and beam prediction accuracy than baseline method. In addition, the performance of Simulation Case 0, where the “best” Rx beam for the best Tx-RX beam pair in Set C is selected and universally used for all the Tx beams, is better than Simulation Case 1, where the “best” Rx beam for each Tx beam in Set C is selected. The reason is the same Rx beam may provide similar channel characteristic across different Tx beams, and may make it easier for AI/ML model to learn the regularity.

Observation 1: For Tx beam prediction with Rx beams assumption Option 1:

- The beam prediction accuracy as well as average L1-RSRP are obviously better than baseline.

- The “best” Rx beam for the best Tx-Rx beam pair in Set C provides better performance than the “best” Rx beam for each Tx beam in Set C.

Table 7. Evaluation results for  Rx beam assumption Option 1 and Option 2a

|  |  |  | NTT DOCOMO Rx beam Option1: Simulation Case 1 | NTT DOCOMO Rx beam Option 2a: Simulation Case 3 |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 64 |  |
|  |  | Number of beam pairs in Set B | 8 |  |
|  |  | Baseline scheme | Option 1 and Option 2 |  |
| AI/ML model input/output |  | Model input | L1-RSRP values of the 8 beam pairs |  |
|  |  | Model output | L1-RSRP values of the 64 beam pairs |  |
| Data Size |  | Training | 320010 |  |
|  |  | Testing | 80040 |  |
| AI/ML model |  | Short model description | FNN |  |
|  |  | Model complexity [No. of para./ model size] | 266K/1.1MB |  |
|  |  | Computational complexity | 267K |  |
| Evaluation results [AI/ML /Baseline Option 1 /Baseline Option 2] | Beam prediction accuracy (%) | Top-1 (%) | 44.9 / 100 / 2.83 | 45.7 / 100 / 3.22 |
|  |  | Top-1/5 (%) | 87.6 / 100 / 28.3 | 86.8 / 100 / 30.9 |
|  |  | Top-5/1 (%) | 89.4 / 100 / 2.83 | 89.5 / 100 / 3.22 |
|  | L1-RSRP Diff. | Average L1-RSRP diff.(dB) | 1.60 / 0 / 7.37 | 1.65 / 0 / 7.23 |
|  | System performance | RS overhead Reduction (%) | 0.80 / 0 / 0.88 |  |
|  |  | RS overhead | 13 / 64 / 8 |  |

As shown in Table.7, since the available Rx beams to be measured at the UE side are restricted for Simulation Case 3, the “best” Rx beam among all Rx beams for each Tx beam   may not be selected after Set C beam measurements. This restriction on specific Rx beams leads to a bit performance loss in the average L1-RSRP difference loss and beam prediction accuracy. Moreover, due to the measurement of restricted Rx beams in Set C, additional Rx sweeping after the beam prediction may be required.

Observation 2: For Tx beam prediction with Rx beams assumption Option 1 and Option 2a:

- When the Rx beams in Set C is specific beams instead of all Rx beams (Option 2a), the prediction performance deteriorates a bit from the exhaustive Rx sweeping (Option 1) under the same Rx beam determination method.

2.3.1.2 Generalization performance

In this sub-section, for Tx beam prediction, the performance of GC1 and GC3 for Option 1 and Option 2a are provided. In addition, for Tx-Rx beam pair prediction, the performance of GC1 and GC3 for different Set B Tx beam configurations are provided.

Table 8. Evaluation results of GC1 and GC3 for Rx beam assumption Option 1 for Tx beam prediction

|  |  |  | NTT DOCOMO GC1: Simulation Case 1 | NTT DOCOMO GC2: Simulation Case 2 |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 64 |  |
|  |  | Number of beam pairs in Set B | 8 |  |
|  |  | Baseline scheme | Option 1 and Option 2 |  |
| AI/ML model input/output |  | Model input | L1-RSRP values of the 8 beam pairs |  |
|  |  | Model output | L1-RSRP values of the 64 beam pairs |  |
| Data Size |  | Training | 320010 |  |
|  |  | Testing | 80040 |  |
| AI/ML model |  | Short model description | FNN |  |
|  |  | Model complexity [No. of para./ model size] | 266K/1.1MB |  |
|  |  | Computational complexity | 267K |  |
| Evaluation results [AI/ML /Baseline Option 1 /Baseline Option 2] | Beam prediction accuracy (%) | Top-1 (%) | 44.9 / 100 / 2.83 | 45.8 / 100 / 2.78 |
|  |  | Top-1/5 (%) | 87.6 / 100 / 28.3 | 87.6 / 100 / 29.0 |
|  |  | Top-5/1 (%) | 89.4 / 100 / 2.83 | 90.1 / 100 / 2.78 |
|  | L1-RSRP Diff. | Average L1-RSRP diff.(dB) | 1.60 / 0 / 7.37 | 1.61 / 0 / 7.40 |
|  | System performance | RS overhead Reduction (%) | 0.80 / 0 / 0.88 |  |
|  |  | RS overhead | 13 / 64 / 8 |  |

Table 9. Evaluation results of GC1 and GC3 for Rx beam assumption Option 2a for Tx beam prediction

|  |  |  | NTT DOCOMO GC1: Simulation Case 3 | NTT DOCOMO GC2: Simulation Case 4 |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 64 |  |
|  |  | Number of beam pairs in Set B | 8 |  |
|  |  | Baseline scheme | Option 1 and Option 2 |  |
| AI/ML model input/output |  | Model input | L1-RSRP values of the 8 beam pairs |  |
|  |  | Model output | L1-RSRP values of the 64 beam pairs |  |
| Data Size |  | Training | 320010 |  |
|  |  | Testing | 80040 |  |
| AI/ML model |  | Short model description | FNN |  |
|  |  | Model complexity [No. of para./ model size] | 266K/1.1MB |  |
|  |  | Computational complexity | 267K |  |
| Evaluation results [AI/ML /Baseline Option 1 /Baseline Option 2] | Beam prediction accuracy (%) | Top-1 (%) | 45.7 / 100 / 3.22 | 46.1 / 100 / 3.08 |
|  |  | Top-1/5 (%) | 86.8 / 100 / 30.9 | 86.7 / 100 / 31.1 |
|  |  | Top-5/1 (%) | 89.5 / 100 / 3.22 | 89.6 / 100 / 3.08 |
|  | L1-RSRP Diff. | Average L1-RSRP diff.(dB) | 1.65 / 0 / 7.23 | 1.67 / 0 / 7.26 |
|  | System performance | RS overhead Reduction (%) | 0.80 / 0 / 0.88 |  |
|  |  | RS overhead | 13 / 64 / 8 |  |

As shown in Table.8 and Table.9, the AI/ML model trained with mixed assumptions of available Rx beams at UE side could have acceptable generalization performance, regardless of Option 1 and Option 2a. In this case, gNB could use a unified AI/ML model for different UEs even if UEs have different assumptions on the Rx beams for the Set B measurement.

Observation 3: For Tx beam prediction with Rx beam assumptions Option 1 and Option 2a:

- The AI/ML model trained with mixed Rx beam assumptions at UE side could provide acceptable generalization performance.

Table 10. Evaluation results of GC1 and GC3 for Tx-Rx beam pair prediction

|  |  |  | NTT DOCOMO GC1: Simulation Case 5 | NTT DOCOMO GC2: Simulation Case 6 |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 64*8 |  |
|  |  | Number of beam pairs in Set B | 8*2 |  |
|  |  | Baseline scheme | Option 1 and Option 2 |  |
| AI/ML model input/output |  | Model input | L1-RSRP values of the 16 beam pairs |  |
|  |  | Model output | L1-RSRP values of the 512 beam pairs |  |
| Data Size |  | Training | 320010 |  |
|  |  | Testing | 80040 |  |
| AI/ML model |  | Short model description | FNN |  |
|  |  | Model complexity [No. of para./ model size] | 325K/1.3MB |  |
|  |  | Computational complexity | 326K |  |
| Evaluation results [AI/ML /Baseline Option 1 /Baseline Option 2] | Beam prediction accuracy (%) | Top-1 (%) | 24.1 / 100 /7.60 | 20.3 / 100 / 4.32 |
|  |  | Top-1/5 (%) | 58.8 / 100 / 33.3 | 52.5 / 100 / 20.0 |
|  |  | Top-5/1 (%) | 61.1 / 100 / 7.60 | 53.2 / 100 / 4.32 |
|  | L1-RSRP Diff. | Average L1-RSRP diff.(dB) | 3.63 / 0 / 6.29 | 4.51 / 0 / 8.60 |
|  | System performance | RS overhead Reduction (%) | 0.96 / 0 / 0.97 |  |
|  |  | RS overhead | 21 / 512 / 16 |  |

As shown in Table.10, the AI/ML model trained with mixed configurations of Set B Tx beams could have acceptable generalization performance. In this case, when the gNB changes the Set B Tx beam configuration or the UE is handed over to a different gNB, as long as the AI/ML to be used is trained with the mixed configurations, it can provide similar performance over different cells.

Observation 4: For Tx-Rx beam prediction:

- The AI/ML model trained with mixed Set B Tx beam configurations could provide acceptable generalization performance.

    - Temporal beam prediction (BM-Case 2)

The simulation in this section follows the assumption described in Section 2.2.2. ‘average L1-RSRP difference of Top-1 predicted beam’ is adopted as the KPI for simplicity.

2.3.2.1 Generalization performance

In this sub-section, for the time domain prediction Pattern A and Pattern B, the performance of GC1, GC2 and GC3 are provided respectively.

Table 11. Evaluation results of GC1, GC2 and GC3 for Pattern A

| Testing/inference dataset Training dataset |  |  | 30km/h | 60km/h | 90km/h |
| --- | --- | --- | --- | --- | --- |
| L1-RSRP Diff. | Baseline(Option 2) |  | 3.1 | 8.0 | 8.0 |
|  | GC1 GC2 | 30km/h | 3.52 (Case1) | 5.71 | 6.04 |
|  |  | 60km/h | 11.45 | 2.93 (Case1) | 3.09 |
|  |  | 90km/h | 12.26 | 3.34 | 3.04 (Case1) |
|  | GC3 | 30:60:90km/h=1:1:1:1:1:1 | 3.79 | 3.05 | 3.07 |

As shown in Table.11 for Pattern A, the AI/ML model trained with low speed data could still perform better than baseline even when it is applied to make prediction with higher speed data under the same time parameters. For example, the AI/ML model trained with data generated with UE speed 30km/h achieves the average L1-RSRP difference 5.71dB when it is applied to data generated with UE speed 60km/h, which is still 8.0 dB smaller than baseline. However, it is not the applied vice versa; model inference of low speed UE via the model trained with high speed UE. The reason is the data generated with high UE speed may have some information loss, which is difficult for AI/ML model to predict the low UE speed. However, if the AI/ML is trained with mixed data from different UE speed, the performance is still acceptable.

Table 12. Evaluation results of GC1, GC2 and GC3 for Pattern B

| Testing/inference dataset Training dataset |  |  | 30km/h | 60km/h | 90km/h |
| --- | --- | --- | --- | --- | --- |
| L1-RSRP Diff. | Baseline(Option 2) |  | 1.1 | 3.8 | 7.0 |
|  | GC1 GC2 | 30km/h | 3.63 (Case1) | 4.95 | 6.96 |
|  |  | 60km/h | 4.05 | 3.77 (Case1) | 5.01 |
|  |  | 90km/h | 5.82 | 4.06 | 3.10 (Case1) |
|  | GC3 | 30:60:90km/h=1:1:1:1:1:1 | 3.62 | 3.79 | 4.11 |

As shown in Table.12 for Pattern B, the similar tendency could be observed. For example, the AI/ML model trained with data generated with UE speed 60km/h achieves the average L1-RSRP difference 6.96dB when it is applied to data generated with UE speed 90km/h, which is still 7.0dB smaller than baseline. Also, the AI/ML trained with the mixed data from different UE speed provides the acceptable performance.

Observation 5: For temporal beam prediction Pattern A and Pattern B:

- The AI/ML model trained with mixed data from different UE speed could provide acceptable generalization performance.

- The AI/ML model trained with data from low speed UE could still provide better performance than baseline method, when it is applied to the data from high speed UE with the same time parameters.

- Conclusion

In this contribution, we discussed evaluation on AI/ML for beam management. Based on the discussion we made the following observations and proposals.

Observation 1: For Tx beam prediction with Rx beams assumption Option 1:

- The beam prediction accuracy as well as average L1-RSRP are obviously better than baseline.

- The “best” Rx beam for the best Tx-Rx beam pair in Set C provides better performance than the “best” Rx beam for each Tx beam in Set C.

Observation 2: For Tx beam prediction with Rx beams assumption Option 1 and Option 2a:

- When the Rx beams in Set C is specific beams instead of all Rx beams, (Option 2a), the prediction performance deteriorates a bit from the exhaustive Rx sweeping (Option 1) under the same Rx beam determination method.

Observation 3: For Tx beam prediction with Rx beam assumptions Option 1 and Option 2a:

- The AI/ML trained with mixed Rx beam assumptions at UE side could provide acceptable generalization performance.

Observation 4: For Tx-Rx beam prediction:

- The AI/ML trained with mixed Set B Tx beam configurations could provide acceptable generalization performance.

Observation 5: For temporal beam prediction Pattern A and Pattern B:

- The AI/ML model trained with mixed data from different UE speed could provide acceptable generalization performance.

- The AI/ML model trained with data from low speed UE could still provide better performance than baseline method, when it is applied to the data from high speed UE with the same time parameters.

Proposal 1: Study the Rx beam(s) determination methods for the Set B measurement for Tx beam prediction.

Proposal 2: Consider both Pattern A and Pattern B for temporal beam  prediction.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Moderator (OPPO), R1-2205454, “Discussion summary#4 for other aspects on AI/ML for beam management”, May. 2022.

- “Chair's notes RAN1_111 v15”, Nov. 2022.

- NTT DOCOMO, INC., R1-2209898, “Discussion on evaluation on AI/ML for beam management”, Oct. 2022.