# 3GPP TSG-RAN-WG1 #112	R1-2301563

Meeting, Feb. 27th – Mar. 3rd, 2023

Agenda Item:	9.9.1

Source:	Google Inc.

Title:	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion and decision

# 1. Introduction

In this document, we provide our views on multi-cell PUSCH/PDSCH scheduling with a single DCI.

# 2. Discussion on DCI size alignment procedure

According to the discussion in the last meeting, at least DCI fields FDRA, HARQ process ID, and MCS are belong to Type-2 fields (i.e., separate field for each of the co-scheduled cells). Thus, DCI size of the DCI format 0_X/1_X will be much larger than legacy DCI formats (e.g., DCI formats 0_0/1_0, 0_1/1_1, and/or 0_2/1_2). In addition, based on the previous agreement made in RAN1 #110b meeting, gNB can configure UE to monitor multi-cell scheduling DCI format 0_X/1_X and legacy single cell scheduling DCI formats simultaneously on the same scheduling cell. These facts may impact the current DCI size alignment procedure.

RAN1 #110b Agreement Confirm below working assumption reached in RAN1#110 meeting with revision. Working Assumption - For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), if configured from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_0/1_0/0_1/1_1/0_2/1_2 can be monitored simultaneously. - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

According to the current DCI size alignment procedure in TS. 38.212, a UE may monitor up to 4 different DCI sizes, wherein up to 3 DCI sizes can associate with C-RNTI and up to 1 DCI size can associate with other RNTI. However, there is no agreement on whether to introduce a new RNTI or use a legacy RNTI for the DCI format 0_X/1_X.

Observation 1: It is agreed that DCI format 0_X/1_X will be a new DCI format. However, there is no agreement on whether to introduce a new RNTI for the new DCI format or use a legacy RNTI (e.g., C-RNTI) for the new DCI format.

If the DCI format 0_X/1_X associate with C-RNTI, and a UE monitors DCI format 0_X/1_X and DCI formats 0_0/1_0, 1_0/1_1, and 0_2/1_2, simultaneously, the total number of monitored DCI size that associates with C-RNTI may exceed 3.

Observation 2: Regardless the RNTI that the DCI format 0_X/1_X associate with, if a UE monitors DCI format 0_X/1_X and legacy DCI formats 0_0/1_0/0_1/1_1/0_2/1_2, simultaneously, the total number of monitored DCI sizes may violate the DCI size alignment 3+1 rule.

Proposal 1: Introduce a new RNTI for DCI format 0_X/1_X.

Proposal 2: For DCI size alignment procedure, down select from the following alternatives

- Alt-1: Use legacy Rel. 17 DCI size alignment procedure for DCI format 0_X/1_X. It is based on base station implementation to make sure the number of monitored DCI size is under the DCI size budget.

- Alt-2: If the number of monitored DCI size exceeds the DCI size budget, the UE aligns the size of DCI format 0_1/1_1 to the DCI format 0_X/1_X.

RAN1 #111 Agreement Confirm the RAN1#110bis-e working assumption with the following changes: Working Assumption For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells. - DCI size of the DCI format 0_X/1_X is counted on the reference cell. - BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells. - BD/CCE of the DCI format 0_X/1_X is counted on the reference cell. - Same reference cell is used for both DCI format 0_X and DCI format 1_X. - The reference cell is - the scheduling cell if the scheduling cell is included in the set of cells and search space of the DCI format 0_X/1_X is configured only on the scheduling cell; - one cell of the set of cells which search space of DCI format 0_X/1_X is configured on and associated with the search space of the scheduling cell with the same search space ID if search space of the DCI format 0_X/1_X is configured on the cell in addition to the scheduling cell. - It is up to gNB on which cell the SS of the DCI format 0_X/1_X is configured on. - To address Rel-17 BD/CCE limit for any given cell (operating the feature under Rel-17 BD/CCE limit) - For the reference cell, a total number of configured BD/CCEs for both DCI formats 0_X/1_X and legacy DCI formats (if configured) does not exceed the Rel-17 limits. - For other cells in the sets of cells, Rel-17 limits for PDCCH/DCI monitoring and BD/CCE counting rules for legacy DCI formats (not including DCI formats 0_X/1_X) apply

In RAN1 #111 meeting, the group has confirmed to count the DCI size of DCI format 0_X/1_X on a reference cell. If the scheduling cell is not in the set of cells for multi-cell scheduling, the gNB can configure one cell in the set of cells for multi-cell scheduling as the reference cell. In this case, the reference cell can be a SCell, which can be activated/deactivated by using MAC CE. However, it is not clear whether the DCI size and BD/CCE budget of the DCI format 0_X/1_X can be counted on a deactivated SCell or not.

Observation 3: If the reference is a SCell, it is not clear whether the DCI size and BD/CCE budget of the DCI format 0_X/1_X can be counted on a deactivated reference cell or not.

Proposal 3: Down select a UE behaviour from following alternatives when the reference cell is deactivated.

- Alt-1: The UE does not expect the reference cell to be deactivated (i.e., based on gNB implementation).

- Alt-2: The UE counts the DCI size and BD/CCE budget of DCI format 0_X/1_X on the reference cell, regardless the reference cell is activated or deactivated.

- Alt-3: The UE counts the DCI size and BD/CCE budget of DCI format 0_X/1_X on another cell (e.g., scheduling cell or another cell in the set of cell for multi-cell scheduling) when the reference cell is deactivated by the gNB.

According to the RAN1 meeting #111 agreement, if the scheduling cell is included in the set of cells for multi-cell scheduling, the reference cell is the scheduling cell. With the SCell activation/deactivation operation, it is possible that the scheduling cell is the only active cell in the set of cells for multi-cell scheduling. In this case, should the UE continue monitoring DCI format 0_X/1_X, if the UE simultaneously monitors DCI format 0_X/1_X and non-fallback DCI formats (e.g., DCI format 0_1/1_1 or 0_2/1_2).

RAN1 #109 Agreement - (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling - DCI format 0_X can be used for single cell PUSCH scheduling. - DCI format 1_X can be used for single cell PDSCH scheduling. - FFS: UE monitors one of or both multi-cell scheduling DCI and legacy single cell scheduling DCI for a scheduled

According to another agreement from RAN1 meeting #109, it has been agreed that DCI format 0_X/1_X can be used for single cell scheduling. However, it does not mean that the UE should simultaneously monitor DCI format 0_X/1_X and non-fallback DCI formats or when the scheduling cell is the only active cell in the set of cells for multi-cell scheduling. Using non-fallback DCI formats provides more reliability due to its smaller DCI sizes. On top of that, non-fallback DCI formats also provide completed scheduling features, such as finer FDRA granularity, CBG transmission, etc. In this regard, we think the UE should suspend monitoring DCI format 0_X/1_X when the scheduling cell is the only active cell in the set of cells for multi-cell scheduling.

Observation 4: It is not clear that if a UE simultaneously monitors DCI format 0_X/1_X and DCI format 0_1/1_1, should the UE continue monitoring DCI format 0_X/1_X when the scheduling cell is the only active cell in the set of cells for multi-cell scheduling?

Proposal 4: If a UE simultaneously monitors DCI format 0_X/1_X and legacy DCI format 0_1/1_1 on a scheduling cell, the UE suspends monitoring DCI format 0_X/1_X when the scheduling cell is the only one active cell in the set of cells for multi-cell scheduling.

# 3. Conclusion

Observation 1: It is agreed that DCI format 0_X/1_X will be a new DCI format. However, there is no agreement on whether to introduce a new RNTI for the new DCI format or use a legacy RNTI (e.g., C-RNTI) for the new DCI format.

Observation 2: Regardless the RNTI that the DCI format 0_X/1_X associate with, if a UE monitors DCI format 0_X/1_X and legacy DCI formats 0_0/1_0/0_1/1_1/0_2/1_2, simultaneously, the total number of monitored DCI sizes may violate the DCI size alignment 3+1 rule.

Proposal 1: Introduce a new RNTI for DCI format 0_X/1_X.

Proposal 2: For DCI size alignment procedure, down select from the following alternatives

- Alt-1: Use legacy Rel. 17 DCI size alignment procedure for DCI format 0_X/1_X. It is based on base station implementation to make sure the number of monitored DCI size is under the DCI size budget.

- Alt-2: If the number of monitored DCI size exceeds the DCI size budget, the UE aligns the size of DCI format 0_1/1_1 to the DCI format 0_X/1_X.

Observation 3: If the reference is a SCell, it is not clear whether the DCI size and BD/CCE budget of the DCI format 0_X/1_X can be counted on a deactivated reference cell or not.

Proposal 3: Down select a UE behaviour from following alternatives when the reference cell is deactivated.

- Alt-1: The UE does not expect the reference cell to be deactivated (i.e., based on gNB implementation).

- Alt-2: The UE counts the DCI size and BD/CCE budget of DCI format 0_X/1_X on the reference cell, regardless the reference cell is activated or deactivated.

- Alt-3: The UE counts the DCI size and BD/CCE budget of DCI format 0_X/1_X on another cell (e.g., scheduling cell or another cell in the set of cell for multi-cell scheduling) when the reference cell is deactivated by the gNB.

Observation 4: It is not clear that if a UE simultaneously monitors DCI format 0_X/1_X and DCI format 0_1/1_1, should the UE continue monitoring DCI format 0_X/1_X when the scheduling cell is the only active cell in the set of cells for multi-cell scheduling?

Proposal 4: If a UE simultaneously monitors DCI format 0_X/1_X and legacy DCI format 0_1/1_1 on a scheduling cell, the UE suspends monitoring DCI format 0_X/1_X when the scheduling cell is the only one active cell in the set of cells for multi-cell scheduling.

# 4. Reference

[1]	R1-2212924, “Feature lead summary #5 on multi-cell PUSCH/PDSCH scheduling with a single DCI,” Lenovo, Nov. 2022.