3GPP TSG RAN WG1 #112	R1-2301567

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		9.11.4

Source:				Nokia, Nokia Shanghai Bell

Title:	Enhancements for long connections in NB-IoT/eMTC over NTN

Document for:		Discussion and Decision

# Introduction

The release 18 work item [1] on IoT support of non-terrestrial networks includes the following objective

Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

In release 17, the work was limited to short and sporadic connection and therefore it was feasible that a UE upon GNSS validity timer expiry could move to RRC Idle mode.

However, the release 18 work will address the broader use case of long connection time. Therefore, it is important to consider how the UE can obtain a new GNSS-based position fix during the long connection. It is also good to note that the simultaneous use of GNSS and IoT operation is not assumed as was also the case in release 17.

In this contribution we provide our view on the improved GNSS operations for IoT NTN.

# Discussion

At RAN1#110bis the following was agreed:

Agreement Support eNB to at least aperiodically trigger UE to make GNSS measurement.

At RAN1 #111 the agreement was further refined:

Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration

The agreement is good because it enables the UE and eNB to have a common understanding of when the UE is unavailable for communication due to the UE performing a GNSS measurements. This is of key importance to ensure the UE is not scheduled, when it is actually unavailable due to the GNSS measurement, and the best approach to facilitate this is to let eNB control the GNSS measurements and the UE to follow the eNB’s command.

The agreement defines the eNB can aperiodically trigger the UE to make the GNSS measurement, but it is not clear when the measurement is exactly required. However, it is important that the UE shall not become uplink unsynchronized during the long connection. This essentially means the UE has to complete the GNSS measurement before the current remaining GNSS validity duration expires.

Proposal 1: RAN1 to define the GNSS measurement shall be completed before the remaining GNSS validity duration expiry.

In release 17, the UE would move to RRC Idle if a GNSS measurement was needed. Therefore, the impact of GNSS measurements on RRC Connected mode behavior was not discussed. However, in release 18 it is important to define the UE behavior, when the GNSS measurement gap collides with other UE procedures. For example, the procedure of using uplink or downlink repetitions and the Radio Link Monitoring procedure including Radio Link Failure. This is particularly important for GNSS measurement gaps, because they could potentially be in the order of seconds. As one example of legacy specification, for PUSCH the transmission gaps, the TS 36.211 defines the UE shall count the time units for the resource mapping but not use them for transmission. This may be acceptable for the 256 ms transmission gap, but could be problematic for a GNSS measurement gap of several seconds.

BL/CE UL subframes within the gap of time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH.

Proposal 2: RAN1 to discuss the impact of GNSS measurement gaps on RAN1-related UE behavior including use of repetitions in uplink/downlink.

The FFS regarding the details of the configuration needs further discussion. Similar to other measurement gap configurations it shall include at least the start time of the gap and the gap duration. The gap duration may depend on the UE’s reported GNSS position fix time duration, while the start time can depend on e.g. the time the triggering is received and the remaining GNSS validity duration. In our view, the RAN2 WG can discuss the details.

Proposal 3: RAN1 can leave it to RAN2 to define the details of the GNSS measurement gap configuration, which shall include the gap duration and indicate the start time of the gap.

The RAN1 agreements above define the eNB performing aperiodic triggering of UE GNSS measurements. This is useful if the eNB is able to either detect the need for a new measurement based on UE uplink transmission time/frequency drift or based on the GNSS assistance information in terms of GNSS position fix time duration and GNSS validity duration. However, there is a certain signaling overhead related to triggering the GNSS measurements and in some scenarios the need for future GNSS measurements can be predicted. This would be the case when the UE moves in a predictable way, for example when it is onboard a ship or a train. In such a case periodic GNSS measurements may be useful, because it can be predicted when the measurement is needed and thus the signaling overhead can be reduced by only required a one-time (or long-term) GNSS measurement configuration.

Observation 1: The UE may move in a predictable way, e.g. on a ship or train, and therefore make periodic GNSS measurements useful.

Proposal 4: Support eNB to configure the UE to make periodic GNSS measurements.

The periodic GNSS measurements can be configured via RRC, but it may also be an option to provide multiple different GNSS measurement configurations (e.g. different periodicity and gap) in System Information Broadcast. Each UE can then select the configuration, which it finds most suitable and inform the eNB about its selection. This approach will also address the issue of the CP-only solution not allowing RRC Reconfiguration.

Proposal 5: Support eNB to broadcast multiple GNSS measurement configurations and UE to report which configuration it applies.

The RAN1s agreements above rely on the eNB to trigger the measurement. If the GNSS assistance information has become invalid, e.g. due to the UE starting to move in a different/unexpected way, the eNB is not able to trigger the GNSS measurement timely. If the UE is transmitting the eNB may be able to detect uplink transmission time/frequency drift, but if the UE is not currently transmission the eNB will not be aware of the UE movement and the need for a new, aperiodic GNSS measurement. Therefore, it seems beneficial to also allow the UE to request the eNB to trigger a GNSS measurement. According to the work item assumptions the UE is not able to operate the GNSS and 3GPP modem simultaneously. Therefore, the UE cannot rely on the GNSS to update the location information during an ongoing 3GPP communication instance. However, it may be feasible for the UE to compare the expected time/frequency of a downlink signal, based on the previous determined location, and the actual time/frequency of the received downlink signal. If the difference between the expected and measured downlink signals exceeds a threshold the UE could trigger a request for a new GNSS measurement.

Observation 2: If the UE does not transmit, the eNB is not aware of potential UE movement and therefore only the UE is able to determine the need for a new GNSS position fix and trigger aperiodic GNSS measurement.

Observation 3: The UE may detect the mobility state change based on determining the time/frequency difference between a received downlink signal and an estimate based on the last known location.

Proposal 6: Support the UE to request eNB to trigger at least an aperiodic GNSS measurement.

At RAN1 #111 the following was agreed:

Agreement The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

In our view, this allows the UE to perform the GNSS measurement autonomously, if the eNB has not triggered it. The issue is the timing for the autonomous measurement. First, the UE autonomous GNSS measurement shall rely on the GNSS measurement gap configuration in terms of the gap duration, such that the eNB knows how long the UE is not available. Second, the eNB and UE must have a common understanding of when the UE starts the autonomous measurement, for example if the measurement is within the C-DRX Active where the eNB expects the UE to monitor the PDCCH, or if it collides with transmission/reception of control or data channels or reference signals. It seems logical to define the GNSS measurement to start before the remaining GNSS validity duration expires, because the UE will not be uplink synchronized after this expiry. Therefore, RAN1 shall discuss the exact timing definition of when the UE autonomously can start the measurement relative to the expiry of the GNSS validity duration, and RAN1 shall also discuss whether there are any conditions, which would allow the UE to shift such measurement in the time domain.

Observation 4: UE and eNB must have a common understanding of when the UE is performing the autonomous GNSS measurement.

Proposal 7: RAN1 to define the UE is allowed to initiate the GNSS measurement (according to the network’s measurement gap configuration) autonomously x seconds before the expiry of the GNSS validity duration.

Additionally, for the IoT UE, the repetition period may last for a long time, e.g. several seconds or even tens of seconds. When considering the largest number of repetitions, number of resources units (RU), and number of slot in RU, the maximum transmission time could be 10.24 s for 15kHz SCS and 40.96 s for 3.75kHz SCS. During such a long repetition period the GNSS validity duration timer may expire. Since the UE and eNB are both aware of the GNSS validity duration timer and the duration of the repetitions it could be considered to allow the UE to perform a GNSS measurement during the repetitions by creating a GNSS measurement gap at a known point in time within the repetition period.

Observation 5: There may be a need for a GNSS measurement during a long period of downlink/uplink repetitions.

Proposal 8: RAN1 to discuss handling a GNSS measurement during a long period of downlink/uplink repetitions.

Alternatively, it can be discussed whether the eNB instead of triggering the UE to perform a new GNSS measurement can command the UE to extend the GNSS validity duration timer. This would be useful in the scenario of a stationary UE, where the eNB may notice the UE’s transmissions remain well aligned in time and frequency during the course of a repetition period. Therefore, the eNB could extend the UE’s GNSS validity timer and save the time and UE energy, which would be spend on a new GNSS measurement, and furthermore avoid the interruption of the repetitions.

Proposal 9: RAN1 to discuss the possibility of eNB extending a UE’s GNSS validity duration timer.

At RAN1#110bis the following was agreed:

Agreement UE reports GNSS position fix time duration for measurement at least during the initial access stage - which message carries this information is up to RAN2

The agreement enables the eNB to be aware of the GNSS position fix time duration such that the eNB knows the minimum length of the GNSS measurement gap. The agreement states the UE can report this information at least during the initial access stage. The information is required at this point in time such that the eNB can prepare the scheduling of the GNSS measurement gap. However, if the UE starts moving in a different way or the (GNSS) propagation conditions change the information reported during the initial accesss stage may be outdated, because the UE would now have a different GNSS position fix time duration.

Observation 6: The UE may move and/or the propagation conditions may change and therefore cause a change in the required GNSS position fix time duration.

To support the scenarios where the UE’s GNSS position fix time duration changes during a long connection it will be beneficial to discuss support for UE reporting updated GNSS position fix time duration if the duration has changed compared to the duration reported in the initial access stage.

Proposal 10: RAN1 to discuss UE reporting GNSS position fix time duration if the duration has changed since the initial access stage report.

At RAN1#110bis the following was agreed:

Agreement In connected mode, UE may report GNSS validation duration with MAC CE.

As noted above the UE may start moving in a different way and therefore the above agreement is useful to ensure the UE informs the eNB about any changes in the GNSS validity duration. Note the agreement refers to validation instead of validity.

To ensure timely reporting of the GNSS validaty duration it is necessary to define triggers for the UE. A simple approach is to define the UE reports the GNSS validity duration after every GNSS measurement or alternatively only if the current GNSS validity duration is different from the previously reported value. This works well in predictable scenarios. However, the UE’s mobility state/location may also changed unexpectedly and therefore it will be beneficial to discuss how the UE can detect such unexpected movement and how the UE can trigger a new GNSS measurement, for example by reporting a new GNSS validity duration to the eNB (since such validity duration will also change based on the movement).

Observation 7: The UE’s mobility state/location may change and therefore cause a change in the expected GNSS validity duration.

In release 17 it was discussed (by RAN2) that the UE is expected to have a valid GNSS fix before it establishes a RRC Connection. However, it may not be strictly necessary for the UE to re-acquire the GNSS fix before setting up the connection. This is for example the case when the UE has been very briefly in RRC Idle due to an RLF or is performing a handover / cell reselection. In such scenarios it would be important to define whether the UE can reuse the previous GNSS fix and whether the UE shall report the GNSS validity duration. Thus UE triggers to control this behavior would be good to discuss in RAN1.

Observation 8: GNSS reacquisition may not always be needed if the UE has been RRC Idle for a short period during a long connection.

Furthermore, in the RAN1 #111 it was discussed that the UE can report the new remaining GNSS validity duration to indicate a successful GNSS measurement. This is useful, but not strictly needed if the new duration is the same as the previous duration. In that case, it would be sufficient to indicate “successful GNSS measurement” without providing the GNSS validity duration.

Proposal 11: RAN1 to discuss how UE can trigger the reporting of GNSS validity duration, including after a successful GNSS measurement, if the duration has changed, and after a short period in RRC Idle.

Besides the reporting of the GNSS validity duration, it may be important for the UE to report the GNSS measurement was successful such that UE and network remain synchronized in terms of the understanding of the GNSS status including the remaining GNSS validity duration. In our view, a successful measurement means a new GNSS position fix was obtained. If the GNSS measurement has failed, the UE may have zero or very limited remaining GNSS validity duration left and therefore further communication with the network is correspondingly limited. Depending on when the GNSS measurement gap is scheduled relative to the GNSS validity duration expiry, the UE may not even be allowed to transmit an indication of a failed GNSS measurement, because its uplink is unsynchronized. Based on these challenges, we propose that RAN1 defines the UE is required to report the success, and potentially failure, of a GNSS measurement.

Proposal 12: RAN1 to define the UE is required to report the success of a GNSS measurement.

At RAN1#110bis and #110 it was discussed whether the UE can reacquire the GNSS position fix outside the Connected DRX Active Time.

Relying on the inactive state of Connected DRX for re-acquiring GNSS position fix is a challenging solution. First of all, there is no guarantee that the “next” inactive state of C-DRX will happen, because the Active Time may be extended if there is new data incoming or retransmissions occur. Even if the inactive state were to happen it may be shorter than expected due to the extension of the Active Time.

Observation 9: Inactive state of C-DRX may not occur at all or be shorter than expected due to extension of the active state because of new data and retransmissions.

The GNSS position fix time duration is expected to be in the order of 1-2 seconds. Therefore, the inactive state would need to be at least this long to accommodate the GNSS reacquisition. This essentially means the C-DRX cycle should be 2.56 s or longer, while at least eMTC devices could have a significantly shorter cycle to reduce data latency. Considering the short cell availability time (about 6-7 seconds for a 50 km EMC) it may also be reasonable to assume that C-DRX is not even configured to ensure the UE has sufficient time to communicate with the cell, while it is available.

In addition, the UE may also be configured with measurement gaps, e.g. for inter-frequency and inter-RAT measurements. Such gaps are configured indenpendently of the C-DRX and therefore the gap and the corresponding RRM measurement may take place during the inactive state of C-DRX. This would mean the UE cannot perform the GNSS measurement in order to perform the configured RRM measurement.

Observation 10: RRM measurement gaps may occur during the inactive state of Connected DRX and therefore prohibit the GNSS measurement.

Finally, the NTN UE may also have to acquire serving cell satellite assistance information (SIB31) during the inactive state of C-DRX. This will be necessary if the validity timer T317 for SIB31 is about to expire/has expired. Therefore, the UE will not be able to perform the GNSS measurement during the part of the inactive state, which collides with the reception of the SIB31.

TS 36.331: In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT).

Observation 11: UE may need to reacquire SIB31 during the inactive state of Connected DRX and is therefore not able to perform the GNSS measurement.

Based on the above discussion it is clear that it is challenging for the UE to autonomously reacquire GNSS during the inactive state of Connected DRX. As discussed there are multiple different events which could prevent or collide with such a measurement. Our view is that RAN1 shall first specify the basic GNSS measurement functionality during a long connection, i.e. the configuration and use of measurement gaps, and then RAN1 may discuss whether there is a need for GNSS measurements outside the Connected DRX Active Time as a potential next step

Proposal 13: RAN1 to postpone the discussion on whether the UE can reacquire the GNSS position fix outside the Connected DRX Active Time.

# Conclusion

In this contribution we have made the following observations:

Observation 1: The UE may move in a predictable way, e.g. on a ship or train, and therefore make periodic GNSS measurements useful.

Observation 2: If the UE does not transmit, the eNB is not aware of potential UE movement and therefore only the UE is able to determine the need for a new GNSS position fix and trigger aperiodic GNSS measurement.

Observation 3: The UE may detect the mobility state change based on determining the time/frequency difference between a received downlink signal and an estimate based on the last known location.

Observation 4: UE and eNB must have a common understanding of when the UE is performing the autonomous GNSS measurement.

Observation 5: There may be a need for a GNSS measurement during a long period of downlink/uplink repetitions.

Observation 6: The UE may move and/or the propagation conditions may change and therefore cause a change in the required GNSS position fix time duration.

Observation 7: The UE’s mobility state/location may change and therefore cause a change in the expected GNSS validity duration.

Observation 8: GNSS reacquisition may not always be needed if the UE has been RRC Idle for a short period during a long connection.

Observation 9: Inactive state of C-DRX may not occur at all or be shorter than expected due to extension of the active state because of new data and retransmissions.

Observation 10: RRM measurement gaps may occur during the inactive state of Connected DRX and therefore prohibit the GNSS measurement.

Observation 11: UE may need to reacquire SIB31 during the inactive state of Connected DRX and is therefore not able to perform the GNSS measurement.

And the following proposals:

Proposal 1: RAN1 to define the GNSS measurement shall be completed before the remaining GNSS validity duration expiry.

Proposal 2: RAN1 to discuss the impact of GNSS measurement gaps on RAN1-related UE behavior including use of repetitions in uplink/downlink.

Proposal 3: RAN1 can leave it to RAN2 to define the details of the GNSS measurement gap configuration, which shall include the gap duration and indicate the start time of the gap.

Proposal 4: Support eNB to configure the UE to make periodic GNSS measurements.

Proposal 5: Support eNB to broadcast multiple GNSS measurement configurations and UE to report which configuration it applies.

Proposal 6: Support the UE to request eNB to trigger at least an aperiodic GNSS measurement.

Proposal 7: RAN1 to define the UE is allowed to initiate the GNSS measurement (according to the network’s measurement gap configuration) autonomously x seconds before the expiry of the GNSS validity duration.

Proposal 8: RAN1 to discuss handling a GNSS measurement during a long period of downlink/uplink repetitions.

Proposal 9: RAN1 to discuss the possibility of eNB extending a UE’s GNSS validity duration timer.

Proposal 10: RAN1 to discuss UE reporting GNSS position fix time duration if the duration has changed since the initial access stage report.

Proposal 11: RAN1 to discuss how UE can trigger the reporting of GNSS validity duration, including after a successful GNSS measurement, if the duration has changed, and after a short period in RRC Idle.

Proposal 12: RAN1 to define the UE is required to report the success of a GNSS measurement.

Proposal 13: RAN1 to postpone the discussion on whether the UE can reacquire the GNSS position fix outside the Connected DRX Active Time.

# References

- RP-223519, Revised WID on IoT NTN enhancements, 3GPP TSG RAN Meeting #98, December, 2022.