3GPP TSG RAN WG1 #112	R1-2301577

Athens, Greece, February 27th – March 3rd, 2023

Source: 	MediaTek Inc.

Title: 	Evaluation on low power WUS

Agenda item:	9.13.1

Document for:	Discussion

# Introduction

In RAN1#111, evaluation assumptions and methodologies have been agreed upon for the main radio (MR), and low-power wake-up radio (LR). However, the details of introducing ultra-deep sleep for MR need further study.

In this contribution, we provide our recommendation on power consumption models for MR and LR and provide initial evaluations based on the agreed baseline parameters. The endorsed agreements are provided in the appendix.

# Evaluations

## Power Model for Main Radio

### eMBB

In RAN1#111, companies are encouraged to provide recommendations for MR at least for FR1 evaluation.

Agreement in RAN1#111 For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption - Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112

eMBB UE typically contains multiple sub-systems such as multi-band 5G NR, LTE-FDD, LTE-TDD, HSPA+, and multiple GNSS modules. Turning off one of the sub-systems may have certain interaction with the rest of the sub-systems.

For ultra-deep sleep, the main difference from deep sleep is that the wake-up module is not needed if a dedicated LP-WUR exists. In this case, it seems reasonable to turn off more hardware components than deep sleep, e.g., the controller and the low-complexity oscillator, with a cost of a high ramp-up latency.

Table 1: Different usage of hardware among deep sleep, ultra-deep sleep, and power off

| Modem related hardware | Latency | Ultra-deep sleep | Deep sleep | Comments |
| --- | --- | --- | --- | --- |
| Oscillator (XO) | Medium | Off | Low power | For deep sleep, UE roughly maintains the system frame number (SFN) and the slot number for waking up on time. Resume time takes a few milliseconds. |
| RF/front-end | Low | Off | Off |  |
| Baseband modem | Medium | Off | Off |  |
| Controller | High | Off | Low power | For deep sleep, UE needs a controller to keep the time base. Resume time takes hundreds of milliseconds. |
| DDR memory | Very high | Off | Low power | DDR memory resume time takes a few seconds if it is turned off. |

If UE only turns off the low-complexity oscillator and the controller from a deep sleep, the ramp-up time of 400ms should be sufficient for XO and the controller to reboot, initiate, and reload programs and data.

For power off (ultra-deep sleep), considering both cold boot or warm boot (refers to whether RF channel and PLMN ID are still valid), the ramp-up time needs several seconds for a system reboot, initiations, and DDR memory reload.

Based on this example, we have the following recommendation for the ultra-deep sleep power state.

If ultra-deep sleep aims for a power consumption of less than LPWUR (e.g., < 1mW), it requires at least 2s to ramp up based on the current implementation for NR eMBB.

For eMBB, consider the following power models for an “ultra-deep sleep” power state for the main ratio for R18 LPWUS evaluation.

| Use case | Power State | Relative Power (unit) | Ramp-up and down transition energy (unit multiplied by ms) | Ramp-up time | Ramp-down time | Time for sync/re-sync |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB | Ultra-deep sleep | 0.015 (<<1mW) | At least 10000 | At least 2 seconds | At least 1 seconds | At least 82 ms based on R1-2212261 |

### IoT and wearables

IoT and wearables have diverse data and range requirements and result in quite different power consumption and hardware properties. It would be difficult to consider one or two modules and provide our recommendation. To tackle this issue, it is reasonable to leverage LPHAP’s agreement in RAN1#110-b.

Agreement in RAN1#110-b For the LPHAP study only: - For the power consumption model of the ultra-deep sleep type, adopt the following option (i.e. revision of option 1 from previous agreement): - The relative power unit: 0.015 - Additional transition energy: 10000 - Note: Power consumption analysis from individual companies with additional transition energy of 5000 can be optionally evaluated and captured in the TR. - Total transition time: 400ms - Note: Power consumption analysis from individual companies with Option 2 (revised from previous agreement) can be optionally evaluated and captured in the TR. - Option 2 additional transition energy is revised from 450 to 480. - Note: No new device type is expected based on ultra-deep sleep power modeling.

In TS 22.104, SA1 has introduced requirements for LPHAP (Low Power High Accuracy Positioning) for industrial IoT scenarios including use cases such as massive asset tracking, AGV (Automatic Guided Vehicle) tracking in industrial factories, and person localization in danger zones.

On the other hand, in the revised SID RP-222644, the LPWUS study will primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, and controllers) and wearables. Other use cases are not precluded, e.g., smart glasses, and smartphones.

We can see a synergy between LPHAP and LPWUS at least for industrial IoT scenarios. However, wearable devices may involve a more complex process than industrial IoT, for a system reboot, initiations, and DDR memory reload.

Based on this observation, we have the following recommendation for the ultra-deep sleep power state.

LPWUS and LPHAP (Low Power High Accuracy Positioning) both target industrial IoT scenarios and have similar considerations on hardware requirements.

For IoT and wearables, consider the following power models for an “ultra-deep sleep” power state for the main ratio for R18 LPWUS evaluation.

| Use case | Power State | Relative Power (unit) | Ramp-up and down transition energy (unit multiplied by ms) | Ramp-up time | Ramp-down time | Time for sync/re-sync |
| --- | --- | --- | --- | --- | --- | --- |
| IoT and wearables | Ultra-deep sleep | 0.015 | 10000 | Total transition time: 400ms; Option: 1s |  | At least 82 ms based on R1-2212261 |

## Power Model for LPWUR

### Non-OFDM-based LPWUR

In RAN1#111, companies have determined the relative power values of LPWUR power-on from 0.01 to 4 and determined the relative power of power-off as 0.001. The ramp-up time of LPWUR is FFS.

Agreement in RAN1#111 The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed. FFS: whether further categorization/sub-categorization is needed and how.

To support a non-OFDM-based LPWUR with a power consumption of less than 1 mW, the relative power of 1, 2, and 4 cannot be considered. This is because the deep sleep mode (relative power of 1) cannot be lower than 1 mW based on the current implementation.

Also, we can ignore the ramp-up time for the non-OFDM-based LPWUR. A typical non-OFDM-based LPWUR has no PLL, AGC, advanced CPU, or memory architectures. No process requires significant ramp-up time.

For a non-OFDM-based LPWUR, RAN1 considers removing the relative power values of 1, 2, and 4 because the non-OFDM-based LPWUR is typically less than 1 mW.

For a non-OFDM-based LPWUR, RAN1 considers a zero ramp-up time due to a simple receiver architecture without PLL, AGC, advanced CPU, or heavy memory usage.

Consider the following power model for non-OFDM-based LPWUR for FR1 evaluation.

| Power State of LPWUR | Relative Power (unit) | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| LPWUR Off | 0.001 | 0 | 0 |
| LPWUR On | 0.01/0.05/0.1/0.5 | N/A | N/A |

### OFDM-based LPWUR

In RAN1#111, companies have agreed to study OFDMA-based signals considering the existing signal, e.g., SSS. The details of the agreement are given below.

Agreement in RAN1#111 - Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Note that above does not preclude DFT-S-OFDMA - Study generation and link performance of multi-carrier (MC)-FSK waveforms - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g., CSI-RS, SSS) - Other signal/channel structures are not precluded - For next meeting, companies to provide input on aspects to consider that might impact link performance

Note that MC-OOK and FSK can be modulated by OFDMA techniques, being adjacent subcarrier interference-free, and supporting non-coherent detection using a low-power and low-cost WUR. However, considering the existing signal is another story. For example, the NR SSS search procedure requires coherent detection, including timing alignment, frequency correction, OFDM demodulation, etc.

Presumably, an OFDM-based LPWUR can reuse the existing PSS to achieve coarse timing and frequency correction, which makes coherent detection possible for an SSS-like LPWUS. Based on our LPWUR architecture and analysis given in R1-2301578, relative power during various states can be modelled as the following proposal.

OFDM-based LPWUR can reuse LO, ADC, and DFT modules from the MR to perform coherent detection on an SSS-like LP-WUS.

The power states for an OFDM-based LPWUR can be modeled as follows:

| Power State of LPWUR | Relative power | Transition energy | Ramp-up time (ms) | Notes |
| --- | --- | --- | --- | --- |
| OFDM-WUR ON | 40 | N/A | N/A | Share LO and ADC |
| OFDM-WUR OFF | 0.2 | 0 (ideally) | 0 (ideally) | Maintain the 26 MHz-CLK |

# Evaluation Assumptions and Results

## Simulation Assumptions

Table 2 gives the power state model based on TR 38.875 for IoT and wearables use cases.

Table 2: Power state modelling for an MR based on TR 38.875 and UDS

| Power state of MR | Relative power | Transition time | Transition energy |
| --- | --- | --- | --- |
| Ultra-Deep Sleep (UDS) | 0.015 | 400ms | 10000 |
| Deep sleep | 0.8 | 20ms | 450 |
| Light sleep | 18 | 6ms | 100 |
| Micro sleep | 31 | 0ms | 0 |
| SSB/CSI-RS proc. | 50 | N/A | N/A |
| PDCCH monitoring | 50 | N/A | N/A |
| Intra-inter frequency RRM | 60 | N/A | N/A |

### Non-OFDM-based LPWUR

#### IDLE/INACTIVE

Figure 1 shows a processing timeline for MR ramp-up (re)-synchronization. Presumably, non-OFDM-based LPWUR has independent CLK (real time clock) and LO (local oscillator). Once MR turns off its CLK and LO to enter ultra-deep sleep, it loses timing and frequency synchronization immediately. To re-synchronize, the initial timing search takes 20ms in the worst case depending on the UE implementation, and frequency lock from 10ppm to 2ppm needs 3 SSB.

Figure 1: I-DRX timeline for ultra-deep sleep UE processing before PO for low SNR

Figure 2 shows a processing timeline to evaluate the non-OFDM-based LPWUR. We consider the low SINR scenario and assume re-synchronization is needed. The LPWUS occasion determines whether UE should process the paged timeline (MR wakes up for paging) or the not paged timeline (MR wakes up for RRM requirement). The probability of receiving LPWUS in the occasion is given by the group UE paging rate. Details are given in Table 7 and Table 8.

Figure 2: Reference timeline to evaluate the non-OFDM-based LPWUR

Figure 3 shows the eDRX timeline, given 4 DRX cycles within a PTW and each eDRX cycle has 61.44 seconds. Assume UE enters deep sleep within the PTW and ultra-deep sleep outside the PTW. Details are given in Table 9.

Figure 3: e-DRX timeline for ultra-deep sleep UE processing

#### CONNECTED

In the RRC-connected mode, UE can monitor DCP to know whether to start the on-duration timer of the upcoming DRX cycle. On the other hand, DCP cannot be used to adjust PDCCH monitoring behavior during the on-duration time, or there is no DRX configured. In the use case of XR with large timing jitter (e.g., [-4 ms, 4 ms]), a large on-duration timer may be used, or there will be no DRX configured, and the DCP solution cannot reduce UE power consumption for such use cases. An alternative is to reuse the Rel-17 PS adaptation (SSSG switching). A gNB can configure one UE-specific SSSG for LR to monitor LP-WUS and another SSSG for MR to monitor DCI.

Suppose UE monitors LP-WUS in SSSG#0 via LP-WUR and monitors PDCCH in SSSG#1 via MR. In SSSG#0, MR enters microsleep, and LR stays ON. In SSSG#1, MR performs PDCCH monitoring (and PDSCH processing), and LR stays OFF. UE switches from SSSG#0 to SSSG#1 if LP-WUR detects a wake-up indication. If there is no new data after the SSSG timer (4 ms) in SSSG#1, UE switches to SSSG#0. Settings and scenarios are in Appendix 6.2.

Figure 4: LP-WUS assistant SSSG switching for XR traffic

### OFDM-based LPWUR

Figure 5 shows a processing timeline to evaluate the OFDM-based LPWUR. We consider the low SINR scenario and assume no timing search (or SSB search) is needed because LPWUR shares the LO and CLK from the MR. Also, we assume that LPWUS and PSS can be allocated in the same time slot. The evaluation timeline is illustrated below.

Figure 5: Reference timeline to evaluate the OFDM-based LPWUR

## Simulation Results

### Non-OFDM-based LPWUR

#### IDLE/INACTIVE

Table 3 provides evaluations and assumptions for Case A7: provide power and latency for different RRM assumptions for IoT and wearables. The main power consumption can be found in Ramp-up/down energy and Sync/re-sync.

Table 3: IDLE Power Consumption Analysis with I-DRX for IoT/wearables

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Continuously monitoring |
|  | ON state power[unit] = 0.05 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 10000 |
|  | Sync/re-sync = 110 ms Energy consumption = 4225.25 |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 11529.95 |
|  | Power saving gain = -12% |
|  | baseline schemes = 10249.32 |

Introducing of LPWUS has no power saving benefit if IDLE UE has RRM duty per I-DRX cycle of 1.28s. The power saving gain is -12% compared to R17 PEI with 0.05 as LPWUR ON power.

RAN1 to postpone the study on LPWUS used with I-DRX until the current NR RRM requirement can be relaxed.

Table 5 provides an evaluation for Case A7 given an eDRX cycle of 61.44s.

Table 4: IDLE Power Consumption Analysis with eDRX for IoT/wearables

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| eDRX configuration | e-DRX cycle = 61.44s |
|  | i-DRX cycle in PTW [s] = 5.12 s |
| LP-WUR (LP) | Continuously monitoring |
|  | ON state power[unit] = 0.05 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 10000 |
|  | Sync/re-sync = 110 ms Energy consumption = 4225.25 |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 28307.99 |
|  | Power saving gain = 51% |
|  | baseline schemes = 57544.84 |

LPWUS can achieve a 51% power-saving gain with an eDRX cycle of 61.44s.

RAN1 to consider focusing on the study of LPWUS used with eDRX in RRC IDLE.

#### CONNECTED

Table 5 provides an evaluation result in RRC CONNECTED. About 16%-19% can be observed using LP-WUS-assisted SSSG switching compared to Rel-17 SSSG switching. The PS gain highly depends on the time UE stays in SSSG#0, where MR enters microsleep. On the other hand, power consumption for LP-WUS monitoring only contributes a few. For example, there is no significant difference if LP-ON takes a power value of 0.005 or 0.05.

Table 5: LP-WUS assistant SSSG switching for XR traffic

| # | Power saving scheme | PS Gain (w.r.t Rel-17) |  |
| --- | --- | --- | --- |
|  |  | LP-WUS 0.005 | LP-WUS 0.05 |
| 1 | SSSG switching (XR 30 Mbps) | 19.02% | 19.01% |
| 2 | SSSG switching (XR 45 Mbps) | 17.58% | 17.56% |

LP-WUR-assisted SSSG switching shows 16% to 19% PS gains compared to R17 SSSG switching.

RAN1 to consider LP-WUR-assisted SSSG switching in RRC CONNECTED, where UE is configured with one SSSG for LR to monitor LP-WUS and another SSSG for MR to monitor DCI.

### OFDM-based LPWUR

Figure 6 shows the performance of the OFDM-based LPWUR. It is beneficial only when e-DRX is configured, if the e-DRX cycle is greater than or equal to 20480ms (20.48s), and if LPWUR only monitors LPWUS once (active for 1ms) per e-DRX cycle and is powered off in the rest of time.

Figure 6: Power saving gain analysis with the following assumptions: MR model (TR 38.875), group paging rate = 10%, SCS = 15kHz, and the OFDM-based LPWUR monitoring PSS and LPWUS only one time per e-DRX cycle.

The key advantage is to reuse the existing NR PSS for LPWUR synchronization. It could prevent NW from sending a new broadcast signal for LPWUR. The downside is the high-power consumption for coherent detection, including a high sampling rate and high accuracy for LO and CLK. The key disadvantage is no latency gain because the LPWUR also needs a duty cycle to lower its power consumption, and that eliminates potential latency gain.

Based on the result, we recommend RAN1 should stop pursuing OFDM-based LPWUR in the R18 SI.

OFDM-based LPWUR has no power-saving gain to R17 PEI without a duty cycle.

OFDM-based LPWUR has no power-saving gain to R17 PEI if its duty cycle is smaller than 20.48s.

RAN1 should stop pursuing OFDM-based LPWUR for SSS-like LPWUS in the R18 LPWUR/WUS SI.

### Link Budget

Presumably, OFDM-based WUR has a similar coverage range of NR PSS/SSS, which could be the same level as the coverage of PDCCH for paging. Thus, we only highlight the MIL for a non-OFDMM-based OOK LPWUR listed below.

Table 6: Link Budget (MIL) Analysis

| Scenarios | Urban 2.6GHz |  |
| --- | --- | --- |
| Description of LP WUS | Seq + Data + CRC | Seq only |
| System configuration |  |  |
| Carrier frequency (GHz) | 2.60 |  |
| Pathloss model | NLOS TDL-C |  |
| Target error rate (BLER/MDR etc.) | 1% initial BLER (no retransmission) |  |
| Company reporting Assumptions for LP-WUS/WUR |  |  |
| False alarm rate (FAR) | <0.1% | <1% |
| Channel Structure | sync: 32 chips data: 32bits (64 chips) CRC: 8 bits (16 chips) | Seq only: 32 chips |
| Number of information bits delivered | 32 | 1 |
| Waveform | OOK |  |
| Coding Scheme | Manchester 1/2 |  |
| Frequency Domain Allocation (MHz) | 4.32 |  |
| Guard band (MHz) | 0.72 |  |
| Time Domain Allocation (Y ms) | 0.5 | 0.142857143 |
| Efficiency(bit/s/Hz) | 0.012698413 | 0.001388889 |
| Adjacent subcarrier interference | PDSCH mapped on resources other than that for WUS and guard band; same power density as WUS |  |
| Receiver structure | Homodyne/zero-IF architecture with baseband envelope detection |  |
| Frequency error/drifts | 200 ppm |  |
| ADC bit-width | 4 |  |
| Sampling rate (MHz) | 3.84 |  |
| Parameters for BB BPF/LPF | 5th Order Butterworth with 4.32MHz bandwidth and with cutoff frequency at approximately 2.16 MHz |  |
| Other assumptions if not listed above |  |  |
| - Preamble: Reuse 802.11ba WUS [1 0 1 0 0 1 0 0 1 0 1 1 1 0 1 1 0 0 0 1 0 1 1 1 0 0 1 1 1 0 0 0] - AWGN: Baseband SNR is measured after the 4.32MHz LPF - Channel: TDL-C with a mobile speed of 3km per hour and RMS delay spread of 300ns (long) - CFO: 200 ppm CFO only results in phase rotation, no sampling impact - Mixer: In-phase only (single RX branch rather than both in-phase and quadrature components) DBB: Only include frame boundary detection and payload detection |  |  |
| Transmitter |  |  |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 100000000 |  |
| (3c) bandwidth used for the evaluated channel (Hz) | 4320000 |  |
| Receiver |  |  |
| (10) Number of receiving antenna elements | 1 |  |
| (13) Receiver noise figure (dB) | 15 |  |
| (19) Required SNR (dB) | 5.00 | 1.25 |
| (22) Receiver sensitivity | -85.26 | -89.01 |
| (22bis) MCL | 134.67 | 138.42 |
| (23) Hardware link budget, MIL | 140.80 | 144.55 |
| Calculation of available path loss |  |  |
| (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) | 150.76 | 188.03 |

The required SNR values are obtained by the link-level simulations (LLS). Given 224kbps of the raw data rate, the data payload required 5dB SNR to achieve 1% initial BLER, and the preamble requires 1.25 dB of SNR to achieve 1% MDR. Note that the SNR values are measured after the 4.32MHz LPF in the LLS.

Figure 7: Required baseband SNR at 1% initial BLER in TDL-C channel

MIL (maximum isotropic loss) in PDCCH for Paging is 157.62 dB with a maximum range of 406.24 m. MIL in PUSCH for eMBB is 138.28 dB with a maximum range of 155.88m. Presumably, up to the raw data rate of 224kbps can achieve the same coverage range of NR PUSCH for eMBB, given the 4.32MHz bandwidth in TDL-C with 3km per hour.

To achieve the range of 406.24m the same as NR PDCCH for paging, the targeted SNR will be -12dB. We observe a 7dB gap between our best result of -5dB and the target SNR. The best result (raw data rate = 28kbps) carries only 1 information bit and uses 32 OFDM symbols, resulting in an LPWUS duration of 1.143ms at 30kHz SCS.

LPWUR cannot achieve the MIL of NR PDCCH for paging of AL16 with 4RX and there is a 7dB required SNR gap between the required SNR target and the performance given the raw data rate of 28kbps.

LPWUR can support the MIL of NR PUSCH for eMBB with a raw data rate of up to 224kbps.

RAN1 should stop pursuing the same coverage range of NR PDCCH for paging of AL16 with 4RX (406.24m) for LPWUR in the Rel-18 LPWUS/WUR study at least for Urban 2.6GHz.

Since the coverage range of NR PUSCH for eMBB can be supported/guaranteed by LPWUR, RAN1 should focus at least on LPWUS use cases in RRC CONNECTED.

# Conclusion

In this contribution, we have the following observations and proposals.

Observation 1	If ultra-deep sleep aims for a power consumption of less than LPWUR (e.g., < 1mW), it requires at least 2s to ramp up based on the current implementation for NR eMBB.

Proposal 1	For eMBB, consider the following power models for an “ultra-deep sleep” power state for the main ratio for R18 LPWUS evaluation.

| Use case | Power State | Relative Power (unit) | Ramp-up and down transition energy (unit multiplied by ms) | Ramp-up time | Ramp-down time | Time for sync/re-sync |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB | Ultra-deep sleep | 0.015 (<<1mW) | At least 10000 | At least 2 seconds | At least 1 seconds | At least 82 ms based on R1-2212261 |

Observation 2	LPWUS and LPHAP (Low Power High Accuracy Positioning) both target industrial IoT scenarios and have similar considerations on hardware requirements.

Proposal 2	For IoT and wearables, consider the following power models for an “ultra-deep sleep” power state for the main ratio for R18 LPWUS evaluation.

| Use case | Power State | Relative Power (unit) | Ramp-up and down transition energy (unit multiplied by ms) | Ramp-up time | Ramp-down time | Time for sync/re-sync |
| --- | --- | --- | --- | --- | --- | --- |
| IoT and wearables | Ultra-deep sleep | 0.015 | 10000 | Total transition time: 400ms; Option: 1s |  | At least 82 ms based on R1-2212261 |

Observation 3	For a non-OFDM-based LPWUR, RAN1 considers removing the relative power values of 1, 2, and 4 because the non-OFDM-based LPWUR is typically less than 1 mW.

Observation 4	For a non-OFDM-based LPWUR, RAN1 considers a zero ramp-up time due to a simple receiver architecture without PLL, AGC, advanced CPU, or heavy memory usage.

Proposal 3	Consider the following power model for non-OFDM-based LPWUR for FR1 evaluation.

| Power State of LPWUR | Relative Power (unit) | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| LPWUR Off | 0.001 | 0 | 0 |
| LPWUR On | 0.01/0.05/0.1/0.5 | N/A | N/A |

Observation 5	OFDM-based LPWUR can reuse LO, ADC, and DFT modules from the MR to perform coherent detection on an SSS-like LP-WUS.

Proposal 4	The power states for an OFDM-based LPWUR can be modeled as follows:

| Power State of LPWUR | Relative power | Transition energy | Ramp-up time (ms) | Notes |
| --- | --- | --- | --- | --- |
| OFDM-WUR ON | 40 | N/A | N/A | Share LO and ADC |
| OFDM-WUR OFF | 0.2 | 0 (ideally) | 0 (ideally) | Maintain the 26 MHz-CLK |

Observation 6	Introducing of LPWUS has no power saving benefit if IDLE UE has RRM duty per I-DRX cycle of 1.28s. The power saving gain is -12% compared to R17 PEI with 0.05 as LPWUR ON power.

Proposal 5	RAN1 to postpone the study on LPWUS used with I-DRX until the current NR RRM requirement can be relaxed.

Observation 7	LPWUS can achieve a 51% power saving gain with an eDRX cycle of 61.44s.

Proposal 6	RAN1 to consider focusing on the study of LPWUS used with eDRX in RRC IDLE.

Observation 8	LP-WUR-assisted SSSG switching shows 16% to 19% PS gains compared to R17 SSSG switching.

Proposal 7	RAN1 to consider LP-WUR-assisted SSSG switching in RRC CONNECTED, where UE is configured with one SSSG for LR to monitor LP-WUS and another SSSG for MR to monitor DCI.

Observation 9	OFDM-based LPWUR has no power-saving gain to R17 PEI without a duty cycle.

Observation 10	OFDM-based LPWUR has no power-saving gain to R17 PEI if its duty cycle is smaller than 20.48s.

Proposal 8	RAN1 should stop pursuing OFDM-based LPWUR for SSS-like LPWUS in the R18 LPWUR/WUS SI.

Observation 11	LPWUR cannot achieve the MIL of NR PDCCH of AL16 with 4RX for paging and there is a 7dB required SNR gap between the required SNR target and the performance given the raw data rate of 28kbps.

Observation 12	LPWUR can support the MIL of NR PUSCH for eMBB with a raw data rate of up to 224kbps.

Proposal 9	RAN1 should stop pursuing the same coverage range of NR PDCCH for paging of AL16 with 4RX  (406.24m) for LPWUR in the Rel-18 LPWUS/WUR study at least for Urban 2.6GHz.

Proposal 10	Since the coverage range of NR PUSCH for eMBB can be supported/guaranteed by LPWUR, RAN1 should focus at least on LPWUS use cases in RRC CONNECTED.

# Reference

R1-2213005, Final FL summary of evaluation on low power WUS, Moderator (vivo)

R1-2212261, Evaluation on low power WUS, MediaTek Inc.

R1-2301578, Low power WUS receiver architectures, MediaTek Inc.

# Appendix

## Processing timelines

### I-DRX

Table 7: Power consumption timeline for the Paged case

| UE operations in a paging cycle | Time duration (ms) | Energy contribution (relative power * ms) | value |
| --- | --- | --- | --- |
| LP-WUR ON | 1280 | PLP-WUS-ON*1280 | 64 |
| initial SSB search | 20 | PSSB * 20 | 1000 |
| SSB processing | 2 | PSSB * 2 | 100 |
| Light sleep | 18 | PLS * 18 + 100 | 424 |
| SSB processing | 2 | PSSB * 2 | 100 |
| Light sleep | 18 | PLS * 18 + 100 | 424 |
| SSB processing | 2 | PSSB * 2 | 100 |
| Light sleep | 18 | PLS * 18 + 100 | 424 |
| SSB processing | 2 | PSSB * 2 | 100 |
| Light sleep | 18 | PLS * 18 + 100 | 424 |
| SSB processing | 2 | PSSB * 2 | 100 |
| Light sleep | 18 | PLS * 18 + 100 | 424 |
| SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4] + Pintra, meas-only * (1 – [1/4] ) ) ) * 0.85* 2 | 361.25 |
| Light sleep | 8 | PLS * 8 + 100 | 244 |
| PO reception | 4 | PPDCCH+PDSCH * 4 | 480 |
| Light sleep | 6 | PLS * 6 + 100 | 208 |
| Switch to another frequency layer | 0.5 | PMS * 0.5 | 15.5 |
| Inter-freq. RRM measurement | 5 | (Pinter, search-only * [1/4] + Pinter, meas-only * (1 – [1/4] ) ) * 5 | 750 |
| Switch back to serving frequency | 0.5 | PMS * 0.5 | 15.5 |
| Ultra-Deep sleep | 1194 | PUDS * 1194 + EUDS | 10017.91 |
| (Total) | 1280 | Total Energy | 15776.16 |

Table 8: Power consumption timeline for the Not Paged case

| UE operations in a paging cycle | Time duration (ms) | Energy contribution (relative power * ms) | value |
| --- | --- | --- | --- |
| LP-WUR ON | 1280 | PLP-WUS-ON*1280 | 64 |
| Inter-freq. RRM measurement | 5 | (Pinter, search-only * [1/4] + Pinter, meas-only * (1 – [1/4]) ) * 5 | 750 |
| Ultra-Deep sleep | 1275 | PUDS * 1275 + EUDS | 10019.13 |
| (Total) | 1280 | Total Energy | 10833.13 |

### eDRX

Table 9: Power consumption for R18 LPWUS with the eDRX cycle of 61.44s

| UE operation | time duration (ms) | Energy contribution | value |
| --- | --- | --- | --- |
| LPWUR on | 61440 | P_LP_WUS-ON*total_time | 3072.00 |
| resync | (wihtin one IDRX) | P_resync*(1-R_nosync) | 2549.15 |
| IDRX | 5120 | P_IDRX | 11842.04 |
| Ultra-deep sleep | 56320 | P_UDS*UDP_time+E_UDS | 10844.8 |
| total time | 61440 | total energy | 28307.99 |

Table 10: Power consumption for R17 PEI with the eDRX cycle of 61.44s

| UE operation | time duration (ms) | Energy contribution | value |
| --- | --- | --- | --- |
| PEI | 4 | P_PDCCH*4 | 200 |
| IDRX | 5120 | P_IDRX | 11842.04 |
| Deep sleep | 56316 | P_DS*DP_time+E_DS | 45502.8 |
| total time | 61440 | total energy | 57544.84 |

## Evaluation Models for LP-WUR-Assisted SSSG Switching

The power model and simulation parameters are in this subsection.

| Simulation parameters | Value |
| --- | --- |
| Deployment | 3GPP Dense Urban, 7 sites, 3 sectors per site, ISD = 200m 80% indoor UE, 20% outdoor UE |
| Channel model | 3D-UMa |
| Channel estimation | Ideal |
| SCS | 30 kHz |
| Frame structure for TDD | DDDSU |
| Scheduling scheme | Proportional fair |
| Bandwidth | 100MHz; 2.08% guard band (272 360kHz RB in 100MHz) |
| BS Tx antenna config. | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) with 64 TXRU (dH, dV) = (0.5λ, 0.5λ) Down tilt: 12 degrees SU-MIMO |
| BS height | 25 m |
| UE Rx antennas config. | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) 2T/4R (dH, dV) = (0.5λ, N/A) |
| UE height | 1.5 m |
| UE speed | 3 km/h |
| Antenna Gain | BS: 8 dBi, 3-sector antenna UE: 0 dBi, omni-directional |
| Noise figure | BS: 5dB; UE: 9dB |
| Max Tx power | BS: 44 dBm per 20 Mhz UE: 23 dBm |

| Traffic | AR/VR (Follow the setting in TR 38.838) |  |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| PDB (ms) | 10 |  |
| Packet size distribution | Truncated Gaussian |  |
| mean packet size (Bytes) | 62500 | 93750 |
| Ratio: STD/mean pkt size | 0.105 |  |
| Ratio: Max/mean packet size | 1.5 |  |
| Ratio: min/mean packet size | 0.5 |  |
| Packet arrival rate (sec) | Periodic (1/FR), FR = 60 |  |
| Jitter value distribution | Truncated Gaussian |  |
| Avg. jitter value (ms) | 0 |  |
| Jitter value STD (ms) | 2 |  |
| Max jitter value (ms) | 4 |  |
| Min jitter value (ms) | -4 |  |

| Basic power consumption for PDCCH/LP-WUS monitoring (uint) |  |
| --- | --- |
| PDCCH monitoring in SSSG 0/1 | 100 (PDCCH monitoring in TR 38.840) |
| LP-WUS monitoring | 45 (micro sleep power in TR 38.840) |
| Additional power consumption for LR (unit) |  |
| LP-WUS off | 0.001 |
| LP-WUS on | 0.005, 0.05, 4 |

## Agreements

### RAN1#111

Agreement

For system impact analysis, the following performance metrics are considered to be provided,

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |
| Capacity impact | Evaluate the system capacity impact due to introducing of LP-WUS Note: it is for UEs which are in connected mode. Definition is the same as in XR TR. |
| FFS: NW power consumption / Energy Efficiency | [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.] |

For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study

| Performance Metric | Note |
| --- | --- |
| Power consumption | Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme). |
| Latency | For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency . In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included |
| UPT | The definition is the same as in [TR38.840] Note: it is for connected mode purpose. |

Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI

Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact

Other performance metrics (e.g., mobility) can be reported by companies (if any)

Agreement

Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2,

- The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where

  - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s

  - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group

    - FFS: Value of N

- For LP-WUS

  - Both per group and UE paging can be assumed.

- Note：

- For i-DRX with i-DRXcycle durationY second,

  - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF

- For e-DRX with K i-DRXcyclesduration, L PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second

  - Per UE paging probability is

    - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW

    - RE = 1 – (1 – RE, REF )LY/YREF for each of the remaining L-1 i-DRX cycles within the PTW

  - L=4 (as agreed in RAN1#110bis)

Agreement

For MR, at least for FR1 evaluation,

- Number of SSBs for sync/re-sync for MR is up to 10

  - Companies to report timeline and energy consumption

- Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112

Agreement

The following power model for LP-WUR is used for evaluation for FR1,

| Power State | Relative Power (unit) | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| Off | 0.001 | [TLR, ramp-up *(PON+POFF)/2] | TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up |
| On | 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed |  |  |

FFS: whether further categorization/sub-categorization is needed and how.

### RAN1#110bis-e

For future meetings on LP WUS:

Use the following terminology for future discussion,

- Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up

- LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

Agreement in RAN1#110b-e

For evaluation, 1 Rx chain for LP-WUS receiver is baseline.

Agreement in RAN1#110b-e

Both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI.

- FFS: Further prioritization if needed during the study item.

Agreement in RAN1#110b-e

Take the following power model for main radio for evaluation in LP-WUS/WUR SI,

- For IoT and wearable cases, reuse TR38.875 power model as baseline.

- For eMBB and other cases, reuse TR38.840 power model as baseline.

- Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver

  - FFS: The details of ‘Ultra-deep sleep’ power state

Agreement in RAN1#110b-e

- The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation

| Power State | Relative power (unit) | Ramp-up and down transition energy (Note1): (unit multiplied by ms) | Ramp-up time | Time for sync/re-sync |
| --- | --- | --- | --- | --- |
| Ultra-deep sleep | [0.015] | [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection | [400ms], FFS: 100ms | X |

Note1:

- Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.],

- Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.],

  - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio

  - Companies can report the assumption of X in the initial evaluation.

- Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated.

- The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync.

  - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption

Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state.

Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS

Agreement in RAN1#110b-e

- The following power model for LP-WUR/WUS evaluation is considered,

  - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring:

    - [0.001]

  - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring:

    - [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4]

    - Other values are not precluded to be evaluated.

    - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation

  - No additional transition energy and transition time between ‘on’ and ‘off’ state as start point, FFS any transition energy and transition time if needed.

Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver.

Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion.

Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture)

FFS: LP-WUR power consumption values for FR2.

Agreement in RAN1#110b-e

For R18 LP-WUS/WUR power evaluation in RRC connected mode, the following can be considered,

- XR traffic model with evaluation methodologies and assumptions captured in TR 38.838.

- eMBB traffic model with evaluation methodologies and assumptions captured in TR 38.840

- Heartbeat traffic models in 3GPP TR 38.875.

- Other models are not precluded.

Company to further provide the followings,

  - Parameters (e.g., frame rate, data rate, jitter range, DRX configurations and etc if needed.)

  - How to use LP-WUS, e.g., LP-WUS to trigger/adapt PDCCH monitoring

  - Other details if any

Agreement in RAN1#110b-e

- For LP-WUS coverage evaluation, the noise figure of LP-WUR is

  - Options: [9, 12, 15, 18, 21, 24], Other values can be reported by companies

- FFS: how to determine the NF option.

- The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion.

Agreement in RAN1#110b-e

For the performance evaluations of LP-WUS candidate designs, it is assumed that

- The miss-detection rate (MDR) of LP-WUS [1%],

- The false-alarm rate (FAR) of LP-WUS

  - [0.1%, 1%, 10%]

  - Other values are not precluded for studying reported by companies

- Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly

- The above values applied in both RRC CONNECTED and IDLE/INACTIVE mode.

- FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead

- FFS: Note: FAR should be evaluated both in the absence of gNB transmissions and in the presence of transmissions from gNB. Proponent to provide the details.

Agreement in RAN1#110b-e

For system impact analysis, the following performance metrics are considered to be provided,

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |
| FFS: Capacity impact | [Evaluate the system capacity impact due to introducing of LP-WUS] |
| FFS: NW power consumption / Energy Efficiency | [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.] |

For power and latency evaluation of the LP-WUS, the following performance metrics are considered to be provided.

| Performance Metric | Note |
| --- | --- |
| Power consumption | Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme). |
| Latency | For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included For CONNECTED state, TBD |
| FFS: UPT | FFS Note: it is for connected mode purpose. |

Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI

Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact

Other performance metrics (e.g., mobility) can be reported by companies (if any)

Agreement in RAN1#110b-e

The following is assumed for RRC IDLE/INACTIVE evaluation,

| Parameters | Value |
| --- | --- |
| i-DRX cycle length | 1.28s and other values not precluded and reported by companies, consider both with PEI/ without PEI |
| e-DRX cycle length | 20.48s, 61.44s and other values not precluded, company to report which value(s) are used. Note: ‘ultra-deep sleep’ state can be assumed for eDRX whenever necessary for baseline UE |
| Number of POs in Paging Frame | 1 |
| Number of DRXs per PTW | 4 |
| Number of SSB before PO / PEI | 1, 2 or 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) company to report which value(s) are used Note: the assumptions is for MR wakes from ‘Deep sleep’ |
| Sync/re-sync after ultra-deep sleep | companies to report the timeline of sync/re-sync and X value, X is the time for sync/re-sync |
| RRM Measurement | Company to report whether and how the RRM measurement is assumed, e.g., whether RRM performed by main radio or LP-WUR, whether RRM is relaxed or not. |
| LP-WUS monitoring | Option 1: continuously monitoring Option 2: discontinuously monitoring, with [T] ms as the period for complete an on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle. |
| Traffic | Option 1 (baseline): Per UE paging rate (R_E)= ([1%]) or ([0.1%]) or ([0.01%]) or ([0.001%]) within duration Y, [FFS Y is an i-DRX cycle length or an absolute time duration length] - R_G denotes as the group paging rate and R_E denotes as UE paging rate, and 1-R_G=(1-R_E)^N, where N is the number of UEs in the group, and N is [TBD] - FFS: how (R_G, R_E) for e-DRX derived from FFS: Option 2 (optional): Reusing TR 38.875 heart beat traffic model Model; FTP3. Packet size; 100 Bytes. Mean inter-arrival time; 60s (per UE paging rate≈2%). Model RRC connection phase power consumption as follows, RRC connection duration; [30ms]. Relative energy consumption of RRC connection block (Relative power x ms); [=3000]. Other options are not precluded can be reported by companies. |
| Others | Reported by companies |

Agreement in RAN1#110b-e

For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline.

- MIL is used as the metric for LP-WUS coverage evaluation

- urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded.

Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830.

Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss

Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS