3GPP TSG-RAN WG1 Meeting #112	R1-2301607

# Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on validation of PRACH and PUSCH occasions with SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For unpaired spectrum (TDD) operation, it is not clear how a RedCap UE should determine valid PRACH and PUSCH occasions based on current Clause 8.1 and 8.1A, respectively, when the UE is configured with NCD-SSB or is not configured with any SSB in a DL BWP. The reason is whether the SS/PBCH blocks in Clauses 8.1 and 8.1A are referred to only SS/PBCH blocks configured in a BWP or referred to all SSBs across all configured BWPs in a cell. Since NCD-SSB can be configured with a non-zero time offset relative to CD-SSB, it can invalidate different PRACH/PUSCH occasions than those invalidated by CD-SSB. Furthermore, since NCD-SSB is configured via dedicated BWP configuration, some UEs may be configured with NCD-SSB while the others may not even when these UEs are in the same BWP. Therefore, if different UEs apply different SSBs and obtain different sets of valid PRACH/PUSCH occasions, it may cause the same PRACH/PUSCH resource to be associated with different SSBs by different UEs. This further causes SSB (QCL assumption) ambiguity for Msg2/MsgB/Msg4 transmission (reception) at gNB (UE) side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that UE should always apply CD-SSB to determine valid PRACH and PUSCH occasions in Clause 8.1 and 8.1A regardless of whether UE is configured with CD-SSB, NCD-SSB, or no SSB in a BWP in TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether a RedCap UE should apply NCD-SSB or both CD-SSB and NCD-SSB to determine valid PRACH/PUSCH occasions when the UE is configured with NCD-SSB in a BWP. It is also not clear whether a UE should apply CD-SSB or no SSB to determine valid PRACH/PUSCH occasions when the UE is not provided any SSB in a BWP. If different UEs in the same cell apply different SSBs (CD-SSB, NCD-SSB, both, or none) to determine valid PRACH/PUSCH occasions and obtain different SSB-PRACH/PUSCH association patterns, it may potentially cause SSB (QCL) ambiguity for transmitting (receiving) Msg2/MsgB/Msg4 at gNB (UE). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | There is a supplemental t-doc in R1-2301606. Isolated impact analysis: This CR has isolated impact because it corrects the RedCap UE behaviour on determining valid PRACH and PUSCH occasions in TDD. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 17.1	RedCap UE procedures

Procedures for a RedCap UE are same as described for a UE in all other clauses of this document unless stated otherwise. In this clause, the term 'UE' refers to a RedCap UE.

[… Irrelevant text omitted …]

For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB, collision handling between downlink receptions or uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated.

If a UE is indicated the presence of SS/PBCH blocks by NonCellDefiningSSB or if a UE is not indicated the presence of any SS/PBCH blocks within a DL BWP, the SS/PBCH block in Clause 8.1 for determining valid PRACH occasions in unpaired spectrum corresponds to the SS/PBCH blocks that the UE used to obtain SIB1.

If a UE is indicated the presence of SS/PBCH blocks by NonCellDefiningSSB or if a UE is not indicated the presence of any SS/PBCH blocks within a DL BWP, the SS/PBCH block in Clause 8.1A for determining valid PUSCH occasions in unpaired spectrum corresponds to the SS/PBCH block that the UE used to obtain SIB1.

For monitoring of a PDCCH candidate by a UE configured with NonCellDefiningSSB, if the UE

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by NonCellDefiningSSB,

the UE is not required to monitor the PDCCH candidate.