3GPP TSG-RAN WG1 #112	R1-2301633

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:		9.5.1.2

Source:	MediaTek Inc.

Title:	Measurement and reporting design for sidelink positioning

Document for:		Discussion

# Introduction

Three positioning methods, SL-RTT, SL-AOA and SL-TDOA are agreed to support for sidelink positioning. The corresponding measurement types and the reporting need to be specified.

In this contribution, we further elaborate the TX timing adjustment issue for RTT measurements.

# Measurements for SL-RTT

The RTT information between 2 UEs requires the measurement, RX-TX time difference, to be performed at two sides. The main design of the measurement type may consider

- The RX (reception) and TX (transmission) timing

The RTT between 2 UEs could be derived combining the measurements between them. For the UE at each side, the measurement and transmission are performed. Currently in downlink, the TX timing within the UE RX-TX time difference measurement is the timing of intended TX when RX timing is measured. This also means, the transmission may not really happen when measuring the timing of the received signal. The actual transmission may happen at a different time instance. As such, the actual TX timing could be different from the intended TX timing.

The change of TX timing could be both aware and unaware to the UE. The timing change due to the residual sampling frequency offset (SFO), or the relative SFO between 2 UEs could be treated as unaware. For example, the clock count is not changed for next transmission, but the clock duration does change. As such the actual TX timing is changed accordingly.

To mitigate the offset between intended TX timing and actual TX timing, two methods could be considered,

- The TX timing within the RX-TX time difference measurement applies the actual TX timing

- Indicate a timestamp for TX timing change and the corresponding adjustment value

Fig. 2-1 gives an example. For the first method, when UE B changes the TX timing at t4, the measurements by UE B at t3 based on previous TX timing can be modified to apply the new TX timing, since the reporting will happen later at t6. Then the measurement by UE B at t3 and by UE A at t5 could be combined. However, it is not clear for a server functionality to figure out whether the measurements at t1, t2 and t3 could be combined.

As we further look at Fig. 2-4, if actual TX timing is considered, then the TX timing for measurement at t2 may select using the actual TX timing at t1 or at t4, which doesn't solve the problem.

If the TX timing adjustment happens after the transmission, as shown in Fig. 2-2, it is still unknown for a server functionality whether the measurements at t1, t2 and t4 could be combined, since the server functionality doesn’t know when the UE adjusts TX timing. The combination may not be limited within two transmissions, one for each side. The combination on more number of transmissions maybe helpful to mitigate the timing change due to residual SFO.

Fig. 2-3 shows the case that when the TX timing adjustment at t4 is indicated, the server functionality may know that the measurements at t1, t2 and t3 could be combined. Moreover, if the timing adjustment value can be further provided, it further provides the flexibility for the server functionality to combine the measurements at t1, t2, t3, t5 and t6.

Then, the definition of SL RX-TX time difference could reuse UE RX-TX time difference that TX timing is the intended TX timing during the measurement on the RX timing of received signal. The additional add-on to improve the accuracy of the combination of the measurements may provide the timestamp of TX timing adjustment and the corresponding adjustment value.

A RTT measurement could be derived based on the combination of SL RX-TX time difference measurements between two UEs when each UE transmits SL-PRS once.

Proposal 2-1: For SL RX-TX time difference definition, the RX and TX timing could follow the definition for UE RX-TX time difference by modifying as the sidelink subframe

Proposal 2-2: When the TX timing is adjusted during the SL RX-TX time difference measurement, the corresponding timestamp and the adjustment value could be provided

Fig. 2-1,

Fig. 2-2,

Fig. 2-3,

Fig. 2-4,

# Measurements for SL-TDOA

SL-TDOA may have two transmission directions, target to anchors, which is similar to UL-TDOA as one-to-many (groupcast), and anchors to target, which is similar to DL-TDOA as many-to-one (unicast).

The SL-TDOA use case maybe limited to the absolute positioning, especially when RSUs are deployed. Both the transmission directions are applicable for absolute positioning. The RSUs could also transmit and receive in between to acquire the synchronization error estimation between a pair of RSUs. In [1], we have shown how the combination on the measurement results by DL-TDOA and UL-TDOA could estimate the synchronization error.

This also means, even the DL-TDOA like or UL-TDOA like transmission could choose one to apply, the hybrid mode is still useful to solve the synchronization problem. The transmission periodicity for two different directions don't need to be the same.

It is beneficial to support SL-TDOA in two transmission directions. Then the corresponding measurement types SL-RTOA and SL-RSTD are supported to be defined.

Proposal 3-1: Support both SL-RSTD and SL-RTOA measurements for SL-TDOA

# Reporting

The positioning method agreed for SL positioning has been narrowed down as compared to that for DL/UL positioning. Some common measurement types apply for different positioning method, for example the PRS RSRP measurement.

We consider the reporting by measurement types directly. As such, the duplication could be avoided when multiple positioning methods apply.

Proposal 4-1: The reporting is by the measurement types directly

# Conclusion

Proposal 2-1: For SL RX-TX time difference definition, the RX and TX timing could follow the definition for UE RX-TX time difference by modifying as the sidelink subframe

Proposal 2-2: When the TX timing is adjusted during the SL RX-TX time difference measurement, the corresponding timestamp and the adjustment value could be provided

Proposal 3-1: Support both SL-RSTD and SL-RTOA measurements for SL-TDOA

Proposal 4-1: The reporting is by the measurement types directly

# Reference

[1] R1-2101142