3GPP TSG RAN WG1 #112			R1-2301732

Athens, Greece, February 27th – March 3rd, 2023

Source:	WILUS Inc.

Title:	Discussion on subband non-overlapping full duplex

Agenda item:	9.3.2

Document for:	Discussion

Introduction

In the previous RAN1#111 meeting, the following agreements were made on subband non-overlapping full duplex (SBFD) [1].

Agreement at RAN1#111: For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement at RAN1#111: For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously Agreement at RAN1#111: For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. Agreement at RAN1#111: Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured. Agreement at RAN1#111: Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

In this contribution, we provide our views on subband non-overlapping full duplex.

Discussion on subband non-overlapping full duplex

Subband aware UE behavior

In the previous RAN1#111 meeting, agreements were made on baseline UE behaviors of SBFD aware UE that semi-statically configured with UL subband in a SBFD symbol configured as DL and flexible in TDD-UL-DL-ConfigCommon. There are two options and some FFSs on agreement for SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon. We provide our views on those two options and FFSs.

With Option 1, SBFD aware UE follows UE behavior in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon. Thus, SBFD aware UE can only transmit UL signal/channel within UL subband and receive DL signal/channel within DL subband(s), if FFS is not considered as the following.

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol.

Some companies proposed to study DL reception outside DL subband(s) to reflect dynamic and variable traffic conditions. For example, if DL traffic increases for a UE operating with SBFD, a gNB dynamically indicates SBFD aware UE to receive PDSCH via DCI format 1_0, 1_1, or 1_2 across DL subband(s) and UL subband by utilizing more RBs for DL reception.

With Option 2, the RBs outside the UL subband can be used as either UL or DL excluding guardband(s) if used, in the symbol from gNB’s perspective. Thus, UL transmission for SBFD aware UE is not limited within UL subband. Compared to Option 1, Option 2 can utilize more RBs for UL transmission that is beneficial for enhanced UL coverage, reduced latency, and improved capacity.

- FFS: Whether or not the symbol can be converted to a DL-only symbol.

Similar to issue about DL reception outside DL subband(s) in Option 1, this FFS in Option 2 was originated from the necessity to schedule more RBs for DL reception in a symbol operating with SBFD (i.e., SBFD symbol). In other words, this FFS in Option 2 is also motivated to allow DL reception outside DL subband(s). Difference with the FFS in Option 2 is explicit allowance via symbol conversion, while the above FFS in option 1is implicit allowance via gNB scheduling.

Comparison among two FFSs in both options for DL reception outside DL subband(s) is showed in Figure 1. Figure 1-(a) is the implicit method (i.e., via gNB scheduling) for DL reception outside DL subband(s) and Figure 1-(b) is the explicit method (i.e., via symbol conversion) for DL reception outside DL subband(s).

Figure 1. Comparison between implicit and explicit method for DL reception outside DL subband(s).

In Figure 1-(a), a gNB cannot schedule PDSCH for UE#1 due to resource collision with CG-PUSCH for UE#2 in slot#n. On the other hand, an gNB can schedule PDSCH for UE#1 in slot#n by explicitly converting SBFD symbol to DL-only symbol. In this case, CG-PUSCH for UE#2 in the slot#n is cancelled according to UE behavior in DL symbol. When considering that motivation of DL reception outside DL subband(s) is to reflect dynamically increased DL traffic, explicit method is more likely to schedule DL reception instantly with more RBs in the same symbol than implicit method for DL reception outside DL subband(s).

- Observation 1: Explicit method (symbol conversion to a DL-only) to allow DL reception outside DL subband(s) has more occasions in time domain to schedule DL reception than implicit method (gNB scheduling) for DL reception outside DL subband(s).

To allow DL reception outside DL subband(s) via explicit method, dynamic SFI can be reused for the SBFD aware UE. Note that DL symbol configured by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDeicated can be indicated as DL, while flexible symbol not configured by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDeicated can be indicated as DL, UL, or flexible.

Regarding that the dynamic SFI is transmitted via GC (group-common) PDCCH including DCI format 2_0 with CRC scrambled by SFI-RNTI, both SBFD aware UE and non-SBFD aware UE can be configured to monitor GC-PDCCH with the same SFI-RNTI value in a given cell. Impact and potential enhancements on dynamic SFI can be further studied by considering the co-existence of SBFD aware UE and non-SBFD aware UE in a given cell and the case of dynamic SFI detection/non-detection from the UE’s point of view. For example, if the SBFD aware UE does not detect a dynamic SFI, the SBFD aware UE transmits PUSCH indicated by DCI, while the SBFD aware UE does not transmit PUSCH configured by higher layer.

Based on above discussion, we propose RAN1 to study Option 2 for SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

- Proposal 1: RAN1 to study Option 2 for SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

  - SBFD aware UE can be indicated with explicit method (symbol conversion to a DL-only) to allow DL reception outside DL subband(s) via dynamic SFI.

  - FFS: impact and potential enhancements on dynamic SFI for the co-existence of SBFD aware UE and non-SBFD aware UE in a given cell.

  - FFS: impact and potential enhancements on UE behaviors in case of SFI detection/non-detection point of view.

Additionally, dynamic conversion to DL-only symbol can be commonly applied to a SBFD symbol regardless of symbol type (i.e., DL or flexible). Such a UE behavior for DL reception outside DL subband(s) can also be applied to SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon.

- Proposal 2: SBFD aware UE can be indicated with explicit method (symbol conversion to a DL-only) to allow DL reception outside DL subband(s) via dynamic SFI even in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon.

- FFS: Whether or not signalling of guardband(s) is needed.

With SBFD operation, a symbol can be divided into different link directions, i.e., UL subband and DL subband(s). Although a gNB performs full duplex in a SBFD symbol over non-overlapping RBs, DL reception in RBs close to RBs for UL transmission can create gNB self-interference. Also, UL-to-DL inter-UE/inter-subband CLI can be occurred.

To address those interference issues, guardband(s) can be placed between UL subband and DL subband(s). The size of guardband(s) can be variable according to interference level and the number of RBs in UL subband. Since RBs in guardband(s) cannot be used for scheduling, cell-level throughput would be impacted if many RBs are used for guardband(s).

For this reason, fixed size of guardband(s) can be used for SBFD operation. It can be determined with the minimum number of RBs to protect the gNB from the self-interference and CLI among UEs. The minimum fixed size of guardband(s) can be determined based on the RAN4 study. RBs in the minimum fixed size of guardband(s) are not used for scheduling.

In addition to the minimum fixed size of guardband(s), RBs in UL subband or DL subband(s) can be used for additional guardband(s) according to the interference status and/or dynamic traffic changes. It can depend on the gNB’s implementation to schedule DL reception or UL transmission in RBs outside guardband(s). With the minimum fixed size of guardband(s) and additional flexible guardband(s), cell-level throughput is less affected than allocating too many RBs on the fixed guardband(s) only.

Based on the agreements at the RAN1#111 meeting that the frequency location of UL subband and DL subband(s) are known to a SBFD aware UE. Because the SBFD aware UE also can implicitly know the frequency location of fixed guardband(s), the signalling of guardband(s) is not needed. Even for the RBs outside the UL subband, which can be used either UL or DL, the signalling of guardband(s) is not needed since the frequency location of fixed guardband(s) is not changed.

- Proposal 3: Signalling of guardband(s) is not needed for SBFD aware UE.

  - SBFD aware UE can know the frequency location of fixed guardband(s) based on knowledge of the frequency location of UL subband and DL subband(s).

- FFS: SBFD aware UE behaviors.

In Option 2, the RBs outside the UL subband can be used as either UL or DL. Those RBs are not belonging to any type of subband, such as UL suband and DL subband(s), and guardband(s). Regarding that those RBs are in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the SBFD aware UE can still regard those RBs in flexible symbol as the flexible symbol. Therefore, the SBFD aware UE can follow legacy UE behavior in the RBs outside the UL subband in the flexible symbol. For example, if the SBFD aware UE does not detect a dynamic SFI, the SBFD aware UE transmits PUSCH indicated dynamically by DCI, while the SBFD aware UE does not transmit PUSCH configured semi-statically by higher layer.

- Proposal 4: SBFD aware UE follows legacy UE behavior in the RBs outside the UL subaand that can be used as either UL, or DL excluding guardband(s) in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

Resource allocation

In the previous RAN1#111 meeting, it was agreed to study the impact and potential enhancements to resource allocation for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols. Also, the agreement was made to study the impact and benefits of potential enhancements to resource allocation in frequency domain considering unaligned boundaries between resource block group(s)/reporting subband(s), and SBFD subbands. We provide our views on impact and potential enhancements to resource allocation for subband non-overlapping full duplex operation.

    - PDSCH

PDSCH frequency domain resource allocation

With {D-U-D} subband frequency pattern, the available DL resources are distributed across two non-contiguous DL subbands. Current FDRA type 0 can be used for resource allocation across non-contiguous RBGs, thus it can be reused for SBFD aware UE. Note that the number of PRBs within a RBG (i.e., P) depends on the number of PRBs within a DL BWP.

Figure 2. PDSCH FDRA type 0 with {D-U-D} subband frequency pattern.

In Figure 2, a UE is configured with DL BWP size as 275 PRBs (100MHz with 30kHz SCS) and allocated with DL subbands as 110 PRBs (40MHz) and UL subband as 55 PRBs (20MHz). In this case, the UE determines P = 16 regardless of RBG-Size configured from gNB. Then, RBG 6-10, which overlap with UL subband cannot be used for PDSCH FDRA for SBFD aware UE. Especially, PRB 96-109 of RBG 6 and PRB 165-175 of RBG 10 cannot be used even those are included in DL subbands.

Enhancements to utilize those PRBs via FDRA type 0 can be studied. One possible approach is to enhance RBG mapping of FDRA type 0. For example, SBFD aware UE can determine RBGs per DL subband separately. Otherwise, non-contiguous PRBs can be mapped as a single RBG for SBFD aware UE. Alternatively, PDSCH rate-matching mechanism can be reused, i.e., a UE shall assume that PRBs in UL subband (and guard band, if specified) are not available for PDSCH reception. In other words, a SBFD aware UE shall assume that PDSCH is rate-matched around the PRBs in UL subband. This approach has less specification impact and it’s worth noting that FDRA type 1 also can be used to allocate PDSCH across non-contiguous DL subbands via PDSCH rate-matching mechanism.

- Proposal 5: Reusing PDSCH rate-matching mechanism to address unaligned boundaries of DL/UL subband and configuration granularity of RBG for PDSCH FDRA (i.e., FDRA type 0) in terms of specification impact.

PDSCH with slot aggregation

In the previous RAN1#111 meeting, it was also discussed about the impact and potential enhancements on PDSCH receptions with slot aggregation (repetitions) across non-SBFD symbols and SBFD symbols. As we proposed above, impact and potential enhancements on PDSCH FDRA for resource allocation across non-contiguous DL subbands with {D-U-D} subband frequency pattern (e.g., PDSCH is rate matched around PRBs in UL subband) can be studied. In addition, a UE can be configured or indicated to receive PDSCH of the same TB across multiple slots (i.e., slot aggregation). In this case, multiple slots can include both non-SBFD symbols and SBFD symbols. Then, TBS of PDSCH on non-SBFD symbols can be differently determined from that of PDSCH on SFBD symbols. However, TBs with different TBSs are hard to be combined. Therefore, enhancements can be studied for SBFD aware UE to assume that PRBs in UL subband are not available not only for SBFD symbols, but also for non-SBFD symbols as shown in Figure 3 to maximize the combining gain.

Figure 3. PDSCH receptions with slot aggregation across non-SBFD symbols and SBFD symbols.

- Proposal 6: 	Same TBS can be determined among PDSCH receptions with slot aggregation (repetitions) across SBFD symbols and non-SBFD symbols.

    - CSI-RS

In the RAN1#110b-e meeting, it was agreed to study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands. We provide our views on potential options to study CSI-RS resource set frequency domain resource allocation and CSI reporting.

CSI-RS resource set frequency domain resource allocation

Option 1: Two CSI-RS resources are configured.

With this option, RRC signalling overhead can be an issue regarding that a SBFD aware UE should be configured with two CSI-RS resources with different CSI-RS resource IDs. Also, the number of configurable CSI-RS resources is restricted than legacy operation regarding that two CSI-RS resource IDs are necessary for CSI-RS reception in the same symbol.

Option 2: Non-contiguous CSI-RS resource is configured.

For this option, enhancement on CSI-RS resource configuration can be necessary. For example, multiple CSI-RS frequency domain resource allocations (i.e., startingRB and nrofRBs) with the same CSI-RS resource ID should be introduced. This option has no impact on the number of CSI-RS resource configurations. However, it still requires enhancement on CSI-RS resource configuration, which can increase the RRC signalling overhead.

Option 3: One contiguous CSI-RS resource is configured.

With this option, a SBFD aware UE can assume that PRBs in UL subband (and guardband, if specified) are not available for CSI-RS reception. It’s similar to the PDSCH rate-matching mechanism, but different in that CSI-RS cannot be rate-matched. Thus, the SBFD aware UE can implicitly assume that two separate sequences for CSI-RS with the same ID are received across non-contiguous DL subbands, thus it does not have any changes on CSI-RS resource configuration (i.e., one contiguous resource configuration overlapping UL subband). Therefore, it has no specification impact and no additional RRC signalling overhead, which are concerns of Option 1 and Option 2.

Based on above discussion, we propose to study Option 3 (one contiguous CSI-RS resource configuration across non-contiguous DL subbands) in terms of specification impact.

- Proposal 7: RAN1 to study Option 3 (one contiguous CSI-RS resource configuration across non-contiguous DL subbands) in terms of specification impact.

  - A UE can assume that PRBs in UL subband (and guardband, if specified) are not available for CSI-RS reception.

    - PUSCH/PUCCH

PUSCH/PUCCH frequency hopping

If a UE is configured or indicated to transmit PUSCH with frequency hopping (FH), the UE determines FH pattern, i.e., starting RB of 1st hop and 2nd hop respectively. In the current specification, FH pattern can be determined according to the texts from TS 38.214 V17.4.0 [2] as follows.

### 6.3 UE PUSCH frequency hopping procedure ### 6.3.1 Frequency hopping for PUSCH repetition Type A … In case of intra-slot frequency hopping, the starting RB in each hop is given by: , where i=0 and i=1 are the first hop and the second hop respectively, and is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) or as calculated from the resource assignment for MsgA PUSCH (described in [6, TS 38.213]) and is the frequency offset in RBs between the two frequency hops. The number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: , where is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops.

According to the specification, the UE determines FH pattern based on the size of UL BWP. In case that the UL subband is included in the UL BWP, the UE may determine 2nd hop RBs of UL transmission outside the UL subband.

To handle this issue, one possible method is to introduce new configuration of RB offset value for SBFD slot. Otherwise, current modulo mechanism can be modified without new configuration. For this purpose, the size of UL BWP (i.e., NBWPsize) can be modified as the size of UL subband with additional parameters such as starting RB index of the UL subband to align the different RB starting between non-SBFD symbols and SBFD symbols as shown in Figure 4.

Figure 4. PUSCH transmission with frequency hopping across non-SBFD symbols and SBFD symbols.

If a UE is configured to transmit PUCCH with frequency hopping (FH), the UE determines FH pattern, i.e., starting RB of 1st hop and 2nd hop respectively. In current specification, FH pattern can be determined as the configured values, i.e., the UE transmits the PUCCH starting from a first PRB, provided by startingPRB and starting from a second PRB, provided by secondHopPRB.

Different with PUSCH, both starting RBs of 1st hop and 2nd hop are configured by gNB. Thus, new configuration of first PRB and second PRB can be introduced for SBFD slot.

Based on above discussion, we propose to study enhancement on frequency hopping (both intra-slot and inter-slot) for PUSCH and PUCCH in SBFD operation. Additionally, different frequency hopping patterns can be determined across non-SBFD symbols and SBFD symbols. In non-SBFD symbols, legacy FH pattern should be determined for SBFD aware UE to guarantee frequency diversity as legacy UE.

- Proposal 8: Different frequency hopping patterns can be determined for PUSCH and PUCCH transmission(s) across SBFD symbols and non-SBFD symbols.

Available slot determination for PUSCH/PUCCH repetition

In Rel-17 coverage enhancement WI, enhancement on PUSCH repetition Type A was introduced. Like inter-slot PUCCH repetition case, PUSCH repetition Type A supports repetitions counted based on available slots in 2-step procedure: in the 1st step, the UE determines available slots for K PUSCH repetitions; in the 2nd step the UE determines whether to drop each of the K PUSCH repetition or not according to PUSCH dropping rules, where the PUSCH repetition is still counted in the K repetitions even if it is dropped.

In the 1st step (i.e., available slot determination), a UE determines available slots based on the symbol-level availability. For unpaired spectrum, the slot is determined as not available for PUSCH or PUCCH transmission if at least one of the symbols for PUSCH or PUCCH transmission overlaps with a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. In other words, the slot is determined as available for PUSCH or PUCCH transmission starting in an UL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or in a flexible symbol that is not SS/PBCH block symbol indicated by ssb-PositionsInBurst in consecutive UL symbols indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or in flexible symbols that are not SS/PBCH block symbols indicated by ssb-PositionsInBurst.

Recall the SBFD operation, UL subband can be allocated in a symbol configured as DL or flexible indicated by TDD-UL-DL-ConfigCommon. In this case, a SBFD aware UE may not determine the slot with symbols within UL subband since those symbols are configured as DL and flexible indicated by TDD-UL-DL-ConfigCommon even the set of symbols for PUSCH or PUCCH are available for transmission.

To address this, frequency domain availability can be considered in addition to time domain availability. Thus, the SBFD aware UE can determine the slot as available for PUSCH or PUCCH transmission in the set of symbols within UL subband. Even a symbol for PUSCH or PUCCH transmission overlaps with a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the slot can be determined as available if symbols for PUSCH or PUCCH transmission are scheduled within UL subband.

- Proposal 9: In addition to time domain availability, frequency domain availability can be considered to determine available slot of PUSCH/PUCCH repetition for a SBFD aware UE.

PUSCH rate matching in RE-level

In Rel-15, RE-level rate matching for PDSCH was specified around REs for LTE CRS (cell-specific reference signal) or ZP (zero power) CSI-RS to guarantee LTE UE’s CRS detection for DL channel estimation or other NR UE’s ZP CSI-RS detection for beam management. A UE can be configured with RE-level resource pattern for LTE CRS or ZP CSI-RS, then the UE assumes that those REs are not available for PDSCH reception. If the UE is indicated to receive PDSCH in REs overlapped with configured RE-level resource pattern, the UE receives PDSCH by rate matching REs for PDSCH around the REs for LTE CRS or ZP CSI-RS. Note that LTE CRS and ZP CSI-RS are likely to be configured in DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated.

For SBFD aware UE, UL transmission can be scheduled within UL subband in a symbol configured as DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated. Then, resources of PUSCH scheduled to the SBFD aware UE within UL subband can overlap with REs of LTE CRS or ZP CSI-RS. It can be a more critical issue than Rel-15 PDSCH case due to CLI between DL reception and UL transmission. For this, RE-level rate matching for PUSCH can be studied, and the Rel-15’s RE-level PDSCH rate matching can be a starting point.

- Proposal 10: For a SBFD UE, PUSCH scheduled in UL subband is rate matched around REs of LTE CRS or ZP CSI-RS.

  - Rel-15 RE-level PDSCH rate matching can be a starting point.

HPN determination for multi-slot PUSCH

In Rel-17 60GHz WI, multi-slot PUSCH scheduling with a single DCI was specified, where time domain allocations of multiple PUSCH can be discontinuous. Those PUSCH transmissions in multi-slot have different TBs, thus different HPNs (HARQ process numbers) are determined for each PUSCH transmission. Because a single HPN value is contained in a DCI, HPN determination rule was specified as follows.

When the UE is scheduled with multiple PUSCHs by a DCI, HPN indicated by DCI applies to the first PUSCH not overlapping with a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. HPN is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation. HPN is not incremented if at least one of the symbols for PUSCH or PUCCH transmission overlaps with a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

Similar with available slot determination as we discussed above, enhancement on HPN determination of mutl-slot PUSCH scheduled with a single DCI within UL subband is necessary as showed in Figure 5.

Figure 5. HPN determination for multi-slot PUSCH across SBFD symbols and non-SBFD symbols.

In Figure 5, a SBFD aware UE can be scheduled to transmit multi-slot PUSCH across slot#1 and slot#2 with a single DCI, where slot#1 is consisted of non-SBFD symbols and slot#2 is consisted of SBFD symbols. Even the SBFD aware UE can transmit PUSCH#2 in the slot#2, the HPN of PUSCH#2 would not be incremented according to the current rule. In this case, ambiguity due to the same HPN between the PUSCHs with different TBs can occur, thus it can affect PUSCH HARQ process.

To address this, availability of frequency domain can be considered in addition to time domain availability. Thus, the SBFD aware UE increments HPN for PUSCH transmission in the set of symbols within UL subband. Even a symbol for PUSCH transmission overlaps with a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the HPN can be incremented if symbols for PUSCH transmission are scheduled within UL subband.

- Proposal 11: In addition to time domain availability, frequency domain availability can be considered to determine HPN of multi-slot PUSCH scheduled by a single DCI for a SBFD aware UE.

UE collision handling

In the RAN1#110b-e meeting, it was agreed to identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE, and if there are, study whether/how to avoid/handle such collision cases. We provide our views on cases of time domain conflict and potential UE behavior to avoid/handle collision cases.

Valid RO vs. DL signal/channel

In legacy TDD operation, a UE can determine valid RO (RACH occasion) within UL symbol or within flexible symbol (Ngap symbol after DL and SS/PBCH symbol). For SBFD aware UE, it can be extended to determine valid RO within UL symbol in UL subband.

In this case, collision can occur between valid RO and DL signal/channel as shown in Figure 6.

Figure 6. Collision between valid RO and PDSCH.

In Figure 6, symbols for valid RO and PDSCH are overlapped in time domain. To handle this, new UE behavior for SBFD aware UE can be studied since the above collision case is not identified in legacy TDD operation. Also, different UE behaviors can be studied for the cases whether PDSCH is semi-statically scheduled via RRC configuration or PDSCH is dynamically indicated via DCI. If PDSCH is semi-statically scheduled, PDSCH can be received for the case when PRACH is not transmitted in valid RO. PDSCH is not received for the case when PRACH is transmitted. Alternatively, PDSCH can be rate-matched even when PRACH is transmitted. If PDSCH is dynamically scheduled, PDSCH can be received and PRACH is not transmitted even in valid RO.

DL signal/channel vs. UL signal/channel

Collision cases between DL signal/channel and UL signal/channel can be listed for SBFD aware UE as followings:

Case 1) DCI indicated DL (or UL) signal/channel vs. DCI indicated UL (or DL) signal/channel.

In this case, a SBFD aware UE may not expect to be indicated with DL reception and UL transmission in the same set of symbols. Thus, it can be handled by gNB implementation as in the current TDD operation.

Case 2) Higher layer configured DL (or UL) signal/channel vs. higher layer configured UL (or DL) signal/channel.

In this case, a SBFD aware UE can determine to select one of DL reception or UL transmission. For example, SBFD operation in a symbol configured as DL by tdd-UL-DL-ConfigurationCommon, the UE receives DL signal/channel and does not transmit UL signal/channel since the set of symbols was originally configured as DL. Otherwise, the UE neither receives DL signal/channel nor transmits UL.

Case 3) DCI indicated DL (or UL) signal/channel vs. higher layer configured UL (or DL) signal/channel.

In this case, a UE can prioritize to receive or transmit signal/channel indicated by DCI as in legacy TDD operation (overwriting rule). For example, if a collision occurs between DCI indicated CSI-RS and higher layer configured PUSCH, the UE receives the DCI indicated CSI-RS, and does not transmit the higher layer configured PUSCH as shown in Figure 7 (a). In addition to that, enhancements on SBFD aware UE can be further studied. For example, the UE receives the DCI indicated CSI-RS and transmits the higher layer configured PUSCH by rate-matching around collided symbols including DL-UL switching gap as shown in Figure 7 (b).

Figure 7. Collision between DCI indicated CSI-RS and higher layer configured PUSCH.

- Proposal 12: RAN1 to study following collision cases for SBFD aware UE:

  - Valid RO vs. DL signal/channel.

  - Higher layer configured DL (or UL) signal/channel vs. higher layer configured UL (or DL) signal/channel.

  - DCI indicated DL (or UL) signal/channel vs. higher layer configured UL (or DL) signal/channel.

Conclusion

In this contribution, we discussed subband non-overlapping full duplex and summarize our views as the followings:

- Observation 1: Explicit method (symbol conversion to a DL-only) to allow DL reception outside DL subband(s) has more occasions in time domain to schedule DL reception than implicit method (gNB scheduling) for DL reception outside DL subband(s).

- Proposal 1: RAN1 to study Option 2 for SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

  - SBFD aware UE can be indicated with explicit method (symbol conversion to a DL-only) to allow DL reception outside DL subband(s) via dynamic SFI.

  - FFS: impact and potential enhancements on dynamic SFI for the co-existence of SBFD aware UE and non-SBFD aware UE in a given cell.

  - FFS: impact and potential enhancements on UE behaviors in case of SFI detection/non-detection point of view

- Proposal 2: SBFD aware UE can be indicated with explicit method (symbol conversion to a DL-only) to allow DL reception outside DL subband(s) via dynamic SFI even in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon.

- Proposal 3: Signalling of guardband(s) is not needed for SBFD aware UE.

  - SBFD aware UE can know the frequency location of fixed guardband(s) based on knowledge of the frequency location of UL subband and DL subband(s).

- Proposal 4: SBFD aware UE follows legacy UE behavior in the RBs outside the UL subaand that can be used as either UL, or DL excluding guardband(s) in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

- Proposal 5: Reusing PDSCH rate-matching mechanism to address unaligned boundaries of DL/UL subband and configuration granularity of RBG for PDSCH FDRA (i.e., FDRA type 0) in terms of specification impact.

- Proposal 6: 	Same TBS can be determined among PDSCH receptions with slot aggregation (repetitions) across SBFD symbols and non-SBFD symbols.

- Proposal 7: RAN1 to study Option 3 (one contiguous CSI-RS resource configuration across non-contiguous DL subbands) in terms of specification impact.

  - A UE can assume that PRBs in UL subband (and guardband, if specified) are not available for CSI-RS reception.

- Proposal 8: Different frequency hopping patterns can be determined for PUSCH and PUCCH transmission(s) across SBFD symbols and non-SBFD symbols.

- Proposal 9: In addition to time domain availability, frequency domain availability can be considered to determine available slot of PUSCH/PUCCH repetition for a SBFD aware UE.

- Proposal 10: For a SBFD UE, PUSCH scheduled in UL subband is rate matched around REs of LTE CRS or ZP CSI-RS.

  - Rel-15 RE-level PDSCH rate matching can be a starting point.

- Proposal 11: In addition to time domain availability, frequency domain availability can be considered to determine HPN of multi-slot PUSCH scheduled by a single DCI for a SBFD aware UE.

- Proposal 12: RAN1 to study following collision cases for SBFD aware UE:

  - Valid RO vs. DL signal/channel.

  - Higher layer configured DL (or UL) signal/channel vs. higher layer configured UL (or DL) signal/channel.

  - DCI indicated DL (or UL) signal/channel vs. higher layer configured UL (or DL) signal/channel.

References

- Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0

- 3GPP TS 38.214 V17.4.0 (2022-12)