3GPP TSG RAN WG1 Meeting #112	        		   R1-2301749

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDCCH skipping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2023-2-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2#120 meeting, a LS (R2-2213349) from RAN2 is sent to RAN1. The details of the LS are provided as following. RAN2 thanks RAN1 for the LS and would like to provide following answer and agreements. Question: What is the meaning of “ignores” in the sentence “UE ignores PDCCH skipping while contention resolution timer is running.” quoted from the previous RAN2 LS to RAN1 on PDCCH skipping in RRC_CONNECTED (R2-2201960)? RAN2 answer: The sentence “UE ignores PDCCH skipping while contention resolution timer is running.” indicates that the UE shall monitor PDCCH regardless of skipping command on SpCell while contention resolution timer is running. RAN2 has agreed the attached CR to remove the cases for ignoring PDCCH skipping in stage 2 specification. RAN2 has further discussed the PDCCH skipping aspects and have agreed that the following cases are missing in the RAN1 specification: - UE shall monitor PDCCH regardless of previously received skipping command on SpCell after successful contention resolution. - UE shall monitor PDCCH regardless of skipping command on all serving cells of the corresponding Cell Group if UE detect the PDCCH skipping command after SR is sent and is pending. - UE shall monitor PDCCH regardless of previously received skipping command on all serving cells of the corresponding Cell Group after the pending SR is cancelled.. In this contribution, we capture the above cases that were missed into the RAN1 specification. |
| Summary of change: |  | Capture the following cases into the RAN1 specification: - UE shall monitor PDCCH regardless of previously received skipping command on SpCell after successful contention resolution. - UE shall monitor PDCCH regardless of skipping command on all serving cells of the corresponding Cell Group if UE detect the PDCCH skipping command after SR is sent and is pending. - UE shall monitor PDCCH regardless of previously received skipping command on all serving cells of the corresponding Cell Group after the pending SR is cancelled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some cases are missed in specification. Incomplete support for UE power saving enhancements in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

## 10.4	Search space set group switching and skipping of PDCCH monitoring

*** Unchanged text is omitted ***

When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. If a DCI format providing the PDCCH monitoring adaptation field is indicated to the UE to skip PDCCH monitoring for a duration on the active DL BWP of the serving cell after the UE transmits a PUCCH providing a positive SR, the UE monitors PDCCH in all serving cells of the corresponding Cell Group and ignores the indication of the PDCCH skipping indication when the SR is pending [11, TS 38.321]. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. If contention resolution succeedssss [11, TS38.321] after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for a duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring on SpCell after the successful contention resolution. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell.

*** Unchanged text is omitted ***