3GPP TSG-RAN WG1 Meeting #112	R1-2301765

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on RAN3 LS on RB set configuration

Document for:	Discussion, Decision

# Introduction

RAN1 agreed the following with respect to frequency domain H/S/NA configuration:

Agreement (RAN1#106-e) A Reference SCS is configured for frequency domain H/S/NA configuration. Agreement (RAN1#108-e) The start RB index of the first RB set for the Rel-17 IAB-DU HSNA resource configuration is the lowest index of RB of the IAB-DU cell

RAN3 sent an LS to RAN1 [1] expressing uncertainty on how to define the start RB index for the Rel-17 IAB-DU HSNA resource configuration of an IAB-DU cell.

RAN3 has two different understandings:

- Understanding 1: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the SCS-specific offset defined in the NR Carrier List IE based on the SCS included in the RB Set Configuration IE.

- Understanding 2: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the smallest SCS-specific frequency offset across all numerologies provided by the NR Carrier List IE for this cell. Note that in this understanding, the start RB index does not depend on the SCS value included in the RB Set Configuration IE in TS 38.473.

In both RAN3 understandings, the start RB is aligned with point A in case the NR Carrier List IE is not present.

RAN3 asks for clarification on which of the two understandings is correct.

In this paper, we present our position in the above matter. A draft reply LS is further provided in [2].

# Discussion

RAN1 agreed that the start RB index of the first RB set for the Rel-17 IAB-DU HSNA resource configuration is the lowest index of RB of the IAB-DU cell. The F1AP signalling for configuring RB sets needs to reflect this. RAN1’s intention when taking the agreement on the start (in frequency) of Rel-17 IAB-DU HSNA was to ensure that the whole spectrum used for transmission is configured with frequency HSNA, and no frequency resource is left out. The latter is clear when considering the agreement from RAN1 #109 “If the RB sets of a Rel-17 IAB-DU H/S/NA resource configuration do not cover the entire carrier bandwidth: The remaining RBs not part of an RB set configuration are considered as included in the last RB set.”

RAN1’s intention when agreeing on the first and last frequency resources included in Rel-17 IAB-DU HSNA resource configuration was to include all transmission resources in the configuration.

IAB-DU cell configuration is contained in Served Cell Information IE, in clause 9.3.1.10 of TS 38.473 [3]. Table 1 lists the relevant parts of this IE. Consequently, All IEs below the NR PCI IE, including the Transmission Bandwidth and Carrier List IEs, pertain to the same IAB-DU cell.

Table 1: Excerpt from clause 9.3.1.10 of TS 38.473

| NR CGI | M |  | 9.3.1.12 |  | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>>NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |

The configuration in clause 9.3.1.10 of TS 38.473, including the IEs Transmission Bandwidth as well as NR Carrier List pertain to one and only one gNB-DU cell.

Clause 5.3 of TS 38.104 [4] explains the relations between channel bandwidth, transmission bandwidth, and configuration(s) of numerologies. In TS 38.473 [3], the cell configuration information of numerologies is provided either via Transmission Bandwidth IE (clause 9.3.1.15), or via NR Carrier List IE (9.3.1.137), in case of a single or multiple numerology configured for a cell (and multiplexed in an OFDM symbol, according to TS 38.104), respectively.

The F1AP Transmission Bandwidth IE (clause 9.3.1.15) configures a single numerology to a single IAB-DU cell.

The F1AP NR Carrier List IE (clause 9.3.1.137) configures multiple numerologies to a single IAB-DU cell.

Since TS 38.300 [5] clause 3.2 states that a numerology “corresponds to one subcarrier spacing in the frequency domain”, we use the terms numerology and SCS interchangeably. TS 38.211 specifies physical resource blocks (PRB) for a certain sub-carrier spacing only within a bandwidth part (BWP). Since the RAN3 question is concerning a DU configuration, we disregard any consideration or properties of PRBs.

## IEs for IAB-DU cell configuration with a single numerology

Figure 5.3.3-1 in TS 38.104 [4] shows how an IAB-DU cell is configured with only one numerology. IE Transmission Bandwidth (9.3.1.15, Table 2) provides a numerology (IE NR SCS) and information about the transmission bandwidth (IE NRB). IE NRB is denoted in units of RBs where an RB always amounts to 12 subcarriers, i.e., the RB bandwidth is always 12x(NR SCS). Figure 1 illustrates the application of IEs NR SCS and NRB.

Table 2: Excerpt of IE Transmission Bandwidth, clause 9.3.1.15 of TS 38.473

| NR SCS | M |  | ENUMERATED (…) | The values scs15, scs30, scs60, scs120, scs480 and scs960 corresponds to the sub carrier spacing in TS 38.104 [17]. |
| --- | --- | --- | --- | --- |
| NRB | M |  | ENUMERATED (…) | This IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [17]). The values nrb11, nrb18, etc. correspond to the number of resource blocks "NRB" 11, 18, etc. |

Figure 1: IAB-DU Cell configuration with one numerology by IE Transmission Bandwidth (9.3.1.15)

## IEs for IAB-DU cell configuration with multiple numerologies

Figure 5.3.3-2 in TS 38.104 [4], shows how an IAB-DU cell configuration is done when “multiple numerologies are multiplexed in the same symbol”. The F1AP NR Carrier List IE (clause 9.3.1.137, shown in Table 3 below) provides for each (what TS 38.473 [3] calls a) SCS-specific carrier: a numerology (NR SCS IE), the offset between a reference point (Point A) and the lowest sub-carrier (Offset to Carrier IE), and the frequency extent of the configured numerology (Carrier Bandwidth IE).

Figure 2 illustrates the application of IEs NR SCS, Offset to Carrier, and Carrier Bandwidth.

Table 3: IE NR Carrier List, clause 9.3.1.137 of TS 38.473, “This IE indicates the SCS-specific carriers per TDD, per DL, per UL or per SUL of an NR cell.”

| >NR SCS | M |  | ENUMERATED (…) | SCS for the corresponding carrier. |
| --- | --- | --- | --- | --- |
| >Offset to Carrier | M |  | INTEGER (0…) | Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the NR SCS IE defined for this carrier). The maximum value corresponds to 275×8−1. See TS 38.211 [33], clause 4.4.2. |
| >Carrier Bandwidth | M |  | INTEGER (1…) | Width of this carrier in number of PRBs (using the NR SCS IE defined for this carrier). See TS 38.211 [33], clause 4.4.2. |

Figure 2: IAB-DU Cell configuration with multiple (2) numerologies by NR Carrier List IE (9.3.1.137). Two offsets indicated between ‘Point A’ and the beginning of a first (green) and second (blue) numerology.

## The lowest RB index of the IAB-DU cell

Figure 1 and Figure 2 depict how an IAB-DU cell is configured when either Transmission Bandwidth IE or NR Carrier List IE is provided.

As can be observed, regardless of which numerologies are configured for a cell, there is a lowest frequency resource. This lowest frequency is unique and determining the lowest index of RB of the IAB-DU cell. The numerology of this lowest RB might have an impact on the width of this lowest RB, but it has no impact on the fact that there is a lowest RB in a cell, which RB it is, and where it starts. Figure 3 illustrates the lowest RB index of the IAB-DU for both cases of when an IAB-DU cell is configured with numerologies by Transmission Bandwidth IE or NR Carrier List IE. The RB that starts the first RB set for the Rel-17 IAB-DU HSNA resource configuration is indicated in red (depicted as RB 0). The start RB index for the Rel-17 IAB-DU HSNA resource configuration depends on Point A plus the smallest SCS-specific frequency offset across all numerologies (in case multiple numerologies are configured). If it does not depend on the smallest SCS-specific frequency offset, there is an index of an RB of the IAB-DU cell with lower index than the start RB index of the first of the first RB set. That would violate the RAN1 agreement.

Figure 3: Location of the lowest RB (in red) of the IAB-DU cell with one numerology (top) and for an IAB-DU cell with multiple numerologies (bottom)

Regardless of which numerologies are configured for a cell, for a given cell configuration, there is a unique lowest frequency resource, determining the lowest RB index of the IAB-DU cell.

The lowest index of RB of the IAB-DU cell only depends on the location of Point A, the subcarrier spacing of the first numerology (i.e., with lowest location in spectrum) and its signalled carrier offset from Point A.

If the start RB index for the Rel-17 IAB-DU HSNA resource configuration does not depend on the smallest SCS-specific frequency offset (given NR Carrier List IE), there may be an index of an RB of the IAB-DU cell with lower index than the start RB index of the first of the first RB set. That would violate the RAN1 agreement.

## On the use of the reference SCS in the RB set configuration

The description for the Subcarrier Spacing IE in clause 9.3.230 of TS 38.473 [3], states that “Subcarrier spacing used as reference for the RB set configuration” (Table 4).

Table 4: Excerpt from IE RB Set Configuration, clause 9.3.1.230 of TS 38.473

| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. |
| --- | --- | --- | --- | --- |

This subcarrier spacing parameter is used as the reference frequency unit (in kHz, represented in configurable SCS units) for the configuration of RB sets. It is not required that this SCS is used for scheduling resources or creating OFDM symbols for transmission to/from UEs. For example, an IAB-DU cell can be configured to transmit OFDM symbols based on an SCS of 15kHz. However, RB sets – for HSNA frequency domain configuration – can be configured assuming RBs based on a reference SCS of 30, 60 or even 120kHz.

This is completely in line with the resource configuration of the cells served by an IAB-DU, i.e., the TDD/FDD resource parameters for each activated cell (TS 38.473 [3], 9.3.1.107). Table 5 provides an excerpt highlighting the relation of reference SCS and the meaning of what is a Number of DL symbols (in IE gNB-DU Cell Resource Configuration, Table 5) – for the configuration of transmission symbols, a time unit is required. This time unit is expressed by the reference SCS equivalent to a reference OFDM symbol duration. It is probably a good practice to configure such reference SCS for TDD configuration to the one used for actual OFDM symbol transmission, if only one numerology is configured (e.g., IE Transmission Bandwidth). However, the need for a reference time unit/reference SCS for TDD configuration becomes evident if a cell is configured with multiple numerologies that are multiplexed into the same OFDM symbol with a single common OFDM symbol duration. A TDD configuration is not done per numerology but for a cell – otherwise a cell cannot have a single common OFDM symbol duration.

A TDD configuration is done per cell using a reference SCS.

An RB set configuration is done per cell using a reference SCS.

Table 5: Excerpt from IE gNB-DU Cell Resource Configuration, clause 9.3.1.107 of TS 38.473

| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| DUF Slot Configuration List |  | 0..1 |  |  |  |  |
| >DUF Slot Configuration Item |  | 1..<maxnoofDUFSlots> |  | The maxNrofSlots in TS 38.331 [8]. | - |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  | - |  |
| >>>Explicit Format |  |  |  |  | - |  |
| >>>>Number of Downlink Symbols | O |  | INTEGER (0..14) |  | - |  |
| >>>>Number of Uplink Symbols | O |  | INTEGER (0..14) |  | - |  |

Figure 4 illustrates the configuration of RB sets assuming RB set reference SCS (9.3.1.230) four times a common OFDM symbol SCS (9.3.1.15) and RB Set size (9.3.1.230) of RB2 (2). Example combination of SCS could be a 30kHz OFDM symbol SCS and 120kHz RB set configuration reference SCS.

Figure 4: RB set configuration with RB set reference SCS (9.3.1.230) four times a common OFDM symbol SCS (9.3.1.15) and RB Set size (9.3.1.230) of RB2 (2).

Any gNB cell has a lowest index of RB for scheduling transmissions – this does not depend on any IAB configuration. The new IAB-specific reference Subcarrier Spacing, i.e., the SCS included in the IE RB Set Configuration, will not change the cell’s RB configuration, which is completely determined by either IE Transmission Bandwidth or IE NR Carrier List. Since Understanding 1 implies that the IAB-specific reference SCS has impact on what is the lowest RB in a cell, Understanding 1 cannot be a correct reflection of the RAN1 agreement.

Understanding 1 implies that the IAB-specific reference SCS has impact on what is the lowest RB in a cell, although the lowest RB in a cell is defined outside the IAB context. Hence, Understanding 1 cannot be a correct reflection of the RAN1 agreement.

The start RB index of the first RB set is the lowest RB index of the IAB-DU cell, and it uniquely determines the start of an RB set configuration in frequency domain. When the NR Carrier List configuration is not present, it has no impact on the question of what the RB of the IAB-DU cell with the lowest index is.

Reply to RAN3 that the start RB index for the Rel-17 IAB-DU HSNA resource configuration depends on Point A plus the smallest SCS-specific frequency offset across all numerologies (in case multiple numerologies are configured).

# Conclusion

In the previous sections we made the following observations:

Observation 1	RAN1’s intention when agreeing on the first and last frequency resources included in Rel-17 IAB-DU HSNA resource configuration was to include all transmission resources in the configuration.

Observation 2	The configuration in clause 9.3.1.10 of TS 38.473, including the IEs Transmission Bandwidth as well as NR Carrier List pertain to one and only one gNB-DU cell.

Observation 3	The F1AP Transmission Bandwidth IE (clause 9.3.1.15) configures a single numerology to a single IAB-DU cell.

Observation 4	The F1AP NR Carrier List IE (clause 9.3.1.137) configures multiple numerologies to a single IAB-DU cell.

Observation 5	Regardless of which numerologies are configured for a cell, for a given cell configuration, there is a unique lowest frequency resource, determining the lowest RB index of the IAB-DU cell.

Observation 6	The lowest index of RB of the IAB-DU cell only depends on the location of Point A, the subcarrier spacing of the first numerology (i.e., with lowest location in spectrum) and its signalled carrier offset from Point A.

Observation 7	If the start RB index for the Rel-17 IAB-DU HSNA resource configuration does not depend on the smallest SCS-specific frequency offset (given NR Carrier List IE), there may be an index of an RB of the IAB-DU cell with lower index than the start RB index of the first of the first RB set. That would violate the RAN1 agreement.

Observation 8	A TDD configuration is done per cell using a reference SCS.

Observation 9	An RB set configuration is done per cell using a reference SCS.

Observation 10	Understanding 1 implies that the IAB-specific reference SCS has impact on what is the lowest RB in a cell, although the lowest RB in a cell is defined outside the IAB context. Hence, Understanding 1 cannot be a correct reflection of the RAN1 agreement.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reply to RAN3 that the start RB index for the Rel-17 IAB-DU HSNA resource configuration depends on Point A plus the smallest SCS-specific frequency offset across all numerologies (in case multiple numerologies are configured).

# References

R1-2300006 (R3-226781), “LS on RB set configuration,” RAN3, RAN1 #112, Athens, Greece, February 2023.

R1-2301764, “Draft Reply LS on RB set configuration,” Ericsson, RAN1 #112, Athens, Greece, February 2023.

TS 38.473, “NR; F1 application protocol (F1AP) (Release 17)”, 3GPP, V17.3.0, December 2022.

TS 38.104, “NR; Base Station (BS) radio transmission and reception (Release 17)”, 3GPP, V17.8.0, December 2022.

TS 38.300, “NR; NR and NG-RAN Overall description; Stage 2 (Release 17)”, 3GPP, V17.3.0, December 2022.